3GPP TSG-RAN WG4 Meeting #112	R4-2413281

Maastricht, NL, 19 - 23 August, 2024

Source: 	Huawei, HiSilicon

Title: 	AAS modelling considerations for IMT base stations

Agenda Item:	8.2.5

Document for:	Approval

1 Introduction

WP 5D sent a LS to 3GPP requesting information to make their compatibility studies (R4-2400333), if available, already by June 2024 and final submission June 2025 if the studies are extended.

Considering the system-level simulation work typically conducted in ITU-R WP 5D for sharing and compatibility studies using 3D modelling of antenna pattern, ITU-R WP 5D is seeking information on whether for implementation of IMT BS multiuser spatial beamforming techniques, such as zero-forcing (ZF) or minimum mean-square error (MMSE) based schemes, would be necessary to improve the assessment of interference and the accuracy of studies. If considered necessary, guidance on the process of deriving the necessary beamforming weights for the IMT AAS BS to compute its radiation pattern.

In this paper, we implement zero-forcing (ZF) multiuser spatial beamforming method and evaluate the antenna radiate pattern towards to a number of interfered-with receivers located in the upper hemisphere. Based on the simulation results, ZF impact on the AAS BS gain towards potential interfered-with receivers is minor and the interference is even less than M.2101.

2 ZF simulation results

Zero-forcing (ZF) beamforming is an appealing solution as it has a moderate computational complexity while achieving spatial multiplexing. This technique decouples the multi-user channel into multiple independent subchannels by nullifying unwanted signals in the direction of other users, ensuring that each user receives their intended signal with minimal interference. Mathematically, the ZF precoder can be derived as the pseudo-inverse of the channel matrix:

| WZF=HHHHH-1 |  |
| --- | --- |

where H represents the channel matrix with its number of rows corresponding to the number of Rx antennas (i.e., UEs) and its number of columns corresponding to the number of Tx antennas (BS antenna ports), and HH is the conjugate transpose of H.

In this study, we normalize ZF precoder by diving its column wise Fresenius norm squared. Furthermore, individual per-antenna power constraints are also considered since each transmit antenna has its own power amplifier limitation in practise.

Channel matrix H between BS and UE link is normally assumed line-of-sight and then AAS beam is formed towards to its serving UEs in ITU-R sharing study. In the paper, the channel matrix H used for ZF beamforming follows similar assumptions and assume all BS and UE links are line-of-sight. Channel matrix H can be generated reflecting the Azimuth/ Zenith angle Of Departure (AoD/ZoD) from each BS to its serving UEs.

Figure 1

Geometry distribution of AAS with multiple columns array

dH

z

u

dv

φ

θ

x

y

……

In order to assess the necessity and accuracy of interference comes from IMT systems for ITU-R sharing and compatibility studies, the IMT BS gain towards a number of potential interfered-with receivers located in the upper hemisphere are simulated. The angular separation between sample points is assumed 10 degrees.

Figure 2

10-degree sample points in the upper hemisphere above the BS

The cumulative distribution functions (CDFs) curves of the IMT BS gain towards the sample points of the different scenarios are shown in Figures 3. It can be seen that the difference between ITU-R M.2101 and ZF gains are minor.

Figure 3

BS gain towards all sample points (composite)

To assess the impact on long- and short-term statistics, the 80th percentile and 99.9th percentile are considered respectively. Table 1 contains the summary of the BS antenna gain difference between ITU-R M.2101 model and ZF beamforming model. It shows that ZF beamforming get slightly less antenna gain than M.2101 towards to the sample points:

TABLE 1

BS antenna gain difference between ITU-R M.2101 beamforming model and ZF beamforming

| C band Urban (with subarrays) Scenario | Long-term (80th percentile) | Short-term (99.9th percentile) |
| --- | --- | --- |
| M.2101 | -13.720 | 20.861 |
| ZF | -15.526 | 19.903 |
| M.2101- ZF difference (dB) | 1.806 | 0.958 |

Observation: multiuser spatial beamforming zero-forcing (ZF) impact on the AAS BS gain towards potential interfered-with receivers located in the upper hemisphere is minor and the interference to the receiver sample points is even less.

Deployment related parameters of BSs and UEs are based on the agreed technical and operational characteristics in the document 5D/716 (Annex 4.4 - WRC-23), and beamforming antenna characteristics are referred to 3GPP liaison statement on parameters for 4400 to 4800 MHZ of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-27, see 5D/136. More detail simulation methods and assumptions are provided in the Attachment.

3 Proposal

The simulation results show multiuser spatial beamforming zero-forcing (ZF) impact on the AAS BS gain towards potential interfered-with receivers located in the upper hemisphere is minor and the interference to the receiver sample points is even less.

In theory, AAS beam is firstly shaped by 1*3 sub-array weighting which is already covered by extended AAS modelling for sub-array. Additional ZF or M.2101 weighting is still in the same antenna envelop formed by extended AAS model. In order to achieve the zero-forcing to other serving UEs direction, the target serving UE’s antenna gain will be influenced and the whole antenna array power is also very difficult to be fully used, so the modelling method in M.2101 can be considered as the upper bound of power utilization.

Based on the simulation and theory analysis, there is no strong motivation to change the beamforming assumptions in Recommendation ITU-R M.2101.

Proposal 1: there is no strong motivation to change the beamforming assumptions in Recommendation ITU-R M.2101.

Attachment

1	IMT BS AAS related parameters

Table 2 contains the deployment related parameters of BSs and UEs based on the agreed technical and operational characteristics in the document 5D/716 (Annex 4.4 - WRC-23), and Table 3 contains the beamforming antenna characteristics in 3GPP liaison statement on parameters for 4400 to 4800 MHZ of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-27, see 5D/136.

TABLE 2

IMT deployment-related parameters

|  | Urban macro |
| --- | --- |
| Base station |  |
| Carrier frequency | 4.5 GHz |
| Channel bandwidth | 100 MHz |
| BS Antenna height | 20 m |
| Cell radius | 400 m |
| Sectorization | 1 sector (Note 1) |
| Frequency reuse | 1 |
| User terminal |  |
| UE height | 1.5 m |
| UE density for terminals that are transmitting simultaneously | 3 UEs per sector |
| Note 1: This study considers only a single-entry scenario. |  |

TABLE 3

IMT base station beamforming antenna characteristics for IMT

|  | Urban Macro |
| --- | --- |
| Antenna pattern | Refer to the extended AAS model in 5D/716 (Annex 4.4 - WRC23) |
| Element gain (incl. Ohmic loss) (dBi) | 6.4 |
| Horizontal/vertical 3 dB beamwidth of single element (degree) | 90º for H 65º for V |
| Horizontal/vertical front-to-back ratio (dB) | 30 for both H/V |
| Antenna polarization | Linear ±45º |
| Antenna array configuration (Row × Column) | 4 × 8 (Note 1) |
| Horizontal/Vertical radiating sub-array spacing | 0.5 of wavelength for H 2.1 of wavelength for V |
| Number of element rows in sub-array | 3 |
| Vertical radiating element spacing in sub-array | 0.7 of wavelength |
| Pre-set sub-array downtilt (degrees) | 3 |
| Array Ohmic loss (dB) | 2 |
| Base station maximum coverage angle in the horizontal plane (degrees) | ±60 |
| Base station vertical coverage range (degrees) | 90-100 (Note 2) |
| Mechanical downtilt (degrees) | 6 |
| Note 1: For the sake of completeness, additional results are provided with no subarrays. By keeping the same physical dimensions, an antenna array of size 12 × 8 elements (Row × Column) is considered. Note 2: The vertical coverage range includes the mechanical downtilt. A minimum BS-UE distance along the ground of 35 m should be used for urban/suburban and rural macro environments. |  |

An extended version of the AAS array antenna model is created to support vertical sub-array geometries with fixed sub-array down-tilt. The model equations are summarized in below Table 4.

Table 4

Extended AAS model

| Description | Equation |
| --- | --- |
| Peak normalized element radiation pattern | Aθ,φ=-min--min12φφ3dB2,Am-min12θ-90θ3dB2,SLAv ,Am |
| Peak gain normalized element radiation pattern | AEθ,φ=GE,max+Aθ,φ |
| Sub-array excitation | wm=1Msubexpj2πm-1dv,subλsinθsubtilt |
| Sub-array radiation pattern | Asubθ,φ=AEθ,φ+10log10m=1Msubwmvm2 , where vm=expj2πm-1dv,subλcosθ |
| Array excitation | wm,n=1MNexpj2πm-1dvλsinθetilt-n-1dhλcosθetiltsinφescan Where M and N is corresponding to (Row × Column). |
| Composite array radiation pattern | AAθ,φ=Asubθ,φ+10log10m=1Mn=1Nwm,nvm,n2 , where vm,n=expj2πm-1dvλcosθ+n-1dhλsinθsinφ Where M and N is corresponding to (Row × Column). |

The IMT network consists of a single site with a single sector. In each snapshot of the Monte Carlo simulation, 3 UEs are uniformly distributed within the sector, and the BS serves each UE with a dedicated beam.

Figure 4

Uniform UE distributions

A minimum 35m BS-UE distance along the ground is simulated and the vertical angle distribution is plotted below.

Figure 5

Probability density function (PDF) of the elevation angle from the BS to the UEs

The max vertical steering angle is considered if the deployed UEs are outside of this limitation provided in Table 3.

3	Configuration of minimum angular separation for ZF

In order to evaluate the influence of minimum azimuth angular separation between serving UEs, AAS beamforming gain towards to target serving UE under different minimum angular separation assumptions is simulated. It’s obvious ZF will decrease the target serving UE RX gain, the larger minimum angular separation the less influence.

Figure 6

AAS beamforming gain towards to target serving UE

Minimum 5 degrees angular separation        Minimum 10 degrees angular separation

Minimum 20 degrees angular separation        Minimum 40 degrees angular separation

In order to avoid more than 2 dB antenna gain decrease of 50% probability, this study set minimum 20 degrees as the default assumption.

4	Effectiveness of the ZF beamforming technique

In this section, we validate the effectiveness of the ZF beamforming technique, Figure 7 shows the BS gain towards the co-scheduled UEs when serving a specific UE. As expected, ZF reduces the BS gain towards the co-scheduled UEs.

Figure 7

BS gain towards the co-scheduled UEs when serving a specific UE

References

[1] ITU-R M.2101 Modelling and simulation of IMT networks and systems for use in sharing and compatibility studies, 02/2017

[2] ITU-R 5D/158 Assessment of the effect of zero-forcing on gain distribution for active antenna of IMT BS, France, New Zealand, 11 June 2024

[3] ITU-R 5D/197 Consideration for the active antenna array radiation pattern model for IMT Base Stations, Telefon AB - LM Ericsson, 14 June 2024

[4]	TR 37.842	3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Evolved Universal Terrestrial Radio Access (E-UTRA) and Universal Terrestrial Radio Access (UTRA; Radio Frequency (RF) requirement background for Active Antenna System (AAS) Base Station (BS) (Release 13)

[5]	TR 37.840	3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Study of Radio Frequency (RF) and Electromagnetic Compatibility (EMC) requirements for Active Antenna Array System (AAS) base station (Release 12)