3GPP RAN WG4 Meeting #112		R4-2413282

# Maastricht, Netherlands, 19th – 23rd August, 2024

Source: 	Huawei, HiSilicon

Title: 	RF requirements for Ambient IoT BS

Agenda Item:	8.20.3.1

Document for:	Approval

1 Introduction

The new SID on solutions for Ambient IoT was approved at the RAN#102 meeting [1] and later revised [2] at the RAN#103 meeting. The RAN4-led objectives are copied as below:

- RAN4-led:

  - Coexistence study of Ambient IoT and NR/LTE.

  - RF requirements study for Ambient IoT:

    - Ambient IoT BS transmission and reception

    - Ambient IoT Device, as per the General Scope, transmission and reception

    - Intermediate node (UE), as per the General Scope, transmission and reception

In this contribution, we provide some initial discussion on Ambient IOT BS RF requirements.

2 Discussion

2.1 A-IoT System Parameters

Issue 1-1: System parameter

Agreement:

- RAN4 will define the D2R and/or R2D channel bandwidth and operating bands for A-IoT

  - Wait for the conclusions from other WGs to discuss the detailed parameters.

  - FFS on whether to have the same or different channel bandwidths for devices, BS and intermediate node

As per the SID [1], the operating bands are limited to FR1 licensed spectrum in FDD. Furthermore, the centre frequency agreed for RAN1’s evaluation is 900MHz (mandatory) [2].

As per WF [2], on the D2R and/or R2D channel bandwidth RAN4 will wait for the conclusions form other WGs to discuss the detailed definitions.

As per the SID [1], the spectrum deployment includes: in-band to NR, in guard-band to LTE/NR, in standalone band(s). Since there would be no cell search on the device side, sync raster is not needed. For channel raster, although it may be transparent to devices which use RF-ED receiver, for RF requirements it might be needed. For in-band to NR spectrum, Ambient IOT will take use of one or multiple PRBs of NR spectrum, and correspondingly NR need to blank the PRBs for co-existence. 100 kHz channel raster is defined he legacy FR1 bands below 3 GHz and in Rel-18 enhanced channel raster is introduced to handle the odd-even transmission configuration issue (e.g. 20 MHz redcap UE in 25 MHz network). Hence, enhanced channel raster (i.e. 10 kHz) can be reused to support the flexible spectrum deployments for Ambient IOT.

Proposal 1: RAN4 to study the necessity of channel raster for Ambient RF requirement. If needed, it is proposed to reuse the enhanced channel raster (i.e. 10 kHz)

2.2 BS RF

For the ambient IOT BS RF requirement, since ambient IOT is operating within an LTE/NR channel, for the requirements related to out of LTE/NR channel, the existing BS RF requirements can be used as starting point. Within the LTE/NR channel, we need further consider the ACLR/ACS requirement based on the co-existence study. In the following we look at each TX and RX requirement for ambient IOT BS.

BS type

For D1T1 deployment is indoor scenarios. An indoor BS is typically deployed by ceiling installation, especially for the scenarios of factory or warehouse. The main requirement is wide beam width on the hemispherical surface, so beam forming does not provide benefits for indoor deployment. And the target IMT bands are low frequency band, e.g. 800/900 MHz. It is proposed to define BS type 1-C for Ambient-IoT indoor scenarios.

Proposal 2: It is proposed to define BS type 1-C for Ambient-IoT indoor scenarios.

Issue 2-4: TX

Agreement:

- Use the following table as starting point for RF requirements impact study. The table is for information.

| RF Requirement for A-IoT BS- TX part |  |  |  |
| --- | --- | --- | --- |
| TX requirement | Transmit output power | Maximum output power |  |
|  | Output power dynamic |  |  |
|  | Transmit ON/OFF power |  |  |
|  | Transmission times |  |  |
|  | Transmit signal quality | Frequency error |  |
|  |  | EVM |  |
|  |  | TAE |  |
|  | Unwanted emissions | Occupied bandwidth |  |
|  |  | SEM |  |
|  |  | ACLR |  |
|  |  | Operating band unwanted emissions |  |
|  |  | Transmitter spurious emissions |  |
|  | Transmitter intermodulation |  |  |

Base station output power

BS output power is a key parameter for coverage. In existing NR specification, the rated carrier output power of the BS type 1-C is 38 dBm for MR BS, which is characterised by requirements derived from Micro Cell scenarios.

Table 6.2.1-1: BS type 1-C rated output power limits for BS classes

| BS class | Prated,c,AC |
| --- | --- |
| Wide Area BS | (Note) |
| Medium Range BS | ≤ 38 dBm |
| Local Area BS | ≤ 24 dBm |
| NOTE: There is no upper limit for the Prated,c,AC rated output power of the Wide Area Base Station. |  |

The above rated output power limits for Medium range BS is applicable to Ambient IOT BS for D1T1 deployment scenarios.

As a reference, the UHF RFID reader can transmit with 35 dBm EIRP, according to the ISM band regulation. From the view of better coverage than UHF RFID, it is reasonable to assume the transmit power of Ambient IoT BS to be no lower than that of the RFID reader, e.g. 33 dBm transmit power and 2 dBi antenna gain.

Proposal 3: The existing rated output power limits for NR Medium range BS is applicable to Ambient IOT BS for D1T1 deployment scenarios.

Output power dynamic

The general output power dynamics are not applicable to Ambient IOT BS.

Deployment scenarios for D1T1 was discussed in previous meetings. Option 2-1 scenario is AIOT and NR BS share the hardware. From the study in [R4-2413030], it is shown that power boosting need be larger than 24 dB for device 1 and 2a to decode the R2D signal within certain conditions. But we know current specification define a 6 dB power boosting for NB-IOT in-band operation. It is a huge gap for BS to support that A-IOT and NR BS share the hardware to generate the two RATs with 24 dB power boosting. Hence the power boosting for BS support both Ambient IOT and NR can be discussed in future release.

Proposal 3: The power boosting for BS support both Ambient IOT and NR can be discussed in future release.

Transmit ON/OFF power

It is a TDD requirements and not applicable to Ambient IOT BS.

Transmission times

The on-duration and the off-duration requirement is defined for RFID. More discussion is covered below for transmit signal quality.

Transmit signal quality

For R2D link, OOK waveform generated by OFDM modulator. Pulse interval encoding (PIE) and Manchester encoding have been agreed as the study baseline line code candidates for the R2D transmission. Device 1 and Device 2a use RF envelope detection to decode the R2D signal. The RF envelope requirement might be needed to guarantee the decoding performance.

Figure 2-1. Manchester encoding and decoding

Figure 2-2. PIE encoding and decoding

As a reference, RFID uses ASK modulation with PIE data encoding. The ASK modulation mask was defined as shown in Figure 2-3 below. The pulsewidth (PW), Modulation Depth ((A–B)/A), RF Envelope Ripple (M), RF envelope Rise time (tr,10–90), and RF envelope fall time (tf,10–90) are the major requirements.

Figure 2-3: Interrogator-to-Tag RF envelope

Proposal 4: it is proposed to define RF envelope mask as the transmit signal quality requirement.

Adjacent Channel Leakage power Ratio

ACLR can be defined based on the co-existence study

Operating band unwanted emissions

Need more study for OBUE after ACLR is defined.

Spurious emission

Existing spurious emission requirement is applicable

Transmitter intermodulation

Traditionally the transmitter intermodulation requirement was created to guarantee emission levels in a co-location scenario. The requirement is based on robustness against injection of a reverse interference signal from a co-located BS. for indoor deployment. For the same band it is not possible to have co-location transmitter since it will have interference to the device on the RF-ED. Hence no need to define transmitter intermodulation for A-IOT BS.

Proposal 5: no need to define transmitter intermodulation for A-IOT BS

Based on the above discussion, a summary is captured in the Table

| RF Requirement for A-IoT BS- TX part |  |  |  |
| --- | --- | --- | --- |
| TX requirement | Transmit output power | Maximum output power | The rated output power limits for Medium range BS is applicable to Ambient IOT BS |
|  | Output power dynamic |  | The general output power dynamics are not applicable to Ambient IOT BS. The power boosting for BS support both Ambient IOT and NR can be discussed in future release. |
|  | Transmit ON/OFF power |  | Not applicable |
|  | Transmission times |  | Not applicable. More discussion is covered below for signal quality. |
|  | Transmit signal quality | Frequency error | Reused from NR BS |
|  |  | EVM | to define RF envelope mask as the transmit signal quality requirement. |
|  |  | TAE | N/A |
|  | Unwanted emissions | Occupied bandwidth | Channel bandwidth for Ambient IOT might be needed. |
|  |  | SEM | Duplicated with OBUE |
|  |  | ACLR | ACLR can be defined based on the co-existence study |
|  |  | Operating band unwanted emissions | Need more study for OBUE after ACLR is defined. |
|  |  | Transmitter spurious emissions | Existing spurious emission requirement is applicable |
|  | Transmitter intermodulation |  | not needed |

Issue 2-5: RX

Agreement:

- Use the following table as starting point for RF requirements impact study. The table is for information.

| RF Requirement for A-IoT BS- RX part |  |  |
| --- | --- | --- |
| RX requirement | Reference sensitivity |  |
|  | Dynamic range |  |
|  | In-channel selectivity |  |
|  | Adjacent Channel Selectivity |  |
|  | In-band blocking |  |
|  | Out-of-band blocking |  |
|  | Receiver intermodulation |  |
|  | Narrowband intermodulation |  |
|  | Rx spurious emission |  |
|  | Receiver intermodulation |  |

Reference sensitivity level

The residual noise from CW interference is higher than the noise floor. So it should be considered when defining receiver sensitivity. Reference sensitivity shall be specified according following formula:

PREFSENS(dBm)=-174dBm+10×log10BW+NF+IM+Icw+SNR

Where:

-	BW is the maximum transmission bandwidth

-	NF is BS noise figure, equal to 10 dB for Medium Range BS and 13 dB for Local Area BS.

-	IM is the implementation margin, equal to 2dB.

-	Icw is IoT due the residual CW interference, value will be defined in the WI.

-	SNR is the SNR value for which we reach 10% BLER target. Each company provided simulation results, and average will be done for each BW.

Proposal 6: IoT due the residual CW interference should be considered when define RX reference sensitivity

Dynamic range

FFS

In-band selectivity and blocking

ACS can be defined based on the co-existence study. In-band blocking need more study after ACS is defined

Out-of-band blocking

Out of band blocking is a measure of the receiver ability to receive a wanted signal at its assigned channel in the presence of an unwanted interferer out of the operating band, which is a CW signal for out-of-band blocking. Existing out-of-band blocking requirement is applicable.

Proposal 7: existing out-of-band blocking requirement is applicable

Receiver spurious emissions

Existing RX spurious emission requirement is applicable

Receiver intermodulation

FFS

7.8	In-channel selectivity

Not applicable

Based on the above discussion, a summary for RX part is captured in the following Table

| RF Requirement for A-IoT BS- RX part |  |  |
| --- | --- | --- |
| RX requirement | Reference sensitivity | IoT due the residual CW interference should be considered when define RX reference sensitivity |
|  | Dynamic range | FFS |
|  | In-channel selectivity | Not applicable |
|  | Adjacent Channel Selectivity | ACS can be defined based on the co-existence study. |
|  | In-band blocking | In-band blocking need more study after ACS is defined |
|  | Out-of-band blocking | Existing out-of-band blocking requirement is applicable |
|  | Narrowband intermodulation | FFS |
|  | Rx spurious emission | Existing RX spurious emission requirement is applicable |
|  | Receiver intermodulation | FFS |

2.3 CW for D1T1

Issue 2-6: CW for D1T1

Agreement:

- To further investigate output power, emission requirements for CW node

  - FFS for other requirements.

CW is using the IMT spectrum. From the perspective of spectrum regulations, the transmit power of carrier-wave is assumed not to exceed the allowed transmit power in the corresponding spectrum. For CW uses the UL spectrum in 900 MHz band, 26 dBm used for NR UE and 33 dBm for GSM should be considered. For the emissions related to out of NR channel, the existing specifications of target spectrum can be reused. And for the emission within the NR channel, whether to define in-band emission is FFS.

Observation 1: For CW uses the UL spectrum in 900 MHz band, 26 dBm used for NR UE and 33 dBm for GSM should be considered.

3	Conclusion

This contribution discusses some proposals on Ambient IOT BS RF requirements.

Proposal 1: RAN4 to study the necessity of channel raster for Ambient RF requirement. If needed, it is proposed to reuse the enhanced channel raster (i.e. 10 kHz)

Proposal 2: It is proposed to define BS type 1-C for Ambient-IoT indoor scenarios.

Proposal 3: The existing rated output power limits for NR Medium range BS is applicable to Ambient IOT BS for D1T1 deployment scenarios.

Proposal 4: it is proposed to define RF envelope mask as the transmit signal quality requirement.

Proposal 5: no need to define transmitter intermodulation for A-IOT BS

Proposal 6: IoT due the residual CW interference should be considered when define RX reference sensitivity

Proposal 7: existing out-of-band blocking requirement is applicable

a summary for TX part is captured in below Table

| RF Requirement for A-IoT BS- TX part |  |  |  |
| --- | --- | --- | --- |
| TX requirement | Transmit output power | Maximum output power | The rated output power limits for Medium range BS is applicable to Ambient IOT BS |
|  | Output power dynamic |  | The general output power dynamics are not applicable to Ambient IOT BS. The power boosting for BS support both Ambient IOT and NR can be discussed in future release. |
|  | Transmit ON/OFF power |  | Not applicable |
|  | Transmission times |  | Not applicable. More discussion is covered below for signal quality. |
|  | Transmit signal quality | Frequency error | Reused from NR BS |
|  |  | EVM | to define RF envelope mask as the transmit signal quality requirement. |
|  |  | TAE | N/A |
|  | Unwanted emissions | Occupied bandwidth | Channel bandwidth for Ambient IOT might be needed. |
|  |  | SEM | Duplicated with OBUE |
|  |  | ACLR | ACLR can be defined based on the co-existence study |
|  |  | Operating band unwanted emissions | Need more study for OBUE after ACLR is defined. |
|  |  | Transmitter spurious emissions | Existing spurious emission requirement is applicable |
|  | Transmitter intermodulation |  | not needed |

a summary for RX part is captured in the following Table

| RF Requirement for A-IoT BS- RX part |  |  |
| --- | --- | --- |
| RX requirement | Reference sensitivity | IoT due the residual CW interference should be considered when define RX reference sensitivity |
|  | Dynamic range | FFS |
|  | In-channel selectivity | Not applicable |
|  | Adjacent Channel Selectivity | ACS can be defined based on the co-existence study. |
|  | In-band blocking | In-band blocking need more study after ACS is defined |
|  | Out-of-band blocking | Existing out-of-band blocking requirement is applicable |
|  | Narrowband intermodulation | FFS |
|  | Rx spurious emission | Existing RX spurious emission requirement is applicable |
|  | Receiver intermodulation | FFS |

Observation 1: For CW uses the UL spectrum in 900 MHz band, 26 dBm used for NR UE and 33 dBm for GSM should be considered.

References

[1] RP-240829, New WID: NR base station (BS) RF requirement evolution for FR1/FR2 and testing, RAN4 Chair (Huawei)

[2] R4-2410597, WF on impacts of A-IoT on RF requirements, Huawei