3GPP RAN WG4 Meeting #112		R4-2413283

# Maastricht, Netherlands, 19th – 23rd August, 2024

Source: 	Huawei, HiSilicon

Title: 	On modification of existing BS TX requirements for SBFD

Agenda Item:	8.19.2.2

Document for:	Approval

Introduction

During RAN4#111, BS RF requirements for SBFD have been discussed. Following conclusions for existing TX requirements are reproduced from [1].

- Issue 2-1-1: PSD scaling for normal and SBFD slots/symbols - Agreement: - PSD scaling for normal and SBFD slots/symbols: - No need to introduce the restriction on PSD scaling for normal and SBFD slots/symbols - Vendors can declare different TX power values for normal and SBFD slots/symbols - Issue 2-2-1: TX intermodulation requirement - Agreement: - The transmitter intermodulation requirement is applicable in SBFD slots/symbols: - Follow the existing requirement - No receiver requirement is specified. - Issue 2-3-1: The necessity of Co-location ACLR requirement - Agreement: - RAN4 further study the ACLR requirement by applying the existing ACLR requirement for SBFD-capable BS in SBFD symbols/slots - FFS the applicable deployment scenario where different ACLR requirement could be useful, whether it justify the different ACLR requirement. - Issue 2-3-2: OBUE - Agreement: - Further discuss how to define OBUE requirement for SBFD capable BS in order to ensure feasible co-existence performance for SBFD BSs operating in adjacent frequency.

Good progress was made in last meeting. In this contribution, we would like to share our views regarding the open issues.

Discussion

While it was agreed to still apply the existing OBW requirement for the whole BS channel bandwidth in SBFD symbols/slots, both ACLR and OBUE requirements have also been agreed to be defined from the edge of the whole carrier instead of sub-band for SBFD DL symbols/slots. In order to guarantee feasible co-existence performance between different SBFD BSs operating in adjacent frequency, ACLR and especially OBUE requirements for SBFD capable BS are agreed to be further discussed [x].

For protection of SBFD UL, when SBFD BSs are operating in adjacent channels of the same band, additional requirement is needed. As shown in Table xx-x, the in-band emission at xx MHz offset of Operator A is -15 dBm/MHz. If no additional emission limit is specified, in general co-existence scenarios assumption, the interference from DL TX of operator A to SBFD UL of operator B is higher than acceptable. Hence new OBUE requirements based on per operator filter are needed to guarantee the normal operation of SBFD.

Proposal 1: it is proposed to specify additional OBUE requirement for SBFD capable BS in order to ensure co-existence performance for SBFD BSs operating in adjacent frequency.

The cross link interference to SBFD UL, which is node B to node B interference requirements traditionally are defined by assuming a MCL to consider the applicable deployment scenarios. With this purpose we did some evaluation on the coupling loss. Element to element coupling for n104 AAS is measured and the average value is shown in the Table 2-1. It can be found that 60~ 65 dB MCL can cover most of deployments scenarios since the measured cases can present a certain worse case.

- Horizontal 0°

- Vertical 0°

- Horizontal 180° 0°

Table 2-1: Adjacent channel co-existence

| Case | Average e2e coupling loss (dB) |  |
| --- | --- | --- |
|  | in chamber | in- field |
| Case a) Horizontal 0° spacing 50 cm | 68 | 64 |
| Case b) Vertical spacing 70 cm | 77 | 68 |
| Case c) Horizontal 180° spacing 1 m | 81 | 73 |

Observation 1: 60~ 65 dB MCL can cover most of deployments scenarios for FR1 high frequency band

The value 65 dB is close to co-existence spurious emissions MCL assumption (i.e. 67 dB), which is derived for 3G system at an intermediate case scenario (577/2 m base station shifting, 2GHz UMTS band). Based on above discussion, it can be found at higher frequency band, e.g. n104 upper 6GHz, similar MCL assumption cover more scenarios. Table 2-2 shows an example link budget for adjacent channel co-existence if 67 dB MCL is assumed.

Table 2-2: Adjacent channel co-existence

| Aggressor | Victim | Aggressor (Operator A) Victim (Operator B) | Link budget |
| --- | --- | --- | --- |
| SBFD | SBFD | D D U U |  |
| TX |  | -15 dBm/MHz | -15 dBm |
| TX Filter | RX Filter |  | Required attenuation from TX filter: -15+49=34 dB |
| Pathloss/coupling loss |  | e.g. co-existence spurious emissions assume 67 dB MCL | 67 dB |
| Emission level |  | -49 dBm/1MHz in UL, assuming 67 dB MCL +7dB lower than noise floor. | -49 dBm/MHz |

As shown in Table 2-3, for protection of SBFD UL of operator B, the SBFD DL of operator B can be adopted as guard band for filter implementation of adjacent operator A. In existing FR1 networks, usually operators hold less than or equal to 400 MHz. For the band n104 which has 700 MHz band width, considering it is allocated to 2 ~ 3 operators, each may hold 200 MHz, 300 MHz or 400 MHz. Table 2-3 shows the available guard band for different operator’s spectrum.

Table 2-3: available guard band for different operator’s spectrum

| Operator's spectrum | Achieved guard band (MHz) |  | min Ratio of transition guard/sub frequency band | Comments |
| --- | --- | --- | --- | --- |
|  | 2 operator case | 3 operator case (in middle) |  |  |
| 100 MHz | 80 | 40 | 40% | Configuration 1: 40-20-40 Configuration 2: 80-20 |
| 200 MHz | 180 | 80 | 30% | Configuration 1: SBFD 20-80+ TDD 100 |
| 300 MHz | 260 | 100 | 33.30% | Configuration 1: SBFD 40-60+ TDD 100+ TDD 100 |
| 400 MHz | 360 | N/A | 90% | Configuration 1: SBFD 40-60+ TDD 100+ TDD 100+ TDD 100 |

As a reference, in existing TX requirement, ΔfOBUE is defined based on filter implementation. For FR1 operating band less than 100 MHz, ΔfOBUE is 10 MHz, and for band wider than 100 MHz except n104, ΔfOBUE is extended to 40 MHz. ΔfOBUE is further extended to 100 MHz for band n104. It can be found min ratio of transition guard/sub frequency band is much higher than that of ΔfOBUE, which means it would not impose additional implementation difficulty for taking the advantage of per operator filter.

Table 2-4: Maximum offset ΔfOBUE outside the downlink operating band

| BS type | Operating band characteristics (MHz) | ΔfOBUE (MHz) | minimum Ratio of ΔfOBUE /bandwidth |
| --- | --- | --- | --- |
| BS type 1-O | FDL,high – FDL,low < 100 | 10 | 10% |
|  | 100 MHz ≤ FDL,high – FDL,low ≤ 900 (Note: operators hold less than or equal to 400 MHz ) | 40 | 10% |
|  | Band n104 | 100 | 14.3% |

Observation 2: available guard band for different operators’ spectrum cases are different. The Ratio of transition guard/sub frequency band is higher than that of ΔfOBUE, which means it would not impose additional implementation difficulty for taking the advantage of per operator filter.

Table 2-5 shows one example to define the additional OBUE requirements with different offsets considering different operators’ spectrum cases.

Table 2-5: additional OBUE requirement for protection of SBFD UL

| SBFD sub-band characteristics |  | Basic limits | Measurement bandwidth |
| --- | --- | --- | --- |
| 100 MHz ≤Band width < 200 | [40 MHz] f fmax | [-56 dBm] | 1MHz |
| 200 MHz ≤Band width < 300 | [80 MHz] f fmax | [-56 dBm] | 1MHz |
| 300 MHz ≤Band width ≤ 400 | [100 MHz] f fmax | [-56 dBm] | 1MHz |

Proposal 2: the additional OBUE requirement should be defined at different frequency offsets from the sub frequency band edge.

Conclusion

In this contribution we discussed on the BS TX requirement for SBFD. We have the following observations and proposals:

Proposal 1: it is proposed to specify additional OBUE requirement for SBFD capable BS in order to ensure co-existence performance for SBFD BSs operating in adjacent frequency.

Observation 1: 60~ 65 dB MCL can cover most of deployments scenarios for FR1 high frequency band

Observation 2: available guard band for different operators’ spectrum cases are different. The Ratio of transition guard/sub frequency band is higher than that of ΔfOBUE, which means it would not impose additional implementation difficulty for taking the advantage of per operator filter.

Proposal 2: the additional OBUE requirement should be defined at different frequency offsets from the sub frequency band edge.

- References

- R4-2406107, “Way Forward for [110bis][313] NR_duplex_evo”, Samsung, 3GPP TSG-RAN4 Meeting #110bis, Changsha, China, April 15-19, 2024.

- R4-2409958	Way Forward for [111][313] NR_duplex_evo	Samsung