3GPP RAN WG4 Meeting #112		R4-24

# Maastricht, Netherlands, 19th – 23rd August, 2024

Source: 	Huawei, HiSilicon

Title: 	On modification of existing RX RF requirements for SBFD

Agenda Item:	8.19.2.3

Document for:	Approval

Introduction

During RAN4#111, BS RF requirements for SBFD have been discussed. Following conclusions for existing RX requirements are reproduced from [1].

- Issue 3-4-1 & Issue 3-4-2: Necessity for additional co-existence study for In-band blocking and Scenario/Case for additional co-existence study (if agreed) - Way forward: - Companies are encouraged to provide the evaluation results for in-band blocking in the next meeting: - The scenarios to be studied are Scenario 1, 3, 5, 6, 9, defined in TR 38.858 - The case to be studied is Case 3 (TDD DL to SBFD UL subband). - FFS the power level which should be collected from the simulation results to derive in-band blocking levels: - Option-1: absolute power level of interference from adjacent channel only, i.e., SBFD network internal interference should not be considered in the simulation, from the perspective of definition of in-band blocking requirement. - Other options are not precluded. - FFS the reference point for the power level which should be before or after RX beamforming. - Option-1: after RX subarray beamforming gain and before array gain, i.e, at TAB. - FFS different grid-shift values should be considered. - Option-1: 10%. - Option-2: Other percentage values are not precluded. - FFS how to use the power level probability distributions to derive the in-band blocking levels - Option-1: Find the X% tile of the UL SBFD wideband received power CDF to define the blocking requirement in each of the down-selected scenarios. - FFS the value of X - FFS the impact of CLI handling scheme introduced in RAN1.

In this contribution, we would like to share our views regarding the open issues.

Discussion

2.1 Reference sensitivity and OTA sensitivity

During Rel-18 SI, value range [0.5~1] dB has been considered for the allowed sensitivity degradation due to self-interference. RAN4 may need to continue the work from e.g. whether to consider interferences other than self-interference for OTA sensitivity.

From the previous discussion in [R4-2410108], firstly we need to decide the assumption for the interference considered in the OTA sensitivity degradation.

  - Alt. 1: RAN4 to consider self-interference, inter-site interference, and inter-sector interference when defining the OTA reference sensitivity.

  - Alt. 2: Only self-interference considered

  - Alt. 3: Others

Inter-site interference and inter-sector interference involve at least two BSs and the required high isolation may be guaranteed by deployments and some CLI coordination solutions, i.e. coordinated beamforming, which may not be covered by a RF requirement for a BS. hence it is preferred to Alt.2, i.e. only self-interference is considered.

For the value range, ~1 dB sensitivity degradation was used in the SI in general. If consider overall impact to system performance, self-interference, inter-site interference, and inter-sector interference all will contribute on the sensitivity degradation. Meanwhile we define 20 dB dynamic range requirement to consider co-channel interference. Considering there may be higher IoT in the real network. It is proposed to add 1 dB addition to reference sensitivity due to self-interference.

Proposal 1: for OTA reference sensitivity, it is proposed to add 1 dB addition to reference sensitivity due to self-interference.

2.2 Dynamic range requirement

During Rel-18 SI, it is agreed that dynamic range is still applicable for SBFD capable BS. Receiver dynamic range requirement is to ensure that the base station can receive high throughput also in the presence of increased interference levels. 20 dB interference over thermal noise is used for NR and LTE. From a scenario of view, 20 dB IOT level is high enough to reflect an acceptable coverage.

Proposal 2: 20 dB IoT level assumption is reused for dynamic range requirement.

2.3 In-band selectivity and blocking

We reproduce some related conclusions from TR 38.858 for better illustration.

- ACS requirement and the interference level shall be determined by RAN4 co-existence study, and for the definition of ACS requirement: - Conducted ACS: Take the existing wanted signal of ACS requirement by using the existing reference sensitivity level. - OTA ACS: The OTA sensitivity degradation shall be taken into account to determine the level of wanted signal and interference signal mean power. - In-band blocking requirement and the interference level shall be determined by RAN4 co-existence study, and for the definition of In-band blocking requirement: - Conducted In-band blocking: Take the existing wanted signal of In-band blocking requirement by using the existing reference sensitivity level. - OTA In-band blocking: The OTA sensitivity degradation shall be taken into account to determine the level of wanted signal and interference signal mean power. - For in-band selectivity and blocking, the requirements shall be defined out of the BS channel bandwidth instead of uplink subband bandwidth.

In our view, in order to guarantee feasible co-existence performance between different BSs operating in adjacent frequency, in-band blocking requirement for SBFD should be further discussed since the blocker outside of carrier could be DL transmission from other BS, which is other than the UL transmission from other UEs had been considered for legacy requirement. Whether the blocker signal level can be scenario specific can be further discussed as well.

As discussed for TX part, the BS to BS cross link interference requirements can be decided by assuming a MCL to consider the applicable deployment scenarios. From the evaluation in [R4-2413283]. 60~ 65 dB MCL can cover most of deployments scenarios since the measured cases can represent a certain worse case. So the blocking level is about -10~ -15 dBm considering a 50 dBm BS TX. The value is in line with the power level of existing out of band blocking requirement.

Meanwhile we perform some simulation to study the blocking level with following assumption.

  - The scenarios to be studied are Scenario 1, defined in TR 38.858

  - The case to be studied is Case 3

  - absolute power level of interference from adjacent channel only, i.e., SBFD network internal interference should not be considered in the simulation, from the perspective of definition of in-band blocking requirement.

  - the reference point: before array gain

  - Grid-shift: Option-1: 10%, 20%, 50%, 100%

  - Percentage values: 99%

The simulation results are shown in following figures and table.

Figure 2.3-1 blocking level CDF 10% grid shift   Figure 2.3-2 blocking level CDF 20% grid shift

Figure 2.3-3 blocking level CDF 50% grid shift     Figure 2.3-4 blocking level CDF 100% grid shift

Table 2.3-1 blocking level @ 99%CDF

| Blocking level(dBm) |  |
| --- | --- |
| Grid-shift | 99% CDF |
| 10% | -5.1 |
| 20% | -12.4 |
| 50% | -22.4 |
| 100% | -25.8 |

Traditionally the in-band blocking level has been analysis on a statistical basis based on the 99.99% probability of an interferer being possible, e.g. UMTS. The NR OFDMA scheme does not suffer so greatly due to a blocking event hence such a high probability is not required, probabilities 99% have been considered.

For in-band selectivity and blocking, both requirements are defined out of the BS channel bandwidth instead of uplink subband bandwidth. The previous agreements on conducted ACS may be less important, since the in-band blocking will be increased by dozens of dB. One approach is to merge the ACS and in-band blocking requirements with a wider channel bandwidth, e.g. the same channel bandwidth as wanted signal.

Proposal 3: -10~ -15 dBm is considered as the in-band blocking level.

Conclusion

In this contribution we discussed on the BS RX requirement for SBFD. We have the following observations and proposals:

Proposal 1: for OTA reference sensitivity, it is proposed to add 1 dB addition to reference sensitivity due to self-interference.

Proposal 2: 20 dB IoT level assumption is reused for dynamic range requirement.

Proposal 3: -10~ -15 dBm is considered as the in-band blocking level.

- References

- R4-2406107, “Way Forward for [110bis][313] NR_duplex_evo”, Samsung, 3GPP TSG-RAN4 Meeting #110bis, Changsha, China, April 15-19, 2024.

- R4-2409958	Way Forward for [111][313] NR_duplex_evo,	Samsung