3GPP TSG-RAN WG4 Meeting #112	R4-2413293

# Maastricht, Netherlands, August 19th – 23rd, 2024

Source:	Nokia

Title:	RRM Performance Requirements for LPHAP

Agenda item:	6.1.2.3

Document for:	Discussion

Introduction

The Rel-18 WID [1] for NR positioning includes the enhancements for LPHAP. The WID depicts the RAN4 work related to this accuracy improvement technology. The RAN4#111 meeting [2] has made the following agreements on RRM performance requirements for LPHAP.

Issue 1-2-1: eDRX related configuration in the test cases Agreements (from Mon adhoc session) - In TC configuration, agree to use same eDRX cycle and PTW length for CN eDRX and RAN eDRX. - Reuse DRX cycle configuration from corresponding R17 TCs. - eDRX cycle length: longer than reporting periodicity. - eDRX cycle length configuration: 4 x 10.24 s Issue 1-2-3: configurations for cell reselection TCs Agreements (from Thu adhoc session) - UE is not configured with PRS measurement - UE is configured with 5.12s SRS periodicity in Cell 1 - Simplify existing cell reselection TCs by using only two time periods - Time period (T3) for reselection back is not needed

This contribution further investigates impacts to RRM performance requirements for NR positioning enhancements.

Discussion

The impacts to RRM performance requirements are discussed in this section.

RRM testing for PRS measurements

To discuss the test cases for PRS measurements, the open issue [2] is summarized.

Issue 1-2-2: whether to configure PRS measurement reporting periodicity in the test cases Wayforwards - Option 1: No - Option 2: Yes, configured as 20s (smaller than eDRX cycle)

For NR RSTD measurement reporting delay test case, the open issue is whether to specify the PRS measurement reporting periodicity. UE can be configured to report positioning measurements with a periodicity that is smaller than eDRX cycle, via reportingInterval in nr-DL-TDOA-RequestLocationInformation. For instance, if a reporting periodicity is 20 sec, the value of reportingInterval is set to ri20. In this case, the reporting periodicity is configured by LMF, and the configured value can be independent on the eDRX cycle. Therefore, in a reporting delay test case, a different reporting periodicity can be flexibly configured by LMF. Thus, it is unnecessary to define a reporting periodicity in a corresponding test case.

RAN4 does not have to specify PRS measurement reporting periodicity in the test cases for LPHAP.

Conclusion

The following proposal is made in this contribution.

- RAN4 does not have to specify PRS measurement reporting periodicity in the test cases for LPHAP.

- References

- RP-230328, “Revised WID on Expanded and Improved NR Positioning,” Intel Corporation, RAN#99.

- R4-2410194, “WF on NR Positioning – LPHAP,” Huawei, HiSilicon, RAN4#111.