3GPP TSG-RAN4 Meeting #112	R4-2413296

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 2498 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | ( HPUE_NR_FR1_FDD) CR for 38.101-1 NS_06 corrections |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | T-Mobile USA, AT&T, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | HPUE_FR1_FDD_NR_CADC_R18 |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | NS_06 in Table 6.2.3.1-1 had an error in that 3 and 15 MHz were listed in the row for n13 and the table was more complicated as needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Moved n13 to a row with n14 where only 5 and 10 MHz are indicated, and added sentences about PC3 and PC1 to 6.2.3.32 so that Table 6.2.3.1-1 can be simplified. Also removed Note 11 from n71 in Table 6.2.3.1-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Errors persist in the spec Table 6.2.3.1-1 is more complicated than necessary. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3.1, 6.2.3.32 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### <First change>

### 6.2.3	UE additional maximum output power reduction

#### 6.2.3.1	General

Additional emission requirements can be signalled by the network. Each additional emission requirement is associated with a unique network signalling (NS) value indicated in RRC signalling by an NR frequency band number of the applicable operating band and an associated value in the field additionalSpectrumEmission. Throughout this specification, the notion of indication or signalling of an NS value refers to the corresponding indication of an NR frequency band number of the applicable operating band, the IE field freqBandIndicatorNR and an associated value of additionalSpectrumEmission in the relevant RRC information elements [7].

To meet the additional requirements, additional maximum power reduction (A-MPR) is allowed for the maximum output power as specified in Table 6.2.1-1. Unless stated otherwise, the total reduction to UE maximum output power is max(MPR, A-MPR) where MPR is defined in clause 6.2.2. Outer and inner allocation notation used in clause 6.2.3 is defined in clause 6.2.2. Unless stated otherwise, Edge RB allocations get the same AMPR as Outer RB allocations. In absence of modulation and waveform types the A-MPR applies to all modulation and waveform types.

Table 6.2.3.1-1 specifies the additional requirements with their associated network signalling values and the allowed A-MPR and applicable operating band(s) for each NS value. In case of a power class 3 UE, when IE powerBoostPi2BPSK is set to 1, power class 2 A-MPR values apply. When IE [powerBoostPi2BPSKRel18] or [powerBoostQPSKRel18] is enabled, A-MPR, if larger than zero, is increased by ΔPPowerBoost. The mapping of NR frequency band numbers and values of the additionalSpectrumEmission to network signalling labels is specified in Table 6.2.3.1-1A.

For almost contiguous allocations in CP-OFDM waveforms in power class 1.5, 2 and 3, the allowed A-MPR defined in clause 6.2.3 is increased by CEIL{ 10 log10(1 + NRB_gap / NRB_alloc), 0.5 } dB, where CEIL{x, 0.5} means x rounding upwards to closest 0.5dB, NRB_gap is the total number of unallocated RBs between allocated RBs and NRB_alloc is the total number of allocated RBs, and the parameter LCRB is replaced by NRB_alloc + NRB_gap in specifying the RB allocation regions.

Unless otherwise specified, pi/2 BPSK in following A-MPR tables refers to both variants of pi/2 BPSK referenced in 6.2.2 tables 6.2.2-1.

Table 6.2.3.1-1: Additional maximum power reduction (A-MPR)

| Network signalling label | Requirements (clause) | NR Band | Channel bandwidth (MHz) | Resources blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_01 |  | Table 5.2-1 (NOTE 8) | 3, 5, 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | Table 5.3.2-1 | N/A |
| NS_03 | 6.5.2.3.3 | n2, n25, n66, n70, n86 |  |  | Clause 6.2.3.7 |
| NS_03U | 6.5.2.3.3, 6.5.2.4.2 | n2, n25, n66, n86 (NOTE 1) |  |  | Clause 6.2.3.7 |
| NS_04 | 6.5.2.3.2, 6.5.3.3.1 | n41, n90 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |  | Clause 6.2.3.2 |
| NS_05 | 6.5.3.3.4 | n1, n65, n84 (NOTE 1) | 5, 10, 15, 20 (NOTE 2) |  | Clause 6.2.3.4 (NOTE 7) |
| NS_05U | 6.5.3.3.4, 6.5.2.4.2 | n1, n65, n84 | 5, 10, 15, 20 |  | Clause 6.2.3.4 (NOTE 7) |
| NS_06 | 6.5.2.3.4 | n12 | 5, 10, 15 |  | Clause 6.2.3.32 |
|  |  | n13, n14 | 5, 10 |  |  |
|  |  | n85 | 3, 5, 10, 15 |  |  |
| NS_07 | 6.5.2.3.4 6.5.3.3.26 | n13 | 5,10 | Table 6.2.3.29-1, Table 6.2.3.29-3 | Table 6.2.3.29-2. Table 6.2.3.29-4 |
| NS_10 |  | n20, n82 | 15, 20 | Table 6.2.3.3-1 | Table 6.2.3.3-1 |
| NS_12 | 6.5.3.3.17 | n26 | 3,5,10 | Table 6.2.3.21-1 | Table 6.2.3.21-2 |
| NS_13 | 6.5.3.3.18 | n26 | 3, 5 | Table 6.2.3.22-1 | Table 6.2.3.22-2 |
| NS_14 | 6.5.3.3.19 | n26 | 10,15,20 | Table 6.2.3.23-1 | Table 6.2.3.23-2 |
| NS_15 | 6.5.3.3.20 | n26 | 3,5,10,15,20 | Table 6.2.3.24-1 | Table 6.2.3.24-2 |
| NS_17 | 6.5.3.3.2 | n28, n8313 | 3,5,10 | Table 5.3.2-1 | N/A |
| NS_18 | 6.5.3.3.3 | n28, n8313, n109 | 3, 5 |  | Table 6.2.3.13-1, A1 |
|  |  |  | 10, 15, 20 |  | Table 6.2.3.13-1, A2 |
|  |  |  | 25, 30 |  | Table 6.2.3.13-1, A3, A4, A5 |
| NS_21 | 6.5.2.3.9 6.5.3.3.12 | n30 | 5, 10 |  | Clause 6.2.3.14 |
| NS_24 | 6.5.3.3.13 | n65 (NOTE 4) | 5, 10, 15, 20 | Table 6.2.3.15-1 | Clause 6.2.3.15 |
| NS_27 | 6.5.2.3.8 6.5.3.3.14 | n48 | 5, 10, 15, 20, 30, 40 | Table 6.2.3.16-1 | Table 6.2.3.16-2 |
| NS_35 | 6.5.2.3.1 | n71 | 5, 10, 15, 20, 25, 30 | Table 5.3.2-1 | Clause 6.2.3.31 |
| NS_37 | 6.5.3.3.6 | n74 (NOTE 3) | 10, 15 | Table 6.2.3.8-1 | Table 6.2.3.8-1 |
| NS_38 | 6.5.3.3.7 | n74 | 5, 10, 15, 20 | Table 6.2.3.9-1 | Table 6.2.3.9-1 |
| NS_39 | 6.5.3.3.8 | n74 | 10, 15, 20 | Table 6.2.3.10-1 | Table 6.2.3.10-1 |
| NS_40 | 6.5.3.3.9 | n51 | 5 |  | Table 6.2.3.5-1 |
| NS_41 | 6.5.3.3.10 | n50 | 5, 10, 15, 20, 30, 40, 50, 60 |  | Table 6.2.3.11-1 |
| NS_42 | 6.5.3.3.11 | n50 | 5, 10, 15, 20, 30, 40, 50, 60 |  | Table 6.2.3.12-1 |
| NS_43 | 6.5.3.3.5 | n8, n81 | 5, 10, 15 |  | Clause 6.2.3.6 |
| NS_43U | 6.5.3.3.5, 6.5.2.4.2 | n8, n81 (NOTE 1) | 5, 10, 15 |  | Clause 6.2.3.6 |
| NS_44 | 6.5.3.3.24 | n38 | 25, 30, 40 | Table 6.2.3.20-1 | Table 6.2.3.20-1 |
| NS_45 | 6.5.3.3.21 | n53 | 5, 10 |  | Clause 6.2.3.25 |
| NS_46 | 6.5.3.3.25 | n7 | 10, 15, 20, 25, 30, 35, 40, 50 | Table 6.2.3.17-1 Table 6.2.3.17-311 Table 6.2.3.17-5 | Table 6.2.3.17-2 Table 6.2.3.17-411 Table 6.2.3.17-6 |
| NS_47 | 6.5.3.3.15 | n41 (Note 5) | 30 | Table 6.2.3.18-1 Table 6.2.3.18-3 | Table 6.2.3.18-2 Table 6.2.3.18-4 |
| NS_48 | 6.5.3.3.22 | n1 and n84 | 10, 15, 20, 25, 30, 40, 45, 50 | Table 6.2.3.26-1, Table 6.2.3.26-3 | Table 6.2.3.26-2, Table 6.2.3.26-4 (NOTE 7) |
| NS_49 | 6.5.3.3.23 | n1 and n84 | 10, 15, 20, 25, 30, 40, 45, 50 | Table 6.2.3.27-1, Table 6.2.3.27-3 | Table 6.2.3.27-2, Table 6.2.3.27-4 (NOTE 7) |
| NS_50 | 6.5.3.3.16 | n39, n98 | 10, 15, 20, 25, 30, 35, 40 |  | Clause 6.2.3.19 |
| NS_51 | 6.5.3.3.22 | n65 | 50 | Table 6.2.3.28-1 | Table 6.2.3.28-2 |
| NS_55 | NOTE 6 | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  | N/A |
| NS_56 | 6.5.3.3.27 | n24, n99 | 5, 10 |  | Clause 6.2.3.30 |
| NS_57 | NOTE 10 | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  | N/A |
| NS_62 | 6.5.3.3.28 | n54 | 5 |  | N/A |
| NS_100 | 6.5.2.4.2 | n1, n2, n3, n5, n8, n18, n25, n26, n65, n66, n80, n81, n84, n86, n89 (NOTE 1) |  |  | Table 6.2.3.1-2 |
| NOTE 1: This NS can be signalled for NR bands that have UTRA services deployed. NOTE 2: No A-MPR is applied for 5 MHz BWChannel where the upper channel edge is ≥ 1930 MHz,10 MHz BWChannel where the upper channel edge is ≥ 1950 MHz and 15 MHz BWChannel where the upper channel edge is ≥ 1955 MHz and 20 MHz BWChannel where the upper channel edge is ≥ 1970 MHz. NOTE 3: Applicable when the NR carrier is within 1447.9 – 1462.9 MHz. NOTE 4: Applicable when the upper edge of the channel bandwidth frequency is greater than 1980 MHz. NOTE 5: Applicable when the NR carrier is within 2545 – 2575 MHz. PC1 operation is not allowed. NOTE 6: This NS value is applicable for cells in the range 3450 – 3550 MHz for operations in the USA. This NS value does not indicate any additional spurious emission and maximum output power reduction requirements. NOTE 7: The 1Tx architecture is assumed. For power class 2 UE indicating txDiversity-r16 or txDiversity2Tx-r18 [TS 38.306], the additional relaxation of [2] dB is applicable. NOTE 8: The NS_01 label with the field additionalPmax [7] absent is default for all NR bands. NOTE 9: Void NOTE 10: This NS value is applicable for cells below 3980 MHz that are partly or fully within the range 3650-3980 MHz for operations in Canada. This NS value does not indicate any additional spurious emission and maximum output power reduction requirements. NOTE 11: Void. NOTE 12: Void. NOTE 13: Void. NOTE 14: Void. |  |  |  |  |  |

Table 6.2.3.1-1A: Mapping of network signalling label

| NR band | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| n1 | NS_01 | NS_100 | NS_05 | NS_05U | NS_48 | NS_49 |  | Reserved |
| n2 | NS_01 | NS_100 | NS_03 | NS_03U |  |  |  | Reserved |
| n3 | NS_01 | NS_100 |  |  |  |  |  | Reserved |
| n5 | NS_01 | NS_100 |  |  |  |  |  | Reserved |
| n7 | NS_01 | NS_46 |  |  |  |  |  | Reserved |
| n8 | NS_01 | NS_100 | NS_43 | NS_43U |  |  |  | Reserved |
| n12 | NS_01 | NS_06 |  |  |  |  |  | Reserved |
| n13 | NS_01 | NS_06 | NS_07 |  |  |  |  | Reserved |
| n14 | NS_01 | NS_06 |  |  |  |  |  | Reserved |
| n18 | NS_01 | NS_100 |  |  |  |  |  | Reserved |
| n20 | NS_01 | Void | NS_10 |  |  |  |  | Reserved |
| n24 | NS_01 | NS_56 |  |  |  |  |  | Reserved |
| n25 | NS_01 | NS_100 | NS_03 | NS_03U |  |  |  | Reserved |
| n26 | NS_01 | NS_100 | NS_12 | NS_13 | NS_14 | NS_15 |  | Reserved |
| n28 | NS_01 | NS_17 | NS_18 |  |  |  |  | Reserved |
| n30 | NS_01 | NS_21 |  |  |  |  |  | Reserved |
| n31 | NS_01 |  |  |  |  |  |  | Reserved |
| n34 | NS_01 |  |  |  |  |  |  | Reserved |
| n38 | NS_01 | NS_44 |  |  |  |  |  | Reserved |
| n39 | NS_01 | NS_50 |  |  |  |  |  | Reserved |
| n40 | NS_01 |  |  |  |  |  |  | Reserved |
| n41 | NS_01 | NS_04 | NS_47 |  |  |  |  | Reserved |
| n48 | NS_01 | NS_27 |  |  |  |  |  | Reserved |
| n50 | NS_01 | NS_41 | NS_42 |  |  |  |  | Reserved |
| n51 | NS_01 | NS_40 |  |  |  |  |  | Reserved |
| n53 | NS_01 | NS_45 |  |  |  |  |  | Reserved |
| n54 | NS_01 | NS_62 |  |  |  |  |  | Reserved |
| n65 | NS_01 | NS_24 | NS_100 | NS_05 | NS_05U | NS_51 |  | Reserved |
| n66 | NS_01 | NS_100 | NS_03 | NS_03U |  |  |  | Reserved |
| n70 | NS_01 | NS_03 |  |  |  |  |  | Reserved |
| n71 | NS_01 | NS_35 |  |  |  |  |  | Reserved |
| n72 | NS_01 |  |  |  |  |  |  | Reserved |
| n74 | NS_01 | NS_37 | NS_38 | NS_39 |  |  |  | Reserved |
| n77 | NS_01 | NS_55 | NS_57 |  |  |  |  | Reserved |
| n78 | NS_01 |  |  |  |  |  |  | Reserved |
| n79 | NS_01 |  |  |  |  |  |  | Reserved |
| n80 | NS_01 | NS_100 |  |  |  |  |  | Reserved |
| n81 | NS_01 | NS_100 | NS_43 | NS_43U |  |  |  | Reserved |
| n82 | NS_01 | Void | NS_10 |  |  |  |  | Reserved |
| n83 | NS_01 | NS_17 | NS_18 |  |  |  |  | Reserved |
| n84 | NS_01 | NS_100 | NS_05 | NS_05U | NS_48 | NS_49 |  | Reserved |
| n85 | NS_01 | NS_06 |  |  |  |  |  | Reserved |
| n86 | NS_01 | NS_100 | NS_03 | NS_03U |  |  |  | Reserved |
| n89 | NS_01 | NS_100 |  |  |  |  |  | Reserved |
| n90 | NS_01 | NS_04 |  |  |  |  |  | Reserved |
| n91 | NS_01 |  |  |  |  |  |  | Reserved |
| n92 | NS_01 |  |  |  |  |  |  | Reserved |
| n93 | NS_01 |  |  |  |  |  |  | Reserved |
| n94 | NS_01 |  |  |  |  |  |  | Reserved |
| n95 | NS_01 |  |  |  |  |  |  | Reserved |
| n97 | NS_01 |  |  |  |  |  |  | Reserved |
| n98 | NS_01 | NS_50 |  |  |  |  |  | Reserved |
| n99 | NS_01 | NS_56 |  |  |  |  |  | Reserved |
| n1042 | NS_01 |  |  |  |  |  |  | Reserved |
| n105 | NS_01 |  |  |  |  |  |  | Reserved |
| n106 | NS_01 |  |  |  |  |  |  | Reserved |
| n109 | NS_01 | NS_18 |  |  |  |  |  | Reserved |
| NOTE 1: additionalSpectrumEmission corresponds to an information element of the same name defined in clause 6.3.2 of TS 38.331 [7]. NOTE 2: Additional emission requirements and associated network signalling for Band n104 are not defined in this version of the specification but may be forthcoming in the future. |  |  |  |  |  |  |  |  |

Table 6.2.3.1-2: A-MPR for NS_100 (UTRA protection) (Power Class 3 and Power Class 2)

| Modulation/Waveform |  | Outer (dB) |
| --- | --- | --- |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 2 |
|  | QPSK | ≤ 2 |
|  | 16 QAM | ≤ 2.5 |
|  | 64 QAM | ≤ 3 |
|  | 256 QAM | ≤ 4.5 |
| CP-OFDM | QPSK | ≤ 4 |
|  | 16 QAM | ≤ 4 |
|  | 64 QAM | ≤ 4 |
|  | 256 QAM | ≤ 6.5 |
| NOTE 1: Void NOTE 2: Void |  |  |

#### <Unchanged clauses omitted>

#### 6.2.3.32	A-MPR for NS_06

For power class 3 operation on bands n12, n13, n14 and n85, no A-MPR is applicable.

For power class 1 operation on band n14, no A-MPR is applicable.

For power class 1 operation on band n85 A-MPR = 8.5 dB if

( LCRB ≤ 0.20 ∙ NRB and ( RBstart = 0 or RBstart + LCRB = NRB ) )

or

( LCRB = 1 and 5 ∙ | RBstart + 0.5 – NRB / 2 | ∙ 12 ∙ SCS ≥ 1.5 ∙ CBW + 5 MHz ).

For power class 2 operation on bands n13, n14 and n85, the PC2 A-MPR requirements for NS_06 are defined below in Table 6.2.3.32-1.

Table 6.2.3.32-1: A-MPR for NS_06 (Power Class 2)

| Modulation/Waveform |  | Outer (dB) |
| --- | --- | --- |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 1.0 |
|  | QPSK | ≤ 1.5 |
|  | 16 QAM | ≤ 2.5 |
|  | 64 QAM | ≤ 3.0 |
|  | 256 QAM | ≤ 4.5 |
| CP-OFDM | QPSK | ≤ 3.5 |
|  | 16 QAM | ≤ 3.5 |
|  | 64 QAM | ≤ 4.0 |
|  | 256 QAM | ≤ 6.5 |

#### <End of changes>