3GPP TSG-RAN WG4 Meeting # 112	R4-2413305

# Maastricht, Netherland, August 19 – 23, 2024

Source:			T-Mobile USA

Title:			NTN S-Band coexistence

Agenda item:		7.17.2

Document for:		Discussion

# Introduction

At the last RAN#104, a new WI [1] has been approved to specify new NTN S-Band.

This contribution discusses issues related to coexistence with other 3GPP bands.

# Discussion

## Coexistence with other 3GPP bands

For UE-UE and BS-BS coexistence, for FDD bands regulators have typically located uplink next to uplink, and downlink next to downlink. Some bands, such as B71/n71 and B20/n20, and B13/n13, B14/n14 use reverse duplexing, where the uplink frequency is higher than the downlink frequency to keep uplink s next to the uplinks of adjacent bands.

Observation 1: Regulators typically keep uplink next to uplink and downlink next to downlink for adjacent or nearby bands to minimize interference issues.

In the case of NTN band B256/n256 for Region 1 and Region 3, the uplink is adjacent to the B1/n1 uplink and overlaps with the B65/n65 uplink, and the B256 DL is adjacent to the B1/n1 uplink and overlaps with the B65/n65 DL.

Observation 2: In the case of NTN band B256/n256, the uplink is adjacent to the B1/n1 uplink and overlaps with the B65/n65 uplink, and the B256 DL is adjacent to the B1/n1 uplink and overlaps with the B65/n65 DL.

Figure 1: Adjacent and overlapping bands to the new NTN S-band

For the proposed S-Band, the uplink overlaps with the n70 downlink and is only 5 MHz above the n25 downlink, and 10 MHz above the n2 downlink.

Observation 3: For the proposed S-Band, the uplink overlaps with the n70 downlink and is only 5 MHz above the n25 downlink, and 10 MHz above the n2 downlink.

In the US, LTE Band 23 was deprecated in favor of Band 70 and n70. There are not UE-UE coexistence issues between B70 and n70 because the B70/n70 uplink is adjacent to the B25/n25 uplink.

Observation 4: In the US, LTE Band 23 was deprecated in favor of Band 70 and n70 which coexists better with Band2/n2 and Band 25/n25.

While coexistence between the proposed S-Band uplink that the Band 2/n2, Band 25/n25 downlinks will be challenging, coexistence between the S-Band uplink and the Band 70/n70 downlink will likely be impossible for the same geographic region because the S-Band UL would overlap with the Band70/n70 DL.

Observation 5: While coexistence between the proposed S-Band uplink that the Band 2/n2, Band 25/n25 downlinks will be challenging, coexistence between the S-Band uplink and the Band 70/n70 downlink will likely be impossible for the same geographic region.

In addition to UE-UE interference there could also be BS-BS interference because B70/n70 downlink transmissions may impact the S-Band uplink. While there may be minimal terrestrial B70/n70 deployments today, as terrestrial B70/n70 coverage expands, there will be more and more coexistence issues between the S-Band and B70/n70.

Observation 6: As terrestrial deployments in B70/n70 expand, it will become more difficult to achieve adequate coexistence between the S-Band and Band70/n70.

One way to consider coexistence with terrestrial 3GPP bands is the following scenarios:

- Areas with terrestrial deployments of B70/n70, and also deployment of B2/n2 and/or B25/n25

- Areas with terrestrial deployments of B2/n2 and/or B25/n25

- Areas with no terrestrial deployment of B70/n70, B2/n2 or B25/n25

Scenario 1: It is believed that since the S-Band proponents are also deploying B70/n70, they will be able to coordinate coexistence between their S-band NTN network and their terrestrial B70/n70 network. Given the potential for UE-UE interference between the S-Band uplink and the Band70/n70 downlink, and also BS-BS interference from the B70/n70 downlink and the S-band uplink, it is unlikely that the S-Band and Band 70/n70 can be deployed in the same geographic region.

Scenario 2: Since the deployment of B2/n2 and B25/n25 are currently much larger than the deployment of B70/n70, it is likely that the proponents will want to use S-Band for satellite coverage in areas where there is no B70/n70 coverage but there is B2/n2 and/or B25/n25 coverage. In these scenarios there is potential for UE-UE interference from the S-Band uplink into the B2/n2 and/or B25/n25 downlink. These scenarios could be heavily populated, meaning that users could be in close proximity. Even in less crowded areas, since people tend to gather together, there is a high likelihood in many situations that users could be in close proximity.

Scenario 3: Even in areas where there is no terrestrial Band 2/n2 or Band 25/n25 coverage, there are plans to use the Band 25 G-Block for mobile satellite communications. Sinde these are likely low population density areas, it may be tempting to assume there will be no UE-UE coexistence issues. While it is true that there are situations where a single UE is alone with no victim UEs around, it is also quite possible that even in remote areas multiple people could be gathered, with some using S-Band satellite service and others using Band 25/n25 satellite service. This UE-UE interference could be especially serious in first responder accident, disaster or rescue scenarios. It is even possible that a given first responder may have a personal device in one pocket and their work device in another and they may be using different networks.

Observation 7: In dense urban, urban, suburban and rural areas, the incidence of UE-UE interference between the S-Band and terrestrial bands must be considered.

Another issue to consider for UE-UE coexistence is that for terrestrial networks there may be some UEs with good coverage transmitting at reduced power, and only UEs near the cell edge transmitting at higher power. With NTN, it is likely that all UEs will be transmitting at or near their maximum power. And while the current WID only includes PC3 the trend we have seen in NTN is interest in PC2, PC1.5 and even PC1.

Observation 8: Compared to terrestrial UEs, coexistence studies need to consider that it is more likely that NTN UEs will be operating at maximum power

## A potential alternative band plan

Currently, the primary proponents of the NTN S-Band are also the US operator for B70/n70, so they may be able to coordinate coexistence between the S-Band and Band 70/n70, but if either the satellite or terrestrial spectrum license changes to a different operator (if that is possible) then coexistence will be much more difficult to manage.

We understand that the proposed S-Band configuration is based on an existing MSS allocation. However, to facilitate coexistence between the NTN service and the terrestrial networks, it might be beneficial to consider an NTN band that re-uses the Band 70/n70 configuration instead of a configuration where the proposed S-Band uplink overlaps with the Band 70/n70 downlink.

Proposal 1: The proponents should discuss with regulators the possibility of using the Band 70/n70 uplink/downlink configuration for satellite NTN, rather than 2000-2020 MHz UL, 2180-2200 DL.

When protected spectrum like the downlinks of other bands are too close to 3GPP band uplinks, NS signalling and A-MPR is usually defined, which enables a UE to meet strict nearby emissions limits by lowering transmit power.

When the former LTE Band 23 was introduced, there were many contentious discussions about coexistence between Band 23 and Band 25 and Band 2 because of the close proximity between the Band 23 uplink and the B25 and B2 downlinks. For Band 23, a compromise was reached that the protection level for 1990-1995 MHz would be -40 dBm/MHz because both Band 23 and Band 25 were new bands at the time. Because Band 23 has since been deprecated for terrestrial use in favor of Band 70/n70 and Band 25/ n25 is a well-established legacy band, the starting point for protection should be -50 dBm/MHz for 1990-1995 MHz.

Proposal 2:  The baseline protection level for emissions into 1990-1995 should be -50 dBm/MHz.

## An alternative to A-MPR for protecting other bands

To facilitate coexistence between the bands, high levels of A-MPR were defined for NS_11 and NS_20 to assure protection of spectrum below the B23 uplink. The following compromise emission limits were agreed to for protecting the spectrum below the B23 uplink:

Table 6.6.3.3.14-1: Additional requirements

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 5, 10, 15, 20 MHz |  |
| 1990 ≤ f < 1999 | -40 | 1 MHz |
| 1999 ≤ f ≤ 2000 | -40 | NOTE 1 |
| NOTE 1: The measurement bandwidth is 1% of the applicable E-UTRA channel bandwidth. |  |  |

Even to meet the weaker protection levels of -40 dBm/MHz rather than the standard -50 dBm/MHz level for UE emissions into other 3GPP band downlinks, the amount of A-MPR allowed is quite high. For many possible configurations the amount of A-MPR (or alternatively total backoff) is greater than 10 dB.

Table 6.2.4-15: A-MPR for "NS_20"

| Channel Bandwidth [MHz] | Parameters |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | - Fc [MHz] | < 2007.5 |  |  | 2007.5 ≤ Fc < 2012.5 |  |  |  |  |  |  |  |  |  |  |  | 2012.5 ≤ Fc ≤ 2017.5 |  |  |  |
|  | - RBstart | ≤24 |  |  | 0-3 |  |  |  |  |  |  |  | 4-6 |  |  |  | ≤24 |  |  |  |
|  | - LCRB [RBs] | >0 |  |  | 15-19 |  |  |  | ≥20 |  |  |  | ≥18 |  |  |  | 1-25 |  |  |  |
|  | - A-MPR [dB] | ≤17 |  |  | ≤1 |  |  |  | ≤4 |  |  |  | ≤2 |  |  |  | ≤ 0 |  |  |  |
| 10 | - Fc [MHz] | 2005 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | - RBstart | 0-25 |  |  |  |  | 26-34 |  |  |  |  |  |  |  |  | 35-49 |  |  |  |  |
|  | - LCRB [RBs] | >0 |  |  |  |  | 8-15 |  |  |  |  | >15 |  |  |  | >0 |  |  |  |  |
|  | - A-MPR [dB] | ≤16 |  |  |  |  | ≤2 |  |  |  |  | ≤5 |  |  |  | ≤ 6 |  |  |  |  |
|  | - Fc [MHz] | 2015 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | - RBstart | 0-5 |  |  |  |  |  |  |  |  | 6-10 |  |  |  |  |  |  |  |  |  |
|  | - LCRB [RBs] | ≥32 |  |  |  |  |  |  |  |  | ≥40 |  |  |  |  |  |  |  |  |  |
|  | - A-MPR [dB] | ≤4 |  |  |  |  |  |  |  |  | ≤2 |  |  |  |  |  |  |  |  |  |
| 15 | - Fc [MHz] | 2012.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | - RBstart | 0-14 |  |  |  |  |  | 15-24 |  |  |  |  |  |  |  |  | 25-39 |  | 61-74 |  |
|  | - LCRB [RBs] | 1-9 & 40-75 |  | 10-39 |  |  |  | 24-29 |  |  |  |  |  | ≥30 |  |  | ≥36 |  | ≤6 |  |
|  | - A-MPR [dB] | ≤11 |  | ≤6 |  |  |  | ≤1 |  |  |  |  |  | ≤7 |  |  | ≤5 |  | ≤6 |  |
| 20 | - Fc [MHz] | 2010 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | - RBstart | 0-21 | 22-31 |  |  |  |  |  |  | 32-38 |  |  |  |  | 39-49 |  |  | 50-68 |  | 69-99 |
|  | - LCRB [RBs] | >0 | 1-9 & 31-75 |  |  | 10-30 |  |  |  | ≥15 |  |  |  |  | ≥24 |  |  | ≥25 |  | >0 |
|  | - A-MPR [dB] | ≤17 | ≤12 |  |  | ≤6 |  |  |  | ≤9 |  |  |  |  | ≤7 |  |  | ≤5 |  | ≤16 |
| NOTE 1: When NS_20 is signaled the minimum requirements for the 10 MHz bandwidth are specified for E-UTRA UL carrier center frequencies of 2005 MHz or 2015 MHz. NOTE 2: When NS_20 is signaled the minimum requirements for the 15 MHz channel bandwidth are specified for E-UTRA UL carrier center frequency of 2012.5 MHz. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

In a terrestrial network NS signalling and A-MPR reduces the coverage for impacted RB allocations. In a satellite network, the use of A-MPR could mean the difference between being able to close the link and not. Also, in a terrestrial network often an operator has multiple bands available, so high A-MPR in one band can be avoided by switching a user to a different band. In satellite networks, a satellite operator may only have a single band, so there may not be a work-around to high A-MPR.

While it would be possible to define A-MPR for protection of spectrum below the proposed S-Band uplink, since NS signalling is usually voluntary, it is unlikely that a satellite operator would ever signal the NS value because such power reduction would break the link budget.

Observation 9: It is unlikely that a satellite network would use NS signalling because the A-MPR could prevent the link budget from closing.

To protect spectrum neighbors while still enabling the uplink link budget to close, 3GPP should consider alternative approaches to A-MPR for satellite networks. One approach would be for the standard to forbid certain uplink RB configurations to avoid the worst-case emissions when A-MPR is applied. For example, RAN4 could look at the A-MPR table for NS_20 and decide that the emissions would probably be unacceptable for configurations where the A-MPR is above a certain threshold. Looking at the NS_20 A-MPR table above as an example, RAN4 might decide to ban any configurations where the allowed A-MPR was greater than 10 dB (in red) would not be allowed. Alternatively, a value like 6 dB A-MPR might be chosen for the allowed/not allowed threshold.

In LTE maximum allowed back-off is MPR + A-MPR, whereas for NR the maximum allowed back-off is max(MPR, A-MPR), so the total back-off should be considered.

Maybe there would be a table similar to the A-MPR in the UE specification to indicate allowed vs. unallowed RB allocations, For the example of the 10 dB threshold for NS_20:

Table 1: S-band allowed allocations

| Channel Bandwidth [MHz] | Parameters |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | - Fc [MHz] | < 2007.5 |  |  | 2007.5 ≤ Fc < 2012.5 |  |  |  |  |  |  |  |  |  |  |  | 2012.5 ≤ Fc ≤ 2017.5 |  |  |  |
|  | - RBstart | ≤24 |  |  | 0-3 |  |  |  |  |  |  |  | 4-6 |  |  |  | ≤24 |  |  |  |
|  | - LCRB [RBs] | >0 |  |  | 15-19 |  |  |  | ≥20 |  |  |  | ≥18 |  |  |  | 1-25 |  |  |  |
|  | Allowed/Not allowed | Not allowed |  |  | Allowed |  |  |  | Allowed |  |  |  | Allowed |  |  |  | Allowed |  |  |  |
| 10 | - Fc [MHz] | 2005 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | - RBstart | 0-25 |  |  |  |  | 26-34 |  |  |  |  |  |  |  |  | 35-49 |  |  |  |  |
|  | - LCRB [RBs] | >0 |  |  |  |  | 8-15 |  |  |  |  | >15 |  |  |  | >0 |  |  |  |  |
|  | Allowed/Not allowed | Not allowed |  |  |  |  | Allowed |  |  |  |  | Allowed |  |  |  | Allowed |  |  |  |  |
|  | - Fc [MHz] | 2015 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | - RBstart | 0-5 |  |  |  |  |  |  |  |  | 6-10 |  |  |  |  |  |  |  |  |  |
|  | - LCRB [RBs] | ≥32 |  |  |  |  |  |  |  |  | ≥40 |  |  |  |  |  |  |  |  |  |
|  | Allowed/Not allowed | Allowed |  |  |  |  |  |  |  |  | Allowed |  |  |  |  |  |  |  |  |  |
| 15 | - Fc [MHz] | 2012.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | - RBstart | 0-14 |  |  |  |  |  | 15-24 |  |  |  |  |  |  |  |  | 25-39 |  | 61-74 |  |
|  | - LCRB [RBs] | 1-9 & 40-75 |  | 10-39 |  |  |  | 24-29 |  |  |  |  |  | ≥30 |  |  | ≥36 |  | ≤6 |  |
|  | Allowed/Not allowed | Not allowed |  | Allowed |  |  |  | Allowed |  |  |  |  |  | Allowed |  |  | Allowed |  | Allowed |  |
| 20 | - Fc [MHz] | 2010 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | - RBstart | 0-21 | 22-31 |  |  |  |  |  |  | 32-38 |  |  |  |  | 39-49 |  |  | 50-68 |  | 69-99 |
|  | - LCRB [RBs] | >0 | 1-9 & 31-75 |  |  | 10-30 |  |  |  | ≥15 |  |  |  |  | ≥24 |  |  | ≥25 |  | >0 |
|  | Allowed/Not allowed | Not allowed | Not allowed |  |  | Allowed |  |  |  | Allowed |  |  |  |  | Allowed |  |  | Allowed |  | Not allowed |

While this would not guarantee a certain emission level would be met, it would certainly help to avoid the worst-case emissions configurations.

Proposal 3: RAN4 should consider banning certain problematic uplink RB configurations to minimize the chances of causing unacceptable levels of interference into neighboring downlink bands.

# Summary

Observation 1: Regulators typically keep uplink next to uplink and downlink next to downlink for adjacent or nearby bands to minimize interference issues.

Observation 2: In the case of NTN band B256/n256, the uplink is adjacent to the B1/n1 uplink and overlaps with the B65/n65 uplink, and the B256 DL is adjacent to the B1/n1 uplink and overlaps with the B65/n65 DL.

Observation 3: For the proposed S-Band, the uplink overlaps with the n70 downlink and is only 5 MHz above the n25 downlink, and 10 MHz above the n2 downlink.

Observation 4: In the US, LTE Band 23 was deprecated in favor of Band 70 and n70 which coexists better with Band2/n2 and Band 25/n25.

Observation 5: While coexistence between the proposed S-Band uplink that the Band 2/n2, Band 25/n25 downlinks will be challenging, coexistence between the S-Band uplink and the Band 70/70 downlink will likely be impossible for the same geographic region.

Observation 6: As terrestrial deployments in B70/n70 expand, it will become more difficult to achieve adequate coexistence between the S-Band and Band70/n70.

Observation 7: In dense urban, urban, suburban and rural areas, the incidence of UE-UE interference between the S-Band and terrestrial bands must be considered.

Observation 8: Compared to terrestrial UEs, coexistence studies need to consider that it is more likely that NTN UEs will be operating at maximum power

Observation 9: It is unlikely that a satellite network would use NS signalling because the A-MPR could prevent the link budget from closing.

Proposal 1: The proponents should discuss with regulators the possibility of using the Band 70/n70 uplink/downlink configuration for satellite NTN, rather than 2000-2020 MHz UL, 2180-2200 DL.

Proposal 2:  The baseline protection level for emissions into 1990-1995 should be -50 dBm/MHz.

Proposal 3: RAN4 should consider banning certain problematic uplink RB configurations to minimize the chances of causing unacceptable levels of interference into neighboring downlink bands.

# References

- RP-241661, New WID on Introduction of NR-NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL), DISH Network, EchoStar, TerreStar, OmniSpace, Telus

- TR 36.811, Adding 2 GHz band LTE Frequency Division Duplex (FDD) (Band 23) for Ancillary Terrestrial Component (ATC) of Mobile Satellite Services (MSS) in North America