3GPP TSG-RAN WG4 Meeting #112	R4-2413311

Maastricht, Netherlands, August, 19 - 23, 2024

Source:	Nokia, Nokia Shanghai Bell

Title: 	Simulation results for eRedCap CQI reporting

Agenda item:	5.28.2.1

Document for:	Discussion

Introduction

The demodulation part of the work item on enhanced support for reduced capability NR devices (eRedCap) [1] has progressed and PDSCH demodulation performance requirements for UE for eRedCap were finalized at RAN4 #111 [2].

In addition, eRedCap simulation results were contributed both for CQI reporting for static channel and CQI for fading channel. It was agreed to specify CQI reporting requirements for static channel, whilst CQI reporting requirements for fading channel were kept open [3].

This contribution contains a summary of our CQI reporting simulation results for static channel and fading channel. It is proposed to include them in the simulation summary document for eRedCap.

CQI reporting simulation results summary

The agreed configurations for CQI reporting performance are depicted in [4]. Results presented in this section refer to static condition and fading condition for FR.

  - CQI reporting performance under static condition

Simulation results are reported in this section for agreed simulation assumptions contained in [4].

FDD 15kHz FR1

| 1Rx UE or 2Rx UE | CBW / SCS | CQI table | Rank | Antenna configuration | SNR (dB) | CQI metric (%) | Median CQI |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | 10 / 15 | Table 1 | 1 | 2 x 1 with static channel (Annex B.1 in TS 38.101-4) | 5 dB | 97.875 | 7 |
|  |  |  |  |  | 6 dB | 99.65 | 7 |
|  |  |  |  |  | 11 dB | 100 | 9 |
|  |  |  |  |  | 12 dB | 99.95 | 9 |
| 2Rx UE | 10 / 15 | Table 1 | 2 | 2 x 2 with static channel (Annex B.1 in TS 38.101-4) | 8 dB | 100 | 8 |
|  |  |  |  |  | 9 dB | 100 | 8 |
|  |  |  |  |  | 14 dB | 100 | 9 |
|  |  |  |  |  | 15 dB | 100 | 10 |

TDD 30kHz FR1

| 1Rx UE or 2Rx UE | CBW / SCS | CQI table | Rank | Antenna configuration | SNR (dB) | CQI metric (%) | Median CQI |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | 20 / 30 | Table 1 | 1 | 2 x 1 with static channel (Annex B.1 in TS 38.101-4) | 5 dB | 97.725 | 7 |
|  |  |  |  |  | 6 dB | 99.65 | 7 |
|  |  |  |  |  | 11 dB | 99.975 | 9 |
|  |  |  |  |  | 12 dB | 99.975 | 9 |
| 2Rx UE | 20 / 30 | Table 1 | 2 | 2 x 2 with static channel (Annex B.1 in TS 38.101-4) | 8 dB | 100 | 8 |
|  |  |  |  |  | 9 dB | 100 | 8 |
|  |  |  |  |  | 14 dB | 100 | 9 |
|  |  |  |  |  | 15 dB | 100 | 10 |

It is observed that CQI metric > 90% is fulfilled in all cases. Thus, it is justified to specify CQI reporting requirements under static condition for eRedCap.

Under static condition, the CQI metric > 90% is fulfilled in all cases. Thus, it is justified to specify CQI reporting requirements under static condition for eRedCap.

CQI reporting performance under fading condition

Simulation results are reported in this section for agreed simulation assumptions contained in [4].

FDD 15kHz FR1

| 1Rx UE or 2Rx UE | CBW / SCS | CQI table | Rank | Antenna configuration | SNR (dB) | CQI metric (%) | Median CQI |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | 10 / 15 | Table 1 | 1 | 2 x 1 ULA high | 9 dB | 27.9 | 7 |
|  |  |  |  |  | 10 dB | 18.325 | 8 |
|  |  |  |  |  | 15 dB | 7.3 | 9 |
|  |  |  |  |  | 16 dB | 4.65 | 9 |
| 2Rx UE | 10 / 15 | Table 1 | 1 | 2 x 2 ULA high | 6 dB | 37.3 | 8 |
|  |  |  |  |  | 7 dB | 37.4 | 8 |

TDD 30kHz FR1

| 1Rx UE or 2Rx UE | CBW / SCS | CQI table | Rank | Antenna configuration | SNR (dB) | CQI metric (%) | Median CQI |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | 20 / 30 | Table 1 | 1 | 2 x 1 ULA high | 9 dB | 21.95 | 7 |
|  |  |  |  |  | 10 dB | 13.5 | 8 |
|  |  |  |  |  | 15 dB | 5.625 | 9 |
|  |  |  |  |  | 16 dB | 3.65 | 9 |
| 2Rx UE | 20 / 30 | Table 1 | 1 | 2 x 2 ULA high | 6 dB | 27.25 | 8 |
|  |  |  |  |  | 7 dB | 26.975 | 8 |

It is observed that CQI metric > alpha=20% is not fulfilled in all cases. Especially for 1Rx UE it is not fulfilled for SNR values > 9 dB. Thereagainst, for the 2Rx UE, the CQI metric is fulfilled.

Under fading condition, the CQI metric > alpha=20% is not fulfilled in all cases. Especially for 1Rx UE it is not fulfilled for SNR values > 9 dB. Thereagainst, for the 2Rx UE, the CQI metric is fulfilled.

- And the following proposal is made.

RAN4 to remove CQI reporting requirements under fading condition for eRedCap, or to remove them at least for 1 Rx UE.

Conclusion

In this contribution, we provided a summary of our CQI reporting simulation results for eRedCap for agreed configurations in [4].

Following observations are made.

- Under static condition, the CQI metric > 90% is fulfilled in all cases. Thus, it is justified to specify CQI reporting requirements under static condition for eRedCap.

- Under fading condition, the CQI metric > alpha=20% is not fulfilled in all cases. Especially, for 1Rx UE it is not fulfilled for SNR values > 9 dB. Thereagainst, for 2Rx UE, the CQI metric is fulfilled.

And the following proposal is made.

- RAN4 to remove CQI reporting requirements under fading condition for eRedCap, or to remove them at least for 1 Rx UE.

- References

| [1] | RP-223544, “Revised WID on Enhanced support of reduced capability NR devices”, Ericsson, RAN #98-e |
| --- | --- |
| [2] | R4-2410123, "Topic summary for [111][330] NR_redcap_enh_demod", Moderator (Ericsson) |
| [3] | “RAN4 #111 BDatT Session Report v08 Post meeting”, BaDT session chair |

[4]	R4-2406052, “Way Forward for [110bis][330] NR_redcap_enh_demod”, Ericsson