3GPP TSG-RAN WG4 Meeting #112	R4-2413313

# Maastricht, Netherlands, August 19th – 23rd, 2024

Source:	Nokia, Nokia Shanghai Bell

Title:	Simulation results for DL RSCPD and DL RSCP

Agenda item:	6.1.2.6

Document for:	Discussion

Introduction

The Rel-18 WID [1] for NR positioning contains several enhancements, including:

- Support of NR DL and UL carrier phase positioning for UE-based, UE-assisted, and NG-RAN node assisted positioning

NR carrier phase positioning (NR CPP) has been studied in the FS_NR_pos_enh2 study item by RAN1 with company contributions and outcome of the study being captured in TR 38.859 [2].  Based on Rel-16 and Rel-17 link level simulation assumptions for RSTD and PRS-RSRP, respectively, in [3] and [4], RAN4 has agreed NR CPP link level simulation assumptions for RSCPD in [5] and updated them in [6].

In this contribution we list the used parameters according to the updated simulation assumptions and then provide simulation results for DL RSCPD, together with the corresponding ones for DL RSCP (which are the same we provided for RAN4 #110). It is noted that these results do not contain any RF/BB impairments. However, great potential of this positioning technology in terms of improved positioning accuracy can be seen. In addition, we provide an updated preliminary analysis of the impact of the carrier frequency offset (CFO) impairment on DL RSCPD measurement accuracy, as agreed to be investigated during RAN4 #110.

Simulation Assumptions

Simulation assumptions for DL RSCPD captured in  are considered. The PRS configurations as listed in  were used in the simulation study. Note that the repetition factor relates to repetitions in adjacent slots.

| Parameter | Value |  |  |  |
| --- | --- | --- | --- | --- |
| SCS, RB num, Repetition | SCS (kHz) | RB num | Repetition (Note) | Sample rate (Tc) |
|  | 15 | 24 | 4 | 256 |
|  |  | 52 | 1 | 128 |
|  |  | 104 | 1 | 64 |
|  | 30 | 24 | 4 | 128 |
|  |  | 48 | 1 | 64 |
|  | 60, FR1 | 24 | 4 | 64 |
|  | 60, FR2 | 24 | 4 | 64 |
|  |  | 64 | 1 | 32 |
|  |  | 132 | 1 | 16 |
|  | 120 | 32 | 4 | 32 |
|  |  | 64 | 1 | 16 |
| PRS comb size | 4 |  |  |  |
| PRS symbol size | 4 |  |  |  |
| Number of samples | 1 |  |  |  |
| Es/Iot for three cells (cell 1, cell 2, cell 3), [dB] | (-6, -13, -13)dB for NSample = 1; (-3, -6, -6)dB for NSample = 1. (-3, -13, -13)dB for NSample = 1; (0, -6, -6)dB for NSample = 1. |  |  |  |
| PRS periodicity | 40 ms |  |  |  |
| Network synchronization | Synchronous with time shifts <0, 0, 3 us> |  |  |  |

Table 1: PRS configurations used for NR RSCPD study for SCS = 15kHz, 30kHz, 60kHz (FR1 and FR2), and 120 kHz.

In the simulations, no offset between the 4 PRS symbols of different cells in the slot was configured. Furthermore, we focused on AWGN and two-tap channels and no RF/BB impairments are included. Moreover, for the simulations with the two-tap channel a single UE receive antenna was used, therefore results for the two-tap channel should be considered as preliminary. However, for the next contribution we plan to use 2 UE receive antennas also for the two-tap channel.

Simulation Results

In this section we provide the simulation results for the above listed simulation assumptions. The results are reported for 5-percentile and 95-percentile CDF of the DL RSCPD and DL RSCP errors in degree for given Es/Iot for three cells .

AWGN channel

DL RSCPD

The DL RSCPD error in degree for AWGN channel, derived from the 5-percentile and 95-percentile of the CDF, for Es/IoT of three cells (-6, -13, -13) dB, (-3, -6, -6) dB, (-3, -13, -13) dB, and (0, -6, -6) dB,  respectively, are listed in Table 2 to Table 5.

| SCS (kHz) | Carrier frequency (GHz) | PRS BW in RB | Sampling rate (Tc) | Rep | Number of samples | RSCPD (cell-1, cell-2) (-6, -13) dB |  | RSCPD (cell-1, cell-3) (-6, -13) dB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 5%-tile | 95%-tile | 5%-tile | 95%-tile |
| 15 | 2 | 24 | 256 | 4 | 1 | -6.9295 | 6.3495 | -6.8691 | 6.5551 |
|  |  | 52 | 128 | 1 | 1 | -10.7501 | 10.3782 | -9.5727 | 10.1098 |
|  |  | 104 | 64 | 1 | 1 | -7.517 | 7.6379 | -7.2621 | 7.459 |
|  | 4 | 24 | 256 | 4 | 1 | -6.5493 | 6.883 | -6.7753 | 6.6872 |
|  |  | 52 | 128 | 1 | 1 | -10.4405 | 10.1618 | -10.1452 | 10.3021 |
|  |  | 104 | 64 | 1 | 1 | -7.0983 | 7.7247 | -7.3849 | 7.5049 |
| 30 | 2 | 24 | 128 | 4 | 1 | -6.664 | 6.4319 | -6.8896 | 6.3101 |
|  |  | 48 | 64 | 1 | 1 | -10.0486 | 10.2636 | -10.3496 | 10.5377 |
|  | 4 | 24 | 128 | 4 | 1 | -6.9295 | 6.3495 | -6.5976 | 6.3496 |
|  |  | 48 | 64 | 1 | 1 | -10.8823 | 10.8862 | -11.2272 | 11.0691 |
| 60 | 2 | 24 | 64 | 4 | 1 | -6.6243 | 6.7694 | -6.9958 | 6.7986 |
|  | 4 | 24 | 64 | 4 | 1 | -6.664 | 6.4319 | -6.7515 | 6.3268 |
|  | 40 | 24 | 64 | 4 | 1 | -6.664 | 6.4319 | -6.7515 | 6.3268 |
|  |  | 64 | 32 | 1 | 1 | -9.4117 | 10.33 | -9.7957 | 10.4628 |
|  |  | 132 | 16 | 1 | 1 | -6.664 | 6.4319 | -6.7515 | 6.3268 |
| 120 | 2 | 32 | 32 | 4 | 1 | -6.6639 | 6.1021 | -6.5018 | 6.7241 |
|  |  | 64 | 16 | 1 | 1 | -10.2227 | 10.4705 | -2.5386 | 11.7873 |
|  | 4 | 32 | 32 | 4 | 1 | -5.9844 | 6.4595 | -6.123 | 6.6137 |
|  |  | 64 | 16 | 1 | 1 | -9.362 | 10.0318 | -3.5872 | 11.0669 |

Table 2: DL RSCPD error for AWGN channel in degree for Es/Iot (cell-1, cell-2, cell-3) = (-6, -13, -13) dB.

| SCS (kHz) | Carrier frequency (GHz) | PRS BW in RB | Sampling rate (Tc) | Rep | Number of samples | RSCPD (cell-1, cell-2) (-3, -6) dB |  | RSCPD (cell-1, cell-3) (-3, -6) dB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 5%-tile | 95%-tile | 5%-tile | 95%-tile |
| 15 | 2 | 24 | 256 | 4 | 1 | -3.43 | 3.1465 | -3.3991 | 3.2851 |
|  |  | 52 | 128 | 1 | 1 | -5.4249 | 5.176 | -4.8311 | 5.1138 |
|  |  | 104 | 64 | 1 | 1 | -3.785 | 3.6766 | -3.6592 | 3.7185 |
|  | 4 | 24 | 256 | 4 | 1 | -3.3039 | 3.3657 | -3.4974 | 3.3261 |
|  |  | 52 | 128 | 1 | 1 | -5.3824 | 5.0907 | -5.2903 | 5.1927 |
|  |  | 104 | 64 | 1 | 1 | -3.5021 | 3.9081 | -3.5395 | 3.7459 |
| 30 | 2 | 24 | 128 | 4 | 1 | -3.3571 | 3.2634 | -3.4436 | 3.1603 |
|  |  | 48 | 64 | 1 | 1 | -4.9102 | 5.21 | -5.0967 | 5.4025 |
|  | 4 | 24 | 128 | 4 | 1 | -3.43 | 3.1465 | -3.2591 | 3.1655 |
|  |  | 48 | 64 | 1 | 1 | -5.4639 | 5.5631 | -5.5513 | 5.5624 |
| 60 | 2 | 24 | 64 | 4 | 1 | -3.3982 | 3.3662 | -3.5209 | 3.3504 |
|  | 4 | 24 | 64 | 4 | 1 | -3.3571 | 3.2634 | -3.3858 | 3.2271 |
|  | 40 | 24 | 64 | 4 | 1 | -3.3571 | 3.2634 | -3.3858 | 3.2271 |
|  |  | 64 | 32 | 1 | 1 | -4.8857 | 5.2973 | -5.0554 | 5.3925 |
|  |  | 132 | 16 | 1 | 1 | -3.4347 | 3.7117 | -2.6838 | 3.7236 |
| 120 | 2 | 32 | 32 | 4 | 1 | -3.2189 | 2.9836 | -3.2777 | 3.2793 |
|  |  | 64 | 16 | 1 | 1 | -4.9286 | 5.2306 | -2.4039 | 5.4489 |
|  | 4 | 32 | 32 | 4 | 1 | -3.0452 | 3.2109 | -2.9831 | 3.3404 |
|  |  | 64 | 16 | 1 | 1 | -4.6866 | 5.1082 | -3.3786 | 5.3216 |

Table 3. DL RSCPD error for AWGN channel in degree for Es/Iot (cell-1, cell-2, cell-3) = (-3, -6, -6) dB.

| SCS (kHz) | Carrier frequency (GHz) | PRS BW in RB | Sampling rate (Tc) | Rep | Number of samples | RSCPD (cell-1, cell-2) (-3, -13) dB |  | RSCPD (cell-1, cell-3) (-3, -13) dB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 5%-tile | 95%-tile | 5%-tile | 95%-tile |
| 15 | 2 | 24 | 256 | 4 | 1 | -6.5852 | 6.2227 | -6.6836 | 6.3376 |
|  |  | 52 | 128 | 1 | 1 | -10.3397 | 9.7615 | -9.2951 | 9.6634 |
|  |  | 104 | 64 | 1 | 1 | -7.0543 | 7.2182 | -7.0008 | 7.1883 |
|  | 4 | 24 | 256 | 4 | 1 | -6.2495 | 6.4371 | -6.6812 | 6.3118 |
|  |  | 52 | 128 | 1 | 1 | -9.9281 | 10.0262 | -9.4784 | 9.8648 |
|  |  | 104 | 64 | 1 | 1 | -6.9085 | 7.3687 | -7.0375 | 7.1288 |
| 30 | 2 | 24 | 128 | 4 | 1 | -6.3644 | 6.2534 | -6.7725 | 6.1191 |
|  |  | 48 | 64 | 1 | 1 | -9.6328 | 10.1912 | -10.0035 | 10.1889 |
|  | 4 | 24 | 128 | 4 | 1 | -6.5852 | 6.2227 | -6.085 | 6.1075 |
|  |  | 48 | 64 | 1 | 1 | -10.2711 | 10.3892 | -10.6369 | 10.341 |
| 60 | 2 | 24 | 64 | 4 | 1 | -6.4889 | 6.4799 | -6.7991 | 6.4683 |
|  | 4 | 24 | 64 | 4 | 1 | -6.3644 | 6.2534 | -6.5649 | 5.9721 |
|  | 40 | 24 | 64 | 4 | 1 | -6.3644 | 6.2534 | -6.5649 | 5.9721 |
|  |  | 64 | 32 | 1 | 1 | -9.1285 | 9.7435 | -9.2952 | 9.9489 |
|  |  | 132 | 16 | 1 | 1 | -6.4168 | 7.0431 | -2.3818 | 7.0666 |
| 120 | 2 | 32 | 32 | 4 | 1 | -6.3998 | 5.8797 | -6.1042 | 6.411 |
|  |  | 64 | 16 | 1 | 1 | -9.9104 | 10.0166 | -1.7349 | 11.2142 |
|  | 4 | 32 | 32 | 4 | 1 | -5.8562 | 6.2439 | -6.0096 | 6.3084 |
|  |  | 64 | 16 | 1 | 1 | -9.0117 | 9.6249 | -2.7063 | 10.3538 |

Table 4: DL RSCPD error for AWGN channel in degree for Es/Iot (cell-1, cell-2, cell-3) = (-3, -13, -13) dB.

| SCS (kHz) | Carrier frequency (GHz) | PRS BW in RB | Sampling rate (Tc) | Rep | Number of samples | RSCPD (cell-1, cell-2) (0, -6) dB |  | RSCPD (cell-1, cell-3) (0, -6) dB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 5%-tile | 95%-tile | 5%-tile | 95%-tile |
| 15 | 2 | 24 | 256 | 4 | 1 | -3.2109 | 2.8079 | -3.1338 | 3.0123 |
|  |  | 52 | 128 | 1 | 1 | -4.9508 | 4.7564 | -4.4007 | 4.7364 |
|  |  | 104 | 64 | 1 | 1 | -3.4042 | 3.5082 | -3.368 | 3.4476 |
|  | 4 | 24 | 256 | 4 | 1 | -2.9203 | 3.107 | -3.0898 | 3.0352 |
|  |  | 52 | 128 | 1 | 1 | -4.8334 | 4.773 | -4.7802 | 4.7364 |
|  |  | 104 | 64 | 1 | 1 | -3.2526 | 3.568 | -3.3492 | 3.446 |
| 30 | 2 | 24 | 128 | 4 | 1 | -2.9933 | 2.9431 | -3.2278 | 2.9319 |
|  |  | 48 | 64 | 1 | 1 | -4.617 | 4.6567 | -4.7295 | 4.9297 |
|  | 4 | 24 | 128 | 4 | 1 | -3.2109 | 2.8079 | -3.0085 | 2.9097 |
|  |  | 48 | 64 | 1 | 1 | -4.948 | 5.0971 | -5.126 | 5.0497 |
| 60 | 2 | 24 | 64 | 4 | 1 | -3.0398 | 3.1283 | -3.1922 | 3.0285 |
|  | 4 | 24 | 64 | 4 | 1 | -2.9933 | 2.9431 | -3.1551 | 2.9503 |
|  | 40 | 24 | 64 | 4 | 1 | -2.9933 | 2.9431 | -3.1551 | 2.9503 |
|  |  | 64 | 32 | 1 | 1 | -4.4231 | 4.8501 | -4.5321 | 4.827 |
|  |  | 132 | 16 | 1 | 1 | -3.1293 | 3.3732 | -2.2571 | 3.3622 |
| 120 | 2 | 32 | 32 | 4 | 1 | -3.0486 | 2.7093 | -2.9339 | 2.9962 |
|  |  | 64 | 16 | 1 | 1 | -4.6134 | 4.7202 | -1.791 | 5.17 |
|  | 4 | 32 | 32 | 4 | 1 | -2.7871 | 2.9017 | -2.6995 | 2.9962 |
|  |  | 64 | 16 | 1 | 1 | -4.3127 | 4.6752 | -2.7066 | 4.7386 |

Table 5: DL RSCPD error for AWGN channel in degree Es/Iot (cell-1, cell-2, cell-3) = (0, -6, -6) dB.

DL RSCP

The DL RSCP error in degree for AWGN channel, derived from the 5-percentile and 95-percentile of the CDF, for Es/IoT of three cells (-6, -13, -13) dB, (-3, -6, -6) dB, (-3, -13, -13) dB, and (0, -6, -6) dB,  respectively, are listed in Table 6 to Table 9.

| SCS (kHz) | Carrier frequency (GHz) | PRS BW in RB | Sampling rate (Tc) | Rep | Number of samples | RSCP (cell-1) -6 dB, 0 μs |  | RSCP (cell-2) -13 dB, 0 μs |  | RSCP (cell-3) -13 dB, 3 μs |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 5%-tile | 95%-tile | 5%-tile | 95%-tile | 5%-tile | 95%-tile |
| 15 | 2 | 24 | 256 | 4 | 1 | -2.6667 | 2.6658 | -6.2551 | 6.1742 | -6.1745 | 5.9168 |
|  |  | 52 | 128 | 1 | 1 | -4.2691 | 4.2702 | -9.4018 | 9.6857 | -8.6974 | 8.8846 |
|  |  | 104 | 64 | 1 | 1 | -2.8854 | 2.9664 | -6.5126 | 7.0146 | -6.7411 | 6.7466 |
|  | 4 | 24 | 256 | 4 | 1 | -2.9339 | 2.7548 | -5.7608 | 5.8188 | -6.1432 | 5.8984 |
|  |  | 52 | 128 | 1 | 1 | -4.1562 | 4.3747 | -9.3582 | 9.4594 | -9.5305 | 9.7699 |
|  |  | 104 | 64 | 1 | 1 | -2.8711 | 2.9804 | -6.6479 | 6.8092 | -6.8241 | 6.7488 |
| 30 | 2 | 24 | 128 | 4 | 1 | -2.6398 | 2.7701 | -6.1181 | 5.9664 | -6.272 | 6.1811 |
|  |  | 48 | 64 | 1 | 1 | -4.4878 | 4.4103 | -9.1952 | 9.7644 | -9.5622 | 10.0271 |
|  | 4 | 24 | 128 | 4 | 1 | -6.9295 | 6.3495 | -6.5976 | 6.3496 | -6.9295 | 6.3495 |
|  |  | 48 | 64 | 1 | 1 | -4.5358 | 4.3206 | -9.318 | 10.1034 | -9.7193 | 9.4497 |
| 60 | 2 | 24 | 64 | 4 | 1 | -2.7045 | 2.8274 | -6.1234 | 6.0468 | -6.3802 | 6.1936 |
|  | 4 | 24 | 64 | 4 | 1 | -2.6398 | 2.7701 | -6.1181 | 5.9664 | -5.9047 | 5.9006 |
|  | 40 | 24 | 64 | 4 | 1 | -2.6398 | 2.7701 | -6.1181 | 5.9664 | -5.9047 | 5.9006 |
|  |  | 64 | 32 | 1 | 1 | -4.4258 | 3.9858 | -9.1297 | 9.2277 | -8.5242 | 9.4599 |
|  |  | 132 | 16 | 1 | 1 | -2.9057 | 2.973 | -6.3703 | 6.6748 | -1.5767 | 6.8451 |
| 120 | 2 | 32 | 32 | 4 | 1 | -2.6122 | 2.6115 | -5.9618 | 5.8402 | -5.5878 | 6.3191 |
|  |  | 64 | 16 | 1 | 1 | -4.4135 | 4.0179 | -9.093 | 9.262 | -0.6401 | 10.1282 |
|  | 4 | 32 | 32 | 4 | 1 | -2.4677 | 2.4874 | -5.3338 | 5.7701 | -5.5165 | 5.9383 |
|  |  | 64 | 16 | 1 | 1 | -4.3171 | 4.1205 | -9.0184 | 9.2927 | -1.5143 | 9.5141 |

Table 6: DL RSCP error for AWGN channel in degree Es/Iot (cell-1, cell-2, cell-3) = (-6, -13, -13) dB.

| SCS (kHz) | Carrier frequency (GHz) | PRS BW in RB | Sampling rate (Tc) | Rep | Number of samples | RSCP (cell-1) -3 dB, 0 μs |  | RSCP (cell-2) -6 dB, 0 μs |  | RSCP (cell-3) -6 dB, 3 μs |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 5%-tile | 95%-tile | 5%-tile | 95%-tile | 5%-tile | 95%-tile |
| 15 | 2 | 24 | 256 | 4 | 1 | -1.921 | 1.8935 | -2.7478 | 2.7167 | -2.8293 | 2.651 |
|  |  | 52 | 128 | 1 | 1 | -3.0439 | 3.0274 | -4.1926 | 4.192 | -3.9496 | 4.084 |
|  |  | 104 | 64 | 1 | 1 | -2.0534 | 2.1137 | -2.8992 | 3.1476 | -3.0137 | 3.0356 |
|  | 4 | 24 | 256 | 4 | 1 | -2.1001 | 1.9352 | -2.5973 | 2.6409 | -2.7935 | 2.6464 |
|  |  | 52 | 128 | 1 | 1 | -2.9484 | 3.0883 | -4.2015 | 4.3831 | -4.3818 | 4.2956 |
|  |  | 104 | 64 | 1 | 1 | -2.0462 | 2.0711 | -2.9325 | 3.0063 | -2.9405 | 3.0343 |
| 30 | 2 | 24 | 128 | 4 | 1 | -1.8547 | 1.9968 | -2.6733 | 2.6833 | -2.84 | 2.7528 |
|  |  | 48 | 64 | 1 | 1 | -3.201 | 3.1159 | -4.0838 | 4.3355 | -4.3124 | 4.4146 |
|  | 4 | 24 | 128 | 4 | 1 | -1.921 | 1.8935 | -2.7478 | 2.7167 | -2.6596 | 2.6583 |
|  |  | 48 | 64 | 1 | 1 | -3.2275 | 3.083 | -4.2345 | 4.524 | -4.3011 | 4.2564 |
| 60 | 2 | 24 | 64 | 4 | 1 | -1.9127 | 2.0281 | -2.7571 | 2.6108 | -2.858 | 2.6846 |
|  | 4 | 24 | 64 | 4 | 1 | -1.8547 | 1.9968 | -2.6733 | 2.6833 | -2.6384 | 2.6751 |
|  | 40 | 24 | 64 | 4 | 1 | -1.8547 | 1.9968 | -2.6733 | 2.6833 | -2.6384 | 2.6751 |
|  |  | 64 | 32 | 1 | 1 | -3.1303 | 2.8241 | -4.0102 | 4.1331 | -3.9115 | 4.2221 |
|  |  | 132 | 16 | 1 | 1 | -2.0569 | 2.1166 | -2.8754 | 3.0682 | -1.6857 | 2.9273 |
| 120 | 2 | 32 | 32 | 4 | 1 | -1.8463 | 1.8714 | -2.7429 | 2.6006 | -2.5955 | 2.8443 |
|  |  | 64 | 16 | 1 | 1 | -3.1282 | 2.8274 | -4.043 | 4.1918 | -0.7541 | 4.1964 |
|  | 4 | 32 | 32 | 4 | 1 | -1.7461 | 1.7596 | -2.4354 | 2.6371 | -2.5164 | 2.7015 |
|  |  | 64 | 16 | 1 | 1 | -3.0781 | 2.8965 | -4.0777 | 4.1917 | -1.6387 | 3.9511 |

Table 7. DL RSCP error for AWGN channel in degree for Es/Iot (cell-1, cell-2, cell-3) = (-3, -6, -6) dB.

| SCS (kHz) | Carrier frequency (GHz) | PRS BW in RB | Sampling rate (Tc) | Rep | Number of samples | RSCP (cell-1) -3 dB, 0 μs |  | RSCP (cell-2) -13 dB, 0 μs |  | RSCP (cell-3) -13 dB, 3 μs |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 5%-tile | 95%-tile | 5%-tile | 95%-tile | 5%-tile | 95%-tile |
| 15 | 2 | 24 | 256 | 4 | 1 | -1.921 | 1.8935 | -6.2551 | 6.1742 | -6.1745 | 5.9168 |
|  |  | 52 | 128 | 1 | 1 | -3.0439 | 3.0274 | -9.4018 | 9.6857 | -8.6974 | 8.8846 |
|  |  | 104 | 64 | 1 | 1 | -2.0534 | 2.1137 | -6.5126 | 7.0146 | -6.7411 | 6.7466 |
|  | 4 | 24 | 256 | 4 | 1 | -2.1001 | 1.9352 | -5.7608 | 5.8188 | -6.1432 | 5.8984 |
|  |  | 52 | 128 | 1 | 1 | -2.9484 | 3.0883 | -9.3582 | 9.4594 | -9.5305 | 9.7699 |
|  |  | 104 | 64 | 1 | 1 | -2.0462 | 2.0711 | -6.6479 | 6.8092 | -6.8241 | 6.7488 |
| 30 | 2 | 24 | 128 | 4 | 1 | -1.8547 | 1.9968 | -6.1181 | 5.9664 | -6.272 | 6.1811 |
|  |  | 48 | 64 | 1 | 1 | -3.201 | 3.1159 | -9.1952 | 9.7644 | -9.5622 | 10.0271 |
|  | 4 | 24 | 128 | 4 | 1 | -1.921 | 1.8935 | -6.2551 | 6.1742 | -5.9229 | 6.0331 |
|  |  | 48 | 64 | 1 | 1 | -3.2275 | 3.083 | -9.318 | 10.1034 | -9.7193 | 9.4497 |
| 60 | 2 | 24 | 64 | 4 | 1 | -1.9127 | 2.0281 | -6.1234 | 6.0468 | -6.3802 | 6.1936 |
|  | 4 | 24 | 64 | 4 | 1 | -1.8547 | 1.9968 | -6.1181 | 5.9664 | -5.9047 | 5.9006 |
|  | 40 | 24 | 64 | 4 | 1 | -1.8547 | 1.9968 | -6.1181 | 5.9664 | -5.9047 | 5.9006 |
|  |  | 64 | 32 | 1 | 1 | -3.1303 | 2.8241 | -9.1297 | 9.2277 | -8.5242 | 9.4599 |
|  |  | 132 | 16 | 1 | 1 | -2.0569 | 2.1166 | -6.3703 | 6.6748 | -1.5767 | 6.8451 |
| 120 | 2 | 32 | 32 | 4 | 1 | -1.8463 | 1.8714 | -5.9618 | 5.8402 | -5.5878 | 6.3191 |
|  |  | 64 | 16 | 1 | 1 | -3.1282 | 2.8274 | -9.093 | 9.262 | -0.6401 | 10.1282 |
|  | 4 | 32 | 32 | 4 | 1 | -1.7461 | 1.7596 | -5.3338 | 5.7701 | -5.5165 | 5.9383 |
|  |  | 64 | 16 | 1 | 1 | -3.0781 | 2.8965 | -9.0184 | 9.2927 | -1.5143 | 9.5141 |

Table 8: DL RSCP error for AWGN channel in degree for Es/Iot (cell-1, cell-2, cell-3) = (-3, -13, -13) dB.

| SCS (kHz) | Carrier frequency (GHz) | PRS BW in RB | Sampling rate (Tc) | Rep | Number of samples | RSCP (cell-1) 0 dB, 0 μs |  | RSCP (cell-2) -6 dB, 0 μs |  | RSCP (cell-3) -6 dB, 3 μs |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 5%-tile | 95%-tile | 5%-tile | 95%-tile | 5%-tile | 95%-tile |
| 15 | 2 | 24 | 256 | 4 | 1 | -1.3526 | 1.356 | -2.7478 | 2.7167 | -2.8293 | 2.651 |
|  |  | 52 | 128 | 1 | 1 | -2.158 | 2.1314 | -4.1926 | 4.192 | -3.9496 | 4.084 |
|  |  | 104 | 64 | 1 | 1 | -1.4459 | 1.5015 | -2.8992 | 3.1476 | -3.0137 | 3.0356 |
|  | 4 | 24 | 256 | 4 | 1 | -1.4631 | 1.371 | -2.5973 | 2.6409 | -2.7935 | 2.6464 |
|  |  | 52 | 128 | 1 | 1 | -2.0883 | 2.1908 | -4.2015 | 4.3831 | -4.3818 | 4.2956 |
|  |  | 104 | 64 | 1 | 1 | -1.473 | 1.4894 | -2.9325 | 3.0063 | -2.9405 | 3.0343 |
| 30 | 2 | 24 | 128 | 4 | 1 | -1.3106 | 1.4443 | -2.6733 | 2.6833 | -2.84 | 2.7528 |
|  |  | 48 | 64 | 1 | 1 | -2.2845 | 2.1813 | -4.0838 | 4.3355 | -4.3124 | 4.4146 |
|  | 4 | 24 | 128 | 4 | 1 | -1.3526 | 1.356 | -2.7478 | 2.7167 | -2.6596 | 2.6583 |
|  |  | 48 | 64 | 1 | 1 | -2.2871 | 2.1721 | -4.2345 | 4.524 | -4.3011 | 4.2564 |
| 60 | 2 | 24 | 64 | 4 | 1 | -1.3586 | 1.4233 | -2.7571 | 2.6108 | -2.858 | 2.6846 |
|  | 4 | 24 | 64 | 4 | 1 | -1.3106 | 1.4443 | -2.6733 | 2.6833 | -2.6384 | 2.6751 |
|  | 40 | 24 | 64 | 4 | 1 | -1.3106 | 1.4443 | -2.6733 | 2.6833 | -2.6384 | 2.6751 |
|  |  | 64 | 32 | 1 | 1 | -2.2179 | 2.0197 | -4.0102 | 4.1331 | -3.9115 | 4.2221 |
|  |  | 132 | 16 | 1 | 1 | -1.4418 | 1.521 | -2.8754 | 3.0682 | -1.6857 | 2.9273 |
| 120 | 2 | 32 | 32 | 4 | 1 | -1.2861 | 1.3169 | -2.7429 | 2.6006 | -2.5955 | 2.8443 |
|  |  | 64 | 16 | 1 | 1 | -2.2166 | 2.0131 | -4.043 | 4.1918 | -0.7541 | 4.1964 |
|  | 4 | 32 | 32 | 4 | 1 | -1.225 | 1.2749 | -2.4354 | 2.6371 | -2.5164 | 2.7015 |
|  |  | 64 | 16 | 1 | 1 | -2.1573 | 2.0452 | -4.0777 | 4.1917 | -1.6387 | 3.9511 |

Table 9: DL RSCP error for AWGN channel in degree for Es/Iot (cell-1, cell-2, cell-3) = (0, -6, -6) dB.

Two-tap channel

DL RSCPD

The DL RSCPD error in degree for two-tap channel, derived from the 5-percentile and 95-percentile of the CDF, for Es/IoT of three cells (-6, -13, -13) dB, (-3, -6, -6) dB, (-3, -13, -13) dB, and (0, -6, -6) dB, respectively, are listed in Table 10 to Table 13. These results are to be considered as preliminary since they are derived using 1 UE receive antenna (deviating from the simulation assumptions in [6] where 2 UE receive antennas are indicated).

| SCS (kHz) | Carrier frequency (GHz) | PRS BW in RB | Sampling rate (Tc) | Rep | Number of samples | RSCPD (cell-1, cell-2) (-6, -13) dB |  | RSCPD (cell-1, cell-3) (-6, -13) dB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 5%-tile | 95%-tile | 5%-tile | 95%-tile |
| 15 | 2 | 24 | 256 | 4 | 1 | -19.1186 | 22.907 | -27.1381 | 45.9378 |
|  |  | 52 | 128 | 1 | 1 | -50.773 | 55.0535 | -59.5446 | 76.9358 |
|  |  | 104 | 64 | 1 | 1 | -20.2801 | 36.4185 | -35.4387 | 58.6672 |
|  | 4 | 24 | 256 | 4 | 1 | -19.1866 | 22.4811 | -25.6942 | 45.9 |
|  |  | 52 | 128 | 1 | 1 | -50.7687 | 55.0761 | -59.5596 | 76.9402 |
|  |  | 104 | 64 | 1 | 1 | -20.2767 | 36.4185 | -35.4387 | 58.6672 |
| 30 | 2 | 24 | 128 | 4 | 1 | -26.8962 | 25.0842 | -26.1744 | 38.8631 |
|  |  | 48 | 64 | 1 | 1 | -51.6379 | 42.9979 | -78.4669 | 96.7505 |
|  | 4 | 24 | 128 | 4 | 1 | -26.8866 | 24.9657 | -26.1503 | 38.8983 |
|  |  | 48 | 64 | 1 | 1 | -51.6379 | 42.9979 | -78.4669 | 96.2431 |
| 60 | 2 | 24 | 64 | 4 | 1 | -20.4325 | 22.0202 | -28.1383 | 29.7192 |
|  | 4 | 24 | 64 | 4 | 1 | -20.4336 | 22.0202 | -28.1383 | 29.7192 |
|  | 40 | 24 | 64 | 4 | 1 | -20.3595 | 22.0063 | -28.1383 | 29.7192 |
|  |  | 64 | 32 | 1 | 1 | -54.2997 | 66.4184 | -87.6091 | 88.745 |
|  |  | 132 | 16 | 1 | 1 | -35.1563 | 25.606 | -57.637 | 54.0888 |
| 120 | 2 | 32 | 32 | 4 | 1 | -15.7714 | 17.7136 | -22.023 | 30.314 |
|  |  | 64 | 16 | 1 | 1 | -51.1752 | 41.8067 | -87.1786 | 85.6112 |
|  | 4 | 32 | 32 | 4 | 1 | -15.8193 | 17.5031 | -22.4516 | 29.8199 |
|  |  | 64 | 16 | 1 | 1 | -51.171 | 41.8067 | -87.1786 | 85.6112 |

Table 10. DL RSCPD error for two-tap channel in degree for Es/Iot (cell-1, cell-2, cell-3) = (-6, -13, -13) dB.

| SCS (kHz) | Carrier frequency (GHz) | PRS BW in RB | Sampling rate (Tc) | Rep | Number of samples | RSCPD (cell-1, cell-2) (-3, -6) dB |  | RSCPD (cell-1, cell-3) (-3, -6) dB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 5%-tile | 95%-tile | 5%-tile | 95%-tile |
| 15 | 2 | 24 | 256 | 4 | 1 | -9.2213 | 10.4345 | -12.8116 | 13.622 |
|  |  | 52 | 128 | 1 | 1 | -15.7302 | 17.2933 | -15.8351 | 15.8153 |
|  |  | 104 | 64 | 1 | 1 | -9.2628 | 8.8224 | -10.7092 | 9.406 |
|  | 4 | 24 | 256 | 4 | 1 | -9.2144 | 10.4905 | -12.7873 | 13.6416 |
|  |  | 52 | 128 | 1 | 1 | -16.0061 | 17.2928 | -15.8354 | 15.8111 |
|  |  | 104 | 64 | 1 | 1 | -9.2528 | 8.8216 | -10.7092 | 9.5211 |
| 30 | 2 | 24 | 128 | 4 | 1 | -11.3545 | 10.9857 | -10.657 | 10.7485 |
|  |  | 48 | 64 | 1 | 1 | -16.1613 | 17.5397 | -16.6705 | 19.278 |
|  | 4 | 24 | 128 | 4 | 1 | -11.252 | 10.9883 | -10.6599 | 10.7249 |
|  |  | 48 | 64 | 1 | 1 | -16.1613 | 17.5397 | -16.6705 | 19.2742 |
| 60 | 2 | 24 | 64 | 4 | 1 | -8.7773 | 9.4486 | -9.4862 | 8.2269 |
|  | 4 | 24 | 64 | 4 | 1 | -8.7773 | 9.4486 | -9.471 | 8.2269 |
|  | 40 | 24 | 64 | 4 | 1 | -8.7773 | 9.4486 | -9.61 | 8.1814 |
|  |  | 64 | 32 | 1 | 1 | -13.8475 | 16.8359 | -14.8658 | 15.7676 |
|  |  | 132 | 16 | 1 | 1 | -9.2581 | 9.4795 | -9.9381 | 9.4063 |
| 120 | 2 | 32 | 32 | 4 | 1 | -7.7356 | 7.5642 | -7.9645 | 7.3332 |
|  |  | 64 | 16 | 1 | 1 | -13.5395 | 14.1875 | -14.0252 | 15.1414 |
|  | 4 | 32 | 32 | 4 | 1 | -7.733 | 7.5588 | -7.9645 | 7.3798 |
|  |  | 64 | 16 | 1 | 1 | -13.5395 | 14.1875 | -14.0252 | 15.1414 |

Table 11. DL RSCPD error for two-tap channel in degree for Es/Iot (cell-1, cell-2, cell-3) = (-3, -6, -6) dB.

| SCS (kHz) | Carrier frequency (GHz) | PRS BW in RB | Sampling rate (Tc) | Rep | Number of samples | RSCPD (cell-1, cell-2) (-3, -13) dB |  | RSCPD (cell-1, cell-3) (-3, -13) dB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 5%-tile | 95%-tile | 5%-tile | 95%-tile |
| 15 | 2 | 24 | 256 | 4 | 1 | -17.6274 | 24.409 | -22.7108 | 47.0039 |
|  |  | 52 | 128 | 1 | 1 | -40.0868 | 56.4135 | -54.2231 | 82.0802 |
|  |  | 104 | 64 | 1 | 1 | -18.7818 | 27.6657 | -34.7045 | 50.9101 |
|  | 4 | 24 | 256 | 4 | 1 | -17.6365 | 24.3335 | -22.6921 | 47.0063 |
|  |  | 52 | 128 | 1 | 1 | -40.0868 | 56.4135 | -54.215 | 82.0803 |
|  |  | 104 | 64 | 1 | 1 | -18.5953 | 27.6903 | -34.7045 | 50.9101 |
| 30 | 2 | 24 | 128 | 4 | 1 | -23.7961 | 21.6874 | -25.782 | 37.0526 |
|  |  | 48 | 64 | 1 | 1 | -48.036 | 41.713 | -79.3176 | 91.8228 |
|  | 4 | 24 | 128 | 4 | 1 | -23.8204 | 21.5678 | -25.782 | 37.0526 |
|  |  | 48 | 64 | 1 | 1 | -48.036 | 41.713 | -79.3176 | 91.1238 |
| 60 | 2 | 24 | 64 | 4 | 1 | -19.1454 | 20.075 | -29.0828 | 26.3862 |
|  | 4 | 24 | 64 | 4 | 1 | -19.1454 | 20.075 | -28.901 | 26.3859 |
|  | 40 | 24 | 64 | 4 | 1 | -19.1445 | 20.2135 | -29.7719 | 26.3892 |
|  |  | 64 | 32 | 1 | 1 | -49.0445 | 55.6294 | -85.4666 | 87.6859 |
|  |  | 132 | 16 | 1 | 1 | -30.3508 | 22.3138 | -55.3139 | 49.6825 |
| 120 | 2 | 32 | 32 | 4 | 1 | -14.4164 | 15.5726 | -20.639 | 25.3424 |
|  |  | 64 | 16 | 1 | 1 | -50.4593 | 42.1306 | -86.8111 | 87.1571 |
|  | 4 | 32 | 32 | 4 | 1 | -14.418 | 15.8167 | -20.6383 | 27.2133 |
|  |  | 64 | 16 | 1 | 1 | -50.4593 | 42.1305 | -86.8111 | 87.1571 |

Table 12. DL RSCPD error for two-tap channel in degree for Es/Iot (cell-1, cell-2, cell-3) = (-3, -13, -13) dB.

| SCS (kHz) | Carrier frequency (GHz) | PRS BW in RB | Sampling rate (Tc) | Rep | Number of samples | RSCPD (cell-1, cell-2) (0, -6) dB |  | RSCPD (cell-1, cell-3) (0, -6) dB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 5%-tile | 95%-tile | 5%-tile | 95%-tile |
| 15 | 2 | 24 | 256 | 4 | 1 | -8.6958 | 10.2313 | -10.7912 | 12.6174 |
|  |  | 52 | 128 | 1 | 1 | -12.7643 | 15.4333 | -13.7043 | 13.8491 |
|  |  | 104 | 64 | 1 | 1 | -8.293 | 7.9893 | -8.9253 | 8.7552 |
|  | 4 | 24 | 256 | 4 | 1 | -8.7001 | 10.2841 | -10.714 | 12.6174 |
|  |  | 52 | 128 | 1 | 1 | -12.7333 | 15.4396 | -13.7168 | 13.8464 |
|  |  | 104 | 64 | 1 | 1 | -8.2857 | 7.9841 | -8.9253 | 8.7552 |
| 30 | 2 | 24 | 128 | 4 | 1 | -9.7475 | 8.6581 | -9.7569 | 9.7262 |
|  |  | 48 | 64 | 1 | 1 | -13.6528 | 15.7888 | -13.9038 | 16.8513 |
|  | 4 | 24 | 128 | 4 | 1 | -9.7488 | 8.8849 | -9.7596 | 9.7262 |
|  |  | 48 | 64 | 1 | 1 | -13.658 | 15.7888 | -13.8999 | 16.8812 |
| 60 | 2 | 24 | 64 | 4 | 1 | -7.7324 | 8.5904 | -8.5815 | 7.7444 |
|  | 4 | 24 | 64 | 4 | 1 | -7.581 | 8.5904 | -8.6105 | 7.7544 |
|  | 40 | 24 | 64 | 4 | 1 | -7.5557 | 8.5904 | -8.5815 | 7.7444 |
|  |  | 64 | 32 | 1 | 1 | -12.2401 | 14.6178 | -13.2846 | 14.0124 |
|  |  | 132 | 16 | 1 | 1 | -8.5007 | 8.5681 | -8.4706 | 8.1559 |
| 120 | 2 | 32 | 32 | 4 | 1 | -7.039 | 7.1643 | -7.1216 | 6.5541 |
|  |  | 64 | 16 | 1 | 1 | -12.0236 | 12.9459 | -12.881 | 13.2667 |
|  | 4 | 32 | 32 | 4 | 1 | -7.0391 | 7.1643 | -7.1216 | 6.5541 |
|  |  | 64 | 16 | 1 | 1 | -12.0216 | 12.9459 | -12.881 | 13.2667 |

Table 13. DL RSCPD error for two-tap channel in degree for SCS = 15kHz. Es/Iot (cell-1, cell-2, cell-3) = (0, -6, -6) dB.

DL RSCP

The DL RSCP error in degree for two-tap channel, derived from the 5-percentile and 95-percentile of the CDF, for Es/IoT of three cells (-6, -13, -13) dB, (-3, -6, -6) dB, (-3, -13, -13) dB, and (0, -6, -6) dB, respectively, are listed in Table 14 to Table 17. These results are to be considered as preliminary since they are derived using 1 UE receive antenna (deviating from the simulation assumptions in  where 2 UE receive antennas are indicated).

| SCS (kHz) | Carrier frequency (GHz) | PRS BW in RB | Sampling rate (Tc) | Rep | Number of samples | RSCP (cell-1) -6 dB, 0 μs |  | RSCP (cell-2) -13 dB, 0 μs |  | RSCP (cell-3) -13 dB, 3 μs |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 5%-tile | 95%-tile | 5%-tile | 95%-tile | 5%-tile | 95%-tile |
| 15 | 2 | 24 | 256 | 4 | 1 | -83.3134 | 80.3118 | -84.3325 | 83.6978 | -83.6926 | 95.1042 |
|  |  | 52 | 128 | 1 | 1 | -86.0788 | 93.2411 | -85.4821 | 97.1709 | -87.7483 | 106.0892 |
|  |  | 104 | 64 | 1 | 1 | -87.06 | 92.6728 | -88.3761 | 99.4904 | -89.2993 | 97.9327 |
|  | 4 | 24 | 256 | 4 | 1 | -83.4657 | 79.5938 | -84.6747 | 83.6978 | -83.3003 | 95.1042 |
|  |  | 52 | 128 | 1 | 1 | -86.141 | 92.9867 | -85.4821 | 97.1709 | -87.7618 | 106.0891 |
|  |  | 104 | 64 | 1 | 1 | -87.06 | 92.6725 | -88.3761 | 99.4904 | -89.2993 | 97.9327 |
| 30 | 2 | 24 | 128 | 4 | 1 | -42.5051 | 88.9661 | -38.6984 | 81.2945 | -41.5831 | 91.5791 |
|  |  | 48 | 64 | 1 | 1 | -35.0923 | 90.0713 | -47.4891 | 98.4806 | -78.8636 | 115.3922 |
|  | 4 | 24 | 128 | 4 | 1 | -42.5051 | 88.9739 | -38.7124 | 81.151 | -41.5831 | 91.5791 |
|  |  | 48 | 64 | 1 | 1 | -35.0923 | 90.0319 | -47.4891 | 98.453 | -78.8636 | 115.3922 |
| 60 | 2 | 24 | 64 | 4 | 1 | -46.5371 | 94.2246 | -50.4784 | 92.8137 | -58.4812 | 100.5354 |
|  | 4 | 24 | 64 | 4 | 1 | -46.5371 | 94.2263 | -50.4784 | 92.8137 | -58.4798 | 100.6261 |
|  | 40 | 24 | 64 | 4 | 1 | -46.5371 | 94.228 | -50.3081 | 93.067 | -58.4812 | 100.5354 |
|  |  | 64 | 32 | 1 | 1 | -43.9526 | 92.6071 | -53.1939 | 103.6174 | -82.8536 | 117.2895 |
|  |  | 132 | 16 | 1 | 1 | -49.2042 | 91.1929 | -52.2269 | 101.0544 | -66.4791 | 106.1165 |
| 120 | 2 | 32 | 32 | 4 | 1 | -51.9461 | 90.1638 | -52.3906 | 92.0823 | -60.5719 | 93.8773 |
|  |  | 64 | 16 | 1 | 1 | -48.6882 | 92.8896 | -62.6854 | 94.0498 | -80.4165 | 114.6625 |
|  | 4 | 32 | 32 | 4 | 1 | -52.0302 | 91.1628 | -52.3906 | 92.0823 | -60.5719 | 93.8773 |
|  |  | 64 | 16 | 1 | 1 | -48.6882 | 92.8896 | -62.6824 | 94.0432 | -80.4165 | 114.6625 |

Table 14: DL RSCP error for two-tap channel in degree for Es/Iot (cell-1, cell-2, cell-3) = (-6, -13, -13) dB.

| SCS (kHz) | Carrier frequency (GHz) | PRS BW in RB | Sampling rate (Tc) | Rep | Number of samples | RSCP (cell-1) -3 dB, 0 μs |  | RSCP (cell-2) -6 dB, 0 μs |  | RSCP (cell-3) -6 dB, 3 μs |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 5%-tile | 95%-tile | 5%-tile | 95%-tile | 5%-tile | 95%-tile |
| 15 | 2 | 24 | 256 | 4 | 1 | -83.0543 | 73.4367 | -82.887 | 72.5827 | -85.4465 | 78.6139 |
|  |  | 52 | 128 | 1 | 1 | -86.7897 | 89.9234 | -84.261 | 88.9335 | -86.8976 | 88.8109 |
|  |  | 104 | 64 | 1 | 1 | -87.4756 | 92.3456 | -88.7985 | 90.5652 | -88.8116 | 91.601 |
|  | 4 | 24 | 256 | 4 | 1 | -83.0543 | 73.4367 | -82.887 | 72.5827 | -85.4465 | 78.6139 |
|  |  | 52 | 128 | 1 | 1 | -86.7897 | 89.9234 | -84.261 | 88.9335 | -86.8712 | 88.8085 |
|  |  | 104 | 64 | 1 | 1 | -87.4756 | 92.3721 | -88.9153 | 90.5647 | -88.8284 | 91.8143 |
| 30 | 2 | 24 | 128 | 4 | 1 | -41.8395 | 86.1352 | -40.6387 | 85.0732 | -40.1693 | 87.5786 |
|  |  | 48 | 64 | 1 | 1 | -33.2853 | 90.1439 | -34.998 | 95.043 | -42.7897 | 101.508 |
|  | 4 | 24 | 128 | 4 | 1 | -41.8395 | 86.1229 | -40.6387 | 85.0732 | -40.1693 | 87.5786 |
|  |  | 48 | 64 | 1 | 1 | -33.2853 | 90.1439 | -34.998 | 95.043 | -42.7897 | 101.508 |
| 60 | 2 | 24 | 64 | 4 | 1 | -47.262 | 95.935 | -45.0751 | 93.7813 | -48.8054 | 97.5841 |
|  | 4 | 24 | 64 | 4 | 1 | -47.262 | 95.935 | -45.0751 | 93.7813 | -48.8054 | 97.5841 |
|  | 40 | 24 | 64 | 4 | 1 | -47.262 | 95.935 | -45.0751 | 93.7813 | -48.8054 | 97.5841 |
|  |  | 64 | 32 | 1 | 1 | -45.835 | 93.0437 | -45.2851 | 97.0709 | -48.9043 | 94.3873 |
|  |  | 132 | 16 | 1 | 1 | -48.6014 | 92.6265 | -45.2668 | 94.6595 | -48.3772 | 95.1664 |
| 120 | 2 | 32 | 32 | 4 | 1 | -50.3105 | 91.4829 | -49.6617 | 87.4133 | -48.0351 | 90.743 |
|  |  | 64 | 16 | 1 | 1 | -48.6149 | 91.1915 | -51.349 | 87.8262 | -52.6663 | 90.4685 |
|  | 4 | 32 | 32 | 4 | 1 | -50.2273 | 91.2636 | -49.6617 | 87.4133 | -48.0351 | 90.743 |
|  |  | 64 | 16 | 1 | 1 | -48.6149 | 91.1915 | -51.349 | 87.8262 | -52.6663 | 90.4546 |

Table 15. DL RSCP error for two-tap channel in degree for Es/Iot (cell-1, cell-2, cell-3) = (-3, -6, -6) dB.

| SCS (kHz) | Carrier frequency (GHz) | PRS BW in RB | Sampling rate (Tc) | Rep | Number of samples | RSCP (cell-1) -3 dB, 0 μs |  | RSCP (cell-2) -13 dB, 0 μs |  | RSCP (cell-3) -13 dB, 3 μs |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 5%-tile | 95%-tile | 5%-tile | 95%-tile | 5%-tile | 95%-tile |
| 15 | 2 | 24 | 256 | 4 | 1 | -83.0543 | 73.4367 | -84.3325 | 83.6978 | -83.6926 | 95.1042 |
|  |  | 52 | 128 | 1 | 1 | -86.7897 | 89.9234 | -85.4821 | 97.1709 | -87.7483 | 106.0892 |
|  |  | 104 | 64 | 1 | 1 | -87.4756 | 92.3456 | -88.3761 | 99.4904 | -89.2993 | 97.9327 |
|  | 4 | 24 | 256 | 4 | 1 | -83.0543 | 73.4367 | -84.6747 | 83.6978 | -83.3003 | 95.1042 |
|  |  | 52 | 128 | 1 | 1 | -86.7897 | 89.9234 | -85.4821 | 97.1709 | -87.7618 | 106.0891 |
|  |  | 104 | 64 | 1 | 1 | -87.4756 | 92.3721 | -88.3761 | 99.4904 | -89.2993 | 97.9327 |
| 30 | 2 | 24 | 128 | 4 | 1 | -41.8395 | 86.1352 | -38.6984 | 81.2945 | -41.5831 | 91.5791 |
|  |  | 48 | 64 | 1 | 1 | -33.2853 | 90.1439 | -47.4891 | 98.4806 | -78.8636 | 115.3922 |
|  | 4 | 24 | 128 | 4 | 1 | -41.8395 | 86.1229 | -38.7124 | 81.151 | -41.5831 | 91.5791 |
|  |  | 48 | 64 | 1 | 1 | -33.2853 | 90.1439 | -47.4891 | 98.453 | -78.8636 | 115.3922 |
| 60 | 2 | 24 | 64 | 4 | 1 | -47.262 | 95.935 | -50.4784 | 92.8137 | -58.4812 | 100.5354 |
|  | 4 | 24 | 64 | 4 | 1 | -47.262 | 95.935 | -50.4784 | 92.8137 | -58.4798 | 100.6261 |
|  | 40 | 24 | 64 | 4 | 1 | -47.262 | 95.935 | -50.3081 | 93.067 | -58.4812 | 100.5354 |
|  |  | 64 | 32 | 1 | 1 | -45.835 | 93.0437 | -53.1939 | 103.6174 | -82.8536 | 117.2895 |
|  |  | 132 | 16 | 1 | 1 | -48.6014 | 92.6265 | -52.2269 | 101.0544 | -66.4791 | 106.1165 |
| 120 | 2 | 32 | 32 | 4 | 1 | -50.3105 | 91.4829 | -52.3906 | 92.0823 | -60.5719 | 93.8773 |
|  |  | 64 | 16 | 1 | 1 | -48.6149 | 91.1915 | -62.6854 | 94.0498 | -80.4165 | 114.6625 |
|  | 4 | 32 | 32 | 4 | 1 | -50.2273 | 91.2636 | -52.3906 | 92.0823 | -60.5719 | 93.8773 |
|  |  | 64 | 16 | 1 | 1 | -48.6149 | 91.1915 | -62.6824 | 94.0432 | -80.4165 | 114.6625 |

Table 16: DL RSCP error for two-tap channel in degree for Es/Iot (cell-1, cell-2, cell-3) = (-3, -13, -13) dB.

| SCS (kHz) | Carrier frequency (GHz) | PRS BW in RB | Sampling rate (Tc) | Rep | Number of samples | RSCP (cell-1) 0 dB, 0 μs |  | RSCP (cell-2) -6 dB, 0 μs |  | RSCP (cell-3) -6 dB, 3 μs |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 5%-tile | 95%-tile | 5%-tile | 95%-tile | 5%-tile | 95%-tile |
| 15 | 2 | 24 | 256 | 4 | 1 | -82.8905 | 72.1397 | -82.887 | 72.5827 | -85.4465 | 78.6139 |
|  |  | 52 | 128 | 1 | 1 | -87.4314 | 90.5053 | -84.261 | 88.9335 | -86.8976 | 88.8109 |
|  |  | 104 | 64 | 1 | 1 | -88.0224 | 91.9566 | -88.7985 | 90.5652 | -88.8116 | 91.601 |
|  | 4 | 24 | 256 | 4 | 1 | -82.9493 | 72.1397 | -82.887 | 72.5827 | -85.4465 | 78.6139 |
|  |  | 52 | 128 | 1 | 1 | -87.4314 | 90.5053 | -84.261 | 88.9335 | -86.8712 | 88.8085 |
|  |  | 104 | 64 | 1 | 1 | -88.0224 | 91.9566 | -88.9153 | 90.5647 | -88.8284 | 91.8143 |
| 30 | 2 | 24 | 128 | 4 | 1 | -41.3538 | 85.9036 | -40.6387 | 85.0732 | -40.1693 | 87.5786 |
|  |  | 48 | 64 | 1 | 1 | -37.6348 | 92.1988 | -34.998 | 95.043 | -42.7897 | 101.508 |
|  | 4 | 24 | 128 | 4 | 1 | -41.3801 | 85.9036 | -40.6387 | 85.0732 | -40.1693 | 87.5786 |
|  |  | 48 | 64 | 1 | 1 | -37.6348 | 92.1988 | -34.998 | 95.043 | -42.7897 | 101.508 |
| 60 | 2 | 24 | 64 | 4 | 1 | -47.1933 | 95.0664 | -45.0751 | 93.7813 | -48.8054 | 97.5841 |
|  | 4 | 24 | 64 | 4 | 1 | -47.1933 | 95.0664 | -45.0751 | 93.7813 | -48.8054 | 97.5841 |
|  | 40 | 24 | 64 | 4 | 1 | -47.1933 | 95.0664 | -45.0751 | 93.7813 | -48.8054 | 97.5841 |
|  |  | 64 | 32 | 1 | 1 | -47.6993 | 93.3637 | -45.2851 | 97.0709 | -48.9043 | 94.3873 |
|  |  | 132 | 16 | 1 | 1 | -49.5247 | 93.5688 | -45.2668 | 94.6595 | -48.3772 | 95.1664 |
| 120 | 2 | 32 | 32 | 4 | 1 | -49.8615 | 93.143 | -49.6617 | 87.4133 | -48.0351 | 90.743 |
|  |  | 64 | 16 | 1 | 1 | -49.0847 | 90.9395 | -51.349 | 87.8262 | -52.6663 | 90.4685 |
|  | 4 | 32 | 32 | 4 | 1 | -49.8615 | 93.143 | -49.6617 | 87.4133 | -48.0351 | 90.743 |
|  |  | 64 | 16 | 1 | 1 | -49.0847 | 90.9395 | -51.349 | 87.8262 | -52.6663 | 90.4546 |

Table 17: DL RSCP error for two-tap channel in degree for Es/Iot (cell-1, cell-2, cell-3) = (0, -6, -6) dB.

CFO impact to NR CPP

For deriving the CFO impact to NR CPP, we first assume that the UE is not exposed to the full CFO derived from its frequency stability, but only a reduced one. Note, indeed, that RAN1 previously assumed for the residual CFO (after frequency tracking) in TR 38.859, Annex A.3, following values for UE and TRP in FR1/FR2, listed in Table 18 below.

| Frequency errors – NOTE 1 | Ideal | Practical |
| --- | --- | --- |
| Initial residual CFO (is the same for one measurement instances [or multiple phase measurement instances]) | 0 (UE/TRP) | Uniform distribution within: - [-30, +30] Hz (FR1, UE), [-100, +100] Hz (FR1, UE), - [-120, +120] Hz (FR2, UE), [-400, +400] Hz (FR2, UE), - [-10, +10] Hz (for each TRP, FR1), - [-40, +40] Hz (for each TRP, FR2). |

Table 18. Assumptions for evaluation of NR carrier phase positioning [TR 38.859].

The DL RSCPD error in degree for AWGN channel with carrier frequency of 2GHz, derived from the 5-percentile and 95-percentile of the CDF, for Es/IoT of three cells (-3, -6, -6) dB without and with CFO impairment is listed in Table 19. In particular, the CFO impairment is applied according the first and third bullets of Table 18, that is, a residual CFO with uniform distribution within [-10, +10] Hz (and [-30, +30] Hz) is applied independently to the reference cell-1,to the target cell cell-2 and to the target cell cell-3. The residual CFO is applied to each of the samples.

| SCS (kHz) | Carrier frequency (GHz) | PRS BW in RB | Sampling rate (Tc) | Rep | Number of samples | Carrier frequency offset Uniform distribution within [-CFO, +CFO] (Hz) | RSCPD (cell-1, cell-2) (-3, -6) dB, 0 μs |  | RSCPD (cell-1, cell-3) (-3, -6) dB, 3 μs |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 5%-tile | 95%-tile | 5%-tile | 95%-tile |
| 15 | 2 | 24 | 256 | 4 | 1 | 0 | -4.8321 | 4.591 | -4.733 | 4.7812 |
|  |  |  |  |  |  | 10 | -13.529 | 13.2723 | -13.5594 | 13.4196 |
|  |  |  |  |  |  | 30 | -38.9133 | 38.1948 | -38.8894 | 38.1494 |
|  |  | 52 | 128 | 1 | 1 | 0 | -7.1032 | 7.5174 | -7.5198 | 7.7361 |
|  |  |  |  |  |  | 10 | -38.4427 | 39.5845 | -37.1375 | 38.8765 |
|  |  |  |  |  |  | 30 | -114.4769 | 119.9431 | -117.8187 | 121.73 |
|  |  | 104 | 64 | 1 | 1 | 0 | -5.0522 | 4.8322 | -5.0587 | 5.085 |
|  |  |  |  |  |  | 10 | -72.5865 | 75.5745 | -75.0201 | 77.2241 |
|  |  |  |  |  |  | 30 | -150.7528 | 149.7346 | -151.0678 | 148.7707 |
| 30 | 2 | 24 | 128 | 4 | 1 | 0 | -10.5154 | 10.869 | -10.7145 | 10.3805 |
|  |  |  |  |  |  | 10 | -25.3354 | 24.688 | -25.3887 | 24.9858 |
|  |  |  |  |  |  | 30 | -77.3254 | 75.0174 | -77.744 | 75.9488 |
|  |  | 48 | 64 | 1 | 1 | 0 | -7.5669 | 7.8325 | -7.471 | 7.3823 |
|  |  |  |  |  |  | 10 | -74.8393 | 79.2079 | -79.7808 | 77.7613 |
|  |  |  |  |  |  | 30 | -236.528 | 224.433 | -215.9307 | 230.7789 |
| 60 | 2 | 24 | 64 | 4 | 1 | 0 | -11.083 | 10.664 | -10.3078 | 10.5648 |
|  |  |  |  |  |  | 10 | -77.8178 | 76.2797 | -77.8842 | 73.3778 |
|  |  |  |  |  |  | 30 | -235.4035 | 229.8257 | -250.8722 | 223.4235 |

Table 19: DL RSCPD error for AWGN channel in degree with CFO and for Es/Iot (cell-1, cell-2, cell-3) = (-3, -6, -6) dB.

Discussion

From the simulation results reported above in sections 3.1 and 3.2 without RF/BB impairments, we observed:

- For the AWGN channel, for a given bandwidth, the DL RSCPD measurement accuracy does not significantly deviate for different carrier frequencies. Thus, the DL RSCPD accuracy dependency on the carrier frequency is not so significant.

- For the AWGN, the DL RSCPD measurement accuracy depends more on the cells with lower Es/Iot. For example, this effect can be noticed by comparing Table 2 with Table 4, where just the Es/Iot of cell 1 changes, and indeed the performance are very similar; meanwhile, by comparing Table 3 with Table 4 where the Es/Iot of cell 1 remains fixed and the Es/Iot of cell 2 and cell 3 changes, we notice that the performance difference is significant. Thus, the DL RSCPD accuracy mainly depends on the cells with lower Es/Iot.

Thus, following observations are made.

For the AWGN channel, the DL RSCPD accuracy dependency on carrier frequency is not so significant.

For the AWGN channel, the DL RSCPD accuracy mainly depends on the cells with lower Es/Iot.

Similar observations can be made for the two-tap channel, however we did not include it above since the results for it were derived using 1 UE receive antenna (deviating from the simulation assumptions in [6] where 2 UE receive antennas are indicated).

From the preliminary simulation results in section 3.3, we observed:

- In both configurations without time shift of the target cell cell-2 and with 3 us time shift of the target cell cell-3 versus reference cell cell-1, the DL RSCPD error is considerably increased including the CFO impairment.

- Within the same SCS configuration, the impact of the CFO impairment on the DL RSCPD error is strongly dependent on the sampling rate and the assumed maximum residual CFO.

Thus, following observation is made.

Preliminary results show that CFO does have a non-negligible impact on DL RSCPD measurement accuracy.

Current frequency stability requirements appear to be not sufficient for a solid CP estimation.

And the following proposal is made.

RAN4 to further investigate how to reduce the impact of CFO to DL RSCPD measurement accuracy.

Conclusion

In this contribution we have provided first simulation results for DL RSCPD and DL RSCP error without RF/BB impairments based on simulation assumptions in [5].

Following observations are made.

- For the AWGN channel, the DL RSCPD accuracy dependency on carrier frequency is not so significant.

For the AWGN channel, the DL RSCPD accuracy mainly depends on the cells with lower Es/Iot.

Preliminary results show that CFO does have a non-negligible impact on DL RSCPD measurement accuracy.

Current frequency stability requirements appear to be not sufficient for a solid CP estimation.

And the following proposal is made.

- RAN4 to further investigate how to reduce the impact of CFO to DL RSCPD measurement accuracy.

- References

| [1] | RP-233382, "Revised WID on Expanded and Improved NR Positioning," 3GPP TSG RAN#102. |
| --- | --- |
| [2] | TS 38.859, "Study on Expanded and Improved NR Positioning (Release 18)," V18.0.0. |
| [3] | R4-2002287, "Updated link level simulation assumption for RSTD and RSRP," Huawei. |
| [4] | R4-2211061, "Simulation assumption for PRS-RSRPP," Huawei. |
| [5] | R4-2310078, "Simulation assumptions for CPP measurement," CATT. |
| [6] | R4-2321459, "Updated simulation assumptions for CPP measurement," CATT. |