3GPP TSG-RAN WG4 Meeting #112	R4-2413314

Maastricht, Netherlands, Aug 19 – Aug 23, 2024

Source:	Nokia

Title:	Discussion on RRM requirements for XR phase 3

Agenda item:	8.24.2

Document for:	Discussion

Introduction

This paper presents Nokia’s views on the requirements for XR enhancements phase 3. The objectives agreed for XR_ph3 are as follow:

### 4.1 Objective of SI or Core part WI or Testing part WI The Rel-19 XR Phase 3 objectives are as follows: - Study and if justified, specify aspects related to multi-modality (intra-UE) (with coordination with SA2/SA4 as needed by LS request). Aim to facilitate efficient and effective support for XR application with Multiple QoS flows with multi-modal inter-dependencies, meeting multi-modal QoS requirements, e.g. synchronization and/or coordination. Efficiency enhancements are expected to be visible in terms of capacity or power consumption. [RAN2]. NOTE: Check in RAN#105 (check also other WG involvement if needed). - Specify enhancements to enable transmission/reception in gaps/restrictions that are caused by RRM measurements (from inter-frequency RRM measurement gaps, or intra-frequency measurements, or other scheduling restrictions etc). [RAN1, RAN2, RAN4] - Specify the corresponding measurement gap and scheduling restriction to enable the identified enhancements with RRM performance impact taken into consideration, work being triggered by LS. [RAN4] - Specify Enhancements for Scheduling, as follows: - For the UL, Study and if justified, Specify enhancements using delay/deadline information, for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs. [RAN2]. NOTE: LCP implementation complexity need to be taken into account when evaluating solutions. NOTE: Check in RAN#105 - Specify the following user plane enhancements [RAN2] - RLC re-transmission related enhancements for operation of RLC Acknowledged Mode (AM) with small packet delay budget. - Specify Core requirements related to the above objectives as necessary [RAN4] - Extend Release 18 standalone mechanism to support NR-NR dual connectivity as follows [RAN3] - PDU set based handling - ECN marking - Burst Arrival Time reporting, if needed - PSI Discard coordination, if needed - Note: No RAN2 impact from above items NOTE: Whether / to what extent network exposure / RAN awareness / e.g. RAN involved rate control, possibly additional info for DL scheduling, parallel with SA2 work, shall be covered in this WI is TBD.

As part of those objectives, there are clear RAN4 RRM impacts related to objective 2, regarding enhancements to enable transmission-reception during measurement gaps or scheduling restrictions. RAN1 already started the discussion regarding that objective, where RAN1 is defining methods for skipping measurement occasions.

Background of XR use case

As discussed in TR 38.838, the latency requirement of XR traffic in RAN side (i.e., air interface) is modelled as packet delay budget (PDB). The PDB is a limited time budget for a packet to be transmitted over the air from a gNB to a UE.

For a given packet, the delay of the packet incurred in air interface is measured from the time that the packet arrives at the gNB to the time that it is successfully transferred to the UE. If the delay is larger than a given PDB for the packet, then, the packet is said to violate PDB, otherwise the packet is said to be successfully delivered.

The value of PDB may vary for different applications and traffic types. Without loss of generality, we summarize the latency requirement for DL single stream only.

The AR/VR DL single stream follows generic single stream DL video traffic model in clause 5.1.1 of TR 38.838 with following parameters in Table 5.3.1-1 [3]:

Table 5.3.1-1: Statistical parameters for single stream DL VR traffic model Parameters; Unit; Baseline values for evaluation; Optional values for evaluation. data rate: R; Mbps; 30, 45; 60. frame generation rate: F; fps or Hz; 60. PDB; Ms; 10; 5, 20.

The CG DL stream follows generic single stream DL video traffic model in clause 5.1.1 of TR 38.838 with following parameters in Table 5.4.1-1 [3]:

Table 5.4.1-1: Statistical parameters for single stream CG traffic model Parameters; unit; Baseline values for evaluation; Optional values for evaluation. data rate: R; Mbps; 30, 8; 45. frame generation rate: F; fps or Hz; 60. PDB; ms; 15; 10, 30.

As shown above, for the cases that were analysed in TR 38.383, the packet delay budget may be between 5 ms to 30 ms, depending on the application. From that RAN1 selected the values of 10 and 15 ms for their discussion.

RAN1 is discussing XR measurement enhancements considering a packet delay budget between 10  and 15 ms.

As discussed at earlier RAN1 meetings (RAN1#109-e-RAN1#110bis-e), UEs performing RRM measurements does not come for free, as there are cases where this imposes scheduling restrictions, either due to potential measurement gaps for inter-frequency RRM measurements, or alike restrictions for FR2 intra-frequency RRM measurements as is discussed in greater details in this section. In line with the Rel-17 XR simulation assumptions (TR 38.838) for FR2, UEs are equipped with multiple antenna panels, where UEs may only be able to measure (and Tx/Rx) on a single panel at a time. The UE selects its best antenna panel for Tx/Rx with its serving cell based on local RRM (RSRP) measurements. The UE also performs intra-frequency RRM measurements for mobility and beam management purposes. However, for FR2 operation, it is important to notice that there are additional scheduling restrictions due to intra-frequency RRM measurements that challenge the XR performance. Notice that earlier FR2 XR performance results included in the Rel-17 XR TR 38.838 did not consider the effects of such measurement restrictions.

As per the current NR specifications, the network configures the SMTC in order to control the UE’s schedule of when the UE has to performing measurements. The time-resolution of SMTC is on subframe level, corresponding to 1 ms intervals. It should be noted that the SMTC only instruct the UE when (in time domain) it could/should measure e.g. RSRP from SSB, while it is left completely open for UE implementation exactly when it will actually measure, and which antenna panel to be used for conducting such measurement during those “SMTC measurement windows”.

Scheduling restrictions apply for the UEs during time-intervals where it may be performing SSB based measurements as per the SMTC configuration appears in 38.133, Section 9.5.6.3. In particular for FR2 and L1-RSRP on SSB, “The UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/CSI-RS…”. Typical network configurations use a setting with SMTC windows of 5 ms every 20 ms (aligned to SSB periodicity), meaning that this poses serious scheduling restrictions that likely challenge the networks capability to efficiently schedule and serve its XR users according to their QoS constraint, severely limiting the XR capacity if such scheduling restrictions are valid. Accounting for that scheduling restrictions would apply on SSBs to be measured, starting from one symbol before and ending one symbol after, means that in practice means every slot where SSBs are to be measured is restricted from PDSCH perspective, resulting in that nearly 25% of the time (i.e., SMTC windows of 5ms every 20ms time-period) can be blocked if 64 SSBs are to be measured.

Secondly, current NR specs does not mention anything related to how/when the UE shall/may measure on different antenna panels, and/or when to only measure on its currently selected best antenna panel (aligned with serving cell). One example with the timing of the SMTC windows of 5 ms with scheduling restrictions, as well as the arrival of XR frames is illustrated in Figure 1. As can be seen, the SMTC induced scheduling restrictions come every 20 ms, while the average XR frame inter-arrival time is 16.6 ms (assuming 60 fps). Additionally, the XR traffic is not deterministic, which means that each XR frame arrival is subject to +/-4ms jitter as illustrated with the dashed line. From Figure 1 it is visible that the SMTC windows with scheduling restrictions often collide with time periods where the gNB would have preferred to schedule the XR transmission. This will impact the experienced XR QoE and will negatively impact the obtained network XR capacity.

Figure 1: Sketch of timing of SMTC windows with scheduling restrictions as well as arrival of XR frames.

In a typical scenario, measurement gaps of 6 ms or SMTC of 5 ms are configured for RRM measurements. When this is done, the latency of any data packet arriving during or shortly before the gap/SMTC occasion will potentially be extended by the SMTC/gap duration.

Measurement gaps and scheduling restrictions are large in comparison to the total packet delay budget of typical XR applications.

Review of RAN1 agreements

As part of the RAN1 discussion, it is under discussion how measurement skipping can be used for enhancing performance of XR traffic. As part of the RAN1 discussion, it is being decided how it can be signalled for the UE that a given measurement occasion can be skipped, in order to prevent scheduling restrictions.

Some agreements related to that topic are shown as below:

RAN1#116-bis (April 2024): Agreement Confirm the working assumption from RAN1 #116 with updates: - RAN1 aims to develop/identify solution(s) to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements agnostic in RAN1 normative work to types of gaps/restrictions that are caused by RRM measurements. - It is up to RAN4 to discuss which type of gaps/restrictions caused by RRM measurements can be cancelled/skipped - Note: UE features related to the developed solution(s) is a separate discussion

As part of the agreement above, it can be highlighted that RAN1 did not make a decision on which type of gaps or measurement restrictions the RAN1 enhancements are related to. It is up to RAN4 to make a decision on that.

RAN1 left it up to RAN4 the discussion on which types of gaps/restrictions can be skipped.

Furthermore, RAN1 is discussing how the measurement skipping can be indicated to the UE. The agreement below shows the alternatives that are currently under discussion in RAN1:

For solutions based on triggering/enabling by network signaling to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements consider the following alternatives or combinations for further down-selection: - Alt. 1: Dynamic indication to enable Tx/Rx in particular gap(s)/restriction(s) that are caused by RRM measurements. - FFS: Alt 1-1: Explicit indication by DCI to skip a particular gap(s)/restriction(s); - Indication is included as part of scheduling DCI: - FFS: Bit-field size is one bit; - FFS: Bit-field size is >1 bit; - Note: Minimum time offset(s) between the end of [the first] received dynamic indication and start of corresponding gap(s)/restriction(s) occasion that is going to be skipped shall be introduced. - FFS: Alt 1-2: Explicit indication by DCI to indicate a time window where to skip a particular gap(s)/restriction(s); - Note: Minimum time offset between the end of received dynamic indication and start of gap(s)/restriction(s) occasion in time window that is going to be skipped shall be introduced. - FFS: Alt 1-3: Implicit indication by DCI scheduling a transmission/reception overlapping with a gap(s)/restriction(s) to skip the gap(s)/restriction(s); - Note: Minimum time offset between the end of received dynamic indication and start of gap(s)/restriction(s) occasion that is going to be skipped shall be introduced. o FFS: DCI format, DCI content, DCI bit-field size; o FFS: Whether indication is for one or more occasions; o FFS: How to consider time offset between the end of received dynamic indication and start of gap(s)/restriction(s) occasion that is going to be skipped. - Alt. 3: Semi-static solution to enable TX/RX in gaps/restrictions that are caused by RRM measurements. - FFS: Alt 3-1: Configure a pattern(s) via RRC to indicate occasions where to skip gaps/restrictions; - FFS: Details of pattern: - FFS: Pattern is based on periodicity, offset and duration; - FFS: Pattern is based on a bitmap; - FFS: whether a pattern is applied to all or subset of configured MG configurations/scheduling restrictions. - FFS: Alt 3-3: Gaps/restrictions that are caused by RRM measurements are skipped if collided with particular semi-statically pre-configured Tx/Rx occasions. - FFS: Alt. 3-4: Gaps/restrictions that are caused by RRM measurements are skipped based on semi-statically configured priority information for particular semi-statically pre-configured Tx/Rx and/or particular gaps/restrictions.

In the agreement above the 2 main alternatives relate to DCI-based or RRC-based signalling. In the DCI-based signalling, the decision of skipping is more dynamic, and can be more easily adapted to variation of the traffic, since the network has more flexibility on the decision of skipping a measurement occasion or not based on the data to be scheduled. On the other hand, the RRC based solution is more static, since previous information on the XR traffic is needed in order to optimize the skipping pattern.

RAN1 is discussing measurement occasion skipping based on dynamic DCI-based indication or semi-static indication based on RRC signalling.

Discussion

Scenarios for measurement skipping

Firstly, we would like to focus the attention to which scenarios the enhancements of scheduling during gaps or scheduling restrictions would apply.

With respect to frequency ranges, RAN1 has provided simulation results only considering FR1. The enhancements proposed with measurement skipping could be translated in extension of measurement delays. Therefore we need to consider the baseline measurement delay requirements before deciding which FRs to apply the enhancements. As an example, the PSS/SSS detection minimum delay is 600 ms, and for 20 ms MGRP TPSS/SSS_sync_inter=800 ms for FR2-1. On the other hand, for FR2-2, the beam sweeping scaling factor is extended to 12 instead of 8, and there is an additional scaling factor of KFR=2 and KFR=3 for 480 kHz and 960 kHz respectively. As a result the measurement delay in FR2-2 is 1.5x, 3x and 4.5x times larger than the delay for FR2-1 for 120 kHz, 480 kHz and 960 kHz SCS respectively. Considering the long measurement delays for FR2-2 we believe it is best to avoid further extensions that would be resulting from measurement skipping.

FR2-2 measurement delay requirements are 1.5x, 3x and 4.5x longer than FR2-1 measurements for 120 kHz, 480 kHz and 960 kHz SCS.

Measurement skipping apply for FR1 and FR2-1 measurements

With respect with types of measurements, RAN4 needs to discuss and conclude on which cases to include as part of this WID. As discussed in Section 3, RAN1 has an agreement stating that RAN1 conclusions are agnostic to the types of measurement gaps for the enhancements in XR_ph3. Therefore it must be clarified which types of gaps and scheduling restrictions apply for measurement skipping enhancements.

RAN1 is defining enhancements for skipping measurements that cause scheduling restrictions which are agnostic to the types of measurement gaps or scheduling restrictions.

Legacy measurements with gaps are natural candidates for the measurement skipping enhancements. Here we refer to legacy measurement gaps as excluding pre-configured, NCSG, MUSIM, Positioning, NTN gaps. When the UE is configured with a measurement gap the latency of any data transmission that is arriving shortly before the gap will be extended by at least the measurement duration.

As for measurements without gaps, they may cause scheduling restrictions in some situations. One typical situation for scheduling restrictions of measurements without gaps is when performing measurements in FR2. In that case the UE is expected to beam sweep during L3 measurements, and the beams will in most cases not coincide with beams for data reception. For this reason, scheduling restrictions are needed for FR2 measurements without gaps.

Intra-frequency measurements always cause scheduling restrictions in FR2, around SSB symbols to measure.

For measurements without gaps, another aspect to consider is whether interruptions are needed or not. Inter-frequency measurements without gaps can typically be performed using a spare RF chain present in the UE. When this RF chain is turned on, or retuned, it may cause some internal UE interference that causes data loss in the main RF chains used for data reception/transmission. This is referred to as interruptions in RAN4 RRM requirements.

Interruptions will cause the UE to lose some data/control before and after SMTC occasions to be measured. This will typically be more problematic for XR traffic than for normal eMBB traffic. The first reason for interruptions being more damaging to XR traffic, is that losing data/DCI scheduling data will cause extra delay which can be large in relation to the total delay budget available for the UE.

Another reason why interruptions need to be avoided for XR traffic is that they may cause the UE to miss commands for measurement skipping. If DCI command containing that command is missed, that network could still think that the UE is available during the SMTC occasions, and many data transmissions during that time would be missed. That would further increase the XR data reception latency by the duration of the SMTC occasion.

Interruptions will cause scheduling problems for latency sensitive traffic.

RAN4 has defined RRM requirements for interruptions for measurements without gaps in Rel-18 NR_MG_enh. During this work item, it was defined that interruptions are not restricted to a location. That means that although interruptions are expected to happen close to a SMTC to measure, there is no requirement of how close they should be. During this discussion, some companies pointed out that it was not clear whether Rel-16 UEs supporting NeedForGaps-r16 are causing interruptions.

If measurement skipping requirements are to be defined for measurements without gaps with interruptions it is important that some method to limit the interruptions over XR traffic is used. As an example a window where interruption is avoided could be defined. This would avoid the problems described above.

Requirements for measurements without gaps with interruptions using NeedForInterruptions-r18 do not limit interruption location.

Measurements without gaps based on NeedForGaps-r16 do not have interruption requirements.

Prioritize measurement skipping for following scenarios:

  - Measurements with gaps

    - Intra-frequency

    - Inter-frequency

  - Measurements without gaps without interruptions

    - Intra-frequency for FR2

Need for UE assistance information

RAN1 sent LS to RAN4, requesting RAN4 to consider whether or not to introduce UE assistance information related to measurement occasions [4]. As part of this LS, the RAN1 agreement included FFS points related to measurement occasions, channel conditions, traffic, and UE mobility.

In relation to channel conditions, the RAN1 agreement includes an FFS point on s-MeasureConfig. The network can configure the UE with a search threshold (s-MeasureConfig, see below text from TS 38.331) for a UE in connected mode to enable reduction of the measurement effort.

s-MeasureConfig Threshold for NR SpCell RSRP measurement controlling when the UE is required to perform measurements on non-serving cells. Choice of ssb-RSRP corresponds to cell RSRP based on SS/PBCH block and choice of csi-RSRP corresponds to cell RSRP of CSI-RS.

If the network has configured the UE with the s-MeasureConfig condition for conducting RRM measurements, it is important that the network knows if this condition has been fulfilled. This is needed for the network to know if the UE perform RRM measurements (and hence is subject to corresponding scheduling restrictions), or not. Once the s-MeasureConfig condition is fulfilled for a UE, it is suggested that the UE informs the network as it would then need to take corresponding scheduling restrictions into account.

For UEs configured with search threshold (s-MeasureConfig), the UE shall inform the network when the condition is met (i.e. defined RSRP threshold is exceeded).

Similarly, for a UE that no longer has its s-MeasureConfig condition fulfilled, and as a consequence may stop conducting RRM measurements, we also suggest that the UE informs the network. This is important for the network to know as it no longer needs to take scheduling restrictions from RRM measurements into accounts (as the UE has stopped such measurements). This leads to:

For UEs configured with search threshold (s-MeasureConfig), the UE shall inform the network that it is stopping doing RRM measurements because the s-MeasureConfig condition is no longer fulfilled.

The basic principle of the solution is as follows and shown in Figure 2. The UE is first configured with the s-MeasureConfig conditions, and asked to report back to the gNB when this condition is fulfilled (e.g. SS-RSRP< s-MeasureConfig) and it starts performing RRM measurements that causes scheduling restrictions. Similarly, when the condition is no longer fulfilled, and the UE is not required to do RRM measurements on neighbouring cells, is shall also report that to the gNB. Thereby, the gNB always knows when the UE is conducting RRM measurements, and hence knows when it can freely schedule the UE, and when it needs to respect certain scheduling restrictions.

Figure 2 Illustration of case where the UE is configured to report when it starts, or stops, conducting RRM measurements as per the s-MeasureConfig condition that results in scheduling restrictions.

We note that the proposals related to UE to network reporting of s-MeasureConfig conditions, has some relations to the already defined RRC Events A1 (Serving becomes better than threshold) and A2 (Serving becomes worse than threshold). However, while A1/A2 events solely links to serving cell RSRP power levels as compared to a threshold, what we suggest in the listed Proposals links also to UE behaviour of performing RRM measurements (and is related to scheduling restrictions).

From RAN1 LS several other FFS points are included related to UE assistance information related to measurement occasions. Those include:

- FFS: UE assistance information related to measurement occasions: - FFS: The number of needed measurement gaps/SMTC with restrictions within a time period; - FFS: The maximum number or ratio of MGs/SMTC with restrictions that can be skipped within a time period; - FFS: The number of required SSBs within a time period; - FFS: The number of consecutive RRM measurements that can be skipped; - FFS: The maximum interval between two consecutively reserved gap/restriction occasions for RRM measurements; - FFS: The patterns of gap(s)/restriction(s) where skipping is feasible or acceptable;

Additionally, it is also included in the RAN1 agreement the following related to gNB behaviour:

Note: From specification point of view, there is no mandated gNB behavior in response to any of the UE assistance information.

The benefit of UAI related to measurement occasions can be potentially useful provided that measurement skipping can be performed without extension of measurement delay. As an example, if the UE indicates that X SSBs are required in  T ms, and the gNB sends skipping commands leaving at least X SSBs in the window of T ms, we would expect that this operation would not cause any extension of the measurement delay.

UAI only makes sense if UAI information is used to provide skipping implies on no extension of measurement delay and no impact on measurement accuracy.

As part of the FFS points, one many bullets relate to the number of SSBs that need to be available in a time window. The existing measurement delay and accuracy requirements are defined considering a minimum SNR. From that SNR a minimum number of samples for averaging is derived, which result on the accuracy and measurement delay. The measurement requirements also include delay regarding detection of new cells. The UE might know the SNR of cells that were previously measured, and assume a smaller number of samples is needed for those based on previous information. But to cells that are still not known it is impossible for the UE to estimate the SNR for cells that are not yet detected.

Therefore, for the UAI information related to measurement occasions, RAN4 has to provide analysis on the following questions:

- Can the UE provide information that guarantee that measurement delay and measurement accuracy is not impacted by measurement skipping?

- Are UEs capable of determining how many measurement occasions are needed for measuring beforehand for unknown cells?

- How is the XR capacity improved if the UE is reporting UAI related to measurement occasions?

- How is the measurement performance improved if the UE is reporting UAI related to measurement occasions?

Discuss the feasibility and accuracy of UE reporting UAI in relation to XR capacity;

Discuss the feasibility and accuracy of UE reporting UAI in relation to measurement performance.

Discuss the benefits of UE reporting UAI in relation to XR capacity;

Discuss the benefits of UE reporting UAI in relation to measurement performance.

When considering the feasibility of UAI related to measurement occasions, it should be ensured that measurement delay and measurement accuracy is not impacted by measurement skipping

Any proposals for UAI related to measurement occasions should be justified by clear performance benefit.

Timeline for dynamic skipping of RRM measurement

As mentioned, RAN1 is currently discussing Alt 1 (dynamic) solutions where the gNB sent command(s) to the UE to skip RRM measurements occasions for the sake of having the UE prioritize PDCCH/PDSCH decoding and/or PUCCH/PUSCH transmissions. Alt 1 relies on the gNB sending a skipping indication command with DCI on PDCCH. For this solution, the UE will naturally need time for decoding the DCI with skipping indication command, and secondly to act on the command to actually skip the RRM measurement. The current 3GPP specs already include requirements for a UE to decode a DCI in 3GPP TS 38.214. For cases where the DCI include a scheduling grant (as is also being the default assumption in RAN1 for carrying skipping indication), the current specs define processing times that include both DCI and decoding PDCSH, or PUSCH preparation. We suggest relying on these existing processing values also for a UE to decode a DCI that carries skipping indication commands, i.e.,

The UE processing time for decoding a DCI with RRM measurement skipping indication should be the same as the currently defined processing requirements in 3GPP TS 38.214 for decoding of DCI and PDSCH decoding, or PUSCH preparation.

Once the UE has realized that it has received a RRM measurement skipping indication command, it needs time to react on this. This time could be rather short as the UE in principle only need to continue normal operation (i.e. listening to DCI and receiving PDSCH / preparing PUSCH transmissions as per the received scheduling commands). Thus, it does, for instance, not involve any RF retuning or alike actions. In setting the UE requirements for acting on reception of a RRM measurement skipping indication command, we could get inspiration from alike UE behaviours of other commands carried by DCI that instruct the UE to take actions. Among others, those include a power control command received on DCI, a command for a UE to cancel an ongoing PUSCH transmission (aka as cancellation indication). Common for those cases is that the UE react rather fast and implement the action received on the DCI in less than a slot time. We therefore suggest,

RAN4 shall define processing requirements for a UE to act on a DCI with RRM measurement skipping indication. Inspired by UE requirements for e.g. acting on power control commands and uplink cancellation indication in DCI, the processing could be fraction of a slot.

Conclusion

The paper has discussed implications of XR measurement enhancements on RAN4 RRM requirements. As part of this discussion, the following Observations and Proposals were made:

Observation 1: RAN1 is discussing XR measurement enhancements considering a packet delay budget between 10  and 15 ms.

Observation 2: Measurement gaps and scheduling restrictions are large in comparison to the total packet delay budget of typical XR applications.

Observation 3: RAN1 left it up to RAN4 the discussion on which types of gaps/restrictions can be skipped.

Observation 4: RAN1 is discussing measurement occasion skipping based on dynamic DCI-based indication or semi-static indication based on RRC signalling.

Observation 5: FR2-2 measurement delay requirements are 1.5x, 3x and 4.5x longer than FR2-1 measurements for 120 kHz, 480 kHz and 960 kHz SCS.

Proposal 1: Measurement skipping apply for FR1 and FR2-1 measurements

Observation 6: RAN1 is defining enhancements for skipping measurements that cause scheduling restrictions which are agnostic to the types of measurement gaps or scheduling restrictions.

Observation 7: Intra-frequency measurements always cause scheduling restrictions in FR2, around SSB symbols to measure.

Observation 8: Interruptions will cause scheduling problems for latency sensitive traffic.

Observation 9: Requirements for measurements without gaps with interruptions using NeedForInterruptions-r18 do not limit interruption location.

Observation 10: Measurements without gaps based on NeedForGaps-r16 do not have interruption requirements.

Proposal 2: Prioritize measurement skipping for following scenarios:

a.	Measurements with gaps

i.	Intra-frequency

ii.	Inter-frequency

b.	Measurements without gaps without interruptions

i.	Intra-frequency for FR2

Proposal 3: For UEs configured with search threshold (s-MeasureConfig), the UE shall inform the network when the condition is met (i.e. defined RSRP threshold is exceeded).

Proposal 4: For UEs configured with search threshold (s-MeasureConfig), the UE shall inform the network that it is stopping doing RRM measurements because the s-MeasureConfig condition is no longer fulfilled.

Observation 11: UAI only makes sense if UAI information is used to provide skipping implies on no extension of measurement delay and no impact on measurement accuracy.

Observation 12: Discuss the feasibility and accuracy of UE reporting UAI in relation to XR capacity;

Observation 13: Discuss the feasibility and accuracy of UE reporting UAI in relation to measurement performance.

Observation 14: Discuss the benefits of UE reporting UAI in relation to XR capacity;

Observation 15: Discuss the benefits of UE reporting UAI in relation to measurement performance.

Proposal 5: When considering the feasibility of UAI related to measurement occasions, it should be ensured that measurement delay and measurement accuracy is not impacted by measurement skipping

Proposal 6: Any proposals for UAI related to measurement occasions should be justified by clear performance benefit.

Proposal 7: The UE processing time for decoding a DCI with RRM measurement skipping indication should be the same as the currently defined processing requirements in 3GPP TS 38.214 for decoding of DCI and PDSCH decoding, or PUSCH preparation.

Proposal 8: RAN4 shall define processing requirements for a UE to act on a DCI with RRM measurement skipping indication. Inspired by UE requirements for e.g. acting on power control commands and uplink cancellation indication in DCI, the processing could be fraction of a slot.

- References

- RP-240791, Revised WID on XR (eXtended Reality) for NR Phase 3, Nokia

- 3GPP TS 38.331, NR; Radio Resource Control (RRC) protocol specification

- 3GPP TR 38.838, Study on XR (Extended Reality) evaluations for NR

- R1-2405736, LS on UE assistance information