3GPP TSG-RAN WG4 Meeting#112							         			R4-2413315

Maastricht, Netherland, 19 – 23 August 2024

Title: 	On NS_27 applicability to CA_n48-n77 ULCA

Source: 	Skyworks Solutions, Inc.

Agenda Item:	7.3.3	UE RF requirements for NR inter-band CA/DC configurations including inter band CA for 2 DL with up to 2UL (NR_CADC_R19_2BDL_xBUL)	[NR_CADC_SUL_R19-Core]

Document for:	Approval

# 1	Background

In RAN#104, a new NR CA basket was introduced which include 2Band DL/2Band UL CA combination. There is a request to support CA_n48-n77 in UL. In this contribution, given that those two bands are overlapping, we discuss what would be the consequences if NS_27 applied.

# Discussion

Figure 1 provides an overview of the overlapping spectrum between n48 and n77 in the U.S. together with NS_27 emission requirements from Table 1. IMD3 of CA_n48-n77 are also represented for two cases where the n77 carrier is in the lower or upper part of the n77 band.

Figure 1: n48 and n77 spectrum in the US, NS_27 emission limit and ULCA_n48-n77 IMD3

Table 1: Additional requirements for "NS_27"

| Frequency range (MHz) | Channel bandwidth (MHz) / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 5, 10, 15, 20, 30, 40 |  |
| 9 kHz – 3530 MHz | -40 | 1 MHz |
| 3530 MHz – 3540 MHz | -25 |  |
| 3710 MHz – 3720 MHz | -25 |  |
| 3720 MHz – 12.75 GHz | -40 |  |

Observation: There are a number of cases where the IMD3 and higher orders IMDs of an n48 UL carrier and a lower or higher part n77 UL carrier fall in the stringent -40dBm/MHz NS_27 requirement. Since the bands are supported with the same paths, even when using a separate PA for the Band n48 UL and the Band n77 UL, if the NS_27 requirement applies, a similar backoff as for n48 intra-band ULCA of 20dB would be needed, making the CA_n48-n77 UL configuration of less interest than a single UL in band n48 or n77.

Proposal:

- It should be clarified whether NS_27 emission requirements apply to CA_n48-n77

- In general, it should be clarified whether the OOB -40dBm/MHz NS_27 emission requirement still applies once n77 is deployed on each side of band n48.

# Conclusion

In this contribution, we discuss the n40 and n77 band spectrum in the U.S. together with the NS_27 emission requirement and formulate the following request for clarification.

Proposal:

- It should be clarified whether NS_27 emission requirements apply to CA_n48-n77

- In general, it should be clarified whether the OOB -40dBm/MHz NS_27 emission requirement still applies once n77 is deployed on each side of band n48.

# References