3GPP TSG-RAN WG4 Meeting #112	R4-2413319

Maastricht, Netherlands, 19th – 23rd August, 2024

Title:	(NR_RF_FR1_enh-Core) DL interruptions for 2Tx vs 1Tx switching

Source:	Qualcomm Incorporated

Agenda item:	4.2

Work Item: 	NR_RF_FR1_enh-Core, DL_intrpt_combos_TxSW_R17-Core

Document for:	Approval

# 1.	Introduction

Mandating DL interruptions has been made under a basket WI after the primary WI from Rel-17. Rel-16 introduced 1Tx-2Tx switching and Rel-17 introduced 2Tx-2Tx switching and DL interruption mandating was discussed for Rel-17 feature during Rel-18.

# 2. 	Discussion

Depending on antennas architectures, for the same device, 2Tx-2Tx may need an interruption but 1Tx-2Tx may not. The different need for DL interruptions was not discussed for Rel-17 feature [2] and in clause 5.2 tables there is no distinction between different switching cases and the capability uplinkTxSwitching-DL-Interruption-r16 that is part of ULTxSwitchingBandPair-v1700 does not distinguish if the interruption is related to the uplinkTxSwitchingPeriod-r16 or uplinkTxSwitchingPeriod2T2T-r17.

Figure 1. Antenna assignments for 1Tx-2Tx and 2Tx-2Tx switch cases.

In Figure 1, 2Tx-2Tx switch requires reassignment of RX antennas (main antennas and Rx3) and therefore performing a switch creates an interruption but 2Tx-1Tx may not need it.

Proposal:  Consider “no DL interruption” mandate for earlier releases to be dependent on the switching case

This may need some work on the capabilities side.

# Conclusion

We discussed the need for DL interruption and its relation to the switching case and made one proposal:

Proposal:  Consider “no DL interruption” mandate for earlier releases to be dependent on the switching case

# REFERENCE

[1]	R4-2110071, “CR to 38.101-1 Introduce DL interruption clarification for CA conduting Tx Switching”, China Telecom, 3GPP TSG-RAN WG4 Meeting #99-e, Electronic Meeting, May. 19-27, 2021

[2]	R4-2103236, “Switching time mask for 2Tx-2Tx switching between two carriers and 1Tx-2Tx/2Tx-2Tx switching between two bands in Rel-17”, China Telecom, 3GPP TSG-WG RAN4 Meeting #98e, Electronic Meeting, 25 Jan - 5 Feb, 2021