3GPP TSG-RAN4 Meeting #112	R4-2413321

# Maastricht, Netherlands, 19th – 23rd August, 2024

Title:	Energy harvesting considerations

Source:	Qualcomm Incorporated

Agenda item:	8.20.3.2

Release:	Rel-18

SI: 	FS_Ambient_IoT_solutions

Lead WG: 	RAN1

Document for:	Approval

# 1.	Introduction

Energy harvesting aspect has been discussed in previous plenaries and waveform design has been precluded from the SI scope but the impact of energy harvesting can be discussed and studied. In this paper we discuss where energy harvesting has impact.

# 2. 	Discussion

## 2.1	Principle of device behaviour under Energy harvesting

RAN1 is discussing states for the devices and device is assumed to have at least ON and OFF states and maybe also SLEEP. Device goes from OFF state to ON state when it detects energy and has been able to accumulate sufficient amount of energy for the parts to function. We can use a tentative value of [-35] dBm for the energy harvesting sensitivity. In previous paper [8] we discussed the importance of clear reference plane. It is our understanding that RAN4 task will be to define the energy harvesting sensitivity with proper accuracy and needed reference plane definitions as we discussed in [8]. The value may not be needed during the study item for the detailed value but some aspects how this new type of parameter should be studied.

This Energy Harvesting Sensitivity (EHS) is the value when device starts to harvest energy. As such it may not be useful alone but we will discuss more details on that in the next section. Device behaviour principle with non-constant energy availability is shown in Figure 1.

Figure 1. Energy harvesting principle

Energizing signal is therefore trigger for device to turn on. This is specifically for device type 1. After device turns on, it needs to monitor the WUS and then after successfully receiving signal to transmit, it needs to transmit back what was requested. Device may also decide to sleep or continue monitoring. RAN1 is discussing details the device states behaviour [9] and once there is sufficient information about the expected device behaviour and device states from their studies RAN4 can extract minimum performance requirements for the device. Expectation is that there will be some minimum device operation pattern such as one shown in Figure 2 frow which RAN4 can deduct device ON-OFF dutycycle.

Figure 2. Energy harvesting frame structure

In simple form, this translates to some ON-OFF dutycycle. We will discuss energy harvesting parameters in more detail in next section.

## 2.2	Energy harvesting parameters

When incoming RF signal level exceeds EHS, device starts harvesting energy. Energy harvesting in the device is not perfect but happens with some efficiency < 100% and the efficiency is proportional to the incoming signal level. The accumulated energy can be calculated as a function of incoming signal strength, time and efficiency.

Figure 3. Accumulated energy

Simplifying the frame in Figure 2 so that the ON time is 2 msec and considering that device power consumption is always 1uW when ON the energy consumed is 5 nWs. Time to harvest 5 nWs is 21 msec with -29 dBm received signal strength and 18% harvester efficiency.

This is the method RAN4 can define EHS. EHS currently in the discussion is the level when device starts harvesting energy but it does not define how much energy device can harvest. That alone should not be defined but the energy harvesting capability, energy storage size and device energy consumption need to be bundled together and in order to define a testable metric, a energy harvesting signal power level should be defined with the minimum time device is ready to receive and transmit after some power level signal is provided to it.

While waveform design for energy harvesting is not in the study item scope, there should be some understanding on device capability for energy harvesting so that the device functionality in a system can be guaranteed. If energy harvesting capability is not standardised, the inventory process completion can not be guaranteed since some devices may need much more time to accumulate the energy they need.

Proposal 1: RAN4 shall study device energy harvesting capability

Proposal 2: RAN4 shall study how device energy harvesting capability can be included in the requirements

Proposal 2 can mean for example definition of turn on time with a specified energy harvesting signal.

We have understood that some companies are not in favour to define device energy harvesting but prefer to leave it to the implementation. It is not clear how can a device then full fill system requirements if it is not known if when the device will turn on after inventory process begins, or if it will turn on at all. At least minimum, there should be an understating, maybe even a requirement on devices turn on time after energizing signal has been provided to it. This requires a definition of energizing signal.

Proposal 3: RAN4 should study how the availability of energy harvesting signal impacts device behavior

# Conclusion

We discussed device energy harvesting and made three proposals:

Proposal 1: RAN4 shall study device energy harvesting capability

Proposal 2: RAN4 shall study how device energy harvesting capability can be included in the requirements

Proposal 3: RAN4 should study how the availability of energy harvesting signal impacts device behavior

# Reference

[1]	RP-240826, “Study on solutions for Ambient IoT (Internet of Things) in NR”, 3GPP TSG RAN Meeting #103, Maastricht, Netherlands, March 18-21, 2024

[2]	R1-2403195, “Ambient IoT Device Architecture”, Qualcomm Incorporated, 3GPP TSG RAN WG1 #116-bis, Changsha, Hunan Province, China, April 15th – 19th, 2024, https://www.3gpp.org/ftp/tsg_ran/WG1_RL1/TSGR1_116b/Docs/R1-2403195.zip

[3] 	R1-2403194, “Evaluation Assumptions and Results”, Qualcomm Incorporated, 3GPP TSG RAN WG1 #116-bis, Changsha, Hunan Province, China, April 15th – 19th, 2024, https://www.3gpp.org/ftp/tsg_ran/WG1_RL1/TSGR1_116b/Docs/R1-2403194.zip

[4]	A Low-Power Backscatter Modulation System Communicating Across Tens of Meters With Standards-Compliant Wi-Fi Transceivers

[5]	ETSI EN 302 208, “Radio Frequency Identification Equipment operating in the band 865 MHz to 868 MHz with power levels up to 2 W and in the band 915 MHz to 921 MHz with power levels up to 4 W; Harmonised Standard for access to radio spectrum”, ETSI

[6]	R4-2406714, “WF on Ambient IoT in NR”, 3GPP TSG-RAN WG4 Meeting #110bis, Changsha, China, 15th – 19th April, 2024

[7]	R4-2410597, “WF on impacts of A-IoT on RF requirements​”, Huawei, 3GPP TSG-RAN WG4 Meeting # 111, Fukuoka, Japan, May 20 – May 24, 2024

[8]	R4-2407588, “A-IoT device study and RF requirements aspects”, Qualcomm, 3GPP TSG-RAN4 Meeting #111, Fukuoka City, Fukuoka , Japan, 20th – 24th May, 2024

[9]	R1-2407032, “Ambient IoT Device Architecture”, Qualcomm Incorporated, 3GPP TSG RAN WG1 #118, Maastricht, NL, August 19th – 23rd, 2024