3GPP TSG-RAN4 Meeting #112	R4-2413322

# Maastricht, Netherlands, 19th – 23rd August, 2024

Title:	CW cancellation capability

Source:	Qualcomm Incorporated

Agenda item:	8.20.3.3

Release:	Rel-18

SI: 	FS_Ambient_IoT_solutions

Lead WG: 	RAN1

Document for:	Discussion

# 1.	Introduction

CW cancellation is one of the topics discussed in the co-ex simulation work. In this paper we discuss practical challenges with the CW cancellation.

# 2. 	Discussion

CW cancellation is a key parameter in achieving reader coverage. If CW in the reader is very high, the receiver non-linearity will cause received SNR degradation. There was no agreement how to handle the CW cancellation capability in the co-ex simulations [10, issue 2-4-2]. There are multiple design aspects to consider for the self interference mitigation for CW and different techniques are available for the gNB and the intermediate node.

For the intermediate node, the LNA and ADC linearity is defined by the expected ACS performance. RAN4 specification defines ACS as 33 dB for most stringent case at the moment. In order for the CW analogue part cancellation to operate, the CW level compared to the received signal level (D2R) muct be cancelled so that the CW to signal ratio is within receiver (LNA+ADC) lineary. Otherwise D2R signal will be distorted due to saturation and is not detectable anymore. Once CW is cancelled to the level where it does not saturate the receiver anymore, the DC compensation network can further remove the remaining CW even if it is much above the wanted signal.

## 2.1	TX-RX Antenna isolation in gNB

gNB uses different antenna panels for TX and RX and the panels are highly directive thus isolation between TX and and RX antennas is big, in the order of > 50 dB. Therefore the problem is not as severe in gNB. In addition gNB can rely on high precision DSP with large amount of bits to cancel the CW in the receiver and take advantage of larger physical distances between CW TX circuits and receivers, maybe even locate them in different enclosures.

Figure 1. gNB antenna arrangement

## 2.2	CW canceller circuitry

In principle, the opposite phased CW can be injected to the receiver and then two signal will cancel each other out. The problem is how to know the phase of the CW in the receiver and how to adjust the cancelling signal perfectly. This is especially difficult in the intermediate node since it has smaller dimensions and shared antennas. The circuitry and relevant aspects are shown in Figure 2.

Figure 2. CW cancellation in an Intermediate node

From Figure 2 we can see that CW has many leakage paths and conclude that the cancellation circuitry has to cancel very high-level CW compared to the backscattered signal level.

RAN4 has not discussed techniques for active cancellation but it should be noted that the processing accuracy in a handset receiver typically results in the order of 1.5-4 % Rx EVM [12]. This is the level Intermediate node baseband can provide information to the phase and amplitude cancellation circuits for the analogue cancellation. In the LS [12], the typical UE Rx EVM is 2-4 % for a UE that supports 1024QAM. UE fulfilling minimum requirements and not support 1024QAM would have even smaller minimum implied assumption for Rx EVM. The analogue domain phase and amplitude controller also have finite resolution and 1% accuracy would mean 1.8 degrees phase step assuming signal can be inverted separately. This level of granularity maybe challenging in practice.

Observation 1: Intermediate node CW cancellation circuitry processing accuracy is in the order of 1.5-4 %.

Translating this to CW cancellation in dB, we need to split this in to phase accuracy and amplitude accuracy. With a simple mathematical analysis, if we assume that waveform with an amplitude of Y is cancelled with:

Cancellation waveform = Y*(1-amplitude_error)*sin(wt+phase_error)

And calculate the resulting CW as

Z= Y - Cancellation waveform,

We can get cancellation value of Z compared to Y as in Figure 3 below. The EVM % is calculated from the amplitude_error and phase_error.

Figure 3. Cancellation as a function of EVM in the cancellation circuitry.

The curve in Figure 3 is not explicitly defined since the contribution of phase error to the EVM % is not same as its contribution to the cancellation. In other words, cancellation is more sensitive to the amplitude error than it is for phase error. In Figure 4 we show the dependency as surface plot for reference.

Figure 4. Cancellation sensitivity as a function of amplitude and phase error

We can see that the ampltude error and phase have different impact to cancellation.

Observation 2: Intermediate node realistic analogue domain CW cancellation capability is at most 50 dB and more likely typical value is closer to less than 40 dB.

Coming back to the incoming signal characteristics, this means that the UE can cope with the CW of the level:

CW_to_signal= Analogue CW cancellation + Digital domain cancellation.

The digital domain cancellation is same process as DC offset compensation. Normally, the DC offset to be compensated is small since it is a result of non-perfections in the circuitry. In this case, the CW is much larger than the receiver modulated signal so the DC offset cancellation circuitry needs some tuning to work in this new situation. We can assume here that the digital domain cancellation can cope at least with the CW to signal ratio where LNA remain linear enough not to distort the signal.

Observation 3: Intermediate node feasible CW cancellation capability is approximately 70-80 dB.

With the observation 3, the range of the intermediate node reader should be further analysed to undersytand if it is still feasible for the system.

Proposal: RAN4 to study methods and feasable values for CW cancellation in Intermediate node.

# Conclusion

We discussed CW cancellation and observed that

Observation 1: Intermediate node CW cancellation circuitry processing accuracy is in the order of 1.5-4 %.

Observation 2: Intermediate node realistic analogue domain CW cancellation capability is at most 50 dB and more likely typical value is closer to less than 40 dB.

Observation 3: Intermediate node feasible CW cancellation capability is approximately 70-80 dB.

And made one proposal:

Proposal: RAN4 to study methods and feasable values for CW cancellation in Intermediate node.

# Reference

[1]	RP-240826, “Study on solutions for Ambient IoT (Internet of Things) in NR”, 3GPP TSG RAN Meeting #103, Maastricht, Netherlands, March 18-21, 2024

[2]	R1-2403195, “Ambient IoT Device Architecture”, Qualcomm Incorporated, 3GPP TSG RAN WG1 #116-bis, Changsha, Hunan Province, China, April 15th – 19th, 2024, https://www.3gpp.org/ftp/tsg_ran/WG1_RL1/TSGR1_116b/Docs/R1-2403195.zip

[3] 	R1-2403194, “Evaluation Assumptions and Results”, Qualcomm Incorporated, 3GPP TSG RAN WG1 #116-bis, Changsha, Hunan Province, China, April 15th – 19th, 2024, https://www.3gpp.org/ftp/tsg_ran/WG1_RL1/TSGR1_116b/Docs/R1-2403194.zip

[4]	A Low-Power Backscatter Modulation System Communicating Across Tens of Meters With Standards-Compliant Wi-Fi Transceivers

[5]	ETSI EN 302 208, “Radio Frequency Identification Equipment operating in the band 865 MHz to 868 MHz with power levels up to 2 W and in the band 915 MHz to 921 MHz with power levels up to 4 W; Harmonised Standard for access to radio spectrum”, ETSI

[6]	R4-2406714, “WF on Ambient IoT in NR”, 3GPP TSG-RAN WG4 Meeting #110bis, Changsha, China, 15th – 19th April, 2024

[7]	R4-2410597, “WF on impacts of A-IoT on RF requirements​”, Huawei, 3GPP TSG-RAN WG4 Meeting # 111, Fukuoka, Japan, May 20 – May 24, 2024

[8]	R4-2407588, “A-IoT device study and RF requirements aspects”, Qualcomm, 3GPP TSG-RAN4 Meeting #111, Fukuoka City, Fukuoka, Japan, 20th – 24th May, 2024

[10]	R4-2410567, “WF on co-existence study for ambient IoT and NR/LTE”, CMCC, 3GPP TSG-RAN4 Meeting #111, Fukuoka City, Fukuoka, Japan, 20th – 24th May, 2024

[11]	R4-2410597, “WF on impacts of A-IoT on RF requirements”, Huawei, 3GPP TSG-RAN4 Meeting #111, Fukuoka City, Fukuoka, Japan, 20th – 24th May, 2024

[12]	R4-1706112, “Reply LS regarding 1024QAM”, RAN1(Huawei), 3GPP TSG-RAN WG4 Meeting #83, Hangzhou, China, 15 - 19 May, 2017