3GPP TSG-RAN WG4 Meeting #112	R4-2413324

Maastricht, Netherlands, August 19 – August 23, 2024

Agenda Item:	8.22.4.1

Source:	MediaTek Inc.

Title:	Simulation assumptions for R19 LP-WUS

Document for:	Discussion

# Introduction

In RAN4#111 meeting, it was agreed to bring forth simulation assumptions for LP-WUS/WUR in FR1, using the recommended framework provided in the appendix of WF [1].

Issue 1-4-5: Simulation assumptions Agreement: First start the simulation for FR1, and FFS for FR2 pending on the RF progress. Recommendations: Companies are encouraged to provide simulation assumptions based on framework in the appendix.

In this contribution, the simulation assumption for RRM performance evaluation for LP-WUS/WUR UE is provided including the general parameters and cell specific parameters. Given the limited coverage of LP-WUR UE, i.e., close to the cell center, the impact of inter-cell interference on the RRM measurement performance is minor/negligible. Therefore, neighbor cell parameters are not significant for simulation assumptions and can be removed.

# Simulation assumptions

Table 1: General parameters

| Simulation parameters | Comments/values |
| --- | --- |
| Carrier frequency | FR1: 2.6 GHz |
| Channel bandwidth | 20 MHz |
| DRX | No |
| BS transmit antennas for LP-SS blocks | 1 tx or single layer transmissions |
| UE receive antennas | 1 rx |
| Data and control channel subcarrier spacing | OFDM based: The same as SS block subcarrier spacing OOK based: the same as one of the SCS(s) used for other NR transmissions in the same CP-OFDM symbol |
| Measurement period (in number of measurement samples) | OOK based/OFDM based: [2] |
| Subcarrier spacing | 15 kHz and 30 kHz |
| LP-SS periodicity | 320 ms |
| LP-SS block BW | 132 subcarriers for SCS=30kHz; 264 subcarriers for SCS=15kHz |
| Actual LP-SS block transmissions | always transmitted |

Table 2: Cell-specific parameters

| Parameter | Unit | Cell 1 |
| --- | --- | --- |
| RF Channel number | - | Channel 1 |
| PBCH and DMRS power offset with respect to NR-PSS, NR-SSS and LP-SS | dB | 0 |
| Data and control PSD relative to NR-PSS,NR-SSS and LP-SS | dB | 0 |
| RB Utilization | % | 100 |
| LP-SS waveform |  | OOK-1; OOK-4 with M value based on RAN1 conclusion |
| LP-SS duration (Number of OFDM symbols) |  | Based on RAN1 conclusion |
| LP-SS pattern |  | Binary sequence based on RAN1 conclusion |
| Slot length | - | 14 symbols |
| CP Length | - | Normal |
| SNR | dB | OFDM based: -6 dB OOK based: -6dB |
| Channel |  | TDL-C 300ns |
| UE speed |  | 30km/h |

At least the following performance characteristics are to be provided for LP-SS RSRP:

- RSRP error CDF for serving cell.

- At least 5%, 50%, and 95%-ile of the RSRP errors

In the above,

- RSRP error (dB) = estimated RSRP – ideal RSRP (based on perfect channel knowledge).

# References

- R4-2410296, WF for RRM core requirements for LP-WUS/WUR, vivo.