3GPP TSG-RAN WG4 Meeting #112	R4-2413325

Maastricht, Netherlands, August 19 – August 23, 2024

Agenda Item:	8.22.4.2

Source:	MediaTek Inc.

Title:	Discussion on the RRM core requirements for LP-WUS

Document for:	Discussion

# Introduction

In RAN4#111 meeting, RAN4 discussed the general aspects in LP-WUS/WUR RRM core requirements based on the approved objectives in the WID as captured below [1]. In this paper, we provide our view on the related RRM issues discussed in the WF [2].

The objectives of the work item are the following: - To specify an LP-WUS design commonly applicable to both IDLE/INACTIVE and CONNECTED modes (RAN1, RAN4) - Specify OOK (OOK-1 and/or OOK-4) based LP-WUS with overlaid OFDM sequence(s) over OOK symbol - The LP-WUS design shall ensure that for IDLE/INACTIVE operation, the same information is delivered irrespective of LP-WUR type. The OFDM sequence can carry information. - At least duty-cycled monitoring of LP-WUS is supported - For IDLE/INACTIVE modes - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4) - Specify LP-SS with periodicity with Yms for LP-WUR, for synchronization and/or RRM for serving cell. (RAN1, RAN4) - LP-SS is based on OOK-1 and/or OOK-4 waveform with or without overlaid OFDM sequences. Further down selection between with and without overlaid OFDM sequences is to be done within WI. - Note: For LP-WUR that can receive existing PSS/SSS, existing PSS/SSS can be used for synchronization and RRM instead of LP-SS. - Y will be decided within WI. 320ms is the start point. - Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions (RAN4, RAN2) - For CONNECTED mode, specify procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring (RAN2, RAN1) - Check in RAN#105 for potential TU adjustment in RAN2 - Note: In CONNECTED mode, UE MR ultra-deep sleep is not considered, and UE RRM/RLM/BFD/CSI measurements are performed by MR - Note: The target coverage of LP-WUS and LP-SS shall be the coverage of PUSCH for message3. - Note: The optimization of LP-WUS signal design for idle/inactive mode is prioritized over the optimization for connected mode. - Specify the necessary RAN4 core requirement(s) to support the feature (RAN4). - Specify UE low-power wake-up receiver requirements, at least REFSENS, ACS and ASCS requirements with consideration of possible new methodology to assess the low-power wake-up receiver performance - Define guard RBs for ACS and ASCS cases - Study testability of above requirements - Consider impacts of different architecture and impairments, and set requirements that enable all types of reasonable implementation - Study and if necessary specify or support by declaration, the corresponding BS requirements, e.g., dynamic range for LP-WUS/LP-SS. - Current NR BS requirements is baseline - Specify necessary RRM requirements

# Discussion

In the last meeting, RAN4 discussed the possible cases/states of LP-WUR UE that can be considered for RRM relaxation. In our view, there are three main cases:

- Case A (RRM offloading): when MR is deactivated and LR is activated.

- Case B (RRM relaxation): when both MR and LR are activated.

- Case C (Legacy UE): when MR is activated and LR is deactivated.

In Case A, RRM measurements of serving cell are exclusively performed by LR, based on LP-SS or PSS/SSS (depending on LR implementation). In Case B, MR RRM measurements are relaxed for serving cell, neighbour cell or both. In Case C, UE behaviour is same as legacy.

In the last meeting, RAN4 agreed to discuss the requirements for fully offloading case (i.e., Case A), while RRM relaxation case (i.e., Case B) is still FFS as captured below in the WF [2].

Issue 1-1-2: Cases/states to be considered for RRM relaxation Agreement: Discuss the RAN4 requirements first for the following case #1, and FFS for case #2 to #5. RRM measurement case index; MR serving cell measurement; MR neighboring cell measurement; LR measurement. #1 Fully offloading case; Off; Off: FFS the condition and the details; ON. RAN4 to further discuss case #2 to #4: RRM measurement case index; MR serving cell measurement; MR neighboring cell measurement; LR measurement. #2 Relaxed case a; On with relaxation measurement; Off; ON. #3 Relaxed case b; On with relaxation measurement; On with relaxation measurement; ON. #4 Relaxed case c; Off; On, FFS the condition and the details; ON.

For the fully offloading case, MR is Off and LR is ON, LR should at least perform wake-up signal monitoring and serving cell measurements in IDLE/INACTIVE mode to determine whether the UE remains camped on a suitable cell.

Proposal 1: For fully offloading case, MR is Off and LR is ON, LR should at least perform wake-up signal monitoring and serving cell measurements in IDLE/INACTIVE mode.

For the RRM relaxation case (i.e., Case B), there are several sub-cases to consider:

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #2 Relaxed case a | On with relaxation measurement | Off | ON |
| #3 Relaxed case b | On with relaxation measurement | On with relaxation measurement | ON |
| #4 Relaxed case c | Off | On with relaxation measurement | ON |

The RRM relaxation cases above can achieve different power saving gain. However, as a starting point RAN4 can consider RRM relaxation for intra-frequency measurements in IDLE/INACTIVE mode since both serving and neighbour cells can be measured in the same frequency layer.

Proposal 2: As a starting point for RRM relaxation case (MR is relaxed and LR is ON), RAN4 can consider RRM relaxation for intra-frequency measurements in IDLE/INACTIVE mode where both serving and neighbour cells can be measured in the same frequency layer.

The MR RRM relaxation can be achieved by using a scaling factor for the RRM measurements performed in IDLE/INACTIVE mode. It is noted that different scaling factors can offers different power saving gain for the UE. RAN4 can discuss the following options for RRM relaxation:

  - Option 1: MR RRM Relaxation from 8 to 16 times

  - MR RRM Relaxation >=16 times

In our view, RRM relaxation is not necessary to be based on a single scaling factor. In other words, multiple stages of RRM relaxation with different scaling factors can be considered depending on several conditions (e.g., channel condition, UE mobility, UE location)

Proposal 3: RAN4 to discuss the following options for RRM relaxation:

- Option 1: MR RRM Relaxation from 8 to 16 times

- Option 2: MR RRM Relaxation >=16 times

Proposal 4: RAN4 to study the consideration of single stage of RRM relaxation or multiple stages of RRM relaxation (e.g., with different scaling factors) depending on conditions (e.g., channel condition, UE mobility, UE location)

Regarding the target SNR issue for LP-WUS and LP-SS evaluation, RAN1 has agreed on three values for target SNR in the last meeting as captured below:

Agreement For evaluation of LP-WUS and LP-SS design to achieve coverage of PUSCH for message3 from RAN1 perspective, at least the following SNR values should be considered: - SNR=-3dB for NF of LR = NF of MR+ 8dB - SNR= -0.5dB for NF of LR = NF of MR+ 5dB - SNR=2dB for NF of LR = NF of MR+ 2dB - Note 1: The NF of MR is assumed as 7dB

However, RAN4 can consider the worst case scenario from RAN1 agreement (i.e., SNR=-3dB for NF of LR = NF of MR+ 8dB) to define the final side condition for RRM measurement. In our view, for Idle/Inactive mode, it is important to ensure SNR no less than -3dB for LP-WUS monitoring in order not to miss any paging. For the case determination of LP-WUS monitoring is based on LP-SS measurement by LR, 2 to 3 dB measurement inaccuracy need to be considered as a margin to define the final side condition.

Proposal 5: For Idle/Inactive mode, it is important to ensure SNR no less than -3dB for LP-WUS monitoring in order not to miss any paging. For the case determination of LP-WUS monitoring is based on LP-SS measurement by LR, 2 to 3 dB measurement inaccuracy need to be considered as a margin to define the final side condition. Therefore, RAN4 to define -6dB as the final side condition.

# Summary

In this contribution, the following proposal is concluded:

Proposal 1: For fully offloading case, MR is Off and LR is ON, LR should at least perform wake-up signal monitoring and serving cell measurements in IDLE/INACTIVE mode.

Proposal 2: As a starting point for RRM relaxation case (MR is relaxed and LR is ON), RAN4 can consider RRM relaxation for intra-frequency measurements in IDLE/INACTIVE mode where both serving and neighbour cells can be measured in the same frequency layer.

Proposal 3: RAN4 to discuss the following options for RRM relaxation:

- Option 1: MR RRM Relaxation from 8 to 16 times

- Option 2: MR RRM Relaxation >=16 times

Proposal 4: RAN4 to study the consideration of single stage of RRM relaxation or multiple stages of RRM relaxation (e.g., with different scaling factors) depending on conditions (e.g., channel condition, UE mobility, UE location).

Proposal 5: For Idle/Inactive mode, it is important to ensure SNR no less than -3dB for LP-WUS monitoring in order not to miss any paging. For the case determination of LP-WUS monitoring is based on LP-SS measurement by LR, 2 to 3 dB measurement inaccuracy need to be considered as a margin to define the final side condition. Therefore, RAN4 to define -6dB as the final side condition.

# References

- RP-240801, Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR), vivo, NTT DOCOMO, Ericsson, MediaTek, Samsung, Sony.

- R4-2410296, WF for RRM core requirements for LP-WUS/WUR, vivo.

- 3GPP TR 38.869, “Study on low-power wake-up signal and receiver for NR (Release 18)”, V18.0.0.