3GPP TSG-RAN WG4 Meeting #112	R4-2413326

Maastricht, Netherlands, August 19 – August 23, 2024

Agenda Item:	8.15.2.1

Source:	MediaTek Inc.

Title:	Discussion on FR2-1 SSB based L3 measurement delay reduction by optimizing Rx beam sweeping factor

Document for:	Discussion

# Introduction

The objective for R19 NR RRM Phase-5 work item was agreed in the WID as captured below [1]. In this paper, we continue the discussion on the first objective, particularly on optimizing Rx beam sweeping factor for L3 measurement delay reduction when UE support multiple-Rx simultaneous reception.

- FR2-1 SSB based L3 measurement delay reduction for connected mode - For UE supporting multiple-Rx simultaneous reception on single carrier: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - Rx beam sweeping factor - For UE not in multiple-Rx simultaneous reception mode: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - CSSF outside gap in CA/DC scenarios - Baseline assumption on number of searchers is 2 - Fast SCell activation for UE supporting Rel-18 EMR - Study and, if feasible, to reduce the SCell activation delay with valid EMR reporting - Apply fast scell activation in FR1 and FR2-1 - Note: RAN4 to start this work from Q3’2024 and aim for completion in Dec’2024. Workplan for this bullet can be discussed in May’2024

# Discussion

In R18, the feature of multiple-Rx simultaneous reception was introduced to enhance the RRM requirements (L1-RSRP measurement delay, RLM, BFD/CBD etc.) for UEs capable of multi-beam/chain simultaneous DL reception on a single component carrier in FR2-1. In R19, similar enhancement is brought to L3 measurements (SSB based) in FR2-1. For UE capable of multiple-Rx, beam sweeping can be optimized and performed via multiple Rx panels simultaneously, as a result, optimizing/reducing L3 measurement delay in FR2-1.

It is clear that L3 measurement is widely used in the specs for different RRM purposes/procedures, thus it is not expected to activate multi-Rx all the time for those procedures even when it is not necessary to enhance L3 measurements delay. This could impact UE power consumption negatively. Therefore, activating/deactivating multi-Rx (for fast beam sweeping in L3 measurements) shall depend on the scenario and when the corresponding conditions apply.

Observation 1: L3 measurement is widely used in the specs for different RRM purposes/procedures, thus it is not expected to activate multi-Rx all the time for those procedures even when it is not necessary to enhance L3 measurements delay, whereas UE power consumption can be significant.

In the last meeting, RAN4 discussed the conditions to apply the enhancement of FR2-1 SSB based L3 measurement delay reduction. Some companies proposed to reuse the same conditions defined in R18 multi-Rx simultaneous reception WI for R19 L3 measurement delay reduction when UE support multi-Rx. However, if the same conditions specified in Rel-18 multi-Rx are applied, UE multi-Rx would be always activated for any L3 measurements whenever GBBR is configured, regardless which RRM scenario or UE current conditions.

Observation 2: If the same conditions specified in Rel-18 multi-Rx are applied to R19 L3 measurement delay reduction, UE multi-Rx would be always activated for any L3 measurements whenever GBBR is configured (i.e., regardless which RRM scenario or UE current conditions).

In our view, these conditions (i.e., for R18 multi-Rx) are not applicable to R19 L3 measurement enhancement. RAN4 needs to discuss these conditions taking into account the power consumption resulting from activating multi-Rx for L3 measurements. Activating multi-Rx for L3 measurements (intra/inter-frequency) may or may not be always necessary, depending on UE current conditions:

- UE location (cell centre or cell edge)

- UE mobility (stationary or moving)

- Both above

Proposal 1: Do not reuse the same applicable conditions specified in Rel-18 multi-Rx for Rel-19 L3 measurement delay reduction with UE supporting multi-Rx.

Proposal 2: Activating multi-Rx for L3 measurements (intra/inter-frequency) may or may not be always necessary, depending on UE current conditions:

- UE location (cell centre or cell edge)

- UE mobility (stationary or moving)

- Both above

In the last meeting, RAN4 also discussed whether to consider HST in this WI (i.e., high mobility). It is known that HST feature is discussed under a separate WI and therefore it can be precluded from the discussion on FR2-1 SSB based L3 measurement delay reduction. However, low/medium speed mobility of UEs should be considered as one of the conditions to activate multi-Rx for L3 measurement delay reduction.

Proposal 3: On UE mobility status, RAN4 should consider low/medium speed mobility of the UEs as one of the conditions to activate multi-Rx for L3 measurement delay reduction.

UE power consumption due to activating multiple panels for simultaneous DL reception is an important aspect to discuss and consider in RAN4. If UE fulfills the conditions to enable FR2-1 SSB based L3 measurement delay reduction by activating multi-Rx, UE should also have the capability to indicate to the NW whenever UE needs to deactivate multi-Rx, for example, due to overheating resulting from activating multiple panels for simultaneous reception for long time.

Proposal 4: RAN4 to discuss UE indication capability to the NW whenever UE requires to deactivate multi-Rx for FR2-1 SSB based L3 measurement delay reduction (e.g., indication due to overheating resulting from activating multiple panels for long time).

The enhancement of using multi-Rx for L3 measurement delay reduction can include the following scenarios: Intra-frequency measurement with/without gap and Inter-frequency measurement with/without gap. By optimizing Rx beam sweeping for UE supporting multi-Rx, the aforementioned L3 measurements can be enhanced in FR2. However, these measurements are widely used in the specs for different RRM purposes/procedures, thus it is not expected to activate multi-Rx all the time for those procedures even when it is not necessary to enhance L3 measurements delay for intra/inter frequency. Therefore, activating multi-Rx for intra/inter frequency measurements should be conditional when the under-discussion conditions in this WI are met.

Proposal 5: L3 measurement delay reduction by optimizing Rx beam sweeping factor are applied to the following scenarios only when the conditions (under discussion) for activating multi-Rx in R19 are met:

- Intra-frequency measurements with/without gap

- Inter-frequency measurements with/without gap

Optimizing beam sweeping for L3 measurements can also include other RRM scenarios that are based on one-time events/procedures and at the same time they are crucial to complete as fast as possible such as

- Handover event, DAPS Handover event, handover with PSCell event

- SCell activation event, SCG activation event

- PSCell addition event

Proposal 6: L3 measurement delay reduction by optimizing Rx beam sweeping factor are applied to the following scenarios only when the conditions (under discussion) for activating multi-Rx in R19 are met:

•	Handover event, DAPS Handover event, handover with PSCell event

•	SCell activation event, SCG activation event

•	PSCell addition event

In addition, RAN4 also discussed the supported power class for the UE supporting multi-Rx for FR2-1 SSB based L3 measurement delay reduction. In our view, the same UE power class supported in R18 multi-Rx is applicable to this WI. This means UE supporting FR2-1 power class 3 as first priority.

Proposal 7: RAN4 to consider UE supporting FR2-1 power class 3 as first priority.

# Summary

In this contribution, the following observations and proposals are concluded:

Observation 1: L3 measurement is widely used in the specs for different RRM purposes/procedures, thus it is not expected to activate multi-Rx all the time for those procedures even when it is not necessary to enhance L3 measurements delay, whereas UE power consumption can be significant.

Observation 2: If the same conditions specified in Rel-18 multi-Rx are applied to R19 L3 measurement delay reduction, UE multi-Rx would be always activated for any L3 measurements whenever GBBR is configured (i.e., regardless which RRM scenario or UE current conditions).

Proposal 1: Do not reuse the same applicable conditions specified in Rel-18 multi-Rx for Rel-19 L3 measurement delay reduction with UE supporting multi-Rx.

Proposal 2: Activating multi-Rx for L3 measurements (intra/inter-frequency) may or may not be always necessary, depending on UE current conditions:

- UE location (cell centre or cell edge)

- UE mobility (stationary or moving)

- Both above

Proposal 3: On UE mobility status, RAN4 should consider low/medium speed mobility of the UEs as one of the conditions to activate multi-Rx for L3 measurement delay reduction.

Proposal 4: RAN4 to discuss UE indication capability to the NW whenever UE requires to deactivate multi-Rx for FR2-1 SSB based L3 measurement delay reduction (e.g., indication due to overheating resulting from activating multiple panels for long time).

Proposal 5: L3 measurement delay reduction by optimizing Rx beam sweeping factor are applied to the following scenarios only when the conditions (under discussion) for activating multi-Rx in R19 are met:

- Intra-frequency measurements with/without gap

- Inter-frequency measurements with/without gap

Proposal 6: L3 measurement delay reduction by optimizing Rx beam sweeping factor are applied to the following scenarios only when the conditions (under discussion) for activating multi-Rx in R19 are met:

•	Handover event, DAPS Handover event, handover with PSCell event

•	SCell activation event, SCG activation event

•	PSCell addition event

Proposal 7: RAN4 to consider UE supporting FR2-1 power class 3 as first priority.

# References

- R4-2410260, Way Forward for [111][228] NR_RRM_Ph5, Apple.