3GPP TSG-RAN WG4 Meeting #112	R4-2413327

Maastricht, Netherlands, August 19 – August 23, 2024

Agenda Item:	8.15.3

Source:	MediaTek Inc.

Title:	Discussion on the RRM requirements for fast SCell activation for UE supporting Rel-18 EMR

Document for:	Discussion

# Introduction

The objective of R19 NR RRM Phase-5 work item was agreed in the WID as captured below [1]. In this meeting, RAN4 needs to start the work on the second objective of the WID “Fast SCell activation for UE supporting Rel-18 EMR”. Therefore, in this paper, we discuss the RRM core requirements for achieving fast SCell activation for UE supporting R18 EMR.

- FR2-1 SSB based L3 measurement delay reduction for connected mode - For UE supporting multiple-Rx simultaneous reception on single carrier: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - Rx beam sweeping factor - For UE not in multiple-Rx simultaneous reception mode: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - CSSF outside gap in CA/DC scenarios - Baseline assumption on number of searchers is 2 - Fast SCell activation for UE supporting Rel-18 EMR - Study and, if feasible, to reduce the SCell activation delay with valid EMR reporting - Apply fast scell activation in FR1 and FR2-1 - Note: RAN4 to start this work from Q3’2024 and aim for completion in Dec’2024. Workplan for this bullet can be discussed in May’2024

# Discussion

For the UE supporting Rel-18 EMR, the UE can be configured to perform RRM measurements in idle/inactive mode and then check the validity of these measurements before reporting them in the connected mode. The validity of Rel-18 EMR can be checked based on whether the measurement is performed within [X] seconds before msg1 transmission for RRC resume/setup request.

In order to utilize Rel-18 EMR reporting for fast SCell activation, RAN4 needs to discuss the required conditions that ensure the applicability of using R18 EMR towards enhancing SCell activation delay.

Proposal 1: RAN4 to discuss the conditions required to ensure the applicability of using R18 EMR reporting to achieve fast SCell activation.

One of the main issues that RAN4 need to address is that whether the reported Rel-18 EMR in connected mode is fresh enough to be considered for SCell activation delay enhancement. One scenario can be, for example, the measurements performed in idle/inactive mode are valid (based on validity check of R18 EMR) but happened at the beginning of X window used for validity check, where X value can be very large according to R18 Specs. Then if these measurement results are reported in connected mode after some time, RAN4 need to study whether the report can still be used for SCell activation enhancement. In our view, RAN4 need to introduce a new time window Y to represent the validity of R18 EMR for SCell activation delay reduction.

Figure 1: Fast SCell activation for UE supporting Rel-18 EMR

Proposal 2: The applicability of Rel-18 EMR reporting for SCell activation delay reduction can be based on the following conditions:

- Rel-18 EMR of the SCell-to-be-activated is sent within [Y] time window before the reception of the SCell activation command.

- The SSB measured of the SCell-to-be-activated remains detectable during [Y]

- FFS [Y]

If the above conditions are met, then the unknown SCell can be activated by applying the activation delay used for the known SCell scenario, i.e., significant delay reduction can be achieved with respect to the unknown SCell activation delay. Otherwise, if the above conditions are not met, legacy SCell activation shall apply.

Proposal 3: If the introduced conditions in RAN4 for fast SCell activation using R18 EMR are met, then the unknow SCell can be activated by applying the activation delay used for the known SCell scenario.

Since SCell activation can be triggered in different ways in Connected mode, for example, using MAC CE command for normal SCell activation, or using RRC command for direct SCell activation. The solution RAN4 will define in this feature shall be applicable to both cases.

Proposal 4: Fast SCell activation using Rel-18 EMR is applicable to normal SCell activation (triggered by MAC CE command) and direct SCell activation (triggered by RRC command).

# Summary

In this contribution, the following proposals are concluded:

Proposal 1: RAN4 to discuss the conditions required to ensure the applicability of using R18 EMR reporting to achieve fast SCell activation.

Proposal 2: The applicability of Rel-18 EMR reporting for SCell activation delay reduction can be based on the following conditions:

- Rel-18 EMR of the SCell-to-be-activated is sent within [Y] time window before the reception of the SCell activation command.

- The SSB measured of the SCell-to-be-activated remains detectable during [Y]

- FFS [Y]

Proposal 3: If the introduced conditions in RAN4 for fast SCell activation using R18 EMR are met, then the unknow SCell can be activated by applying the activation delay used for the known SCell scenario.

Proposal 4: Fast SCell activation using Rel-18 EMR is applicable to normal SCell activation (triggered by MAC CE command) and direct SCell activation (triggered by RRC command).

# References

- R4-2410260, Way Forward for [111][228] NR_RRM_Ph5, Apple.