3GPP TSG-RAN WG4 Meeting # 112	R4-2413328

Maastricht City, Maastricht, NL, Aug 19 – Aug 23, 2024

Agenda Item: 8.18.3

Source: MediaTek Inc.

Title: Discussion on R19 MIMO for RRM core part requirement

Document for: Discussion

# Introduction

According to WID [1], it mentioned several objectives below for R19 MIMO:

- UE-initiated/event-driven beam management

- CSI support for up to 128 CSI-RS ports

- UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul

- Non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions

- Enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios

# Discussion

In this document, we provided our view on whether each objective has RRM impact or not on the following section.

## 2.1 UE-initiated/event-driven beam management

[1] RP-233090 New WID - Rel-19 MIMO.doc ## 4 Objective ### 4.1 Objective of SI or Core part WI or Testing part WI The detailed objectives are as follows: RAN1: - Specify enhancement to facilitate UE-initiated/event-driven beam management for reducing overhead and/or latency, assuming the unified TCI while leveraging (as much as possible) legacy CSI measurement and reporting configuration frameworks, targeting FR2 and sTRP with intra- and inter-cell beam management - UL signaling content(s) for UE-initiated/event-driven beam reporting facilitating fast beam switching - UL signaling medium/container considering the UE-initiated/event-driven nature of the UL transmission, designed primarily for the purpose of beam reporting [2] R1-2404713 Moderator Summary #1 on UE-initiated/event-driven beam management Moderator (ZTE) Agreement On UE-initiated/event-driven beam reporting, regarding UL signaling content(s) of L1-RSRP report depending on Event-2, in a report instance, at least Option-3 is supported. - Option-3: N ≥ 1 beam(s) are reported in the report instance, - At least one of N reported beam(s) should satisfy the condition of Event-2 - N is configured by gNB - FFS: candidate value of ‘N’. - FFS: RRC can enable or disable whether current beam is always reported in addition to the N beams - FFS: Option-1/1a/1b/2. - Above applies at least for the single CC case

According to objective [1] and RAN1 agreements [2], UE initiated event-driven beam measurement and reporting is a kind of L1-RSRP measurement and reporting. In our understanding, it’s quite like aperiodic L1-RSRP reporting in legacy. Therefore, there’re RRM impact naturally for this event triggered beam reporting at least for measurement requirement and applicability.

Proposal 1: RAN4 to discuss the RRM impact for UE-initiated/event-driven beam management at least for L1-RSRP measurement requirement and applicability.

## 2.2 CSI support for up to 128 CSI-RS ports

[1] RP-233090 New WID - Rel-19 MIMO.doc ## 4 Objective ### 4.1 Objective of SI or Core part WI or Testing part WI The detailed objectives are as follows: RAN1: - Specify CSI support for up to 128 CSI-RS ports, targeting FR1 - Type-I codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks - Type-II codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks, without modifying any codebook parameter other than introducing additional values for the number of ports codebook parameter(s) - Extension of CRI(s)-based CSI reporting (CQI/PMI/RI calculated per CRI for ≥1 CRIs) for hybrid beamforming supporting up to a total of 128 CSI-RS ports across all resources, with up to 32 CSI-RS ports per resource, without new codebook design

In our understanding, the CSI enhancement is mainly focus on codebook enhancement for up to 128 CSI-RS ports without RRM impact. Therefore, the following proposal is suggested:

Proposal 2: No RRM impact for CSI support for up to 128 CSI-RS ports.

## 2.3 UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul

[1] RP-233090 New WID - Rel-19 MIMO.doc ## 4 Objective ### 4.1 Objective of SI or Core part WI or Testing part WI The detailed objectives are as follows: RAN1: - Specify UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul, targeting FR1, both FDD and TDD - Inter-TRP time misalignment and frequency/phase offset measurement and reporting, assuming legacy CSI-RS design, with stand-alone aperiodic reporting on PUSCH [3] R1-2402459 Moderator summary on Rel-19 CSI enhancement Agreement For the Rel-19 aperiodic standalone CJT calibration reporting of {(Dn,offset, dn), n=0, 1, …, NTRP – 1, n≠nref}, regarding the interval [δi,δi+1] which Dn,offset falls into, δ0,δ1,…,δMD-1 is uniformly spaced between 0 and AD, i.e. δi=i×ADMD-1, i=0,1,…, MD-1, with MD=2B and [δMD-1,∞] represent ‘out-of-range’. [4] R1-2403602 Moderator Summary#3 on Rel-19 CSI enhancements: ROUND 3 Moderator (Samsung) From Wednesday session Agreement For the Rel-19 aperiodic standalone CJT calibration reporting, regarding frequency offset reporting, FOi=i×AFOMFO-1, i=0,1,…,MFO-2 and FOMFO-1 represents an ‘invalid’ state. [5] R1-2404108 Moderator summary on Rel-19 CSI enhancements Moderator (Samsung) Agreement For the Rel-19 aperiodic standalone CJT calibration reporting, regarding the dynamic range for delay offset reporting Dn,offset, i.e. AD, at least support the following values: {0.5CP, CP} - Decide, by RAN1#117, whether any of the following candidate values are supported: {0.75CP, 1.5CP, 14∆f, 112∆f,124∆f} Agreement For the Rel-19 aperiodic standalone CJT calibration reporting, regarding the dynamic range for frequency offset reporting FOn, i.e. AFO, at least support the following values: {0.1ppm, 0.2ppm} - Decide, by RAN1#117, whether any of the following candidate values are supported: {0.025ppm, 0.05ppm, 1/(8Dt), 1/(16Dt), 1/(32Dt)} Agreement For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset), S=1 only (agreed in RAN1#116bis) implies that the measured/reported phase offsets {Fn,0, n=0, 1, …, NTRP – 1, n≠nref} are associated with the entire configured CSI reporting band (i.e. ‘wideband’)

RAN1 has already specified some mechanism of UE reporting enhancement for CJT deployments including timing misalignment, frequency offset and phase offset reporting. In our understanding, these are new feature for reporting in terms of the compensation of delay, frequency, and phase offset. The details for the enhancements are still under discussion on RAN1. We can further discuss whether these CJT calibration reporting with RRM impact or not in RAN4 at least for the delay requirement or the accuracy requirement.

Proposal 3: RAN4 to discuss whether there’re RRM impact for the Rel-19 aperiodic CJT calibration reporting.

- Delay requirement

- RRM accuracy requirement

## 2.4 Non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions

[1] RP-233090 New WID - Rel-19 MIMO.doc ## 4 Objective ### 4.1 Objective of SI or Core part WI or Testing part WI The detailed objectives are as follows: RAN1: - Specify non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions, without enhancement on UL full power transmission and without enhancement on SRS resource [6] R1-2403851 FL Summary Support for 3TX CB-based Uplink; First Round Moderator (InterDigital, Inc.) Agreement Update the agreement made in RAN1 #116bis as the following. Agreement For a 3TX UE, to support 3-port SRS transmission with reusing a 4-port SRS resource, support the following for muting one of the ports of the configured 4-port SRS resource, Option 3: Always a same port is muted, e.g., i.e., the 4th port.

The non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions is mainly focus on some enhancement related to codebook-based mechanism. From RAN1 agreement, 3TX UE to support 3-port SRS transmission could reusing the configured 4-port SRS resource. It means one of the ports will be muted from the 4-port SRS resource. Therefore, there’s no RRM impact for 3-antenna-port codebook-based transmissions.

Proposal 4: No RRM impact for 3TX CB-based Uplink (3-antenna-port codebook-based transmissions) since there's no enhancement for 3-port SRS antenna switching in Rel-19.

## 2.5 Enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios

[1] RP-233090 New WID - Rel-19 MIMO.doc ## 4 Objective ### 4.1 Objective of SI or Core part WI or Testing part WI The detailed objectives are as follows: RAN1: 5. Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2 - Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP. [7] R1-2403415 Summary #2 on Rel-19 asymmetric DL sTRP/UL mTRP Moderator (OPPO) From Tuesday session Agreement Support applying PL offset on PDCCH-order PRACH towards a UL TRP in FR1. - Note: The DL reference timing determination for PDCCH-order PRACH transmission to an UL TRP is still based on the DL RS defined in current RAN4 specification. - Above is subject to a separate UE capability signaling. [8] R1-2405346 Summary #1 on Rel-19 asymmetric DL sTRP/UL mTRP Moderator (OPPO) Agreement For indicating a PL offset for PDCCH-order PRACH transmission at least for FR1, further study and down-select one from the Alt1 and Alt3 by RAN1#118 meeting: - Alt1: RRC configures multiple PL offset values [in PRACH-Config] and PDCCH-order DCI indicates one of them through one DCI field - Alt3: The PL offset associated with one of the indicated joint/UL TCI state for UL TRP in unified TCI framework is applied on the PDCCH-order PRACH transmission - FFS: the details of DCI field design. Conclusion For the asymmetric DL sTRP/UL mTRP deployment scenario, reuse the rel-17 unified TCI/ICBM and rel-18 unified TCI framework: - When rel-17 unified TCI/ICBM is configured: - For FR1: one joint TCI state or {one DL TCI state + one UL TCI state} can be applied. - For FR2: {one DL TCI state + one UL TCI state} can be applied. - When rel-18 unified TCI is configured: - For FR1: up to two joint TCI states or {one DL TCI state + up to two UL TCI state} can be applied. - For FR2: {one DL TCI state + up to two UL TCI states} can be applied.

In our understanding, NW will indicate PL offset for UE to calibrate for asymmetric DL sTRP and UL mTRP. From RAN1 agreement, the DL reference timing definition for PDCCH-order PRACH is based on current RAN4 specification. The unified TCI state for the asymmetric DL sTRP/UL mTRP can totally reuse Rel-17 and Rel-18 TCI framework based on RAN1 agreement. Therefore, the following proposal is suggested.

Proposal 5: No RRM impact at least for unified TCI state switching since RAN1 agreed to reuse Rel-17/Rel-18 unified TCI framework on the enhancement of asymmetric DL sTRP/UL mTRP deployment scenarios.

# Summary

In this paper, the discussion of R19 MIMO is provided. We have the following proposals:

Proposal 1: RAN4 to discuss the RRM impact for UE-initiated/event-driven beam management at least for L1-RSRP measurement requirement and applicability.

Proposal 2: No RRM impact for CSI support for up to 128 CSI-RS ports.

Proposal 3: RAN4 to discuss whether there’re RRM impact for the Rel-19 aperiodic CJT calibration reporting.

- Delay requirement

- RRM accuracy requirement

Proposal 4: No RRM impact for 3TX CB-based Uplink (3-antenna-port codebook-based transmissions) since there's no enhancement for 3-port SRS antenna switching in Rel-19.

Proposal 5: No RRM impact at least for unified TCI state switching since RAN1 agreed to reuse Rel-17/Rel-18 unified TCI framework on the enhancement of asymmetric DL sTRP/UL mTRP deployment scenarios.

# Reference

[1] RP-233090 New WID - Rel-19 MIMO

[2] R1-2404713	Moderator Summary #1 on UE-initiated/event-driven beam management	Moderator (ZTE)

[3] R1-2402459 Moderator summary on Rel-19 CSI enhancement (Samsung)

[4] R1-2403602 Moderator Summary#3 on Rel-19 CSI enhancements: ROUND 3 Moderator (Samsung)

[5] R1-2404108	Moderator summary on Rel-19 CSI enhancements Moderator (Samsung)

[6] R1-2403851 FL Summary Support for 3TX CB-based Uplink; First Round	Moderator (InterDigital, Inc.)

[7] R1-2403415 Summary #2 on Rel-19 asymmetric DL sTRP/UL mTRP	Moderator (OPPO)

From Tuesday session

[8] R1-2405346 Summary #1 on Rel-19 asymmetric DL sTRP/UL mTRP	Moderator (OPPO)