3GPP TSG-RAN4 Meeting #112	   											R4-2413333

# Maastricht, NL, 19th of August 2024 – 23rd of August 2024

Title: 	Discussion on dynamic NTN testing for NGSO

Agenda Item:	8.8.4

Source: 	THALES

Document for:	Discussion

# For information

As described in RP-240857 (New WID: Enhanced requirements and test methodology for NR and IoT NTN), given that the mobile services in the NGSO satellite is a rapidly growing market segment, the NTN UE performance is expected to be fully verified to ensure that 3GPP tests cover the operation of UE with NGSO networks i.e. dynamic conditions, which may require new channel model with varying Doppler shift and/or timing shift matching the motion trajectory of NGSO satellite.

The objective of the performance part WI for NTN testing for NGSO was described as:

- Specify the TE-emulated channel model with varying Doppler and delay shifts for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands

  - Match the satellite motion trajectory based on the ephemeris for NGSO (Non-Geostationary Orbit) scenarios

  - Inform RAN5 to assist specifying RF frequency error tests, if needed

    - Checking if the existing frequency error requirement, can be met under the TE-emulated channel model without intention of modifying the existing core requirement

  - Specify UE RRM performance test of uplink timing for NGSO for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands based on the above channel model

    - Checking if the existing RRM uplink timing requirements, can be met under the TE-emulated channel model without intention of modifying the existing core requirement

- Specify UE demodulation performance requirement(s) of PDSCH for NGSO for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands based on the above channel model

  - Minimize the number of demodulation performance requirements

- Specify the applicability of the tests under the TE-emulated channel model

However, it is also important to mention that several solutions have been already provided in the past in contributions submitted to RAN5 and RAN4, where dynamic Doppler and Delay values have been computed with respect to the satellite movement and specific UE position.

As the matter of fact, as shown e.g. in THALES contributions:

1/ R5-237213 “GSO satellite Ephemeris file generation methodology for NTN NR UE testing” presented at RAN4#101, R5-241792 “Neighbor GSO satellite ephemeris file for NTN NR UE testing” presented at RAN5#102;

2/ R5-237216 “NGSO satellite Ephemeris file generation methodology for NTN NR UE testing” presented at RAN5#101.

Moreover, in these contributions THALES provided not only variable Doppler and Delay values, but also associated satellite ephemeris data for both GSO and NGSO.

As a matter of fact, a summary of the results from these contributions have been even submitted to RAN4#109, see “Summary of SIB19/SIB31 parameters for NGSO and GSO NTN UE/NTN IoT testing”, THALES, R4-2320975, November 2023.

For instance, one can represent the following few “selected” scenarios (as examples, to give some idea about the ranges) computed with a min elevation angle target of 30°:

| Frequency | Constellation | Elevation | Doppler shift (kHz) | OneWayDelay_ms | Xsat_ECEF_Coded_m | Ysat_ECEF_Coded_m | Zsat_ECEF_Coded_m | VXsat_ECEF_Coded_m | VYsat_ECEF_Coded_m | VZsat_ECEF_Coded_m | sma_ECEF_Coded | Excentricity_ECEF_Coded | Inc_ECEF_Coded | RAAN_ECEF_Coded | AOP_ECEF_Coded | MeanAnom_ECEF_Coded |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2,00 GHz | LEO@600 km | 30,0 | 39,734 | 3,6 | -2654979,0 | 4388008,0 | 1590654,0 | 14542,0 | -34415,0 | 120208,0 | 127433469,0 | 625589,0 | 65357030,0 | 89802029,0 | 197030161,0 | 11181850,0 |
| 2,00 GHz | LEO@600 km | 149,9 | -39,899 | 3,7 | -2401021 | 3830217 | 2907041 | 28879 | -61410 | 105774 | 126505185 | 585942 | 64768697 | 89517532 | 197339614 | 22986454 |
| 2,00 GHz | LEO@600 km | 89,6 | 0,026 | 2,0 | -2550081 | 4151097 | 2263034 | 21970 | -48164 | 114109 | 127026053 | 607046 | 65099618 | 89701469 | 196621435 | 16735395 |
| 2,00 GHz | LEO@1200 km | 29,9 | 35,127 | 6,7 | -2935802 | 4896147 | 1184965 | 8471 | -23252 | 118268 | 273627907 | 769347 | 65474780 | 89834882 | 198836074 | 7798966 |
| 2,00 GHz | LEO@1200 km | 149,9 | -35,273 | 6,8 | -2471684 | 3860360 | 3621839 | 31517 | -67464 | 94643 | 272191987 | 715100 | 64360117 | 89258597 | 199900686 | 27917535 |
| 2,00 GHz | LEO@1200 km | 89,5 | 0,007 | 4,0 | -2771542 | 4509243 | 2455572 | 20837 | -46176 | 109528 | 273057241 | 744972 | 65034798 | 89682433 | 197978669 | 17169725 |
| 30,00 GHz | LEO@600 km | 30,0 | 596,011 | 3,6 | -2654979 | 4388008 | 1590654 | 14542 | -34415 | 120208 | 127433469 | 625589 | 65357030 | 89802029 | 197030161 | 11181850 |
| 30,00 GHz | LEO@600 km | 149,9 | -598,486 | 3,7 | -2401021 | 3830217 | 2907041 | 28879 | -61410 | 105774 | 126505185 | 585942 | 64768697 | 89517532 | 197339614 | 22986454 |
| 30,00 GHz | LEO@600 km | 89,6 | 0,384 | 2,0 | -2550081 | 4151097 | 2263034 | 21970 | -48164 | 114109 | 127026053 | 607046 | 65099618 | 89701469 | 196621435 | 16735395 |
| 30,00 GHz | LEO@1200 km | 29,9 | 526,908 | 6,7 | -2935802 | 4896147 | 1184965 | 8471 | -23252 | 118268 | 273627907 | 769347 | 65474780 | 89834882 | 198836074 | 7798966 |
| 30,00 GHz | LEO@1200 km | 149,9 | -529,095 | 6,8 | -2471684 | 3860360 | 3621839 | 31517 | -67464 | 94643 | 272191987 | 715100 | 64360117 | 89258597 | 199900686 | 27917535 |
| 30,00 GHz | LEO@1200 km | 89,5 | 0,110 | 4,0 | -2771542 | 4509243 | 2455572 | 20837 | -46176 | 109528 | 273057241 | 744972 | 65034798 | 89682433 | 197978669 | 17169725 |
| 2,00 GHz | GEO | 90,14 | 0,000 | 119,37 | -16975960 | 27636606 | 68083 | 0 | 0 | -48 | 8393577246 | 0 | 224827774 | 67241608 | 268307824 | 18691093 |
| 2,00 GHz | GSO | 58,85 | 0,19 | 121,99 | -16926361 | 27652344 | -903320 | 387 | 36 | -6130 | 8393577246 | 0 | 224829171 | 69524249 | 1191777 | 143241486 |
| 2,00 GHz | GSO | 58,85 | -0,19 | 121,99 | -17018430 | 27595775 | -903373 | 207 | 328 | 6130 | 8393577246 | 0 | 90637528 | 69524136 | 133006899 | 257895279 |
| 2,00 GHz | GSO | 68,87 | 0,000 | 120,59 | -16850867 | 27425327 | 3983665 | -330 | -203 | 0 | 8393577246 | 0 | 23571205 | 5259905 | 201262439 | 66369698 |
| 30,00 GHz | GEO | 90,14 | 0,000 | 119,37 | -16975960 | 27636606 | 68083 | 0 | 0 | -48 | 8393577246 | 0 | 224827774 | 67241608 | 268307824 | 18691093 |
| 30,00 GHz | GSO | 58,85 | 2,87 | 121,99 | -16926361 | 27652344 | -903320 | 387 | 36 | -6130 | 8393577246 | 0 | 224829171 | 69524249 | 1191777 | 143241486 |
| 30,00 GHz | GSO | 58,85 | -2,87 | 121,99 | -17018430 | 27595775 | -903373 | 207 | 328 | 6130 | 8393577246 | 0 | 90637528 | 69524136 | 133006899 | 257895279 |
| 30,00 GHz | GSO | 68,87 | 0,000 | 120,59 | -16850867 | 27425327 | 3983665 | -330 | -203 | 0 | 8393577246 | 0 | 23571205 | 5259905 | 201262439 | 66369698 |

Similarly, one can represent the following few “selected” scenarios (as examples, to give some idea about the ranges) computed with a min elevation angle target of 10°:

| Frequency | Constellation | Elevation | Doppler shift (kHz) | OneWayDelay_ms | Xsat_ECEF_Coded_m | Ysat_ECEF_Coded_m | Zsat_ECEF_Coded_m | VXsat_ECEF_Coded_m | VYsat_ECEF_Coded_m | VZsat_ECEF_Coded_m | sma_ECEF_Coded | Excentricity_ECEF_Coded | Inc_ECEF_Coded | RAAN_ECEF_Coded | AOP_ECEF_Coded | MeanAnom_ECEF_Coded |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2,00 GHz | LEO@600 km | 9,9 | 45,304 | 6,5 | -2717798,0 | 4550992,0 | 849128,0 | 6122,0 | -19350,0 | 124295,0 | 127718928,0 | 639459,0 | 65536691,0 | 89845489,0 | 198612804,0 | 5766116,0 |
| 2,00 GHz | LEO@600 km | 170,0 | -45,239 | 6,6 | -2199272 | 3404229 | 3535794 | 35394 | -74414 | 94682 | 125875956 | 565437 | 64365904 | 89211929 | 199233028 | 30409196 |
| 2,00 GHz | LEO@600 km | 89,6 | 0,026 | 2,0 | -2550081 | 4151097 | 2263034 | 21970 | -48164 | 114109 | 127026053 | 607046 | 65099618 | 89701469 | 196621435 | 16735395 |
| 2,00 GHz | LEO@1200 km | 10,0 | 40,062 | 10,4 | -2962861 | 5019276 | 97850 | -2615 | -3875 | 120784 | 273800787 | 777311 | 65607366 | 89853215 | 201096313 | 632648 |
| 2,00 GHz | LEO@1200 km | 170,0 | -39,918 | 10,7 | -2142675 | 3156398 | 4434942 | 38457 | -82453 | 78501 | 271404201 | 698334 | 63736334 | 88651622 | 203306610 | 37653139 |
| 2,00 GHz | LEO@1200 km | 89,5 | 0,007 | 4,0 | -2771542 | 4509243 | 2455572 | 20837 | -46176 | 109528 | 273057241 | 744972 | 65034798 | 89682433 | 197978669 | 17169725 |
| 30,00 GHz | LEO@600 km | 9,9 | 679,567 | 6,458 | -2717798 | 4550992 | 849128 | 6122 | -19350 | 124295 | 127718928 | 639459 | 65536691 | 89845489 | 198612804 | 5766116 |
| 30,00 GHz | LEO@600 km | 170,0 | -678,582 | 6,605 | -2199272 | 3404229 | 3535794 | 35394 | -74414 | 94682 | 125875956 | 565437 | 64365904 | 89211929 | 199233028 | 30409196 |
| 30,00 GHz | LEO@600 km | 89,6 | 0,384 | 2,031 | -2550081 | 4151097 | 2263034 | 21970 | -48164 | 114109 | 127026053 | 607046 | 65099618 | 89701469 | 196621435 | 16735395 |
| 30,00 GHz | LEO@1200 km | 10,0 | 600,927 | 10,441 | -2962861 | 5019276 | 97850 | -2615 | -3875 | 120784 | 273800787 | 777311 | 65607366 | 89853215 | 201096313 | 632648 |
| 30,00 GHz | LEO@1200 km | 170,0 | -598,774 | 10,656 | -2142675 | 3156398 | 4434942 | 38457 | -82453 | 78501 | 271404201 | 698334 | 63736334 | 88651622 | 203306610 | 37653139 |
| 30,00 GHz | LEO@1200 km | 89,5 | 0,110 | 4,039 | -2771542 | 4509243 | 2455572 | 20837 | -46176 | 109528 | 273057241 | 744972 | 65034798 | 89682433 | 197978669 | 17169725 |
| 2,00 GHz | GEO | 90,14 | 0,000 | 119,37 | -16975960 | 27636606 | 68083 | 0 | 0 | -48 | 8393577246 | 0 | 224827774 | 67241608 | 268307824 | 18691093 |
| 2,00 GHz | GSO | 58,85 | 0,19 | 121,99 | -16926361 | 27652344 | -903320 | 387 | 36 | -6130 | 8393577246 | 0 | 224829171 | 69524249 | 1191777 | 143241486 |
| 2,00 GHz | GSO | 58,85 | -0,19 | 121,99 | -17018430 | 27595775 | -903373 | 207 | 328 | 6130 | 8393577246 | 0 | 90637528 | 69524136 | 133006899 | 257895279 |
| 2,00 GHz | GSO | 68,87 | 0,000 | 120,59 | -16850867 | 27425327 | 3983665 | -330 | -203 | 0 | 8393577246 | 0 | 23571205 | 5259905 | 201262439 | 66369698 |
| 30,00 GHz | GEO | 90,14 | 0,000 | 119,37 | -16975960 | 27636606 | 68083 | 0 | 0 | -48 | 8393577246 | 0 | 224827774 | 67241608 | 268307824 | 18691093 |
| 30,00 GHz | GSO | 58,85 | 2,87 | 121,99 | -16926361 | 27652344 | -903320 | 387 | 36 | -6130 | 8393577246 | 0 | 224829171 | 69524249 | 1191777 | 143241486 |
| 30,00 GHz | GSO | 58,85 | -2,87 | 121,99 | -17018430 | 27595775 | -903373 | 207 | 328 | 6130 | 8393577246 | 0 | 90637528 | 69524136 | 133006899 | 257895279 |
| 30,00 GHz | GSO | 68,87 | 0,000 | 120,59 | -16850867 | 27425327 | 3983665 | -330 | -203 | 0 | 8393577246 | 0 | 23571205 | 5259905 | 201262439 | 66369698 |

# Proposals

For deriving NGSO Doppler and Delay required for the SAT-UE NTN dynamic channel model, RAN4 can use the following example files (already provided to RAN5):

with the UE reference coordinates considered such as:

- Latitude 25.08439333 degrees;

- Longitude 121.56076999 degrees;

- Altitude 0 m.

The values have been generated for both LEO@600km and LEO@1200km with a step size of 640ms (multiple of 160ms) but the intermediate values for Doppler and Delay can be easily interpolated from the points provided in these files allowing sufficient level of granularity to generate the channel dynamics with granularity below OFDM symbol level.

Moreover, information on channel dynamics has been also integrated in the Annex E (Examples of Delay and Doppler shift variation as a function of time for different satellite orbit types) of TR 38.863, for instance:

1/ For this example of LEO@600km constellation, a satellite orbit inclination with respect of the Equator plane of 88° is considered.

Figure E.1-1: LEO600km with 88° orbit inclination

2/ For this example of LEO@1200km constellation, a satellite orbit inclination with respect of the Equator plane of 88° is considered.

Figure E.2-1: LEO1200km with 88° orbit inclination

Moreover, Doppler values are expressed in ppm and can be easily adapted for any carrier frequency/satellite frequency band.

As such, depending on the elevation range and satellite orbit, RAN4 can easily identify several sets of profiles as a combination of Doppler – Delay:

1/ Channel variation Profile 1:

- Doppler decreasing slowly,

- Delay decreasing fast.

2/ Channel variation Profile 2:

- Doppler decreasing fast,

- Delay decreasing fast.

3/ Channel variation Profile 3:

- Doppler near zero range (positive and then negative),

- Delay variation low (decreasing and then increasing).

4/ Channel variation Profile 4:

- Doppler decreasing fast,

- Delay increasing fast.

5/ Channel variation Profile 5:

- Doppler decreasing slowly,

- Delay increasing fast.

These profiles can be extracted directly from the previous files as in the following examples:

- LEO@600km

- LEO@1200km

# Conclusions

Proposal 1: For deriving NGSO Doppler and Delay required for the SAT-UE NTN dynamic channel model, RAN4 can use the following example files:

Proposal 2: The intermediate values for Doppler and Delay can be easily interpolated from the points provided in these files allowing sufficient level of granularity to generate the channel dynamics with granularity below OFDM symbol level.

Observation 1: Doppler values are expressed in ppm and can be easily adapted for any carrier frequency/satellite frequency band.

Proposal 3: Depending on the elevation range and satellite orbit, RAN4 can easily identify several sets of profiles as a combination of Doppler – Delay:

1/ Channel variation Profile 1:

- Doppler decreasing slowly,

- Delay decreasing fast.

2/ Channel variation Profile 2:

- Doppler decreasing fast,

- Delay decreasing fast.

3/ Channel variation Profile 3:

- Doppler near zero range (positive and then negative),

- Delay variation low (decreasing and then increasing).

4/ Channel variation Profile 4:

- Doppler decreasing fast,

- Delay increasing fast.

5/ Channel variation Profile 5:

- Doppler decreasing slowly,

- Delay increasing fast.

Proposal 4: These profiles can be extracted directly from the previous files as in the following example for LEO@600km:

Proposal 5: These profiles can be extracted directly from the previous files as in the following example for LEO@1200km: