3GPP TSG-RAN WG4 Meeting # 112						R4-2413339

Maastricht Meeting, Aug 19th – Aug 23rd, 2024

Source:	Nokia

Title:	Discussion on methods for reducing the number of UE Rx chains for Fragmented Carriers

Agenda item:	8.6.2

Document for:	Discussion

Introduction

At RAN#104 a new study item [1] was agreed on addressing the issues raised by operators to optimize UE receiver conditions with fragmented carriers.

The objective of this study is as follows:

- Identify methods for reducing the number of UE Rx chains (e.g. from separate RF chains per CC to shared RF chains ) needed for single DL band with frequency span ≤ 100 MHz, containing two non-contiguous CCs within a CA combination for the inter-operator co-located scenario, considering:

-	Which RF requirements could be adjusted for the inter-operator co-located BS scenario, e.g. existing UE RF requirements such as ΔRIBNC, ACS and in-band blocking [RAN4];

- Other requirements are not precluded, if identified

-	The ability to semi-statically switch hardware resources (i.e. Rx chains) [RAN4, RAN2 – See note 2];

-	Up to 6 dB DL received power spectral density imbalance between the two non-contiguous CCs [RAN4];

-	Determine a reasonable level for the power spectral density difference between carriers of co-located adjacent channel operators for study [RAN4]

-	Impacts on DL performance [RAN4] ;

-	Means for a UE to inform the network of appropriate CA configuration it can support with adjusted RF requirements [RAN4, RAN2 – See note 2].

NOTE 1:	No RAN1 impact is foreseen

NOTE 2:	RAN2 work, if necessary, will be triggered by RAN4 LS

NOTE 3:	This study starts from single DL band. Sharing RF chain is not considered among inter-band DL carriers.

- Whether and how to apply the conclusions to intra-band component of inter-band CA will be decided after the study on 2CC in a single DL band is completed.

This contribution discusses the aspects of methods for reducing the number of UE Rx chains for Fragmented Carriers.

Discussion

The following will discuss what is understood by fragmented carriers and how it relates to the number of needed UE Rx chains. Additionally, assumptions about what ‘UE Rx chain’ is will be elaborated. Finally, some suggestions for what RAN4 shall focus on when wanting to reduce the number of needed UE Rx chains for fragmented carriers are discussed.

Fragmented Carriers and their relation to the number of UE Rx chains

An example to present the case of fragmented carriers are shown in the figure below from [3].

Fig. 1 Examples of fragmented carriers [2]

Fragmented carriers is not something completely new for RAN4, since it is essentially non-contiguous intra-band carrier aggregation (NC IB CA). However, proponents of the new SI have chosen to name it Fragmented Carriers, since this is how it is perceived mainly from an operator perspective. Adding a specific name also makes sense to associate the non-contiguous intra-band carrier aggregation with the request for UEs to support downlink reception of the fragmented spectrum block with a single “receive chain”, “receive path” or “receive branch”.

Fragmented carriers are in some sense similar to non-contiguous carrier aggregation which is already specified for intra-band carrier aggregation in TS 38.101-1.

The discussion on what is meant by “Rx chain”, “Rx path” or “Rx branch” is a returning topic within RAN4.

It is not always clear within RAN4 what it meant by “Rx Chain”, “Rx path”  or “Rx Branch”.

Apple has in [3] provided what in their view is meant by a single Rx chain: “A single Rx chain consists of one main Rx and one diversity Rx for a UE with 2Rx”.

In [4] MediaTek shows a block diagram of a single Rx chain and appllying Apple's definition in [3] of a Rx chain to the main and diversity branch in MediaTek’s block diagram results in a definition of a single Rx chain as shown in Fig. 2.

| Single RX chain (consists of one main Rx and one diversity Rx) |  |
| --- | --- |
| Main receiver with RF carrier cantered between CCs | Diversity receiver with RF carrier centred between CCs |

Fig. 2 Single RX chain as presented in [4], where DAC must mean ADCs, since this is at the receiver.

A single Rx chain for a 2Rx UE consists of one main Rx and one diversity Rx branch according to previously shared understandings from UE vendors.

This is illustrated in Fig. 3 with the main and diversity receiver branch.

Fig. 3 Single Rx Chain for a 2Rx UE consisting of a main and diversity receiver branch (FDD).

It would be beneficial for further discussion to agree what the underlying assumptions are, as a basis for a common understanding, to align terminology which can help companies to ensure they discuss the same thing.

For a 2Rx UE, the common RAN4 understanding is that a single Rx chain consists of one main Rx branch and one diversity Rx branch.

Similar to the previous discussion for 2Rx UEs, it becomes even more inconsistent what is meant by a Rx chain for 4Rx UEs when reviewing literature and previous contributions in RAN4. However, it seems the most predominant interpretation is to have 1 main receiver branch and 3 diversity receiver branches. Meaning, when using Fig. 3 as a reference, there will be an addition of two diversity receiver branches.

For a 4Rx UE, the common RAN4 understanding is that a single Rx chain consists of one main Rx branch and three diversity Rx branches.

For the current UE implementation, the descriptions in [3] and [4] assume a separate UE RF chain per fragmented carrier which results in the limitations of the maximum number of supported CCs.

For the current UE implementation, a separate UE RF chain per fragmented carrier is assumed.

This study item challenges the need for applying a separate UE Rx chain per fragment. In other words, the ambition is to reduce the number of needed Rx chains for fragmented carrier operation.

Types of UE receiver architectures for reducing the number of Rx chains for fragmented carriers

As previously mentioned, the concept of fragmented carries is in some way similar to non-contiguous intra-band carrier aggregation (NC IB CA). The reception at the UE of non-contiguous intra-band carrier aggregation in a system with a single Rx chain will be impacted by interference especially in the “gap” between the fragments of the carrier as illustrated with a different colour in Fig. 1. The impact on UE requirements is further discussed in R4-2413340 while the following focuses on the UE receiver architectures.

The in-gap interference will, in a single analogue Rx chain system, pass un-attenuated as the UE does not have the ability to suppress/attenuate the in-gap interference via analogue filtering. Suggested solutions are shown in [4] where the UE, by utilizing additional Rx chains, can attend to this presence of an in-gap interferer. In [4] the single Rx chain system, as also shown in Fig. 2, is denoted Type 1 while two other multi-chain systems are denoted Type 2 and Type 3.

Type 1 UE receiver does not have the ability to suppress/attenuate in-gap interference via analogue filtering.

The differences between the Type 2 and Type 3 UE architectures of [4] may be a bit detailed to follow. Our understanding from [4] is that there is a split at the eLNA (external transceiver LNA) towards two iLNA (internal transceiver LNA) (Type2) and a split only at the DACs (Edit: They must mean ADCs, since this is at the receiver) for CC0 and CC1 (Type 3). However, it is not explaining how the UE actually may receive the two carriers individually.

To extend the understanding of the different choices for transceiver implementation, we have chosen to follow the principal presented by other companies and also split into three different types with emphasis on Type 2 and Type 3 in the following. This as Type 1 is the hardware state we want to free up hardware resources i.e. Rx chains as described in the introduction.

It can be noted that there is also the possibility to suppress some of the in-gap interference in the digital domain. However, this does not solve the dynamic range issues given by the near-far problem. It can mitigate some of the adjacent channel interference but only when within proper dynamic range provided from the analog and mixed signal Rx chain. Hence, the focus of this work is on solutions related to the analog Rx path.

Even though there may be a possibility to suppress some of the in-gap interference in the digital domain, the objective of the SI is to free up Rx resources hence the focus is on the analog Rx chains.

Type 2 – Internal RX path split:

There are two ways that the signal can be treated by separate Rx chains. It may be split at the front-end components (eLNA), which we see as Type 3, and it may be split inside the transceiver, which we see as Type 2. For Type 2 we would like to show a full picture without discussing the iLNA (internal LNA) split. The UE has two Rx chains active, and this allows the UE to center the LO1 and LO2 at the respective centers of each CC, so that the following filtering (channel filters) may attenuate also the in-gap interferer as shown here. In the example of Fig. 4 we consider non-collocation, so the in-gap interferer is seen as a “worst case” condition. In the collocated case, the wanted to interference ratio will be less.

Fig. 4 Type 2 – Internal Rx path split. A single Rx chain after frontend filtering is marked with a dashed square.

If this UE is to transition to a single Rx chain state (Type 1), it will have to accept an in-gap interferer as shown in the next view of Type 2, that visualizes the “Extra Receiver chain” as toned-out colours:

Fig. 5 Type 2 – Internal Rx path split. Unused Rx chain is toned-out. Type 2 becomes Type 1.

In this state, the UE is without analogue filtering to suppress the interferer, and RAN4 will have to discuss the relaxation of the receiver requirements for the UE to tolerate some level of in-gap interference presence. Since the requirement is based on the presence of an interfering signal, it is closer related to the ACS requirement than to the Reference sensitivity requirement for non-contiguous intra-band carrier aggregation, and a new set of requirements must capture the conditions and the relaxation for the UE state that uses a single Rx chain for the reception of the two fragmented carriers in the presence of an in-gap interferer.

For the case that the UE uses a single Rx chain for the reception of the two fragmented carriers in the presence of an in-gap interferer, a new set of requirements must capture the conditions and the relaxation.

This means that when a Type 2 implementation is using a subset of its Rx chains when operating in fragmented carrier mode the new set of requirements will apply.

A new set of requirements shall apply for a Type 2 implementations when the UE uses a single Rx chain for the reception of the fragmented carries.

Type 3 – External Rx path split:

The second way that the signal can be treated by separate Rx chains is a split at the front-end components by multiple external LNAs (eLNA). This can either be within the main/diversity module, or at an extension from the main/diversity module that allows connection to another receive pin at the transceiver. For Type 3 we would like to show a full picture and without discussion of detail whether front-end modules splits internally at the eLNA or if there’s a separate front-end module that accompanies the main/diversity modules to extend the connection possibilities towards the transceiver(s). Some solutions also have 2 transceivers that allow split of reception in NC IB CA between the two transceivers. The full system view is that the UE has two Rx chains active, and this allows the UE to center the LO1 and LO2 at the respective centers of each CC, so that the following filtering (channel filters) may attenuate also the in-gap interferer as shown here.

Fig. 6 Type 3 – External Rx path split. A single Rx chain after frontend filtering is marked with a dashed square.

If this UE is to transition to a single Rx chain state (Type 1) it will have to tolerate an in-gap interferer as shown in the next view of Type 3, that visualizes the “Extra Receiver chain” as toned-out colours:

Fig. 7 Type 3 – External Rx path split. Unused Rx chain is toned-out. Type 3 becomes Type 1.

In this state, the UE is in the same conditions as in Type 2, bringing both Type 3 and Type 2 into the state of Type 1 single Rx chain reception, so the relaxation of the requirements is the same for Type 2 and Type 3, and the same set of requirements can apply regardless of how the UE supports non-contiguous intra-band carrier aggregation.

For type 3 implementations the same new set of requirements as for type 2 implementations can apply.

As presented previously it has been shown that with both a Type 2 and a Type 3 transceiver implementation it is possible to separate the Rx chains (i.e. operate as Type 1). When separating or freeing up Rx chains in the Type 2 and Type implementations it is possible to apply these now idle Rx chains for reception of other carries if new RAN4 requirements for this type of operation is introduced to the specification.

RAN4 shall focus on Type 2 (Internal Rx path split) and Type 3 (External Rx path split) implementations when discussion reducing the number of Rx chains for fragmented carriers.

Use of “extra” Rx chain after reducing the number of Rx chains for fragmented carriers

To understand the motivation for reducing the number of Rx chains for fragmented carriers; the ambition is to “free up” UE’s receive chains to allow the UE to support higher order inter/intra-band carrier aggregation combinations using the freed-up resources.

A motivation for the SI is to “free up” Rx chains as these can then be used for additional CCs in a higher order inter/intra-band combination.

Some obvious limitation becomes notable from the details of the systems shown for Type 2 and Type 3, as presented in the previous section.

For Type 2, the spare Rx chain will be restricted in use for inter-band combinations that are allowed for other bands to connect towards the internal split inside the transceiver. The UE could have a dedicated non-contiguous intra-band carrier aggregation receiver (i.e. additional hardware resources) at the transceiver, or it could be able to share the receiver chains through more advanced multiplexers that route multiple transceiver receivers to multiple receiver chains. However, there will be limitations in these options to re-route the extra receiver chain, which may also have dependencies to the LO connections and limitations. Hence, there is a finite combination possibility from the extra receive chain.

For Type 3, the spare Rx chain will be restricted in use for inter-band combinations that can be routed from the same front-end module, which is stricter than the potential of the Type 2 architecture, which relies on LO and internal port connections among band-groups. The implementation of Type 3 is however not a lesser solution since it has more options in routing and integration of the RF front-end including regional cost optimization. Hence, again there is a finite combination possibility from the extra receive chain.

Therefore, the outcome of this study may not be a requirement for a UE to support all expected extensions in higher order inter-band combinations, but UE designs may use this feature to expand the inter-band combinations, provided they have appropriate requirements when operated in the single Rx chain state for non-contiguous intra-band carrier aggregation.

There are multiple limiting factors to the availability of Rx chains and thereby possible number of DL CCs. A solution for fragmented carriers may depend on the specific combinations of bands.

RAN4 shall focus on the fundamental issue of fragmented spectrum and solutions to reduce the needed Rx chains to receive these. However, the end goal of higher order inter/intra-band combinations shall be considered as a part of any solution.

Scheduling of the fragmented carriers within a single Rx chain

Currently, when a UE is supporting non-contiguous intra-band carrier aggregation (NC IB CA), the configuration consists of the two or more non-contiguous component carriers (CCs). This according to normal CA operation and example of current configurations in TS 38.101-1 is CA_n7(2A), CA_n41(4A) and CA_n77(2A).

Current DL non-contiguous intra-band carrier aggregation consists of at least two and up to four component carriers (CCs).

As seen in the examples from TS 38.101-1 for NC IB CA, it can be noted that up to four non-contiguous CCs are specified. For the SI on fragmented carriers, it is stated RAN4 shall consider a single DL band with a frequency span ≤ 100 MHz, containing two non-contiguous CCs within a CA combination for the inter-operator co-located scenario.

The SID limits the scope to two non-contiguous CCs within a CA combination.

From that it could be concluded that RAN4 are to consider each fragmented as a single CC, similar to NC IB CA. However, it may focus the further discussion for RAN4 to firmly agree this point. Being whether the different fragments shall be contained within separate CCs or a single CC can span all the fragments.

RAN4 shall discuss whether each fragment is treated as an individual CC or a single CC can cover the fragments.

It can be noted that the discussion on either single or multiple CCs have implications to base station schedular considerations, placement of recourse grid(s) and also the number of needed FFT procedures required by the UE. However, these aspects can be further discussed when there is clarification on proposal 5.

Additionally, it may be beneficial for RAN4 to keep in mind that some scenarios may result in more than two spectrum fragments and a solution developed by RAN4 would benefit for being able to also function in this scenario.

RAN4 can also consider solutions for fragmented spectrum with more than two spectrum fragments.

Duplex modes for fragmented carrier operation

Currently the examples presented in [2] are all for FDD bands and it is not explicitly mentioned in the SID [1] whether or not both FDD and TDD bands are to be considered.

The SID does not explicitly mention whether both FDD and TDD bands are under consideration for fragmented carrier operation.

It should be clarified in RAN4 whether a solution for fragmented carriers is to be applicable to both TDD and FDD bands, only to FDD, or whether there can be separate solutions for TDD and FDD fragmented carriers.

RAN4 shall clarify whether a solution for fragmented carriers is to be applicable for both TDD and FDD bands, only to FDD, or whether there can be separate solutions for TDD and FDD fragmented carriers.

Conclusion

In this contribution, we discuss the study item on fragmented carriers. The following Observations and Proposals were made:

Observation 1: Fragmented carriers are in some sense similar to non-contiguous carrier aggregation which is already specified for intra-band carrier aggregation in TS 38.101-1.

Observation 2: It is not always clear within RAN4 what it meant by “Rx Chain”, “Rx path”  or “Rx Branch”.

Observation 3: A single Rx chain for a 2Rx UE consists of one main Rx and one diversity Rx branch according to previously shared understandings from UE vendors.

Proposal 1: For a 2Rx UE, the common RAN4 understanding is that a single Rx chain consists of one main Rx branch and one diversity Rx branch.

Proposal 2: For a 4Rx UE, the common RAN4 understanding is that a single Rx chain consists of one main Rx branch and three diversity Rx branches.

Observation 4: For the current UE implementation, a separate UE RF chain per fragmented carrier is assumed.

Observation 5: Type 1 UE receiver does not have the ability to suppress/attenuate in-gap interference via analogue filtering.

Observation 6: Even though there may be a possibility to suppress some of the in-gap interference in the digital domain, the objective of the SI is to free up Rx resources hence the focus is on the analog Rx chains.

Observation 7: For the case that the UE uses a single Rx chain for the reception of the two fragmented carriers in the presence of an in-gap interferer, a new set of requirements must capture the conditions and the relaxation.

Observation 8: A new set of requirements shall apply for a Type 2 implementations when the UE uses a single Rx chain for the reception of the fragmented carries.

Observation 9: For type 3 implementations the same new set of requirements as for type 2 implementations can apply.

Proposal 3: RAN4 shall focus on Type 2 (Internal Rx path split) and Type 3 (External Rx path split) implementations when discussion reducing the number of Rx chains for fragmented carriers.

Observation 10: A motivation for the SI is to “free up” Rx chains as these can then be used for additional CCs in a higher order inter/intra-band combination.

Observation 11: There are multiple limiting factors to the availability of Rx chains and thereby possible number of DL CCs. A solution for fragmented carriers may depend on the specific combinations of bands.

Proposal 4: RAN4 shall focus on the fundamental issue of fragmented spectrum and solutions to reduce the needed Rx chains to receive these. However, the end goal of higher order inter/intra-band combinations shall be considered as a part of any solution.

Observation 12: Current DL non-contiguous intra-band carrier aggregation consists of at least two and up to four component carriers (CCs).

Observation 13: The SID limits the scope to two non-contiguous CCs within a CA combination.

Proposal 5: RAN4 shall discuss whether each fragment is treated as an individual CC or a single CC can cover the fragments.

Observation 14: RAN4 can also consider solutions for fragmented spectrum with more than two spectrum fragments.

Observation 15: The SID does not explicitly mention whether both FDD and TDD bands are under consideration for fragmented carrier operation.

Proposal 6: RAN4 shall clarify whether a solution for fragmented carriers is to be applicable for both TDD and FDD bands, only to FDD, or whether there can be separate solutions for TDD and FDD fragmented carriers.

- References

- RP-241669, New SID: Study on NR FR1 DL Fragmented Carriers, MediaTek Inc.

- RP-233374, Fragmented carriers in the DL, TELUS, Bell Mobility, T-Mobile USA, Telstra Limited, US Cellular Corporation, Nokia, Nokia Shanghai Bell

- RP-241269, On Fragmented Carriers in Rel-19, Apple

- RP-241361, Recommended WF on Fragmented Carriers, MediaTek