3GPP TSG-RAN WG4 Meeting # 112						R4-2413340

Maastricht Meeting, Aug 19th – Aug 23rd, 2024

Source:	Nokia

Title:	Discussion on impacts on UE RF requirements and DL performance for Fragmented Carriers

Agenda item:	8.6.3

Document for:	Discussion

Introduction

At RAN#104 a new study item [1] was agreed on addressing the issues raised by operators to optimize UE receiver conditions with fragmented carriers.

The objective of this study is as follows:

- Identify methods for reducing the number of UE Rx chains (e.g. from separate RF chains per CC to shared RF chains ) needed for single DL band with frequency span ≤ 100 MHz, containing two non-contiguous CCs within a CA combination for the inter-operator co-located scenario, considering:

-	Which RF requirements could be adjusted for the inter-operator co-located BS scenario, e.g. existing UE RF requirements such as ΔRIBNC, ACS and in-band blocking [RAN4];

- Other requirements are not precluded, if identified

-	The ability to semi-statically switch hardware resources (i.e. Rx chains) [RAN4, RAN2 – See note 2];

-	Up to 6 dB DL received power spectral density imbalance between the two non-contiguous CCs [RAN4];

-	Determine a reasonable level for the power spectral density difference between carriers of co-located adjacent channel operators for study [RAN4]

-	Impacts on DL performance [RAN4] ;

-	Means for a UE to inform the network of appropriate CA configuration it can support with adjusted RF requirements [RAN4, RAN2 – See note 2].

NOTE 1:	No RAN1 impact is foreseen

NOTE 2:	RAN2 work, if necessary, will be triggered by RAN4 LS

NOTE 3:	This study starts from single DL band. Sharing RF chain is not considered among inter-band DL carriers.

- Whether and how to apply the conclusions to intra-band component of inter-band CA will be decided after the study on 2CC in a single DL band is completed.

This contribution discusses the impacts on UE requirements and DL performance for Fragmented Carriers.

Discussion

Existing requirements

Fragmented carriers is not something completely new for RAN4, since it is essentially non-contiguous intra-band carrier aggregation (NC IB CA). However, adding a specific name makes sense to associate the non-contiguous intra-band carrier aggregation with the request for UEs to support downlink reception of the fragmented spectrum block with a single Rx chain as defined in R4-2413339.

Fragmented carriers are in some sense similar to non-contiguous intra-band carrier aggregation (NC IB CA) which is already supported by the specification.

The reception at the UE of non-contiguous intra-band carrier aggregation in a system with a single Rx chain will be impacted by two sources of interference.

The first source of interference is the impact from the UE’s own UL transmission, which is reflected in the Reference sensitivity (REFSENS) requirements for non-contiguous intra-band carrier aggregation that allow an MSD in the sense of ΔRIBNC as specified in TS 38.101-1 clause 7.3A.2.2. This is however only present for FDD carriers and considers the placement of the secondary component carrier (SCC) at a distance less than the normal duplexing distance of the band of operation. Although there are existing requirements, they are different from what is needed for this SI. Hence, something similar may be used as a new requirement for fragmented carriers but it must be separated from the existing FDD non-contiguous intra-band carrier aggregation ΔRIBNC.

New requirement for fragmented carriers shall be separated from the existing FDD non-contiguous intra-band carrier aggregation ΔRIBNC.

The second source of interference is the impact an adjacent channel interference may have on the reception of the two fragmented carriers, which is reflected in the ACS requirements for non-contiguous intra-band carrier aggregation. This already today mandates a UE to include the potential in-gap interference as written in TS 38.101-1 Clause 7.5A.2:

“The UE shall fulfil the minimum requirements all values of a single adjacent channel interferer in-gap and out-of-gap up to a –25 dBm interferer power while all downlink carriers are active”.

Current specification already has a requirement to the UE that it shall fulfil the requirements with an in-gap interferer of up to -25dBm.

It can be noted that the in-gap interference is not tested in the existing ACS test specifications to our understanding. This may need to be addresses within this SI.

Testing of any new requirements for fragmented carriers may need to be addresses within this SI.

The existing requirement of being able to fulfil requirements with a –25 dBm in-gap interferer is based on the assumption that multiple Rx chains can be applied with potential attenuation/suppression of the interferer. This additional attenuation/suppression of the interferer is not possible when a single Rx chain has to cover the fragmented carriers.

A main challenge of non-contiguous intra-band carrier aggregation with a single Rx chain is the additional sources of interference since the single Rx chain has to cover multiple component carriers (CCs) with a potential in-gap interferer.

Requirements for fragmented carriers

As described in the previous section, there would be a need to address at least REFSENS and ACS requirements related to operation with fragmented carriers and a single UE Rx chain.

At least REFSENS and ACS requirements shall be discussed in relation to fragmented carriers in DL.

For REFSENS, it is expected that some relaxation may be introduced as compared to the current requirements. These relaxations would mainly be justified by the widening of the total transmission bandwidth to cover both of the two spectrum fragments as compared to a scenario with two narrower filters.

REFSENS relaxation for fragmented carrier operation can be related to the widening of the total transmission bandwidth.

The wider transmission bandwidth would need a larger guard band, as further discussed in section 2.6, as there is a higher attenuation of the RBs at the outermost edges of the CCs and thus a degradation of the sensitivity. Additionally, suppression of on Tx noise may be lower in a FDD scenario, requiring additional relaxation.

The sensitivity degradation related to the widening of the transmission bandwidth is dependent on specific UE implementations, why input from UE vendors is needed to progress this discussion.

RAN4 encourage UE vendors to supply information on expected sensitivity degradation related to widening of the transmission bandwidth for fragmented carrier operation.

For ACS, there is a need to split the discussion into the selectivity towards a channel adjacent to the fragmented carrier combination (i.e. out-of-gap interference) and the selectivity within the gab of the fragmented carrier (i.e. in-gap interference). There should be no need for relaxation outside of the fragmented carrier combination. On the other hand, some relaxation should be expected within the fragmented carrier configuration since there is no possibility for the single Rx chains filters to suppress the in-gap interferer. This difference is illustrated in Fig 1.

Fig. 1 ACS comparison between single carrier and fragmented carrier operation.

In a TDD scenario the ACS attenuation is illustrated on the left-hand side of Fig. 1, where the analogue filter serves to suppress the adjacent channel interfering signal from saturating the ADC of the receiver. At the right-hand side of Fig. 1, the two CCs (wanted signals) are received with an in-gap interfering signal using 1 Rx chain with an analogue filter bandwidth that covers both carriers and the full in-gap interfering signal with no attenuation. The attenuation would have been provided at the level on the left of Fig. 1, which means the relaxation of the ACS requirement at the right of Fig. 1 should be equal to the ACS attenuation of the left of Fig 1.

To further explain the impact of the analogue filter an example, in this case a low pass filter, is presented in Fig. 1.

Fig. 2 Example of analogue channel filter attenuation

The example in Fig. 2 presents around 15dB of attenuation to the adjacent channel of a 5MHz wanted signal and the presence of a 5MHz adjacent channel interferer. Noting that the current ACS requirements for non-contiguous intra-band carrier aggregation is for both in-gap and out-of-gap is –25 dBm interferer power it is, based on this example, a significant of that which may be provided by the analogue filters.

The current ACS requirements for non-contiguous intra-band carrier aggregation may depend on the presence of analogue filters which are not available for fragmented carrier operation.

The difference in attenuation/suppression of in-gap interferes between current implementations and expected implementation or fragmented carries can be used to develop the in-gap ACS requirement. The difference in attenuation to is heavily impacted by multiple factors related to both the specific scenario of fragmented carriers and the UE implementation. Therefore, to form a basis for discussion of potential ACS relaxations input from UE vendors are welcomed.

RAN4 shall seek input from UE vendors on expected difference in interference attenuation with current implementation and fragmented carrier operation.

PSD imbalance of the fragmented carriers

In [3] it is discussed whether or not the power imbalance of the wanted carriers within the fragmented carriers shall be considered. While this is a valid point that there may be an imbalance of the power of the wanted carriers, the current SI limits the scenario to be co-located via the maximum of 6dB power spectral density (PSD) imbalance requirement/prerequisite.

It is already given by the SID that a PSD imbalance of up to 6dB between the wanted carriers within the fragmented carriers shall be accepted.

This means that the imbalance of up to 6dB is to be considered with any relaxation RAN4 may allow to enable support of reception of fragmented carriers with a single receiver.

If power imbalance of the wanted carriers within the fragmented carriers is to be considered it means that the level of signal-to-in-gap interference ratio will always be towards the CC of lowest PSD. For that reason, the imbalance should not be treated as an aspect of fragmented CA requirements but treated similarly as it would for all other test cases, even such as the NC IB CA FDD REFSENS test that also does not include aspects of power imbalance of the wanted component carriers. Hence, we believe that aspects of power imbalance should not be a part of this study for relaxation requirements for fragmented carrier operation.

RAN4 shall not consider power imbalance of the wanted carriers further in relation to the SI on fragmented carriers.

Expected interference power level

When referring to Fig. 1 the in-gap interference is illustrated between the wanted fragmented carriers. The power level of the interferer is in the presented example significantly larger than the wanted signals mostly for illustration purpose. However, what power level of the in-gap interference a UE can tolerate also needs to be addressed by RAN4. A starting point for the discussion could be the current ACS or IBB requirements.

RAN4 shall consider what level of in-gap interference fragmented carrier operation shall be able to tolerate.

Additionally, RAN4 needs to consider whether the power level of the in-gap interferer needs to be used as a determining factor for the UE to report support of fragmented carrier operation and thereby semi-statically switch hardware resources to enable/free-up Rx chains. If so, the procedure for measuring this power level may need involvement of RAN4 RRM experts.

RAN4 can consider if the power level of the in-gap interferer shall be used for enabling or disabling fragmented carrier operation.

Regardless of the above, RAN4 needs to consider how the UE is expected to behave if an in-gap interferer precludes the UE to operate in fragmented carrier mode. Is the UE expected to choose one of the fragments or simply drop the configuration completely? This UE fallback behaviour needs to be known by the network, maybe even controlled by the network to facilitate scheduling.

RAN4 shall discuss expected UE fallback behaviour when an in-gap interferer precludes the UE to operate in fragmented carrier mode.

Means for a UE to inform the network of CA configuration it can support with adjusted RF requirements

As mentioned multiple times, fragmented carrier operation has similarities to NC IB CA for which support is mainly indicated via the supported band combination list provided to the network from the UE. Examples of NC IB CA combinations could be CA_n7(2A), CA_n41(4A) and CA_n77(2A) where the non-contiguous part is indicated in the parentheses with the number for the number of CCs and where the letter indicates their size/type.

RAN4 specification already have an agreed syntax for indicating non-contiguous parts of a band combination.

It can be suggested that RAN4 introduces a new syntax for indicating band combinations supported also with fragmented carrier operation like CA_n7(2A1) where the 2nd number indicates the tolerated in-gap interferer. By indicating the support of fragmented carrier operation directly in the band combination (i.e. CA configurations), the new type of band combinations would have to be introduced in a similar manner as now done for NC IB CA where each case is assessed individually for needed potential relaxations. Additionally, it would be possible to list all supported band combinations in the specification and a UE would be able to indicate only those which it supports to the network.

RAN4 can consider a new syntax for indicating support of fragmented carrier operation in a CA configuration

With this it would be up for further discussion whether there is a need for other signalling introduced by RAN2 together with a new UE capability. RAN2 will not introduce the new capability before a WI phase for fragmented carriers but they may be needed to provide input on other signalling aspects during the SI phase.

RAN4 needs to further discuss whether to ask RAN2 on input on signalling to support the feature at a later stage of the SI.

DL performance impact

The starting point for introducing a new CA configuration type, in this case fragmented carrier operation, should be that there should be no performance degradation. By that meant, that in the absence of in-gap interference, the performance should be similar to current CA configurations and when there is a strong in-gap interferer, the performance should not be worse than the performance with a single carrier.

There should be no performance degradation when introducing a new CA configuration type, in this case fragmented carrier operation.

This means that aspects as throughput and spectrum utilization should be considered. In relation to this a discussion needed in RAN4 is how the Maximum transmission bandwidth and related Minimum guardbands should be defined when operating in fragmented carrier mode. By this meant whether the total bandwidth, including the gap(s), is used as the baseline or it is the sum of the wanted signals or something different.

RAN4 shall discuss what is the basis for Maximum transmission bandwidth and related Minimum guardbands when operating in fragmented carrier mode.

This has an impact on available PRBs and hence the expected DL performance.

Conclusion

In this contribution, we discuss the study item on fragmented carriers. The following Observations and Proposals were made:

Observation 1: Fragmented carriers are in some sense similar to non-contiguous intra-band carrier aggregation (NC IB CA) which is already supported by the specification.

Proposal 1: New requirement for fragmented carriers shall be separated from the existing FDD non-contiguous intra-band carrier aggregation ΔRIBNC.

Observation 2: Current specification already has a requirement to the UE that it shall fulfil the requirements with an in-gap interferer of up to -25dBm.

Observation 3: Testing of any new requirements for fragmented carriers may need to be addresses within this SI.

Observation 4: A main challenge of non-contiguous intra-band carrier aggregation with a single Rx chain is the additional sources of interference since the single Rx chain has to cover multiple component carriers (CCs) with a potential in-gap interferer.

Proposal 2: At least REFSENS and ACS requirements shall be discussed in relation to fragmented carriers in DL.

Observation 5: REFSENS relaxation for fragmented carrier operation can be related to the widening of the total transmission bandwidth.

Proposal 3: RAN4 encourage UE vendors to supply information on expected sensitivity degradation related to widening of the transmission bandwidth for fragmented carrier operation.

Observation 6: The current ACS requirements for non-contiguous intra-band carrier aggregation may depend on the presence of analogue filters which are not available for fragmented carrier operation.

Proposal 4: RAN4 shall seek input from UE vendors on expected difference in interference attenuation with current implementation and fragmented carrier operation.

Observation 7: It is already given by the SID that a PSD imbalance of up to 6dB between the wanted carriers within the fragmented carriers shall be accepted.

Proposal 5: RAN4 shall not consider power imbalance of the wanted carriers further in relation to the SI on fragmented carriers.

Proposal 6: RAN4 shall consider what level of in-gap interference fragmented carrier operation shall be able to tolerate.

Observation 8: RAN4 can consider if the power level of the in-gap interferer shall be used for enabling or disabling fragmented carrier operation.

Proposal 7: RAN4 shall discuss expected UE fallback behaviour when an in-gap interferer precludes the UE to operate in fragmented carrier mode.

Observation 9: RAN4 specification already have an agreed syntax for indicating non-contiguous parts of a band combination.

Proposal 8: RAN4 can consider a new syntax for indicating support of fragmented carrier operation in a CA configuration

Observation 10: RAN4 needs to further discuss whether to ask RAN2 on input on signalling to support the feature at a later stage of the SI.

Observation 11: There should be no performance degradation when introducing a new CA configuration type, in this case fragmented carrier operation.

Proposal 9: RAN4 shall discuss what is the basis for Maximum transmission bandwidth and related Minimum guardbands when operating in fragmented carrier mode.

- References

- RP-241669, New SID: Study on NR FR1 DL Fragmented Carriers, MediaTek Inc.

- RP-233374, Fragmented carriers in the DL, TELUS, Bell Mobility, T-Mobile USA, Telstra Limited, US Cellular Corporation, Nokia, Nokia Shanghai Bell

- RP-241269, On Fragmented Carriers in Rel-19, Apple

- RP-241361, Recommended WF on Fragmented Carriers, MediaTek