3GPP TSG-RAN WG4 Meeting # 112						R4-2413350

Maastricht Meeting, Aug. 19th – Aug 23rd, 2024

Title:	TP to TR 38.718-03-01 Addition of CA_n3A-n5A-n8A

Source:	Nokia, Vodafone

Agenda item:	7.3.4

Document for:	Approval

# 1	Introduction

This is a TP to TR 38.718-03-01 to add CA_n3A-n5A-n8A.

************************************* Start of TP*****************************************

## 5.x	CA_n3-n5-n8

### 5.x.1	Common for 1 band UL and 2 bands UL CA

#### 5.x.1.1	Operating bands for CA

Table 5.x.1.1-1: CA band combination constituent bands definition

| NR CA Band | NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n3-n5-n8 | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |

#### 5.x.1.2	Channel bandwidths per operating band for CA

Table 5.x.1.2-1: Supported bandwidths per CA band combination of band n3-n5-n8

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n3A-n5A-n8A | - | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n8 | 5, 10, 15, 20 |  |

#### 5.x.1.3	∆TIB,c and ∆RIB,c values

For CA_n1-n5-n8, since there is only single carrier UL the following TIB,c  values are proposed:

Table 5.x.1.3-1: ΔTIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)8 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |  |
| CA_n3-n5-n8 | - | - | - |
| NOTE 8: “-” denotes ΔTIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

The RIB,c values are proposed to be defined following CA_n5-n8 as in the following table

Table 5.x.1.3-2: ΔRIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)9 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |  |
| CA_n3-n5-n8 | - | 0.4 | 0.4 |
| NOTE 9: “-” denotes ΔRIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n8 the band order from left to right is n1, n3 and n8. |  |  |  |

#### 5.x.2 	Specific for 2 bands UL CA

#### 5.x.2.1	UE co-existence studies

Since CA_n3-n5, CA_n3-n8 and CA_n5-n8 are the fallback combination of CA_n3-n5-n8, and the studies for the corresponding MSD have been covered by these fallback combinations.

************************************* End of TP******************************************