3GPP TSG-RAN4 Meeting #112	   											R4-2413352

# Maastricht, NL, 19th of August 2024 – 23rd of August 2024

Title: 	Discussion on FR1-NTN High Power UE (HPUE)

Agenda Item:	8.8.2.1

Source: 	THALES, Magister Solutions Ltd

Document for:	Discussion

# For information

As described in RP-240857 (New WID: Enhanced requirements and test methodology for NR and IoT NTN), until Rel-18, the 3GPP specifications limit maximum non-terrestrial network (NTN) UE transmission (Tx) power to the level of 23dBm, which may not allow to use the full potential of satellite communication. On the other hand, power class 2 (PC2 26dBm) and power class 1.5 (PC1.5 29dBm for TDD only) are widely supported for terrestrial network (TN), and power class 1 (PC1 31dBm) is also supported for particular device for public safety.

When compared to the existing TN deployments, NTN becomes critical in adverse conditions such as severe weather events or natural disasters, where or when the terrestrial deployments are not present or temporally unavailable. However, satellite communication involves transmitting signals over long distances, which results in larger path loss. Thus increasing the UE maximum transmit power will significantly improve UL performance in terms of coverage, availability and throughput, which would be needed to meet the above market demands associated with fixed wireless, public safety and automotive usage.

HPUE for NTN is feasible. For a number of NTN use cases, including automotive, maritime, logistics, outdoor industrial monitoring and other scenarios not reliant on battery power, such that the higher power requirements of PC2, PC1.5 and PC1 are not with a battery power issue. Regarding the regulation, the existing regulation and specifications used by the current satellite services (primarily MSS), such as ECC/CEPT and FCC, typically allow significantly higher UE transmit powers even for handheld (e.g. satellite phones and trackers, which are different from smartphones) and automotive UE, and operate in narrow-band spectrum. Therefore, regulation is not expected to be a barrier. To address the peak spatial-average specific absorption rate (SAR) restriction for some UE (e.g. smartphones) the proper solution similar to TN HPUE would be needed.

Both NR-NTN and IoT-NTN need be included since the techniques share the commonality for them.

The objective of the core part WI for enhanced requirements related to High Power UE (HPUE) for NTN was therefore described as:

- Conduct the coexistence analysis based on TR 38.863 for the above example bands and corresponding power classes

  - Evaluate the impact on the uplink performance of TN (Terrestrial network)

    - Example bands include bands n256 and 256, n255 and 255

    - PC2, PC1.5 (for NR-NTN only) and PC1 will be evaluated for ACLR and ACS

    - For NR-NTN and IoT-NTN: coexistence studies shall be carried out under the same conditions, including geographical separation between NTN UE and Terrestrial Networks (TN), as the co-existence studies in Rel-17 and Rel-18

- Specify high power UE (HPUE) for NR-NTN (Non-Terrestrial Networks) and IoT-NTN (NB-IoT and eMTC based NTN) in FR1-NTN bands and the corresponding LTE NTN bands for the single uplink (UL) carrier scenario

  - Support UE with power class 2 (PC2, 26 dBm) that applies to both handheld and non-handheld based on the co-existence study.

  - Support other power classes based on the co-existence and feasibility study

    - Non-hand held UE is prioritized

  - Specify RF requirements for all considered power classes for NR-NTN and IoT-NTN in FR1-NTN bands and the corresponding LTE NTN bands

    - Necessary Tx requirements, at least including

      - Investigate and if necessary, specify the solution to address potential SAR (Specific Absorption Rate restriction) requirements for handheld UE

      - Specify MPR and A-MPR for the example bands considering the regulation for the corresponding bands, if necessary

    - Necessary Rx requirements, at least including

      - Specify the RSD requirements on the example bands, if necessary

- Specify the necessary signalling to support the above objectives

However, it is also important to mention that FR1-NTN coexistence studies have been intensively studied as part of Rel-17 and therefore Rel-19 work could reuse results and related methodology from Rel-17.

# Recall coexistence scenarios from Rel-17

The coexistence scenarios from Rel-17 could be found in the Table and Figures below.

Table 1: Aggressor and victim

| No. | Combination | Aggressor | Victim | Notes | Study Phase |
| --- | --- | --- | --- | --- | --- |
| 1 | TN with NTN | TN DL | NTN DL | Applicable for satellite operating in e.g. S-band, for e.g. coexistence with n1 FDD. | Phase 1 |
| 2 | TN with NTN | TN UL | NTN UL | Applicable for satellite operating in e.g. S-band, for e.g. coexistence with n1 FDD. | Phase 1 |
| 3 | TN with NTN | NTN DL | TN DL | Applicable for satellite operating in e.g. S-band, for e.g. coexistence with n1 FDD. | Phase 1 |
| 4 | TN with NTN | NTN UL | TN UL | Applicable for satellite operating in e.g. S-band, for e.g. coexistence with n1 FDD. | Phase 1 |
| 5 | TN with NTN | NTN UL | TN DL | Applicable for satellite operating in S-band, for e.g. coexistence with n34 TDD. | Phase 1 |
| 6 | TN with NTN | TN DL | NTN UL | Applicable for satellite operating in S-band, for e.g. coexistence with n34 TDD. | Phase 1 |
| 7 | NTN with NTN | NTN DL | NTN DL | HAPS-HAPS | Phase 2 |
|  |  | NTN UL | NTN UL | HAPS-HAPS | Phase 2 |

For further visualisation, the interference charts with respect to scenarios 1 to 6 are further represented with respect to NTN-TN coexistence scenarios in Figure 1 (scenarios 1 to 4) and Figure 2 (scenarios 5 and 6).

Figure 1: S-band NTN-TN adjacent band coexistence scenarios with TN in FDD mode

Figure 2: S-band NTN-TN adjacent band coexistence scenarios with TN in TDD mode (e.g. n34)

It can be easily noticed that the impacted coexistence scenarios by HPUE studies are only the Scenario 4 and the Scenario 5.

Proposal 1: RAN4 to de-scope HPUE related coexistence scenarios only to Scenario 4 and Scenario 5.

# Simulation Results

Under exact the same conditions, including geographical separation between NTN UE and Terrestrial Networks (TN), NTN-TN coexistence simulations have been performed for HPUE with 31dBm transmission power corresponding to Power Class 1 (which is the worst case compared to 26dBm and 29dBm transmission power corresponding to Power Class 2 and Power Class 1.5).

Simulations have been further performed only based on Scenario 4 and 5 with the following conclusions:

1/ Scenario 4 – 23dBm vs 31dBm UE Tx power

LEO@600km

LEO@1200km

GEO

Observation 1: For Scenario 4, throughput loss increases from 0.035% to 0.20% with 12 dB ACIR from 23 dBm to 31 dBm UE.

Observation 2: For Scenario 4, throughput loss is 0 at 30 dB ACIR for 31 dBm UE.

Observation 3: Increased UE Transmission Power (TxP) has no co-existence impact in Scenario 4 with studied system parameters as per WID description.

2/ Scenario 5 – 23dBm vs 31dBm UE Tx power

LEO@600km

LEO@1200km

GEO

Observation 4: For Scenario 5, throughput loss increases from 0.07% to 0.14% with 12 dB ACIR from 23 dBm to 31 dBm UE.

Observation 5: For Scenario 5, throughput loss is 0.02% at 30 dB ACIR for 31 dBm UE.

Observation 6: Increased UE Transmission Power (TxP) has no co-existence impact in Scenario 5 with studied system parameters as per WID description.

Proposal 2: From adjacent channel interference impact point of view for HPUE, under WID assumption, there are no problems identified even with 31dBm NTN HPUE (31dBm NTN UL Transmission Power).

Proposal 3: Rel-19 NTN HPUE to reuse Rel-17 NTN UE ACLR for all power classes up to 31dBm (PC1).

Proposal 4: Since no impact on Rx side with respect to Rel-17 according to coexistence analysis, Rel-19 NTN HPUE to reuse Rel-17 NTN UE ACS.

# Conclusions

Proposal 1: RAN4 to de-scope HPUE related coexistence scenarios only to Scenario 4 and Scenario 5.

Observation 1: For Scenario 4, throughput loss increases from 0.035% to 0.20% with 12 dB ACIR from 23 dBm to 31 dBm UE.

Observation 2: For Scenario 4, throughput loss is 0 at 30 dB ACIR for 31 dBm UE.

Observation 3: Increased UE Transmission Power (TxP) has no co-existence impact in Scenario 4 with studied system parameters as per WID description.

Observation 4: For Scenario 5, throughput loss increases from 0.07% to 0.14% with 12 dB ACIR from 23 dBm to 31 dBm UE.

Observation 5: For Scenario 5, throughput loss is 0.02% at 30 dB ACIR for 31 dBm UE.

Observation 6: Increased UE Transmission Power (TxP) has no co-existence impact in Scenario 5 with studied system parameters as per WID description.

Proposal 2: From adjacent channel interference impact point of view for HPUE, under WID assumption, there are no problems identified even with 31dBm NTN HPUE (31dBm NTN UL Transmission Power).

Proposal 3: Rel-19 NTN HPUE to reuse Rel-17 NTN UE ACLR for all power classes up to 31dBm (PC1).

Proposal 4: Since no impact on Rx side with respect to Rel-17 according to coexistence analysis, Rel-19 NTN HPUE to reuse Rel-17 NTN UE ACS.

# Annex – basic simulation assumptions for both Scenarios 4 & 5

- UE Tx Power: 23, 26, 29, 31 dBm

- Frequency band : S-band with 20 MHz DL and UL bands for both networks

- TN network of 7 sites with 3 cells per site

- 10 TN UEs deployed per cell in the cell range

- 50 NTN UEs deployed on the edge of TN; isolation distance of 2*ISD in Urban scenarios

- NTN LEO@600km/LEO@1200km/GEO satellite on top of TN

- Full buffer traffic model

- Scenarios: Urban or Rural