3GPP TSG-RAN WG4 Meeting #112									                        R4-2413357

Maastricht, Netherlands, August 19 – 23, 2024

Agenda item:			5.9.1

Source:	Ericsson

Title:	(NR_ENDC_RF_FR1_enh2-Core) On DeltaP_PowerClass for SRS AS for 4Tx

Document for:	Approval

# Introduction

In RAN4#110, an issue was raised in [1] that ΔPPowerClass requirement for antenna switching (AS) is incomplete for 4Tx case. The issue has been discussed for three meetings, but it was not possible to reach an agreement between the companies. Since the WI on “Further RF requirements enhancement for NR and EN-DC in frequency range 1 (FR1)” is now closed, it was decided that the issue will be discussed further under the maintenance agenda for UE RF enhancement for Rel-18. In this contribution, we propose a solution which provides a good balance between the simplicity and the network performance.

# Discussion

## ΔPPowerClass requirement for SRS AS for 2Tx

ΔPPowerClass requirement definition has been extended in Rel-17 (and updated in Rel-18 to account for 8Rx case) to apply for SRS transmission occasions with the usage in SRS-ResourceSet set as ‘antennaSwitching’. The main motivation was to prevent UEs which indicate PC1.5 or PC2 together with txDiversity-r16 to virtualize the SRS ports by using 2 Tx chains. The following is an excerpt from TS-38.101-1 V18.5.0 [2]:

- 3dB is applied during SRS transmission occasions with usage in SRS-ResourceSet set as ‘antennaSwitching’ with configured SRS resources in each SRS resource set(s) consisting of one SRS port when PC2 UE with txDiversity-r16 or txDiversity2Tx-r18 capability or PC1.5 UE further indicates SRS-TxSwitch capability ‘t1r2’ or ‘t1r4’ or ‘t1r1-t1r2’ or ‘t1r1-t1r2-t1r4’ or further indicates srs-AntennaSwitchingBeyond4RX-r17 as ‘t1r8’;

If such requirement didn’t exist, e.g. the PC2 UE indicating txDiversity-r16 capability would be assumed to use two half-power PAs of 23dBm to achieve 26dBm by virtualization even for ‘t1ry’ (y=2,4,8) indicated AS capability, which shall not be allowed. Even if such UE were equipped with one PA of 26dBm, the network couldn’t have known it and it could only assume the UE behavior based on its reported capability. By applying ΔPPowerClass = 3dB such uncertainty at the gNB is avoided, where the improvement in accuracy of channel estimation (by avoiding such uncertainty) may overcome the drawback of “unnecessarily” capping the SRS transmission power to 23dBm for UE implementations where 26dBm could be achieved with one PA. An obvious work-around for such UEs, equipped with a PA of 26dBm which is used for sounding of all SRS ports, is not to indicate txDiversity-r16 or txDiversity2Tx-r18 capability (which would make ΔPPowerClass = 0 dB).

We would like to note that the antenna virtualization is not completely prevented even by this solution as e.g. nothing prevents the PC3 UE to achieve 23dBm by setting the power of the two PAs to 20 dBm + 20 dBm (using the power back-off at both PAs). Still, the probability for the antenna virtualization occurring significantly reduces. The main benefit of the present solution is that it prevents the virtualization of SRS ports for the range of the allowed maximum configured output power settings where the UE equipped uniquely with the PC3 PAs would be forced to do it (by reducing the power), for example for PC2 UE indicating Tx Diversity in the range between 23dBm and 26dBm.

Observation 1: ΔPPowerClass requirement definition has been extended in Rel-17 to apply for SRS transmission occasions with usage in SRS-ResourceSet set as ‘antennaSwitching’, where the main motivation was to prevent UEs which indicate PC1.5 or PC2 together with txDiversity-r16 to virtualize the SRS ports by using 2 Tx chains.

Observation 2: The main benefit of the present solution is that it prevents the virtualization of SRS ports for the range of the allowed maximum configured output power settings where the UE equipped uniquely with the PC3 PAs would be forced to do it (by reducing the power), for example for PC2 UE indicating Tx Diversity in the range between 23 dBm and 26 dBm.

On the other hand, motivated by a very particular case where a UE is equipped with both a PA of 26 dBm and a PA of 23 dBm and where the UE does not indicate txDiversity-r16 or txDiversity2Tx-r18 capability and where some SRS ports are sounded with 26dBm PA and others are sounded with 23dBm PA, ΔTRxSRS requirement was further relaxed by 3dB as it can be seen in the following excerpt from TS-38.101-1 [2] for e.g. ‘t1r2’ AS capability:

- The value of ∆TRxSRS is 4.5dB for bands whose FUL_high is higher than the FUL_low of n79 and 3 dB for bands whose FUL_high is lower than the FUL_low of n79 when the device is power class 3 or power class 5 or power class 1.5 in the band, or when the device is power class 2 in the band and ΔPPowerClass = 3 dB, or when UE indicating Tx diversity capability. - The value of ∆TRxSRS is 7.5dB for bands whose FUL_high is higher than the FUL_low of n79 and 6 dB for bands whose FUL_high is lower than the FUL_low of n79 during SRS transmission occasions with configured SRS resources consisting of one SRS port when the device is power class 2 in the band and ΔPPowerClass = 0 dB and not indicating Tx diversity capability.

The problem with combining ΔTRxSRS and ΔPPowerClass is that additional 3dB relaxation of ΔTRxSRS is also allowed for UEs equipped uniquely with PAs of 26dBm or for UEs equipped with both 23 dBm and 26 dBm PAs and where only the PA of 26dBm is used for SRS sounding. ΔTRxSRS is a maximum allowed relaxation due to the insertion loss where the actual relaxation can have any value between 0 dB and the value of ΔTRxSRS. For that reason, the uncertainty at gNB remains (and even increases), so there is a limited benefit from increasing the power of SRS transmission if the network cannot know which power was used (since the purpose of ‘antennaSwitching’ usage of SRS is the accurate channel estimation).

Observation 3: The introduction of additional 3dB relaxation of ΔTRxSRS for PC2 UEs not indicating the Tx Diversity was motivated by the case of a UE sounding some SRS ports with 26 dBm while other with 23 dBm. Due to the nature of ΔTRxSRS, which is a maximum allowed relaxation, UEs equipped uniquely with PAs of 26 dBm or with both 23 dBm and 26 dBm PAs and where only the PA of 26 dBm is used for SRS sounding could also apply the relaxed values of ΔTRxSRS which should not be allowed.

Proposal 1: To address the power limitation issue for SRS transmissions for AS for UEs equipped with 4 Tx chains ΔPPowerClass should be used, just as in 2Tx case. Avoid combining ΔPPowerClass and ΔTRxSRS requirements since those two have a different nature and introduce uncertainty at the gNB and thus degrade the channel estimation accuracy.

## Proposed solution for SRS AS for 4Tx

Similar to 2Tx case where ΔPPowerClass requirement definition was extended in Rel-17 to apply for SRS AS, we propose a simple solution for 4Tx case which solely relies on ΔPPowerClass requirement, and which takes into account the important points raised in the previous clause.

We propose that different values should be applicable for ΔPPowerClass as a function of the indicated AS capability:

- For ‘t4ry’ AS capability, ΔPPowerClass = 0 dB.

- For ‘t2ry’ and similar AS capabilities, ΔPPowerClass = 3 dB.

- For ‘t1ry’ and similar AS capabilities, ΔPPowerClass = 6 dB.

With this solution the uncertainty for the network on SRS transmission power being used is minimized. In our view the fact that e.g. the UE equipped with at least one PA of 26 dBm indicating ‘t1ry’ AS capability cannot sound an SRS port with 26 dBm power is not a huge drawback since there would be no way for the network to know the UE’s power setting and thus the accuracy of the channel estimation would be negatively affected. Moreover, the absence of knowledge of the increased SRS power being used for AS may introduce performance degradation rather than a gain.

Observation 4: The absence of knowledge of the increased SRS power being used for AS may introduce performance degradation rather than a gain.

Proposal 2: We propose that different values should be applicable for ΔPPowerClass as a function of the indicated AS capability:

- For ‘t4ry’ AS capability, ΔPPowerClass = 0 dB.

- For ‘t2ry’ and similar AS capabilities, ΔPPowerClass = 3 dB.

- For ‘t1ry’ and similar AS capabilities, ΔPPowerClass = 6 dB.

In previous meetings, there have been some proposals not to apply ΔPPowerClass to the upper bound of the configured transmitted power inequality PCMAX_H,f,c. Similar to 2Tx case, for ‘t1ry’ and similar AS capabilities, if ΔPPowerClass didn’t figure in the upper bound of the configured transmitted power inequality PCMAX_H,f,c, the UE indicating PC1.5 would be forced to use antenna virtualization for the range of the allowed configured transmitted power settings near PCMAX_H,f,c. On one hand, a UE equipped with 4 x 23 dBm PAs and indicating PC1.5 would be forced to use antenna virtualization for any configured transmitted power in the range between 23 dBm and 29 dBm. On another hand, a UE equipped with 4 x 26 dBm PAs and indicating PC1.5 would be forced to use antenna virtualization for any configured transmitted power in the range between 26 dBm and 29 dBm. For UEs with any other PA configuration (n x 23 dBm + (4-n) x 26 dBm PAs, 0 < n < 4), depending on the UE implementation and the PA tied to the given SRS antenna port, the same range is between [23 or 26dBm] and 29dBm. This means that for all possible PA configurations at a 4Tx UE indicating ‘t1ry’ or “similar” AS capabilities, there is a 3 to 6 dB range of the allowed configured transmitted output power where the UE would be forced (obliged) to use the antenna virtualization to achieve it. Similarly, for ‘t2ry’ and “similar” AS capabilities, there is a 0 to 3 dB range (depending on PA configuration) where the UE would be forced to use the antenna virtualization to achieve the configured maximum output power.

Since the antenna virtualization is strictly forbidden for SRS AS, the same as it was discussed already in previous Releases, ΔPPowerClass shall not be removed from the upper bound of the configured transmitted power inequality PCMAX_H,f,c for 2Tx UEs. For 4Tx UEs, ΔPPowerClass should be introduced to the upper bound of the configured transmitted power inequality PCMAX_H,f,c. The values 3 dB and 6 dB for ‘t2ry’ and ‘t1ry’ AS capabilities respectively in Proposal 2 are chosen such that all possible PA configurations at a UE are covered and so that a UE is never forced to perform the antenna virtualization. Any other solution which would allow a UE to optionally reduce its configured maximum output power (depending on its PA configuration) would only introduce the uncertainty to the network and thus a potential degradation of the channel estimation accuracy.

Observation 5: If ΔPPowerClass weren’t introduced to PCMAX_H,f,c, for all possible PA configurations at a 4Tx UE there would be a range of the allowed configured transmitted output power settings where a UE would be forced (obliged) to use the antenna virtualization to achieve it.

Extending ΔPPowerClass definition in the specifications in this way leads however to the scenario where for a PC1.5 UE and “t1ry or similar” indicated AS capability, the applicable value of ΔPPowerClass depends on the number of equipped Tx chains by the UE. The issue is that the gNB cannot always know the number of Tx chains the UE is equipped with and thus whether the UE applies ΔPPowerClass of 3 dB (2 Tx chains) or 6 dB (4 Tx chains). That information can be derived from some reported UE capabilities, e.g. maxNumberMIMO-LayersCB-PUSCH, maxNumberMIMO-LayersNonCB-PUSCH, txDiversity2Tx-r18, txDiversity4Tx-r18 etc. but since those capabilities are not mandatory there is no guarantee that the UE will report one of these.

Observation 6: Without introducing some kind of reporting, there is no perfect solution among the ones proposed so far, but we believe that our proposed one (Proposal 2) would minimize the uncertainty at the gNB and thus maximize the performance of the channel state estimation in the field, which is the main goal of the antenna switching usage of SRS.

For this meeting, we have also submitted a CR [3] which implements the proposed changes from Proposal 2.

## Introducing reporting as an improved solution

In our view, the only way to optimally solve this issue is to introduce some kind of reporting. In the following, we present two possible approaches.

### 2.3.1   Reporting of ΔPPowerClass

A solution which has already been discussed in the past but to which unfortunately some companies have been reluctant consists of UE reporting the ΔPPowerClass depending on its PA configuration. For example, if a PC1.5 UE is equipped with 1 x 26 dBm + 3 x 23 dBm PAs configuration and indicates “t1ry or similar” AS capability, it could report ΔPPowerClass = 3 dB for the first SRS port and ΔPPowerClass = 6 dB for the second, third and fourth SRS port. In that way both the transmitted power of each SRS port would be maximized and there would be no uncertainty at the network about the SRS transmission power being used and thus the performance of the channel estimation would be maximized.

In the current version of TS38.101-1 specification [2], ΔPPowerClass is reported but only for limited number of scenarios:

NOTE: UE reports ∆PPowerClass when deltaPowerClassReporting-r18 is present, dpc-Reporting-FR1 [7] is configured and the reporting is triggered only by uplink duty cycle exceedance or by return to the ue-PowerClass after the duty cycle exceedance.

Nevertheless, since ΔPPowerClass reporting is already present in the specifications, its extension would not induce too large specification changes.

Proposal 3: A simple way to improve both the channel estimation accuracy and the SRS transmission power is to introduce reporting of ΔPPowerClass, which would not induce too large specification changes.

### 2.3.2   Reporting of the actual SRS transmission power

In our view, the optimal solution consists of UE reporting the actual SRS transmission power measured at the antenna connector (the same plane of reference as for the DL pathloss estimation). In that way, the UE could use the available output power for SRS transmission (depending on a PA) and the network would not have any ambiguities on which power has been used, and thus the channel estimation accuracy and the DL MIMO performance would be optimal.

Observation 7: The optimal solution consists of a UE reporting the actual SRS transmission power at the antenna connector. In that way, the UE could use the maximum output power for SRS transmission and the network would not have any ambiguities on the power being used, and thus the channel estimation accuracy would be optimal.

Basically, this issue could be addressed with the same kind of reporting as the one being discussed in the ongoing Rel-19 WI on “UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4”, topic “6Rx for handheld and FWA UE”. There, the insertion loss imbalance issue across SRS ports has been recognized, which is non-reciprocal on the DL, and the need for reporting is currently being studied.

Observation 8: This issue could be addressed with the same kind of reporting as the one being discussed in the ongoing Rel-19 WI on “UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4”, topic “6Rx for handheld and FWA UE”.

In our contribution [4] to “6Rx for handheld and FWA UE” topic, we have proposed the following reporting solution: the UE should report the configured maximum output power per SRS resource, the power headroom per SRS resource and ΔPPowerClass. In that way the network can have a relatively precise information on each SRS output power at the connector. In addition, the information on the configured maximum output power per SRS resource allows the network to be aware of whether a UE operates at its maximum power.

Proposal 4: If introducing of reporting to solve this issue is agreeable to all companies, the discussion could be merged with the ongoing one on the SRS insertion loss imbalance reporting in Rel-19 WI on “UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4”, topic “6Rx for handheld and FWA UE”, since the same solution could solve both issues.

# Conclusion

In the previous sections, we made the following observations:

Observation 1: ΔPPowerClass requirement definition has been extended in Rel-17 to apply for SRS transmission occasions with usage in SRS-ResourceSet set as ‘antennaSwitching’, where the main motivation was to prevent UEs which indicate PC1.5 or PC2 together with txDiversity-r16 to virtualize the SRS ports by using 2 Tx chains.

Observation 2: The main benefit of the present solution is that it prevents the virtualization of SRS ports for the range of the allowed maximum configured output power settings where the UE equipped uniquely with the PC3 PAs would be forced to do it (by reducing the power), for example for PC2 UE indicating Tx Diversity in the range between 23 dBm and 26 dBm.

Observation 3: The introduction of additional 3dB relaxation of ΔTRxSRS for PC2 UEs not indicating the Tx Diversity was motivated by the case of a UE sounding some SRS ports with 26 dBm while other with 23 dBm. Due to the nature of ΔTRxSRS, which is a maximum allowed relaxation, UEs equipped uniquely with PAs of 26 dBm or with both 23 dBm and 26 dBm PAs and where only the PA of 26 dBm is used for SRS sounding could also apply the relaxed values of ΔTRxSRS which should not be allowed.

Observation 4: The absence of knowledge of the increased SRS power being used for AS may introduce performance degradation rather than a gain.

Observation 5: If ΔPPowerClass weren’t introduced to PCMAX_H,f,c, for all possible PA configurations at a 4Tx UE there would be a range of the allowed configured transmitted output power settings where a UE would be forced (obliged) to use the antenna virtualization to achieve it.

Observation 6: Without introducing some kind of reporting, there is no perfect solution among the ones proposed so far, but we believe that our proposed one (Proposal 2) would minimize the uncertainty at the gNB and thus maximize the performance of the channel state estimation in the field, which is the main goal of the antenna switching usage of SRS.

Observation 7: The optimal solution consists of a UE reporting the actual SRS transmission power at the antenna connector. In that way, the UE could use the maximum output power for SRS transmission and the network would not have any ambiguities on the power being used, and thus the channel estimation accuracy would be optimal.

Observation 8: This issue could be addressed with the same kind of reporting as the one being discussed in the ongoing Rel-19 WI on “UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4”, topic “6Rx for handheld and FWA UE”.

We made the following proposals:

Proposal 1: To address the power limitation issue for SRS transmissions for AS for UEs equipped with 4 Tx chains ΔPPowerClass should be used, just as in 2Tx case. Avoid combining ΔPPowerClass and ΔTRxSRS requirements since those two have a different nature and introduce uncertainty at the gNB and thus degrade the channel estimation accuracy.

Proposal 2: We propose that different values should be applicable for ΔPPowerClass as a function of the indicated AS capability:

- For ‘t4ry’ AS capability, ΔPPowerClass = 0 dB.

- For ‘t2ry’ and similar AS capabilities, ΔPPowerClass = 3 dB.

- For ‘t1ry’ and similar AS capabilities, ΔPPowerClass = 6 dB.

Proposal 3: A simple way to improve both the channel estimation accuracy and the SRS transmission power is to introduce reporting of ΔPPowerClass, which would not induce too large specification changes.

Proposal 4: If introducing of reporting to solve this issue is agreeable to all companies, the discussion could be merged with the ongoing one on the SRS insertion loss imbalance reporting in Rel-19 WI on “UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4”, topic “6Rx for handheld and FWA UE”, since the same solution could solve both issues.

# References

- R4-2400341, “Delta PpowerClsss for 4Tx for SRS antenna switching”, Nokia, Nokia Shanghai Bell, RAN4#110

- 3GPP TS 38.101-1 V18.6.0, “NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”

- R4-2413358, “(NR_ENDC_RF_FR1_enh2-Core) CR to 38.101 Rel-18: DeltaP_PowerClass correction for SRS AS for 4Tx”, Ericsson, RAN4#112

- R4-2413360, “On need for SRS insertion loss imbalance reporting”, Ericsson, RAN4#112