3GPP TSG-RAN WG4 Meeting #112									                        R4-2413359

Maastricht, Netherlands, August 19 – 23, 2024

Agenda item:			8.1.1.3.3

Source:	Ericsson

Title:	On DeltaT_RxSRS requirement for 6Rx UE RF

Document for:	Approval

# Introduction

In RAN#104, a revised WID has been approved for “UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4” WI [1]. One of the three main parts of the WI is to specify 6Rx UE RF requirements for both handheld and FWA UEs, with the following objectives:

- Specify the core requirements to enable 6Rx for higher frequency bands (>2.5GHz) targeting at support of handheld UE for NR FR1 single carrier scenario

  - Example bands: n41, n77/n78, n79, n104

  - Support 4 MIMO layers at least, and study the gain and feasibility and if feasible, support 6 MIMO layers

  - Specify the Rx requirements including reference sensitivity requirements for support 6Rx

    - Note: the specified requirements can be applicable to both handheld UE and FWA devices

  - Specify the requirements to support SRS antenna switching including t1r6, t2r6, t3r6, depending on UE capability

    - RAN1 to study and specify, if necessary, SRS antenna switching to support 3T6R depending on UE capability.

      - Note: RAN1 discussion starts from RAN1#118bis (Oct. 2024)

  - Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding solution.

In this contribution, we focus on the second-to-last bullet point, which is to specify the requirements to support SRS antenna switching. As it can be seen above, the SRS antenna switching (AS) capabilities included in the WID are ‘t1r6’, ‘t2r6’ and ‘t3r6’. Both ‘t3r6’ and ‘t4r6’ have figured in the initial version of the WID but since it has been observed that those AS capabilities have never been specified by RAN1 and are not supported according to TS38.306, an LS was sent from RAN4 to RAN1 on the support for such capabilities. Since there was no agreement in RAN1, the discussion moved to RAN where it was finally decided to remove ‘t4r6’ from the WID in the latest revision and that: “RAN1 to study and specify, if necessary, SRS antenna switching to support 3T6R depending on UE capability”. Since the work in RAN1 will not start before RAN1#118bis meeting (Oct. 2024), in this contribution we will focus on ‘t1r6’, ‘t2r6’ as well as ‘t1r6-t2r6’ SRS AS capabilities and propose the corresponding ΔTRxSRS requirements.

# Discussion

In the approved WF from RAN4#111 [2], the following was agreed:

Issue 2-1-1: SRS antenna switching configurations

Way forward: RAN4 to focus on discussing ∆TRxSRS for 1tr6 and t2r6 until RAN1 concludes work on 3t6r and 4t6r. Pending RAN1 feedback, RAN4 can consider any necessary updates to the work scope in a Revised WID for the SRS antenna configurations subject to RAN Plenary approval.

Issue 2-1-2: Whether to use different ∆TRxSRS based on operating frequency

Way forward: RAN4 defines different ∆TRxSRS values based on operating frequency. Further discuss whether to introduce an additional breakpoint for bands whose FUL_high is higher than the FUL_low of n104.

Issue 2-2-1: Proposed ∆TRxSRS values

Way forward: Further discuss the following options for the ∆TRxSRS requirements.

-	Option 1: Consider average value of ∆TRxSRS for further discussion.

| Operating bands | ΔTRxSRS requirement for t1r6 | ΔTRxSRS requirement for t2r6 | ΔTRxSRS requirement for t1r6-t2r6 |
| --- | --- | --- | --- |
| Band n41, n77, n78 | 4.0 dB | 3.4 dB | 4.4 dB |
| Band n79, [n104] | 5.3 dB | 4.5 dB | 5.7 dB |

-	Option 2: Consider range of ∆TRxSRS in brackets for each case below for further discussion.

| Operating bands | ΔTRxSRS requirement for t1r6 | ΔTRxSRS requirement for t2r6 | ΔTRxSRS requirement for t1r6-t2r6 |
| --- | --- | --- | --- |
| Band n41, n77, n78 | [2.8 ~ 5.0] dB | [2.3 ~ 4.0] dB | [3.3 ~ 5.5] dB |
| Band n79, [n104] | [3.6 ~ 6.0] dB | [3.0 ~ 5.5] dB | [4.2 ~ 6.5] dB |

Note: Maximum level for t1r6 corrected for OPPO proposals per OPPO.

-	Option 3: Other options are not precluded.

ΔTRxSRS relaxation requirement is measured at the antenna connector, and it is important not to have too loose margins in order to limit degradation of DL-MIMO performance, i.e. to have an adequate reciprocal CSI-RS estimation, especially since the gNB measures the resulting radiated SRS power per SRS port(s) which include different antenna element gains (in a typical implementation, antenna elements on diversity paths have a poorer quality compared with the antenna elements on the main path).

As a reminder, ΔTRxSRS is a maximum allowed relaxation applied to the lower bound of the maximum configured transmitted power PCMAX,f,c due to the additional insertion loss of a given diversity branch relative to the main branch. The main “contributors” to insertion loss are antenna switches, filters, as well as PCB trace/RF cable routing located between the PA which sounds the given SRS port and the antenna connector.

## ‘t1r6’ SRS AS capability

On Figure 1, we have shown two alternatives (out of many possible) for the UE architecture for ‘t1r6’ SRS AS capability.

The exact value of the insertion loss introduced by each antenna switch, PCB trace/RF cable routing etc. depends on the particular component being used where values can vary considerably between different manufacturers. In addition, for the given component, the insertion loss depends mainly on the operating frequency point and for that reason in current specifications there are different ΔTRxSRS requirements for bands whose FUL_high is higher than the FUL_low of n79 and for bands whose FUL_high is lower than the FUL_low of n79. The insertion loss of the given component also depends on the temperature conditions, but we will assume normal temperature conditions in all cases.

- (b)

Figure 1	Two alternatives for UE architecture for ‘t1r6’ antenna switching.

Instead of providing some values for insertion losses of different components which, as we have just mentioned, can vary significantly from a UE to UE (based on its architecture and concrete choice of components), we will do a rough comparison with ‘t1r4’ and ‘t1r8’ UE architectures and propose some reasonable values for ΔTRxSRS requirement.

For bands whose FUL_high is lower than the FUL_low of n79, in current specifications [3] ΔTRxSRS is specified as 3dB for ‘t1r4’ SRS AS capability where at the time of its specification both handheld and FWA/CPE types of UE were considered. For the same group of bands, in current specifications [3] ΔTRxSRS is specified as 4dB for ‘t1r8’ SRS AS capability where at the time of its specification only FWA/CPE/vehicle/industrial types of UE were considered.

Comparing the architecture of alt. (a) on Figure 1 for ‘t1r6’ AS capability with a typical ‘t1r8’ architecture, for the “worst-case” positioned antenna of ‘t1r6’ (antenna 5 or antenna 6) the insertion loss is lower than the insertion loss of the “worst-case” positioned antenna of ‘t1r8’ (see e.g. [4] for an example of ‘t1r8’ UE architecture) as much as the insertion loss of SPDT antenna switch is lower than the insertion loss of SP4T antenna switch, which is ~0.3dB – 0.5dB. Similarly, the insertion loss of SP6T antenna switch (alt. (b) on Figure 1) is around 0.5dB smaller than the insertion loss of SP8T antenna switch, which should be needed for a similar ‘t1r8’ implementation.

Given that in this WI both handheld and FWA types of UE are considered, and that the requirement should cover both types of devices, where for handheld type of UEs PCB traces and RF cables are shorter and thus IL are smaller, we propose ΔTRxSRS = 3.5 dB for bands whose FUL_high is lower than the FUL_low of n79. For bands whose FUL_high is higher than the FUL_low of n79, up to Rel-18 the practice has been to adopt 1.5dB higher value than for bands whose FUL_high is lower than the FUL_low of n79. For band n79 in particular, 1.5dB is quite a relaxed value according to the analysis made by different companies and the proposed values in the previous meeting (Issue 2-2-1 in the WF [2]), where the average difference between the two groups of frequency bands is around 1.3dB for ‘t1r6’ and 1.1dB for ‘t2r6’ AS capability. We recognize that the insertion loss of different components increases with the frequency, but due to already relaxed ΔTRxSRS values, consistency with previous releases and specification simplicity (using a single requirement for multiple bands), we propose to adopt the same value for both n79 and n104 bands: ΔTRxSRS = 5.0 dB.

Proposal 1: For ‘t1r6’ AS capability, for bands n41 and n77/78 ΔTRxSRS should be specified as 3.5 dB, while for bands n79 and n104 ΔTRxSRS should be specified as 5.0 dB.

## ‘t2r6’ SRS AS capability

The typical implementation architecture for ‘t2r6’ SRS AS capability (where no fallback to 1 PA is needed to be supported) corresponds to having two ‘t1r3’ architectures in parallel as shown on Figure 2 (there are other possibilities as well). Compared with architecture for ‘t1r8’ AS, SP3T antenna switches are used instead of SP4T, which typically have a smaller insertion loss.

Figure 2	An example of UE architecture for ‘t2r6’ antenna switching.

For bands whose FUL_high is lower than the FUL_low of n79, in current specifications [3] ΔTRxSRS is specified as 3 dB for ‘t1r4’ SRS AS capability and 4 dB for ‘t1r8’ SRS AS capability (under the conditions stated in clause 2.1). Even though the IL of ‘t2r8’ AS can be as low as the IL of ‘t1r4’ (since it corresponds to two ‘t1r4’ architectures in parallel), the requirement was agreed to be equal as for ‘t1r8’ case.

Sticking to the same principle, we propose the following:

Proposal 2: For ‘t2r6’ AS capability, for bands n41 and n77/78 ΔTRxSRS should be specified as 3.5 dB, while for bands n79 and n104 ΔTRxSRS should be specified as 5.0 dB.

## ‘t1r6-t2r6’ SRS AS capability

For ‘t1r6-t2r6’ AS, one out of two PAs should be implemented as a fallback in case the other one is switched off, and thus the architecture looks a slightly more complex than in previous two cases, where one example is shown on Figure 3.

Figure 3	An example of UE architecture for ‘t1r6-t2r6’ antenna switching.

For bands whose FUL_high is lower than the FUL_low of n79, in current specifications [4] ΔTRxSRS is specified as 4.5 dB for ‘t1r8-t2r8’ SRS AS capability. Compared with a typical ‘t1r8-t2r8’ architecture (see e.g. [4]), the insertion loss of the “worst-cased” positioned antenna (for ‘t1r6-t2r6’) is lower than the insertion loss of the worst-case positioned antenna for ‘t1r8-t2r8’ as much as the IL of DP3T antenna switch is smaller than the IL of DP4T antenna switch, and in addition one SPDT antenna switch could be removed.

For that reason, and since ΔTRxSRS for ‘t1r8-t2r8’ AS capability is higher by 0.5dB compared with both ‘t1r8’ and ‘t2r8’ AS capabilities, the difference between ΔTRxSRS for ‘t1r6-t2r6’ and both ‘t1r6’ and ‘t2r6’ should be at most 0.5 dB. Thus, we propose the following:

Proposal 3: For ‘t1r6-t2r6’ AS capability, for bands n41 and n77/78 ΔTRxSRS should be specified as 4.0 dB, while for bands n79 and n104 ΔTRxSRS should be specified as 5.5 dB.

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1: For ‘t1r6’ AS capability, for bands n41 and n77/78 ΔTRxSRS should be specified as 3.5 dB, while for bands n79 and n104 ΔTRxSRS should be specified as 5.0 dB.

Proposal 2: For ‘t2r6’ AS capability, for bands n41 and n77/78 ΔTRxSRS should be specified as 3.5 dB, while for bands n79 and n104 ΔTRxSRS should be specified as 5.0 dB.

Proposal 3: For ‘t1r6-t2r6’ AS capability, for bands n41 and n77/78 ΔTRxSRS should be specified as 4.0 dB, while for bands n79 and n104 ΔTRxSRS should be specified as 5.5 dB.

# References

- RP-241656, “New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4”, RAN#104.

- R4-2410751, “WF on requirements for 6Rx”, AT&T, RAN4#111.

- 3GPP TS 38.101-1 V18.6.0, “NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”

- R4-2216437, “R18 Discussion on 8Rx FWA”, OPPO, RAN4#104bis-e.