3GPP TSG-RAN WG4 Meeting #112									                        R4-2413360

Maastricht, Netherlands, August 19 – 23, 2024

Agenda item:			8.1.1.3.4

Source:	Ericsson

Title:	On need for SRS insertion loss imbalance reporting

Document for:	Approval

# Introduction

In RAN#103, a revised WID has been approved [1] for “UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4” WI. One of the three main parts of the WI is to specify 6Rx UE RF requirements for both handheld and FWA UEs, with the following objectives:

- Specify the core requirements to enable 6Rx for higher frequency bands (>2.5GHz) targeting at support of handheld UE for NR FR1 single carrier scenario

  - Example bands: n41, n77/n78, n79, n104

  - Support 4 MIMO layers at least, and study the gain and feasibility and if feasible, support 6 MIMO layers

  - Specify the Rx requirements including reference sensitivity requirements for support 6Rx

    - Note: the specified requirements can be applicable to both handheld UE and FWA devices

  - Specify the requirements to support SRS antenna switching including t1r6, t2r6, t3r6, depending on UE capability

    - RAN1 to study and specify, if necessary, SRS antenna switching to support 3T6R depending on UE capability.

      - Note: RAN1 discussion starts from RAN1#118bis (Oct. 2024)

  - Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding solution.

In this contribution, we focus on the last bullet point, which is to study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding solution. This topic has been discussed in depth already in Rel-18 under “Enable 8Rx for CPE/FWA/vehicle/industrial devices” topic of “Further RF requirements enhancement for NR and EN-DC in frequency range 1 (FR1)” WI. There, the discussion was mostly focused on the concrete solution rather than the justification to introduce the SRS IL imbalance reporting.

# Discussion

The discussion on this topic has made little progress so far. According to the approved WF [2], the companies should first determine a set of initial conditions which will allow them to have a common understanding for the study, and afterwards the discussion on the candidate solution could start. In the following, we will address both subjects.

## General considerations for SRS IL imbalance issue

In the approved WF [2], the following was captured on this sub-topic:

Sub-topic 4-1:	General considerations for SRS IL imbalance issue

Issue 4-1-2: Initial Considerations for SRS IL imbalance issue

Way forward: Given the different views amongst companies, RAN4 to further discuss the set of initial considerations which will allow companies to have a common understanding for the study including existing UE behavior for SRS transmissions in case of SRS IL.

-	Companies are encouraged to bring analysis on the existing UE behavior and achievable power imbalance for SRS transmissions based on current specification and UE implementations in case of SRS IL.

-	Companies are encouraged to analyse the impact of SRS IL imbalance on NW performance degradation.

After analyzing the existing UE behavior and the impact of SRS IL imbalance on NW performance degradation we will bring up another important argument to introduce a reporting for SRS AS, related to the ongoing discussion on specifying ΔPPowerClass requirement for SRS AS for 4Tx (maintenance agenda for UE RF enhancement for Rel-18).

### Existing UE behavior

In the following, we will analyze the UE behavior when it comes to power compensation for SRS AS transmissions in case of SRS IL according to current specifications.

According to TS38.213 [3], the power control equations for SRS transmission occasions are defined at the antenna connector (all transmission requirements in FR1 are defined at the antenna connector, as stated in TS38.101-1 [4]):

[dBm]

According to TS38.215 [5], the path loss (PL) measurements, both CSI-RSRP and SSB-RSRP, are also defined at the antenna connector in the same plane of reference (RSRP reported can include more than one connector, but the use of diversity is not mandated). In the above equation, the PL is a function of RS resource index qd (e.g. SSB resource) which is the same for all SRS resources in the set, which makes sense assuming that the plane of reference is the same for all RX antenna ports (antenna connectors). This is also consistent with the above observation that the plane of reference for the SRS output power is the antenna connector.

The PCMAX,f,c must be defined in the same plane of reference as the PL measurements, that is, in the plane of reference of the RSRP (for both FR1 and FR2). For FR1, this plane is also consistent with the measurement of the output power at the antenna connectors. The NW can therefore only assume that the PCMAX,f,c(i)  reported in the PHR is referred to the antenna connectors for FR1.

The PCMAX,f,c(i) is set per transmission occasion i, i.e. per SRS resource and can include the IL on top of the MPR applied for the SRS (likely the same MPR for all AS resources). Any reported PCMAX,f,c(i) for SRS transmissions should therefore include the actual IL TX/RX imbalance on top of the MPR applied for the transmitted resource (the internal power setting is of course up to UE implementation).

Note that according to TS38.213 [3], clause 7.3, if the non-TDMed SRS resource consists of multiple ports then its transmission power is equally divided across all ports (at all power levels): “- else, a UE splits a linear value PSRS,b,f,c(i,qs,l) of the transmit power PSRS,b,f,c(i,qs,l) on active UL BWP b of carrier f of serving cell c equally across the configured antenna ports for SRS.” In that case, the mapping between the different SRS ports and the actual antenna connectors in the UE is up to UE implementation (but different pairs should be used for each resource).

When it comes to compensation, as discussed in the Rel-18 WI, according to the above equation, the UE shall compensate for the IL below the maximum configured output power for a resource, as the insertion loss is not accounted for. It can be understood that the UE can compensate for such losses by setting the appropriate power of PA supplying PSRS,b,f,c(i,qs,l) at the antenna connector(s) for the transmitted resource, at least below the maximum output power for the SRS transmission.

Observation 1	According to the current specifications, the UE shall compensate for the IL below the maximum configured output power for a resource, as the insertion loss is not accounted for in the SRS power control equations.

However, it is recognized that in practice for SRS resources for which the allowed DTRxSRS > 0dB the insertion loss may not be fully accounted for (and thus compensated) in implementations since the allowed tolerance of the absolute power level in device testing can be of order of 10dB when the setting is below the maximum power, i.e. the insertion loss may be absorbed by the large absolute tolerance. Hence, for such implementations the UL power as set by the power-control equations in 38.213 below the maximum would be reduced by the IL imbalance at the connector (notwithstanding other power-setting inaccuracy), and this would not be captured by any RAN4 requirements since power tolerances are large. Moreover, conformance testing of SRS is limited.

Potential inconsistencies in the field on whether the UE, depending on its implementation, tries to compensate (at least in some cases) the IL is in our view a reason more to introduce SRS IL imbalance reporting since the UE behavior is almost impossible to predict for the network.

Observation 2	It is recognized that in practice the insertion loss may not be fully accounted for (and thus compensated) for all UE implementations. That is a reason to introduce SRS IL imbalance reporting since the behavior of different UE implementations is almost impossible to predict for the network.

### Impact of SRS IL imbalance on NW performance degradation

For an accurate DL CSI estimation and good reciprocity-based DL-MIMO performance based on SRS antenna switching the gNB would ideally be aware of any non-reciprocal IL for each sounded antenna connector/port. However, depending on the architecture, the power sounded from each receive antenna port/connector may be different due to non-reciprocal insertion losses on given paths and the implementation of the SRS power control on diversity connectors, and RAN4 has only specified a maximum allowed relaxation ΔTRxSRS for such ports at maximum output power.

The introduction of some kind of IL (insertion loss) reporting mechanism for SRS Antenna Switching is necessary given how large the specified ΔTRxSRS values are, up to more than 10dB for certain SRS-AS capabilities for higher frequency bands when the device is capable of power class 2. In addition, in the field even under static conditions a very large variations (as a function of time) of received SRS power/SNR have been observed per SRS port due to the power setting inaccuracies etc.

Observation 3	In the field, even under static conditions a very large variations (as a function of time) of received SRS power/SNR have been observed per SRS port due to the power setting inaccuracies etc.

To further justify the need for introducing an IL reporting mechanism for AS we have performed non-full-buffer system-level simulations in [6]. We have considered a UE with 4 RX chains and 1 TX chain configured with 1T4R antenna switching and two cases of the SRS IL imbalance:

- Without imbalance: [0, 0, 0, 0] dB for SRS port 0—3.

- With imbalance: [0, -3, -6, -6] dB for SRS port 0—3.

Note that the case “without imbalance” corresponds to the case when gNB can perfectly compensate for SRS IL imbalances (e.g., if UE reports the exact SRS IL imbalance) and the case “with imbalance” correspond to the case when gNB has no knowledge of the SRS IL imbalance and, hence, cannot compensate for it. The remaining/detailed simulation assumptions can be found in [6].

On Figure 1, the mean and the cell-edge user throughput are compared for cases “with” and “without” SRS IL imbalance for MU-MIMO scenario. It can be observed that the SRS IL imbalance, without reporting/compensation, results in a non-negligible performance loss. This can be understood from Figure 2 where it is shown that the average rank (per gNB) is lower in the presence of non-compensated SRS IL imbalances. Hence, for reciprocity-based DL precoding, SRS IL imbalance results in the measured DL CSI quality being worse than the actual DL CSI quality such that the gNB select a suboptimal precoder and a lower rank, which results in throughput loss.

Observation 4	The SRS IL imbalance without reporting/compensation results in a non-negligible DL MIMO performance loss.

In [6], the simulations have been performed for SU-MIMO scenario as well where similar conclusions can be drawn.

Figure 1	Mean and cell-edge MU-MIMO user throughput with/without SRS IL imbalance. Here, the round, square, and diamond markers correspond to 20%, 50%, and 70% resource utilization, respectively.

Figure 2	MU-MIMO rank with/without SRS imbalance.

Finally, we believe that this issue is not only relevant for 6Rx, but also for 2Rx, 4Rx and 8Rx and the specified solution should be applicable to all the mentioned cases, which should even be the design criterion.

- Introduce reporting for the SRS insertion loss imbalance issue in Rel-19.

- The introduced solution for the SRS insertion loss imbalance issue should be applicable to all 2Rx/4Rx/6Rx/8Rx cases.

### Issue with SRS antenna switching and 2Tx / 4Tx

Another important argument to introduce some kind of reporting for SRS AS can be seen from the discussion on “ΔPPowerClass requirement for SRS AS for 4Tx” issue.

As a reminder, in Rel-17, ΔPPowerClass requirement definition has been extended to apply for SRS transmission occasions with the usage in SRS-ResourceSet set as ‘antennaSwitching’. The main motivation was to prevent UEs which indicate PC1.5 or PC2 together with txDiversity-r16 to virtualize the SRS ports by using 2 Tx chains. The following is an excerpt from TS-38.101-1 [4]:

- 3dB is applied during SRS transmission occasions with usage in SRS-ResourceSet set as ‘antennaSwitching’ with configured SRS resources in each SRS resource set(s) consisting of one SRS port when PC2 UE with txDiversity-r16 or txDiversity2Tx-r18 capability or PC1.5 UE further indicates SRS-TxSwitch capability ‘t1r2’ or ‘t1r4’ or ‘t1r1-t1r2’ or ‘t1r1-t1r2-t1r4’ or further indicates srs-AntennaSwitchingBeyond4RX-r17 as ‘t1r8’;

If such requirement didn’t exist, e.g. the PC2 UE indicating txDiversity-r16 capability would be assumed to use two half-power PAs of 23dBm to achieve 26dBm by virtualization even for ‘t1ry’ (y=2,4,8) indicated AS capability, which shall not be allowed. Even if such UE were equipped with one PA of 26dBm, the network couldn’t have known it and it could only assume the UE behavior based on its reported capability. By applying ΔPPowerClass = 3dB such uncertainty at the network is avoided, where the improvement in accuracy of channel estimation (by avoiding such uncertainty) may even overcome the drawback of “unnecessarily” capping the SRS transmission power to 23dBm for UE implementations where 26dBm could be achieved with one PA. Nonetheless, such UEs, equipped with a PA of 26dBm, are not expected to indicate txDiversity-r16 or txDiversity2Tx-r18 capability, so this limitation is targeted for 23 dBm + 23 dBm PA architectures.

In Rel-18 WI on “Further RF requirements enhancement for NR frequency range 1 (FR1)”, UE RF requirements have been specified to support 4 Tx. All the necessary requirements have been specified except for ΔPPowerClass requirement for AS, where the discussion is still ongoing as part of the maintenance agenda for UE RF enhancement for Rel-18 since an agreement cannot be reached between the companies. The main reason is the increased complexity of 4Tx case compared with 2Tx, since there are many more possibilities for PA configurations, such as 4 x 23 dBm, 2 x 23 dBm + 2 x 26 dBm, 4 x 26 dBm etc. On one hand, there is a proposal to specify ΔPPowerClass = 6dB for ‘t1ry’, ΔPPowerClass = 3dB for ‘t2ry’ and ΔPPowerClass = 0dB for ‘t4ry’ AS capabilities, which minimizes the uncertainty about the SRS transmission power at the network side at the price of limiting it (more detail in [7]). On another hand, there are proposals to either remove ΔPPowerClass from the upper bound of PCMAX,f,c, which would force the UE to virtualize antenna ports for SRS transmission powers near PCMAX,f,c, or to combine ΔPPowerClass with DTRxSRS, which would introduce more uncertainty at the network side about the SRS transmission power. Either way, for 4Tx there is no optimal solution without the introduction of reporting.

Observation 5	“ΔPPowerClass requirement for SRS AS for 4Tx” issue could not be optimally solved without the introduction of reporting of the SRS transmission power or similar.

If the UE were reporting the SRS transmission power, which would include the information on the insertion loss since it is defined at the antenna connector, there would be no uncertainty at the network about the SRS transmission power nor power limitations at the UE. This solution would be the most beneficial one for the performance of reciprocity-based CSI estimation for DL MIMO, and it could be realized as presented in clause 2.2. Therefore, a common reporting could be introduced for both “SRS IL imbalance” issue and “ΔPPowerClass requirement for SRS AS for 4Tx” issue, and the two discussions could be merged.

- The discussion on “ΔPPowerClass requirement for SRS AS for 4Tx” issue could be merged with the discussion on “SRS IL imbalance” issue if reporting related to the SRS transmission power is agreed to be introduced.

## Proposed SRS IL reporting solution

On candidate solutions for the SRS IL imbalance issue the following was captured in the WF [2]:

Sub-topic 4-2:	Candidate solutions for the SRS IL imbalance issue

Issue 4-2-1: Candidate solutions for the SRS IL imbalance issue

Way forward: After determining the set of initial considerations for the study, discuss candidate solutions considering the UE behaviour in terms of the IL imbalance compensation and whether the UE can compensate the IL imbalance for AS-SRS for all power levels and whether NW and UE have consistent understanding for the possible compensation.

Based on the discussion in the previous clause and the conclusion that the UE behavior when it comes to the compensation of the additional IL of a given Rx branch is unpredictable, reporting of the actual insertion loss (or IL imbalance) per branch is in general insufficient for the accurate DL CSI estimation as in many scenarios the UE Tx power is below the maximum configured transmission power, and thus it is unclear how the network would use this information. The actual insertion loss should be directly included in the SRS output power.

Observation 6	Since the UE behavior when it comes to the compensation of the additional IL of a given Rx branch is unpredictable, reporting of the actual insertion loss (or IL imbalance) per branch is in general insufficient for the accurate DL CSI estimation (it is unclear how the network would use this information).

Our proposed solution is that reporting should include the configured maximum output power per SRS resource and the power headroom (PH) per SRS resource as well as an information on a power class fallback or the return to the supported power class (ΔPPowerClass). In that way the network can have a relatively precise information on each SRS output power at the connector, which is in line with the reasoning in clause 2.1.3. The information on the configured maximum output power per SRS resource allows the network to be aware of whether a UE operates at its maximum power. The configured maximum output power includes the actual insertion loss (TX/RX) imbalance (for resources for which this is allowed) on top of the MPR used for the SRS resource. If the MPR is the same between the different ports (which is expected), by inspecting the differences between the configured maximum output power per SRS resource the information on IL imbalance can be obtained. On the other hand, by transmitting in addition the power headroom (PH) report per SRS resource, the NW can obtain the information on the actual output power per SRS resource and observe whether the IL compensation has been performed. So, both cases when UEs perform or do not perform a compensation for insertion losses are covered by the proposed solution.

- IL imbalance reporting mechanism for SRS AS should include the configured maximum output power per SRS resource, the power headroom per SRS resource and ΔPPowerClass.

The PH used for the SRS resource can be a Type 3 but used for a new MAC-CE “SRS resource power report” (instead of PHR MAC-CE) and can be used also for a carrier configured for PUSCH transmission. This would be a RAN1 change but very limited (i.e. apply the Type 3 for the new MAC-CE and also on a PUSCH carrier). A new MAC-CE specified for SRS power reporting should be dynamic since SRS AS power levels can change.

- The PH used for the SRS resource can be a Type 3 but used for a new MAC-CE “SRS resource power report” and can be used also for a carrier configured for PUSCH transmission.

Another way to resolve the SRS IL imbalance reporting issue (especially if companies could not find an agreement on one solution) would be to introduce two types of reporting for a UE: a “baseline” and an “advanced” reporting. A UE could indicate which one it is using. The baseline reporting would not require any calibration at the UE and it could include e.g. the PHR and ΔPPowerClass information. That information alone is not enough for the accurate knowledge of the SRS transmission power but it would give at least the information on whether the UE operates at its maximum output power, as well as which power class is being used. The advanced reporting could in addition include the information on e.g. the configured maximum output power which would substantially increase the knowledge of the SRS transmission output power by the network.

- Another way to resolve the SRS IL imbalance reporting issue would be to introduce two types of reporting for a UE: a “baseline” and an “advanced” reporting. The baseline reporting would not require any calibration at the UE and it could include e.g. the PHR and ΔPPowerClass information. The advanced reporting could in addition include the information on e.g. the configured maximum output power.

# Conclusion

In the previous sections, we made the following observations:

Observation 1	According to the current specifications, the UE shall compensate for the IL below the maximum configured output power for a resource, as the insertion loss is not accounted for in the SRS power control equations.

Observation 2	It is recognized that in practice the insertion loss may not be fully accounted for (and thus compensated) for all UE implementations. That is a reason to introduce SRS IL imbalance reporting since the behavior of different UE implementations is almost impossible to predict for the network.

Observation 3	In the field, even under static conditions a very large variations (as a function of time) of received SRS power/SNR have been observed per SRS port due to the power setting inaccuracies etc.

Observation 4	The SRS IL imbalance without reporting/compensation results in a non-negligible DL MIMO performance loss.

Observation 5	“ΔPPowerClass requirement for SRS AS for 4Tx” issue could not be optimally solved without the introduction of reporting of the SRS transmission power or similar.

Observation 6	Since the UE behavior when it comes to the compensation of the additional IL of a given Rx branch is unpredictable, reporting of the actual insertion loss (or IL imbalance) per branch is in general insufficient for the accurate DL CSI estimation (it is unclear how the network would use this information).

Based on the discussion in the previous sections we propose the following:

Proposal 1	Introduce reporting for the SRS insertion loss imbalance issue in Rel-19.

Proposal 2	The introduced solution for the SRS insertion loss imbalance issue should be applicable to all 2Rx/4Rx/6Rx/8Rx cases.

Proposal 3	The discussion on “ΔPPowerClass requirement for SRS AS for 4Tx” issue could be merged with the discussion on “SRS IL imbalance” issue if reporting related to the SRS transmission power is agreed to be introduced.

Proposal 4	IL imbalance reporting mechanism for SRS AS should include the configured maximum output power per SRS resource, the power headroom per SRS resource and ΔPPowerClass.

Proposal 5	The PH used for the SRS resource can be a Type 3 but used for a new MAC-CE “SRS resource power report” and can be used also for a carrier configured for PUSCH transmission.

Proposal 6	Another way to resolve the SRS IL imbalance reporting issue would be to introduce two types of reporting for a UE: a “baseline” and an “advanced” reporting. The baseline reporting would not require any calibration at the UE and it could include e.g. the PHR and ΔPPowerClass information. The advanced reporting could in addition include the information on e.g. the configured maximum output power.

# References

- RP-241656, “New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4”, RAN#104.

- R4-2410751, “WF on requirements for 6Rx”, AT&T, RAN4#111.

- 3GPP TS38.213 V18.3.0, “NR; Physical layer procedures for control”

- 3GPP TS 38.101-1 V18.6.0, “NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”

- 3GPP TS38.215 V18.3.0, “NR; Physical layer measurements”

- R1-2305911, “Discussion on RAN4 LS on the UE SRS imbalance issue”, Ericsson, RAN1#113

- R4-2413357, “(NR_ENDC_RF_FR1_enh2-Core) On DeltaP_PowerClass for SRS AS for 4Tx”, Ericsson, RAN4#112