3GPP TSG-RAN WG4 Meeting #112									                        R4-2413361

Maastricht, Netherlands, August 19 – 23, 2024

Agenda item:			8.8.3.1

Source:	Ericsson

Title:	On system parameters for less than 5 MHz for NTN

Document for:	Approval

# Introduction

In RAN#104, a revised WID has been approved for “Enhanced requirements and conductive test methodology for NR NTN and IoT NTN” WI [1]. One of the three working areas of the WI is to specify less than 5 MHz for NTN with the following objectives:

- Specify the system parameters for channel bandwidth of less than 5 MHz for NR-NTN (NR based Non-Terrestrial Networks) in FR1-NTN bands

  - Consider the NR-NTN bands n255, n256, and n254

  - Add less than 5MHz channel bandwidth

    - 3MHz with the RAN1 design in Rel-18

  - Investigate and if necessary, specify the additional synchronization (sync) raster(s) for the additional channel bandwidth

    - Leverage the work on less than 5MHz for TN

  - Note: no RAN1 impact is expected

- Specify the necessary SAN RF core requirements for NR-NTN in FR1-NTN bands

- Specify the necessary NR-NTN UE RF core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands

  - Focus on power class 3 (PC3) for Tx RF requirements

- Specify the necessary RRM core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands

- Specify the necessary signaling to support the above objectives

In this contribution, we focus on the system parameters for channel bandwidths of less than 5 MHz for NR-NTN in FR1-NTN bands and we provide our initial considerations on their expected impact.

# Discussion

## System parameters channel bandwidths scope

In the title of this working area “less than 5 MHz for NTN” is mentioned, while in the first bullet point it is mentioned to “Specify the system parameters for channel bandwidth of less than 5 MHz for NR-NTN (NR based Non-Terrestrial Networks) in FR1-NTN bands”. However, in one of a sub-bullet point it is mentioned “3MHz with the RAN1 design in Rel-18”, so it should be clarified that this WI should only focus on 3 MHz channel bandwidth and the associated system parameters.

Proposal 1: This WI should only focus on 3 MHz channel bandwidth and the associated system parameters.

## Maximum transmission bandwidth configuration

One of the key parameters when it comes to the definition of a new channel bandwidth is to specify the maximum transmission bandwidth configuration NRB. Since the maximum transmission bandwidth configuration for all channel bandwidths of 5 MHz or larger in FR1-NTN bands are exactly the same as for the TN FR1 bands, the same should be followed for 3 MHz channel bandwidth, i.e. to specify 15 RB value (which was originally adopted from LTE 3 MHz CBW). Moreover, only SCS = 15kHz should be considered as in FR1 TN bands.

Proposal 2: Specify the maximum transmission bandwidth configuration NRB = 15 RB for 3 MHz channel bandwidth for all FR1-NTN bands (only consider SCS = 15 kHz).

## Minimum guardband

For each UE channel bandwidth and SCS, the associated parameter with the maximum transmission bandwidth configuration is the minimum guardband. Since the minimum guardbands for all channel bandwidths of 5 MHz or larger for FR1-NTN bands are the same as for the TN FR1 bands, the same should be followed for 3 MHz channel bandwidth, i.e. to specify 142.5 kHz value (which is obtained from equation GBChannel = (BWChannel x 1000 (kHz) - NRB x SCS x 12) / 2 - SCS/2).

Proposal 3: Specify the minimum guardband 142.5 kHz for SCS = 15kHz for 3 MHz channel bandwidth for all FR1-NTN bands.

## Channel raster

All considered bands n256, n255 and n254 in this working area of the WI are specified with the channel raster of 100 kHz, which was the value specified for TN FR1 FDD bands which are re-farmed from the LTE to allow the co-existence with it. Typically, the channel raster is not channel bandwidth dependent and to avoid any potential deployment issue with the coexistence with the UEs which do not support the “less than 5MHz channel bandwidth”, 100 kHz channel raster should also be specified for 3 MHz channel bandwidth.

Proposal 4: Specify 100 kHz channel raster for bands n256, n255 and n254 for spectrum where 3 MHz channel bandwidth is supported.

## Synchronization raster

In the Rel-18 WI on “NR support for dedicated spectrum less than 5MHz for FR1”, one of the main discussion points was the specification of a new synchronization raster since the legacy NR SSB has a 20 RBs width, which is wider than the maximum transmission bandwidth configuration for a 3 MHz channel bandwidth (15 RBs). For that reason, RAN1 has specified a punctured SSB of 12 RBs to fit in this narrower channel bandwidth. The value of 12 RBs has been originally motivated by a specific case of band n100, but in the end it was agreed to be applied to all bands.

Based on the RAN1 agreement, RAN4 agreed on the new sync raster design, similar to NR legacy one but with a step size of 600 kHz. The main motivation for this smaller step is that the legacy step of 1200 kHz is too large for a 3 MHz channel bandwidth, as it would not allow flexibility for the SS block reference frequency SSREF. The new sync raster has an offset of 300 kHz from the legacy one to ensure that there is no overlap between the two, so that the legacy UEs do not mistakenly try to decode the new sync raster points. It is specified as in the following table [2]:

Table 5.4.3.1-2: GSCN parameters for the global frequency for 3 MHz channel bandwidth

| Range of frequencies (MHz) | SS block frequency position SSREF | GSCN | Range of GSCN |
| --- | --- | --- | --- |
| 0 – 1000 | N * 600 kHz + M * 50 kHz + 300 kHz, N = 1:1665, M ϵ {1,3,5} (Note 1) | 26638+3N + (M-3)/2 | 26640 – 31634 |
| NOTE 1: Only applicable for 15 PRB transmission bandwidth configuration within 3 MHz channel bandwidth with punctured PBCH defined in TS 38.211 [6] clause 7.4.3.1. |  |  |  |

It was also agreed to specify 2 dedicated sync raster points for band n100 to cover the specific need of RMR when migrating from GSM-R to FRMCS: one sync raster point at the lower edge of the band n100 when transmissions span up to 12 RBs under the umbrella of a 3 MHz channel bandwidth, and another one at the lower edge of the band n100 when transmissions span up to 20 RBs under the umbrella of a 5 MHz channel bandwidth [2]:

Table 5.4.3.1-3: Additional GSCN parameters for band n100

| SS Block frequency position SSREF (MHz) | GSCN | Note |
| --- | --- | --- |
| 920.73 | 41637 | Only applicable for 12 PRB transmission bandwidth configuration within 3 MHz channel with punctured PBCH defined in TS 38.211 [6] clause 7.4.3.1. |
| 921.45 | 41638 | Only applicable for 20 PRB transmission bandwidth configuration within 5 MHz channel with unpunctured PBCH defined in TS 38.211 [6] clause 7.4.3.1. |

When the SSB width was discussed another candidate value was 15 RBs. The SSB of 15 RBs brings the 2-3 dB coverage gain compared with 12 RBs, but on the other hand there was a common understanding that the sync raster would have to be equal to the channel raster in that case, i.e. 100 kHz, which would considerably increase the UE search time and power consumption as the number of sync raster entries would increase. For this WI, there is a note in the objectives which states that “no RAN1 impact is expected”. For that reason, we propose that the same sync raster design as for the TN could be adopted for the NTN.

Proposal 5: The same sync raster design with the step size of 600 kHz and an offset of 300 kHz as for the TN FR1 3 MHz channel bandwidth could be specified for FR1-NTN.

For FR1-NTN bands n256, n255 and n254 it should be further studied whether there is a need to introduce some additional sync raster points (GSCN parameters).

Proposal 6: For FR1-NTN bands n256, n255 and n254 it should be further studied whether there is a need to introduce some additional sync raster points (GSCN parameters).

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1: This WI should only focus on 3 MHz channel bandwidth and the associated system parameters.

Proposal 2: Specify the maximum transmission bandwidth configuration NRB = 15 RB for 3 MHz channel bandwidth for all FR1-NTN bands (only consider SCS = 15 kHz).

Proposal 3: Specify the minimum guardband 142.5 kHz for SCS = 15kHz for 3 MHz channel bandwidth for all FR1-NTN bands.

Proposal 4: Specify 100 kHz channel raster for bands n256, n255 and n254 for spectrum where 3 MHz channel bandwidth is supported.

Proposal 5: The same sync raster design with the step size of 600 kHz and an offset of 300 kHz as for the TN FR1 3 MHz channel bandwidth could be specified for FR1-NTN.

Proposal 6: For FR1-NTN bands n256, n255 and n254 it should be further studied whether there is a need to introduce some additional sync raster points (GSCN parameters).

# References

- RP-241281, “Revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN”, RAN#104.

- 3GPP TS 38.101-1 V18.6.0, “NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”