3GPP TSG-RAN WG4 Meeting #112									                        R4-2413362

Maastricht, Netherlands, August 19 – 23, 2024

Agenda item:			8.8.3.2

Source:	Ericsson

Title:	On UE RF requirements for less than 5 MHz for NTN

Document for:	Approval

# Introduction

In RAN#104, a revised WID has been approved for “Enhanced requirements and conductive test methodology for NR NTN and IoT NTN” WI [1]. One of the three working areas of the WI is to specify less than 5 MHz for NTN with the following objectives:

- Specify the system parameters for channel bandwidth of less than 5 MHz for NR-NTN (NR based Non-Terrestrial Networks) in FR1-NTN bands

  - Consider the NR-NTN bands n255, n256, and n254

  - Add less than 5MHz channel bandwidth

    - 3MHz with the RAN1 design in Rel-18

  - Investigate and if necessary, specify the additional synchronization (sync) raster(s) for the additional channel bandwidth

    - Leverage the work on less than 5MHz for TN

  - Note: no RAN1 impact is expected

- Specify the necessary SAN RF core requirements for NR-NTN in FR1-NTN bands

- Specify the necessary NR-NTN UE RF core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands

  - Focus on power class 3 (PC3) for Tx RF requirements

- Specify the necessary RRM core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands

- Specify the necessary signaling to support the above objectives

In this contribution, we focus on the UE RF requirements for channel bandwidths of less than 5 MHz for NR-NTN in FR1-NTN bands and we provide our initial considerations on their expected impact.

# Discussion

In the following we will analyze which Tx and Rx requirements will have an impact in the TS 38.101-5 specifications [2] by the introduction of a 3 MHz channel bandwidth and wherever applicable we propose to reuse the requirements for 3 MHz UE CBW for TN as the starting point.

Proposal 1: As the starting point, consider the Tx and Rx RF requirements specified for 3 MHz channel bandwidth for TN UE wherever possible, as presented in Table 1 and Table 2 of this contribution.

## Tx requirements

Table 1: UE RF Tx requirements overview - 3 MHz channel BW

| Requirements | Impact |
| --- | --- |
| Transmitter power |  |
| UE maximum output power | Study whether to add 3 MHz to Table 6.2.1-2 for the additional requirements for maximum mean transmission power density when NS_04N or NS_05N is signalled for band n254. |
| UE MPR | Study whether Pi/2 BPSK edge allocation needs to be increased to 1dB as for 3MHz CBW for TN. |
| UE AMPR | For the NS applicable to bands n256, n255 and n254, the following impacts have been identified in Table 6.2.3.1-1: - NS_01: 3 MHz shall be added to the list of channel BWs. - NS_24: 3 MHz shall be added to the list of channel BWs. - NS_02N: 3 MHz shall be added to the list of channel BWs. - NS_100: No impact identified. - NS_03N: 3 MHz shall be added to the list of channel BWs. - NS_04N: it should be studied whether 3 MHz should be added. - NS_05N: 3 MHz shall be added to the list of channel BW. |
| Configured transmitted power | No specification impact. |
| Output power dynamics |  |
| Minimum output power | 3 MHz shall be added in table 6.3.1-1 and the same value as for TN could be reused: -40dBm. |
| Transmit OFF power | 3 MHz shall be added in table 6.3.2-1 and the same value as for TN could be reused: -50dBm (which is channel agnostic). |
| Transmit ON/OFF time mask | No impact expected. |
| Power control | No impact expected. |
| Transmitted signal quality |  |
| Frequency error | No impact expected. |
| Transmit modulation quality | No impact expected. |
| Output RF spectrum emissions |  |
| Occupied bandwidth | 3 MHz shall be added in table 6.5.1-1 |
| Out of band emissions | 3 MHz shall be added in table 6.5.2.2-1 (SEM), same values as for TN should be specified. 3 MHz shall be added in table 6.5.2.4.1-1 (NR ACLR measurement bandwidth) in the same column as 5, 10, 15 and 20 MHz channel BW (the same as for TN). |
| Spurious emissions |  |
| General spurious | Study whether the same boundary FOOB = 6MHz as for TN should be specified for 3MHz channel bandwidth. |
| Spurious emissions for UE-coexistence | To be further studied. |
| Transmit intermodulation | No impact expected. |

## Rx requirements

Table 2: UE RF Rx requirements overview - 3 MHz channel BW

| Requirements | Impact |
| --- | --- |
| Reference sensitivity level | REFSENS limit and associated uplink configuration shall be specified for the considered bands. |
| Maximum input level | 3 MHz shall be added in table 7.4-1, same column as for 5, 10, 15, 20 MHz channel BW (same as for TN). |
| ACS | 3 MHz shall be added in table 7.5-1, same column as for 5 and 10 MHz channel BW (same as for TN). For tables 7.5-2 and 7.5-3, 3 MHz channel BW shall also be added but considering a 3 MHz interferer BW and 3/-3 interferer offset. |
| Blocking characteristics |  |
| In-band blocking | 3 MHz shall be added in table 7.6.2-1 considering a 3 MHz interferer and consider whether the same interference offsets could be reused from TN. |
| Out of band blocking | 3 MHz shall be added in table 7.6.3-1, same value as for 5 MHz channel BW could be adopted (same as for TN). |
| Narrow band blocking | 3 MHz shall be added in table 7.6.4-1, considering 18dB Pw (same as for TN). |
| Spurious response | 3 MHz shall be added in table 7.7-1, same column as for 5 and 10 MHz channel BW (same as for TN). |
| Intermodulation characteristics | 3 MHz shall be added in table 7.8.2-1, considering REFSENS +8dB for Pw, 3 MHz bandwidth for interferer 2 and interferer offset 1 at ±BW/2 ± 4.5 MHz (same as for TN). |
| Spurious emissions | No impact expected. |

# Conclusion

Based on the discussion in the previous sections we have the following proposal:

Proposal 1: As the starting point, consider the Tx and Rx RF requirements specified for 3 MHz channel bandwidth for TN UE wherever possible, as presented in Table 1 and Table 2 of this contribution.

# References

- RP-241281, “Revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN”, RAN#104.

- 3GPP TS 38.101-5 V18.6.0, “NR; User Equipment (UE) radio transmission and reception; Part 5: Satellite access Radio Frequency (RF) and performance requirements”