3GPP TSG-RAN WG4 Meeting #112									                        R4-2413363

Maastricht, Netherlands, August 19 – 23, 2024

Agenda item:			8.8.3.3

Source:	Ericsson

Title:	On SAN RF core requirements for less than 5 MHz for NTN

Document for:	Approval

# Introduction

In RAN#104, a revised WID has been approved for “Enhanced requirements and conductive test methodology for NR NTN and IoT NTN” WI [1]. One of the three working areas of the WI is to specify less than 5 MHz for NTN with the following objectives:

- Specify the system parameters for channel bandwidth of less than 5 MHz for NR-NTN (NR based Non-Terrestrial Networks) in FR1-NTN bands

  - Consider the NR-NTN bands n255, n256, and n254

  - Add less than 5MHz channel bandwidth

    - 3MHz with the RAN1 design in Rel-18

  - Investigate and if necessary, specify the additional synchronization (sync) raster(s) for the additional channel bandwidth

    - Leverage the work on less than 5MHz for TN

  - Note: no RAN1 impact is expected

- Specify the necessary SAN RF core requirements for NR-NTN in FR1-NTN bands

- Specify the necessary NR-NTN UE RF core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands

  - Focus on power class 3 (PC3) for Tx RF requirements

- Specify the necessary RRM core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands

- Specify the necessary signaling to support the above objectives

In this contribution, we focus on the SAN RF core requirements for channel bandwidths of less than 5 MHz for NR-NTN in FR1-NTN bands and we provide our initial considerations on their expected impact.

# Discussion

In the following we will analyze which Tx and Rx requirements will have an impact in the TS 38.108 specifications [2] by the introduction of a 3 MHz channel bandwidth and wherever applicable we propose to reuse the requirements for 3 MHz BS CBW for TN as the starting point.

Proposal 1: As the starting point, consider reusing the Tx and Rx RF requirements specified for 3 MHz channel bandwidth for TN BS wherever possible, as presented in Table 1 and Table 2 of this contribution.

## Tx requirements

Table 1: SAN RF Tx requirements overview - 3 MHz channel BW

| Requirements | Impact |
| --- | --- |
| SAN output power | No impact expected. |
| Output power dynamics |  |
| RE power control dynamic range | No impact expected. |
| Total power dynamic range | Requirement for 3 MHz channel BW shall be added, consider 11.7dB as for TN BS. |
| Transmit ON/OFF power | No impact expected. |
| Transmitted signal quality |  |
| Frequency error | No impact expected. |
| Modulation quality | No impact expected. |
| Time alignment error | No impact expected. |
| Unwanted emissions |  |
| Occupied bandwidth | No impact expected. |
| Adjacent Channel Leakage Power Ratio | Requirement for 3 MHz channel BW shall be added for both GEO and LEO class. |
| Out-of-band emissions | Study whether a new OBUE mask shall be specified for 3MHz (for OOBE basic limits). |
| Transmitter spurious emissions | No impact expected. |
| Transmitter intermodulation | No impact expected. |

## Rx requirements

Table 2: SAN RF Rx requirements overview - 3 MHz channel BW

| Requirements | Impact |
| --- | --- |
| Reference sensitivity level | Requirement for 3 MHz channel BW shall be added in Table 7.2.2-1 and Table 7.2.2-2. Consider scaling from 5 MHz. Study whether the existing FRC G-FR1-A1-7 can be reused for 3 MHz CBW Refsens (same as for TN BS). |
| Dynamic range | Requirement for 3 MHz channel BW shall be added. Add minimum requirements for 3 MHz in Table 7.3.2-1. Consider scaling from 5 MHz. Study whether to introduce FRC G-FR1-A2-15 for 3 MHz (same as for TN BS). |
| ACS | Requirement for 3 MHz channel BW shall be added. Study whether PREFSENS+8dB wanted signal mean power should be specified in Table 7.4.1.2-1. In Table 7.4.1.2-2, study whether the same type of interfering signal and the interfering signal center frequency offset could be reused from 3 MHz BS CBW for TN. |
| In-band blocking | No impact expected. |
| Out of band blocking | No impact expected. |
| Receiver spurious emission | No impact expected. |
| Receiver intermodulation | No impact expected. |
| In-channel selectivity | Requirement for 3 MHz channel BW (interferer) shall be added in Table 7.8.2-1 and Table 7.8.2-2. Study whether to introduce ICS FRC G-FR1-A1-20 for 3 MHz (same as for TN BS). |

# Conclusion

Based on the discussion in the previous sections we have the following proposal:

Proposal 1: As the starting point, consider reusing the Tx and Rx RF requirements specified for 3 MHz channel bandwidth for TN BS wherever possible, as presented in Table 1 and Table 2 of this contribution.

# References

- RP-241281, “Revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN”, RAN#104.

- 3GPP TS 38.108 V18.3.0, “NR; Satellite Access Node radio transmission and reception”