3GPP TSG-RAN WG4 Meeting #112									                        R4-2413365

Maastricht, Netherlands, August 19 – 23, 2024

Agenda item:			8.8.2.2

Source:	Ericsson

Title:	On UE RF Tx requirements for NTN HPUE

Document for:	Approval

# Introduction

In RAN#104, a revised WID has been approved for “Enhanced requirements and conductive test methodology for NR NTN and IoT NTN” WI [1]. One of the three working areas of the WI is to specify High power UE (HPUE) for NTN with the following objectives:

- Conduct the coexistence analysis based on TR 38.863 for the above example bands and corresponding power classes

  - Evaluate the impact on the uplink performance of TN (Terrestrial network)

    - Example bands include bands n256 and 256, n255 and 255

    - PC2, PC1.5 (for NR-NTN only) and PC1 will be evaluated for ACLR and ACS

    - For NR-NTN and IoT-NTN: coexistence studies shall be carried out under the same conditions, including geographical separation between NTN UE and Terrestrial Networks (TN), as the co-existence studies in Rel-17 and Rel-18

- Specify high power UE (HPUE) for NR-NTN (Non-Terrestrial Networks) and IoT-NTN (NB-IoT and eMTC based NTN) in FR1-NTN bands and the corresponding LTE NTN bands for the single uplink (UL) carrier scenario

  - Support UE with power class 2 (PC2, 26 dBm) that applies to both handheld and non-handheld based on the co-existence study.

  - Support other power classes based on the co-existence and feasibility study

    - Non-hand held UE is prioritized

  - Specify RF requirements for all considered power classes for NR-NTN and IoT-NTN in FR1-NTN bands and the corresponding LTE NTN bands

    - Necessary Tx requirements, at least including

      - Investigate and if necessary, specify the solution to address potential SAR (Specific Absorption Rate restriction) requirements for handheld UE

      - Specify MPR and A-MPR for the example bands considering the regulation for the corresponding bands, if necessary

    - Necessary Rx requirements, at least including

      - Specify the RSD requirements on the example bands, if necessary

- Specify the necessary signaling to support the above objectives

In this contribution, we provide our initial considerations on the supported power classes in this WI impact on Tx requirements for HPUE for NTN.

# Discussion

## Supported power classes

According to the WID objectives [1], the main goal of this WI is to specify the power class 2 (PC2, 26dBm) that applies to both handheld and non-handheld UEs based on the co-existence study. Support of other power classes will be based on the co-existence and feasibility study, which in our understanding means that their support is not guaranteed as it will depend on the outcome of the feasibility study. Therefore, until the feasibility of other power classes is confirmed, we propose that RAN4 should focus on specifying power class 2.

Proposal 1: Until the feasibility of other power classes is confirmed based on the co-existence and feasibility study, we propose that RAN4 should focus on specifying power class 2.

In one of the objectives of the coexistence analysis part, particularly PC1.5 and PC1 are mentioned for NR-NTN and PC1 only for IoT-NTN. In TS38.101-1 [2], the UE RF requirements for TN, PC1.5 is specified for TDD bands only. Unless there is a specific need to introduce PC1.5 for FR1-NTN bands n256, n255 and n254 (which are all FDD bands), we propose to remove PC1.5 from the scope of this WI.

Proposal 2: Since PC1.5 is specified for TN TDD bands only, unless there is a specific need to introduce PC1.5 for FR1-NTN bands n256, n255 and n254 (which are all FDD bands), we propose to remove PC1.5 from the scope of this WI.

When it comes to UE type, according to the WID objectives both handheld and non-handheld UEs are in the scope, where for other than PC2 high power classes “non-handheld UE is prioritized”. In TS38.101-1 [2] “Table 6.2-1: UE Power Class”, there is a note that “Generally, PC1 UE is not targeted for smartphone form factor.” As the starting point, the same assumption should hold for FR1 NTN. Similar is the case with LTE bands as stated in TS36.101 [3]. Thus, unless NTN handheld UEs prove to be able to support PC1 and there is a specific need to introduce it, we propose to target PC1 for non-handheld UEs only.

Proposal 3: Similar to FR1 TN, for both FR1-NTN and IoT-NTN PC1 should be targeted for non-handheld UEs only.

## SAR requirements for handheld UEs

In TS38.101-1 [2] clause “6.2.1 UE maximum output power”, Specific Absorption Rate restriction (SAR) requirements are specified for PC2 and PC1.5. As the starting point, we propose to reuse the same requirements for FR1-NTN handheld UEs as well (but to keep only PC2 related requirements).

As part of the discussions on the scope of this WI, there were some ideas shared that perhaps UEs do not have to comply with SAR requirements at least in some extreme use cases. For example, one could argue that in an emergency situation in a very remote area where a TN is not available, the ability to establish the reliable communication with a high FR1-NTN power class outweighs the risk to the health by exposing to more than recommended strength of EMF. However, in our view, since SAR requirements are regulatory ones which always need to be respected, these requirements should be specified for handheld UEs and have no exceptions when it comes to their applicability.

In LTE, SAR requirements do not apply for the only FDD band for which PC2 is specified (n14). So, it is FFS how SAR requirements would be specified for FR1-NTN bands.

Proposal 4: As the starting point, for FR1-NTN PC2 handheld UEs (at least smartphones) consider reusing the same SAR requirements as for TN PC2 handheld UEs.

## Other impacted Tx requirements

In the following we will provide the analysis on the expected impact of PC2 introduction (PC1 introduction depends on the outcome of the feasibility study) on Tx requirements in TS38.101-5 and TS36.102 specifications, respectively.

Table 1: Impact of PC2 introduction on Tx requirements in TS38.101-5

| Requirements | Impact |
| --- | --- |
| Transmitter power |  |
| UE maximum output power | Introduction of PC2 in Table 6.2.1-1. |
| UE MPR | Study whether MPR requirements should be different between FR1 TN and FR1-NTN for PC2 (for PC3 MPR requirements are the same). |
| UE AMPR | Study whether there would be an impact on NS_03N, NS_04N or NS_05N. |
| Configured transmitted power | No impact expected. |
| Output power dynamics |  |
| Minimum output power | No impact expected. |
| Transmit OFF power | No impact expected. |
| Transmit ON/OFF time mask | No impact expected. |
| Power control | No impact expected. |
| Transmitted signal quality |  |
| Frequency error | No impact expected. |
| Transmit modulation quality | No impact expected. |
| Output RF spectrum emissions |  |
| Occupied bandwidth | No impact expected. |
| Out of band emissions | Add a PC2 column to Table 6.5.2.4.1-2 for NR ACLR and Table 6.5.2.4.2-1 for UTRA ACLR. The values will depend on the outcome of the coexistence study. |
| Spurious emissions | No impact expected. |
| Transmit intermodulation | No impact expected. |

Table 2: Impact of PC2 introduction on Tx requirements in TS36.102

| Requirements | Impact |
| --- | --- |
| Transmitter power |  |
| UE maximum output power | Introduction of PC2 in Table 6.2A.1-1 and Table 6.2B.1-1. |
| UE MPR | Study whether MPR requirements should be different between IoT-TN and IoT-NTN for PC2 (for PC3 MPR requirements are the same). |
| UE AMPR | Study whether there would be an impact on NS_24 (if PC2 column should be added) in Table 6.2A.3-1, Table 6.2A.3-2 and Table 6.2B.3-1. |
| Configured transmitted power | No impact expected. |
| Output power dynamics |  |
| Minimum output power | No impact expected. |
| Transmit OFF power | No impact expected. |
| Transmit ON/OFF time mask | No impact expected. |
| Power control | No impact expected. |
| Transmitted signal quality |  |
| Frequency error | No impact expected. |
| Transmit modulation quality | No impact expected. |
| Output RF spectrum emissions |  |
| Occupied bandwidth | No impact expected. |
| Out of band emissions | Study whether there is an impact by introducing PC2. |
| Spurious emissions | Study whether there is an impact on spurious emission for UE co-existence. |
| Transmit intermodulation | No impact expected. |

# Conclusion

Based on the discussion in the previous sections we have the following proposals:

Proposal 1: Until the feasibility of other power classes is confirmed based on the co-existence and feasibility study, we propose that RAN4 should focus on specifying power class 2.

Proposal 2: Since PC1.5 is specified for TN TDD bands only, unless there is a specific need to introduce PC1.5 for FR1-NTN bands n256, n255 and n254 (which are all FDD bands), we propose to remove PC1.5 from the scope of this WI.

Proposal 3: Similar to FR1 TN, for both FR1-NTN and IoT-NTN PC1 should be targeted for non-handheld UEs only.

Proposal 4: As the starting point, for FR1-NTN PC2 handheld UEs (at least smartphones) consider reusing the same SAR requirements as for TN PC2 handheld UEs.

# References

- RP-241281, “Revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN”, RAN#104.

- 3GPP TS 38.101-1 V18.6.0, “NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”

- 3GPP TS 36.101 V18.6.0, “Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception”