3GPP TSG-RAN WG4 Meeting #112									                        R4-2413366

Maastricht, Netherlands, August 19 – 23, 2024

Agenda item:			8.8.2.3

Source:	Ericsson

Title:	On UE RF Rx requirements for NTN HPUE

Document for:	Approval

# Introduction

In RAN#104, a revised WID has been approved for “Enhanced requirements and conductive test methodology for NR NTN and IoT NTN” WI [1]. One of the three working areas of the WI is to specify High power UE (HPUE) for NTN with the following objectives:

- Conduct the coexistence analysis based on TR 38.863 for the above example bands and corresponding power classes

  - Evaluate the impact on the uplink performance of TN (Terrestrial network)

    - Example bands include bands n256 and 256, n255 and 255

    - PC2, PC1.5 (for NR-NTN only) and PC1 will be evaluated for ACLR and ACS

    - For NR-NTN and IoT-NTN: coexistence studies shall be carried out under the same conditions, including geographical separation between NTN UE and Terrestrial Networks (TN), as the co-existence studies in Rel-17 and Rel-18

- Specify high power UE (HPUE) for NR-NTN (Non-Terrestrial Networks) and IoT-NTN (NB-IoT and eMTC based NTN) in FR1-NTN bands and the corresponding LTE NTN bands for the single uplink (UL) carrier scenario

  - Support UE with power class 2 (PC2, 26 dBm) that applies to both handheld and non-handheld based on the co-existence study.

  - Support other power classes based on the co-existence and feasibility study

    - Non-hand held UE is prioritized

  - Specify RF requirements for all considered power classes for NR-NTN and IoT-NTN in FR1-NTN bands and the corresponding LTE NTN bands

    - Necessary Tx requirements, at least including

      - Investigate and if necessary, specify the solution to address potential SAR (Specific Absorption Rate restriction) requirements for handheld UE

      - Specify MPR and A-MPR for the example bands considering the regulation for the corresponding bands, if necessary

    - Necessary Rx requirements, at least including

      - Specify the RSD requirements on the example bands, if necessary

- Specify the necessary signaling to support the above objectives

In this contribution, we provide our initial analysis on the impact of the introduction of higher power classes for FR1-NTN and IoT-NTN in this WI on Rx requirements.

# Discussion

For FR1-NTN, the only identified potential impact on Rx requirements by introducing PC2 is on a reference sensitivity. In TS38.101-1[2], in clause 7.3.2 there is a Table 7.3.2-1c on “Reference Sensitivity Degradation from PC3 to PC2 for FDD bands for UE not supporting Tx Diversity”, where the allowed reference sensitivity degradation is specified for FDD bands. Since in FR1-NTN all specified bands are FDD (n256, n255 and n254), the similar table should be studied to be introduced to clause 7.3.2 on “Reference sensitivity power level” of TS38.101-5 specifications.

Proposal 1: Study the introduction of a table on reference sensitivity degradation from PC3 to PC2 in clause 7.3.2 of TS38.101-5.

For IoT-NTN, the only identified potential impact on Rx requirements by introducing PC2 is also on a reference sensitivity. In TS36.101[3], in clause 7.3.1 there is Table 7.31-1 on “Reference sensitivity QPSK PREFSENS”, where a separate value for the reference sensitivity for PC2 is specified for n14 FDD band. For bands 253, 254, 255 and 256, it should be studied whether to introduce separate tables or to add additional rows in the existing tables “Table 7.3A-1: Reference sensitivity for FDD UE category M1 QPSK PREFSENS” and “Table 7.3A-2: Reference sensitivity for HD-FDD UE category M1 QPSK PREFSENS” in clause 7.3A and “Table 7.3B-1: Reference sensitivity for UE category NB1 and NB2” in clause 7.3B.

Proposal 2: Study the introduction of tables/adding additional rows in the existing tables on reference sensitivity degradation from PC3 to PC2 in clauses 7.3A and 7.3B of TS36.102.

# Conclusion

Based on the discussion in the previous sections we have the following proposals:

Proposal 1: Study the introduction of a table on reference sensitivity degradation from PC3 to PC2 in clause 7.3.2 of TS38.101-5.

Proposal 2: Study the introduction of tables/adding additional rows in the existing tables on reference sensitivity degradation from PC3 to PC2 in clauses 7.3A and 7.3B of TS36.102.

# References

- RP-241281, “Revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN”, RAN#104.

- 3GPP TS 38.101-1 V18.6.0, “NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”

- 3GPP TS 36.101 V18.6.0, “Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception”