3GPP TSG-RAN WG4 Meeting #112									                        R4-2413367

Maastricht, Netherlands, August 19 – 23, 2024

Agenda item:			8.18.2

Source:	Ericsson

Title:	On UE RF requirements impact for NR MIMO Phase 5

Document for:	Approval

# Introduction

In RAN#104, a revised WID has been approved for a RAN1-led WI on “NR MIMO Phase 5” WI [1]. The objectives of the WI are the following:

RAN1:

- Specify enhancement to facilitate UE-initiated/event-driven beam management for reducing overhead and/or latency, assuming the unified TCI while leveraging (as much as possible) legacy CSI measurement and reporting configuration frameworks, targeting FR2 and sTRP with intra- and inter-cell beam management

  - UL signaling content(s) (and procedure(s) as required) for UE-initiated/event-driven beam reporting facilitating fast beam switching

  - UL signaling medium/container considering the UE-initiated/event-driven nature of the UL transmission, designed primarily for the purpose of beam reporting

- Specify CSI support for up to 128 CSI-RS ports, targeting FR1

  - Type-I codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks

  - Type-II codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks, without modifying any codebook parameter other than introducing additional values for the number of ports codebook parameter(s)

  - Extension of CRI(s)-based CSI reporting (CQI/PMI/RI calculated per CRI for ≥1 CRIs) for hybrid beamforming supporting up to a total of 128 CSI-RS ports across all resources, with up to 32 CSI-RS ports per resource, without new codebook design

- Specify UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul, targeting FR1, both FDD and TDD

- Inter-TRP time misalignment and frequency/phase offset measurement and reporting, assuming legacy CSI-RS design, with stand-alone aperiodic reporting on PUSCH

- Specify non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions, without enhancement on UL full power transmission and without enhancement on SRS resource

Note: UL full power transmission mode 1 and 2 are not supported.

- Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2

  - Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP.

Acronyms:

- sTRP: single TRP (transmit-receive point)

- mTRP: multiple TRP (transmit-receive point)

- CJT: coherent joint transmission

- DU: distributed unit

- CRI: CSI-RS resource indicator

RAN2:

Specify higher layer support of the enhancements listed above.

RAN4:

Specify necessary core requirements for the enhancements listed above.

In this contribution, we analyze the impact of “Support for 3-antenna-port codebook-based transmissions” working area on the UE RF requirements in RAN4.

# Discussion

In current specifications TS38.101-1 [2], 2-antenna-port and 4-antenna-port transmissions are specified in clause “6.2D.1 UE maximum output power for UL MIMO”. In the same clause, the requirements for 3-antenna-port codebook-based transmissions could be specified.

Observation 1: In current specifications TS38.101-1, 2-antenna-port and 4-antenna-port transmissions are specified in clause “6.2D.1 UE maximum output power for UL MIMO”.

Proposal 1: The requirements for 3-antenna-port codebook-based transmissions could be specified in clause “6.2D.1 UE maximum output power for UL MIMO” of TS38.101-1.

According to RAN1 agreement in [3]:

Agreement For non-coherent uplink precoding by a 3TX UE, at least following precoders are supported for three-layer transmission. 13100010001

According to RAN1 agreement in [4]:

Agreement For codebook-based UL transmission by a 3TX UE, subject to its capability, - A 3TX UE may report a maximum number of 3 layers - A 3TX UE may report a maximum number of SRS ports of up to 3 Note: SRS resource definition is not changed nor the number of SRS ports in the SRS resource.

Proposal 2: For 3-layer case, the maximum output power requirements shall be met with the UL MIMO configurations of using 3-layer UL MIMO codebook-based transmission with precoding matrix of W=13100010001.

Proposal 3: Regarding the UL MIMO configuration in closed-loop spatial multiplexing scheme for which the maximum output power requirements shall be met, a row can be added to Table 6.2D.1-2 in TS38.101-1 for 3 layers and TPMI index [0] (to be further confirmed).

According to a WID objective [1], UL full power transmission mode 1 and 2 are not supported. Regarding the support of full-power Mode 0, in [3] there was a RAN1 agreement:

Agreement For codebook-based uplink transmission by a 3TX UE, support full-power Mode 0, subject to UE capability.

On the other hand, there was an agreement in [3] on the supported precoders for single-layer transmission:

Agreement For non-coherent uplink precoding by a 3TX UE, at least following precoders are supported for single-layer transmission. 13100,13010,13001

Thus, we make the following proposal:

Proposal 4: For the UE supporting full-power Mode 0 for UL MIMO, a PUSCH configuration for 3 Tx ports and 1 layer could be added to “Mode-full power” row of Table 6.2D.1-3 in TS38.101-1, with TPMI index [0,1,2] (to be further confirmed).

# Conclusion

Based on the discussion in the previous sections we propose the following observation:

Observation 1: In current specifications TS38.101-1, 2-antenna-port and 4-antenna-port transmissions are specified in clause “6.2D.1 UE maximum output power for UL MIMO”.

We also make the following proposals:

Proposal 1: The requirements for 3-antenna-port codebook-based transmissions could be specified in clause “6.2D.1 UE maximum output power for UL MIMO” of TS38.101-1.

Proposal 2: For 3-layer case, the maximum output power requirements shall be met with the UL MIMO configurations of using 3-layer UL MIMO codebook-based transmission with precoding matrix of W=13100010001.

Proposal 3: Regarding the UL MIMO configuration in closed-loop spatial multiplexing scheme for which the maximum output power requirements shall be met, a row can be added to Table 6.2D.1-2 in TS38.101-1 for 3 layers and TPMI index [0] (to be further confirmed).

Proposal 4: For the UE supporting full-power Mode 0 for UL MIMO, a PUSCH configuration for 3 Tx ports and 1 layer could be added to “Mode-full power” row of Table 6.2D.1-3 in TS38.101-1, with TPMI index [0,1,2] (to be further confirmed).

# References

- RP-240087, “WID revision: NR MIMO Phase 5”, RAN#104.

- 3GPP TS 38.101-1 V18.6.0, “NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”

- R1-2401937, “Final Report of 3GPP TSG RAN WG1 #116 v1.0.0”, MCC Support

- R1-2405782, “Final Report of 3GPP TSG RAN WG1 #117 v1.0.0”, MCC Support