3GPP TSG-WG Meeting #112	R4-2413375

Maastricht, Netherlands, 19th – 23rd August, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 4936 | rev | - | Current version: | 16.20.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_unlic-Perf) CR for NR-U TC correction – Clause A.12 (R16) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_unlic-Perf |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - Some editorial corrections in the tables - Update tables with OCNG related text agreed in last meetings. - Incorrect per FR gap configurations - Incorrect Io values |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Some editorial corrections in the tables - Updated tables with OCNG related text agreed in last meetings. - Removed per FR gap configurations as agreed in other WIs - Corrected Io values |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect test configuration |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.12 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### Start of change 1

# A.12	E-UTRA Standalone Tests with at Least One NR Cell under CCA

## A.12.1	RRC_IDLE state mobility

### A.12.1.1	Inter-RAT cell re-selection to NR on a carrier frequency with CCA

#### A.12.1.1.1	E-UTRA Cell reselection to higher priority NR target Cell in FR1 when target cell is subject to CCA

##### A.12.1.1.1.1	Test Purpose and Environment

This test is to verify the requirement for the E-UTRAN to NR inter-RAT cell subject to CCA reselection requirements specified in clause 4.2.2.5.7 in TS 36.133 [15].

The test scenario comprises of 1 E-UTRA cell and 1 NR cell subject to CCA as given in tables A.12.1.1.1.1-1, A.8.2.1.1.1-2, A.8.2.1.1.1-3 and A.8.2.1.1.1-4. The test consists of three successive time periods, with time duration of T1, T2, and T3 respectively. E-UTRA cell 1 is already identified by the UE prior to the start of the test. Cell 2 is of higher priority than cell 1.

Table A.12.1.1.1.1-1: Supported test configurations

| Configuration | Description of a cell without CCA | Description of a cell with CCA |
| --- | --- | --- |
| 1 | LTE FDD | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 2 | LTE TDD | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |  |

Table A.12.1.1.1.1-2: General test parameters for E-UTRA cell re-selection FR1 NR cell subject to CCA test case

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2 | Cell2 | The UE camps on cell 2 in the initial phase |
|  | Neighbour cell |  | 1, 2 | Cell1 |  |
| T1 end condition | Active cell |  |  | Cell1 | During T1 period the UE reselects to cell 1 |
|  | Neighbour cell |  |  | Cell2 |  |
| T3 end condition | Active cell |  | 1, 2 | Cell2 | The UE shall perform reselection to cell 2 during T3 |
|  | Neighbour cell |  | 1, 2 | Cell1 |  |
| RF Channel Number |  |  | 1, 2 | 1, 2 | E-UTRAN radio channel (1) and NR radio channel (2) are used for this test |
| Time offset between cells |  |  | 1, 2 | 3 s | Synchronous cells |
| Access Barring Information |  | - | 1, 2 | Not Sent | No additional delays in random access procedure. |
| DBT Window Configuration |  |  | 1, 2 | DBT.1 | As specified in clause A.3.28.1. |
| DL CCA model |  |  | 1, 2 | As specified in clause A.3.26.2.1 | DL CCA model |
| UL CCA model |  |  | 1, 2 | As specified in clause A.3.26.2.2 | UL CCA model |
| DRX cycle length |  | s | 1, 2 | 1.28 | The value shall be used for all cells in the test. |
| NR PRACH configuration index |  |  | 1, 2 | 102 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| T1 |  | s | 1, 2 | 15 | T1 needs to be defined so that cell re-selection reaction time is taken into account. |
| T2 |  | s | 1, 2 | >7 | During T2, cell 2 shall be powered off, and during the off time the physical cell identity shall be changed. The intention is to ensure that cell 2 has not been detected by the UE prior to the start of period T3. |
| T3 |  | s | 1, 2 | 75 | T3 needs to be defined so that cell re-selection reaction time is taken into account. |

Table A.12.1.1.1.1-3: Cell specific test parameters for NR cell 2 subject to CCA

| Parameter | Unit | Test configuration | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| TDD configuration |  | 1, 2 | TDDConf.1.1.CCA |  |  |
| DL CCA probability for semi-static channel access (PCCA_DL) |  | 1, 2 | 0.9 |  |  |
| DL CCA probability for for dynamic static channel access (PCCA_DL_1) |  | 1, 2 | 0.75 |  |  |
| DL CCA probability for for dynamic static channel access (PCCA_DL_2) |  | 1, 2 | 0.5 |  |  |
| UL CCA probability PCCA_UL |  | 1, 2 | 1 |  |  |
| Md,max |  | 1, 2 | 16 |  |  |
| Mm,max |  | 1, 2 | 4 |  |  |
| Me,max |  | 1, 2 | 8 |  |  |
| PDSCH Reference measurement channel |  | 1, 2 | SR.1.1 CCA |  |  |
| RMSI CORESET Reference Channel |  | 1, 2 | CR.1.1 CCA |  |  |
| RMC CORESET Reference Channel |  | 1, 2 | CCR.1.1 CCA |  |  |
| OCNG Patterns |  | 1, 2 | OP.1 |  |  |
| SSB configuration |  | 1, 2 | SSB.1 CCA Note4 SSB.2 CCA Note5,6 |  |  |
| Initial DL BWP configuration |  | 1, 2 | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | 1, 2 | ULBWP.0.1 |  |  |
| RLM-RS |  | 1, 2 | SSB |  |  |
| Qrxlevmin | dBm/SCS | 1, 2 | -137 |  |  |
| Pcompensation | dB | 1, 2 | 0 |  |  |
| Qhysts | dB | 1, 2 | 0 |  |  |
| Qoffsets, n | dB | 1, 2 | 0 |  |  |
| Cell_selection_and_ reselection_quality_measurement |  | 1, 2 | SS-RSRP |  |  |
|  | dB | 1, 2 | -4 | -infinity | 12 |
| Note2 | dBm/SCS | 1, 2 | -95 |  |  |
| Note2 | dBm/15 kHz | 1, 2 | -98 |  |  |
|  | dB | 1, 2 | -4 | -infinity | 12 |
| SS-RSRP Note3 | dBm/SCS | 1, 2 | -99 | -infinity | -83 |
| Io | dBm/38.16 MHz | 1, 2 | -62.50 | -63.95 | -51.69 |
| Treselection | s | 1, 2 | 0 | 0 | 0 |
| SnonintrasearchP | dB | 1, 2 | 50 |  |  |
| Threshx, highP | dB | 1, 2 | 48 |  |  |
| Threshserving, lowP | dB | 1, 2 | 44 |  |  |
| Threshx, lowP | dB | 1, 2 | 50 |  |  |
| Propagation Condition |  | 1, 2 | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. . For cells with CCA model, OCNG is transmitted only in the slots with downlink transmission burst and is not transmitted during the muted slots or during DBT window. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 5: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. Note 6: For UE supporting both semi-static and dynamic channel access, the UE must be tested under dynamic channel access configuration. |  |  |  |  |  |

Table A.12.1.1.1.1-4: Cell specific test parameters for E-UTRA cell 1

| Parameter | Unit | Cell 1 |  |  |
| --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 |
| E-UTRA RF Channel number |  | 1 |  |  |
| BWchannel | MHz | 10 |  |  |
| OCNG Patterns defined in TS 36.133 [15] clause A.3.2 |  | OP.2 TDD for test configuration 1, 2, 3; OP.2 FDD for test configuration 4, 5, 6 |  |  |
| PBCH_RA | dB | 0 |  |  |
| PBCH_RB | dB |  |  |  |
| PSS_RA | dB |  |  |  |
| SSS_RA | dB |  |  |  |
| PCFICH_RB | dB |  |  |  |
| PHICH_RA | dB |  |  |  |
| PHICH_RB | dB |  |  |  |
| PDCCH_RA | dB |  |  |  |
| PDCCH_RB | dB |  |  |  |
| PDSCH_RA | dB |  |  |  |
| PDSCH_RB | dB |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |
| Qrxlevmin | dBm | -140 |  |  |
| Note 2 | dBm/15 kHz | -98 |  |  |
| RSRP Note 3 | dBm/15 KHz | -84 | -84 | -84 |
|  | dB | 14 | 14 | 14 |
|  | dB | 14 | 14 | 14 |
| TreselectionEUTRAN | S | 0 |  |  |
| SnonintrasearchP | dB | 50 |  |  |
| Threshx, highP | dB | 48 |  |  |
| Threshserving, lowP | dB | 44 |  |  |
| Threshx, lowP | dB | 50 |  |  |
| Propagation Condition |  | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

##### A.12.1.1.1.2	Test Requirements

The cell reselection delay to a higher priority NR cell subject to CCA is defined as the time from the beginning of time period T3, to the moment when the UE camps on cell 2, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Registration procedure for mobility and periodic registration updateon cell 2.

The cell re-selection delay to a higher priority cell shall be less than 60 + 1.28 x (5 + Me) + TSI_CCA s. Me is the number of DRX cycles with at least one SMTC where there are no SSBs available during the Tevaluate,NR_Intra_CCA. If Me > Me,max the UE is required to restart the evaluation of cell 2.

The rate of correct cell reselections observed during repeated tests shall be at least 90%.

NOTE:	The cell re-selection delay to a higher priority cell can be expressed as: Thigher_priority_search + Tevaluate, NR_ inter_CCA + TSI_CCA, and to a lower priority cell can be expressed as: Tevaluate, NR + TSI-NR,

Where:

Thigher_priority_search	See clause 4.2.2 in TS 36.133 [15]

Tevaluate, NR_ inter_CCA	See Table 4.2.2.5.7-1 in clause 4.2.2.5.7

TSI_CCA	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell.

Tevaluate, NR	See Table 4.2.2.5.6-1 in clause 4.2.2.5.6 in TS 36.133 [15]

TSI-NR	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280 ms is assumed in this test case.

This gives a total of 67.68 s, allow 68 s for the cell re-selection delay to a higher priority NR cell and 7.68 s for the cell re-selection delay to a lower priority cell in the test case, which we allow 8 s.

## A.12.2	RRC_CONNECTED state mobility

### A.12.2.1	Handover

#### A.12.2.1.1	E-UTRAN - NR with CCA handover

##### A.12.2.1.1.1	Test Purpose and Environment

This test shall verify the E-UTRAN to NR FR1 handover requirements specified in clause 5.3.4A in TS 36.133 [15].

The test comprises of one E-UTRA carrier and one NR carrier with CCA. There are two cells and one cell on each carrier. Cell 1 is the E-UTRAN cell and Cell 2 is an inter-RAT NR neighbour cell with CCA.

The test consists of three successive time periods, with time durations of T1, T2 and T3 respectively. At the start of time duration T1, the UE does not have any timing information of Cell 2. Starting T2, Cell 2 becomes detectable and the UE is expected to detect and send a measurement report. Gap pattern configuration with id #0 as specified in Table 8.1.2.1-1 of TS 36.133 [15] is configured before T2 begins to enable inter-RAT frequency monitoring. A RRC message implying handover shall be sent to the UE during period T2 after the UE has reported Event B2. The start of T3 is the instant when the last TTI containing the RRC message implying handover is sent to the UE. The handover message shall contain Cell 2 as the target cell.

Supported test configurations are shown in table A.12.2.1.1-1. General test parameters are provided in Table A.12.2.1.1-2. Cell specific test parameters for Cell 1 and Cell 2 are provided in Tables A.12.2.1.1-3 and A.12.2.1.1-4 respectively.

Table A.12.2.1.1-1: Supported test configurations for E-UTRAN inter-RAT NR handover

| Configuration | Description |
| --- | --- |
| 1 | LTE FDD, NR with CCA 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 2 | LTE TDD, NR with CCA 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.12.2.1.1-2: General test parameters for E-UTRAN inter-RAT NR handover

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  |  | 1 | 1 NR carrier frequency with CCA is used in the test |
| LTE RF Channel Number |  |  | 2 | 1 E-UTRAN carrier frequency is used in the test |
| Initial conditions | Active cell |  | Cell 1 | E-UTRAN cell |
|  | Neighbouring cell |  | Cell 2 | NR cell with CCA |
| Final condition | Active cell |  | Cell 2 |  |
| DL CCA model |  |  | As specified in clause A.3.26.2.1 |  |
| UL CCA model |  |  | As specified in clause A.3.26.2.2 |  |
| NR measurement quantity |  |  | SS-RSRP |  |
| E-UTRAN measurement quantity |  |  | RSRP |  |
| b2-Threshold1 |  | dBm | -84 | Absolute E-UTRAN RSRP threshold for event B2 |
| b2-Threshold2NR |  | dBm | As specified in Table A.12.2.1.1-4 | Absolute NR SS-RSRP threshold for event B2 |
| Hysteresis |  | dB | 0 |  |
| TimeToTrigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| DRX |  |  | OFF | Non-DRX test |
| Access Barring Information |  | - | Not sent | No additional delays in random access procedure |
| Time offset between cells |  |  | 3 ms | Asynchronous cells |
| Gap pattern configuration Id |  |  | 0 | As specified in Table 8.1.2.1-1 started before T2 starts [15] |
| T1 |  | s | 5 |  |
| T2 |  | s | £5 |  |
| T3 |  | s | 1 |  |

Table A.12.2.1.1-3: Cell specific test parameters for E-UTRAN inter-RAT NR handover with CCA (Cell 1)

| Parameter | Unit | Configuration | Cell 1 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| RF channel number |  | 1, 2 | 2 |  |  |
| Duplex mode |  | 1 | FDD |  |  |
|  |  | 2 | TDD |  |  |
| TDD special subframe configurationNote1 |  | 1, 2 | 6 |  |  |
| TDD uplink-downlink configurationNote1 |  | 1, 2 | 1 |  |  |
| BWchannel | MHz | 1, 2 | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |  |  |
| PRACH ConfigurationNote2 |  | 1 | 4 |  |  |
|  |  | 2 | 53 |  |  |
| PDSCH parameters: DL Reference Measurement ChannelNote3 |  | 1 | 5 MHz: R.7 FDD 10 MHz: R.3 FDD 20 MHz: R.6 FDD |  |  |
|  |  | 2 | 5 MHz: R.4 TDD 10 MHz: R.0 TDD 20 MHz: R.3 TDD |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote3 |  | 1 | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |  |
|  |  | 2 | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |  |
| OCNG PatternsNote3 |  | 1 | 5 MHz: OP.20 FDD 10 MHz: OP.10 FDD 20 MHz: OP.17 FDD |  |  |
|  |  | 2 | 5 MHz: OP.9 TDD 10 MHz: OP.1 TDD 20 MHz: OP.7 TDD |  |  |
| PBCH_RA | dB | 1, 2 | 0 |  |  |
| PBCH_RB |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |
| OCNG_RANote4 |  |  |  |  |  |
| OCNG_RBNote4 |  |  |  |  |  |
| NocNote5 | dBm/15kHz | 1, 2 | -98 |  |  |
| Ês/Noc | dB | 1, 2 | 7 | 7 | 7 |
| Ês/IotNote6 | dB | 1, 2 | 7 | 7 | 7 |
| RSRPNote6 | dBm/15kHz | 1, 2 | -91 | -91 | -91 |
| SCH_RPNote6 | dBm/15kHz | 1, 2 | -91 | -91 | -91 |
| IoNote6 | dBm/9MHz | 1, 2 | -62.43 | -62.43 | -62.43 |
| Propagation Condition |  | 1, 2 | AWGN |  |  |
| Antenna Configuration and Correlation Matrix Note7 |  | 1, 2 | 1x2 Low |  |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [23]. Note 2: PRACH configurations are specified in table 5.7.1-2 and table 5.7.1-3 in TS 36.211 [23]. Note 3: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [15] respectively. Note 4: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 5: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 6: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 7: Propagation condition and correlation matrix are defined in clause B.2 in TS 36.101 [25]. |  |  |  |  |  |

Table A.12.2.1.1-4: Cell specific test parameters E-UTRAN inter-RAT NR with CCA handover (Cell 2)

| Parameter |  | Unit | Configuration | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T3 |
| RF channel number |  |  | 1, 2 | 1 |  |  |
| DL CCA probability PCCA_DL | Semi-static channel access Note 4, 6 |  | 1, 2 | PCCA_DL=0.9375 |  |  |
|  | Dynamic channel access Note 5, 6 |  | 1, 2 | PCCA_DL_1=0.75 PCCA_DL_2=0.75 |  |  |
| UL CCA probability PCCA_UL | Semi-static channel access Note 4, 6 |  | 1, 2 | PCCA_UL=0.87 |  |  |
|  | Dynamic channel access Note 5, 6 |  | 1, 2 | PCCA_UL=0.75 |  |  |
| LCCA_DL |  | - | 1, 2 | 5 |  |  |
| WCCA_DL |  | ms | 1, 2 | T304 |  |  |
| LCCA_UL |  | - | 1, 2 | 5 |  |  |
| WCCA_UL |  | ms | 1, 2 | T304 |  |  |
| T304 |  | ms | 1, 2 | 500 |  |  |
| Duplex mode |  |  | 1, 2 | TDD |  |  |
| TDD Configuration |  |  | 1, 2 | TDDConf.1.1 CCA |  |  |
| BWchannel |  | MHz | 1, 2 | 40: NRB,c = 106 (TDD) |  |  |
| PDSCH reference measurement channel |  |  | 1, 2 | SR.1.1 CCA |  |  |
| CORESET reference channel |  |  | 1, 2 | CR.1.1 CCA |  |  |
| PRACH configuration |  |  | 1, 2 | FR1 PRACH configuration 1 under CCA |  |  |
| OCNG patternNote1 |  |  | 1, 2 | OP.1 |  |  |
| BWP | Initial DL BWP |  | 1, 2 | DLBWP.0.1 |  |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  |  |
| SMTC configuration |  |  | 1, 2 | SMTC.1 |  |  |
| SSB configuration | Semi-static channel access Note 4, 6 |  | 1, 2 | SSB.1 CCA |  |  |
|  | Dynamic channel access Note 5, 6 |  | 1, 2 | SSB.2 CCA |  |  |
| DBT window configuration |  |  |  | As defined in A.3.28.1 |  |  |
| b2-Threshold2NR |  | dBm | 1 | -105 |  |  |
|  |  |  | 2 | -103 |  |  |
| EPRE ratio of PSS to SSS |  | dB | 1, 2 | 0 |  |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |  |  |  |
| NocNote2 |  | dBm/15 KHz | 1, 2 | -98 |  |  |
| NocNote2 |  | dBm/SCS | 1, 2 | -95 |  |  |
| Ês/Noc |  | dB | 1, 2 | -inifinit | 0 | 0 |
| Ês/IotNote3 |  | dB | 1, 2 | -inifinit | 0 | 0 |
| SS-RSRPNote3 |  | dBm/SCS | 1, 2 | -inifinit | -95 | -95 |
| IoNote3 |  | dBm/38.16 MHz | 1, 2 | -63.96 | -60.94 | -60.94 |
| Propagation condition |  |  | 1, 2 | AWGN |  |  |
| Antenna Configuration and Correlation Matrix |  |  | 1, 2 | 1x2 Low |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. . For cells with CCA model, OCNG is transmitted only in the slots with downlink transmission burst and is not transmitted during the muted slots or during DBT window. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Ês/Iot, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 5: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. Note 6: For UE supporting both semi-static and dynamic cannel access, the UE must be tested under both dynamic and semi-static channel occupancy configurations. |  |  |  |  |  |  |

##### A.12.2.1.1.2	Test Requirements

The UE shall start to transmit the PRACH to Cell 2 less than 112 + (L1´ + L3)*20 ms from the beginning of time period T3.

The rate of correct handovers observed during repeated tests shall be at least 90%.

NOTE:	The handover delay can be expressed as: RRC procedure delay + Tinterrupt, where:

RRC procedure delay = 50 ms and is specified in TS36.331.

Tinterrupt = 62 + ( L1´ + L3) * TSMTC; Tinterrupt is defined in TS36.133 clause 5.3.4A.3 where

L1´ is the number of SMTC occasions not available at the UE during the inter-RAT detection period.

L3 is the number of consecutive SSB to PRACH occasion association periods during which no PRACH occasion is available for PRACH transmission due to UL CCA failure.  L3 = 0 for Type 2C UL channel access procedure as defined in TS 37.213 [33].

TSMTC = 20 ms is the SMTC periodicity ms in the test.

This gives a total of 112 + (L1´ + L3)*20 ms.

## A.12.3	Void

## A.12.4	Measurement procedure

### A.12.4.1	E-UTRANNR inter-RAT SFTD measurements

#### A.12.4.1.1	E-UTRA – NR Inter-RAT SFTD Measurement Delay with NR under CCA in non-DRX

##### A.12.4.1.1.1	Test Purpose and Environment

The purpose of this test is to partly verify that measurement reporting delay for SFTD between E-UTRA PCell and inter-RAT NR neighbour cell under CCA is within the requirements stated in clauses 8.1.2.4.25 and 8.1.2.4.26 of TS 36.133 [15] for E-UTRA FDD and TDD, respectively, when no measurement gaps are provided and no DRX is configured.

The tests consist of a single time period of duration T1. Two carriers are used in the tests: one E-UTRA carrier with the PCell (Cell 1), and one NR carrier under CCA with the NR neighbour cell (Cell 2).

Prior to the start of time duration T1, the UE is connected to Cell 1 and configured to carry out intra-frequency measurements only. The point in time at which the UE receives, at the UE antenna connector(s), a RRC message containing a measurement configuration for SFTD measurements on RF channel 2 defines the start of time duration T1. Following the start of T1 the UE shall detect Cell 2, determine the SFN and frame time difference of Cell 2 relative to Cell 1, and send a measurement report.

The supported test configurations are listed in Table A.12.4.1.1.1-1 below. General test parameters and cell-specific parameters for the NR cell are provided in Tables A.12.4.1.1.1-2 and A.12.4.1.1.1-3 below, respectively. Cell-specific parameters for the E-UTRA cell are provided in clause A.3.7.2.1.

Table A.12.4.1.1.1-1: Applicable test configurations for inter-RAT SFTD measurement delay test with NR under CCA

| Config | Description |
| --- | --- |
| 1 | LTE FDD; NR: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 2 | LTE TDD; NR: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.12.4.1.1.1-2: General test parameters for inter-RAT SFTD measurement delay test with NR under CCA

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| E-UTRA RF Channel Number |  | Config 1,2 | 1 |  | One E-UTRAN carrier frequencies is used. |
| NR RF Channel Number |  | Config 1,2 | 1 |  | One NR carrier frequencies is used. |
| Active cell |  | Config 1,2 | Cell 1 |  | Cell 1 is on E-UTRA RF channel number 1. |
| Neighbour cell |  | Config 1,2 | Cell 2 |  | Cell 2 is on NR RF channel number 1. |
| CP length |  | Config 1,2 | Normal |  | Applicable to both cells. |
| DRX |  | Config 1,2 | OFF |  | DRX is not used |
| Frame time offset between serving and neighbour cells | ms | Config 1 | 3 | 7 | Asynchronous cells. The timing of Cell 2 relative to the timing of Cell 1. |
|  | ms | Config 2 | 3 |  | Synchronous cells. |
| SFN offset between serving and neighbour cells |  | Config 1,2 | 0 | 1 | SFN of Cell 2 relative to SFN of Cell 1. |
| SS-RSRP reporting |  | Config 1,2 | No |  | Only SFTD is reported. |
| T1 | s | Config 1,2 | 2 |  | T1 shall exceed Tmeasure_SFTD_LBT_max = 56 × SMTC |

Table A.12.4.1.1.1-3: Cell specific test parameters for Cell 2 in inter-RAT SFTD measurement delay test with NR under CCA

| Parameter |  | Unit | Cell 2 |
| --- | --- | --- | --- |
| NR RF Channel Number |  |  | 1 |
| Duplex mode |  |  | TDD |
| BWchannel |  | MHz | 40: NRB,c = 106 |
| TDD configuration |  |  | TDDConf.1.1 CCA |
| DL CCA model |  |  | As specified in clause A.3.26.2.1 |
| DL CCA probability for semi-static channel accessNote5,7 | PCCA_DL |  | 0.9375 |
| DL CCA probability for dynamic channel accessNote6,7 | PCCA_DL_1 |  | 0.75 |
|  | PCCA_DL_2 |  | 0.75 |
| OCNG Pattern defined in A.3.2.1.1Note 1 |  |  | OP.1 |
| SMTC configuration defined in A.3.2.11.1 and A.3.2.11.2 |  |  | SMTC.2 |
| SSB configuration for semi-static channel accessNote5,7 |  |  | SSB.1 CCA |
| SSB configuration for dynamic channel accessNote6,7 |  |  | SSB.2 CCA |
| DBT window configuration |  |  | DBT.1 |
| PDSCH/PDCCH subcarrier spacing |  | kHz | 30 |
| EPRE ratio of PSS to SSS |  | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  | dB |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |
| EPRE ratio of OCNG DMRS to SSS Note 1 |  | dB |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  | dB |  |
| Noc Note2 |  | dBm/15 kHz | -98 |
| Noc Note2 |  | dBm/SCS | -95 |
| SS-RSRP Note 3, 4 |  | dBm/SCS | -91 |
| Ês/Iot |  | dB | 4 |
| Ês/Noc |  | dB | 4 |
| Io Note 3 |  | dBm/38.16MHz | -58.50 |
| Propagation Condition |  |  | AWGN |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols in slots with downlink transmission bursts. For cells with CCA model, OCNG is transmitted only in the slots with downlink transmission burst and is not transmitted during the muted slots or during DBT window. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 6: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. Note 7: For UE supporting both semi-static and dynamic cannel access, the UE can be tested under dynamic channel access only.. |  |  |  |

##### A.12.4.1.1.2	Test Requirements

Following the start of T1, the UE shall detect Cell 2 and determine the relative time difference between Cell 1 and Cell 2. At latest at TRRC_procedure_delay + Tmeasure_SFTD_LBT_max after the beginning of time duration T1, the UE shall send a measurement report on SFTD between Cell 1 and Cell 2.

The observed rate of successful SFTD reports in repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2×TTIDCCH longer than the measurement reporting delays above due to TTI insertion uncertainty of the measurement report in DCCH.

### A.12.4.2	E-UTRANNR inter-RAT measurements on NR carrier frequency under CCA

#### A.12.4.2.1	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used

##### A.12.4.2.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the NR inter-RAT cell search requirements in clause 8.1.2.4.21A of TS 36.133 [15] for E-UTRAN FDD-NR measurements under CCA and clause 8.1.2.4.22A of TS 36.133 [15] for E-UTRAN TDD-NR measurements under CCA.

In this test, there are two cells: E-UTRA cell 1 as PCell on E-UTRA RF channel 1 and NR cell 2 as neighbour cell in FR1 on NR RF channel 1 on a carrier frequency with CCA. The test parameters are given in Tables A.12.4.2.1.1-1, A.12.4.2.1.1-2, A.12.4.2.1.1-3 and A.12.4.2.1.1-4. Cell transmits SSBs in DBT windows according to DL CCA model.

In this test, measurement gap pattern configuration # 0 as defined in Table A.12.4.2.1.1-2 is provided.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event B2 (PCell becomes worse than threshold1 and inter RAT neighbour becomes better than threshold2) [16] is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 2.

Table A.12.4.2.1.1-1: NR inter-RAT event triggered reporting tests without SSB index reading for FR1

| Configuration | Description |
| --- | --- |
| 1 | LTE FDD; NR with CCA: SCS 30 kHz, BW 40 MHz, TDD |
| 2 | LTE TDD; NR with CCA: SCS 30 kHz, BW 40 MHz, TDD |
| NOTE: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table A.12.4.2.1.1-2: General test parameters for NR inter-RAT event triggered reporting for FR1 without SSB time index detection

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1, 2 | 1 | One E-UTRA carrier frequency is used. |
| NR RF Chanel Number |  | 1, 2 | 1 | One FR1 NR carrier frequency under CCA is used. |
| DL CCA model |  |  | As specified in clause A.3.26.2.1 |  |
| UL CCA model |  |  | As specified in clause A.3.26.2.2 |  |
| Active cell |  | 1, 2 | E-UTRA cell 1 (PCell) | E-UTRA cell 1 is on E-UTRA RF channel number 1. |
| Neighbour cell |  | 1, 2 | NR cell 2 | NR cell 2 is on NR RF channel number 1. |
| Gap Pattern Id |  | 1, 2 | 0 | As specified in clause Table 8.1.2.1-1 of TS 36.133 [15]. |
| Measurement gap offset |  | 1, 2 | 39 | As specified in TS 36.331 [16]. |
| b2-Threshold1 | dBm | 1, 2 | Note 1 | E-UTRA RSRP/RSRQ/SINR threshold for E-UTRA RSRP measurement on cell 1 for event B2 [16] |
| b2-Threshold2NR | dBm | 1, 2 | Note 2 | SS-RSRP/ SS-RSRQ/ SS-SINR threshold measurement on cell 2 for event B2 [16] |
| Hysteresis | dB | 1, 2 | 0 |  |
| CP length |  | 1, 2 | Normal |  |
| TimeToTrigger | s | 1, 2 | 0 |  |
| Filter coefficient |  | 1, 2 | 0 | L3 filtering is not used |
| DRX |  | 1, 2 | OFF | DRX is not used |
| Time offset between serving and neighbour cells |  | 1, 2 | 3s | Synchronous cells. |
| T1 | s | 1, 2 | 5 |  |
| T2 | s | 1, 2 | ≥Tidentify_irat_cca_without_index | Tidentify_irat_cca_without_index is defined in clause 8.1.2.4.21A.1 and 8.1.2.4.22A.1 in TS 36.133 |
| Note 1: The value of b2-Threshold1 is defined in Table A.12.4.2.1.1-3 Note 2: The value of b2-Threshold2NR is defined in Table A.12.4.2.1.1-4 |  |  |  |  |

Table A.12.4.2.1.1-3: E-UTRAN PCell specific test parameters for NR inter-RAT event triggered reporting in non-DRX with NR neigbour cell in FR1 without SSB time index detection

| Parameter | Unit | Configuration | Cell 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| RF channel number |  | 1, 2 | 1 |  |
| Duplex mode |  | 1 | FDD |  |
|  |  | 2 | TDD |  |
| TDD special subframe configurationNote1 |  | 2 | 6 |  |
| TDD uplink-downlink configurationNote1 |  | 2 | 1 |  |
| BWchannel | MHz | 1, 2 | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |  |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  | 1 | 5 MHz: R.7 FDD 10 MHz: R.3 FDD 20 MHz: R.6 FDD |  |
|  |  | 2 | 5 MHz: R.4 TDD 10 MHz: R.0 TDD 20 MHz: R.3 TDD |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 |  | 1 | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |
|  |  | 2 | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |
| OCNG PatternsNote2 |  | 1 | 5 MHz: OP.20 FDD 10 MHz: OP.10 FDD 20 MHz: OP.17 FDD |  |
|  |  | 2 | 5 MHz: OP.9 TDD 10 MHz: OP.1 TDD 20 MHz: OP.7 TDD |  |
| b2-Threshold1 | dBm | 1, 2 | -77 |  |
| PBCH_RA | dB | 1, 2 | 0 |  |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |
| NocNote4 | dBm/15kHz | 1, 2 | -104 |  |
| Ês/Noc | dB | 1, 2 | 17 | 17 |
| Ês/IotNote5 | dB | 1, 2 | 17 | 17 |
| RSRPNote5 | dBm/15kHz | 1, 2 | -87 | -87 |
| SCH_RPNote5 | dBm/15kHz | 1, 2 | -87 | -87 |
| IoNote5 | dBm/9MHz | 1, 2 | -59.13+10log (NRB,c /50) | -59.13+10log (NRB,c /50) |
| Propagation Condition Note6 |  | 1, 2 | AWGN |  |
| Antenna Configuration and Correlation Matrix Note6 |  | 1, 2 | 1x2 Low |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [23]. Note 2: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [15] respectively. Note 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 5: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 6: Propagation condition and correlation matrix are defined in clause B.2 in TS 36.101 [25]. |  |  |  |  |

Table A.12.4.2.1.1-4: NR neighbour cell specific test parameters for NR inter-RAT event triggered reporting for FR1 without SSB time index detection

| Parameter | Unit | Test configuration | Cell 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| NR RF Channel Number |  | 1, 2 | 2 |  |
| TDD configuration |  | 1, 2 | TDDConf.1.1 CCA |  |
| BWchannel | MHz | 1, 2 | 40: NRB,c = 106 |  |
| PCCA_DL for dynamic channel access Note 6,8 |  | 1, 2 | PCCA_DL_1=0.75 PCCA_DL_2=0.75 |  |
| PCCA_DL for semi-static channel access Note 7,8 |  | 1, 2 | PCCA_DL=0.9375 |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  | 1, 2 | OP.1 |  |
| SMTC configuration defined in A.3.11.1 and A.3.11.2 |  | 1, 2 | SMTC.1 |  |
| DBT window configuration |  | 1, 2 | DBT.1 |  |
| SSB configuration for semi-static channel access |  | 1, 2 | SSB.1 CCA |  |
| SSB configuration for dynamic channel access |  | 1, 2 | SSB.2 CCA |  |
| PDSCH/PDCCH subcarrier spacing | kHz | 1, 2 | 30 |  |
| b2-Threshold2NR | dBm | 1, 2 | -98 |  |
| EPRE ratio of PSS to SSS |  | 1, 2 | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS (Note 1) |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |
| Note2 | dBm/15kHz | 1, 2 | -98 |  |
| Note2 | dBm/SCS | 1, 2 | -95 |  |
| SS-RSRP Note 3,5 | dBm/SCS | 1, 2 | -Infinity | -88 |
| Note 5 | dB | 1, 2 | -Infinity | 7 |
| Note 5 | dB | 1, 2 | -Infinity | 7 |
| IoNote3 | dBm/38.16MHz | 1, 2 | -63.95 | -56.16 |
| Propagation Condition |  | 1, 2 | AWGN |  |
| Antenna Configuration and Correlation Matrix |  | 1, 2 | 1x2 Low |  |
| NOTE 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. For cells with CCA model, OCNG is transmitted only in the slots with downlink transmission burst and is not transmitted during the muted slots or during DBT window. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. NOTE 5: The signal levels apply for SSS REs when the discovery burst is transmitted during DBT windows. NOTE 6: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. NOTE 7: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. NOTE 8: For a UE supporting both semi-static and dynamic channel access, the UE can be tested under dynamic channel occupancy only. |  |  |  |  |

##### A.12.4.2.1.2	Test Requirements

The UE shall send one Event B2 triggered measurement report, with a measurement reporting delay less than Tidentify_irat_cca_without_index ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

The UE is not required to report SSB time index. Tidentify_irat_cca_without_index is defined in defined in clause 8.1.2.4.21A.1 and 8.1.2.4.22A.1 in TS 36.133.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.12.4.2.2	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used

##### A.12.4.2.2.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the NR inter-RAT cell search requirements in clause 8.1.2.4.21A of TS 36.133 [15] for E-UTRAN FDD-NR measurements and clause 8.1.2.4.22A of TS 36.133 [15] for E-UTRAN TDD-NR measurements.

In this test, there are two cells: E-UTRA cell 1 as PCell on E-UTRA RF channel 1 and NR cell 2 as neighbour cell in FR1 on NR RF channel 1 on a carrier frequency with CCA. The test parameters are given in Tables A.12.4.2.2.1-1, A.12.4.2.2.1-2, A.12.4.2.2.1-3 and A.12.4.2.2.1-4. Cell transmits SSBs in DBT windows according to DL CCA model.

In this test, measurement gap pattern configuration # 0 as defined in Table A.12.4.2.2.1-2 is provided.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event B2 (PCell becomes worse than threshold1 and inter RAT neighbour becomes better than threshold2) [16] is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 2.

Table A.12.4.2.2.1-1: NR inter-RAT event triggered reporting tests without SSB index reading for FR1

| Configuration | Description |
| --- | --- |
| 1 | LTE FDD; NR with CCA: SCS 30 kHz, BW 40 MHz, TDD |
| 2 | LTE TDD; NR with CCA: SCS 30 kHz, BW 40 MHz, TDD |
| NOTE: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table A.12.4.2.2.1-2: General test parameters for NR inter-RAT event triggered reporting for FR1 without SSB time index detection

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| E-UTRA RF Channel Number |  | 1, 2 | 1 |  | One E-UTRAcarrier frequency is used. |
| NR RF Chanel Number |  | 1, 2 | 1 |  | One FR1 NR carrier frequency under CCA is used. |
| Active cell |  | 1, 2 | E-UTRA cell 1 (PCell) |  | E-UTRA cell 1 is on E-UTRA RF channel number 1. |
| DL CCA model |  |  | As specified in clause A.3.26.2.1 |  |  |
| UL CCA model |  |  | As specified in clause A.3.26.2.2 |  |  |
| Neighbour cell |  | 1, 2 | NR cell 2 |  | NR cell 2 is on NR RF channel number 1. |
| Gap Pattern Id |  | 1, 2 | 0 |  | As specified in clause Table 8.1.2.1-1 of TS 36.133 [15]. |
| Measurement gap offset |  | 1, 2 | 39 |  | As specified in TS 36.331 [16]. |
| b2-Threshold1 | dBm | 1, 2 | Note 1 |  | E-UTRA RSRP threshold for E-UTRA RSRP measurement on cell 1 for event B2 [16] |
| b2-Threshold2NR | dBm | 1, 2 | Note 2 |  | SS-RSRP threshold for measurement on cell 2 for event B2 [16] |
| Hysteresis | dB | 1, 2 | 0 |  |  |
| CP length |  | 1, 2 | Normal |  |  |
| TimeToTrigger | s | 1, 2 | 0 |  |  |
| Filter coefficient |  | 1, 2 | 0 |  | L3 filtering is not used |
| DRX |  | 1, 2 | DRX.9 | DRX.12 | As specified in clause A.3.3 |
| Time offset between serving and neighbour cells |  | 1, 2 | 3s |  | Synchronous cells. |
| T1 | s | 1, 2 | 5 |  |  |
| T2 | s | 1, 2 | ≥Tidentify_irat_cca_without_index |  | Tidentify_irat_cca_without_index is defined in clause 8.1.2.4.21A.1 and 8.1.2.4.22A.1 in TS 36.133 |
| Note 1: The value of b2-Threshold1 is defined in Table A.12.4.2.1.1-3 Note 2: The value of b2-Threshold2NR is defined in Table A.12.4.2.1.1-4 |  |  |  |  |  |

Table A.12.4.2.2.1-3: E-UTRAN PCell specific test parameters for NR inter-RAT event triggered reporting in non-DRX with NR neigbour cell in FR1 without SSB time index detection

| Parameter | Unit | Configuration | Cell 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| RF channel number |  | 1, 2 | 1 |  |
| Duplex mode |  | 1 | FDD |  |
|  |  | 2 | TDD |  |
| TDD special subframe configurationNote1 |  | 2 | 6 |  |
| TDD uplink-downlink configurationNote1 |  | 2 | 1 |  |
| BWchannel | MHz | 1, 2 | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |  |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  | 1 | 5 MHz: R.7 FDD 10 MHz: R.3 FDD 20 MHz: R.6 FDD |  |
|  |  | 2 | 5 MHz: R.4 TDD 10 MHz: R.0 TDD 20 MHz: R.3 TDD |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 |  | 1 | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |
|  |  | 2 | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |
| OCNG PatternsNote2 |  | 1 | 5 MHz: OP.20 FDD 10 MHz: OP.10 FDD 20 MHz: OP.17 FDD |  |
|  |  | 2 | 5 MHz: OP.9 TDD 10 MHz: OP.1 TDD 20 MHz: OP.7 TDD |  |
| b2-Threshold1 | dBm | 1, 2 | -77 |  |
| PBCH_RA | dB | 1, 2 | 0 |  |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |
| NocNote4 | dBm/15kHz | 1, 2 | -104 |  |
| Ês/Noc | dB | 1, 2 | 17 | 17 |
| Ês/IotNote5 | dB | 1, 2 | 17 | 17 |
| RSRPNote5 | dBm/15kHz | 1, 2 | -87 | -87 |
| SCH_RPNote5 | dBm/15kHz | 1, 2 | -87 | -87 |
| IoNote5 | dBm/9MHz | 1, 2 | -59.13+10log (NRB,c /50) | -59.13+10log (NRB,c /50) |
| Propagation Condition Note6 |  | 1, 2 | AWGN |  |
| Antenna Configuration and Correlation Matrix Note6 |  | 1, 2 | 1x2 Low |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [23]. Note 2: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [15] respectively. Note 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 5: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 6: Propagation condition and correlation matrix are defined in clause B.2 in TS 36.101 [25]. |  |  |  |  |

Table A.12.4.2.2.1-4: NR neighbour cell specific test parameters for NR inter-RAT event triggered reporting for FR1 without SSB time index detection

| Parameter | Unit | Test configuration | Cell 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| NR RF Channel Number |  | 1, 2 | 1 |  |
| TDD configuration |  | 1, 2 | TDDConf.1.1 CCA |  |
| BWchannel | MHz | 1, 2 | 40: NRB,c = 106 |  |
| PCCA_DL for dynamic channel access Note 6,8 |  | 1, 2 | PCCA_DL_1=0.75 PCCA_DL_2=0.75 |  |
| PCCA_DL for semi-static channel access Note 7,8 |  | 1, 2 | PCCA_DL=0.9375 |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  | 1, 2 | OP.1 |  |
| SMTC configuration defined in A.3.11.1 and A.3.11.2 |  | 1, 2 | SMTC.1 |  |
| DBT window configuration |  | 1, 2 | DBT.1 |  |
| SSB configuration for semi-static channel access |  | 1, 2 | SSB.1 CCA |  |
| SSB configuration for dynamic channel access |  | 1, 2 | SSB.2 CCA |  |
| PDSCH/PDCCH subcarrier spacing | kHz | 1, 2 | 30 |  |
| b2-Threshold2NR | dBm | 1, 2 | -98 |  |
| EPRE ratio of PSS to SSS |  | 1, 2 | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS (Note 1) |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |
| Note2 | dBm/15kHz | 1, 2 | -98 |  |
| Note2 | dBm/SCS | 1, 2 | -95 |  |
| SS-RSRP Note 3,5 | dBm/SCS | 1, 2 | -Infinity | -88 |
| Note 5 | dB | 1, 2 | -Infinity | 7 |
| Note 5 | dB | 1, 2 | -Infinity | 7 |
| IoNote3 | dBm/38.16MHz | 1, 2 | -63.95 | -56.16 |
| Propagation Condition |  | 1, 2 | AWGN |  |
| Antenna Configuration and Correlation Matrix |  | 1, 2, | 1x2 Low |  |
| NOTE 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. For cells with CCA model, OCNG is transmitted only in the slots with downlink transmission burst and is not transmitted during the muted slots or during DBT window. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. NOTE 5: The signal levels apply for SSS REs when the discovery burst is transmitted during DBT windows. NOTE 6: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. NOTE 7: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. NOTE 8: For a UE supporting both semi-static and dynamic channel access, the UE can be tested under dynamic channel occupancy only. |  |  |  |  |

##### A.12.4.2.2.2	Test Requirements

In test 1, the UE shall send one Event B2 triggered measurement report, with a measurement reporting delay less than Tidentify_irat_cca_without_index ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 2, the UE shall send one Event B2 triggered measurement report, with a measurement reporting delay less than Tidentify_irat_cca_without_index ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In tests 1 and 2, the UE is not required to report SSB time index.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.12.4.2.3	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used

##### A.12.4.2.3.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the NR inter-RAT cell search requirements in clause 8.1.2.4.21A of TS 36.133 [15] for E-UTRAN FDD-NR measurements and clause 8.1.2.4.22A of TS 36.133 [15] for E-UTRAN TDD-NR measurements.

In this test, there are two cells: E-UTRA cell 1 as PCell on E-UTRA RF channel 1 and NR cell 2 as neighbour cell in FR1 on NR RF channel 1 on a carrier frequency with CCA.  The test parameters are given in Tables A.12.4.2.3.1-1, A.12.4.2.3.1-2, A.12.4.2.3.1-3 and A.12.4.2.3.1-4. Cell transmits SSBs in DBT windows according to DL CCA model.

In this test, measurement gap pattern configuration # 0 as defined in Table A.12.4.2.3.1-2 is provided.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event B2 (PCell becomes worse than threshold1 and inter RAT neighbour becomes better than threshold2) [16] is used. In the measurement configuration the UE shall be indicated to report the SSB index of the identified NR cell. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 2.

Table A.12.4.2.3.1-1: NR inter-RAT event triggered reporting tests with SSB index reading for FR1

| Configuration | Description |
| --- | --- |
| 1 | LTE FDD; NR with CCA: SCS 30 kHz, BW 40 MHz, TDD |
| 2 | LTE TDD; NR with CCA: SCS 30 kHz, BW 40 MHz, TDD |
| NOTE: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table A.12.4.2.3.1-2: General test parameters for NR inter-RAT event triggered reporting for FR1 with SSB time index detection

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1, 2 | 1 | One E-UTRAcarrier frequency is used. |
| NR RF Chanel Number |  | 1, 2 | 1 | One FR1 NR carrier frequency under CCA is used. |
| DL CCA model |  |  | As specified in clause A.3.26.2.1 |  |
| UL CCA model |  |  | As specified in clause A.3.26.2.2 |  |
| Active cell |  | 1, 2 | E-UTRA cell 1 (PCell) | E-UTRA cell 1 is on E-UTRA RF channel number 1. |
| Neighbour cell |  | 1, 2 | NR cell 2 | NR cell 2 is on NR RF channel number 1. |
| Gap Pattern Id |  | 1, 2 | 0 | As specified in clause Table 8.1.2.1-1 of TS 36.133 [15]. |
| Measurement gap offset |  | 1, 2 | 39 | As specified in TS 36.331 [16]. |
| b2-Threshold1 | dBm | 1, 2 | Note 1 | E-UTRA RSRP/RSRQ/SINR threshold for E-UTRA RSRP measurement on cell 1 for event B2 [16] |
| b2-Threshold2NR | dBm | 1, 2 | Note 2 | SS-RSRP/ SS-RSRQ/ SS-SINR threshold measurement on cell 2 for event B2 [16] |
| Hysteresis | dB | 1, 2 | 0 |  |
| CP length |  | 1, 2 | Normal |  |
| TimeToTrigger | s | 1, 2 | 0 |  |
| Filter coefficient |  | 1, 2 | 0 | L3 filtering is not used |
| DRX |  | 1, 2 | OFF | DRX is not used |
| Time offset between serving and neighbour cells |  | 1, 2 | 3s | Synchronous cells. |
| T1 | s | 1, 2 | 5 |  |
| T2 | s | 1, 2 | ≥ Tidentify_irat_cca_with_index | Tidentify_irat_cca_with_index is defined in clause 8.1.2.4.21A.1 and 8.1.2.4.22A.1 in TS 36.133 |
| Note 1: The value of b2-Threshold1 is defined in Table A.12.4.2.3.1-3 Note 2: The value of b2-Threshold2NR is defined in Table A.12.4.2.3.1-4 |  |  |  |  |

Table A.12.4.2.3.1-3: E-UTRAN PCell specific test parameters for NR inter-RAT event triggered reporting in non-DRX with NR neigbour cell in FR1 with SSB time index detection

| Parameter | Unit | Configuration | Cell 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| RF channel number |  | 1, 2 | 1 |  |
| Duplex mode |  | 1 | FDD |  |
|  |  | 2 | TDD |  |
| TDD special subframe configurationNote1 |  | 2 | 6 |  |
| TDD uplink-downlink configurationNote1 |  | 2 | 1 |  |
| BWchannel | MHz | 1, 2 | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |  |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  | 1 | 5 MHz: R.7 FDD 10 MHz: R.3 FDD 20 MHz: R.6 FDD |  |
|  |  | 2 | 5 MHz: R.4 TDD 10 MHz: R.0 TDD 20 MHz: R.3 TDD |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 |  | 1 | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |
|  |  | 2 | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |
| OCNG PatternsNote2 |  | 1 | 5 MHz: OP.20 FDD 10 MHz: OP.10 FDD 20 MHz: OP.17 FDD |  |
|  |  | 2 | 5 MHz: OP.9 TDD 10 MHz: OP.1 TDD 20 MHz: OP.7 TDD |  |
| b2-Threshold1 | dBm | 1, 2 | -77 |  |
| PBCH_RA | dB | 1, 2 | 0 |  |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |
| NocNote4 | dBm/15kHz | 1, 2 | -104 |  |
| Ês/Noc | dB | 1, 2 | 17 | 17 |
| Ês/IotNote5 | dB | 1, 2 | 17 | 17 |
| RSRPNote5 | dBm/15kHz | 1, 2 | -87 | -87 |
| SCH_RPNote5 | dBm/15kHz | 1, 2 | -87 | -87 |
| IoNote5 | dBm/9MHz | 1, 2 | -59.13+10log (NRB,c /50) | -59.13+10log (NRB,c /50) |
| Propagation Condition Note6 |  | 1, 2 | AWGN |  |
| Antenna Configuration and Correlation Matrix Note6 |  | 1, 2 | 1x2 Low |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [23]. Note 2: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [15] respectively. Note 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 5: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 6: Propagation condition and correlation matrix are defined in clause B.2 in TS 36.101 [25]. |  |  |  |  |

Table A.12.4.2.3.1-4: NR neighbour cell specific test parameters for NR inter-RAT event triggered reporting for FR1 with SSB time index detection

| Parameter | Unit | Test configuration | Cell 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| NR RF Channel Number |  | 1, 2 | 1 |  |
| TDD configuration |  | 1, 2 | TDDConf.1.1 CCA |  |
| BWchannel | MHz | 1, 2 | 40: NRB,c = 106 |  |
| PCCA_DL for dynamic channel access Note 6,8 |  | 1, 2 | PCCA_DL_1=0.75 PCCA_DL_2=0.75 |  |
| PCCA_DL for semi-static channel access Note 7,8 |  | 1, 2 | PCCA_DL=0.9375 |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  | 1, 2 | OP.1 |  |
| SMTC configuration defined in A.3.11.1 and A.3.11.2 |  | 1, 2 | SMTC.1 |  |
| DBT window configuration |  | 1, 2 | DBT.1 |  |
| SSB configuration for semi-static channel access |  | 1, 2 | SSB.1 CCA |  |
| SSB configuration for dynamic channel access |  | 1, 2 | SSB.2 CCA |  |
| PDSCH/PDCCH subcarrier spacing | kHz | 1, 2 | 30 |  |
| b2-Threshold2NR | dBm | 1, 2 | -98 |  |
| EPRE ratio of PSS to SSS |  | 1, 2 | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS (Note 1) |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |
| Note2 | dBm/15kHz | 1, 2 | -98 |  |
| Note2 | dBm/SCS | 1, 2 | -95 |  |
| SS-RSRP Note 3,5 | dBm/SCS | 1, 2 | -Infinity | -88 |
| Note 5 | dB | 1, 2 | -Infinity | 7 |
| Note 5 | dB | 1, 2 | -Infinity | 7 |
| IoNote3 | dBm/38.16MHz | 1, 2 | -63.95 | -56.16 |
| Propagation Condition |  | 1, 2 | AWGN |  |
| Antenna Configuration and Correlation Matrix |  | 1, 2, | 1x2 Low |  |
| NOTE 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. For cells with CCA model, OCNG is transmitted only in the slots with downlink transmission burst and is not transmitted during the muted slots or during DBT window. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for NOC to be fulfilled. NOTE 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. NOTE 5: The signal levels apply for SSS REs when the discovery burst is transmitted during DBT windows. NOTE 6: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. NOTE 7: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. NOTE 8: For a UE supporting both semi-static and dynamic channel access, the UE can be tested under dynamic channel occupancy only. |  |  |  |  |

##### A.12.4.2.3.2	Test Requirements

The UE shall send one Event B2 triggered measurement report, with a measurement reporting delay less than Tidentify_irat_cca_with_index ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

The UE is required to report SSB time index.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.12.4.2.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used

##### A.12.4.2.4.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the NR inter-RAT cell search requirements in clause 8.1.2.4.21A of TS 36.133 [15] for E-UTRAN FDD-NR measurements and clause 8.1.2.4.22A of TS 36.133 [15] for E-UTRAN TDD-NR measurements.

In this test, there are two cells: E-UTRA cell 1 as PCell on E-UTRA RF channel 1 and NR cell 2 as neighbour cell in FR1 on NR RF channel 1 on a carrier frequency with CCA. The test parameters are given in Tables A.12.4.2.4.1-1, A.12.4.2.4.1-2, A.12.4.2.4.1-3 and A.12.4.2.4.1-4. Cell transmits SSBs in DBT windows according to DL CCA model.

In this test, measurement gap pattern configuration # 0 as defined in Table A.12.4.2.4.1-2 is provided.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event B2 (PCell becomes worse than threshold1 and inter RAT neighbour becomes better than threshold2) [16] is used. In the measurement configuration the UE shall be indicated to report the SSB index of the identified NR cell. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 2.

Table A.12.4.2.4.1-1: NR inter-RAT event triggered reporting tests with SSB index reading for FR1

| Configuration | Description |
| --- | --- |
| 1 | LTE FDD; NR with CCA: SCS 30 kHz, BW 40 MHz, TDD |
| 2 | LTE TDD; NR with CCA: SCS 30 kHz, BW 40 MHz, TDD |
| NOTE: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table A.12.4.2.4.1-2: General test parameters for NR inter-RAT event triggered reporting for FR1 with SSB time index detection

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| E-UTRA RF Channel Number |  | 1, 2 | 1 |  | One E-UTRAcarrier frequency is used. |
| NR RF Chanel Number |  | 1, 2 | 1 |  | One FR1 NR carrier frequency under CCA is used. |
| DL CCA model |  |  | As specified in clause A.3.26.2.1 |  |  |
| UL CCA model |  |  | As specified in clause A.3.26.2.2 |  |  |
| Active cell |  | 1, 2 | E-UTRA cell 1 (PCell) |  | E-UTRA cell 1 is on E-UTRA RF channel number 1. |
| Neighbour cell |  | 1, 2 | NR cell 2 |  | NR cell 2 is on NR RF channel number 1. |
| Gap Pattern Id |  | 1, 2 | 0 |  | As specified in clause Table 8.1.2.1-1 of TS 36.133 [15]. |
| Measurement gap offset |  | 1, 2 | 39 |  | As specified in TS 36.331 [16]. |
| b2-Threshold1 | dBm | 1, 2 | Note 1 |  | E-UTRA RSRP/RSRQ/SINR threshold for E-UTRA RSRP measurement on cell 1 for event B2 [16] |
| b2-Threshold2NR | dBm | 1, 2 | Note 2 |  | SS-RSRP/ SS-RSRQ/ SS-SINR threshold measurement on cell 2 for event B2 [16] |
| Hysteresis | dB | 1, 2 | 0 |  |  |
| CP length |  | 1, 2 | Normal |  |  |
| TimeToTrigger | s | 1, 2 | 0 |  |  |
| Filter coefficient |  | 1, 2 | 0 |  | L3 filtering is not used |
| DRX |  | 1, 2 | DRX.9 | DRX.12 | As specified in clause A.3.3 |
| Time offset between serving and neighbour cells |  | 1, 2 | 3s |  | Synchronous cells. |
| T1 | s | 1, 2 | 5 |  |  |
| T2 | s | 1, 2 | ≥Tidentify_irat_cca_with_index |  | Tidentify_irat_cca_with_index is defined in clause 8.1.2.4.21A.1 and 8.1.2.4.22A.1 in TS 36.133 |
| Note 1: The value of b2-Threshold1 is defined in Table A.12.4.2.4.1-3 Note 2: The value of b2-Threshold2NR is defined in Table A.12.4.2.4.1-4 |  |  |  |  |  |

Table A.12.4.2.4.1-3: E-UTRAN PCell specific test parameters for NR inter-RAT event triggered reporting in non-DRX with NR neigbour cell in FR1 with SSB time index detection

| Parameter | Unit | Configuration | Cell 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| RF channel number |  | 1, 2 | 1 |  |
| Duplex mode |  | 1 | FDD |  |
|  |  | 2 | TDD |  |
| TDD special subframe configurationNote1 |  | 2 | 6 |  |
| TDD uplink-downlink configurationNote1 |  | 2 | 1 |  |
| BWchannel | MHz | 1, 2 | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |  |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  | 1 | 5 MHz: R.7 FDD 10 MHz: R.3 FDD 20 MHz: R.6 FDD |  |
|  |  | 2 | 5 MHz: R.4 TDD 10 MHz: R.0 TDD 20 MHz: R.3 TDD |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 |  | 1 | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |
|  |  | 2 | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |
| OCNG PatternsNote2 |  | 1 | 5 MHz: OP.20 FDD 10 MHz: OP.10 FDD 20 MHz: OP.17 FDD |  |
|  |  | 2 | 5 MHz: OP.9 TDD 10 MHz: OP.1 TDD 20 MHz: OP.7 TDD |  |
| b2-Threshold1 | dBm | 1, 2 | -77 |  |
| PBCH_RA | dB | 1, 2 | 0 |  |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |
| NocNote4 | dBm/15kHz | 1, 2 | -104 |  |
| Ês/Noc | dB | 1, 2 | 17 | 17 |
| Ês/IotNote5 | dB | 1, 2 | 17 | 17 |
| RSRPNote5 | dBm/15kHz | 1, 2 | -87 | -87 |
| SCH_RPNote5 | dBm/15kHz | 1, 2 | -87 | -87 |
| IoNote5 | dBm/9MHz | 1, 2 | -59.13+10log (NRB,c /50) | -59.13+10log (NRB,c /50) |
| Propagation Condition Note6 |  | 1, 2 | AWGN |  |
| Antenna Configuration and Correlation Matrix Note6 |  | 1, 2 | 1x2 Low |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [23]. Note 2: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [15] respectively. Note 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 5: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 6: Propagation condition and correlation matrix are defined in clause B.2 in TS 36.101 [25]. |  |  |  |  |

Table A.12.4.2.4.1-4: NR neighbour cell specific test parameters for NR inter-RAT event triggered reporting for FR1 with SSB time index detection

| Parameter | Unit | Test configuration | Cell 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| NR RF Channel Number |  | 1, 2 | 1 |  |
| TDD configuration |  | 1, 2 | TDDConf.1.1 CCA |  |
| BWchannel | MHz | 1, 2 | 40: NRB,c = 106 |  |
| PCCA_DL for dynamic channel access Note 6,8 |  | 1, 2 | PCCA_DL_1=0.75 PCCA_DL_2=0.75 |  |
| PCCA_DL for semi-static channel access Note 7,8 |  | 1, 2 | PCCA_DL=0.9375 |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  | 1, 2 | OP.1 |  |
| SMTC configuration defined in A.3.11.1 and A.3.11.2 |  | 1, 2 | SMTC.1 |  |
| DBT window configuration |  | 1, 2 | DBT.1 |  |
| SSB configuration for semi-static channel access |  | 1, 2 | SSB.1 CCA |  |
| SSB configuration for dynamic channel access |  | 1, 2 | SSB.2 CCA |  |
| PDSCH/PDCCH subcarrier spacing | kHz | 1, 2 | 30 |  |
| b2-Threshold2NR | dBm | 1, 2 | -98 |  |
| EPRE ratio of PSS to SSS |  | 1, 2 | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS (Note 1) |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |
| Note2 | dBm/15kHz | 1, 2 | -98 |  |
| Note2 | dBm/SCS | 1, 2 | -95 |  |
| SS-RSRP Note 3,5 | dBm/SCS | 1, 2 | -Infinity | -88 |
| Note 5 | dB | 1, 2 | -Infinity | 7 |
| Note 5 | dB | 1, 2 | -Infinity | 7 |
| IoNote3 | dBm/38.16MHz | 1, 2 | -63.95 | -56.16 |
| Propagation Condition |  | 1, 2 | AWGN |  |
| Antenna Configuration and Correlation Matrix |  | 1, 2, | 1x2 Low |  |
| NOTE 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. For cells with CCA model, OCNG is transmitted only in the slots with downlink transmission burst and is not transmitted during the muted slots or during DBT window. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. NOTE 5: The signal levels apply for SSS REs when the discovery burst is transmitted during DBT windows. NOTE 6: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. NOTE 7: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. NOTE 8: For a UE supporting both semi-static and dynamic channel access, the UE can be tested under dynamic channel occupancy only. |  |  |  |  |

##### A.12.4.2.4.2	Test Requirements

In test 1, the UE shall send one Event B2 triggered measurement report, with a measurement reporting delay less than Tidentify_irat_cca_with_index ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 2, the UE shall send one Event B2 triggered measurement report, with a measurement reporting delay less than Tidentify_irat_cca_with_index ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In tests 1 and 2, the UE is required to report SSB time index.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.12.4.2.5 Void

#### A.12.4.2.6	Void

## A.12.5	Measurement performance

### A.12.5.1	E-UTRANNR SFTD

#### A.12.5.1.1	Inter-RAT SFTD accuracy with NR target cell under CCA

##### A.12.5.1.1.1	Test Purpose

The purpose of this set of tests is to verify that the SFTD measurement accuracy is within the specified limits. This test will verify the requirements as specified in clause 9.1.27 in TS 36.133 [15] for inter-RAT SFTD measurements between E-UTRA PCell and NR target cell under CCA.

##### A.12.5.1.1.2	Test Environment

Supported test configurations are shown in Table A.12.5.1.1.2-1. In this set of test cases there are two cells on different carriers. Cell 1 is E-UTRAN PCell and Cell 2 is inter-RAT NR target cell under CCA. The test parameters of Cell 1 are given in clause A.12.5.1.1.2-2. The test parameters of Cell 2 are given in Table A.12.5.1.1.2-3. The SFTD between PCell and NR target cell shall be set by the test equipment to one of the time differences in Table A.12.5.1.1.2-4.

Table A.12.5.1.1.2-1: Supported test configurations for SFTD accuracy with NR target cell under CCA

| Config | Description |
| --- | --- |
| 1 | LTE FDD NR with CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 2 | LTE TDD NR with CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.12.5.1.1.2-2: Test parameters for inter-RAT SFTD accuracy with NR target cell under CCA (Cell 1)

| Parameter | Unit | Test 1 |
| --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 |
| Duplex mode |  | FDD or TDD |
| TDD special subframe configurationNote1 |  | 6 |
| TDD uplink-downlink configurationNote1 |  | 1 |
| BWchannel |  | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  | 5 MHz: R.7 FDD 10 MHz: R.3 FDD 20 MHz: R.6 FDD 5 MHz: R.4 TDD 10 MHz: R.0 TDD 20 MHz: R.3 TDD |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 |  | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |
| OCNG PatternsNote2 |  | 5 MHz: OP.20 FDD 10 MHz: OP.10 FDD 20 MHz: OP.17 FDD 5 MHz: OP.9 TDD 10 MHz: OP.1 TDD 20 MHz: OP.7 TDD |
| PBCH_RA | dB | 0 |
| PBCH_RB | dB |  |
| PSS_RA | dB |  |
| SSS_RA | dB |  |
| PCFICH_RB | dB |  |
| PHICH_RA | dB |  |
| PHICH_RB | dB |  |
| PDCCH_RA | dB |  |
| PDCCH_RB | dB |  |
| PDSCH_RA | dB |  |
| PDSCH_RB | dB |  |
| OCNG_RANote3 | dB |  |
| OCNG_RBNote3 | dB |  |
| NocNote4 | dBm/15 kHz | -104 |
| Ês/Noc | dB | -3 |
| Ês/Iot | dB | -3 |
| RSRP Note5 | dBm/15 kHz | -107 |
| SCH_RP Note5 | dBm/15 kHz | -107 |
| Io Note5 | dBm/Ch BW | -74.45 +10log (NRB,c /50) |
| Propagation Condition |  | AWGN |
| Antenna Configuration |  | 1x2 |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [23]. Note 2: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [15] respectively. Note 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 5: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |

Table A.12.5.1.1.2-3: Test parameters for inter-RAT SFTD accuracy with NR target cell under CCA (Cell 2)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| TDD Configuration |  |  | TDDConf.1.1 CCA |
| BWchannel |  | MHz | 40: NRB,c = 106 |
| DL CCA model |  |  | As specified in clause A.3.26.2.1 |
| DL CCA probability for semi-static channel accessNote6,8 | PCCA |  | 0.75 |
| DL CCA probability for dynamic channel accessNote7,8 | PCCA_DL_1 |  | 0.75 |
|  | PCCA_DL_2 |  | 0.75 |
| SSB configuration for semi-static channel accessNote6,8 |  |  | SSB.1 CCA |
| SSB configuration for dynamic channel accessNote7,8 |  |  | SSB.2 CCA |
| SMTC configuration |  |  | SMTC.1 |
| DBT window configuration |  |  | DBT.1 |
| OCNG Patterns |  |  | OP.1 |
| EPRE ratio of PSS to SSS |  | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Noc Note2 |  | dBm/15kHz | -104 |
| Noc Note2 |  | dBm/SSB SCS | -101 |
| Ês/Iot |  | dB | -3 |
| Ês/Noc |  | dB | -3 |
| SS-RSRP Note3 |  | dBm/SCS | -104 |
| Io Note3 |  | dBm/38.16 MHz | -68.18 |
| Propagation condition |  |  | AWGN |
| Antenna configuration |  |  | 1x2 |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols in slots with downlink transmission bursts. For cells with CCA model, OCNG is transmitted only in the slots with downlink transmission burst and is not transmitted during the muted slots or during DBT window. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification Note 6: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 7: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. Note 8: For UE supporting both semi-static and dynamic cannel access, the UE must be tested under both dynamic and semi-static channel occupancy configurations. |  |  |  |

Table A.12.5.1.1.2-4: Timing offsets for inter-RAT SFTD accuracy test with NR target cell under CCA

| Configuration | SFN offset between PCell and NR neighbor cell | Frame boundary offset between PCell and NR neighbour cell (Ts) |
| --- | --- | --- |
| 1 | 100 | -122000 |
| 2 | 300 | -60540 |
| 3 | 500 | 1000 |
| 4 | 700 | 62540 |
| 5 | 900 | 124000 |

##### A.12.5.1.1.3	Test Requirements

The SFTD reported by the UE consists of 2 elements, SFN offset and frame boundary offset between PCell and inter-RAT NR target cell. The reported SFTD accuracy shall fulfil the requirement in clause 9.1.27 in TS 36.133 [15].

### A.12.5.2	Void

### A.12.5.3	Void

### A.12.5.4	Void

### A.12.5.5	Void

### A.12.5.6	Void

#### End of change 1