3GPP TSG-RAN4 Meeting #112	R4-2413383

Maastricht, Netherlands, 19th – 23rd August, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 4944 | rev |  | Current version: | 17.14.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_SmallData_INACTIVE-Perf) CR FR2 CG-SDT test (R17) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SmallData_INACTIVE-Perf |  |  |  |  |  | Date: |  |  | 08-04-2024 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - Current CG-SDT Io level exceed -50dBm which is not supported. - Measurement uncertainty (6.5dB) has not considered during test case design. - To make sure the good UE can pass the test, RSRP level should be adjust to accommodate 6.5dB measurement uncertainty. Moreover CG-SDT threshold should be changed accordingly. - Since FR1 CG-SDT test can detect wrong UE who measure RSRP at outside of measurement window, FR2 test can focus on good UE to make sure pass the test case. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Change subtest 1 RSRP1, Noc, and CG-SDT threshold - Change subtest 2 RSRP1, Noc, and CG-SDT threshold - Change RB allocation - Same changes for both NR (A.7.2.1.1) and NR-U (A.17.2.1.1) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE fail the test due to wrong configuration and Io exceeds -50dbm. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.7.2.1.1, A.17.2.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Change 1 >>

#### A.7.2.1.1	TA validation for CG-SDT in FR2

##### A.7.2.1.1.1	Test Purpose and Environment

The purpose of this test is to partly verify that the UE properly perform TA validation for CG-SDT transmission in clause 5.5.3. The test includes two sub-tests, Sub-test#1 for testing valid TA where UE can initiate CG-SDT transmission, and Sub-test#2 for testing invalid TA where UE does not initiate CG-SDT transmission. Subtest#2 is only tested if Sub-test#1 is passed. For each sub-test, UE is configured with CG-SDT configurations when entering RRC Inactive state. Sub-test#1 consists of four successive time periods, with time duration of T1, T2, T3 and T4 respectively. Sub-test#2 consists of two successive time periods, with time duration of T5 and T6 respectively. There is one cell, which is the active NR cell in FR2. Figure A.7.2.1.1.1-1 shows the variation of the RSRP over the duration of Sub-test#1, and Figure A.7.2.1.1.1-2 shows the variation of the RSRP over the duration of Sub-test#2.

In Sub-test#1:

-	Prior to the time point TA, the UE shall be fully synchronized to PCell (Cell 1), be registered to the cell and have entered RRC connected mode.

-	Before starting the test at time point TA, test equipment configures RSRP to P0. At time point TB, RSRP is changed from P0 to P1.

-	At time point TC which is W1 after time point TB, UE expect to receive RRC release with CG SDT configuration and RRC status is changed to INACTIVE status.

-	At time point TD, RSRP is changed from P1 to P0.

-	At time point TE, RSRP is changed from P0 to P1. TE must be W2 before TF.

-	Test equipment triggers UL data arrival at UE lower layer at time point TF. After time point TF, test equipment observes whether UE transmits with CG-SDT no later than TG which is W3 after TF.

-	After time point TG, RRC status is changed from RRC INACTIVE to RRC CONNECTED.

In Sub-test#2:

-	Prior to the time point TA, the UE shall pass Sub-test#1 and have entered RRC connected mode. Otherwise, Sub-test#2 shall not be executed.

-	From time point TA to time point TD, RSRP is set to P1.

-	At time point TC, which is W1 after time point TB, UE expect to receive RRC release with CG SDT configuration and RRC status is changed to INACTIVE status.

-	At time point TD, RSRP is changed from P1 to P2.

-	Test equipment triggers UL data arrival at UE lower layer at time point TF. TF is 3360ms after TD. After time point TF, test equipment observes whether UE transmits with CG-SDT no later than TG which is W3 after TF.

W1 equals to 480ms and W2 equals to 480ms based on requirements in clause 5.5.3. W3 is 1060ms.

Supported test configurations are shown in table A.7.2.1.1.1-1. The test parameters are given in Tables A.7.2.1.1.1-2 and A.7.2.1.1.1-3.

Table A.7.2.1.1.1-1: Supported test configurations for FR2 PCell

| Configuration | Description |
| --- | --- |
| 1 | TDD, SSB SCS 120 KHz, data SCS 120KHz, BW 100 MHz |

Table A.7.2.1.1.1-2: General test parameters for TA validation for CG-SDT in FR2

| Parameter |  | Unit |  | Value |  | Value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | SubTest 1 |  | Subtest 2 |  |
| Active PCell |  |  |  | Cell 1 |  | Cell 1 |  |
| RF Channel Number |  |  |  | 1 |  | 1 |  |
| Duplex mode | Config 1 |  |  | TDD |  | TDD |  |
| BWchannel | Config 1 | MHz |  | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  |
| Data RBs allocated | Config 1 |  |  | 6 |  | 6 |  |
| DL initial BWP configuration | Config 1 |  |  | DLBWP.0.1 |  | DLBWP.0.1 |  |
| UL initial BWP configuration | Config 1 |  |  | ULBWP.0.1 |  | ULBWP.0.1 |  |
| TDD Configuration | Config 1 |  |  | TDDConf.3.1 |  | TDDConf.3.1 |  |
| RMSI CORESET Reference Channel | Config 1 |  |  | CR.3.1 TDD |  | CR.3.1 TDD |  |
| SSB Configuration | Config 1 |  |  | SSB.3 FR2 |  | SSB.3 FR2 |  |
| SMTC Configuration | Config 1 |  |  | SMTC.1 |  | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing | Config 1 | kHz |  | 120 |  | 120 |  |
| PRACH Configuration | Config 1 |  |  | Table A.3.8.3.4 |  | Table A.3.8.3.4 |  |
| OCNG parameters |  |  |  | OP.5 |  | OP.5 |  |
| CP length |  |  |  | Normal |  | Normal |  |
| Correlation Matrix and Antenna Configuration |  |  |  | 2x2 Low |  | 2x2 Low |  |
| DRX |  | s |  | 0.64 |  | 0.64 |  |
| cg-SDT-RSRP-ThresholdSSB |  | dBm |  | -113 |  | -113 |  |
| cg-SDT-RSRP-ChangeThreshold |  | dB |  | 14 |  | 4 |  |
| cg-SDT-TimeAlignmentTime |  |  |  | infinity |  | infinity |  |
| CG-SDT resource period |  | ms |  | 320 |  | 320 |  |
| T1 |  | s |  | 0.8 |  | NA |  |
| T2 |  | s |  | 0.96 |  | NA |  |
| T3 |  | s |  | 3.04 |  | NA |  |
| T4 |  | s |  | 1.54 |  | NA |  |
| T5 |  | s |  | NA |  | 1.76 |  |
| T6 |  | s |  | NA |  | 4.58 |  |

Table A.7.2.1.1.1-3: Cell specific test parameters TA validation for CG-SDT in FR2

| Parameter |  | Unit | Test 1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 |  | T2 |  |  |  | T3 |  | T4 |  | T5 |  |  |  | T6 |  |
| AoA setup |  |  | Setup 1 defined in A.3.15 |  |  |  |  |  |
| Assumption for UE beams Note 4 |  |  | Rough |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  | dB | 4 |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB | 0 |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB | 0 |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |  |  |  |
|  | Config 1 | dBm/15kHz | -104 | -104 |  |  | -104 |  |  |  |  | -104 |  |  | -107 |  |  | -107 |
|  | Config 1 | dBm/SCS | -95 | -95 |  |  | -95 |  |  |  |  | -95 |  |  | -98 |  |  | -98 |
|  | Config 1 | dB | 0 |  | 14 |  |  |  | 0 |  | 14 |  | 18.25 |  |  |  | 0.25 |  |
|  | Config 1 | dB | 0 |  | 14 |  |  |  | 0 |  | 14 |  | 18.25 |  |  |  | 0.25 |  |
| SS-RSRP | Config 1 | dBm/SCS | -95 |  | -81 |  |  |  | -95 |  | -81 |  | -79.75 |  |  |  | -97.75 |  |
| Io | Config 1 | dBm/95.04 MHz | -68.16 |  | -57.00 |  |  |  | -68.16 |  | -57.00 |  | -55.85 |  |  |  | -71.03 |  |
| Propagation condition |  |  | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Information about types of UE beam is given in B.2.1.3 and does not limit UE implementation or test system implementation. |  |  |  |  |  |  |  |  |

Figure A.7.2.1.1.1-1: RSRP variation for TA validation for CG-SDT Sub-test#1

Figure A.7.2.1.1.1-2: RSRP variation for TA validation for CG-SDT Sub-test#2

##### A.7.2.1.1.2	Test Requirements

The UE behaviour in each test during time durations shall be as follows:

During Sub-test#1, UE shall transmit UL data with CG-SDT within 1060ms after time point TF.

During Sub-test#2, after passing Sub-test#1, UE shall not transmit UL data with CG-SDT.

The rate of correct events observed during repeated tests shall be at least 90%.

<< Change 2 >>

#### A.17.2.1.1	TA validation for CG-SDT in FR2 for RedCap

##### A.17.2.1.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly perform TA validation for CG-SDT transmission in clause 5.2B.2.1. The test includes two sub-tests, Sub-test#1 for testing valid TA where UE can initiate CG-SDT transmission, and Sub-test#2 for testing invalid TA where UE does not initiate CG-SDT transmission. Subtest#2 is only tested if Sub-test#1 is passed. For each sub-test, UE is configured with CG-SDT configurations when entering RRC Inactive state. Sub-test#1 consists of four successive time periods, with time duration of T1, T2, T3 and T4 respectively. Sub-test#2 consists of two successive time periods, with time duration of T5 and T6 respectively. There is one cell, which is the active NR cell in FR1. Figure A.17.1.1.2.1-1 shows the variation of the RSRP over the duration of Sub-test#1 and Figure A.17.1.1.2.1-2 shows the variation of the RSRP over the duration of Sub-test#2.

In Sub-test#1:

-	Prior to the time point TA, the UE shall be fully synchronized to PCell (Cell 1), be registered to the cell and have entered RRC connected mode.

-	Before starting the test at time point TA, test equipment configures RSRP to P0.

-	At time point TB, RSRP is changed from P0 to P1.

-	At time point TC which is W1 after time point TB, UE expect to receive RRC release with CG-SDT configuration and RRC status is changed to INACTIVE status.

-	At time point TD, RSRP is changed from P1 to P0.

-	At time point TE, RSRP is changed from P0 to P1. TE must be W2 before TF.

-	Test equipment triggers UL data arrival at UE lower layer at time point TF. After time point TF, test equipment observes whether UE transmits with CG-SDT no later than TG which is W3 after TF.

-	After time point TG, RRC status is changed from RRC INACTIVE to RRC CONNECTED.

In Sub-test#2:

-	Prior to the time point TA, the UE shall pass Sub-test#1 and have entered RRC connected mode. Otherwise, Sub-test#2 shall not be executed.

-	From time point TA to time point TD, RSRP is set to P1.

-	At time point TC, which is W1 after time point TB, UE expect to receive RRC release with CG SDT configuration and RRC status is changed to INACTIVE status.

-	At time point TD, RSRP is changed from P1 to P2.

-	Test equipment triggers UL data arrival at UE lower layer at time point TF. TF is 3520ms after TD. After time point TF, test equipment observes whether UE transmits with CG-SDT no later than TG which is W3 after TF.

W1 equals to 480ms and W2 equals to 480ms based on requirements in clause 5.2.B2.1. W3 is 1060ms.

Table A.17.2.1.1.1-1: Supported test configurations for FR2 PCell

| Configuration | Description |
| --- | --- |
| 1 | NR TDD, SSB SCS 120 kHz, data SCS 120KHz, BW 100 MHz |

Table A.17.2.1.1.1-2: General test parameters for TA validation for CG-SDT in FR2

| Parameter |  | Unit | Value | Value |
| --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Subtest 2 |
| Active PCell |  |  | Cell 1 | Cell 1 |
| RF Channel Number |  |  | 1 | 1 |
| Duplex mode | Config 1 |  | TDD | TDD |
| BWchannel | Config 1 | MHz | 100: NRB,c = 66 | 100: NRB,c = 66 |
| Data RBs allocated | Config 1 |  | 6 | 6 |
| DL initial BWP configuration | Config 1 |  | DLBWP.0.1 | DLBWP.0.1 |
| UL initial BWP configuration | Config 1 |  | ULBWP.0.1 | ULBWP.0.1 |
| TDD Configuration | Config 1 |  | TDDConf.3.1 | TDDConf.3.1 |
| RMSI CORESET Reference Channel | Config 1 |  | CR.3.1 TDD | CR.3.1 TDD |
| SSB Configuration | Config 1 |  | SSB.3 FR2 | SSB.3 FR2 |
| SMTC Configuration | Config 1 |  | SMTC.1 | SMTC.1 |
| PDSCH/PDCCH subcarrier spacing | Config 1 | kHz | 120 | 120 |
| PRACH Configuration | Config 1 |  | Table A.3.8.3.4 | Table A.3.8.3.4 |
| OCNG parameters |  |  | OP.5 | OP.5 |
| CP length |  |  | Normal | Normal |
| Correlation Matrix and Antenna Configuration |  |  | 2x2 Low | 2x2 Low |
| DRX |  | s | 0.64 | 0.64 |
| cg-SDT-RSRP-ThresholdSSB |  | dBm | -113 | -113 |
| cg-SDT-RSRP-ChangeThreshold |  | dB | 14 | 4 |
| cg-SDT-TimeAlignmentTime |  |  | infinity | infinity |
| CG-SDT resource period |  | ms | 320 | 320 |
| T1 |  | s | 0.8 | NA |
| T2 |  | s | 0.96 | NA |
| T3 |  | s | 3.04 | NA |
| T4 |  | s | 1.54 | NA |
| T5 |  | s | NA | 1.76 |
| T6 |  | s | NA | 4.58 |

Table A.17.2.1.1.1-3: Cell specific test parameters TA validation for CG-SDT in FR2

| Parameter |  | Unit | SSB#0 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 |  | T2 |  |  | T3 |  |  | T4 |  |  |  | T5 |  | T6 |  |
| AoA setup |  |  | Setup 1 defined in A.3.15 |  |  |  |  |  |
| Assumption for UE beams Note 4 |  |  | Rough |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  | dB | 4 |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB | 0 |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB | 0 |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |  |  |  |
|  | Config 1 | dBm/15kHz | -104 |  |  | -104 | -104 |  |  | -104 |  |  | -107 |  |  | -107 |  |  |
|  | Config 1 | dBm/SCS | -95 |  |  | -95 | -95 |  |  | -95 |  |  | -98 |  |  | -98 |  |  |
|  | Config 1 | dB | 0 |  | 14 |  |  | 0 |  |  | 14 |  |  |  | 18.25 |  | 0.25 |  |
|  | Config 1 | dB | 0 |  | 14 |  |  | 0 |  |  | 14 |  |  |  | 18.25 |  | 0.25 |  |
| SS-RSRP | Config 1 | dBm/SCS | -95 |  | -81 |  |  | -95 |  |  | -81 |  |  |  | -79.75 |  | -97.75 |  |
| Io | Config 1 | dBm/95.04 MHz | -68.16 |  | -57.00 |  |  | -68.16 |  |  | -57.00 |  |  |  | -55.85 |  | -71.03 |  |
| Propagation condition |  |  | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Information about types of UE beam is given in B.2.1.3 and does not limit UE implementation or test system implementation. |  |  |  |  |  |  |  |  |

Figure A.17.2.1.1.1-1: RSRP variation model for CG-SDT Sub-test#1

Figure A.17.2.1.1.1-2: RSRP variation model for CG-SDT Sub-test#2

##### A.17.2.1.1.2	Test Requirements

The UE behaviour in each test during time durations shall be as follows:

During Sub-test#1, UE shall transmit UL data with CG-SDT within 1060ms after time point TF.

During Sub-test#2, after passing Sub-test#1, UE shall not transmit PUSCH at CG-SDT resources after TF until the end of the test at time point TG.

The rate of correct events observed during repeated tests shall be at least 90%.