3GPP TSG-RAN WG4 Meeting #112 	R4-2413388

# Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda Item:	6.1.2.2

Source: 	Ericsson

Title:	On remaining performance issues for SL positioning

Document for:	Discussion

# Introduction

The latest agreed WF on SL positioning and CPP is in [18].

In this contribution, we discuss issues related to SL positioning performance part.

# Discussion on SL positioning

## Specification structure for SL-PRS configurations in test cases

It was agreed to have SL-PRS configuration for test cases related to SL-PRS measurements for positioning in A.3.X, according to the agreed work split. It is proposed to replace “X” with “21A” in the clause number.

- Proposal 1: The following specification structure is proposed for SL-PRS configuration section:

A.3.21A	NR Sidelink Measurements for Positioning

A.3.21A.1	Introduction

A.3.21A.2	NR SL-PRS configurations

A.3.21A.2.1	NR SL-PRS configurations for FR1

## Accuracy requirements

### SL-PRS bandwidth in accuracy requirements and test cases

The following frequency bands and the applicable bandwidths are specified for V2X in TS 38.101-1

Table 5.3E.1-1 [TS 38.101-1] NR V2X operation channel bandwidths for each operating band

| NR band / SCS / UE Channel bandwidth (MHz) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| NR Band | SCS (kHz) | 5 | 10 | 20 | 30 | 40 |
| n14 | 15 | 5 | 10 |  |  |  |
|  | 30 |  | 10 |  |  |  |
|  | 60 |  |  |  |  |  |
| n38 | 15 |  | 10 | 20 | 30 | 40 |
|  | 30 |  | 10 | 20 | 30 | 40 |
|  | 60 |  | 10 | 20 | 30 | 40 |
| n47 | 15 |  | 10 | 20 | 30 | 40 |
|  | 30 |  | 10 | 20 | 30 | 40 |
|  | 60 |  | 10 | 20 | 30 | 40 |
| n79 | 15 |  | 10 | 20 | 30 | 40 |
|  | 30 |  | 10 | 20 | 30 | 40 |
|  | 60 |  | 10 | 20 | 30 | 40 |

The accuracy requirements have to cover all relevant bands and their bandwidths, i.e., starting from the smallest SL-PRS bandwidth, which is 24 PRBs for 5 MHz with 15 kHz SCS. In [21], the accuracy requirements for SL RSTD and SL Rx-Tx are specified from 10 MHz with 15 kHz SCS.

- Proposal 2: SL-PRS bandwidth 24 PRBs with 15 kHz SCS is included in the accuracy requirements for SL RSTD and SL Rx-Tx measurements.

### NR frequency band groups for SL positioning

Some of the bands are currently not included in the accuracy requirements in the big CR. Below are the frequency bands and corresponding band groups applicable for SL positioning:

n14:	NR_FDD_FR1_G,

n38: 	NR_TDD_FR1_B,

n47: 	NR_TDD_FR1_J,

n79: 	NR_TDD_FR1_C.

- Proposal 3: The following band groups are included in the accuracy requirements for SL positioning and in the conditions for NR SL-PRS based measurements:

  - NR_TDD_FR1_B,

  - NR_TDD_FR1_C,

  - NR_FDD_FR1_G,

  - NR_TDD_FR1_J.

### SL PRS-RSRP and SL PRS-RSRPP measurements

#### One the need for accuracy requirements

According to the received RAN1 LS [19], the requesting and/or reporting only SL PRS-RSRP or SL PRS-RSRPP without any other SL positioning measurements is not supported. Therefore, for core requirements, it is proposed in another Ericsson contribution that the measurement period of these measurements is the same as specified in TS 38.133 for the corresponding mandatory measurement with which they are configured, i.e., SL RSTD, SL Rx-Tx, SL AoA, or SL RTOA. Regardless of how the measurement period is defined and how SL PRS-RSRP/RSRPP are reported, their accuracy requirements still need to be specified.

- Proposal 4: Accuracy requirements for SL-PRS based measurements are specified in TS 38.133, without any impact of the RAN1 LS [19].

#### SL PRS-RSRP accuracy

Based on simulation results, SL PRS-RSRP accuracy is similar to that of PRS RSRP. So, at least for ≥24 PRBs at the same side conditions (Es/Iot ≥-3 dB), PRS RSRP accuracy can be reused for SL PRS-RSRP.

- Proposal 5: SL PRS-RSRP accuracy:

  - ±3.5 dB and	±8 dB for normal and extreme conditions, respectively, for ≥24 PRBs.

  - FFS, for other BWs

#### SL PRS-RSRPP accuracy

SL PRS-RSRPP accuracy is expected to be similar to that of PRS RSRPP. So, at least for ≥24 PRBs at the same side conditions (Es/Iot ≥-3 dB), PRS RSRPP accuracy can be reused for SL PRS-RSRPP.

- Proposal 6: SL PRS-RSRPP accuracy:

  - ±4.1 dB and	±8.6 dB for normal and extreme conditions, respectively, for ≥24 PRBs.

  - FFS, for other BWs

### SL Rx-Tx measurements

#### SL Rx-Tx accuracy

Based on simulation results, SL Rx-Tx accuracy is similar to that of PRS Rx-Tx. So, at least for ≥24 PRBs at the same side conditions (Es/Iot ≥-3 dB), PRS Rx-Tx accuracy can be reused for SL Rx-Tx.

- Proposal 7: SL Rx-Tx accuracy:

  - AWGN:

    - ≥24 PRBs, AWGN:

      - ± 78+, for 15 kHz,

      - ± 57+, for 30 kHz,

      - ± 29+, for 60 kHz,

    - FFS, for other BWs

  - Fading conditions:

    - ≥24 PRBs:

      - ± 137+, for 15 kHz,

      - ± 87+, for 30 kHz,

      - ± 59+, for 60 kHz,

    - FFS, for other BWs

## Test cases

### On the need and configuration of SL PRS-RSRP and SL PRS-RSRPP test cases with other measurements

Earlier related agreement [17]:

Issue 3-1-6: Test case list

Agreements:

- No test cases are defined to specifically verify measurement delay requirements for SL PRS-RSRP/RSRPP, regardless of whether (separate) measurement delay requirements are specified for SL PRS-RSRP/RSRPP.

- Accuracy requirements for SL PRS-RSRP/RSRPP are verified when configured together with RSTD/RX-TX, respectively.

  - Option 1a: separate section for testing SL PRS-RSRP/PRS-RSRPP, without verifying the accuracy of the other (SL RSTD/Rx-Tx) measurement, respectively

  - Option 1b: separate section for testing SL PRS-RSRP/PRS-RSRPP, with verifying also the accuracy of the other (SL RSTD/Rx-Tx) measurement, respectively

  - Option 2: same section for SL PRS-RSRP/RSRPP and SL RSTD/Rx-Tx, but different test cases - with and without SL PRS-RSRP/RSRPP.

From the agreement above, no measurement period test case for SL PRS-RSRP/RSRPP will be specified, but there will be accuracy test cases for SL PRS-RSPP and SL PRS-RSRPP, when they are configured together with SL RSTD and SL Rx-Tx, respectively.

Given that SL PRS-RSRP/RSRPP are separate measurements from SL RSTD and SL Rx-Tx, separate sections for test cases are preferred.

- Proposal 8: Accuracy test case for SL PRS-RSRP with SL RSTD is defined in a separate section from SL RSTD accuracy test case.

- Proposal 9: Accuracy test case for SL PRS-RSRPP with SL Rx-Tx is defined in a separate section from SL Rx-Tx accuracy test case.

- Proposal 10: In the accuracy test case for SL PRS-RSRP with SL RSTD, the SL PRS-RSRP accuracies for both reference SL-PRS and the other anchor SL-PRS are verified.

- Proposal 11: In the accuracy test case for SL PRS-RSRP with SL RSTD, the SL RSTD accuracy may be practically useful to verify together with the SL PRS-RSRP accuracy.

### Signal levels in delay test cases

Given the agreed side conditions, it is proposed to configure SL-PRS in UEs so that SL-PRS of UE1 and UE2 collide, but not with UE3 SL-PRS which is time shifted. Es/Iot level of UE1 SL-PRS is close to 0 dB, while Es/Iot of UE2 SL-PRS and UE3 SL-PRS are close to -3 dB.

- Proposal 12: For SL RSTD delay test case, the following SL-PRS signal level configurations are proposed:

|  | UE1 | UE2 | UE3 |
| --- | --- | --- | --- |
| SL-PRS Es/Noc | 5 dB | 3.2 dB | -3 dB |
| SL-PRS Es/Iot | 0.1 dB | -2.99 dB | -3 dB |

- Proposal 13: For SL Rx-Tx delay test case, the following SL-PRS signal level configurations are proposed:

|  | UE1 | UE2 |
| --- | --- | --- |
| SL-PRS Es/Noc | 5 dB | 3.2 dB |
| SL-PRS Es/Iot | 0.1 dB | -2.99 dB |

- Proposal 14: In SL AoA delay test case, the SL-PRS signal level configurations can be the same as in SL Rx-Tx test case.

- Proposal 15: In SL RTOA delay test case, it is proposed to add one interfering UE, so the total number of UEs in the test case is 3, including the target UE. The SL-PRS signal configuration for the three UEs can then be the same as for SL Rx-Tx:

|  | Interfering UE | Target UE |
| --- | --- | --- |
| SL-PRS Es/Noc | 5 dB | 3.2 dB |
| SL-PRS Es/Iot | 0.1 dB | -2.99 dB |

### SL-PRS configuration

In RAN4#111, the following test configurations were agreed for SL UEs:

| SL_conf1 | NR SL: 15 kHz SSB SCS, 10 MHz bandwidth, HD duplex mode |
| --- | --- |
| SL_conf2 | NR SL: 30 kHz SSB SCS, 10 MHz bandwidth, HD duplex mode |
| SL_conf3 | NR SL: 30 kHz SSB SCS, 20 MHz bandwidth, HD duplex mode |

To align with the above agreed configurations, the numbers of SL-PRS PRBs need to be updated to 24 and 48 PRBs.

- Proposal 16: Update SL-PRS configurations as follows:

| SL PRS Parameters | Values |  |  |  |
| --- | --- | --- | --- | --- |
| Reference channel | SL PRS.1.1 FR1 | SL PRS.1.2 FR1 | SL PRS.1.3 FR1 | SL PRS.1.4 FR1 |
| SCS | 15kHz, 30kHz |  |  |  |
| SL-PRS comb size | 2 |  | 4 |  |
| Number of SL-PRS symbols | 4 |  | 4 |  |
| SL-PRS comb offset Note 1 | [1] |  | [1] |  |
| SL-PRS resource slot offset (slot) Note 1 | [0] | [4] | [0] | [4] |
| RB numbers containing SL PRS within channel Bandwidth Note 1 | [2448] |  | [4896] |  |
| SL-PRS starting PRB | [4] |  | [4] |  |
| Note 1: Unless otherwise specified in the test case |  |  |  |  |

# Summary

The following have been proposed in the current contribution.

- Proposal 1: The following specification structure is proposed for SL-PRS configuration section:

A.3.21A	NR Sidelink Measurements for Positioning

A.3.21A.1	Introduction

A.3.21A.2	NR SL-PRS configurations

A.3.21A.2.1	NR SL-PRS configurations for FR1

- Proposal 2: SL-PRS bandwidth 24 PRBs with 15 kHz SCS is included in the accuracy requirements for SL RSTD and SL Rx-Tx measurements.

- Proposal 3: The following band groups are included in the accuracy requirements for SL positioning and in the conditions for NR SL-PRS based measurements:

  - NR_TDD_FR1_B,

  - NR_TDD_FR1_C,

  - NR_FDD_FR1_G,

  - NR_TDD_FR1_J.

- Proposal 4: Accuracy requirements for SL-PRS based measurements are specified in TS 38.133, without any impact of the RAN1 LS [19].

- Proposal 5: SL PRS-RSRP accuracy:

  - ±3.5 dB and	±8 dB for normal and extreme conditions, respectively, for ≥24 PRBs.

  - FFS, for other BWs

- Proposal 6: SL PRS-RSRPP accuracy:

  - ±4.1 dB and	±8.6 dB for normal and extreme conditions, respectively, for ≥24 PRBs.

  - FFS, for other BWs

- Proposal 7: SL Rx-Tx accuracy:

  - AWGN:

    - ≥24 PRBs, AWGN:

      - ± 78+, for 15 kHz,

      - ± 57+, for 30 kHz,

      - ± 29+, for 60 kHz,

    - FFS, for other BWs

  - Fading conditions:

    - ≥24 PRBs:

      - ± 137+, for 15 kHz,

      - ± 87+, for 30 kHz,

      - ± 59+, for 60 kHz,

    - FFS, for other BWs

- Proposal 8: Accuracy test case for SL PRS-RSRP with SL RSTD is defined in a separate section from SL RSTD accuracy test case.

- Proposal 9: Accuracy test case for SL PRS-RSRPP with SL Rx-Tx is defined in a separate section from SL Rx-Tx accuracy test case.

- Proposal 10: In the accuracy test case for SL PRS-RSRP with SL RSTD, the SL PRS-RSRP accuracies for both reference SL-PRS and the other anchor SL-PRS are verified.

- Proposal 11: In the accuracy test case for SL PRS-RSRP with SL RSTD, the SL RSTD accuracy may be practically useful to verify together with the SL PRS-RSRP accuracy.

- Proposal 12: For SL RSTD delay test case, the following SL-PRS signal level configurations are proposed:

|  | UE1 | UE2 | UE3 |
| --- | --- | --- | --- |
| SL-PRS Es/Noc | 5 dB | 3.2 dB | -3 dB |
| SL-PRS Es/Iot | 0.1 dB | -2.99 dB | -3 dB |

- Proposal 13: For SL Rx-Tx delay test case, the following SL-PRS signal level configurations are proposed:

|  | UE1 | UE2 |
| --- | --- | --- |
| SL-PRS Es/Noc | 5 dB | 3.2 dB |
| SL-PRS Es/Iot | 0.1 dB | -2.99 dB |

- Proposal 14: In SL AoA delay test case, the SL-PRS signal level configurations can be the same as in SL Rx-Tx test case.

- Proposal 15: In SL RTOA delay test case, it is proposed to add one interfering UE, so the total number of UEs in the test case is 3, including the target UE. The SL-PRS signal configuration for the three UEs can then be the same as for SL Rx-Tx:

|  | Interfering UE | Target UE |
| --- | --- | --- |
| SL-PRS Es/Noc | 5 dB | 3.2 dB |
| SL-PRS Es/Iot | 0.1 dB | -2.99 dB |

- Proposal 16: Update SL-PRS configurations as follows:

| SL PRS Parameters | Values |  |  |  |
| --- | --- | --- | --- | --- |
| Reference channel | SL PRS.1.1 FR1 | SL PRS.1.2 FR1 | SL PRS.1.3 FR1 | SL PRS.1.4 FR1 |
| SCS | 15kHz, 30kHz |  |  |  |
| SL-PRS comb size | 2 |  | 4 |  |
| Number of SL-PRS symbols | 4 |  | 4 |  |
| SL-PRS comb offset Note 1 | [1] |  | [1] |  |
| SL-PRS resource slot offset (slot) Note 1 | [0] | [4] | [0] | [4] |
| RB numbers containing SL PRS within channel Bandwidth Note 1 | [2448] |  | [4896] |  |
| SL-PRS starting PRB | [4] |  | [4] |  |
| Note 1: Unless otherwise specified in the test case |  |  |  |  |

# References

[1] RP-230328, Revised WID on Expanded and Improved NR Positioning, Intel Corp, March, 2023.

[2] R4-2303190, WF on Expanded and Improved NR Positioning RRM requirements, Ericsson, March, 2023.

[3] R1-2301950, Summary #1 on Measurements and reporting for SL positioning, vivo, March 2023.

[4] R1-2301951, Summary #2 on Measurements and reporting for SL positioning, vivo, March 2023.

[5] R1-2301952	Summary #3 on Measurements and reporting for SL positioning, vivo, March 2023.

[6] R4-2306351, WF on RRM requirements for NR sidelink positioning and carrier phase positioning, CATT, April 2023.

[7] R4-2310163, WF on NR Positioning – SL positioning and CPP, CATT, May 2023.

[8] R4-2310077, Simulation assumptions for SL positioning, CATT, May 2023.

[9] R4-2314343, Ad-hoc minutes on R18 NR Positioning RRM requirements, Ericsson, Aug. 2023.

[10] R4-2314354, WF on R18 NR positioning – SL positioning and Carrier Phase Positioning, CATT, Aug. 2023.

[11] R4-2314358, LS on SL positioning and CPP measurements report mapping, Aug. 2023.

[12] R4-2312724, On SL positioning, Ericsson, Aug. 2023.

[13] RP-232670, Revised WID on Expanded and Improved NR Positioning, Intel Corporation, CATT, MediaTek, Sep. 2023.

[14] R4-2317380, WF on R18 NR positioning – SL positioning and Carrier Phase Positioning, CATT, Oct. 2023.

[15] R4-2321524, WF on R18 NR positioning – SL positioning and Carrier Phase Positioning, CATT, Nov. 2023.

[16] R4-2403368, WF on R18 NR positioning – SL positioning and Carrier Phase Positioning, CATT, March 2024.

[17] R4-2406384, WF on SL positioning and carrier phase positioning, CATT, Apr. 2024.

[18] R4-2410193, WF on SL positioning and carrier phase positioning, CATT, May 2024.

[19] R4-2411003, Reply LS on SL positioning measurement, RAN1, Aug. 2024.

[20] R4-2410191, Updated work split on test cases for positioning, Ericsson, May 2024.

[21] R4-2410160, endorsed draft big CR on performance, Ericsson, May 2024.