3GPP TSG-RAN WG4 Meeting #112 	R4-2413391

# Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda Item:	8.17.1

Source: 	Ericsson

Title:	On general AI/ML aspects

Document for:	Discussion

# Introduction

The following was agreed in RAN4#112 [1] on general issues for AI/ML:

Issue 1-1: Post deployment testing

Interested companies are invited to bring analysis on the feasibility of the options proposed so far.

Issue 1-4:  Inference latency handling

Agreement:

- Use the following as the starting point

  - Inference latency can be implicitly captured in the delay of corresponding procedures (e.g. reporting delay) and/or performance requirements.

  - Inference latency is part of delay of the corresponding procedure and no separate requirement will be specified for inference latency.

  - Check the above bullets based on per use case, and if other WGs define the inference latency, RAN4 can revisit the above bullets.

Issue 1-5: LCM Requirements

Agreement:

- RAN4 considers defining RRM requirements for LCM procedures based on the specific use cases (beam management and positioning), and candidates are:

- identification

- selection

- activation

- deactivation

- switching

- fallback to non-AI operation

- performance monitoring

- Others are not precluded

Issue 1-6: Legacy RRM requirements handling

Agreement:

- UE shall meet all legacy RRM requirements (non-AI/ML) even during the AI/ML operation mode.

# Discussion

## Post-deployment verification

Post-deployment verification refers to processes for building confidence in continuing adherence to minimum performance requirements (indirectly) for UEs deployed in the field. Post-deployment verification is needed due to the potential for model update, reinforcement learning and the general issue that compliance testing can never exhaustively test model performance in all circumstances.

Existing agreements on post-deployment testing are as follows:

Agreement:

- To ensure the AI performance after device deployment, discuss the following options further

  - Option 1: Conduct the conformance testing for AI model/functionality before deployment

    - FFS on the feasibility

  - Option 2: Design the test to verify the performance monitoring

    - Depend on the other WG progress

    - Monitoring can be used for managing fallback, model update/model switching/model transfer, if applicable

  - Other options are not precluded

In our understanding, the wording in Option 1 can cause some confusion and become misleading. It was already agreed that conformance testing of UEs that are already deployed is not feasible. What Option 1 refers to is testing of new/updated models in a realistic way (considering the UE hardware that they will be deployed to) prior to use of the models in deployed UEs. For example, prior to sending a model to UEs, a UE vendor could verify that the model meets RAN4 requirements in representative hardware.

Also, Option 2 could be updated to include not only the monitoring but also the ability to timely act to prevent performance degradation, based on the performance monitoring.

- Proposal 1: Further clarify the options in the earlier RAN4 agreement as follows (the updates are in red):

- To ensure the AI performance after device deployment, discuss the following options further

  - Option 1: Conduct the conformance testing for new/updated AI model/functionality before its deployment in already deployed UEs (considering the UE hardware that it will be used in)

    - FFS on the feasibility

  - Option 2: Design the test to verify the performance monitoring and proactive recovery from potential performance degradation

    - Depend on the other WG progress

    - Monitoring can be used for managing fallback, model update/model switching/model transfer, if applicable

  - Other options are not precluded

During RAN4#110bis, we proposed a further option, which is for the UE vendor to store internal data on the input to a model during conformance testing (prior to UE deployment), such that model updates can later on be tested using the stored data prior to the models being activated in deployed UEs. This can be done, e.g., as described below:

- During compliance testing under controlled lab conditions, the UE records the inputs to the model under test. The inputs may be UE-specific as they may be after, e.g., analog processing.

- The data on model inputs may be stored in the UE or stored in another location.

- When a model is to be updated, the same data as used for the laboratory test is input to the model.

  - This could be done inside the UE in the field, but it could also be done offline in a test facility.

The latter option requires the capture and storage of UE internal data, which may need to be done by the UE vendor. It may not be feasible to independently verify the UE depending on the test setup. It may, however, provide a further option for post-deployment verification.

- Proposal 2: As a further option relating to post deployment testing, consider the possibility of capturing model input during testing for later testing of new models. Add Option 3 to the earlier agreed Option 1 and Option 2:

  - Option 3: Capture model input during conformance testing for later testing of new models.

- Regarding the feasibility of Option 1 earlier agreed by RAN4, the approach may be not so scalable due to UE implementation differences and may also give non-comparable and non-conclusive results if the models are tested in different conditions, which in turn may lead to wrong conclusions.

- Proposal 3: When discussing the feasibility of Option 1, RAN4 to consider its scalability limitation and potential mismatch between testing and actual environment.

We would also like to note that more feasibility studies in RAN4 are necessary for the above options to facilitate the other groups’ solution discussions. In order to achieve consistent and predictable performance towards the network, such that UEs can be correctly managed and KPIs can be maintained, it is essential that the performance of AI models and their updates can be verified. It is not yet clear which of the two options are more feasible. Option1 implies a greater amount of logistics for updating and deploying new models. Option 2 may involve significant processing to determine the performance, and it has yet to be proven (on a use-case specific basis) whether accurate and reproducible performance monitoring can be achieved.

For Option 1, RAN1 should continue to discuss monitoring frameworks and RAN4 consider how feasible verification of consistency and accuracy of the frameworks is. This may require some interaction between the groups. It would be unfortunate if a RAN1 framework would not be verifiable such that reports from different UE implementations could be comparable from a network perspective.

- Proposal 4: RAN4 to study and compare the feasibility of Options 1, 2, and 3. RAN4 can inform other groups about the conclusions of this study.

## LCM RRM requirements for AI/ML

In RAN4#111, it was agreed that RAN4 will consider defining RRM requirements for selected LCM procedures, one of which is the fallback procedure to non-AI operation.

It is generally not expected that the performance will be worse with AI/ML than the legacy operation, since the UE will have to meet at least the legacy requirements even under AI/ML operation. However, a UE capable of AI/ML operation, may still need to go to non-AI/ML operation mode. Some of the critical reasons for a fallback to non-AI/ML operation can be, e.g.,

- potential performance degradation below the expected legacy performance,

- due to an error, from which it cannot reliably recover in AI/ML mode, etc.

Hence it is important to have a requirement on how fast the UE can go back to the legacy non-AI/ML operation.

- Proposal 5: RAN4 will study a maximum delay requirement for switching between AI/ML and non-AI/ML operation modes, at least in the fallback direction (AI/ML →non-AI/ML).

- Proposal 6: The maximum fallback delay may include one or more components, e.g.,

  - obtaining and starting to use a fallback tx/rx configuration,

  - performing the necessary transmission and/or detection, identification, or measurement procedures,

  - informing the network about the fallback decision (e.g., if the UE decides this autonomously) or about completion of the fallback, etc.

# Summary

The following have been proposed in the current contribution:

- Proposal 1: Further clarify the options in the earlier RAN4 agreement as follows (the updates are in red):

- To ensure the AI performance after device deployment, discuss the following options further

  - Option 1: Conduct the conformance testing for new/updated AI model/functionality before its deployment in already deployed UEs (considering the UE hardware that it will be used in)

    - FFS on the feasibility

  - Option 2: Design the test to verify the performance monitoring and proactive recovery from potential performance degradation

    - Depend on the other WG progress

    - Monitoring can be used for managing fallback, model update/model switching/model transfer, if applicable

  - Other options are not precluded

- Proposal 2: As a further option relating to post deployment testing, consider the possibility of capturing model input during testing for later testing of new models. Add Option 3 to the earlier agreed Option 1 and Option 2:

  - Option 3: Capture model input during conformance testing for later testing of new models.

- Proposal 3: When discussing the feasibility of Option 1, RAN4 to consider its scalability limitation and potential mismatch between testing and actual environment.

- Proposal 4: RAN4 to study and compare the feasibility of Options 1, 2, and 3. RAN4 can inform other groups about the conclusions of this study.

- Proposal 5: RAN4 will study a maximum delay requirement for switching between AI/ML and non-AI/ML operation modes, at least in the fallback direction (AI/ML →non-AI/ML).

- Proposal 6: The maximum fallback delay may include one or more components, e.g.,

  - obtaining and starting to use a fallback tx/rx configuration,

  - performing the necessary transmission and/or detection, identification, or measurement procedures,

  - informing the network about the fallback decision (e.g., if the UE decides this autonomously) or about completion of the fallback, etc.

# References

[1] R4-2401570, WF on AI/ML, Qualcomm Inc., May 2024.

[2]	TR 38.843, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface”.