3GPP TSG-RAN WG4 Meeting #112 	R4-2413392

# Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda Item:	8.24.2

Source: 	Ericsson

Title:	On RRM core requirements for XR

Document for:	Discussion

# Introduction

The RAN4-related objectives of the Rel-19 XR Phase 3 WI are as follows.

Core part:

...

-	Specify enhancements to enable transmission/reception in gaps/restrictions that are caused by RRM measurements (from inter-frequency RRM measurement gaps, or intra-frequency measurements, or other scheduling restrictions etc). [RAN1, RAN2, RAN4]

-	Specify the corresponding measurement gap and scheduling restriction to enable the identified enhancements with RRM performance impact taken into consideration, work being triggered by LS. [RAN4]

...

-	Specify Core requirements related to the above objectives as necessary [RAN4]

...

Performance part:

-	Specify Performance requirements related to the above objectives as necessary [RAN4]

RAN4 also received RAN1 LS [2], were RAN4 is requested to take the below information from [2] into account and decide whether or not to introduce any UE assistance information related to measurement occasions:

RAN1 Agreement

RAN1 continues to discuss and decide whether or not to introduce new UE assistance information for solution(s) to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements. At least the following UE assistance information is considered for further study:

- FFS: UE assistance information related to measurement occasions:

  - FFS: The number of needed measurement gaps/SMTC with restrictions within a time period;

  - FFS: The maximum number or ratio of MGs/SMTC with restrictions that can be skipped within a time period;

  - FFS: The number of required SSBs within a time period;

  - FFS: The number of consecutive RRM measurements that can be skipped;

  - FFS: The maximum interval between two consecutively reserved gap/restriction occasions for RRM measurements;

  - FFS: The patterns of gap(s)/restriction(s) where skipping is feasible or acceptable;

- FFS: UE assistance information related to channel conditions:

  - FFS: RSRP is below/above search threshold (s-MeasureConfig);

- FFS: UE assistance information related to traffic:

  - FFS: PSI (PDU set importance);

- FFS: UE assistance information related to UE mobility:

  - FFS: L3 parameters related to mobility, e.g., static or not

Companies are encouraged to provide additional details (e.g. how often the UE assistance info is provided, timing, applicable scenarios, performance gains, etc) on their preferred scheme.

Note: From specification point of view, there is no mandated gNB behavior in response to any of the UE assistance information.

RAN1 to make decision, from RAN1 perspective, in RAN1#117 on the support of UE assistance information.

Particularly, it was discussed whether impact on RRM performance from skipping measurement occasions may be reduced if UE sends additional information:

- Information about the maximum number of MGs/SMTC with restrictions that can be skipped within a time period.

- Information about the patterns of gap(s)/restriction(s) where skipping is feasible or acceptable.

Below we further discuss the RAN1 LS and the impact of the WI objectives on RAN4 core requirements.

# Discussion

## RRM requirements enhancements for XR

This WI aims to realize system capacity gains by enabling transmission/reception in gaps/restrictions reserved for RRM measurements, while keeping impact to mobility performance limited. This was studied in TR 38.835 B.1.7.

The RRM measurement core and performance requirements are tightly linked to the number of measurement occasions. If some of the measurement occasions are not available for the measurements, the existing requirements may not be met, at least at the existing side conditions. Hence, to maintain the RRM measurement performance, two approaches could be envisioned:

- Approach 1: The simplest approach would be to limit the XR operation to unused measurement occasions, e.g., in good radio conditions, when RRM measurements could be completed by UE faster. No extension of the RRM measurement period would be needed in this case, but the availability of the unused measurement occasions may not be always guaranteed, e.g., at the cell edge.

- Approach 2: A more practical approach is to compensate for the configured measurement occasions reallocated to the XR operation by extending the RRM measurement period during the XR operation (the compensation means a longer measurement period compared to the UE operation without XR), to ensure the availability of the sufficient number of measurement occasions to complete the RRM measurements at the required accuracy level.

With Approach 2, the extension of the measurement period can be fixed or pre-defined or can be dynamically defined based on the availability of the measurement occasions during the UE XR operation. The choice of how the extension is defined depends on the RAN1/RAN2 solution and the specified signalling, but the dynamically defined extended RRM measurement period may be more suitable for XR due to the dynamic nature of the XR traffic.

- Proposal 1: To avoid limiting the XR operation to good network radio conditions only and hereby limiting the XR service coverage, RRM measurement requirements are enhanced for UEs under XR operation.

- Proposal 2: RAN4 to discuss possible enhancements to the legacy RRM requirements to account for the UE XR operation, e.g., whether pre-defined or dynamically extended RRM measurement period is to be specified for UE under XR operation to dynamically account for the availability of the RRM measurement occasions for RRM measurements.

Further, there also exist different types and purposes of RRM measurements and measurement gaps as well as they may have different configurations (e.g., measurement gap pattern ID, MGL, MGRP, per-UE or per-FR measurement gaps, etc.). It may also be so that not all measurement occasion configurations are necessary or relevant for the XR operation, e.g., RAN4 needs to discuss the relevance of too short or too long MGL, maximum acceptable time separation between the XR occasions, etc.

- Proposal 3: RAN4 to discuss which conditions the measurement gap configurations need to meet to be relevant for the UE XR operation, e.g., minimum MGL and/or maximum MGL, maximum MGRP, maximum acceptable time separation between the XR occasions, etc.

At the same time, it is also noted that when some of the measurement occasions are used for the UE XR operation, there will also be impact on RRM measurement samples and how they are combined, since the samples may become non-equally spaced or disrupted, resulting in inaccurate or lost measurements, timing, etc., even if the total measurement period is met. This may result in the RRM measurement accuracy degradation.

- Proposal 4: RAN4 to discuss the impact on RRM samples and how they are combined during the UE XR operation, e.g., maximum acceptable time separation between two measurement occasions available for RRM measurements.

## On UE assistance information for XR

RAN1 has been discussing UE assistance data to suggest which and/or how RRM measurement occasions (e.g., in measurement gaps) are to be reallocated to XR. According to [2], RAN1 discussed whether the impact on RRM performance from skipping measurement occasions may be reduced if UE sends additional information:

- Information about the maximum number of MGs/SMTC with restrictions that can be skipped within a time period,

- Information about the patterns of gap(s)/restriction(s) where skipping is feasible or acceptable.

As correctly also pointed out in [2], from the specification point of view, there is no mandated gNB behavior in response to any of the UE assistance information. This means that there cannot be any explicit or implicit requirement on gNB behaviour upon receiving such assistance data.

- Proposal 5: RAN4 will not specify any explicit or implicit requirement on gNB behaviour in relation to the UE assistance data.

On the information about the maximum number of MGs/SMTC with restrictions that can be skipped within a time period: this is more related to RRM measurement performance and therefore can be pre-defined in the specification (TS 38.133). Furthermore, such number may depend on MGRP, so it can be more relevant to specify the maximum time between the remaining occasions available for RRM measurements.

On the information about the patterns of gap(s)/restriction(s) where skipping is feasible or acceptable: RAN4 can discuss whether this problem can be solved by the above, e.g., by defining in the specification the maximum time between the occasions available for RRM measurements. Also, using patterns may limit network scheduling flexibility and at the same time not all UEs will have the same preferred patterns, while the network will have to support all UEs, while will ultimately result in high complexity in the network. This is in addition to that gNB is not required to follow the UE patterns, anyway.

- Proposal 6: RAN4 to discuss whether the information related to the maximum number of measurement occasions allowed for reallocated to XR over a time can be defined in RAN4 specification.

- Proposal 7: UE assistance containing patterns still cannot guarantee that gNB configuration will follow the UE pattern, since the configuration is ultimately the network decision.

A draft LS is provided below in R4-2413393.

# Summary

The following have been proposed in the current contribution:

- Proposal 1: To avoid limiting the XR operation to good network radio conditions only and hereby limiting the XR service coverage, RRM measurement requirements are enhanced for UEs under XR operation.

- Proposal 2: RAN4 to discuss possible enhancements to the legacy RRM requirements to account for the UE XR operation, e.g., whether pre-defined or dynamically extended RRM measurement period is to be specified for UE under XR operation to dynamically account for the availability of the RRM measurement occasions for RRM measurements.

- Proposal 3: RAN4 to discuss which conditions the measurement gap configurations need to meet to be relevant for the UE XR operation, e.g., minimum MGL and/or maximum MGL, maximum MGRP, maximum acceptable time separation between the XR occasions, etc.

- Proposal 4: RAN4 to discuss the impact on RRM samples and how they are combined during the UE XR operation, e.g., maximum acceptable time separation between two measurement occasions available for RRM measurements.

- Proposal 5: RAN4 will not specify any explicit or implicit requirement on gNB behaviour in relation to the UE assistance data.

- Proposal 6: RAN4 to discuss whether the information related to the maximum number of measurement occasions allowed for reallocated to XR over a time can be defined in RAN4 specification.

- Proposal 7: UE assistance containing patterns still cannot guarantee that gNB configuration will follow the UE pattern, since the configuration is ultimately the network decision.

# References

[1] RP-240791, WID on XR (eXtended Reality) for NR Phase 3, March 2024.

[2] R4-2411007/R1-2405736, LS on UE assistance information, RAN1, May 2024.