3GPP TSG-RAN WG4 Meeting # 112	R4-2413396

# Maastricht, Netherlands, 19th – 23rd August, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | DraftCR | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR 38.133 RRM specification improvement for clauses 10 to A_3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: |  |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | WF on RRM specification improvement in [R4-2410715]. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Section 10: Applied changes: - Removed old [] - “Note“ was removed in some cases outside tables, where the text belongs to the requirements. - Some tabs were added to align formatting. Further comments: - “PRB” or “RB”: both are used, choose one of them? None of them in the Abbreviation section of TS 38.133. - “BW”: used, but not in the Abbreviation section in TS 383.133. - “TRS” is used but not defined in Abbreviations section in TS 38.133. Section 12: Applied changes: - Clarified references. - Changed tabulation in some lists to align with the rest. - EUTRA changed to “E-UTRA”. - Removed the word “Note” in the requirements outside tables. Further comments: - PRB: not defined in Abbreviations section in TS 38.133. Sections A.1-A.3: Applied changes: - A.1 (Purpose of annex): added some simple text, since otherwise the first section of the annex is completely empty. - Duplicate text removed in: Table A.3.21.2-1, Table A.3.32.2-1 - Removed old (Rel-17 or earlier) square brackets - TBDs removed (the referenced clauses are void) - Removed FFS in: A.3.15.2.2 - Added missing references - Removed old (Rel-17 or earlier) editor’s notes in: A.3.12.1.2, A.3.12.2.2, A.3.13.1, A.3.15.3. Further comments: - “PRB” or “RB”: both are used, choose one of them? None of them in the Abbreviation section of TS 38.133. - “BW”: used, but not in the Abbreviations section in TS 383.133. - “TRS”: used, but not in the Abbreviations section in TS 383.133. - “CHO” (Conditional HO): used, but not defined in Abbreviations - “N/A” or “n.a.”: N/A is more common, should “n.a.” be changed to “N.A”? - Old (Rel-17 or earlier) FFS requiring discussion: Table A.3.15.3-1 - Old (Rel-17 or earlier) [] requiring discussion: Table A.3.36.2-1 (replace with the signaling reference?) - Old (Rel-17 or earlier) Editor’s notes requiring technical discussions: A.3.27 - New (Rel-18) []: Table A.3.14.2-3A ([db6]); A.3.15.5, A.3.15.6 - New (Rel-18) FFS: A.3.15.5 - There are many empty columns in test configuration tables – should be removed? - corresponding Band are fulfilled à corresponding band are fulfilled? - Table A.3.13A.5-1: correct references (A.6.3.X…)? |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The RAN4 agreement on specification quality improvement is not followed, the specification quality issue is not addressed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10, 12, 12A, A.1, A.2, A.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# 10	Measurement Performance requirements

## 10.1	NR measurements

### 10.1.1	Introduction

The requirements in clause 10.1 apply as follows:

-	intra-frequency requirements apply for PCell measurements in SA, NR-DC, or NE-DC operaion mode,

-	intra-frequency requirements apply for PSCell measurements in NR-DC or EN-DC operaion mode,

-	intra-frequency requirements apply for SCell measurements in SA operation mode with NR CA or any MR-DC operaion mode with NR CA,

-	inter-frequency requirements apply for non-serving cell measurements on NR carrier frequencies.

-	inter-frequency requirements apply for measurements from one cell on a frequency compared to the measurement from another cell on a different frequency.

In the requirements of clause 10.1, the exceptions for side conditions apply as follows:

-	for the UE capable of CA but not configured with any SCell, the applicable exceptions for side conditions are specified in Annex B, clause B.3.2.1 for UE supporting CA in FR1, and clause B.3.2.3 for UE supporting CA in FR2, respectively;

-	for the UE capable of CA and configured with at least one SCell, the applicable exceptions for side conditions are specified in Annex B, clause B.3.2.2 for UE configured with CA in FR1, and clause B.3.2.4 for UE supporting CA in FR2 respectively;

-	for the UE capable of SUL but not configured with SUL, the applicable exceptions for side conditions are specified in Annex B, clause B.3.4.1 for UE supporting SUL in FR1;

-	for the UE capable of SUL and configured with at least one SUL, the applicable exceptions for side conditions are specified in Annex B, clause B.3.4.2 for UE configured with SUL in FR1.

### 10.1.2	Intra-frequency RSRP accuracy requirements for FR1

#### 10.1.2.1	Intra-frequency SS-RSRP accuracy requirements

##### 10.1.2.1.1	Absolute SS-RSRP Accuracy

Unless otherwise specified, the requirements for absolute accuracy of SS-RSRP in this clause apply to a cell on the same frequency as that of the serving cell in FR1. The accuracy requirements in this clause are also applicable when highSpeedMeasFlag-r16 or highSpeedMeasCA-Scell-r17 is configured.

The accuracy requirements in Table 10.1.2.1.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for intra-frequency measurements are fulfilled according to Annex B.2.2 for a corresponding Band for each relevant SSB.

Table 10.1.2.1.1-1: SS-RSRP Intra frequency absolute accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 2 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
|  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | N/A | -70 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | N/A | -70 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | N/A | -70 |
| 4.5 | 9 | -6 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | N/A | -70 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | N/A | -70 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_N | -114.5 | -111.5 | N/A | -70 |
| 8 | 11 | -6 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A, NR_FDD_FR1_B, NR_TDD_FR1_C, NR_FDD_FR1_D, NR_TDD_FR1_D, NR_FDD_FR1_E, NR_TDD_FR1_E, NR_FDD_FR1_F, NR_FDD_FR1_G, NR_FDD_FR1_H, NR_FDD_FR1_N | N/A | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |

##### 10.1.2.1.2	Relative SS-RSRP Accuracy

The relative accuracy of SS-RSRP is defined as the SS-RSRP measured from one cell compared to the SS-RSRP measured from another cell on the same frequency, or between any two SS-RSRP levels measured on the same cell in FR1. The accuracy requirements in this clause are also applicable when highSpeedMeasFlag-r16 or highSpeedMeasCA-Scell-r17 is configured.

The accuracy requirements in Table 10.1.2.1.2-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for intra-frequency measurements are fulfilled according to Annex B.2.2 for a corresponding Band for each relevant SSB.

Table 10.1.2.1.2-1: SS-RSRP Intra frequency relative accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot Note 2 | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 4 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
|  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | N/A | -50 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | N/A | -50 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | N/A | -50 |
| 2 | 3 | -3 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | N/A | -50 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | N/A | -50 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_N | -114.5 | -111.5 | N/A | -50 |
| 3 | 3 | -6 | Note 3 | Note 3 | Note 3 | N/A | Note 3 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The parameter SSB Ês/Iot is the minimum SSB Ês/Iot of the pair of cells to which the requirement applies. NOTE 3: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 4: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |

#### 10.1.2.2	Void

#### 10.1.2.3	Intra-frequency CSI-RSRP accuracy requirements

##### 10.1.2.3.1	Absolute CSI-RSRP Accuracy

Unless otherwise specified, the requirements for absolute accuracy of CSI-RSRP in this clause apply to a cell where the CSI-RS resources to be measured have the same center frequency as the CSI-RS resources indicated for measurement in the serving cell in FR1.

The accuracy requirements in Table 10.1.2.3.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for intra-frequency measurements are fulfilled according to Annex B.2.2 for a corresponding Band for each associated SSB.

-	Conditions for intra-frequency measurements are fulfilled according to Annex B.2.8 for a corresponding Band for each relevant CSI-RS to be measured.

-	The bandwidth of CSI-RS is 48 PRBs and the density is 3. The performance with larger bandwidth of CSI-RS is equal to or better than the accuracy requirements in Table 10.1.2.3.1-1.

-	The timing offset between the reference measurement timing and the target CSI-RS in one layer is no larger than CP, where

the reference measurement timing for one layer for intra-frequency measurement is serving cell timing.

Table 10.1.2.3.1-1: CSI-RSRP Intra frequency absolute accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | CSI-RS Ês/Iot | Io Note 1 range |  |  |  |  |  |
|  |  |  | NR operating band groups Note 2 | Minimum Io |  |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSCSI-RS |  |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSCSI-RS = 15 kHz | SCSCSI-RS = 30 kHz | SCSCSI-RS = 60 kHz |  |  |
| 4.5 | 9 | -6 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | -115 | N/A | -70 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | -114.5 | N/A | -70 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | -114 | N/A | -70 |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | -113.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | -113 | N/A | -70 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | -112.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | -112 | N/A | -70 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | -111.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_N | -114.5 | -111.5 | -108.5 | N/A | -70 |
| 8 | 11 | -6 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A, NR_FDD_FR1_B, NR_TDD_FR1_C, NR_FDD_FR1_D, NR_TDD_FR1_D, NR_FDD_FR1_E, NR_TDD_FR1_E, NR_FDD_FR1_F, NR_FDD_FR1_G, NR_FDD_FR1_H, NR_FDD_FR1_N | N/A | N/A | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |  |

##### 10.1.2.3.2	Relative CSI-RSRP Accuracy

The relative accuracy of CSI-RSRP is defined as the CSI-RSRP measured from one cell compared to the CSI-RSRP measured from another cell on the same center frequency, or between any two CSI-RSRP levels measured on the same cell in FR1.

The accuracy requirements in Table 10.1.2.3.2-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for intra-frequency measurements are fulfilled according to Annex B.2.2 for a corresponding Band for each associated SSB.

-	Conditions for intra-frequency measurements are fulfilled according to Annex B.2.8 for a corresponding Band for each relevant CSI-RS to be measured.

-	The bandwidth of CSI-RS is 48 PRBs and the density is 3. The performance with larger bandwidth of CSI-RS is equal to or better than the accuracy requirements in Table 10.1.2.3.2-1.

-	The timing offset between the reference measurement timing and the target CSI-RS in one layer is no larger than CP, where

the reference measurement timing for one layer for intra-frequency measurement is serving cell timing.

Table 10.1.2.3.2-1: CSI-RSRP Intra frequency relative accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | CSI-RS Ês/Iot Note 2 | Io Note 1 range |  |  |  |  |  |
|  |  |  | NR operating band groups Note 4 | Minimum Io |  |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSCSI-RS |  |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSCSI-RS = 15 kHz | SCSCSI-RS = 30 kHz | SCSCSI-RS = 60 kHz |  |  |
| 2 | 3 | -3 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | -115 | N/A | -50 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | -114.5 | N/A | -50 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | -114 | N/A | -50 |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | -113.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | -113 | N/A | -50 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | -112.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | -112 | N/A | -50 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | -111.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_N | -114.5 | -111.5 | -108.5 | N/A | -50 |
| 3 | 3 | -6 | Note 3 | Note 3 | Note 3 | Note 3 | N/A | Note 3 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The parameter CSI-RS Ês/Iot is the minimum CSI-RS Ês/Iot of the pair of cells to which the requirement applies. NOTE 3: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 4: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |  |

### 10.1.2B	Intra-frequency RSRP accuracy requirements for FR1 for CA/DC Idle Mode Measurements

#### 10.1.2B.1	Intra-frequency SS-RSRP accuracy requirements

The requirements in this clause are applicable for a UE:

-	in state RRC_IDLE or RRC INACTIVE

-	that is synchronised to the cell that is measured.

#### The requirements are for absolute accuracy of SS-RSRP.

##### 10.1.2B.1.1	Absolute SS-RSRP Accuracy

Unless otherwise specified, the requirements for absolute accuracy of SS-RSRP in this clause apply to the serving cell in FR1.

The accuracy requirements in Table 10.1.2B.1.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for intra-frequency measurements are fulfilled according to Annex B.1.2 for a corresponding Band for each relevant SSB.

Table 10.1.2B.1.1-1: SS-RSRP Intra frequency absolute accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 2 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
|  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | N/A | -70 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | N/A | -70 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | N/A | -70 |
| 6 | 10.5 | -4 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | N/A | -70 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | N/A | -70 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_N | -114.5 | -111.5 | N/A | -70 |
| 9.5 | 12.5 | -4 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A, NR_FDD_FR1_B, NR_TDD_FR1_C, NR_FDD_FR1_D, NR_TDD_FR1_D, NR_FDD_FR1_E, NR_TDD_FR1_E, NR_FDD_FR1_F, NR_FDD_FR1_G, NR_FDD_FR1_H, NR_FDD_FR1_N | N/A | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |

### 10.1.2C	Intra-frequency RSRP accuracy requirements for FR1 SAN

#### 10.1.2C.1	Intra-frequency SS-RSRP accuracy requirements

##### 10.1.2C.1.1	Absolute SS-RSRP Accuracy

Unless otherwise specified, the requirements for absolute accuracy of SS-RSRP in this clause apply to a cell on the same frequency as that of the serving cell in FR1.

The accuracy requirements in Table 10.1.2C.1.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for intra-frequency measurements are fulfilled according to Annex B.2.2 for a corresponding Band for each relevant SSB.

-	Valid information for the SAN serving the target cell has been provided.

Table 10.1.2C.1.1-1: SS-RSRP Intra frequency absolute accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 2 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
| 4.5 | 9 | -6 | NR_FDD_SAB_FR1_A | -121 | -118 | N/A | -70 |
| 8 | 11 | -6 | NR_FDD_SAB_FR1_A | N/A | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |

##### 10.1.2C.1.2	Relative SS-RSRP Accuracy

The relative accuracy of SS-RSRP is defined as the SS-RSRP measured from one cell compared to the SS-RSRP measured from another cell on the same frequency, or between any two SS-RSRP levels measured on the same cell in FR1.

The accuracy requirements in Table 10.1.2C.1.2-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for intra-frequency measurements are fulfilled according to Annex B.2.2 for a corresponding Band for each relevant SSB.

-	Valid information for the SAN serving the target cell has been provided.

Table 10.1.2C.1.2-1: SS-RSRP Intra frequency relative accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot Note 2 | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 4 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
| 2 | 3 | -3 | NR_FDD_SAB_FR1_A | -121 | -118 | N/A | -50 |
| 3 | 3 | -6 | Note 3 | Note 3 | Note 3 | N/A | Note 3 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The parameter SSB Ês/Iot is the minimum SSB Ês/Iot of the pair of cells to which the requirement applies. NOTE 3: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 4: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |

### 10.1.3	Intra-frequency RSRP accuracy requirements for FR2

#### 10.1.3.1	Intra-frequency SS-RSRP accuracy requirements

##### 10.1.3.1.1	Absolute SS-RSRP Accuracy

Unless otherwise specified, the requirements for absolute accuracy of SS-RSRP in this clause apply to a cell on the same frequency as that of the serving cell in FR2.

The accuracy requirements in Table 10.1.3.1.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled.

-	Conditions for intra-frequency measurements are fulfilled according to Annex B.2.2 for a corresponding Band for each relevant SSB.

-	The measured signals are in the directions covered by the percentile EIS spherical coverage of the UE, defined in clause 7.3.4 of TS 38.101-2 [19].

Table 10.1.3.1.1-1: SS-RSRP Intra frequency absolute accuracy in FR2

| Accuracy |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 2 range |  |  |  |
|  |  |  | Minimum Io |  |  | Maximum Io |
| dB | dB | dB | dBm / SCSSSB Note 1 |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  | SCSSSB = 120kHz | SCSSSB = 240kHz |  |  |
| 6 | 9 | ≥-6 | Same value as SSB_RP in Table B.2.2-2, according to UE Power class, operating band and angle of arrival |  | N/A | -70 |
| 8 | 11 |  | N/A |  | -70 | -50 |
| Note 1: Values based on Refsens and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. Note 2: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. Note 3: In the test cases, the SSB Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. |  |  |  |  |  |  |

##### 10.1.3.1.2	Relative SS-RSRP Accuracy

The relative accuracy of SS-RSRP is defined as the SS-RSRP measured from one cell compared to the SS-RSRP measured from another cell on the same frequency, or between any two SS-RSRP levels measured on the same cell in FR2.

-	Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled.

-	Conditions for intra-frequency measurements are fulfilled according to Annex B.2.2 for a corresponding Band for each relevant SSB.

-	The measured signals are in the directions covered by the percentile EIS spherical coverage of the UE, defined in clause 7.3.4 of TS 38.101-2 [19].

Table 10.1.3.1.2-1: SS-RSRP Intra frequency relative accuracy in FR2

| Accuracy |  | Conditions |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 2 range |  |  |
|  |  |  | Minimum Io |  | Maximum Io |
| dB | dB | dB | dBm / SCSSSB Note 1 |  | dBm/BWChannel |
|  |  |  | SCSSSB = 120kHz | SCSSSB = 240kHz |  |
| 6 | 9 | ≥-6 | Same value as SSB_RP in Table B.2.2-2, according to UE Power class, operating band and angle of arrival |  | -50 |
| Note 1: Values based on Refsens and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. Note 2: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. Note 3: In the test cases, the SSB Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. Note 4: The parameter SSB Ês/Iot is the minimum SSB Ês/Iot of the pair of cells to which the requirement applies. |  |  |  |  |  |

#### 10.1.3.2	Void

#### 10.1.3.3	Intra-frequency CSI-RSRP accuracy requirements

##### 10.1.3.3.1	Absolute CSI-RSRP Accuracy

Unless otherwise specified, the requirements for absolute accuracy of CSI-RSRP in this clause apply to a cell where the CSI-RS resources to be measured have the same center frequency as the CSI-RS resources indicated for measurement in the serving cell in FR2.

The accuracy requirements in Table 10.1.3.3.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled.

-	Conditions for intra-frequency measurements are fulfilled according to Annex B.2.2 for a corresponding Band for each associated SSB(s).

-	Conditions for intra-frequency measurements are fulfilled according to Annex B.2.8 for a corresponding Band for each relevant CSI-RS to be measured.

-	The bandwidth of CSI-RS is 48 PRBs and the density is 3. The performance with larger bandwidth of CSI-RS is equal to or better than the accuracy requirements in Table 10.1.3.3.1-1.

-	The timing offset between the reference measurement timing and the target CSI-RS in one layer is no larger than CP, where

thethe reference measurement timing for one layer for intra-frequency measurement is serving cell timing.

-	The measured signals are in the directions covered by the percentile EIS spherical coverage of the UE, defined in clause 7.3.4 of TS 38.101-2 [19].

Table 10.1.3.3.1-1: CSI-RSRP Intra frequency absolute accuracy in FR2

| Accuracy |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | CSI-RS Ês/Iot | Io Note 2 range |  |  |  |
|  |  |  | Minimum Io |  |  | Maximum Io |
| dB | dB | dB | dBm / SCSCSI-RS Note 1 |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  | SCSCSI-RS = 60kHz | SCSCSI-RS = 120kHz |  |  |
| 6 | 9 | ≥-6 | Same value as CSI-RS_RP in Table B.2.8-2, according to UE Power class, operating band and angle of arrival |  | N/A | -70 |
| 8 | 11 |  | N/A |  | -70 | -50 |
| Note 1: Values based on Refsens and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. Note 2: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. Note 3: In the test cases, the CSI-RS Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. |  |  |  |  |  |  |

##### 10.1.3.3.2	Relative CSI-RSRP Accuracy

The relative accuracy of CSI-RSRP is defined as the CSI-RSRP measured from one cell compared to the CSI-RSRP measured from another cell on the same center frequency, or between any two CSI-RSRP levels measured on the same cell in FR2.

-	Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled.

-	Conditions for intra-frequency measurements are fulfilled according to Annex B.2.2 for a corresponding Band for each associated SSB(s).

-	Conditions for intra-frequency measurements are fulfilled according to Annex B.2.8 for a corresponding Band for each CSI-RS to be measured.

-	The bandwidth of CSI-RS is 48 PRBs and the density is 3. The performance with larger bandwidth of CSI-RS is equal to or better than the accuracy requirements in Table 10.1.3.3.2-1.

-	The timing offset between the reference measurement timing and the target CSI-RS in one layer is no larger than CP, where

the reference measurement timing for one layer for intra-frequency measurement is serving cell timing.

-	The measured signals are in the directions covered by the percentile EIS spherical coverage of the UE, defined in clause 7.3.4 of TS 38.101-2 [19].

Table 10.1.3.3.2-1: CSI-RSRP Intra frequency relative accuracy in FR2

| Accuracy |  | Conditions |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | CSI-RS Ês/Iot | Io Note 2 range |  |  |
|  |  |  | Minimum Io |  | Maximum Io |
| dB | dB | dB | dBm / SCS CSI-RS Note 1 |  | dBm/BWChannel |
|  |  |  | SCS CSI-RS = 60kHz | SCS CSI-RS = 120kHz |  |
| 6 | 9 | ≥-6 | Same value as CSI-RS_RP in Table B.2.8-2, according to UE Power class, operating band and angle of arrival |  | -50 |
| Note 1: Values based on Refsens and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. Note 2: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. Note 3: In the test cases, the CSI-RS Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. Note 4: The parameter CSI-RS Ês/Iot is the minimum CSI-RS Ês/Iot of the pair of cells to which the requirement applies. |  |  |  |  |  |

### 10.1.3B	Intra-frequency RSRP accuracy requirements for FR2 for CA/DC Idle Mode Measurements

#### 10.1.3B.1	Intra-frequency SS-RSRP accuracy requirements

The requirements in this clause are applicable for a UE:

-	in state RRC_IDLE or RRC INACTIVE

-	that is synchronised to the cell that is measured.

The requirements are for absolute accuracy of SS-RSRP.

##### 10.1.3B.1.1	Absolute SS-RSRP Accuracy

Unless otherwise specified, the requirements for absolute accuracy of SS-RSRP in this clause apply to the serving cell in FR2.

The accuracy requirements in Table 10.1.3B.1.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled.

-	Conditions for intra-frequency measurements are fulfilled according to Annex B.1.2 for a corresponding Band for each relevant SSB.

-	The measured signals are in the directions covered by the percentile EIS spherical coverage of the UE, defined in clause 7.3.4 of TS 38.101-2 [19].

Table 10.1.3B.1.1-1: SS-RSRP Intra frequency absolute accuracy in FR2

| Accuracy |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 2 range |  |  |  |
|  |  |  | Minimum Io |  |  | Maximum Io |
| dB | dB | dB | dBm / SCSSSB Note 1 |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  | SCSSSB = 120kHz | SCSSSB = 240kHz |  |  |
| 7.5 | 10.5 | ≥-4 | Same value as SSB_RP in Table B.2.2-2, according to UE Power class, operating band and angle of arrival |  | N/A | -70 |
| 9.5 | 12.5 |  | N/A |  | -70 | -50 |
| Note 1: Values based on Refsens and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. Note 2: Io specified at the Reference point and assumed to have constant EPRE across the bandwidth. Note 3: In the test cases, the SSB Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. |  |  |  |  |  |  |

### 10.1.4	Inter-frequency RSRP accuracy requirements for FR1

#### 10.1.4.1	Inter-frequency SS-RSRP accuracy requirements

##### 10.1.4.1.1	Absolute Accuracy of SS-RSRP in FR1

The requirements for absolute accuracy of SS-RSRP in this clause apply to a cell on a frequency in FR1 that has different carrier frequency from the serving cell. The accuracy requirements in this clause are also applicable when highSpeedMeasInterFreq-r17 is configured.

The accuracy requirements in Table 10.1.4.1.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for inter-frequency measurements are fulfilled according to Annex B.2.3 for a corresponding Band for each relevant SSB.

Table 10.1.4.1.1-1: SS-RSRP Inter frequency Absolute accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot Note 2 | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 3 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
|  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | N/A | -70 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | N/A | -70 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | N/A | -70 |
| 4.5 | 9 | -6 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | N/A | -70 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | N/A | -70 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_N | -114.5 | -111.5 | N/A | -70 |
| 8 | 11 | -6 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A, NR_FDD_FR1_B, NR_TDD_FR1_C, NR_FDD_FR1_D, NR_TDD_FR1_D, NR_FDD_FR1_E, NR_TDD_FR1_E, NR_FDD_FR1_F, NR_FDD_FR1_G, NR_FDD_FR1_H, NR_FDD_FR1_N | N/A | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: Void NOTE 3: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |

##### 10.1.4.1.2	Relative Accuracy of SS-RSRP in FR1

The relative accuracy of SS-RSRP in inter frequency case is defined as the RSRP measured from one cell on a frequency in FR1compared to the RSRP measured from another cell on a different frequency in FR1. The accuracy requirements in this clause are also applicable when highSpeedMeasInterFreq-r17 is configured.

The accuracy requirements in Table 10.1.4.1.2-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] Clause 7.3 for reference sensitivity are fulfilled.

-	Conditions for inter-frequency measurements are fulfilled according to Annex B.2.3 for a corresponding Band for each relevant SSB.

-	|SSB_RP1dBm - SSB_RP2dBm| ≤ 27 dB

-	|Channel 1_Io -Channel 2_Io |  20 dB

Table 10.1.4.1.2-1: SS-RSRP Inter frequency relative accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot Note 2 | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 3 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
|  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | N/A | -50 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | N/A | -50 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | N/A | -50 |
| 4.5 | 6 | -6 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | N/A | -50 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | N/A | -50 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_N | -114.5 | -111.5 | N/A | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The parameter SSB Ês/Iot is the minimum SSB Ês/Iot of the pair of cells to which the requirement applies. NOTE 3: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |

#### 10.1.4.2	Void

#### 10.1.4.3	Inter-frequency CSI-RSRP accuracy requirements

##### 10.1.4.3.1	Absolute Accuracy of CSI-RSRP in FR1

The requirements for absolute accuracy of CSI-RSRP in this clause apply to a cell where the CSI-RS resources to be measured have the different center frequency as the CSI-RS resources indicated for measurement in the serving cell in FR1.

The accuracy requirements in Table 10.1.4.3.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for inter-frequency measurements are fulfilled according to Annex B.2.3 for a corresponding Band for each relevant SSB.

-	Conditions for inter-frequency measurements are fulfilled according to Annex B.2.9 for a corresponding Band for each relevant CSI-RS to be measured.

-	The bandwidth of CSI-RS is 48 PRBs and the density is 3. The performance with larger bandwidth of CSI-RS is equal to or better than the accuracy requirements in Table 10.1.4.3.1-1.

-	The timing offset between the reference measurement timing and the target CSI-RS in one layer is no larger than CP, where

the reference measurement timing for one layer for inter-frequency measurement is up to UE implementation and shall be based on the timing of one of the target cells.

Table 10.1.4.3.1-1: CSI-RSRP Inter frequency Absolute accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | CSI-RS Ês/Iot | Io Note 1 range |  |  |  |  |  |
|  |  |  | NR operating band groups Note 2 | Minimum Io |  |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSCSI-RS |  |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSCSI-RS = 15 kHz | SCSCSI-RS = 30 kHz | SCSCSI-RS = 60 kHz |  |  |
|  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | -115 | N/A | -70 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | -114.5 | N/A | -70 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | -114 | N/A | -70 |
| 4.5 | 9 | -6 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | -113.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | -113 | N/A | -70 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | -112.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | -112 | N/A | -70 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | -111.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_N | -114.5 | -111.5 | -108.5 | N/A | -70 |
| 8 | 11 | -6 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A, NR_FDD_FR1_B, NR_TDD_FR1_C, NR_FDD_FR1_D, NR_TDD_FR1_D, NR_FDD_FR1_E, NR_TDD_FR1_E, NR_FDD_FR1_F, NR_FDD_FR1_G, NR_FDD_FR1_H, NR_FDD_FR1_N | N/A | N/A | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |  |

##### 10.1.4.3.2	Relative Accuracy of CS-RSRP in FR1

The relative accuracy of CSI-RSRP in inter frequency case is defined as the CSI-RSRP measured from one cell on a frequency in FR1compared to the CSI-RSRP measured from another cell on a different frequency in FR1.

The accuracy requirements in Table 10.1.4.3.2-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] Clause 7.3 for reference sensitivity are fulfilled.

-	Conditions for inter-frequency measurements are fulfilled according to Annex B.2.3 for a corresponding Band for each relevant SSB.

-	Conditions for inter-frequency measurements are fulfilled according to Annex B.2.9 for a corresponding Band for each relevant CSI-RS to be measured.

-	The bandwidth of CSI-RS is 48 PRBs and the density is 3. The performance with larger bandwidth of CSI-RS is equal to or better than the accuracy requirements in Table 10.1.4.3.2-1.

-	The timing offset between the reference measurement timing and the target CSI-RS in one layer is no larger than CP, where

the reference measurement timing for one layer for inter-frequency measurement is up to UE implementation and shall be based on the timing of one of the target cells.

-	|CSI_RP1d–m - CSI_RP2dBm| ≤ 27 dB

-	|Channel 1_Io -Channel 2_Io |  20 dB

Table 10.1.4.3.2-1: CSI-RSRP Inter frequency relative accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | CSI-RS Ês/Iot Note 2 | Io Note 1 range |  |  |  |  |  |
|  |  |  | NR operating band groups Note 4 | Minimum Io |  |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSCSI-RS |  |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSCSI-RS = 15 kHz | SCSCSI-RS = 30 kHz | SCSCSI-RS = 60 kHz |  |  |
|  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | -115 | N/A | -50 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | -114.5 | N/A | -50 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | -114 | N/A | -50 |
| 4.5 | 6 | -6 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | -113.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | -113 | N/A | -50 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | -112.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | -112 | N/A | -50 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | -111.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_N | -113.5 | -111.5 | -108.5 | N/A | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The parameter CSI-RS Ês/Iot is the minimum CSI-RS Ês/Iot of the pair of CSI-RS resources to which the requirement applies. NOTE 3: Void NOTE 4: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |  |

### 10.1.4B	Inter-frequency RSRP accuracy requirements for FR1 for CA/DC Idle Mode Measurements

#### 10.1.4B.1	Inter-frequency SS-RSRP accuracy requirements

The requirements in this clause are applicable for a UE:

-	in state RRC_IDLE or RRC INACTIVE

-	that is synchronised to the cell that is measured.

The requirements are for absolute accuracy of SS-RSRP.

##### 10.1.4B.1.1	Absolute Accuracy of SS-RSRP in FR1

The requirements for absolute accuracy of SS-RSRP in this clause apply to a cell on a frequency in FR1 that has different carrier frequency from the serving cell.

The accuracy requirements in Table 10.1.4B.1.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for inter-frequency measurements are fulfilled according to Annex B.1.3 for a corresponding Band for each relevant SSB.

Table 10.1.4B.1.1-1: SS-RSRP Inter frequency Absolute accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot Note 2 | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 3 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
|  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | N/A | -70 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | N/A | -70 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | N/A | -70 |
| 6 | 10.5 | -4 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | N/A | -70 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | N/A | -70 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_N | -114.5 | -111.5 | N/A | -70 |
| 9.5 | 12.5 | -4 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A, NR_FDD_FR1_B, NR_TDD_FR1_C, NR_FDD_FR1_D, NR_TDD_FR1_D, NR_FDD_FR1_E, NR_TDD_FR1_E, NR_FDD_FR1_F, NR_FDD_FR1_G, NR_FDD_FR1_H, NR_FDD_FR1_N | N/A | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: Void NOTE 3: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |

### 10.1.4C	Inter-frequency RSRP accuracy requirements for FR1 SAN

#### 10.1.4C.1	Inter-frequency SS-RSRP accuracy requirements

##### 10.1.4C.1.1	Absolute Accuracy of SS-RSRP in FR1

The requirements for absolute accuracy of SS-RSRP in this clause apply to a cell on a frequency in FR1 that has different carrier frequency from the serving cell.

The accuracy requirements in Table 10.1.4C.1.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for inter-frequency measurements are fulfilled according to Annex B.2.3 for a corresponding Band for each relevant SSB.

-	Valid information for the SAN serving the target cell has been provided.

Table 10.1.4C.1.1-1: SS-RSRP Inter frequency Absolute accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot Note 2 | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 3 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
| 4.5 | 9 | -6 | NR_FDD_SAB_FR1_A | -121 | -118 | N/A | -70 |
| 8 | 11 | -6 | NR_FDD_SAB_FR1_A | N/A | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: Void NOTE 3: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |

##### 10.1.4C.1.2	Relative Accuracy of SS-RSRP in FR1

The relative accuracy of SS-RSRP in inter frequency case is defined as the RSRP measured from one cell on a frequency in FR1compared to the RSRP measured from another cell on a different frequency in FR1.

The accuracy requirements in Table 10.1.4C.1.2-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] Clause 7.3 for reference sensitivity are fulfilled.

-	Conditions for inter-frequency measurements are fulfilled according to Annex B.2.3 for a corresponding Band for each relevant SSB.

-	|SSB_RP1d–m - SSB_RP2dBm| ≤ 27 dB

-	|Channel 1_Io -Channel 2_Io |  20 dB

-	Valid information for the SAN serving the target cell has been provided.

Table 10.1.4C.1.2-1: SS-RSRP Inter frequency relative accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot Note 2 | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 3 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
| 4.5 | 6 | -6 | NR_FDD_SAB_FR1_A | -121 | -118 | N/A | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The parameter SSB Ês/Iot is the minimum SSB Ês/Iot of the pair of cells to which the requirement applies. NOTE 3: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |

### 10.1.5	Inter-frequency RSRP accuracy requirements for FR2

#### 10.1.5.1	Inter-frequency SS-RSRP accuracy requirements

##### 10.1.5.1.1	Absolute SS-RSRP Accuracy

Unless otherwise specified, the requirements for absolute accuracy of SS-RSRP in this clause apply to a cell on a frequency in FR2 that is on a different frequency than the serving cell.

The accuracy requirements in Table 10.1.5.1.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled.

-	Conditions for inter-frequency measurements are fulfilled according to Annex B.2.3 for a corresponding Band for each relevant SSB.

-	The measured signals are in the directions covered by the percentile EIS spherical coverage of the UE, defined in clause 7.3.4 of TS 38.101-2 [19].

Table 10.1.5.1.1-1: SS-RSRP Inter frequency absolute accuracy in FR2

| Accuracy |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 2 range |  |  |  |
|  |  |  | Minimum Io |  |  | Maximum Io |
| dB | dB | dB | dBm / SCSSSB Note 1 |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  | SCSSSB = 120kHz | SCSSSB = 240kHz |  |  |
| 6 | 9 | ≥-4 | Same value as SSB_RP in Table B.2.3-2, according to UE Power class, operating band and angle of arrival |  | N/A | -70 |
| 8 | 11 |  | N/A |  | -70 | -50 |
| Note 1: Values based on Refsens and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. Note 2: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. Note 3: In the test cases, the SSB Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. |  |  |  |  |  |  |

##### 10.1.5.1.2	Relative SS-RSRP Accuracy

The relative accuracy of SS-RSRP is defined as the SS-RSRP measured from one cell on a frequency in FR2 compared to the SS-RSRP measured from another cell on another frequency in FR2.

The accuracy requirements in Table 10.1.5.1.2-1 are valid under the following conditions:

-	Conditions defined in 38.101-2 [19] Clause 7.3 for reference sensitivity are fulfilled.

-	Conditions for inter-frequency measurements are fulfilled according to Annex B.2.3 for a corresponding Band for each relevant SSB.

-	|SSB_RP1d–m - SSB_RP2dBm| ≤ 27dB

-	|Channel 1_Io -Channel 2_Io |  20 dB

-	The measured signals are in the directions covered by the percentile EIS spherical coverage of the UE, defined in clause 7.3.4 of TS 38.101-2 [19].

Table 10.1.5.1.2-1: SS-RSRP Inter frequency relative accuracy in FR2

| Accuracy |  | Conditions |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 2 range |  |  |
|  |  |  | Minimum Io |  | Maximum Io |
| dB | dB | dB | dBm / SCSSSB Note 1 |  | dBm/BWChannel |
|  |  |  | SCSSSB = 120kHz | SCSSSB = 240kHz |  |
| 6 | 9 | ≥-4 | Same value as SSB_RP in Table B.2.3-2, according to UE Power class, operating band and angle of arrival |  | -50 |
| Note 1: Values based on Refsens and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. Note 2: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. Note 3: In the test cases, the SSB Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. Note 4: The parameter SSB Ês/Iot is the minimum SSB Ês/Iot of the pair of cells to which the requirement applies. |  |  |  |  |  |

#### 10.1.5.2	Void

#### 10.1.5.3	Inter-frequency CSI-RSRP accuracy requirements

##### 10.1.5.3.1	Absolute CSI-RSRP Accuracy

Unless otherwise specified, the requirements for absolute accuracy of CSI-RSRP in this clause apply to a cell on a frequency in FR2 where the CSI-RS resources to be measured have the different center frequency as the CSI-RS resources indicated for measurement in the serving cell.

The accuracy requirements in Table 10.1.5.3.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled.

-	Conditions for inter-frequency measurements are fulfilled according to Annex B.2.3 for a corresponding Band for each relevant associated SSB.

-	Conditions for inter-frequency measurements are fulfilled according to Annex B.2.9 for a corresponding Band for each relevant CSI-RS to be measured.

-	The bandwidth of CSI-RS is 48 PRBs and the density is 3. The performance with larger bandwidth of CSI-RS is equal to or better than the accuracy requirements in Table 10.1.5.3.1-1.

-	The timing offset between the reference measurement timing and the target CSI-RS in one layer is no larger than CP, where

the reference measurement timing for one layer for inter-frequency measurement is up to UE implementation and shall be based on the timing of one of the target cells.

-	The measured signals are in the directions covered by the percentile EIS spherical coverage of the UE, defined in clause 7.3.4 of TS 38.101-2 [19].

Table 10.1.5.3.1-1: CSI-RSRP Inter frequency absolute accuracy in FR2

| Accuracy |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | CSI-RS Ês/Iot | Io Note 2 range |  |  |  |
|  |  |  | Minimum Io |  |  | Maximum Io |
| dB | dB | dB | dBm / SCSCSI-RS Note 1 |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  | SCSCSI-RS = 60kHz | SCSCSI-RS = 120kHz |  |  |
| 6 | 9 | ≥-4 | Same value as CSI_RP in Table B.2.9-2, according to UE Power class, operating band and angle of arrival |  | N/A | -70 |
| 8 | 11 |  | N/A |  | -70 | -50 |
| Note 1: Values based on Refsens and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. Note 2: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. Note 3: In the test cases, the CSI-RS Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. |  |  |  |  |  |  |

##### 10.1.5.3.2	Relative CSI-RSRP Accuracy

The relative accuracy of CSI-RSRP in inter frequency case is defined as the CSI-RSRP measured from one cell on a frequency in FR2 compared to the CSI-RSRP measured from another cell on another frequency in FR2.

The accuracy requirements in Table 10.1.5.3.2-1 are valid under the following conditions:

-	Conditions defined in 38.101-2 [19] Clause 7.3 for reference sensitivity are fulfilled.

-	Conditions for inter-frequency measurements are fulfilled according to Annex B.2.3 for a corresponding Band for each relevant associated SSB.

-	Conditions for inter-frequency measurements are fulfilled according to Annex B.2.9 for a corresponding Band for each relevant CSI-RS to be measured.

-	The bandwidth of CSI-RS is 48 PRBs and the density is 3. The performance with larger bandwidth of CSI-RS is equal to or better than the accuracy requirements in Table 10.1.5.3.2-1.

-	The timing offset between the reference measurement timing and the target CSI-RS in one layer is no larger than CP, where

the reference measurement timing for one layer for inter-frequency measurement is up to UE implementation and shall be based on the timing of one of the target cells.

-	|CSI_RP1dBm - CSI_RP2dBm| ≤ 27dB

-	|Channel 1_Io -Channel 2_Io |  20 dB

-	The measured signals are in the directions covered by the percentile EIS spherical coverage of the UE, defined in clause 7.3.4 of TS 38.101-2 [19].

Table 10.1.5.3.2-1: CSI-RSRP Inter frequency relative accuracy in FR2

| Accuracy |  | Conditions |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | CSI-RS Ês/Iot | Io Note 2 range |  |  |
|  |  |  | Minimum Io |  | Maximum Io |
| dB | dB | dB | dBm / SCSCSI-RS Note 1 |  | dBm/BWChannel |
|  |  |  | SCSCSI-RS = 60kHz | SCSCSI-RS = 120kHz |  |
| 6 | 9 | ≥-4 | Same value as CSI_RP in Table B.2.9-2, according to UE Power class, operating band and angle of arrival |  | -50 |
| Note 1: Values based on Refsens and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. Note 2: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. Note 3: In the test cases, the CSI-RS Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. Note 4: The parameter CSI-RS Ês/Iot is the minimum CSI-RS Ês/Iot of the pair of cells to which the requirement applies. |  |  |  |  |  |

### 10.1.5B	Inter-frequency RSRP accuracy requirements for FR2 for CA/DC Idle Mode Measurements

#### 10.1.5B.1	Inter-frequency SS-RSRP accuracy requirements

The requirements in this clause are applicable for a UE:

-	in state RRC_IDLE or RRC INACTIVE

-	that is synchronised to the cell that is measured.

The requirements are for absolute accuracy of SS-RSRP.

##### 10.1.5B.1.1	Absolute SS-RSRP Accuracy

Unless otherwise specified, the requirements for absolute accuracy of SS-RSRP in this clause apply to a cell on a frequency in FR2 that is on a different frequency than the serving cell.

The accuracy requirements in Table 10.1.5B.1.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled.

-	Conditions for inter-frequency measurements are fulfilled according to Annex B.1.3 for a corresponding Band for each relevant SSB.

-	The measured signals are in the directions covered by the percentile EIS spherical coverage of the UE, defined in clause 7.3.4 of TS 38.101-2 [19].

Table 10.1.5B.1.1-1: SS-RSRP Inter frequency absolute accuracy in FR2

| Accuracy |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 2 range |  |  |  |
|  |  |  | Minimum Io |  |  | Maximum Io |
| dB | dB | dB | dBm / SCSSSB Note 1 |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  | SCSSSB = 120kHz | SCSSSB = 240kHz |  |  |
| 7.5 | 10.5 | ≥-4 | Same value as SSB_RP in Table B.2.3-2, according to UE Power class, operating band and angle of arrival |  | N/A | -70 |
| 9.5 | 12.5 |  | N/A |  | -70 | -50 |
| Note 1: Values based on Refsens and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. Note 2: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. Note 3: In the test cases, the SSB Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. |  |  |  |  |  |  |

### 10.1.6	RSRP Measurement Report Mapping

The reporting range of SS-RSRP and CSI-RSRP for L3 reporting is defined from -156 dBm to -31 dBm with 1 dB resolution. The reporting range of SS-RSRP and CSI-RSRP for L1 reporting is defined from -140 to -44 dBm with 1 dB resolution.

The mapping of measured quantity is defined in Table 10.1.6.1-1. The range in the signalling may be larger than the guaranteed accuracy range.

The reporting range of differential SS-RSRP and CSI-RSRP for L1 reporting and L3 reporting is defined from 0 dB to -30 dB with 2 dB resolution.

The mapping of measured quantity is defined in Table 10.1.6.1-2. The range in the signalling may be larger than the guaranteed accuracy range.

Table 10.1.6.1-1: SS-RSRP and CSI-RSRP measurement report mapping

| Reported value | Measured quantity value (L3 SS-RSRP) and CSI-RSRP | Measured quantity value (L1 SS-RSRP and CSI-RSRP) | Unit |
| --- | --- | --- | --- |
| RSRP_0 | SS-RSRP<-156 | Not valid | dBm |
| RSRP_1 | -156≤ SS-RSRP<-155 | Not valid | dBm |
| RSRP_2 | -155≤ SS-RSRP<-154 | Not valid | dBm |
| RSRP_3 | -154≤ SS-RSRP<-153 | Not valid | dBm |
| RSRP_4 | -153≤ SS-RSRP<-152 | Not valid | dBm |
| RSRP_5 | -152≤ SS-RSRP<-151 | Not valid | dBm |
| RSRP_6 | -151≤ SS-RSRP<-150 | Not valid | dBm |
| RSRP_7 | -150≤ SS-RSRP<-149 | Not valid | dBm |
| RSRP_8 | -149≤ SS-RSRP<-148 | Not valid | dBm |
| RSRP_9 | -148≤ SS-RSRP<-147 | Not valid | dBm |
| RSRP_10 | -147≤ SS-RSRP<-146 | Not valid | dBm |
| RSRP_11 | -146≤ SS-RSRP<-145 | Not valid | dBm |
| RSRP_12 | -145≤ SS-RSRP<-144 | Not valid | dBm |
| RSRP_13 | -144≤ SS-RSRP<-143 | Not valid | dBm |
| RSRP_14 | -143≤ SS-RSRP<-142 | Not valid | dBm |
| RSRP_15 | -142≤ SS-RSRP<-141 | Not valid | dBm |
| RSRP_16 | -141≤ SS-RSRP<-140 | RSRP<-140 | dBm |
| RSRP_17 | -140≤ SS-RSRP<-139 | -140≤RSRP<-139 | dBm |
| RSRP_18 | -139≤ SS-RSRP<-138 | -139≤ RSRP<-138 | dBm |
| … | … |  | … |
| RSRP_111 | -46≤ SS-RSRP<-45 | -46≤ RSRP<-45 | dBm |
| RSRP_112 | -45≤ SS-RSRP<-44 | -45≤ RSRP<-44 | dBm |
| RSRP_113 | -44≤ SS-RSRP<-43 | -44≤ RSRP | dBm |
| RSRP_114 | -43≤ SS-RSRP<-42 | Not valid | dBm |
| RSRP_115 | -42≤ SS-RSRP<-41 | Not valid | dBm |
| RSRP_116 | -41≤ SS-RSRP<-40 | Not valid | dBm |
| RSRP_117 | -40≤ SS-RSRP<-39 | Not valid | dBm |
| RSRP_118 | -39≤ SS-RSRP<-38 | Not valid | dBm |
| RSRP_119 | -38≤ SS-RSRP<-37 | Not valid | dBm |
| RSRP_120 | -37≤ SS-RSRP<-36 | Not valid | dBm |
| RSRP_121 | -36≤ SS-RSRP<-35 | Not valid | dBm |
| RSRP_122 | -35≤ SS-RSRP<-34 | Not valid | dBm |
| RSRP_123 | -34≤ SS-RSRP<-33 | Not valid | dBm |
| RSRP_124 | -33≤ SS-RSRP<-32 | Not valid | dBm |
| RSRP_125 | -32≤ SS-RSRP<-31 | Not valid | dBm |
| RSRP_126 | -31≤ SS-RSRP | Not valid | dBm |
| RSRP_127 (Note) | Infinity | Infinity | dBm |
| Note: The value of RSRP_127 is applicable for RSRP threshold configured by the network as defined in TS 38.331 [2], but not for the purpose of measurement reporting. |  |  |  |

Table 10.1.6.1-2: Differential SS-RSRP and CSI-RSRP measurement (for L1 reporting and L3 reporting) report mapping

| Reported value | Measured quantity value (difference in measured RSRP from strongest RSRP) | Unit |
| --- | --- | --- |
| DIFFRSRP_0 | 0≥ΔRSRP>-2 | dB |
| DIFFRSRP_1 | -2≥ΔRSRP>-4 | dB |
| DIFFRSRP_2 | -4≥ΔRSRP>-6 | dB |
| DIFFRSRP_3 | -6≥ΔRSRP>-8 | dB |
| DIFFRSRP_4 | -8≥ΔRSRP>-10 | dB |
| DIFFRSRP_5 | -10≥ΔRSRP>-12 | dB |
| DIFFRSRP_6 | -12≥ΔRSRP>-14 | dB |
| DIFFRSRP_7 | -14≥ΔRSRP>-16 | dB |
| DIFFRSRP_8 | -16≥ΔRSRP>-18 | dB |
| DIFFRSRP_9 | -18≥ΔRSRP>-20 | dB |
| DIFFRSRP_10 | -20≥ΔRSRP>-22 | dB |
| DIFFRSRP_11 | -22≥ΔRSRP>-24 | dB |
| DIFFRSRP_12 | -24≥ΔRSRP>-26 | dB |
| DIFFRSRP_13 | -26≥ΔRSRP>-28 | dB |
| DIFFRSRP_14 | -28≥ΔRSRP>-30 | dB |
| DIFFRSRP_15 | -30≥ΔRSRP | dB |

### 10.1.7	Intra-frequency RSRQ accuracy requirements for FR1

#### 10.1.7.1	Intra-frequency SS-RSRQ accuracy requirements in FR1

##### 10.1.7.1.1	Absolute SS-RSRQ Accuracy in FR1

Unless otherwise specified, the requirements for absolute accuracy of SS-RSRQ in this clause apply to a cell on the same frequency as that of the serving cell in FR1. The accuracy requirements in this clause are also applicable when highSpeedMeasFlag-r16 or highSpeedMeasCA-Scell-r17 is configured.

The accuracy requirements in Table 10.1.7.1.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for intra-frequency measurements are fulfilled according to Annex B.2.2 for a corresponding Band for each relevant SSB.

Table 10.1.7.1.1-1: SS-RSRQ Intra frequency absolute accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 3 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
|  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | N/A | -50 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | N/A | -50 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | N/A | -50 |
| 2.5 | 4 | -3 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | N/A | -50 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | N/A | -50 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_N | -114.5 | -111.5 | N/A | -50 |
| 3.5 | 4 | -6 | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 3: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |

#### 10.1.7.2	Intra-frequency CSI-RSRQ accuracy requirements

##### 10.1.7.2.1	Absolute CSI-RSRQ Accuracy

Unless otherwise specified, the requirements for absolute accuracy of CSI-RSRQ in this clause apply to the intra-frequency measurement defined in 9.10.2.1 in FR1.

The accuracy requirements in Table 10.1.7.2.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for intra-frequency measurements are fulfilled according to Annex B.2.2 for a corresponding Band for associated SSB.

-	Conditions for intra-frequency measurements are fulfilled according to Annex B.2.8 for a corresponding Band for each relevant CSI-RS.

-	The bandwidth of CSI-RS is 48 PRBs and the density is 3.

•	The performance with larger bandwidth of CSI-RS is equal to or better than the accuracy requirements in Table 10.1.7.2.1-1.

-	The timing offset between the reference measurement timing and the target CSI-RS in one layer is no larger than CP, where

the reference measurement timing for one layer for intra-frequency measurement is serving cell timing.

Table 10.1.7.2.1-1: CSI-RSRQ Intra frequency absolute accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | CSI-RS Ês/Iot | Io Note 1 range |  |  |  |  |  |
|  |  |  | NR operating band groups Note 3 | Minimum Io |  |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSCSI-RS |  |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSCSI-RS = 15 kHz | SCSCSI-RS = 30 kHz | SCSCSI-RS = 60 kHz |  |  |
|  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | -115 | N/A | -50 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | -114.5 | N/A | -50 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | -114 | N/A | -50 |
| 2.5 | 4 | -3 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | -113.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | -113 | N/A | -50 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | -112.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | -112 | N/A | -50 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | -111.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_N | -114.5 | -111.5 | -108.5 | N/A | -50 |
| 3.5 | 4 | -6 | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 3: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |  |

### 10.1.7B	Intra-frequency RSRQ accuracy requirements for FR1 for CA/DC Idle Mode Measurements

#### 10.1.7B.1	Intra-frequency SS-RSRQ accuracy requirements in FR1

The requirements in this clause are applicable for a UE:

-	in state RRC_IDLE or RRC INACTIVE

-	that is synchronised to the cell that is measured.

The requirements are for absolute accuracy of SS-RSRQ.

##### 10.1.7B.1.1	Absolute SS-RSRQ Accuracy in FR1

Unless otherwise specified, the requirements for absolute accuracy of SS-RSRQ in this clause apply to the serving cell in FR1.

The accuracy requirements in Table 10.1.7B.1.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for intra-frequency measurements are fulfilled according to Annex B.1.2 for a corresponding Band for each relevant SSB.

Table 10.1.7B.1.1-1: SS-RSRQ Intra frequency absolute accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 3 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
|  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | N/A | -50 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | N/A | -50 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | N/A | -50 |
| 4 | 5.5 | -3 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | N/A | -50 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | N/A | -50 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_N | -114.5 | -111.5 | N/A | -50 |
| 5 | 5.5 | -4 | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 3: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |

### 10.1.7C	Intra-frequency RSRQ accuracy requirements for FR1 SAN

#### 10.1.7C.1	Intra-frequency SS-RSRQ accuracy requirements in FR1

##### 10.1.7C.1.1	Absolute SS-RSRQ Accuracy in FR1

Unless otherwise specified, the requirements for absolute accuracy of SS-RSRQ in this clause apply to a cell on the same frequency as that of the serving cell in FR1.

The accuracy requirements in Table 10.1.7C.1.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for intra-frequency measurements are fulfilled according to Annex B.2.2 for a corresponding Band for each relevant SSB.

-	Valid information for the SAN serving the target cell has been provided.

Table 10.1.7C.1.1-1: SS-RSRQ Intra frequency absolute accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 3 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
| 2.5 | 4 | -3 | NR_FDD_SAB_FR1_A | -121 | -118 | N/A | -50 |
| 3.5 | 4 | -6 | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 3: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |

### 10.1.8	Intra-frequency RSRQ accuracy requirements for FR2

#### 10.1.8.1	Intra-frequency SS-RSRQ accuracy requirements in FR2

##### 10.1.8.1.1	Absolute SS-RSRQ Accuracy in FR2

Unless otherwise specified, the requirements for absolute accuracy of SS-RSRQ in this clause apply to a cell on the same frequency as that of the serving cell in FR2.

The accuracy requirements in Table 10.1.8.1.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled.

-	Conditions for intra-frequency measurements are fulfilled according to Annex B.2.2 for a corresponding Band for each relevant SSB.

-	The measured signals are in the directions covered by the percentile EIS spherical coverage of the UE, defined in clause 7.3.4 of TS 38.101-2 [19].

Table 10.1.8.1.1-1: SS-RSRQ Intra frequency absolute accuracy in FR2

| Accuracy |  | Conditions |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 2 range |  |  |
|  |  |  | Minimum Io |  | Maximum Io |
| dB | dB | dB | dBm / SCSSSB Note 1 |  | dBm/BWChannel |
|  |  |  | SCSSSB = 120kHz | SCSSSB = 240kHz |  |
| 2.5 | 4 | ≥-3 | Same value as SSB_RP in Table B.2.2-2, according to UE Power class, operating band and angle of arrival |  | -50 |
| 3.5 | 4 | ≥-6 |  |  |  |
| Note 1: Values based on Refsens and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. Note 2: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. Note 3: In the test cases, the SSB Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. |  |  |  |  |  |

#### 10.1.8.2	Intra-frequency CSI-RSRQ accuracy requirements

##### 10.1.8.2.1	Absolute CSI-RSRQ Accuracy

Unless otherwise specified, the requirements for absolute accuracy of CSI-RSRQ in this clause apply to the intra-frequency measurement defined in 9.10.2.1 in FR2.

The accuracy requirements in Table 10.1.8.2.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled.

-	Conditions for intra-frequency measurements are fulfilled according to Annex B.2.2 for a corresponding Band for each relevant SSB.

-	Conditions for intra-frequency measurements are fulfilled according to Annex B.2.8 for a corresponding Band for each relevant CSI-RS.

-	The bandwidth of CSI-RS is 48 PRBs and the density is 3.

•	The performance with larger bandwidth of CSI-RS is equal to or better than the accuracy requirements in Table 10.1.8.2.1-1.

-	The timing offset between the reference measurement timing and the target CSI-RS in one layer is no larger than CP, where

the reference measurement timing for one layer for intra-frequency measurement is serving cell timing.

-	The measured signals are in the directions covered by the percentile EIS spherical coverage of the UE, defined in clause 7.3.4 of TS 38.101-2 [19].

Table 10.1.8.2.1-1: CSI-RSRQ Intra frequency absolute accuracy in FR2

| Accuracy |  | Conditions |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | CSI-RS Ês/Iot | Io Note 2 range |  |  |
|  |  |  | Minimum Io |  | Maximum Io |
| dB | dB | dB | dBm / SCSCSI-RS Note 1 |  | dBm/BWChannel |
|  |  |  | SCSCSI-RS = 60kHz | SCSCSI-RS = 120kHz |  |
| 2.5 | 4 | ≥-3 | Same value as CSI_RP in Table B.2.8-2, according to UE Power class, operating band and angle of arrival |  | -50 |
| 3.5 | 4 | ≥-6 |  |  |  |
| Note 1: Values based on Refsens and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. Note 2: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. Note 3: In the test cases, the CSI-RS Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. |  |  |  |  |  |

### 10.1.8B	Intra-frequency RSRQ accuracy requirements for FR2 for CA/DC Idle Mode Measurements

#### 10.1.8B.1	Intra-frequency SS-RSRQ accuracy requirements in FR2

The requirements in this clause are applicable for a UE:

-	in state RRC_IDLE or RRC INACTIVE

-	that is synchronised to the cell that is measured.

The requirements are for absolute accuracy of SS-RSRQ.

##### 10.1.8B.1.1	Absolute SS-RSRQ Accuracy in FR2

Unless otherwise specified, the requirements for absolute accuracy of SS-RSRQ in this clause apply to the serving cell in FR2.

The accuracy requirements in Table 10.1.8B.1.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled.

-	Conditions for intra-frequency measurements are fulfilled according to Annex B.1.2 for a corresponding Band for each relevant SSB.

-	The measured signals are in the directions covered by the percentile EIS spherical coverage of the UE, defined in clause 7.3.4 of TS 38.101-2 [19].

Table 10.1.8B.1.1-1: SS-RSRQ Intra frequency absolute accuracy in FR2

| Accuracy |  | Conditions |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 2 range |  |  |
|  |  |  | Minimum Io |  | Maximum Io |
| dB | dB | dB | dBm / SCSSSB Note 1 |  | dBm/BWChannel |
|  |  |  | SCSSSB = 120kHz | SCSSSB = 240kHz |  |
| 4 | 5.5 | ≥-3 | Same value as SSB_RP in Table B.2.2-2, according to UE Power class, operating band and angle of arrival |  | -50 |
| 5 | 5.5 | ≥-4 |  |  |  |
| Note 1: Values based on Refsens and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. Note 2: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. Note 3: In the test cases, the SSB Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. |  |  |  |  |  |

### 10.1.9	Inter-frequency RSRQ accuracy requirements for FR1

#### 10.1.9.1	Inter-frequency SS-RSRQ accuracy requirements in FR1

##### 10.1.9.1.1	Absolute Accuracy of SS-RSRQ in FR1

The requirements for absolute accuracy of SS-RSRQ in this clause apply to a cell on a frequency in FR1 that has different carrier frequency from the serving cell. The accuracy requirements in this clause are also applicable when highSpeedMeasInterFreq-r17 is configured.

The accuracy requirements in Table 10.1.9.1.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for inter-frequency measurements are fulfilled according to Annex B.2.3 for a corresponding Band for each relevant SSB.

Table 10.1.9.1.1-1: SS-RSRQ Inter frequency absolute accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 3 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
|  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | N/A | -50 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | N/A | -50 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | N/A | -50 |
| 2.5 | 4 | -3 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | N/A | -50 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | N/A | -50 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_N | -114.5 | -111.5 | N/A | -50 |
| 3.5 | 4 | -6 | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 3: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |

##### 10.1.9.1.2	Relative Accuracy of SS-RSRQ in FR1

The relative accuracy of SS-RSRQ in inter frequency case is defined as the RSRQ measured from one cell on a frequency in FR1 compared to the RSRP measured from another cell on a different frequency in FR1. The accuracy requirements in this clause are also applicable when highSpeedMeasInterFreq-r17 is configured.

The accuracy requirements in Table 10.1.9.1.2-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for inter-frequency measurements are fulfilled according to Annex B.2.3 for a corresponding Band for each relevant SSB.

-	|SSB_RP1d–m - SSB_RP2dBm| ≤ 27 dB

-	|Channel 1_Io -Channel 2_Io |  20 dB

Table 10.1.9.1.2-1: SS-RSRQ Inter frequency relative accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 1 range |  |  |  |  |
|  |  | Note 2 | NR operating band groups Note 4 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
|  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | N/A | -50 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | N/A | -50 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | N/A | -50 |
| 3 | 4 | -3 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | N/A | -50 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | N/A | -50 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_N | -114.5 | -111.5 | N/A | -50 |
| 4 | 4 | -6 | Note 3 | Note 3 | Note 3 | Note 3 | Note 3 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The parameter SSB Ês/Iot is the minimum SSB Ês/Iot of the pair of cells to which the requirement applies. NOTE 3: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 4: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |

#### 10.1.9.2	Inter-frequency CSI-RSRQ accuracy requirements

##### 10.1.9.2.1	Absolute CSI-RSRQ Accuracy

Unless otherwise specified, the requirements for absolute accuracy of CSI-RSRQ in this clause apply to the inter-frequency measurement defined in 9.10.3.1 in FR1.

The accuracy requirements in Table 10.1.9.2.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for inter-frequency measurements are fulfilled according to Annex B.2.3 for a corresponding Band for associated SSB.

-	Conditions for inter-frequency measurements are fulfilled according to Annex B.2.9 for a corresponding Band for each relevant CSI-RS.

-	The bandwidth of CSI-RS is 48 PRBs and the density is 3.

•	The performance with larger bandwidth of CSI-RS is equal to or better than the accuracy requirements in Table 10.1.9.2.1-1.

-	The timing offset between the reference measurement timing and the target CSI-RS in one layer is no larger than CP, where

the reference measurement timing for one layer for inter-frequency measurement is up to UE implementation and shall be based on the timing of one of the target cells.

Table 10.1.9.2.1-1: CSI-RSRQ Inter frequency absolute accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | CSI-RS Ês/Iot | Io Note 1 range |  |  |  |  |  |
|  |  |  | NR operating band groups Note 3 | Minimum Io |  |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSCSI-RS |  |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSCSI-RS = 15 kHz | SCSCSI-RS = 30 kHz | SCSCSI-RS = 60 kHz |  |  |
|  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | -115 | N/A | -50 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | -114.5 | N/A | -50 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | -114 | N/A | -50 |
| 2.5 | 4 | -3 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | -113.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | -113 | N/A | -50 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | -112.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | -112 | N/A | -50 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | -111.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_N | -114.5 | -111.5 | -108.5 | N/A | -50 |
| 3.5 | 4 | -6 | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 3: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |  |

##### 10.1.9.2.2	Relative CSI-RSRQ Accuracy

The relative accuracy of CSI-RSRQ is defined as the CSI-RSRQ measured from one cell compared to the CSI-RSRQ measured from another cell with the same center frequency, or between any two CSI-RSRQ levels measured on the same cell in FR1.

The accuracy requirements in Table 10.1.9.2.2-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for inter-frequency measurements are fulfilled according to Annex B.2.3 for a corresponding Band for the associated SSB.

-	Conditions for inter-frequency measurements are fulfilled according to Annex B.2.9 for a corresponding Band for each relevant CSI-RS.

-	The bandwidth of CSI-RS is 48 PRBs and the density is 3.

•	The performance with larger bandwidth of CSI-RS is equal to or better than the accuracy requirements in Table 10.1.9.2.2-1.

-	The timing offset between the reference measurement timing and the target CSI-RS in one layer is no larger than CP, where

the reference measurement timing for one layer for inter-frequency measurement is up to UE implementation and shall be based on the timing of one of the target cells.

Table 10.1.9.2.2-1: CSI-RSRQ Inter frequency relative accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | CSI-RS Ês/Iot | Io Note 1 range |  |  |  |  |  |
|  |  | Note 2 | NR operating band groups Note 4 | Minimum Io |  |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSCSI-RS |  |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSCSI-RS = 15 kHz | SCSCSI-RS = 30 kHz | SCSCSI-RS = 60 kHz |  |  |
|  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | -115 | N/A | -50 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | -114.5 | N/A | -50 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | -114 | N/A | -50 |
| 3 | 4 | -3 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | -113.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | -113 | N/A | -50 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | -112.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | -112 | N/A | -50 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | -111.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_N | -114.5 | -111.5 | -108.5 | N/A | -50 |
| 4 | 4 | -6 | Note 3 | Note 3 | Note 3 | Note 3 | Note 3 | Note 3 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The parameter CSI-RS Ês/Iot is the minimum CSI-RS Ês/Iot of the pair of cells to which the requirement applies. NOTE 3: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 4: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |  |

### 10.1.9B	Inter-frequency RSRQ accuracy requirements for FR1 for CA/DC Idle Mode Measurements

#### 10.1.9B.1	Inter-frequency SS-RSRQ accuracy requirements in FR1

The requirements in this clause are applicable for a UE:

-	in state RRC_IDLE or RRC INACTIVE

-	that is synchronised to the cell that is measured.

The requirements are for absolute accuracy of SS-RSRQ.

##### 10.1.9B.1.1	Absolute Accuracy of SS-RSRQ in FR1

The requirements for absolute accuracy of SS-RSRQ in this clause apply to a cell on a frequency in FR1 that has different carrier frequency from the serving cell.

The accuracy requirements in Table 10.1.9B.1.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for inter-frequency measurements are fulfilled according to Annex B.1.3 for a corresponding Band for each relevant SSB.

Table 10.1.9B.1.1-1: SS-RSRQ Inter frequency absolute accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 3 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
|  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | N/A | -50 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | N/A | -50 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | N/A | -50 |
| 4 | 5.5 | -3 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | N/A | -50 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | N/A | -50 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_N | -114.5 | -111.5 | N/A | -50 |
| 5 | 5.5 | -4 | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 3: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |

### 10.1.9C	Inter-frequency RSRQ accuracy requirements for FR1 SAN

#### 10.1.9C.1	Inter-frequency SS-RSRQ accuracy requirements in FR1

##### 10.1.9C.1.1	Absolute Accuracy of SS-RSRQ in FR1

The requirements for absolute accuracy of SS-RSRQ in this clause apply to a cell on a frequency in FR1 that has different carrier frequency from the serving cell.

The accuracy requirements in Table 10.1.9C.1.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for inter-frequency measurements are fulfilled according to Annex B.2.3 for a corresponding Band for each relevant SSB.

-	Valid information for the SAN serving the target cell has been provided.

Table 10.1.9C.1.1-1: SS-RSRQ Inter frequency absolute accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 3 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
| 2.5 | 4 | -3 | NR_FDD_SAB_FR1_A | -121 | -118 | N/A | -50 |
| 3.5 | 4 | -6 | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 3: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |

##### 10.1.9C.1.2	Relative Accuracy of SS-RSRQ in FR1

The relative accuracy of SS-RSRQ in inter frequency case is defined as the RSRQ measured from one cell on a frequency in FR1 compared to the RSRP measured from another cell on a different frequency in FR1.

The accuracy requirements in Table 10.1.9C.1.2-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for inter-frequency measurements are fulfilled according to Annex B.2.3 for a corresponding Band for each relevant SSB.

-	|SSB_RP1dBm - SSB_RP2dBm| ≤ 27 dB

-	|Channel 1_Io -Channel 2_Io |  20 dB

-	Valid information for the SAN serving the target cell has been provided.

Table 10.1.9C.1.2-1: SS-RSRQ Inter frequency relative accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 1 range |  |  |  |  |
|  |  | Note 2 | NR operating band groups Note 4 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
| 3 | 4 | -3 | NR_FDD_SAB_FR1_A | -121 | -118 | N/A | -50 |
| 4 | 4 | -6 | Note 3 | Note 3 | Note 3 | Note 3 | Note 3 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The parameter SSB Ês/Iot is the minimum SSB Ês/Iot of the pair of cells to which the requirement applies. NOTE 3: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 4: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |

### 10.1.10	Inter-frequency RSRQ accuracy requirements for FR2

#### 10.1.10.1	Inter-frequency SS-RSRQ accuracy requirements in FR2

##### 10.1.10.1.1	Absolute Accuracy of SS-RSRQ in FR2

The requirements for absolute accuracy of SS-RSRQ in this clause apply to a cell on a frequency in FR2 that has different carrier frequency from the serving cell.

The accuracy requirements in Table 10.1.10.1.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled.

-	Conditions for inter-frequency measurements are fulfilled according to Annex B.2.3 for a corresponding Band for each relevant SSB.

-	The measured signals are in the directions covered by the percentile EIS spherical coverage of the UE, defined in clause 7.3.4 of TS 38.101-2 [19].

Table 10.1.10.1.1-1: SS-RSRQ Inter frequency absolute accuracy in FR2

| Accuracy |  | Conditions |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 2 range |  |  |
|  |  |  | Minimum Io |  | Maximum Io |
| dB | dB | dB | dBm / SCSSSB Note 1 |  | dBm/BWChannel |
|  |  |  | SCSSSB = 120kHz | SCSSSB = 240kHz |  |
| 2.5 | 4 | ≥-3 | Same value as SSB_RP in Table B.2.2-2, according to UE Power class, operating band and angle of arrival |  | -50 |
| 3.5 | 4 | ≥-4 |  |  |  |
| Note 1: Values based on Refsens and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. Note 2: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. Note 3: In the test cases, the SSB Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. |  |  |  |  |  |

##### 10.1.10.1.2	Relative Accuracy of SS-RSRQ in FR2

The relative accuracy of SS-RSRQ in inter frequency case is defined as the RSRQ measured from one cell on a frequency in FR2 compared to the RSRP measured from another cell on a different frequency in FR2.

The accuracy requirements in Table 10.1.10.1.2-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled.

-	Conditions for inter-frequency measurements are fulfilled according to Annex B.2.3 for a corresponding Band for each relevant SSB.

-	|SSB_RP1dBm - SSB_RP2dBm|  27 dB

-	| Channel 1_Io -Channel 2_Io |  20 dB

-	The measured signals are in the directions covered by the percentile EIS spherical coverage of the UE, defined in clause 7.3.4 of TS 38.101-2 [19].

Table 10.1.10.1.2-1: SS-RSRQ Inter frequency relative accuracy in FR2

| Accuracy |  | Conditions |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 2 range |  |  |
|  |  |  | Minimum Io |  | Maximum Io |
| dB | dB | dB | dBm / SCSSSB Note 1 |  | dBm/BWChannel |
|  |  |  | SCSSSB = 120kHz | SCSSSB = 240kHz |  |
| 3 | 4 | ≥-3 | Same value as SSB_RP in Table B.2.2-2, according to UE Power class, operating band and angle of arrival |  | -50 |
| 4 | 4 | ≥-4 |  |  |  |
| Note 1: Values based on Refsens and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. Note 2: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. Note 3: The parameter SSB Ês/Iot is the minimum SSB Ês/Iot of the pair of cells to which the requirement applies. Note 4: In the test cases, the SSB Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. |  |  |  |  |  |

#### 10.1.10.2	Inter-frequency CSI-RSRQ accuracy requirements

##### 10.1.10.2.1	Absolute CSI-RSRQ Accuracy

Unless otherwise specified, the requirements for absolute accuracy of CSI-RSRQ in this clause apply the inter-frequency measurement defined in 9.10.3.1 in FR2.

The accuracy requirements in Table 10.1.10.2.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled.

-	Conditions for inter-frequency measurements are fulfilled according to Annex B.2.3 for a corresponding Band for associated SSB.

-	Conditions for inter-frequency measurements are fulfilled according to Annex B.2.9 for a corresponding Band for each relevant CSI-RS.

-	The bandwidth of CSI-RS is 48 PRBs and the density is 3.

•	The performance with larger bandwidth of CSI-RS is equal to or better than the accuracy requirements in Table 10.1.10.2.1-1.

-	The timing offset between the reference measurement timing and the target CSI-RS in one layer is no larger than CP, where

the reference measurement timing for one layer for inter-frequency measurement is up to UE implementation and shall be based on the timing of one of the target cells.

Table 10.1.10.2.1-1: CSI-RSRQ Inter frequency absolute accuracy in FR2

| Accuracy |  | Conditions |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | CSI-RS Ês/Iot | Io Note 2 range |  |  |
|  |  |  | Minimum Io |  | Maximum Io |
| dB | dB | dB | dBm / SCSCSI-RS Note 1 |  | dBm/BWChannel |
|  |  |  | SCSCSI-RS = 60kHz | SCSCSI-RS = 120kHz |  |
| 2.5 | 4 | ≥-3 | Same value as CSI_RP in Table B.2.9-2, according to UE Power class, operating band and angle of arrival |  | -50 |
| 3.5 | 4 | ≥-4 |  |  |  |
| Note 1: Values based on Refsens and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. Note 2: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. Note 3: In the test cases, the CSI-RS Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. |  |  |  |  |  |

##### 10.1.10.2.2	Relative CSI-RSRQ Accuracy

The relative accuracy of CSI-RSRQ is defined as the CSI-RSRQ measured from one cell compared to the CSI-RSRQ measured from another cell with the same center frequency, or between any two CSI-RSRQ levels measured on the same cell in FR2.

The accuracy requirements in Table 10.1.10.2.2-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled.

-	Conditions for inter-frequency measurements are fulfilled according to Annex B.2.3 for a corresponding Band for the associated SSB.

-	Conditions for inter-frequency measurements are fulfilled according to Annex B.2.9 for a corresponding Band for each relevant CSI-RS.

-	The bandwidth of CSI-RS is 48 PRBs and the density is 3.

•	The performance with larger bandwidth of CSI-RS is equal to or better than the accuracy requirements in Table 10.1.10.2.2-1.

-	The timing offset between the reference measurement timing and the target CSI-RS in one layer is no larger than CP, where

the reference measurement timing for one layer for inter-frequency measurement is up to UE implementation and shall be based on the timing of one of the target cells.

Table 10.1.10.2.2-1: CSI-RSRQ Inter frequency relative accuracy in FR2

| Accuracy |  | Conditions |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | CSI-RS Ês/Iot | Io Note 2 range |  |  |
|  |  |  | Minimum Io |  | Maximum Io |
| dB | dB | dB | dBm / SCSCSI-RS Note 1 |  | dBm/BWChannel |
|  |  |  | SCSCSI-RS = 60kHz | SCSCSI-RS = 120kHz |  |
| 3 | 4 | ≥-3 | Same value as CSI_RP in Table B.2.9-2, according to UE Power class, operating band and angle of arrival |  | -50 |
| 4 | 4 | ≥-4 |  |  |  |
| Note 1: Values based on Refsens and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. Note 2: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. Note 3: The parameter CSI-RS Ês/Iot is the minimum CSI-RS Ês/Iot of the pair of cells to which the requirement applies. Note 4: In the test cases, the CSI-RS Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. |  |  |  |  |  |

### 10.1.10B		Inter-frequency RSRQ accuracy requirements for FR2 for CA/DC Idle Mode Measurements

#### 10.1.10B.1	Inter-frequency SS-RSRQ accuracy requirements in FR2

The requirements in this clause are applicable for a UE:

-	in state RRC_IDLE or RRC INACTIVE

-	that is synchronised to the cell that is measured.

The requirements are for absolute accuracy of SS-RSRQ.

##### 10.1.10B.1.1	Absolute Accuracy of SS-RSRQ in FR2

The requirements for absolute accuracy of SS-RSRQ in this clause apply to a cell on a frequency in FR2 that has different carrier frequency from the serving cell.

The accuracy requirements in Table 10.1.10B.1.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled.

-	Conditions for inter-frequency measurements are fulfilled according to Annex B.1.3 for a corresponding Band for each relevant SSB.

-	The measured signals are in the directions covered by the percentile EIS spherical coverage of the UE, defined in clause 7.3.4 of TS 38.101-2 [19].

Table 10.1.10B.1.1-1: SS-RSRQ Inter frequency absolute accuracy in FR2

| Accuracy |  | Conditions |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 2 range |  |  |
|  |  |  | Minimum Io |  | Maximum Io |
| dB | dB | dB | dBm / SCSSSB Note 1 |  | dBm/BWChannel |
|  |  |  | SCSSSB = 120kHz | SCSSSB = 240kHz |  |
| 4 | 5.5 | ≥-3 | Same value as SSB_RP in Table B.2.2-2, according to UE Power class, operating band and angle of arrival |  | -50 |
| 5 | 5.5 | ≥-4 |  |  |  |
| Note 1: Values based on Refsens and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. Note 2: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. Note 3: In the test cases, the SSB Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. |  |  |  |  |  |

### 10.1.11	RSRQ report mapping

#### 10.1.11.1	SS-RSRQ and CSI-RSRQ measurement report mapping

The reporting range of SS-RSRQ and CSI-RSRQ measurement is defined from -43 dB to 20 dB with 0.5 dB resolution. The mapping of measured quantity is defined in Table 10.1.11.1-1. The range in the signalling may be larger than the guaranteed accuracy range.

Table 10.1.11.1-1: SS-RSRQ and CSI-RSRQ measurement report mapping

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| SS-RSRQ_0 | SS-RSRQ<-43 | dB |
| SS-RSRQ_1 | -43≤ SS-RSRQ<-42.5 | dB |
| SS-RSRQ_2 | -42.5≤ SS-RSRQ<-42 | dB |
| SS-RSRQ_3 | -42≤ SS-RSRQ<-41.5 | dB |
| SS-RSRQ_4 | -41.5≤ SS-RSRQ<-41 | dB |
| .. | .. | … |
| SS-RSRQ_122 | 17.5≤ SS-RSRQ<18 | dB |
| SS-RSRQ_123 | 18≤ SS-RSRQ<18.5 | dB |
| SS-RSRQ_124 | 18.5≤ SS-RSRQ<19 | dB |
| SS-RSRQ_125 | 19≤ SS-RSRQ<19.5 | dB |
| SS-RSRQ_126 | 19.5≤ SS-RSRQ<20 | dB |
| SS-RSRQ_127 | 20 ≤ SS-RSRQ | dB |

### 10.1.12	Intra-frequency SINR accuracy requirements for FR1

#### 10.1.12.1	Intra-frequency SS-SINR accuracy requirements in FR1

##### 10.1.12.1.1	Absolute SS-SINR Accuracy in FR1

Unless otherwise specified, the requirements for absolute accuracy of SS-SINR in this clause apply to a cell on the same frequency as that of the serving cell in FR1.

The accuracy requirements in Table 10.1.12.1.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for intra-frequency measurements are fulfilled according to Annex B.2.2 for a corresponding Band.

Table 10.1.12.1.1-1: SS-SINR Intra frequency absolute accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 1 range |  |  |  |  |
|  |  | Note 3 | NR operating band groups Note 4 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
|  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | N/A | -50 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | N/A | -50 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | N/A | -50 |
| 3.0 | 4 | -3 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | N/A | -50 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | N/A | -50 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_N | -114.5 | -111.5 | N/A | -50 |
| 3.5 | 4 | -6 | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 3: The requirements apply for SSB Ês/Iot ≤ 25 dB under non-HST scenarios. NOTE 4: NR operating band groups in FR1 are as defined in clause 3.5.2. NOTE 5: The requirements apply for SSB Ês/Iot ≤5 dB with SCS 15kHz or 30kHz under NR high speed scenarios. |  |  |  |  |  |  |  |

#### 10.1.12.2	Intra-frequency CSI-SINR accuracy requirements in FR1

##### 10.1.12.2.1	Absolute CSI-SINR Accuracy in FR1

Unless otherwise specified, the requirements for absolute accuracy of CSI-SINR in this clause apply to a cell on the same frequency as that of the serving cell in FR1.

The accuracy requirements in Table 10.1.12.2.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for intra-frequency measurements are fulfilled according to Annex B.2.8 for a corresponding Band.

-	The timing offset between the reference measurement timing and the target CSI-RS in one layer is no larger than CP, where

the reference measurement timing for intra-frequency measurement is serving cell timing.

-	The bandwidth of CSI-RS is 48 PRBs and the density is 3.

•	The performance with larger bandwidth of CSI-RS is equal to or better than the accuracy requirements in Table 10.1.12.2.1-1.

Table 10.1.12.2.1-1: CSI-SINR Intra frequency absolute accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | CSI-RS Ês/Iot | Io Note 1 range |  |  |  |  |  |
|  |  |  | NR operating band groups | Minimum Io |  |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCS |  |  | dBm/BW Channel | dBm/BW Channel |
|  |  |  |  | SCS (kHz) |  |  |  |  |
| 3 | 4 | -3 |  | 15 | 30 | 60 |  |  |
|  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | -115 | N/A | -70 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | -114.5 | N/A | -70 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | -114 | N/A | -70 |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | -113.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | -113 | N/A | -70 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | -112.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | -112 | N/A | -70 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | -111.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_N | -114.5 | -111.5 | -108.5 | N/A | -70 |
| 3.5 | 4 | -6 | Note 2 | Note 2 | Note 2 | Note 2 | N/A | Note 2 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 3: NR operating band groups in FR1 are as defined in clause 3.5.2. NOTE 4: The requirements apply for CSI-RS Ês/Iot ≤XdB. X=15 if timing offset between the reference measurement timing and the target CSI-RS is no larger than 0.5*CP, and X=4 if timing offset between the reference measurement timing and the target CSI-RS is larger than 0.5*CP but no larger than CP. |  |  |  |  |  |  |  |  |

### 10.1.12C		Intra-frequency SINR accuracy requirements for FR1 SAN

#### 10.1.12C.1	Intra-frequency SS-SINR accuracy requirements in FR1

##### 10.1.12C.1.1	Absolute SS-SINR Accuracy in FR1

Unless otherwise specified, the requirements for absolute accuracy of SS-SINR in this clause apply to a cell on the same frequency as that of the serving cell in FR1.

The accuracy requirements in Table 10.1.12C.1.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for intra-frequency measurements are fulfilled according to Annex B.2.2 for a corresponding Band.

-	Valid information for the SAN serving the target cell has been provided.

Table 10.1.12C.1.1-1: SS-SINR Intra frequency absolute accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 1 range |  |  |  |  |
|  |  | Note 3 | NR operating band groups Note 4 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
| 3 | 4 | -3 | NR_FDD_SAB_FR1_A | -121 | -118 | N/A | -50 |
| 3.5 | 4 | -6 | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 3: The requirements apply for SSB Ês/Iot ≤ 25 dB under non-HST scenarios. NOTE 4: NR operating band groups in FR1 are as defined in clause 3.5.2. NOTE 5: The requirements apply for SSB Ês/Iot ≤5 dB with SCS 15kHz or 30kHz under NR high speed scenarios. |  |  |  |  |  |  |  |

### 10.1.13	Intra-frequency SINR accuracy requirements for FR2

#### 10.1.13.1	Intra-frequency SS-SINR accuracy requirements in FR2

##### 10.1.13.1.1	Absolute SS-SINR Accuracy in FR2

Unless otherwise specified, the requirements for absolute accuracy of SS-SINR in this clause apply to a cell on the same frequency as that of the serving cell in FR2.

The accuracy requirements in Table 10.1.13.1.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled.

-	Conditions for intra-frequency measurements are fulfilled according to Annex B.2.2 for a corresponding Band.

-	The measured signals are in the directions covered by the percentile EIS spherical coverage of the UE, defined in clause 7.3.4 of TS 38.101-2 [19].

Table 10.1.13.1.1-1: SS-SINR Intra frequency absolute accuracy in FR2

| Accuracy |  | Conditions |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 2 range |  |  |
|  |  |  | Minimum Io |  | Maximum Io |
| dB | dB | dB | dBm / SCSSSB Note 1 |  | dBm/BWChannel |
|  |  |  | SCSSSB = 120kHz | SCSSSB = 240kHz |  |
| 3 | 4 | ≥-3 | Same value as SSB_RP in Table B.2.2-2, according to UE Power class, operating band and angle of arrival |  | -50 |
| 3.5 | 4 | ≥-6 |  |  |  |
| Note 1: Values based on Refsens and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. Note 2: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. Note 3: In the test cases, the SSB Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. Note 4: The requirements apply for SSB Ês/Iot ≤ 25 dB. |  |  |  |  |  |

#### 10.1.13.2	Intra-frequency CSI-SINR accuracy requirements in FR2

##### 10.1.13.2.1	Absolute CSI-SINR Accuracy in FR2

Unless otherwise specified, the requirements for absolute accuracy of CSI-SINR in this clause apply to a cell on the same frequency as that of the serving cell in FR2.

The accuracy requirements in Table 10.1.13.2.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled.

-	Conditions for inter-frequency measurements are fulfilled according to Annex B.2.8 for a corresponding Band.

-	The measured signals are in the directions covered by the percentile EIS spherical coverage of the UE, defined in clause 7.3.4 of TS 38.101-2 [19].

-	The timing offset between the reference measurement timing and the target CSI-RS in one layer is no larger than CP, where

the reference measurement timing for intra-frequency measurement is serving cell timing.

-	The bandwidth of CSI-RS is 48 PRBs and the density is 3.

•	The performance with larger bandwidth of CSI-RS is equal to or better than the accuracy requirements in Table 10.1.13.2.1-1.

Table 10.1.13.2.1-1: CSI-SINR Intra frequency absolute accuracy in FR2

| Accuracy |  | Conditions |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | CSI-RS Ês/Iot | Io Note 2 range |  |  |
|  |  |  | Minimum Io |  | Maximum Io |
| dB | dB | dB | dBm / SCSCSI-RS Note 1 |  | dBm/BWChannel |
|  |  |  | SCSCSI-RS = 60kHz | SCSCSI-RS = 120kHz |  |
| 3 | 4 | ≥-3 | Same value as CSI_RP in Table B.2.8-2, according to UE Power class, operating band and angle of arrival |  | -50 |
| 3.5 | 4 | ≥-6 |  |  |  |
| Note 1: Values based on Refsens and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. Note 2: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. Note 3: In the test cases, the CSI-RS Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. Note 4: The requirements apply for CSI-RS Ês/Iot ≤XdB. X=15 if timing offset between the reference measurement timing and the target CSI-RS is no larger than 0.5*CP, and X=4 if timing offset between the reference measurement timing and the target CSI-RS is larger than 0.5*CP but no larger than CP. |  |  |  |  |  |

### 10.1.14	Inter-frequency SINR accuracy requirements for FR1

#### 10.1.14.1	Inter-frequency SS-SINR accuracy requirements in FR1

##### 10.1.14.1.1	Aboslute Accuracy of SS-SINR in FR1

The requirements for absolute accuracy of SS-SINR in this clause apply to a cell on a frequency in FR1 that has different carrier frequency from the serving cell.

The accuracy requirements in Table 10.1.14.1.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for inter-frequency measurements are fulfilled according to Annex B.2.3 for a corresponding Band.

Table 10.1.14.1.1-1: SS-SINR Inter frequency absolute accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot Note 3 | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 4 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
|  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | N/A | -50 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | N/A | -50 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | N/A | -50 |
| 3.0 | 4 | -3 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | N/A | -50 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | N/A | -50 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_N | -114.5 | -111.5 | N/A | -50 |
| 3.5 | 4 | -6 | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 3: The requirements apply for SSB Ês/Iot ≤ 25 dB. NOTE 4: NR operating band groups in FR1 are as defined in clause 3.5.2. NOTE 5: The requirements apply for SSB Ês/Iot ≤ 5 dB with SCS 15kHz or 30kHz under NR high speed scenarios. |  |  |  |  |  |  |  |

##### 10.1.14.1.2	Relative Accuracy of SS-SINR in FR1

The relative accuracy of SS-SINR in inter frequency case is defined as the SS-SINR measured from one cell on a frequency in FR1 compared to the SS-SINR measured from another cell on a different frequency in FR1.

The accuracy requirements in Table 10.1.14.1.2-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for inter-frequency measurements are fulfilled according to Annex B.2.3 for a corresponding Band.

-	|SSB_RP1dBm - SSB_RP2dBm| ≤ 27 dB

-	| Channel 1_Io -Channel 2_Io |  20 dB

Table 10.1.14.1.2-1: SS-SINR Inter frequency relative accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 1 range |  |  |  |  |
|  |  | Note 2,4 | NR operating band groups Note 5 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 120 kHz | SCSSSB = 240 kHz |  |  |
|  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | N/A | -50 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | N/A | -50 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | N/A | -50 |
| 3.5 | 4 | -3 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | N/A | -50 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | N/A | -50 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_N | -114.5 | -111.5 | N/A | -50 |
| 4 | 4 | -6 | Note 3 | Note 3 | Note 3 | Note 3 | Note 3 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The parameter SSB Ês/Iot is the minimum SSB Ês/Iot of the pair of cells to which the requirement applies. NOTE 3: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 4: The requirements apply for SSB Ês/Iot ≤ 25 dB. NOTE 5: NR operating band groups in FR1 are as defined in clause 3.5.2. NOTE 6: The requirements apply for SSB Ês/Iot ≤ 5 dB with SCS 15kHz or 30kHz under NR high speed scenarios. |  |  |  |  |  |  |  |

#### 10.1.14.2	Inter-frequency CSI-SINR accuracy requirements in FR1

##### 10.1.14.2.1	Aboslute Accuracy of CSI-SINR in FR1

The requirements for absolute accuracy of CSI-SINR in this clause apply to a cell on a frequency in FR1 that has different carrier frequency from the serving cell.

The accuracy requirements in Table 10.1.14.2.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for inter-frequency measurements are fulfilled according to Annex B.2.9 for a corresponding Band.

-	The timing offset between the reference measurement timing and the target CSI-RS in one layer is no larger than CP, where

the reference measurement timing for inter-frequency measurement is up to UE implementation and shall be based on the timing of one of the target cells.

-	The bandwidth of CSI-RS is 48 PRBs and the density is 3.

•	The performance with larger bandwidth of CSI-RS is equal to or better than the accuracy requirements in Table 10.1.14.2.1-1.

Table 10.1.14.2.1-1: CSI-SINR Inter frequency absolute accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | CSI-RS Ês/Iot | Io Note 1 range |  |  |  |  |  |
|  |  |  | NR operating band groups | Minimum Io |  |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCS |  |  | dBm/BW Channel | dBm/BW Channel |
|  |  |  |  | SCS (kHz) |  |  |  |  |
| 3 | 4 | -3 |  | 15 | 30 | 60 |  |  |
|  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | -115 | N/A | -70 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | -114.5 | N/A | -70 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | -114 | N/A | -70 |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | -113.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | -113 | N/A | -70 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | -112.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | -112 | N/A | -70 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | -111.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_N | -114.5 | -111.5 | -108.5 | N/A | -70 |
| 3.5 | 4 | -6 | Note 2 | Note 2 | Note 2 | Note 2 | N/A | Note 2 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 3: NR operating band groups in FR1 are as defined in clause 3.5.2. NOTE 4: The requirements apply for CSI-RS Ês/Iot ≤XdB. X=15 if timing offset between the reference measurement timing and the target CSI-RS is no larger than 0.5*CP, and X=4 if timing offset between the reference measurement timing and the target CSI-RS is larger than 0.5*CP but no larger than CP. |  |  |  |  |  |  |  |  |

##### 10.1.14.2.2	Relative Accuracy of CSI-SINR in FR1

The relative accuracy of CSI-SINR in inter frequency case is defined as the CSI-SINR measured from one cell on a frequency in FR1 compared to the CSI-SINR measured from another cell on a different frequency in FR1.

The accuracy requirements in Table 10.1.14.2.2-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for inter-frequency measurements are fulfilled according to Annex B.2.9 for a corresponding Band.

-	|CSI_RP1dBm - CSI_RP2dBm| ≤ 27 dB

-	| Channel 1_Io -Channel 2_Io |  20 dB

-	The timing offset between the reference measurement timing and the target CSI-RS in one layer is no larger than CP, where

the reference measurement timing for inter-frequency measurement is up to UE implementation and shall be based on the timing of one of the target cells.

-	The bandwidth of CSI-RS is 48 PRBs and the density is 3.

•	The performance with larger bandwidth of CSI-RS is equal to or better than the accuracy requirements in Table 10.1.14.2.2-1.

Table 10.1.14.2.2-1: CSI-SINR Inter frequency relative accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | CSI-RS Ês/Iot Note 3 | Io Note 1 range |  |  |  |  |  |
|  |  |  | NR operating band groups | Minimum Io |  |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCS |  |  | dBm/BW Channel | dBm/BW Channel |
|  |  |  |  | SCS (kHz) |  |  |  |  |
| 3.5 | 4 | -3 |  | 15 | 30 | 60 |  |  |
|  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | -115 | N/A | -70 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | -114.5 | N/A | -70 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | -114 | N/A | -70 |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | -113.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | -113 | N/A | -70 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | -112.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | -112 | N/A | -70 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | -111.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_N | -114.5 | -111.5 | -108.5 | N/A | -70 |
| 4 | 4 | -6 | Note 2 | Note 2 | Note 2 | Note 2 | N/A | Note 2 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 3: The parameter CSI-RS Ês/Iot is the minimum CSI-RS Ês/Iot of the pair of cells to which the requirement applies. NOTE 4: NR operating band groups in FR1 are as defined in clause 3.5.2. NOTE 5: The requirements apply for CSI-RS Ês/Iot ≤ XdB. X=15 if timing offset between the reference measurement timing and the target CSI-RS is no larger than 0.5*CP, and X=4 if timing offset between the reference measurement timing and the target CSI-RS is larger than 0.5*CP but no larger than CP. |  |  |  |  |  |  |  |  |

### 10.1.14C		Inter-frequency SINR accuracy requirements for FR1 SAN

#### 10.1.14C.1	Inter-frequency SS-SINR accuracy requirements in FR1

##### 10.1.14C.1.1	Aboslute Accuracy of SS-SINR in FR1

The requirements for absolute accuracy of SS-SINR in this clause apply to a cell on a frequency in FR1 that has different carrier frequency from the serving cell.

The accuracy requirements in Table 10.1.14C.1.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for inter-frequency measurements are fulfilled according to Annex B.2.3 for a corresponding Band.

-	Valid information for the SAN serving the target cell has been provided.

Table 10.1.14C.1.1-1: SS-SINR Inter frequency absolute accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot Note 3 | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 4 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
| 3 | 4 | -3 | NR_FDD_SAB_FR1_A | -121 | -118 | N/A | -50 |
| 3.5 | 4 | -6 | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 3: The requirements apply for SSB Ês/Iot ≤ 25 dB. NOTE 4: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |

##### 10.1.14C.1.2	Relative Accuracy of SS-SINR in FR1

The relative accuracy of SS-SINR in inter frequency case is defined as the SS-SINR measured from one cell on a frequency in FR1 compared to the SS-SINR measured from another cell on a different frequency in FR1.

The accuracy requirements in Table 10.1.14C.1.2-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for inter-frequency measurements are fulfilled according to Annex B.2.3 for a corresponding Band.

-	|SSB_RP1dBm - SSB_RP2dBm| ≤ 27 dB

-	| Channel 1_Io -Channel 2_Io |  20 dB

-	Valid information for the SAN serving the target cell has been provided.

Table 10.1.14C.1.2-1: SS-SINR Inter frequency relative accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 1 range |  |  |  |  |
|  |  | Note 2,4 | NR operating band groups Note 5 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 120 kHz | SCSSSB = 240 kHz |  |  |
| 3.5 | 4 | -3 | NR_FDD_SAB_FR1_A | -121 | -118 | N/A | -50 |
| 4 | 4 | -6 | Note 3 | Note 3 | Note 3 | Note 3 | Note 3 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The parameter SSB Ês/Iot is the minimum SSB Ês/Iot of the pair of cells to which the requirement applies. NOTE 3: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 4: The requirements apply for SSB Ês/Iot ≤ 25 dB. NOTE 5: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |

### 10.1.15	Inter-frequency SINR accuracy requirements for FR2

#### 10.1.15.1	Inter-frequency SS-SINR accuracy requirements in FR2

##### 10.1.15.1.1	Aboslute Accuracy of SS-SINR in FR2

The requirements for absolute accuracy of SS-SINR in this clause apply to a cell on a frequency in FR2 that has different carrier frequency from the serving cell.

The accuracy requirements in Table 10.1.15.1.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled.

-	Conditions for inter-frequency measurements are fulfilled according to Annex B.2.3 for a corresponding Band.

-	The measured signals are in the directions covered by the percentile EIS spherical coverage of the UE, defined in clause 7.3.4 of TS 38.101-2 [19].

Table 10.1.15.1.1-1: SS-SINR Inter frequency absolute accuracy in FR2

| Accuracy |  | Conditions |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 2 range |  |  |
|  |  |  | Minimum Io |  | Maximum Io |
| dB | dB | dB | dBm / SCSSSB Note 1 |  | dBm/BWChannel |
|  |  |  | SCSSSB = 120kHz | SCSSSB = 240kHz |  |
| 3 | 4 | ≥-3 | Same value as SSB_RP in Table B.2.2-2, according to UE Power class, operating band and angle of arrival |  | -50 |
| 3.5 | 4 | ≥-4 |  |  |  |
| Note 1: Values based on Refsens and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. Note 2: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. Note 3: In the test cases, the SSB Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. Note 4: The requirements apply for SSB Ês/Iot ≤ 25 dB. |  |  |  |  |  |

##### 10.1.15.1.2	Relative Accuracy of SS-SINR in FR2

The relative accuracy of SS-SINR in inter frequency case is defined as the SS-SINR measured from one cell on a frequency in FR2 compared to the SS-SINR measured from another cell on a different frequency in FR2.

The accuracy requirements in Table 10.1.15.1.2-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled.

-	Conditions for inter-frequency measurements are fulfilled according to Annex B.2.3 for a corresponding Band.

-	|SSB_RP1dBm - SSB_RP2dBm|  27 dB

-	| Channel 1_Io -Channel 2_Io |  20 dB

-	The measured signals are in the directions covered by the percentile EIS spherical coverage of the UE, defined in clause 7.3.4 of TS 38.101-2 [19].

Table 10.1.15.1.2-1: SS-SINR Inter frequency relative accuracy in FR2

| Accuracy |  | Conditions |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 2 range |  |  |
|  |  |  | Minimum Io |  | Maximum Io |
| dB | dB | dB | dBm / SCSSSB Note 1 |  | dBm/BWChannel |
|  |  |  | SCSSSB = 120kHz | SCSSSB = 240kHz |  |
| 3.5 | 4 | ≥-3 | Same value as SSB_RP in Table B.2.2-2, according to UE Power class, operating band and angle of arrival |  | -50 |
| 4 | 4 | ≥-6 |  |  |  |
| Note 1: Values based on Refsens and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. Note 2: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. Note 3: The parameter SSB Ês/Iot is the minimum SSB Ês/Iot of the pair of cells to which the requirement applies. Note 4: In the test cases, the SSB Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. Note 5: The requirements apply for SSB Ês/Iot ≤ 25 dB. |  |  |  |  |  |

#### 10.1.15.2	Inter-frequency CSI-SINR accuracy requirements in FR2

##### 10.1.15.2.1	Aboslute Accuracy of CSI-SINR in FR2

The requirements for absolute accuracy of CSI-SINR in this clause apply to a cell on a frequency in FR2 that has different carrier frequency from the serving cell.

The accuracy requirements in Table 10.1.15.2.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled.

-	Conditions for inter-frequency measurements are fulfilled according to Annex B.2.9 for a corresponding Band.

-	The measured signals are in the directions covered by the percentile EIS spherical coverage of the UE, defined in clause 7.3.4 of TS 38.101-2 [19].

-	The timing offset between the reference measurement timing and the target CSI-RS in one layer is no larger than CP, where

the reference measurement timing for inter-frequency measurement is up to UE implementation and shall be based on the timing of one of the target cells.

-	The bandwidth of CSI-RS is 48 PRBs and the density is 3.

•	The performance with larger bandwidth of CSI-RS is equal to or better than the accuracy requirements in Table 10.1.15.2.1-1.

Table 10.1.15.2.1-1: CSI-SINR Inter frequency absolute accuracy in FR2

| Accuracy |  | Conditions |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | CSI-RS Ês/Iot | Io Note 2 range |  |  |
|  |  |  | Minimum Io |  | Maximum Io |
| dB | dB | dB | dBm / SCSCSI-RS Note 1 |  | dBm/BWChannel |
|  |  |  | SCSCSI-RS = 60kHz | SCSCSI-RS = 120kHz |  |
| 3 | 4 | ≥-3 | Same value as CSI_RP in Table B.2.9-2, according to UE Power class, operating band and angle of arrival |  | -50 |
| 3.5 | 4 | ≥-4 |  |  |  |
| Note 1: Values based on Refsens and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. Note 2: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. Note 3: In the test cases, the CSI-RS Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. Note 4: The requirements apply for CSI-RS Ês/Iot ≤XdB. X=15 if timing offset between the reference measurement timing and the target CSI-RS is no larger than 0.5*CP, and X=4 if timing offset between the reference measurement timing and the target CSI-RS is larger than 0.5*CP but no larger than CP. |  |  |  |  |  |

##### 10.1.15.2.2	Relative Accuracy of CSI-SINR in FR2

The relative accuracy of CSI-SINR in inter frequency case is defined as the CSI-SINR measured from one cell on a frequency in FR2 compared to the CSI-SINR measured from another cell on a different frequency in FR2.

The accuracy requirements in Table 10.1.15.2.2-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled.

-	Conditions for inter-frequency measurements are fulfilled according to Annex B.2.y for a corresponding Band.

-	|CSI_RP1dBm - CSI_RP2dBm|  27 dB

-	| Channel 1_Io -Channel 2_Io |  20 dB

-	The measured signals are in the directions covered by the percentile EIS spherical coverage of the UE, defined in clause 7.3.4 of TS 38.101-2 [19].

-	The timing offset between the reference measurement timing and the target CSI-RS in one layer is no larger than CP, where

the reference measurement timing for inter-frequency measurement is up to UE implementation and shall be based on the timing of one of the target cells.

-	The bandwidth of CSI-RS is 48 PRBs and the density is 3.

•	The performance with larger bandwidth of CSI-RS is equal to or better than the accuracy requirements in Table 10.1.15.2.2-1.

Table 10.1.15.2.2-1: CSI-SINR Inter frequency relative accuracy in FR2

| Accuracy |  | Conditions |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | CSI-RS Ês/Iot | Io Note 2 range |  |  |
|  |  |  | Minimum Io |  | Maximum Io |
| dB | dB | dB | dBm / SCSCSI-RS Note 1 |  | dBm/BWChannel |
|  |  |  | SCSCSI-RS = 60kHz | SCSCSI-RS = 120kHz |  |
| 3.5 | 4 | ≥-3 | Same value as CSI_RP in Table B.2.9-2, according to UE Power class, operating band and angle of arrival |  | -50 |
| 4 | 4 | ≥-6 |  |  |  |
| Note 1: Values based on Refsens and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. Note 2: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. Note 3: In the test cases, the CSI-RS Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. Note 4: The requirements apply for CSI-RS Ês/Iot ≤XdB. X=15 if timing offset between the reference measurement timing and the target CSI-RS is no larger than 0.5*CP, and X=4 if timing offset between the reference measurement timing and the target CSI-RS is larger than 0.5*CP but no larger than CP. |  |  |  |  |  |

### 10.1.16	SINR report mapping

#### 10.1.16.1	SS-SINR and CSI-SINR measurement report mapping

The reporting range of SS-SINR and CSI-SINR for L3 reporting and L1 reporing is defined from -23 dB to 40 dB with 0.5 dB resolution. The mapping of measured quantity is defined in Table 10.1.16.1-1. The range in the signalling may be larger than the guaranteed accuracy range.

The reporting range of differential SS-SINR and CSI-SINR for L1 reporting and L3 reporting is defined from -15 dB to 0 dB with 1 dB resolution.

The mapping of measured quantity is defined in Table 10.1.16.1-2. The range in the signalling may be larger than the guaranteed accuracy range.

Table 10.1.16.1-1: SS-SINR and CSI-SINR measurement report mapping

| Reported value | Measured quantity value (L3 SS-SINR and L3 CSI-SINR) | Measured quantity value (L1 SS-SINR and L1 CSI-SINR) | Unit |
| --- | --- | --- | --- |
| SINR_0 | SINR<-23 | SINR<-23 | dB |
| SINR_1 | -23≤ SINR<-22.5 | -23≤SINR<-22.5 | dB |
| SINR_2 | -22.5≤ SINR<-22 | -22.5≤SINR<-22 | dB |
| SINR_3 | -22≤ SINR<-21.5 | -22≤SINR<-21.5 | dB |
| SINR_4 | -21.5≤ SINR<-21 | -21.5≤SINR<-21 | dB |
| .. | .. | .. | … |
| SINR_123 | 38≤ SINR<38.5 | 38≤SINR<38.5 | dB |
| SINR_124 | 38.5≤ SINR<39 | 38.5≤SINR<39 | dB |
| SINR_125 | 39≤ SINR<39.5 | 39≤SINR<39.5 | dB |
| SINR_126 | 39.5≤ SINR<40 | 39.5≤SINR<40 | dB |
| SINR_127 | 40≤ SINR | 40≤SINR | dB |

Table 10.1.16.1-2: Differential SS-SINR and CSI-SINR measurement (for L1 reporting and L3 reporting) report mapping

| Reported value | Measured quantity value (difference in measured SINR from largest SINR) | Unit |
| --- | --- | --- |
| DIFFSINR_0 | 0≥ΔSINR>-1 | dB |
| DIFFSINR_1 | -1≥ΔSINR>-2 | dB |
| DIFFSINR_2 | -2≥ΔSINR>-3 | dB |
| DIFFSINR_3 | -3≥ΔSINR>-4 | dB |
| DIFFSINR_4 | -4≥ΔSINR>-5 | dB |
| DIFFSINR_5 | -5≥ΔSINR>-6 | dB |
| DIFFSINR_6 | -6≥ΔSINR>-7 | dB |
| DIFFSINR_7 | -7≥ΔSINR>-8 | dB |
| DIFFSINR_8 | -8≥ΔSINR>-9 | dB |
| DIFFSINR_9 | -9≥ΔSINR>-10 | dB |
| DIFFSINR_10 | -10≥ΔSINR>-11 | dB |
| DIFFSINR_11 | -11≥ΔSINR>-12 | dB |
| DIFFSINR_12 | -12≥ΔSINR>-13 | dB |
| DIFFSINR_13 | -13≥ΔSINR>-14 | dB |
| DIFFSINR_14 | -14≥ΔSINR>-15 | dB |
| DIFFSINR_15 | -15≥ΔSINR | dB |

### 10.1.17	Power Headroom

#### 10.1.17.1	Power Headroom Report

##### 10.1.17.1.1	Power Headroom Report Mapping

The power headroom reporting range is from -32 ...+38 dB. Table 10.1.17.1-1 defines the report mapping.

Table 10.1.17.1-1: Power headroom report mapping

| Reported value | Measured quantity value (dB) |
| --- | --- |
| POWER_HEADROOM_0 | PH -32 |
| POWER_HEADROOM_1 | -32 PH -31 |
| POWER_HEADROOM_2 | -31 PH -30 |
| POWER_HEADROOM_3 | -30 PH -29 |
| POWER_HEADROOM_53 | 20  PH  21 |
| POWER_HEADROOM_54 | 21  PH  22 |
| POWER_HEADROOM_55 | 22  PH  24 |
| POWER_HEADROOM_56 | 24  PH  26 |
| POWER_HEADROOM_57 | 26  PH  28 |
| POWER_HEADROOM_58 | 28  PH  30 |
| POWER_HEADROOM_59 | 30  PH  32 |
| POWER_HEADROOM_60 | 32  PH  34 |
| POWER_HEADROOM_61 | 34  PH  36 |
| POWER_HEADROOM_62 | 36  PH  38 |
| POWER_HEADROOM_63 | PH ≥ 38 |

### 10.1.18	PCMAX,c,f

The UE is required to report the UE configured maximum output  power (PCMAX,c,f) together with the power headroom. This clause defines the requirements for the PCMAX,c,f reporting.

#### 10.1.18.1	Report Mapping

The PCMAX,c,f reporting range is defined from -29 dBm to 33 dBm with 1 dB resolution. Table 10.1.18.1-1 defines the reporting mapping.

Table 10.1.18.1-1 Mapping of PCMAX,c.f

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| PCMAX_C_00 | PCMAX,c,f < -29 | dBm |
| PCMAX_C_01 | -29 PCMAX,c,f < -28 | dBm |
| PCMAX_C_02 | -28 PCMAX,c,f < -27 | dBm |
| … | … | … |
| PCMAX_C_61 | 31 PCMAX,c,f < 32 | dBm |
| PCMAX_C_62 | 32 PCMAX,c,f < 33 | dBm |
| PCMAX_C_63 | 33 PCMAX,c,f | dBm |

### 10.1.19	L1-RSRP accuracy requirements for FR1

#### 10.1.19.1	SSB based L1-RSRP accuracy requirements

Unless otherwise specified, the requirements for absolute accuracy and relative accuracy of SSB based L1-RSRP in this clause apply to all SSBs of the serving cell configured for L1-RSRP measurement and all SSBs of cell(s) with different PCI from serving cell configured for L1-RSRP measurement in FR1.

##### 10.1.19.1.1	Absolute Accuracy

The accuracy requirements in Table 10.1.19.1.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for L1-RSRP measurements are fulfilled according to Annex B.2.4.1 for a corresponding Band for each relevant SSB.

Table 10.1.19.1.1-1: SSB based L1-RSRP absolute accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 2 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
|  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | N/A | -70 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | N/A | -70 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | N/A | -70 |
| ±5.0 | ±9.5 | -3 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | N/A | -70 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | N/A | -70 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_N | -114.5 | -111.5 | N/A | -70 |
| ±8.5 | ±11.5 | -3 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A, NR_FDD_FR1_B, NR_TDD_FR1_C, NR_FDD_FR1_D, NR_TDD_FR1_D, NR_FDD_FR1_E, NR_TDD_FR1_E, NR_FDD_FR1_F, NR_FDD_FR1_G, NR_FDD_FR1_H, NR_FDD_FR1_N | N/A | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |

##### 10.1.19.1.2	Relative Accuracy

The relative accuracy of SSB based L1-RSRP is defined as the L1-RSRP measured from one SSB compared to the largest measured value of L1-RSRP among all SSBs of the cell (serving cell or cell with different PCI from serving cell) on which UE performs L1-RSRP measurements.

The accuracy requirements in Table 10.1.19.1.2-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for L1-RSRP measurements are fulfilled according to Annex B.2.4.1 for a corresponding Band for each relevant SSB.

Table 10.1.19.1.2-1: SSB based L1-RSRP relative accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot Note 2 | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 4 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
|  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | N/A | -50 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | N/A | -50 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | N/A | -50 |
| ±3 | ±4 | -3 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | N/A | -50 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | N/A | -50 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_N | -114.5 | -111.5 | N/A | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The parameter SSB Ês/Iot is the minimum SSB Ês/Iot of the pair of SSBs to which the requirement applies. NOTE 3: Void NOTE 4: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |

#### 10.1.19.2	CSI-RS based L1-RSRP accuracy requirements

##### 10.1.19.2.1	Absolute Accuracy

Unless otherwise specified, the requirements for absolute accuracy of CSI-RS based L1-RSRP in this clause apply to all CSI-RS resources of the serving cell configured for L1-RSRP measurement.

The accuracy requirements in Table 10.1.19.2.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for L1-RSRP measurements are fulfilled according to Annex B.2.4.2 for a corresponding Band for each relevant CSI-RS.

-	The bandwidth of CSI-RS is 48 PRBs and the density is 3.

The performance with larger bandwidth of CSI-RS is equal to or better than the accuracy requirements in Table 10.1.19.2.1-1.

Table 10.1.19.2.1-1: CSI-RS based L1-RSRP absolute accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | CSI-RS Ês/Iot | Io Note 1 range |  |  |  |  |  |
|  |  |  | NR operating band groups Note 2 | Minimum Io |  |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSCSI-RS |  |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSCSI-RS = 15 kHz | SCSCSI-RS = 30 kHz | SCSCSI-RS = 60 kHz |  |  |
|  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | -115 | N/A | -70 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | -114.5 | N/A | -70 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | -114 | N/A | -70 |
| ±5.0 | ±9.5 | -3 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | -113.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | -113 | N/A | -70 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | -112.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | -112 | N/A | -70 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | -111.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_N | -114.5 | -111.5 | -108.5 | N/A | -70 |
| ±8.5 | ±11.5 | -3 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A, NR_FDD_FR1_B, NR_TDD_FR1_C, NR_FDD_FR1_D, NR_TDD_FR1_D, NR_FDD_FR1_E, NR_TDD_FR1_E, NR_FDD_FR1_F, NR_FDD_FR1_G, NR_FDD_FR1_H, NR_FDD_FR1_N | N/A | N/A | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |  |

##### 10.1.19.2.2	Relative Accuracy

The relative accuracy of CSI-RS based L1-RSRP is defined as the L1-RSRP measured from one CSI-RS compared to the largest measured value of L1-RSRP among all CSI-RS resources of the serving cell.

The accuracy requirements in Table 10.1.19.2.2-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for L1-RSRP measurements are fulfilled according to Annex B.2.4.2 for a corresponding Band for each relevant CSI-RS.

-	The bandwidth of CSI-RS is 48 PRBs and the density is 3.

The performance with larger bandwidth of CSI-RS is equal to or better than the accuracy requirements in Table 10.1.19.2.2-1.

Table 10.1.19.2.2-1: CSI-RS based L1-RSRP relative accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | CSI-RS Ês/Iot Note 2 | Io Note 1 range |  |  |  |  |  |
|  |  |  | NR operating band groups Note 4 | Minimum Io |  |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSCSI-RS |  |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSCSI-RS = 15 kHz | SCSCSI-RS = 30 kHz | SCSCSI-RS = 60 kHz |  |  |
|  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | -115 | N/A | -50 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | -114.5 | N/A | -50 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | -114 | N/A | -50 |
| ±3 | ±4 | -3 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | -113.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | -113 | N/A | -50 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | -112.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | -112 | N/A | -50 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | -111.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_N | -114.5 | -111.5 | -108.5 | N/A | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The parameter CSI-RS Ês/Iot is the minimum CSI-RS Ês/Iot of the pair of CSI-RS resources to which the requirement applies. NOTE 3: Void NOTE 4: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |  |

### 10.1.19C		L1-RSRP accuracy requirements for FR1 SAN

#### 10.1.19C.1	SSB based L1-RSRP accuracy requirements

##### 10.1.19C.1.1	Absolute Accuracy

Unless otherwise specified, the requirements for absolute accuracy of SSB based L1-RSRP in this clause apply to all SSBs of the serving cell configured for L1-RSRP measurement.

The accuracy requirements in Table 10.1.19C.1.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for L1-RSRP measurements are fulfilled according to Annex B.2.4.1 for a corresponding Band for each relevant SSB.

-	Valid information for the SAN serving the target cell has been provided.

Table 10.1.19C.1.1-1: SSB based L1-RSRP absolute accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 2 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
| 5 | 9.5 | -3 | NR_FDD_SAB_FR1_A | -121 | -118 | N/A | -70 |
| 8.5 | 11.5 | -3 | NR_FDD_SAB_FR1_A | N/A | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |

##### 10.1.19C.1.2	Relative Accuracy

The relative accuracy of SSB based L1-RSRP is defined as the L1-RSRP measured from one SSB compared to the largest measured value of L1-RSRP among all SSBs of the serving cell.

The accuracy requirements in Table 10.1.19C.1.2-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for L1-RSRP measurements are fulfilled according to Annex B.2.4.1 for a corresponding Band for each relevant SSB.

-	Valid information for the SAN serving the target cell has been provided.

Table 10.1.19C.1.2-1: SSB based L1-RSRP relative accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot Note 2 | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 4 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
| 3 | 4 | -3 | NR_FDD_SAB_FR1_A | -121 | -118 | N/A | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The parameter SSB Ês/Iot is the minimum SSB Ês/Iot of the pair of SSBs to which the requirement applies. NOTE 3: Void NOTE 4: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |

### 10.1.19D	LTM Intra-frequency L1-RSRP accuracy requirements for FR1

#### 10.1.19D.1	SSB based intra-frequency L1-RSRP accuracy requirements

##### 10.1.19D.1.1	Absolute Accuracy

Unless otherwise specified, the requirements for absolute accuracy of SSB based intra-frequency L1-RSRP in this clause apply to all SSBs of candidate neighbour cell(s) on the same frequency as that of the serving cell in FR1.

The accuracy requirements in Table 10.1.19D.1.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for L1-RSRP measurements are fulfilled according to Annex B.2.4.1 for a corresponding Band for each relevant SSB.

Table 10.1.19D.1.1-1: SSB based L1-RSRP absolute accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 2 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
|  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | N/A | -70 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | N/A | -70 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | N/A | -70 |
| ±5.0 | ±9.5 | -3 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | N/A | -70 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | N/A | -70 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_N | -114.5 | -111.5 | N/A | -70 |
| ±8.5 | ±11.5 | -3 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A, NR_FDD_FR1_B, NR_TDD_FR1_C, NR_FDD_FR1_D, NR_TDD_FR1_D, NR_FDD_FR1_E, NR_TDD_FR1_E, NR_FDD_FR1_F, NR_FDD_FR1_G, NR_FDD_FR1_H, NR_FDD_FR1_N | N/A | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |

##### 10.1.19D.1.2	Relative Accuracy

The relative accuracy of SSB based L1-RSRP is defined as the L1-RSRP measured from one SSB compared to the largest measured value of L1-RSRP among all SSBs of the cell(s) on the same frequency in FR1.The accuracy requirements in Table 10.1.19D.1.2-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for L1-RSRP measurements are fulfilled according to Annex B.2.4.1 for a corresponding Band for each relevant SSB.

Table 10.1.19D.1.2-1: SSB based L1-RSRP relative accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot Note 2 | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 4 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
|  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | N/A | -50 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | N/A | -50 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | N/A | -50 |
| ±3 | ±4 | -3 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | N/A | -50 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | N/A | -50 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_N | -114.5 | -111.5 | N/A | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The parameter SSB Ês/Iot is the minimum SSB Ês/Iot of the pair of SSBs to which the requirement applies. NOTE 3: Void NOTE 4: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |

### 10.1.19E			LTM Inter-frequency L1-RSRP accuracy requirements for FR1

#### 10.1.19E.1	SSB based Inter-frequency L1-RSRP accuracy requirements

##### 10.1.19E.1.1	Absolute Accuracy

Unless otherwise specified, the requirements for absolute accuracy of SSB based L1-RSRP in this clause apply to all SSBs of candidate neighbour cell(s) on a frequency in FR1 that is on a different frequency than the serving cell.

The accuracy requirements in Table 10.1.19E.1.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for inter-frequency L1-RSRP measurements are fulfilled according to Annex B.2.4.1 for a corresponding Band for each relevant SSB.

Table 10.1.19E.1.1-1: Inter-frequency L1-RSRP absolute accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 2 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
|  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | N/A | -70 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | N/A | -70 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | N/A | -70 |
| ±5.0 | ±9.5 | -3 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | N/A | -70 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | N/A | -70 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_N | -114.5 | -111.5 | N/A | -70 |
| ±8.5 | ±11.5 | -3 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A, NR_FDD_FR1_B, NR_TDD_FR1_C, NR_FDD_FR1_D, NR_TDD_FR1_D, NR_FDD_FR1_E, NR_TDD_FR1_E, NR_FDD_FR1_F, NR_FDD_FR1_G, NR_FDD_FR1_H, NR_FDD_FR1_N | N/A | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |

##### 10.1.19E.1.2	Relative Accuracy

The relative accuracy of SSB based L1-RSRP is defined as the L1-RSRP measured from one SSB from one cell on a frequency in FR1 compared to the largest measured value of L1-RSRP among all SSBs measured from another cell on another frequency in FR1.

The accuracy requirements in Table 10.1.19E.1.2-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for L1-RSRP measurements are fulfilled according to Annex B.2.4.1 for a corresponding Band for each relevant SSB.

Table 10.1.19E.1.2-1: Inter-frequency L1-RSRP relative accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 2 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
|  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | N/A | -70 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | N/A | -70 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | N/A | -70 |
| ±5.0 | ±9.5 | -3 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | N/A | -70 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | N/A | -70 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_N | -114.5 | -111.5 | N/A | -70 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |

### 10.1.20	L1-RSRP accuracy requirements for FR2

#### 10.1.20.1	SSB based L1-RSRP accuracy requirements

Unless otherwise specified, the requirements for absolute accuracy and relative accuracy of SSB based L1-RSRP in this clause apply to all SSBs of the serving cell configured for L1-RSRP measurement, [all the SSBs of the serving cell configured for L1-RSRP measurement when the UE is configured with groupBasedBeamReporting-r17 set to 'enabled'] and all SSBs of cell(s) with different PCI from serving cell configured for L1-RSRP measurement in FR2.

##### 10.1.20.1.1	Absolute Accuracy

The accuracy requirements in Table 10.1.20.1.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled.

-	Conditions for L1-RSRP measurements are fulfilled according to Annex B.2.4.1 for a corresponding Band for each relevant SSB.

-	The measured signals are in the directions covered by the percentile EIS spherical coverage of the UE, defined in clause 7.3.4 of TS 38.101-2 [19].

Table 10.1.20.1.1-1: SSB based L1-RSRP absolute accuracy in FR2

| Accuracy |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 1 range |  |  |  |
|  |  |  | Minimum Io |  |  | Maximum Io |
| dB | dB | dB | dBm / SCSSSB Note 2 |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  | SCSSSB = 120kHz | SCSSSB = 240kHz |  |  |
| ±6.5 | ±9.5 | ≥-3 | Same value as SSB_RP in Table B.2.4.1-2, according to UE Power class, operating band and angle of arrival |  | N/A | -70 |
| 8.5 | 11.5 | ≥-3 | N/A |  | -70 | -50 |
| NOTE 1: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. NOTE 2: Values based on Refsens and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. NOTE 3: In the test cases, the SSB Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. |  |  |  |  |  |  |

##### 10.1.20.1.2	Relative Accuracy

The relative accuracy of SSB based L1-RSRP is defined as the L1-RSRP measured from one SSB compared to the largest measured value of L1-RSRP among all SSBs of the cell (serving cell or cell with different PCI from serving cell) on which UE performs L1-RSRP measurements.

When the UE is configured with groupBasedBeamReporting-r17 set to 'enabled', the relative accuracy of SSB based L1-RSRP is defined as the L1-RSRP measured from one SSB compared to the largest measured value of L1-RSRP among all SSBs of the serving cell on which UE performs L1-RSRP measurements [with different Rx beams, the all SSBs are in the same or different resource set (s) in one CSI resource setting].

The accuracy requirements in Table 10.1.20.1.2-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled.

-	Conditions for L1-RSRP measurements are fulfilled according to Annex B.2.4.1 for a corresponding Band for each relevant SSB.

-	The measured signals are in the directions covered by the percentile EIS spherical coverage of the UE, defined in clause 7.3.4 of TS 38.101-2 [19].

Table 10.1.20.1.2-1: SSB based L1-RSRP relative accuracy in FR2

| Accuracy |  | Conditions |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 1 range |  |  |
|  |  |  | Minimum Io |  | Maximum Io |
| dB | dB | dB | dBm / SCSSSB Note 3 |  | dBm/BWChannel |
|  |  |  | SCSSSB = 120kHz | SCSSSB = 240kHz |  |
| 6.5 | 9.5 | ≥-3 | Same value as SSB_RP in Table B.2.4.1-2, according to UE Power class, operating band and angle of arrival |  | -50 |
| NOTE 1: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. NOTE 2: The parameter SSB Ês/Iot is the minimum SSB Ês/Iot of the pair of SSBs to which the requirement applies. NOTE 3: Values based on Refsens and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. NOTE 4: In the test cases, the SSB Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. |  |  |  |  |  |

#### 10.1.20.2	CSI-RS based L1-RSRP accuracy requirements

##### 10.1.20.2.1	Absolute Accuracy

Unless otherwise specified, the requirements for absolute accuracy of CSI-RS based L1-RSRP in this clause apply to all CSI-RS resources of the serving cell configured for L1-RSRP measurement, [and all the CSI-RSs of the serving cell configured for L1-RSRP measurement when the UE is configured with groupBasedBeamReporting-r17 set to 'enabled'].

The accuracy requirements in Table 10.1.20.2.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled.

-	Conditions for L1-RSRP measurements are fulfilled according to Annex B.2.4.2 for a corresponding Band for each relevant CSI-RS.

-	The bandwidth of CSI-RS is 48 PRBs and the density is 3.

-	The measured signals are in the directions covered by the percentile EIS spherical coverage of the UE, defined in clause 7.3.4 of TS 38.101-2 [19].

The performance with larger bandwidth of CSI-RS is equal to or better than the accuracy requirements in Table 10.1.20.2.1-1.

Table 10.1.20.2.1-1: CSI-RS based L1-RSRP absolute accuracy in FR2

| Accuracy |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | CSI-RS Ês/Iot | Io Note 1 range |  |  |  |
|  |  |  | Minimum Io |  |  | Maximum Io |
| dB | dB | dB | dBm / SCSCSI-RS Note 2 |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  | SCSCSI-RS = 60kHz | SCSCSI-RS = 120kHz |  |  |
| ±6.5 | ±9.5 | ≥-3 | Same value as CSI-RS_RP in Table B.2.4.2-2, according to UE Power class, operating band and angle of arrival |  | N/A | -70 |
| 8.5 | 11.5 | ≥-3 | N/A |  | -70 | -50 |
| NOTE 1: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. NOTE 2: Values based on Refsens and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. NOTE 3: In the test cases, the CSI-RS Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. |  |  |  |  |  |  |

##### 10.1.20.2.2	Relative Accuracy

The relative accuracy of CSI-RS based L1-RSRP is defined as the L1-RSRP measured from one CSI-RS compared to the largest measured value of L1- For simultaneous reception from multiple directions, when the UE is configured with groupBasedBeamReporting-r17 set to 'enabled', the relative accuracy of CSI-RS based L1-RSRP is defined as the L1-RSRP measured from one CSI-RS compared to the largest measured value of L1-RSRP among all CSI-RSs of the serving cell on which UE performs L1-RSRP measurements [with different Rx beams, the all CSI-RSs are in the same or different resource set (s) in one CSI resource setting].RSRP among all CSI-RS resources of the serving cell.

The accuracy requirements in Table 10.1.20.2.2-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled.

-	Conditions for L1-RSRP measurements are fulfilled according to Annex B.2.4.2 for a corresponding Band for each relevant CSI-RS.

-	The bandwidth of CSI-RS is 48 PRBs and the density is 3.

-	The measured signals are in the directions covered by the percentile EIS spherical coverage of the UE, defined in clause 7.3.4 of TS 38.101-2 [19].

The performance with larger bandwidth of CSI-RS is equal to or better than the accuracy requirements in Table 10.1.20.2.2-1.

Table 10.1.20.2.2-1: CSI-RS based L1-RSRP relative accuracy in FR2

| Accuracy |  | Conditions |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | CSI-RS Ês/Iot | Io Note 1 range |  |  |
|  |  |  | Minimum Io |  | Maximum Io |
| dB | dB | dB | dBm / SCSCSI-RS |  | dBm/BWChannel |
|  |  |  | SCSCSI-RS = 60kHz | SCSCSI-RS = 120kHz |  |
| 6.5 | 9.5 | ≥-3 | Same value as CSI-RS RP in Table B.2.4.2-2, according to UE Power class, operating band and angle of arrival |  | -50 |
| NOTE 1: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. NOTE 2: The parameter CSI-RS Ês/Iot is the minimum CSI-RS Ês/Iot of the pair of CSI-RS resources to which the requirement applies. NOTE 3: Values based on Refsens and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. NOTE 4: In the test cases, the CSI-RS Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. |  |  |  |  |  |

### 10.1.20A			LTM Intra-frequency L1-RSRP accuracy requirements for FR2

#### 10.1.20A.1	SSB based intra-frequency L1-RSRP accuracy requirements

##### 10.1.20A.1.1	Absolute Accuracy

Unless otherwise specified, the requirements for absolute accuracy of SSB based intra-frequency L1-RSRP in this clause apply to all SSBs of candidate neighbour cell(s) on the same frequency as that of the serving cell in FR2.

The accuracy requirements in Table 10.1.20A.1.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled.

-	Conditions for L1-RSRP measurements are fulfilled according to Annex B.2.4.1 for a corresponding Band for each relevant SSB.

-	The measured signals are in the directions covered by the percentile EIS spherical coverage of the UE, defined in clause 7.3.4 of TS 38.101-2 [19].

Table 10.1.20A.1.1-1: SSB based L1-RSRP absolute accuracy in FR2

| Accuracy |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 1 range |  |  |  |
|  |  |  | Minimum Io |  |  | Maximum Io |
| dB | dB | dB | dBm / SCSSSB Note 2 |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  | SCSSSB = 120kHz | SCSSSB = 240kHz |  |  |
| ±6.5 | ±9.5 | ≥-3 | Same value as SSB_RP in Table B.2.4.1-2, according to UE Power class, operating band and angle of arrival |  | N/A | -70 |
| 8.5 | 11.5 | ≥-3 | N/A |  | -70 | -50 |
| NOTE 1: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. NOTE 2: Values based on Refsens and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. NOTE 3: In the test cases, the SSB Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. |  |  |  |  |  |  |

##### 10.1.20A.1.2	Relative Accuracy

The relative accuracy of SSB based L1-RSRP is defined as the L1-RSRP measured from one SSB compared to the largest measured value of L1-RSRP among all SSBs of the cell(s) on the same frequency in FR2.

The accuracy requirements in Table 10.1.20A.1.2-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled.

-	Conditions for L1-RSRP measurements are fulfilled according to Annex B.2.4.1 for a corresponding Band for each relevant SSB.

-	The measured signals are in the directions covered by the percentile EIS spherical coverage of the UE, defined in clause 7.3.4 of TS 38.101-2 [19].

Table 10.1.20A.1.2-1: SSB based L1-RSRP relative accuracy in FR2

| Accuracy |  | Conditions |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 1 range |  |  |
|  |  |  | Minimum Io |  | Maximum Io |
| dB | dB | dB | dBm / SCSSSB Note 3 |  | dBm/BWChannel |
|  |  |  | SCSSSB = 120kHz | SCSSSB = 240kHz |  |
| 6.5 | 9.5 | ≥-3 | Same value as SSB_RP in Table B.2.4.1-2, according to UE Power class, operating band and angle of arrival |  | -50 |
| NOTE 1: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. NOTE 2: The parameter SSB Ês/Iot is the minimum SSB Ês/Iot of the pair of SSBs to which the requirement applies. NOTE 3: Values based on Refsens and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. NOTE 4: In the test cases, the SSB Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. |  |  |  |  |  |

### 10.1.20B			LTM Inter-frequency L1-RSRP accuracy requirements for FR2

#### 10.1.20B.1	SSB based inter-frequency L1-RSRP accuracy requirements

##### 10.1.20B.1.1	Absolute Accuracy

Unless otherwise specified, the requirements for absolute accuracy of SSB based L1-RSRP in this clause apply to all SSBs of candidate neighbour cell(s) on a frequency in FR2 that is on a different frequency than the serving cell.

The accuracy requirements in Table 10.1.20B.1.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled.

-	Conditions for L1-RSRP measurements are fulfilled according to Annex B.2.4.1 for a corresponding Band for each relevant SSB.

-	The measured signals are in the directions covered by the percentile EIS spherical coverage of the UE, defined in clause 7.3.4 of TS 38.101-2 [19].

Table 10.1.20B.1.1-1: SSB based L1-RSRP absolute accuracy in FR2

| Accuracy |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 1 range |  |  |  |
|  |  |  | Minimum Io |  |  | Maximum Io |
| dB | dB | dB | dBm / SCSSSB Note 2 |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  | SCSSSB = 120kHz | SCSSSB = 240kHz |  |  |
| ±6.5 | ±9.5 | ≥-3 | Same value as SSB_RP in Table B.2.4.1-2, according to UE Power class, operating band and angle of arrival |  | N/A | -70 |
| 8.5 | 11.5 | ≥-3 | N/A |  | -70 | -50 |
| NOTE 1: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. NOTE 2: Values based on Refsens and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. NOTE 3: In the test cases, the SSB Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. |  |  |  |  |  |  |

##### 10.1.20B.1.2	Relative Accuracy

The relative accuracy of SSB based L1-RSRP is defined as the L1-RSRP measured from one SSB from one cell on a frequency in FR2 compared to the largest measured value of L1-RSRP among all SSBs measured from another cell on another frequency in FR2.

The accuracy requirements in Table 10.1.20B.1.2-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled.

-	Conditions for L1-RSRP measurements are fulfilled according to Annex B.2.4.1 for a corresponding Band for each relevant SSB.

-	The measured signals are in the directions covered by the percentile EIS spherical coverage of the UE, defined in clause 7.3.4 of TS 38.101-2 [19].

Table 10.1.20B.1.2-1: SSB based L1-RSRP relative accuracy in FR2

| Accuracy |  | Conditions |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 1 range |  |  |
|  |  |  | Minimum Io |  | Maximum Io |
| dB | dB | dB | dBm / SCSSSB Note 3 |  | dBm/BWChannel |
|  |  |  | SCSSSB = 120kHz | SCSSSB = 240kHz |  |
| 6.5 | 9.5 | ≥-3 | Same value as SSB_RP in Table B.2.4.1-2, according to UE Power class, operating band and angle of arrival |  | -50 |
| NOTE 1: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. NOTE 2: The parameter SSB Ês/Iot is the minimum SSB Ês/Iot of the pair of SSBs to which the requirement applies. NOTE 3: Values based on Refsens and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. NOTE 4: In the test cases, the SSB Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. |  |  |  |  |  |

### 10.1.21	SFTD accuracy requirements

#### 10.1.21.1	SFTD acuracy requirements for NE-DC

The SFN and frame timing difference (SFTD) is measured between PCell and E-UTRAN PSCell under NE-DC.

The accuracy requirements in Table 10.1.21.1-4 are appilicable under the following conditions:

For FR1 PCell SFN and frame timing measurement:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Io range deifined in Table 10.1.21.1-1.

Table 10.1.21.1-1: PCell Io range conditions in FR1

| Parameter | Io Note 1 range |  |  |  |
| --- | --- | --- | --- | --- |
|  | NR operating band groups Note 4, 5 | Minimum Io Note 2, 3 |  | Maximum Io |
|  |  | dBm/ SCSSSB |  | dBm/BWChannel |
|  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A | -121 | -118 | -50 |
|  | NR_FDD_FR1_B | -120.5 | -117.5 | -50 |
|  | NR_TDD_FR1_C | -120 | -117 | -50 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | -50 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | -50 |
|  | NR_FDD_FR1_F | -118.5 | -115.5 | -50 |
|  | NR_FDD_FR1_G | -118 | -115 | -50 |
|  | NR_FDD_FR1_H | -117.5 | -114.5 | -50 |
|  | NR_FDD_FR1_N | -114.5 | -111.5 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The condition level is increased by ΔRIB,c as defined in clause 7.3B in TS 38.101-3 [20], depending on E-UTRA – NR band combination. NOTE 3: The condition level is increased by MSD as defined in clause 7.3B in TS 38.101-3 [20], if applicable depending on E-UTRA – NR band combination. NOTE 4: NR operating band groups are as defined in clause 3.5. NOTE 5: Only NR bands within EN-DC band combinations as specified in clause 5.5B in TS 38.101-3 [20] are applicable. |  |  |  |  |

For FR2 PCell SFN and frame timing measurement:

-	Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled.

-	Io range deifined in Table 10.1.21.1-2.

Table 10.1.21.1-2: PCell Io range conditions in FR2

| Parameter | Io Note 1 range |  |  |
| --- | --- | --- | --- |
|  | Minimum Io Note 2, 3 |  | Maximum Io |
|  | dBm/ SCSSSB |  | dBm/BWChannel |
|  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | Same value as SSB_RP in Table B.2.4.1-2, according to UE Power class, operating band and angle of arrival | Same value as SSB_RP in Table B.2.4.1-2, according to UE Power class, operating band and angle of arrival | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth and specified at the Reference point. NOTE 2: Values based on Refsens and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. NOTE 3: In the test cases, the SSB Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. |  |  |  |

For E-UTRA PSCell SFN and frame timing measurement:

-	Cell specific reference signals are transmitted either from one, two or four antenna ports.

-	Conditions defined in TS 36.101 [25] Clause 7.3 for reference sensitivity are fulfilled.

-	No changes to the uplink transmission timing are applied during the measurement period.

-	RSRP|dBm according to Annex B.3.5 in TS 36.101 [25] for a corresponding Band.

-	Io range deifined in Table 10.1.21.1-3.

Table 10.1.21.1-3: E-UTRA PSCell Io range conditions

| Parameter | Io Note 1 range |  |  |
| --- | --- | --- | --- |
|  | E-UTRA operating band groups Note 3 | Minimum Io | Maximum Io |
| Conditions |  | dBm/15kHz Note 2 | dBm/BWChannel |
|  | FDD_A, TDD_A | -121 | -50 |
|  | FDD_C, TDD_C | -120 | -50 |
|  | FDD_D | -119.5 | -50 |
|  | FDD_E, TDD_E | -119 | -50 |
|  | FDD_F | -118.5 | -50 |
|  | FDD_G | -118 | -50 |
|  | FDD_H | -117.5 | -50 |
|  | FDD_N | -114.5 | -50 |
| NOTE 1: When in dBm/15kHz, the minimum Io condition is expressed as the average Io per RE over all REs in that symbol. Io may be different in different symbols within a subframe. NOTE 2: The condition level is increased by ∆>0, when applicable, as described in clauses B.4.2 and B.4.3 in TS36.133 [15]. NOTE 3: E-UTRA operating band groups are as defined in clause 3.5 in TS 36.133 [15]. |  |  |  |

Table 10.1.21.1-4: SFTD measurement accuracy

| Accuracy | Conditions |  |
| --- | --- | --- |
|  | Ês/Iot Note 2 | Frequency range |
| Ts Note 1 | dB |  |
| 40*64*Tc | -3 | FR1 |
| 40*64*Tc |  | FR2 |
| NOTE 1: Tc is the basic timing unit defined in TS 38.211 [6]. NOTE 2: The parameter Ês/Iot is the minimum Ês/Iot of the pair of cells to which the requirement applies. |  |  |

#### 10.1.21.2	SFTD acuracy requirements for NR-DC

The SFN and frame timing difference (SFTD) is measured between PCell in FR1 and PSCell in FR2 under NR dual connectivity.

The accuracy requirements in Table 10.1.21.2-3 are appilicable under the following conditions:

For FR1 PCell SFN and frame timing measurement:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Io range deifined in Table 10.1.21.2-1.

Table 10.1.21.2-1: PCell Io range conditions in FR1

| Parameter | Io Note 1 range |  |  |  |
| --- | --- | --- | --- | --- |
|  | NR operating band groups Note 2 | Minimum Io |  | Maximum Io |
|  |  | dBm/ SCSSSB |  | dBm/BWChannel |
|  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A | -121 | -118 | -50 |
|  | NR_FDD_FR1_B | -120.5 | -117.5 | -50 |
|  | NR_TDD_FR1_C | -120 | -117 | -50 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | -50 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | -50 |
|  | NR_FDD_FR1_F | -118.5 | -115.5 | -50 |
|  | NR_FDD_FR1_G | -118 | -115 | -50 |
|  | NR_FDD_FR1_H | -117.5 | -114.5 | -50 |
|  | NR_FDD_FR1_N | -114.5 | -111.5 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups are as defined in clause 3.5.2. |  |  |  |  |

For FR2 PSCell SFN and frame timing measurement:

-	Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled.

-	Io range deifined in Table 10.1.21.2-2.

-	The measured signals are in the directions covered by the percentile EIS spherical coverage of the UE, defined in clause 7.3.4 of TS 38.101-2 [19].

Table 10.1.21.2-2: PSCell Io range conditions in FR2

| Parameter | Io Note 1 range |  |  |
| --- | --- | --- | --- |
|  | Minimum Io Note 2, 3 |  | Maximum Io |
|  | dBm/ SCSSSB |  | dBm/BWChannel |
|  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | Same value as SSB_RP in Table B.2.4.1-2, according to UE Power class, operating band and angle of arrival | Same value as SSB_RP in Table B.2.4.1-2, according to UE Power class, operating band and angle of arrival | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth and specified at the Reference point. NOTE 2: Values based on Refsens and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. NOTE 3: In the test cases, the SSB Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. |  |  |  |

Table 10.1.21.2-3: SFTD measurement accuracy

| Accuracy | Conditions |  |
| --- | --- | --- |
|  | Ês/Iot Note 2 | Frequency range |
| Ts Note 1 | dB |  |
| 40*64*Tc | -3 | Between FR1 and FR2 |
| NOTE 1: Tc is the basic timing unit defined in TS 38.211 [6]. NOTE 2: The parameter Ês/Iot is the minimum Ês/Iot of the pair of cells to which the requirement applies. |  |  |

#### 10.1.21.3	Inter frequency SFTD acuracy requirements

The SFN and frame timing difference (SFTD) is measured between PCell and inter-frequency neighbour cell.

The accuracy requirements in Table 10.1.21.3-3 are appilicable under the following conditions:

For FR1 PCell, inter frequency neighbour cell SFN and frame timing measurement:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Io range deifined in Table 10.1.21.3-1.

Table 10.1.21.3-1: PCell, inter frequency neighbour cell Io range conditions in FR1

| Parameter | Io Note 1 range |  |  |  |
| --- | --- | --- | --- | --- |
|  | NR operating band groups Note 2 | Minimum Io |  | Maximum Io |
|  |  | dBm/ SCSSSB |  | dBm/BWChannel |
|  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A | -121 | -118 | -50 |
|  | NR_FDD_FR1_B | -120.5 | -117.5 | -50 |
|  | NR_TDD_FR1_C | -120 | -117 | -50 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | -50 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | -50 |
|  | NR_FDD_FR1_F | -118.5 | -115.5 | -50 |
|  | NR_FDD_FR1_G | -118 | -115 | -50 |
|  | NR_FDD_FR1_H | -117.5 | -114.5 | -50 |
|  | NR_FDD_FR1_N | -114.5 | -111.5 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups are as defined in clause 3.5.2. |  |  |  |  |

For FR2 PCell, inter frequency neighbour cell SFN and frame timing measurement:

-	Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled.

-	Io range deifined in Table 10.1.21.3-2.

-	The measured signals are in the directions covered by the percentile EIS spherical coverage of the UE, defined in clause 7.3.4 of TS 38.101-2 [19].

Table 10.1.21.3-2: PCell, inter frequency neighbour cell Io range conditions in FR2

| Parameter | Io Note 1 range |  |  |
| --- | --- | --- | --- |
|  | Minimum Io Note 2, 3 |  | Maximum Io |
|  | dBm/ SCSSSB |  | dBm/BWChannel |
|  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | Same value as SSB_RP in Table B.2.4.1-2, according to UE Power class, operating band and angle of arrival | Same value as SSB_RP in Table B.2.4.1-2, according to UE Power class, operating band and angle of arrival | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth and specified at the Reference point. NOTE 2: Values based on Refsens and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. NOTE 3: In the test cases, the SSB Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. |  |  |  |

Table 10.1.21.3-3: Inter frequency SFTD measurement accuracy

| Accuracy | Conditions |  |
| --- | --- | --- |
|  | Ês/Iot Note 2 | Frequency range |
| Ts Note 1 | dB |  |
| 40*64*Tc | -3 | FR1, FR2 |
| NOTE 1: Tc is the basic timing unit defined in TS 38.211 [6]. NOTE 2: The parameter Ês/Iot is the minimum Ês/Iot of the pair of cells to which the requirement applies. |  |  |

### 10.1.22	CLI measurement accuracy requirements

#### 10.1.22.1	SRS-RSRP

##### 10.1.22.1.1	SRS-RSRP Accuracy

The SRS-RSRP measurement reported by the UE shall fulfil the accuracy requirements defined in Table 10.1.22.1.1-1 for FR1 and Table 10.1.22.1.1-2 for FR2, provided that the following conditions are met. The accuracy requirements in this clause are derived based on AWGN radio propagation conditions.

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for SRS-RSRP measurements are fulfilled according to Annex B.2.z for a corresponding Band for each relevant SRS resource configured for measurement.

-	The time difference between UE’s DL reference timing in the serving cell and SRS arrival time is no larger than Terror_SRS_RSRP, where

-	Terror_SRS_RSRP = TC × NTA_offset + 4.67us for FR1

-	Terror_SRS_RSRP = TC × NTA_offset + 3.67us for FR2

-	NTA_offset is defined in Table 7.1.2-2

-	TC is 0.509ns

-	The number of SRS ports in the SRS resource configured for measurement is 1,

-	The number of symbols in the SRS resource configured for measurement is 1,

-	The number of repetitions in the SRS resource configured for measurement is 1,

-	Frequency hopping, sequence group hopping or sequence hopping is disabled in the SRS resource configured for measurement,

-	The bandwidth of the SRS resource is 48 PRBs.

-	One of the following conditions is met

-	There is no other SRS resource with the same root sequence and on the same symbol and with same comb as the relevant SRS resource.

-	If multiple SRS resources are on the same symbol and with same comb, the distance between cyclic shifts of any two resources is no less than 6 if transmissionComb = n4, and no less than 4 if transmissionComb = n2.

Table 10.1.22.1.1-1:  SRS-RSRP absolute accuracy in FR1

| Accuracy |  |  |  |  |  | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition |  |  | Extreme condition |  |  | SRS Ês/Iot | Io Note 1 range |  |  |  |  |  |
|  |  |  |  |  |  |  | NR operating band groups Note 2 | Minimum Io |  |  |  | Maximum Io |
| dB |  |  |  |  |  | dB |  | dBm / SCSSRS |  |  | dBm/BW Channel | dBm/BW Channel |
| SCSSRS (kHz) |  |  | SCSSRS (kHz) |  |  |  |  |  |  |  |  |  |
| 15 | 30 | 60 | 15 | 30 | 60 |  |  | SCSSRS = 15 kHz | SCSSRS = 30 kHz | SCSSRS = 60 kHz |  |  |
| 3 | 3.5 | 5 | 7.5 | 8 | 9.5 | 1 | NR_TDD_FR1_A, | -120 | -117 | -114 | N/A | -70 |
|  |  |  |  |  |  |  | NR_TDD_FR1_C | -119 | -116 | -113 | N/A | -70 |
|  |  |  |  |  |  |  | NR_TDD_FR1_D | -118.5 | -115.5 | -112.5 | N/A | -70 |
|  |  |  |  |  |  |  | NR_TDD_FR1_E | -118 | -115 | -112 | N/A | -70 |
| 6.5 | 7 | 8.5 | 9.5 | 10 | 11.5 | 1 | NR_TDD_FR1_A, NR_TDD_FR1_C, NR_TDD_FR1_D, NR_TDD_FR1_E | N/A | N/A | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 10.1.22.1.1-2: SRS-RSRP absolute accuracy in FR2

| Accuracy |  |  |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition |  | Extreme condition |  | SRS Ês/Iot | Io Note 1 range |  |  |  |
|  |  |  |  |  | Minimum Io |  |  | Maximum Io |
| dB |  |  |  | dB | dBm / SCSSRS Note 2 |  | dBm/BWChannel | dBm/BWChannel |
| SCSSRS (kHz) |  | SCSSRS (kHz) |  |  | SCSSRS = | SCSSRS = |  |  |
| 60 | 120 | 60 | 120 |  | 60kHz | 120kHz |  |  |
| 6 | 8.5 | 9 | 11.5 | ≥1 | Same value as SRS_RP in Table B.2.7-2, according to UE Power class, operating band and angle of arrival |  | N/A | -70 |
| 9 | 11.5 | 11 | 13.5 | ≥1 | N/A |  | -70 | -50 |
| NOTE 1: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. NOTE 2: Values based on Refsens and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. NOTE 3: In the test cases, the SSB Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. |  |  |  |  |  |  |  |  |

##### 10.1.22.1.2	SRS-RSRP report mapping

The reporting range of SRS-RSRP is defined from -140 dBm to -44 dBm with 1 dB resolution. The mapping of measured quantity is defined in Table 10.1.22.1.2-1. The range in the signalling may be larger than the guaranteed accuracy range.

Table 10.1.22.1.2-1: SRS-RSRP measurement report mapping

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| SRS-RSRP_0 | SRS-RSRP<-140 | dBm |
| SRS-RSRP_1 | -140≤ SRS-RSRP<-139 | dBm |
| SRS-RSRP_2 | -139≤ SRS-RSRP<-138 | dBm |
| SRS-RSRP_3 | -138≤ SRS-RSRP<-137 | dBm |
| SRS-RSRP_4 | -137≤ SRS-RSRP<-136 | dBm |
| .. | .. | … |
| SRS-RSRP_95 | -46≤ SRS-RSRP<-45 | dBm |
| SRS-RSRP_96 | -45≤ SRS-RSRP<-44 | dBm |
| SRS-RSRP_97 | -44≤ SRS-RSRP | dBm |
| SRS-RSRP_98 | Infinity |  |
| Note: ‘Infinity’ means that UE cannot detect SRS due to too strong signal to measure. |  |  |

#### 10.1.22.2	CLI-RSSI

##### 10.1.22.2.1	CLI-RSSI Accuracy

The CLI-RSSI measurement reported by the UE shall fulfil the accuracy requirements defined in Table 10.1.22.2.1-1 for FR1 and Table 10.1.22.2.1-2 for FR2, provided that the following conditions are met.

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

Table 10.1.22.2.1-1: CLI-RSSI absolute accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Io Note 1 range |  |  |  |  |  |
|  |  | NR operating band groups Note 2 | Minimum Io |  |  |  | Maximum Io |
| dB | dB |  | dBm / SCSSRS |  |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  | SCSSRS = 15 kHz | SCSSRS = 30 kHz | SCSSRS = 60 kHz |  |  |
| 3.5 | 6.5 | NR_TDD_FR1_A, | -120 | -117 | -114 | N/A | -70 |
|  |  | NR_TDD_FR1_C | -119 | -116 | -113 | N/A | -70 |
|  |  | NR_TDD_FR1_D | -118.5 | -115.5 | -112.5 | N/A | -70 |
|  |  | NR_TDD_FR1_E | -118 | -115 | -112 | N/A | -70 |
| 5.5 | 8.5 | Note 3 | Note 3 | Note 3 | Note 3 | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups in FR1 are as defined in clause 3.5.2. NOTE 3: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. |  |  |  |  |  |  |  |

Table 10.1.22.2.1-2: CLI-RSSI absolute accuracy in FR2

| Accuracy |  | Conditions |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Io Note 1 range |  |  |  |
|  |  | Minimum Io |  |  | Maximum Io |
| dB | dB | dBm / SCSSRS Note 2 |  | dBm/BWChannel | dBm/BWChannel |
|  |  | SCSSRS = 60kHz | SCSSRS = 120kHz |  |  |
| 5 | 8 | Same value as SRS_RP in Table B.2.7-2, according to UE Power class, operating band and angle of arrival |  | N/A | -70 |
| 7 | 10 | Note 4 |  | -70 | -50 |
| NOTE 1: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. NOTE 2: Values based on Refsens and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. NOTE 3: In the test cases, the SSB Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. NOTE 4: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. |  |  |  |  |  |

##### 10.1.22.2.2	CLI-RSSI report mapping

The reporting range of CLI-RSSI is defined from -100 dBm to -25 dBm with 1 dB resolution. The mapping of measured quantity is defined in Table 10.1.22.2.2-1. The range in the signalling may be larger than the guaranteed accuracy range. UE shall scale the measured CLI-RSSI to report a nominal RSSI equivalent to 6 RB measurement with 15kHz SCS.

Table 10.1.22.2.2-1: CLI-RSSI measurement report mapping

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| CLI-RSSI_00 | CLI-RSSI < -100 | dBm |
| CLI-RSSI_01 | -100 CLI-RSSI < -99 | dBm |
| CLI-RSSI_02 | -99 CLI-RSSI < -98 | dBm |
| … | … | … |
| CLI-RSSI_74 | -27 CLI-RSSI < -26 | dBm |
| CLI-RSSI_75 | -26 CLI-RSSI < -25 | dBm |
| CLI-RSSI_76 | -25 CLI-RSSI | dBm |

### 10.1.23	RSTD Measurements

#### 10.1.23.1	Introduction

The requirements in Clause 10.1.23 shall apply, provided the UE has received nr-DL-TDOA-RequestLocationInformation message from LMF via LPP [34] requesting the UE to report one or more DL RSTD measurements defined in TS 38.215 [4]. The requirements in Clause 10.1.23 shall apply:

-	when UE is in RRC_CONNECTED state and the measurement is performed with MG or without MG,

-	when UE is in RRC_INACTIVE state.

#### 10.1.23.2	Measurement Accuracy Requirements

The accuracy requirements for RSTD measurement shall be within ±(X+Y+Z+Δ) Tc.

X is defined in Table 10.1.23.2-1 for AWGN channel and Table 10.1.23.2-3 for fading channel for FR1, provided that the following conditions are met.

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for RSTD measurements are fulfilled according to Annex B.2.14 for a corresponding Band for each relevant PRS resource configured for measurement.

-	UE does not perform positioning measurement with reduced number of samples.

X is defined in Table 10.1.23.2-2 for AWGN channel and Table 10.1.23.2-4 for fading channel for FR2, provided that the following conditions are met.

-	Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled.

-	Conditions for RSTD measurements are fulfilled according to Annex B.2.14 for a corresponding Band for each relevant PRS resource configured for measurement.

- 	UE does not perform positioning measurement with reduced number of samples.

X is defined in Table 10.1.23.2-7 for AWGN channel in FR1 provided that the following conditions are met.

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for RSTD measurements are fulfilled according to Annex B.2.14 for a corresponding Band for each relevant PRS resource configured for measurement.

-	UE supports positioning measurement with reduced number of sample and is indicated by LMF to perform positioning measurement with reduced number of samples.

X is defined in Table 10.1.23.2-8 for AWGN channel in FR2 provided that the following conditions are met.

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for RSTD measurements are fulfilled according to Annex B.2.14 for a corresponding Band for each relevant PRS resource configured for measurement.

-	UE supports positioning measurement with reduced number of sample and is indicated by LMF to perform positioning measurement with reduced number of samples.

The requriements for fading channel in this clause are derived based on TDL-A (30 ns delay spread, 5Hz) and TDL-C (60 ns delay spread, 300 Hz) channel models for FR1 and FR2 respectively.

When UE measures RSTD on PRS resources belonging to different PFLs, then the RSTD accuracy is defined as the accuracy corresponding to the largest accuracy value among different PFLs.

If the UE doesn’t support Rx TEG reporting for RSTD measurement or when the measurements of reference cell and neighbour cell belong to different Rx TEGs, Y, Z and Δ are defined as follows:

-	When UE measures RSTD on PRS resources belonging to same PFL, Y=32 Tc, provided that the time offset between the two PRS resource instances from the reference cell and the neighbor cell, which are used for a single RSTD estimate, is no greater than 160 ms.

-	When UE measures RSTD on PRS resources belonging different PFLs, Y=256 Tc, provided that the time offset between the two PRS resource instances from the reference cell and the neighbor cell, which are used for a single RSTD estimate, is no greater than 1280 ms.

-	Z is defined in Table 10.1.23.2-5 for FR1 and Table 10.1.23.2-6 for FR2, respectively.

-	Δ is zero for single PFL, and is defined in Table 10.1.23.2-5a for FR1 and Table 10.1.23.2-6a for FR2, respectively, for dual PFL.

If the measurements of reference cell and neighbour cell belong to the same Rx TEG, i.e. associated and reported with a common Rx TEG ID, then the sum of Y+Z+Δ is equal to the timing error margin of the Rx TEG reported in nr-UE-RxTEG-TimingErrorMargin. The timing error margin reported via nr-UE-RxTEG-TimingErrorMargin cannot be larger than the value of (Y+Z+Δ) defined when the UE does not associate the measurements with the same Rx TEG.

Table 10.1.23.2-1: RSTD absolute accuracy in FR1 for AWGN channel

| Accuracy | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | PRS Ês/Iot | PRS SCS | PRS bandwidth Note 1 | PRS resource repetition (TrepPRS*LPRS/KcombPRS) Note 2 | Io Note 3 range |  |  |
|  |  |  |  |  | NR operating band groups Note 4 | Minimum Io | Maximum Io |
| Tc Note 5 | dB | kHz | RB |  |  | dBm/SCS | dBm/BWChannel |
| 132 | (PRS Ês/Iot)ref ≥-6dB (PRS Ês/Iot)i ≥-13dB | 15 | ≥ 24 | ≥ 4 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -127 | -50 |
|  |  |  |  |  | NR_FDD_FR1_B | -126.5 | -50 |
|  |  |  |  |  | NR_TDD_FR1_C | -126 | -50 |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -125.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -125 | -50 |
|  |  |  |  |  | NR_FDD_FR1_F | -124.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_G, NR_TDD_FR1_G | -124 | -50 |
|  |  |  |  |  | NR_FDD_FR1_H | -123.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_N | -120.5 | -50 |
| 98 |  |  | ≥ 52 | ≥ 1 | Note 6 | Note 6 | Note 6 |
| 42 |  |  | ≥ 104 | ≥ 1 | Note 6 | Note 6 | Note 6 |
| 75 |  | 30 | ≥ 24 | ≥ 4 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -124 | -50 |
|  |  |  |  |  | NR_FDD_FR1_B | -123.5 | -50 |
|  |  |  |  |  | NR_TDD_FR1_C | -123 | -50 |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -122.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -122 | -50 |
|  |  |  |  |  | NR_FDD_FR1_F | -121.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_G, NR_TDD_FR1_G | -121 | -50 |
|  |  |  |  |  | NR_FDD_FR1_H | -120.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_N | -117.5 | -50 |
| 48 |  |  | ≥ 48 | ≥ 1 | Note 6 | Note 6 | Note 6 |
| 24 |  |  | ≥ 132 | ≥ 1 | Note 6 | Note 6 | Note 6 |
| 50 |  | 60 | ≥ 24 | ≥ 4 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -50 |
|  |  |  |  |  | NR_FDD_FR1_B | -120.5 | -50 |
|  |  |  |  |  | NR_TDD_FR1_C | -120 | -50 |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -50 |
|  |  |  |  |  | NR_FDD_FR1_F | -118.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_G, NR_TDD_FR1_G | -118 | -50 |
|  |  |  |  |  | NR_FDD_FR1_H | -117.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_N | -114.5 | -50 |
| 24 |  |  | ≥ 64 | ≥ 1 | Note 6 | Note 6 | Note 6 |
| 10 |  |  | ≥ 132 | ≥ 1 | Note 6 | Note 6 | Note 6 |
| NOTE 1: Minimum PRS bandwidth, which is minimum of the PRS bandwidths of the reference resource and the measured neighbour resource i. NOTE 2: Minimum number of PRS resource repetitions among the reference resource and the measured neighbour resource i. TrepPRS, LPRS ,KcombPRS are configured by higher layer parameter dl-PRS-ResourceRepetitionFactor, dl-PRS-NumSymbols and dl-PRS-CombSizeNdefined in TS 37.355 [34], respectively. NOTE 3: Io is assumed to have constant EPRE across the bandwidth. NOTE 4: NR operating band groups in FR1 are as defined in clause 3.5.2. NOTE 5: Tc is the basic timing unit defined in TS 38.211 [6]. NOTE 6: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth of the smallest RB number for the corresponding SCS. NOTE 7: Void |  |  |  |  |  |  |  |

Table 10.1.23.2-2: RSTD absolute accuracy in FR2 for AWGN channel

| Accuracy | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | PRS Ês/Iot | PRS SCS | PRS bandwidth Note 1 | PRS resource repetition (TrepPRS*LPRS/KcombPRS) Note 2 | Io Note 3 range |  |
|  |  |  |  |  | Minimum Io | Maximum Io |
| Tc Note 4 | dB | kHz | RB |  | dBm/SCS | dBm/BWChannel |
| 35 | (PRS Ês/Iot)ref ≥-6dB (PRS Ês/Iot)i ≥-13dB | 60 | ≥ 24 | ≥ 4 | Same value as PRS_RP in Table B.2.14-2, according to UE Power class, operating band and angle of arrival | -50 |
| 24 |  |  | ≥ 64 | ≥ 1 | Note 5 | Note 5 |
| 11 |  |  | ≥ 132 | ≥ 1 | Note 5 | Note 5 |
| 24 |  | 120 | ≥ 32 | ≥ 4 | Same value as PRS_RP in Table B.2.14-2, according to UE Power class, operating band and angle of arrival | -50 |
| 13 |  |  | ≥ 64 | ≥ 1 | Note 5 | Note 5 |
| 6 |  |  | ≥ 128 | ≥ 1 | Note 5 | Note 5 |
| NOTE 1: Minimum PRS bandwidth, which is minimum of the PRS bandwidths of the reference resource and the measured neighbour resource i. NOTE 2: Minimum number of PRS resource repetitions among the reference resource and the measured neighbour resource i. TrepPRS, LPRS ,KcombPRS are configured by higher layer parameter dl-PRS-ResourceRepetitionFactor, dl-PRS-NumSymbols and dl-PRS-CombSizeNdefined in TS 37.355 [34], respectively. NOTE 3: Io is assumed to have constant EPRE across the bandwidth. NOTE 4: Tc is the basic timing unit defined in TS 38.211 [6]. NOTE 5: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth of the smallest RB number for the corresponding SCS. NOTE 6: Void |  |  |  |  |  |  |

Table 10.1.23.2-3: RSTD absolute accuracy in FR1 for fading channel

| Accuracy | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | PRS Ês/Iot | PRS SCS | PRS bandwidth Note 1 | PRS resource repetition (TrepPRS*LPRS/KcombPRS) Note 2 | Io Note 3 range |  |  |
|  |  |  |  |  | NR operating band groups Note 4 | Minimum Io | Maximum Io |
| Tc Note 5 | dB | kHz | RB |  |  | dBm/SCS | dBm/BWChannel |
| 247 | (PRS Ês/Iot)ref ≥-6dB (PRS Ês/Iot)i ≥-13dB | 15 | ≥ 24 | ≥ 4 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -127 | -50 |
|  |  |  |  |  | NR_FDD_FR1_B | -126.5 | -50 |
|  |  |  |  |  | NR_TDD_FR1_C | -126 | -50 |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -125.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -125 | -50 |
|  |  |  |  |  | NR_FDD_FR1_F | -124.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_G, NR_TDD_FR1_G | -124 | -50 |
|  |  |  |  |  | NR_FDD_FR1_H | -123.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_N | -120.5 | -50 |
| 140 |  |  | ≥ 52 | ≥ 1 | Note 6 | Note 6 | Note 6 |
| 86 |  |  | ≥ 104 | ≥ 1 | Note 6 | Note 6 | Note 6 |
| 118 |  | 30 | ≥ 24 | ≥ 4 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -124 | -50 |
|  |  |  |  |  | NR_FDD_FR1_B | -123.5 | -50 |
|  |  |  |  |  | NR_TDD_FR1_C | -123 | -50 |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -122.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -122 | -50 |
|  |  |  |  |  | NR_FDD_FR1_F | -121.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_G, NR_TDD_FR1_G | -121 | -50 |
|  |  |  |  |  | NR_FDD_FR1_H | -120.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_N | -117.5 | -50 |
| 109 |  |  | ≥ 48 | ≥ 1 | Note 6 | Note 6 | Note 6 |
| 28 |  |  | ≥ 132 | ≥ 1 | Note 6 | Note 6 | Note 6 |
| 147 |  | 60 | ≥ 24 | ≥ 4 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -50 |
|  |  |  |  |  | NR_FDD_FR1_B | -120.5 | -50 |
|  |  |  |  |  | NR_TDD_FR1_C | -120 | -50 |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -50 |
|  |  |  |  |  | NR_FDD_FR1_F | -118.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_G, NR_TDD_FR1_G | -118 | -50 |
|  |  |  |  |  | NR_FDD_FR1_H | -117.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_N | -114.5 | -50 |
| 27 |  |  | ≥ 64 | ≥ 1 | Note 6 | Note 6 | Note 6 |
| 21 |  |  | ≥ 132 | ≥ 1 | Note 6 | Note 6 | Note 6 |
| NOTE 1: Minimum PRS bandwidth, which is minimum of the PRS bandwidths of the reference resource and the measured neighbour resource i. NOTE 2: Minimum number of PRS resource repetitions among the reference resource and the measured neighbour resource i. TrepPRS, LPRS ,KcombPRS are configured by higher layer parameter dl-PRS-ResourceRepetitionFactor, dl-PRS-NumSymbols and dl-PRS-CombSizeNdefined in TS 37.355 [34], respectively. NOTE 3: Io is assumed to have constant EPRE across the bandwidth. NOTE 4: NR operating band groups in FR1 are as defined in clause 3.5.2. NOTE 5: Tc is the basic timing unit defined in TS 38.211 [6]. NOTE 6: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth of the smallest RB number for the corresponding SCS. NOTE 7: Void |  |  |  |  |  |  |  |

Table 10.1.23.2-4:  RSTD absolute accuracy in FR2 for fading channel

| Accuracy | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | PRS Ês/Iot | PRS SCS | PRS bandwidth Note 1 | PRS resource repetition (TrepPRS*LPRS/KcombPRS) Note 2 | Io Note 3 range |  |
|  |  |  |  |  | Minimum Io | Maximum Io |
| Tc Note 4 | dB | kHz | RB |  | dBm/SCS | dBm/BWChannel |
| 83 | (PRS Ês/Iot)ref ≥-6dB (PRS Ês/Iot)i ≥-13dB | 60 | ≥ 24 | ≥ 4 | Same value as PRS_RP in Table B.2.14-2, according to UE Power class, operating band and angle of arrival | -50 |
| 64 |  |  | ≥ 64 | ≥ 1 | Note 5 | Note 5 |
| 46 |  |  | ≥ 132 | ≥ 1 | Note 5 | Note 5 |
| 48 |  | 120 | ≥ 32 | ≥ 4 | Same value as PRS_RP in Table B.2.14-2, according to UE Power class, operating band and angle of arrival | -50 |
| 54 |  |  | ≥ 64 | ≥ 1 | Note 5 | Note 5 |
| 36 |  |  | ≥ 128 | ≥ 1 | Note 5 | Note 5 |
| NOTE 1: Minimum PRS bandwidth, which is minimum of the PRS bandwidths of the reference resource and the measured neighbour resource i. NOTE 2: Minimum number of PRS resource repetitions among the reference resource and the measured neighbour resource i. TrepPRS, LPRS ,KcombPRS are configured by higher layer parameter dl-PRS-ResourceRepetitionFactor, dl-PRS-NumSymbols and dl-PRS-CombSizeNdefined in TS 37.355 [34], respectively. NOTE 3: Io is assumed to have constant EPRE across the bandwidth. NOTE 4: Tc is the basic timing unit defined in TS 38.211 [6]. NOTE 5: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth of the smallest RB number for the corresponding SCS. NOTE 6: Void |  |  |  |  |  |  |

Table 10.1.23.2-5: Margin for RSTD measurement accuracy in FR1

| PRS BW (RB number) |  |  | Margin (Tc) |
| --- | --- | --- | --- |
| SCS=15kHz | SCS=30kHz | SCS=60kHz |  |
| ≥ 24 | N/A | N/A | 120 |
| ≥ 52 | ≥ 24 | N/A | 72 |
| ≥ 104 | ≥ 48 | ≥ 24 | 36 |
| N/A | ≥ 132 | ≥ 64 | 16 |
| N/A | N/A | ≥ 132 | 12 |

Table 10.1.23.2-5a: Margin Δ for RSTD measurement accuracy in FR1

| PRS BW (RB number) |  |  | Margin (Tc) |
| --- | --- | --- | --- |
| SCS=15kHz | SCS=30kHz | SCS=60kHz |  |
| ≥ 24 | N/A | N/A | 128 |
| ≥ 52 | ≥ 24 | N/A | 64 |
| ≥ 104 | ≥ 48 | ≥ 24 | 32 |
| N/A | ≥ 132 | ≥ 64 | 16 |
| N/A | N/A | ≥ 132 | 8 |

Table 10.1.23.2-6: Margin for RSTD measurement accuracy in FR2

| PRS BW (RB number) |  | Margin (Tc) |
| --- | --- | --- |
| SCS=60kHz | SCS=120kHz |  |
| ≥ 24 | N/A | 72 |
| ≥ 64 | ≥ 32 | 32 |
| ≥ 132 | ≥ 64 | 16 |
| N/A | ≥ 128 | 12 |

Table 10.1.23.2-6a: Margin Δ for RSTD measurement accuracy in FR2

| PRS BW (RB number) |  | Margin (Tc) |
| --- | --- | --- |
| SCS=60kHz | SCS=120kHz |  |
| ≥ 24 | N/A | 32 |
| ≥ 64 | ≥ 32 | 16 |
| ≥ 132 | ≥ 64 | 8 |
| N/A | ≥ 128 | 4 |

Table 10.1.23.2-7: RSTD absolute accuracy in FR1 for AWGN channel with reduced number of samples

| Accuracy | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | PRS Ês/Iot | PRS SCS | PRS bandwidth Note 1 | PRS resource repetition (TrepPRS*LPRS/KcombPRS) Note 2 | Io Note 3 range |  |  |
|  |  |  |  |  | NR operating band groups Note 4 | Minimum Io | Maximum Io |
| Tc Note 5 | dB | kHz | RB |  |  | dBm/SCS | dBm/BWChannel |
| 98 | (PRS Ês/Iot)ref ≥-3dB (PRS Ês/Iot)i ≥-6dB | 15 | ≥ 52 | ≥ 1 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -127 | -50 |
|  |  |  |  |  | NR_FDD_FR1_B | -126.5 | -50 |
|  |  |  |  |  | NR_TDD_FR1_C | -126 | -50 |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -125.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -125 | -50 |
|  |  |  |  |  | NR_FDD_FR1_F | -124.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_G, NR_TDD_FR1_G | -124 | -50 |
|  |  |  |  |  | NR_FDD_FR1_H | -123.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_N | -120.5 | -50 |
| 42 |  |  | ≥ 104 | ≥ 1 | Note 6 | Note 6 | Note 6 |
| 48 |  | 30 | ≥ 48 | ≥ 1 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -124 | -50 |
|  |  |  |  |  | NR_FDD_FR1_B | -123.5 | -50 |
|  |  |  |  |  | NR_TDD_FR1_C | -123 | -50 |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -122.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -122 | -50 |
|  |  |  |  |  | NR_FDD_FR1_F | -121.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_G, NR_TDD_FR1_G | -121 | -50 |
|  |  |  |  |  | NR_FDD_FR1_H | -120.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_N | -117.5 | -50 |
| 24 |  |  | ≥ 132 | ≥ 1 | Note 6 | Note 6 | Note 6 |
| 24 |  | 60 | ≥ 64 | ≥ 1 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -50 |
|  |  |  |  |  | NR_FDD_FR1_B | -120.5 | -50 |
|  |  |  |  |  | NR_TDD_FR1_C | -120 | -50 |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -50 |
|  |  |  |  |  | NR_FDD_FR1_F | -118.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_G, NR_TDD_FR1_G | -118 | -50 |
|  |  |  |  |  | NR_FDD_FR1_H | -117.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_N | -114.5 | -50 |
| 10 |  |  | ≥ 132 | ≥ 1 | Note 6 | Note 6 | Note 6 |
| NOTE 1: Minimum PRS bandwidth, which is minimum of the PRS bandwidths of the reference resource and the measured neighbour resource i. NOTE 2: Minimum number of PRS resource repetitions among the reference resource and the measured neighbour resource i. TrepPRS, LPRS ,KcombPRS are configured by higher layer parameter dl-PRS-ResourceRepetitionFactor, dl-PRS-NumSymbols and dl-PRS-CombSizeN defined in TS 37.355 [34], respectively. NOTE 3: Io is assumed to have constant EPRE across the bandwidth. NOTE 4: NR operating band groups in FR1 are as defined in clause 3.5.2. NOTE 5: Tc is the basic timing unit defined in TS 38.211 [6]. NOTE 6: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth of the smallest RB number for the corresponding SCS. NOTE 7: Void |  |  |  |  |  |  |  |

Table 10.1.23.2-8: RSTD absolute accuracy in FR2 for AWGN channel with reduced number of samples

| Accuracy | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | PRS Ês/Iot | PRS SCS | PRS bandwidth Note 1 | PRS resource repetition (TrepPRS*LPRS/KcombPRS) Note 2 | Io Note 3 range |  |
|  |  |  |  |  | Minimum Io | Maximum Io |
| Tc Note 4 | dB | kHz | RB |  | dBm/SCS | dBm/BWChannel |
| 24 | (PRS Ês/Iot)ref ≥-3dB (PRS Ês/Iot)i ≥-6dB | 60 | ≥ 64 | ≥ 1 | Same value as PRS_RP in Table B.2.14-2, according to UE Power class, operating band and angle of arrival | Note 5 |
| 11 |  |  | ≥ 132 | ≥ 1 | Note 5 | Note 5 |
| 13 |  | 120 | ≥ 64 | ≥ 1 | Same value as PRS_RP in Table B.2.14-2, according to UE Power class, operating band and angle of arrival | Note 5 |
| 6 |  |  | ≥ 128 | ≥ 1 | Note 5 | Note 5 |
| NOTE 1: Minimum PRS bandwidth, which is minimum of the PRS bandwidths of the reference resource and the measured neighbour resource i. NOTE 2: Minimum number of PRS resource repetitions among the reference resource and the measured neighbour resource i. TrepPRS, LPRS ,KcombPRS are configured by higher layer parameter dl-PRS-ResourceRepetitionFactor, dl-PRS-NumSymbols and dl-PRS-CombSizeN defined in TS 37.355 [34], respectively. NOTE 3: Io is assumed to have constant EPRE across the bandwidth. NOTE 4: Tc is the basic timing unit defined in TS 38.211 [6]. NOTE 5: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth of the smallest RB number for the corresponding SCS. NOTE 6: Void |  |  |  |  |  |  |

#### 10.1.23.3	Report mapping

##### 10.1.23.3.1	Absolute DL RSTD Measurement Reporting

The reporting range for the DL RSTD measurement is defined from -985024Tc to 985024Tc with the resolution step of 2kTc, where

Tc is defined in TS 38.211 [6],

kmin≤k≤kmax,

kmin=2 and kmax=5, when configured PRS resource of at least one of the reference cell and neighbor cell measured for the RSTD measurement is in FR1,

kmin=0 and kmax=5, when configured PRS resource of both the reference cell and neighbor cell measured for the RSTD measurement are in FR2,

k≥ timingReportingGranularityFactor [34] configured by LMF via LPP for the RSTD measurement.

The measurement report mapping for different k values are specified in Tables 10.1.23.3.1-1  10.1.23.3.1-6.

Table 10.1.23.3.1-1: Report mapping for k=0

| Reported Quantity Value, | Measured Quantity Value, | Unit |
| --- | --- | --- |
| RSTD_i | RSTD |  |
| RSTD_0000000 | RSTD < -985024 | Tc |
| RSTD_0000001 | -985024 RSTD < -985023 | Tc |
| RSTD_0000002 | -985023 RSTD < -985022 | Tc |
|  |  | … |
| RSTD_0985024 | -1 RSTD < 0 | Tc |
| RSTD_0985025 | 0 RSTD < 1 | Tc |
| … | … | … |
| RSTD_1970047 | 985022 RSTD < 985023 | Tc |
| RSTD_1970048 | 985023 RSTD < 985024 | Tc |
| RSTD_1970049 | 985024 RSTD | Tc |

Table 10.1.23.3.1-2: Report mapping for k=1

| Reported Quantity Value, | Measured Quantity Value, | Unit |
| --- | --- | --- |
| RSTD_i | RSTD |  |
| RSTD_000000 | RSTD < -985024 | Tc |
| RSTD_000001 | -985024 RSTD < -985022 | Tc |
| RSTD_000002 | -985022 RSTD < -985020 | Tc |
|  |  | … |
| RSTD_492512 | -2 RSTD < 0 | Tc |
| RSTD_492513 | 0 RSTD < 2 | Tc |
| … | … | … |
| RSTD_985023 | 985020 RSTD < 985022 | Tc |
| RSTD_985024 | 985022 RSTD < 985024 | Tc |
| RSTD_985025 | 985024 RSTD | Tc |

Table 10.1.23.3.1-3: Report mapping for k=2

| Reported Quantity Value, | Measured Quantity Value, | Unit |
| --- | --- | --- |
| RSTD_i | RSTD |  |
| RSTD_000000 | RSTD < -985024 | Tc |
| RSTD_000001 | -985024 RSTD < -985020 | Tc |
| RSTD_000002 | -985020 RSTD < -985016 | Tc |
|  |  | … |
| RSTD_246256 | -4 RSTD < 0 | Tc |
| RSTD_246257 | 0 RSTD < 4 | Tc |
| … | … | … |
| RSTD_492511 | 985016 RSTD < 985020 | Tc |
| RSTD_492512 | 985020 RSTD < 985024 | Tc |
| RSTD_492513 | 985024 RSTD | Tc |

Table 10.1.23.3.1-4: Report mapping for k=3

| Reported Quantity Value | Measured Quantity Value, | Unit |
| --- | --- | --- |
| RSTD_i | RSTD |  |
| RSTD_000000 | RSTD < -985024 | Tc |
| RSTD_000001 | -985024 RSTD < -985016 | Tc |
| RSTD_000002 | -985016 RSTD < -985008 | Tc |
|  |  | … |
| RSTD_123128 | -8 RSTD < 0 | Tc |
| RSTD_123129 | 0 RSTD < 8 | Tc |
| … | … | … |
| RSTD_246255 | 985008 RSTD < 985016 | Tc |
| RSTD_246256 | 985016 RSTD < 985024 | Tc |
| RSTD_246257 | 985024 RSTD | Tc |

Table 10.1.23.3.1-5: Report mapping for k=4

| Reported Quantity Value, | Measured Quantity Value, | Unit |
| --- | --- | --- |
| RSTD_i | RSTD |  |
| RSTD_000000 | RSTD < -985024 | Tc |
| RSTD_000001 | -985024 RSTD < -985008 | Tc |
| RSTD_000002 | -985008 RSTD < -984992 | Tc |
|  |  | … |
| RSTD_061564 | -16 RSTD < 0 | Tc |
| RSTD_061565 | 0 RSTD < 16 | Tc |
| … | … | … |
| RSTD_123127 | 984992 RSTD < 985008 | Tc |
| RSTD_123128 | 985008 RSTD < 985024 | Tc |
| RSTD_123129 | 985024 RSTD | Tc |

Table 10.1.23.3.1-6: Report mapping for k=5

| Reported Quantity Value, | Measured Quantity Value, | Unit |
| --- | --- | --- |
| RSTD_i | RSTD |  |
| RSTD_00000 | RSTD < -985024 | Tc |
| RSTD_00001 | -985024 RSTD < -984992 | Tc |
| RSTD_00002 | -984992 RSTD < -984960 | Tc |
|  |  | … |
| RSTD_30782 | -32 RSTD < 0 | Tc |
| RSTD_30783 | 0 RSTD < 32 | Tc |
| … | … | … |
| RSTD_61563 | 984960 RSTD < 984992 | Tc |
| RSTD_61564 | 984992 RSTD < 985024 | Tc |
| RSTD_61565 | 985024 RSTD | Tc |

##### 10.1.23.3.2	Differential Reporting for DL RSTD Measurement

A first DL RSTD measurement is reported by means of differential reporting, i.e. as RSTD, relative to a second DL RSTD measurement (RSTD2), provided that:

-	the absolute measured quantity value of the second DL RSTD measurement (RSTD2) is not larger than the absolute measured quantity value of the first DL RSTD measurement (RSTD1), i.e., RSTD=RSTD1-RSTD2≥0, and

-	the absolute value of the second DL RSTD measurement (RSTD2) is reported together with RSTD for the first DL RSTD measurement.

The reporting range for differential reporting RSTD of the first DL RSTD measurement is defined from 0 up to 8191Tc with the resolution step of 2kTc, where

Tc is defined in TS 38.211 [6],

kmin≤k≤kmax,

kmin=2 and kmax=5, when configured PRS resource of at least one of the reference cell and neighbor cell measured for the first RSTD measurement or second RSTD measurement is in FR1,

kmin=0 and kmax=5, when configured PRS resource of both the reference cell and neighbor cell measured for both of the first RSTD measurement and the second RSTD measurement are in FR2,

k≥ timingReportingGranularityFactor [34] configured by LMF via LPP for the RSTD measurement.

The measurement report mapping for different k values are specified in Tables 10.1.23.3.2-1  10.1.23.3.2-6.

Table 10.1.23.3.2-1: Report mapping for k=0

| Reported Quantity Value, DIFFRSTD_i | RSTD = RSTD1 RSTD2 | Unit |
| --- | --- | --- |
| DIFFRSTD_0000 | 0 RSTD < 1 | Tc |
| DIFFRSTD_0001 | 1 RSTD < 2 | Tc |
| DIFFRSTD_0002 | 2 RSTD < 3 | Tc |
|  |  | … |
| DIFFRSTD_8189 | 8189 RSTD < 8190 | Tc |
| DIFFRSTD_8190 | 8190 RSTD < 8191 | Tc |
| DIFFRSTD_8191 | 8191 RSTD | Tc |

Table 10.1.23.3.2-2: Report mapping for k=1

| Reported Quantity Value, DIFFRSTD_i | RSTD = RSTD1 RSTD2 | Unit |
| --- | --- | --- |
| DIFFRSTD_0000 | 0 RSTD < 2 | Tc |
| DIFFRSTD_0001 | 2 RSTD < 4 | Tc |
| DIFFRSTD_0002 | 4 RSTD < 6 | Tc |
|  |  | … |
| DIFFRSTD_4093 | 8186 RSTD < 8188 | Tc |
| DIFFRSTD_4094 | 8188 RSTD < 8190 | Tc |
| DIFFRSTD_4095 | 8190 RSTD | Tc |

Table 10.1.23.3.2-3: Report mapping for k=2

| Reported Quantity Value, DIFFRSTD_i | RSTD = RSTD1 RSTD2 | Unit |
| --- | --- | --- |
| DIFFRSTD_0000 | 0 RSTD < 4 | Tc |
| DIFFRSTD_0001 | 4 RSTD < 8 | Tc |
| DIFFRSTD_0002 | 8 RSTD < 12 | Tc |
|  |  | … |
| DIFFRSTD_2045 | 8180 RSTD < 8184 | Tc |
| DIFFRSTD_2046 | 8184 RSTD < 8188 | Tc |
| DIFFRSTD_2047 | 8188 RSTD | Tc |

Table 10.1.23.3.2-4: Report mapping for k=3

| Reported Quantity Value, DIFFRSTD_i | RSTD = RSTD1 RSTD2 | Unit |
| --- | --- | --- |
| DIFFRSTD_0000 | 0 RSTD < 8 | Tc |
| DIFFRSTD_0001 | 8 RSTD < 16 | Tc |
| DIFFRSTD_0002 | 16 RSTD < 24 | Tc |
|  |  | … |
| DIFFRSTD_1021 | 8168 RSTD < 8176 | Tc |
| DIFFRSTD_1022 | 8176 RSTD < 8184 | Tc |
| DIFFRSTD_1023 | 8184 RSTD | Tc |

Table 10.1.23.3.2-5: Report mapping for k=4

| Reported Quantity Value, DIFFRSTD_i | RSTD = RSTD1 RSTD2 | Unit |
| --- | --- | --- |
| DIFFRSTD_000 | 0 RSTD < 16 | Tc |
| DIFFRSTD_001 | 16 RSTD < 32 | Tc |
| DIFFRSTD_002 | 32 RSTD < 48 | Tc |
|  |  | … |
| DIFFRSTD_509 | 8144 RSTD < 8160 | Tc |
| DIFFRSTD_510 | 8160 RSTD < 8176 | Tc |
| DIFFRSTD_511 | 8176 RSTD | Tc |

Table 10.1.23.3.2-6: Report mapping for k=5

| Reported Quantity Value, DIFFRSTD_i | RSTD = RSTD1 RSTD2 | Unit |
| --- | --- | --- |
| DIFFRSTD_000 | 0 RSTD < 32 | Tc |
| DIFFRSTD_001 | 32 RSTD < 64 | Tc |
| DIFFRSTD_002 | 64 RSTD < 96 | Tc |
|  |  | … |
| DIFFRSTD_253 | 8096 RSTD < 8128 | Tc |
| DIFFRSTD_254 | 8128 RSTD < 8160 | Tc |
| DIFFRSTD_255 | 8160 RSTD | Tc |

##### 10.1.23.3.3	Additional Path Report Mapping for DL RSTD

The reporting range for the additional path reporting for an RSTD measurement is defined up to the range from -8175Tc to 8175Tc with the resolution step of 2kTc, where

Tc is defined in TS 38.211 [6],

kmin≤k≤kmax,

kmin=2 and kmax=5, when configured PRS resource of at least one of the reference cell and neighbor cell measured for the RSTD measurement is in FR1,

kmin=0 and kmax=5, when configured PRS resource of both the reference cell and neighbor cell measured for the RSTD measurement are in FR2,

k≥ timingReportingGranularityFactor [34] configured by LMF via LPP for the RSTD measurement.

The UE can report the timing of up to two additional paths with respect to the path timing determining the RSTD measurement.

A UE capable of  additionalPathsExtSupport-r17 can report the timing for a number additional paths, up to its capability, with respect to the path timing determining the RSTD measurement.

The report mappings for different k values are specified in Tables 10.1.23.3.3-1  10.1.23.3.3-6.

Table 10.1.23.3.3-1: Report mapping for k=0

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_00000 | path < -8175 | Tc |
| path_00001 | -8175 path < -8174 | Tc |
| path_00002 | -8174 path < -8173 | Tc |
|  |  | … |
| path_08175 | -1 path < 0 | Tc |
| path_08176 | 0 path < 1 | Tc |
| … | … | … |
| path_ 16349 | 8173 path < 8174 | Tc |
| path_ 16350 | 8174 path < 8175 | Tc |
| path_ 16351 | 8175 path | Tc |

Table 10.1.23.3.3-2: Report mapping for k=1

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_0000 | path < -8175 | Tc |
| path_0001 | -8175 path < -8173 | Tc |
| path_0002 | -8173 path < -8171 | Tc |
|  |  | … |
| path_4088 | -1 path < 1 | Tc |
| … | … | … |
| path_8174 | 8171 path < 8173 | Tc |
| path_8175 | 8173 path < 8175 | Tc |
| path_8176 | 8175 path | Tc |

Table 10.1.23.3.3-3: Report mapping for k=2

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_0000 | path < -8174 | Tc |
| path_0001 | -8174 path < -8170 | Tc |
| path_0002 | -8170 path < -8166 | Tc |
|  |  | … |
| path_2044 | -2 path < 2 | Tc |
| … | … | … |
| path_4086 | 8166 path < 8170 | Tc |
| path_4087 | 8170 path < 8174 | Tc |
| path_4088 | 8174 path | Tc |

Table 10.1.23.3.3-4: Report mapping for k=3

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_0000 | path < -8172 | Tc |
| path_0001 | -8172 path < -8164 | Tc |
| path_0002 | -8164 path < -8156 | Tc |
|  |  | … |
| path_1022 | -4 path < 4 | Tc |
| … | … | … |
| path_2042 | 8156 path < 8164 | Tc |
| path_2043 | 8164 path < 8172 | Tc |
| path_2044 | 8172 path | Tc |

Table 10.1.23.3.3-5: Report mapping for k=4

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_0000 | path < -8168 | Tc |
| path_0001 | -8168 path < -8152 | Tc |
| path_0002 | -8152 path < -8136 | Tc |
|  |  | … |
| path_511 | -8 path < 8 | Tc |
| … | … | … |
| path_1020 | 8136 path < 8152 | Tc |
| path_1021 | 8152 path < 8168 | Tc |
| path_1022 | 8168 path | Tc |

Table 10.1.23.3.3-6: Report mapping for k=5

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_000 | path < -8160 | Tc |
| path_001 | -8160 path < -8128 | Tc |
| path_002 | -8128 path < -8096 | Tc |
|  |  | … |
| path_256 | 0 path < 32 | Tc |
| … | … | … |
| path_509 | 8096 path < 8128 | Tc |
| path_510 | 8128 path < 8160 | Tc |
| path_511 | 8160 path | Tc |

### 10.1.24	PRS-RSRP Measurements

#### 10.1.24.1	Introduction

The requirements in Clause 10.1.24 shall apply, provided the UE has received nr-DL-TDOA-RequestLocationInformation or nr-Multi-RTT-RequestLocationInformation or nr-DL-AoD-RequestLocationInformation message from LMF via LPP [34] requesting the UE to report one or more DL PRS-RSRP measurements defined in TS 38.215 [4].

The requirements in clause 10.1.24 apply for UE in RRC_CONNECTED, including PRS-RSRP measurement with MG and outside MG, as well as for UE in RRC_INACTIVE. For PRS-RSRP measurement in FR2, the requirements apply with and without reduced Rx beam sweeping factor.

#### 10.1.24.2	Measurement Accuracy Requirements

##### 10.1.24.2.1	Absolute PRS RSRP accuracy

The absolute accuracy requirements for PRS-RSRP measurement for FR1 defined in Table 10.1.24.2.1-1 are valid under the following conditions:

-	Conditions defined in 38.101-1 Clause 7.3 for reference sensitivity are fulfilled.

-	PRP 1,2|dBm according to Annex B.2.14 for a corresponding Band

-	UE does not support positioning measurements with reduced number of samples, or LMF does not indicate UE to perform positioning measurements with reduced number of samples

The absolute accuracy requirements for PRS-RSRP measurement for FR2 defined in Table 10.1.24.2.1-2 are valid under the following conditions:

-	Conditions defined in 38.101-2 Clause 7.3 for reference sensitivity are fulfilled.

-	PRP 1,2|dBm according to Annex B.2.14 for a corresponding Band

-	UE does not support positioning measurements with reduced number of samples, or LMF does not indicate UE to perform positioning measurements with reduced number of samples

Table 10.1.24.2.1-1: PRS-RSRP absolute accuracy for FR1

| Accuracy |  | Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | PRS Ês/Iot | PRS BW | Repetition factor (TrepPRS*LPRS/KcombPRS) | Io Note 7 range |  |  |  |  |
|  |  |  |  |  | NR operating band groups Note 8 | Minimum Io Note 1 dBm / SCSPRS |  |  | Maximum Io |
| dB | dB | dB | PRB | - |  | dBm / SCSPRS |  |  | dBm/BWChannel |
|  |  |  |  |  |  | dBm/15kHz Note 6 | dBm/30kHz Note 6 | dBm/60kHz Note 6 |  |
| ±3.5 | ±8 | ≥-3dB | ≥24 | All | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -127 | -124 | -121 | -50 |
|  |  |  |  |  | NR_FDD_FR1_B | -126.5 | -123.5 | -120.5 | -50 |
|  |  |  |  |  | NR_TDD_FR1_C | -126 | -123 | -120 | -50 |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -125.5 | -122.5 | -119.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -125 | -122 | -119 | -50 |
|  |  |  |  |  | NR_FDD_FR1_F | -124.5 | -121.5 | -118.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_G, NR_TDD_FR1_G | -124 | -121 | -118 | -50 |
|  |  |  |  |  | NR_FDD_FR1_H | -123.5 | -120.5 | -117.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_N | -120.5 | -117.5 | -114.5 | -50 |
|  |  |  |  |  | Note 4 |  |  |  |  |
|  |  |  |  |  | Note 4 |  |  |  |  |
| ±8.5 | ±13 | ≥-13dB | 24 ≤ BW ≤ 52 | All | Note 4 |  |  |  |  |
| ±6 | ±10.5 |  | 52< BW≤ 104 | All | Note 4 |  |  |  |  |
| ±4.5 | ±9 |  | BW >104 | All | Note 4 |  |  |  |  |
| NOTE 1: This minimum Io condition is expressed as the average Io per RE over all REs in an OFDM symbol. NOTE 2: Void. NOTE 3: PRS bandwidth is as indicated in prs-Bandwidth in the OTDOA or DL-AoD assistance data defined in [34]. NOTE 4: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth ≥ 24 RB. NOTE 5: The serving cell, the reference cell, and the measured neighbour cell i are on the same carrier frequency. NOTE 6: The condition level is increased by ∆>0, when applicable, as described in Sections B.3.2 and B.3.3. NOTE 7: The Io is defined in PRS positioning subframes. The same Io range applies to PRS and non-PRS symbols. Io levels are different in PRS and non-PRS symbols within the same subframe. NOTE 8: NR operating band groups are as defined in Section 3.5.2. |  |  |  |  |  |  |  |  |  |

Table 10.1.24.2.1-2: PRS-RSRP absolute accuracy for FR2

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | PRS Ês/Iot | PRS BW | Repetition factor (TrepPRS*LPRS/KcombPRS) | Io Note 7 range |  |  |
|  |  |  |  |  | Minimum Io Note 1 dBm / SCSPRS |  | Maximum Io |
| dB | dB | dB | PRB | - | dBm / SCSPRS |  | dBm/BWChannel |
|  |  |  |  |  | dBm/120kHz Note 6 | dBm/60kHz Note 6 |  |
| ±5 | ±8 | ≥-3dB | ≥24 | All | Same value as PRP in Table B.2.14-2, according to UE Power class, operating band and angle of arrival |  | -50 |
|  |  |  |  |  | Note 4 |  |  |
|  |  |  |  |  | Note 4 |  |  |
| ±8.5 | ±11.5 | ≥-13dB | 24 ≤ BW ≤ 64 | All | Note 4 |  |  |
| ±6 | ±9 |  | BW >64 | All | Note 4 |  |  |
| NOTE 1: This minimum Io condition is expressed as the average Io per RE over all REs in an OFDM symbol. NOTE 2: Void. NOTE 3: PRS bandwidth is as indicated in prs-Bandwidth in the OTDOA or DL-AoD assistance data defined in [34]. NOTE 4: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth ≥ 24 RB. NOTE 5: The serving cell, the reference cell, and the measured neighbour cell i are on the same carrier frequency. NOTE 6: The condition level is increased by ∆>0, when applicable, as described in Sections B.3.2 and B.3.3. NOTE 7: The Io is defined in PRS positioning subframes. The same Io range applies to PRS and non-PRS symbols. Io levels are different in PRS and non-PRS symbols within the same subframe. NOTE 8: NR operating band groups are as defined in Section 3.5.2. |  |  |  |  |  |  |  |

The absolute accuracy requirements for PRS-RSRP measurement for FR1 defined in Table 10.1.24.2.1-3 are valid under the following conditions:

-	Conditions defined in 38.101-1 Clause 7.3 for reference sensitivity are fulfilled.

-	PRP 1,2|dBm according to Annex B.2.14 for a corresponding Band

-	UE supports positioning measurements with reduced number of samples, and LMF indicates UE to perform positioning measurements with reduced number of samples

-	AWGN channel

The absolute accuracy requirements for PRS-RSRP measurement for FR2 defined in Table 10.1.24.2.1-4 are valid under the following conditions:

-	Conditions defined in 38.101-2 Clause 7.3 for reference sensitivity are fulfilled.

-	PRP 1,2|dBm according to Annex B.2.14 for a corresponding Band

-	UE supports positioning measurements with reduced number of samples, and LMF indicates UE to perform positioning measurements with reduced number of samples

-	AWGN channel

Table 10.1.24.2.1-3: PRS-RSRP absolute accuracy for FR1 with reduced sample number

| Accuracy |  | Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | PRS Ês/Iot | PRS BW | Repetition factor (TrepPRS*LPRS/KcombPRS) | Io Note 6 range |  |  |  |  |  |
|  |  |  |  |  | NR operating band groups Note 7 | Minimum Io Note 1 dBm / SCSPRS |  |  |  | Maximum Io |
| dB | dB | dB | PRB | - |  | dBm / SCSPRS |  |  |  | dBm/BWChannel |
|  |  |  |  |  |  | dBm/15kHz Note 5 |  | dBm/30kHz Note 5 | dBm/60kHz Note 5 |  |
| ±3.5 | ±8 | ≥0 | ≥48 | All | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -127 | -124 |  | -121 | -50 |
|  |  |  |  |  | NR_FDD_FR1_B | -126.5 | -123.5 |  | -120.5 | -50 |
|  |  |  |  |  | NR_TDD_FR1_C | -126 | -123 |  | -120 | -50 |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -125.5 | -122.5 |  | -119.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -125 | -122 |  | -119 | -50 |
|  |  |  |  |  | NR_FDD_FR1_F | -124.5 | -121.5 |  | -118.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_G, NR_TDD_FR1_G | -124 | -121 |  | -118 | -50 |
|  |  |  |  |  | NR_FDD_FR1_H | -123.5 | -120.5 |  | -117.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_N | -120.5 | -117.5 |  | -114.5 | -50 |
| ±8.5 | ±13 | ≥-6 | 48 ≤ BW ≤ 52 | All | Note 3 |  |  |  |  |  |
| ±6 | ±10.5 |  | 52< BW≤ 104 | All | Note 3 |  |  |  |  |  |
| ±4.5 | ±9 |  | BW >104 | All | Note 3 |  |  |  |  |  |
| NOTE 1: This minimum Io condition is expressed as the average Io per RE over all REs in an OFDM symbol. NOTE 2: PRS bandwidth is as indicated in prs-Bandwidth in the OTDOA or DL-AoD assistance data defined in [34]. NOTE 3: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth ≥ 48 RB. NOTE 4: The serving cell, the reference cell, and the measured neighbour cell i are on the same carrier frequency. NOTE 5: The condition level is increased by ∆>0, when applicable, as described in Sections B.3.2 and B.3.3. NOTE 6: The Io is defined in PRS positioning subframes. The same Io range applies to PRS and non-PRS symbols. Io levels are different in PRS and non-PRS symbols within the same subframe. NOTE 7: NR operating band groups are as defined in Section 3.5.2. |  |  |  |  |  |  |  |  |  |  |

Table 10.1.24.2.1-4: PRS-RSRP absolute accuracy for FR2 with reduced sample number

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | PRS Ês/Iot | PRS BW | Repetition factor (TrepPRS*LPRS/KcombPRS) | Io Note 6 range |  |  |
|  |  |  |  |  | Minimum Io Note 1 dBm / SCSPRS |  | Maximum Io |
| dB | dB | dB | PRB | - | dBm / SCSPRS |  | dBm/BWChannel |
|  |  |  |  |  | dBm/120kHz Note 5 | dBm/60kHz Note 5 |  |
| ±5 | ±8 | ≥0 | ≥48 | All | Same value as PRP in Table B.2.14-2, according to UE Power class, operating band and angle of arrival |  | -50 |
| ±8.5 | ±11.5 | ≥-6 | 48 ≤ BW ≤ 64 | All | Note 3 |  |  |
| ±6 | ±9 |  | BW >64 | All | Note 3 |  |  |
| NOTE 1: This minimum Io condition is expressed as the average Io per RE over all REs in an OFDM symbol. NOTE 2: PRS bandwidth is as indicated in prs-Bandwidth in the OTDOA or DL-AoD assistance data defined in [34]. NOTE 3: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth ≥ 48 RB. NOTE 4: The serving cell, the reference cell, and the measured neighbour cell i are on the same carrier frequency. NOTE 5: The condition level is increased by ∆>0, when applicable, as described in Sections B.3.2 and B.3.3. NOTE 6: The Io is defined in PRS positioning subframes. The same Io range applies to PRS and non-PRS symbols. Io levels are different in PRS and non-PRS symbols within the same subframe. NOTE 7: NR operating band groups are as defined in Section 3.5.2. |  |  |  |  |  |  |  |

##### 10.1.24.2.2	Relative PRS RSRP accuracy

The relative accuracy of PRS-RSRP is defined as accuracy of the difference between two PRS-RSRP measurements.

The relative PRS-RSRP accuracy requirements apply for the cases when PRS-RSRP is measured from PRS resources in the same PRS resource set in FR1 or FR2, and measured with same Rx beam in case of FR2.

The accuracy requirements for PRS-RSRP measurement for FR1 defined in Table 10.1.24.2.2-1 are valid under the following conditions:

-	Conditions defined in 38.101-1 Clause 7.3 for reference sensitivity are fulfilled.

-	PRP 1,2|dBm according to Annex B.2.14 for a corresponding Band

-	UE does not support positioning measurements with reduced number of samples, or LMF does not indicate UE to perform positioning measurements with reduced number of samples

The accuracy requirements for PRS-RSRP measurement for FR2 defined in Table 10.1.24.2.2-2 are valid under the following conditions:

-	Conditions defined in 38.101-2 Clause 7.3 for reference sensitivity are fulfilled.

-	PRP 1,2|dBm according to Annex B.2.14 for a corresponding Band

-	UE does not support positioning measurements with reduced number of samples, or LMF does not indicate UE to perform positioning measurements with reduced number of samples

Table 10.1.24.2.2-1: PRS-RSRP relative accuracy for FR1

| Accuracy |  | Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | PRS Ês/Iot | PRS BW | Repetition factor (TrepPRS*LPRS/KcombPRS) | Io Note 7 range |  |  |  |  |
|  |  |  |  |  | NR operating band groups Note 8 | Minimum Io Note 1 dBm / SCSPRS |  |  | Maximum Io |
| dB | dB | dB | PRB | - |  | dBm / SCSPRS |  |  | dBm/BWChannel |
|  |  |  |  |  |  | dBm/15kHz Note 6 | dBm/30kHz Note 6 | dBm/60kHz Note 6 |  |
| ±3.5 | ±5.0 | ≥-3dB | ≥24 | All | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -127 | -124 | -121 | -50 |
|  |  |  |  |  | NR_FDD_FR1_B | -126.5 | -123.5 | -120.5 | -50 |
|  |  |  |  |  | NR_TDD_FR1_C | -126 | -123 | -120 | -50 |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -125.5 | -122.5 | -119.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -125 | -122 | -119 | -50 |
|  |  |  |  |  | NR_FDD_FR1_F | -124.5 | -121.5 | -118.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_G, NR_TDD_FR1_G | -124 | -121 | -118 | -50 |
|  |  |  |  |  | NR_FDD_FR1_H | -123.5 | -120.5 | -117.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_N | -120.5 | -117.5 | -114.5 | -50 |
|  |  |  |  |  | Note 4 |  |  |  |  |
|  |  |  |  |  | Note 4 |  |  |  |  |
| ±9.5 | ±11.0 | ≥-13dB | 24 ≤ BW ≤ 52 | All | Note 4 |  |  |  |  |
| ±6.5 | ±8.0 |  | 52< BW≤ 104 | All | Note 4 |  |  |  |  |
| ±5.0 | ±6.5 |  | BW >104 | All | Note 4 |  |  |  |  |
| NOTE 1: This minimum Io condition is expressed as the average Io per RE over all REs in an OFDM symbol. NOTE 2: Void. NOTE 3: PRS bandwidth is as indicated in prs-Bandwidth in the OTDOA or DL-AoD assistance data defined in [34]. NOTE 4: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth ≥ 24 RB. NOTE 5: The serving cell, the reference cell, and the measured neighbour cell i are on the same carrier frequency. NOTE 6: The condition level is increased by ∆>0, when applicable, as described in Sections B.3.2 and B.3.3. NOTE 7: The Io is defined in PRS positioning subframes. The same Io range applies to PRS and non-PRS symbols. Io levels are different in PRS and non-PRS symbols within the same subframe. NOTE 8: NR operating band groups are as defined in Section 3.5.2. |  |  |  |  |  |  |  |  |  |

Table 10.1.24.2.2-2: PRS-RSRP relative accuracy for FR2

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | PRS Ês/Iot | PRS BW | Repetition factor (TrepPRS*LPRS/KcombPRS) | Io Note 7 range |  |  |
|  |  |  |  |  | Minimum Io Note 1 dBm / SCSPRS |  | Maximum Io |
| dB | dB | dB | PRB | - | dBm / SCSPRS |  | dBm/BWChannel |
|  |  |  |  |  | dBm/120kHz Note 6 | dBm/60kHz Note 6 |  |
| ±5.0 | ±8.0 | ≥-3dB | ≥24 | All | Same value as PRP in Table B.2.14-2, according to UE Power class, operating band and angle of arrival |  | -50 |
|  |  |  |  |  | Note 4 |  |  |
|  |  |  |  |  | Note 4 |  |  |
| ±10 | ±13 | ≥-13dB | 24 ≤ BW ≤ 64 | All | Note 4 |  |  |
| ±7.5 | ±10.5 |  | BW >64 | All | Note 4 |  |  |
| NOTE 1: This minimum Io condition is expressed as the average Io per RE over all REs in an OFDM symbol. NOTE 2: Void. NOTE 3: PRS bandwidth is as indicated in prs-Bandwidth in the OTDOA or DL-AoD assistance data defined in [34]. NOTE 4: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth ≥ 24 RB. NOTE 5: The serving cell, the reference cell, and the measured neighbour cell i are on the same carrier frequency. NOTE 6: The condition level is increased by ∆>0, when applicable, as described in Sections B.3.2 and B.3.3. NOTE 7: The Io is defined in PRS positioning subframes. The same Io range applies to PRS and non-PRS symbols. Io levels are different in PRS and non-PRS symbols within the same subframe. NOTE 8: NR operating band groups are as defined in Section 3.5.2. |  |  |  |  |  |  |  |

The absolute accuracy requirements for PRS-RSRP measurement for FR1 defined in Table 10.1.24.2.2-3 are valid under the following conditions:

-	Conditions defined in 38.101-1 Clause 7.3 for reference sensitivity are fulfilled.

-	PRP 1,2|dBm according to Annex B.2.14 for a corresponding Band

-	UE supports positioning measurements with reduced number of samples, and LMF indicates UE to perform positioning measurements with reduced number of samples

-	AWGN channel

The absolute accuracy requirements for PRS-RSRP measurement for FR2 defined in Table 10.1.24.2.2-4 are valid under the following conditions:

-	Conditions defined in 38.101-2 Clause 7.3 for reference sensitivity are fulfilled.

-	PRP 1,2|dBm according to Annex B.2.14 for a corresponding Band

-	UE supports positioning measurements with reduced number of samples, and LMF indicates UE to perform positioning measurements with reduced number of samples

-	AWGN channel

Table 10.1.24.2.2-3: PRS-RSRP relative accuracy for FR1 with reduced sample number

| Accuracy |  | Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | PRS Ês/Iot | PRS BW | Repetition factor (TrepPRS*LPRS/KcombPRS) | Io Note 6 range |  |  |  |  |
|  |  |  |  |  | NR operating band groups Note 7 | Minimum Io Note 1 dBm / SCSPRS |  |  | Maximum Io |
| dB | dB | dB | PRB | - |  | dBm / SCSPRS |  |  | dBm/BWChannel |
|  |  |  |  |  |  | dBm/15kHz Note 5 | dBm/30kHz Note 5 | dBm/60kHz Note 5 |  |
| ±3.5 | ±5.0 | ≥0 | ≥48 | All | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -127 | -124 | -121 | -50 |
|  |  |  |  |  | NR_FDD_FR1_B | -126.5 | -123.5 | -120.5 | -50 |
|  |  |  |  |  | NR_TDD_FR1_C | -126 | -123 | -120 | -50 |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -125.5 | -122.5 | -119.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -125 | -122 | -119 | -50 |
|  |  |  |  |  | NR_FDD_FR1_F | -124.5 | -121.5 | -118.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_G, NR_TDD_FR1_G | -124 | -121 | -118 | -50 |
|  |  |  |  |  | NR_FDD_FR1_H | -123.5 | -120.5 | -117.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_N | -120.5 | -117.5 | -114.5 | -50 |
| ±9.5 | ±11.0 | ≥-6 | 48 ≤ BW ≤ 52 | All | Note 3 |  |  |  |  |
| ±6.5 | ±8.0 |  | 52< BW≤ 104 | All | Note 3 |  |  |  |  |
| ±5.0 | ±6.5 |  | BW >104 | All | Note 3 |  |  |  |  |
| NOTE 1: This minimum Io condition is expressed as the average Io per RE over all REs in an OFDM symbol. NOTE 2: PRS bandwidth is as indicated in prs-Bandwidth in the OTDOA or DL-AoD assistance data defined in [34]. NOTE 3: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth ≥ 48 RB. NOTE 4: The serving cell, the reference cell, and the measured neighbour cell i are on the same carrier frequency. NOTE 5: The condition level is increased by ∆>0, when applicable, as described in Sections B.3.2 and B.3.3. NOTE 6: The Io is defined in PRS positioning subframes. The same Io range applies to PRS and non-PRS symbols. Io levels are different in PRS and non-PRS symbols within the same subframe. NOTE 7: NR operating band groups are as defined in Section 3.5.2. |  |  |  |  |  |  |  |  |  |

Table 10.1.24.2.2-4: PRS-RSRP relative accuracy for FR2 with reduced sample number

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | PRS Ês/Iot | PRS BW | Repetition factor (TrepPRS*LPRS/KcombPRS) | Io Note 6 range |  |  |
|  |  |  |  |  | Minimum Io Note 1 dBm / SCSPRS |  | Maximum Io |
| dB | dB | dB | PRB | - | dBm / SCSPRS |  | dBm/BWChannel |
|  |  |  |  |  | dBm/120kHz Note 5 | dBm/60kHz Note 5 |  |
| ±5.0 | ±8.0 | ≥0 | ≥48 | All | Same value as PRP in Table B.2.14-2, according to UE Power class, operating band and angle of arrival |  | -50 |
| ±10 | ±13 | ≥-6 | 48 ≤ BW ≤ 64 | All | Note 3 |  |  |
| ±7.5 | ±10.5 |  | BW >64 | All | Note 3 |  |  |
| NOTE 1: This minimum Io condition is expressed as the average Io per RE over all REs in an OFDM symbol. NOTE 2: PRS bandwidth is as indicated in prs-Bandwidth in the OTDOA or DL-AoD assistance data defined in [34]. NOTE 3: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth ≥ 24 RB. NOTE 4: The serving cell, the reference cell, and the measured neighbour cell i are on the same carrier frequency. NOTE 5: The condition level is increased by ∆>0, when applicable, as described in Sections B.3.2 and B.3.3. NOTE 6: The Io is defined in PRS positioning subframes. The same Io range applies to PRS and non-PRS symbols. Io levels are different in PRS and non-PRS symbols within the same subframe. NOTE 8: NR operating band groups are as defined in Section 3.5.2. |  |  |  |  |  |  |  |

#### 10.1.24.3	Report mapping

##### 10.1.24.3.1	Absolute PRS-RSRP Measurement Report Mapping

The reporting range of absolute PRS-RSRP measurement is defined from -156 dBm to -31 dBm with 1 dB resolution.

The mapping of measured quantity is defined in Table 10.1.24.3.1-1. The range in the signalling may be larger than the guaranteed accuracy range.

Table 10.1.24.3.1-1: Measurement report mapping for PRS-RSRP

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| PRS_RSRP_0 | PRS-RSRP<-156 | dBm |
| PRS_RSRP_1 | -156≤PRS-RSRP<-155 | dBm |
| PRS_RSRP_2 | -155≤PRS-RSRP<-154 | dBm |
| PRS_RSRP_3 | -154≤PRS-RSRP<-153 | dBm |
| PRS_RSRP_4 | -153≤PRS-RSRP<-152 | dBm |
| PRS_RSRP_5 | -152≤PRS-RSRP<-151 | dBm |
| PRS_RSRP_6 | -151≤PRS-RSRP<-150 | dBm |
| PRS_RSRP_7 | -150≤PRS-RSRP<-149 | dBm |
| PRS_RSRP_8 | -149≤PRS-RSRP<-148 | dBm |
| PRS_RSRP_9 | -148≤PRS-RSRP<-147 | dBm |
| PRS_RSRP_10 | -147≤PRS-RSRP<-146 | dBm |
| PRS_RSRP_11 | -146≤PRS-RSRP<-145 | dBm |
| PRS_RSRP_12 | -145≤PRS-RSRP<-144 | dBm |
| PRS_RSRP_13 | -144≤PRS-RSRP<-143 | dBm |
| PRS_RSRP_14 | -143≤PRS-RSRP<-142 | dBm |
| PRS_RSRP_15 | -142≤PRS-RSRP<-141 | dBm |
| PRS_RSRP_16 | -141≤PRS-RSRP<-140 | dBm |
| PRS_RSRP_17 | -140≤PRS-RSRP<-139 | dBm |
| PRS_RSRP_18 | -139≤PRS-RSRP<-138 | dBm |
| … | … | … |
| PRS_RSRP_111 | -46≤PRS-RSRP<-45 | dBm |
| PRS_RSRP_112 | -45≤PRS-RSRP<-44 | dBm |
| PRS_RSRP_113 | -44≤PRS-RSRP<-43 | dBm |
| PRS_RSRP_114 | -43≤PRS-RSRP<-42 | dBm |
| PRS_RSRP_115 | -42≤PRS-RSRP<-41 | dBm |
| PRS_RSRP_116 | -41≤PRS-RSRP<-40 | dBm |
| PRS_RSRP_117 | -40≤PRS-RSRP<-39 | dBm |
| PRS_RSRP_118 | -39≤PRS-RSRP<-38 | dBm |
| PRS_RSRP_119 | -38≤PRS-RSRP<-37 | dBm |
| PRS_RSRP_120 | -37≤PRS-RSRP<-36 | dBm |
| PRS_RSRP_121 | -36≤PRS-RSRP<-35 | dBm |
| PRS_RSRP_122 | -35≤PRS-RSRP<-34 | dBm |
| PRS_RSRP_123 | -34≤PRS-RSRP<-33 | dBm |
| PRS_RSRP_124 | -33≤PRS-RSRP<-32 | dBm |
| PRS_RSRP_125 | -32≤PRS-RSRP<-31 | dBm |
| PRS_RSRP_126 | -31≤PRS-RSRP | dBm |

##### 10.1.24.3.2	Differential Report Mapping for PRS-RSRP Measurement

The reporting range of differential PRS-RSRP is defined from -30 dB to 0 dB with 1 dB resolution when nr-DL-AoD-RequestLocationInformation message is received.

The mapping of measured quantity is defined in Table 10.1.24.3.2-1. The range in the signalling may be larger than the guaranteed accuracy range.

The reporting range of differential PRS-RSRP is defined from -30 dB to 30 dB with 1 dB resolution when nr-DL-TDOA-RequestLocationInformation or nr-Multi-RTT-RequestLocationInformation is received.

The mapping of measured quantity is defined in Table 10.1.24.3.2-2. The range in the signalling may be larger than the guaranteed accuracy range or the range supported by the UE receiver for differentail RSRP measured on different PRS resources in frequency domain at the same time.

Table 10.1.24.3.2-1: Measurement report mapping for differential PRS-RSRP

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| DIFFRSRP_0 | -30≥ΔRSRP | dB |
| DIFFRSRP_1 | -29≥ΔRSRP>-30 | dB |
| DIFFRSRP_2 | -28≥ΔRSRP>-29 | dB |
| DIFFRSRP_3 | -27≥ΔRSRP>-28 | dB |
| DIFFRSRP_4 | -26≥ΔRSRP>-27 | dB |
| DIFFRSRP_5 | -25≥ΔRSRP>-26 | dB |
| DIFFRSRP_6 | -24≥ΔRSRP>-25 | dB |
| DIFFRSRP_7 | -23≥ΔRSRP>-24 | dB |
| DIFFRSRP_8 | -22≥ΔRSRP>-23 | dB |
| DIFFRSRP_9 | -21≥ΔRSRP>-22 | dB |
| DIFFRSRP_10 | -20≥ΔRSRP>-21 | dB |
| DIFFRSRP_11 | -19≥ΔRSRP>-20 | dB |
| DIFFRSRP_12 | -18≥ΔRSRP>-19 | dB |
| DIFFRSRP_13 | -17≥ΔRSRP>-18 | dB |
| DIFFRSRP_14 | -16≥ΔRSRP>-17 | dB |
| DIFFRSRP_15 | -15≥ΔRSRP>-16 | dB |
| DIFFRSRP_16 | -14≥ΔRSRP>-15 | dB |
| DIFFRSRP_17 | -13≥ΔRSRP>-14 | dB |
| DIFFRSRP_18 | -12≥ΔRSRP>-13 | dB |
| DIFFRSRP_19 | -11≥ΔRSRP>-12 | dB |
| DIFFRSRP_20 | -10≥ΔRSRP>-11 | dB |
| DIFFRSRP_21 | -9≥ΔRSRP>-10 | dB |
| DIFFRSRP_22 | -8≥ΔRSRP>-9 | dB |
| DIFFRSRP_23 | -7≥ΔRSRP>-8 | dB |
| DIFFRSRP_24 | -6≥ΔRSRP>-7 | dB |
| DIFFRSRP_25 | -5≥ΔRSRP>-6 | dB |
| DIFFRSRP_26 | -4≥ΔRSRP>-5 | dB |
| DIFFRSRP_27 | -3≥ΔRSRP>-4 | dB |
| DIFFRSRP_28 | -2≥ΔRSRP>-3 | dB |
| DIFFRSRP_29 | -1≥ΔRSRP>-2 | dB |
| DIFFRSRP_30 | 0≥ΔRSRP>-1 | dB |

Table 10.1.24.3.2-2: Measurement report mapping for differential PRS-RSRP

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| DIFFRSRP_0 | -30≥ΔRSRP | dB |
| DIFFRSRP_1 | -29≥ΔRSRP>-30 | dB |
| DIFFRSRP_2 | -28≥ΔRSRP>-29 | dB |
| DIFFRSRP_3 | -27≥ΔRSRP>-28 | dB |
| DIFFRSRP_4 | -26≥ΔRSRP>-27 | dB |
| DIFFRSRP_5 | -25≥ΔRSRP>-26 | dB |
| DIFFRSRP_6 | -24≥ΔRSRP>-25 | dB |
| DIFFRSRP_7 | -23≥ΔRSRP>-24 | dB |
| DIFFRSRP_8 | -22≥ΔRSRP>-23 | dB |
| DIFFRSRP_9 | -21≥ΔRSRP>-22 | dB |
| DIFFRSRP_10 | -20≥ΔRSRP>-21 | dB |
| DIFFRSRP_11 | -19≥ΔRSRP>-20 | dB |
| DIFFRSRP_12 | -18≥ΔRSRP>-19 | dB |
| DIFFRSRP_13 | -17≥ΔRSRP>-18 | dB |
| DIFFRSRP_14 | -16≥ΔRSRP>-17 | dB |
| … | … | … |
| DIFFRSRP_25 | -5≥ΔRSRP>-6 | dB |
| DIFFRSRP_26 | -4≥ΔRSRP>-5 | dB |
| DIFFRSRP_27 | -3≥ΔRSRP>-4 | dB |
| DIFFRSRP_28 | -2≥ΔRSRP>-3 | dB |
| DIFFRSRP_29 | -1≥ΔRSRP>-2 | dB |
| DIFFRSRP_30 | 0≥ΔRSRP>-1 | dB |
| DIFFRSRP_31 | 1≥ΔRSRP>0 | dB |
| DIFFRSRP_32 | 2≥ΔRSRP>1 | dB |
| DIFFRSRP_33 | 3≥ΔRSRP>2 | dB |
| DIFFRSRP_34 | 4≥ΔRSRP>3 | dB |
| DIFFRSRP_35 | 5≥ΔRSRP>4 | dB |
| DIFFRSRP_36 | 6≥ΔRSRP>5 | dB |
| … | … | … |
| DIFFRSRP_47 | 17≥ΔRSRP>16 | dB |
| DIFFRSRP_48 | 18≥ΔRSRP>17 | dB |
| DIFFRSRP_49 | 19≥ΔRSRP>18 | dB |
| DIFFRSRP_50 | 20≥ΔRSRP>19 | dB |
| DIFFRSRP_51 | 21≥ΔRSRP>20 | dB |
| DIFFRSRP_52 | 22≥ΔRSRP>21 | dB |
| DIFFRSRP_53 | 23≥ΔRSRP>-22 | dB |
| DIFFRSRP_54 | 24≥ΔRSRP>23 | dB |
| DIFFRSRP_55 | 25≥ΔRSRP>24 | dB |
| DIFFRSRP_56 | 26≥ΔRSRP>25 | dB |
| DIFFRSRP_57 | 27≥ΔRSRP>26 | dB |
| DIFFRSRP_58 | 28≥ΔRSRP>27 | dB |
| DIFFRSRP_59 | 29≥ΔRSRP>28 | dB |
| DIFFRSRP_60 | 30≥ΔRSRP>29 | dB |
| DIFFRSRP_61 | ΔRSRP>30 | dB |

### 10.1.25	UE Rx-Tx Time Difference Measurements

#### 10.1.25.1	Introduction

The requirements in Clause 10.1.25 shall apply, provided the UE has received nr-Multi-RTT-RequestLocationInformation message from LMF via LPP [31] requesting the UE to report one or more UE Rx-Tx time difference measurements defined in TS 38.215 [4]. The requirements in Clause 10.1.25 shall apply:

-	when UE is in RRC_CONNECTED state and the measurement is performed with MG or without MG,

-	when UE is in RRC_INACTIVE state.

#### 10.1.25.2	Measurement Accuracy Requirements

The UE Rx-Tx time difference measurement accuracy requirements in this clause shall not apply, if:

NTA_offset defined in Table 7.1.2-2 changes during the UE Rx-Tx measurement period or

if the uplink transmission timing changes during the UE Rx-Tx measurement period due to the network-configured Timing Advance.

The UE Rx-Tx time difference measurement accuracy requirements in this clause shall apply provided that:

-	The UE transmits SRS within [-160, 160] msec of at least one DL PRS resource of each of the TRPs in the assistance data.

If the uplink transmission timing changes during the UE Rx-Tx measurement period due to the autonomous timing adjustment defined in clause 7.1.2 then:

-	UE Rx-Tx measurement accuracy requirements shall apply for a cell, which is also the downlink reference cell (defined in section 7.1.1) for SRS transmission even if the uplink transmission timing changes during the UE Rx-Tx measurement period due to autonomous adjustment.

-	UE Rx-Tx measurement accuracy requirements shall not apply for a cell, which is not the downlink reference cell (defined in section 7.1.1) for SRS transmission, if the uplink transmission timing changes during the UE Rx-Tx measurement period due to autonomous adjustment.

When a serving cell change occurs during the UE Rx-Tx measurement period, the UE Rx-Tx time difference measurement accuracy requirements in this clause shall apply provided that the serving cell change does not impact SRS configuration for the UE Rx-Tx measurement.

The relative accuracy of UE Rx-Tx measurement in this clause is defined as accuracy of the difference between two UE Rx-Tx measurements.

The accuracy requirements in Table 10.1.25.2-1 for FR1 are valid under the following conditions:

Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

PRP|dBm according to Annex B.2.14 for a corresponding Band.

AWGN propagation condition.

Table 10.1.25.2-1: UE Rx-Tx time difference measurement accuracy in FR1 in AWGN

| Accuracy | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | PRS Ês/Iot | Minimum PRS bandwidth | PRS SCS | PRS resource repetition (TrepPRS*LPRS/KcombPRSNote 3 | NR operating band groupsNote 2 | IoNote 4 range |  |
|  |  |  |  |  |  | Minimum IoNote 1 | Maximum Io |
| TcNote 5 | dB | RB | kHz |  |  | dBm / SCSPRS | dBm/BW |
| ± 78+ | -3 | ≥24 | 15 | ≥4 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -127 | -50 |
|  |  |  |  |  | NR_FDD_FR1_B | -126.5 |  |
|  |  |  |  |  | NR_TDD_FR1_C | -126 |  |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -125.5 |  |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -125 |  |
|  |  |  |  |  | NR_FDD_FR1_F | -124.5 |  |
|  |  |  |  |  | NR_FDD_FR1_G, NR_TDD_FR1_G | -124 |  |
|  |  |  |  |  | NR_FDD_FR1_H | -123.5 |  |
|  |  |  |  |  | NR_FDD_FR1_N | -120.5 |  |
| ± 59+ |  | ≥52 |  | ≥1 | Note 6 | Note 6 | Note 6 |
| ± 30+ |  | >104 |  | ≥1 | Note 6 | Note 6 | Note 6 |
| ± 57+ |  | ≥24 | 30 | ≥4 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -124 | -50 |
|  |  |  |  |  | NR_FDD_FR1_B | -123.5 |  |
|  |  |  |  |  | NR_TDD_FR1_C | -123 |  |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -122.5 |  |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -122 |  |
|  |  |  |  |  | NR_FDD_FR1_F | -121.5 |  |
|  |  |  |  |  | NR_FDD_FR1_G, NR_TDD_FR1_G | -121 |  |
|  |  |  |  |  | NR_FDD_FR1_H | -120.5 |  |
|  |  |  |  |  | NR_FDD_FR1_N | -117.5 |  |
| ± 30+ |  | ≥48 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 15+ |  | ≥132 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 29+ |  | ≥24 | 60 | ≥4 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -50 |
|  |  |  |  |  | NR_FDD_FR1_B | -120.5 |  |
|  |  |  |  |  | NR_TDD_FR1_C | -120 |  |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 |  |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 |  |
|  |  |  |  |  | NR_FDD_FR1_F | -118.5 |  |
|  |  |  |  |  | NR_FDD_FR1_G, NR_TDD_FR1_G | -118 |  |
|  |  |  |  |  | NR_FDD_FR1_H | -117.5 |  |
|  |  |  |  |  | NR_FDD_FR1_N | -114.5 |  |
| ± 15+ |  | ≥ 64 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 7+ |  | ≥ 132 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 101+ | -13 | ≥24 | 15 | ≥4 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 75+ |  | ≥52 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 37+ |  | >104 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 58+ |  | ≥24 | 30 | ≥4 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 39+ |  | ≥48 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 16+ |  | ≥132 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 36+ |  | ≥24 | 60 | ≥4 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 16+ |  | ≥ 64 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 8+ |  | ≥ 132 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| NOTE 1: This minimum Io condition is expressed as the average Io per RE over all REs in an OFDM symbol. NOTE 2: NR operating band groups are as defined in Section 3.5. NOTE 3: TrepPRS, LPRS ,KcombPRS are configured by higher layer parameter dl-PRS-ResourceRepetitionFactor, dl-PRS-NumSymbols and dl-PRS-CombSizeN defined in TS 37.355 [34]. NOTE 4: The Io is defined in PRS slots. The same Io range applies to PRS and non-PRS symbols. Io levels are different in PRS and non-PRS symbols within the same slot. NOTE 5: Tc is the basic timing unit defined in TS 38.211 [6]. NOTE 6: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth of the smallest RB number for the corresponding SCS. NOTE 7: is the margin determined from Table 10.1.25.2-5. |  |  |  |  |  |  |  |

The accuracy requirements in Table 10.1.25.2-1a for FR1 are valid under the following conditions:

Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

PRP|dBm according to Annex B.2.14 for a corresponding Band.

Number of measurement samples is less than 4

AWGN propagation condition.

Table 10.1.25.2-1a: UE Rx-Tx time difference measurement accuracy in FR1 in AWGN with reduced measurement samples

| Accuracy | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | PRS Ês/Iot | Minimum PRS bandwidth | PRS SCS | PRS resource repetition (TrepPRS*LPRS/KcombPRSNote 3 | NR operating band groupsNote 2 | IoNote 4 range |  |
|  |  |  |  |  |  | Minimum IoNote 1 | Maximum Io |
| TcNote 5 | dB | RB | kHz |  |  | dBm / SCSPRS | dBm/BW |
| ± 59+ | 0 | ≥52 | 15 | ≥1 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -127 | -50 |
|  |  |  |  |  | NR_FDD_FR1_B | -126.5 | -50 |
|  |  |  |  |  | NR_TDD_FR1_C | -126 | -50 |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -125.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -125 | -50 |
|  |  |  |  |  | NR_FDD_FR1_F | -124.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_G, NR_TDD_FR1_G | -124 | -50 |
|  |  |  |  |  | NR_FDD_FR1_H | -123.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_N | -120.5 | -50 |
| ± 30+6 |  | >104 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 30+ |  | ≥48 | 30 | ≥1 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -124 | -50 |
|  |  |  |  |  | NR_FDD_FR1_B | -123.5 | -50 |
|  |  |  |  |  | NR_TDD_FR1_C | -123 | -50 |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -122.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -122 | -50 |
|  |  |  |  |  | NR_FDD_FR1_F | -121.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_G, NR_TDD_FR1_G | -121 | -50 |
|  |  |  |  |  | NR_FDD_FR1_H | -120.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_N | -117.5 | -50 |
| ± 15+ |  | ≥132 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 15+ |  | ≥64 | 60 | ≥1 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -50 |
|  |  |  |  |  | NR_FDD_FR1_B | -120.5 | -50 |
|  |  |  |  |  | NR_TDD_FR1_C | -120 | -50 |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -50 |
|  |  |  |  |  | NR_FDD_FR1_F | -118.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_G, NR_TDD_FR1_G | -118 | -50 |
|  |  |  |  |  | NR_FDD_FR1_H | -117.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_N | -114.5 | -50 |
| ± 7+ |  | ≥132 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 75+ | -6 | ≥52 | 15 | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 37+ |  | >104 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 39+ |  | ≥48 | 30 | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 16+ |  | ≥132 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 16+ |  | ≥64 | 60 | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 8+ |  | ≥132 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| NOTE 1: This minimum Io condition is expressed as the average Io per RE over all REs in an OFDM symbol. NOTE 2: NR operating band groups are as defined in Section 3.5. NOTE 3: TrepPRS, LPRS ,KcombPRS are configured by higher layer parameter dl-PRS-ResourceRepetitionFactor, dl-PRS-NumSymbols and dl-PRS-CombSizeN defined in TS 37.355 [34]. NOTE 4: The Io is defined in PRS slots. The same Io range applies to PRS and non-PRS symbols. Io levels are different in PRS and non-PRS symbols within the same slot. NOTE 5: Tc is the basic timing unit defined in TS 38.211 [6]. NOTE 6: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth of the smallest RB number for the corresponding SCS. NOTE 7: is the margin determined from Table 10.1.25.2-5. |  |  |  |  |  |  |  |

The relative accuracy requirements in Table 10.1.25.2-1b for FR1 are valid under the following conditions:

Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

PRP|dBm according to Annex B.2.14 for a corresponding Band.

AWGN propagation condition.

the two UE Rx-Tx time difference measurements are associated with the same RxTx TEG

Table 10.1.25.2-1b: UE Rx-Tx time difference relative measurement accuracy in FR1 in AWGN with TEG reporting

| Accuracy | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | PRS Ês/Iot | PRS SCS | PRS bandwidth Note 1 | PRS resource repetition (TrepPRS*LPRS/KcombPRS) Note 2 | Io Note 3 range |  |  |
|  |  |  |  |  | NR operating band groups Note 4 | Minimum Io | Maximum Io |
| Tc Note 5 | dB | kHz | RB |  |  | dBm/SCS | dBm/BWChannel |
| 132 +ΔNote 7 | (PRS Ês/Iot)j ≥-6dB (PRS Ês/Iot)i ≥-13dB | 15 | ≥ 24 | ≥ 4 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -127 | -50 |
|  |  |  |  |  | NR_FDD_FR1_B | -126.5 | -50 |
|  |  |  |  |  | NR_TDD_FR1_C | -126 | -50 |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -125.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -125 | -50 |
|  |  |  |  |  | NR_FDD_FR1_F | -124.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_G, NR_TDD_FR1_G | -124 | -50 |
|  |  |  |  |  | NR_FDD_FR1_H | -123.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_N | -120.5 | -50 |
| 98 +Δ |  |  | ≥ 52 | ≥ 1 | Note 6 | Note 6 | Note 6 |
| 42 +Δ |  |  | ≥ 104 | ≥ 1 | Note 6 | Note 6 | Note 6 |
| 75 +Δ |  | 30 | ≥ 24 | ≥ 4 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -124 | -50 |
|  |  |  |  |  | NR_FDD_FR1_B | -123.5 | -50 |
|  |  |  |  |  | NR_TDD_FR1_C | -123 | -50 |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -122.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -122 | -50 |
|  |  |  |  |  | NR_FDD_FR1_F | -121.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_G, NR_TDD_FR1_G | -121 | -50 |
|  |  |  |  |  | NR_FDD_FR1_H | -120.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_N | -117.5 | -50 |
| 48 +Δ |  |  | ≥ 48 | ≥ 1 | Note 6 | Note 6 | Note 6 |
| 24 +Δ |  |  | ≥ 132 | ≥ 1 | Note 6 | Note 6 | Note 6 |
| 50 +Δ |  | 60 | ≥24 | ≥ 4 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -50 |
|  |  |  |  |  | NR_FDD_FR1_B | -120.5 | -50 |
|  |  |  |  |  | NR_TDD_FR1_C | -120 | -50 |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -50 |
|  |  |  |  |  | NR_FDD_FR1_F | -118.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_G, NR_TDD_FR1_G | -118 | -50 |
|  |  |  |  |  | NR_FDD_FR1_H | -117.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_N | -114.5 | -50 |
| 24 +Δ |  |  | ≥ 64 | ≥ 1 | Note 6 | Note 6 | Note 6 |
| 10 +Δ |  |  | ≥ 132 | ≥ 1 | Note 6 | Note 6 | Note 6 |
| NOTE 1: Minimum PRS bandwidth, which is minimum of the PRS bandwidths of resource j and resource i. NOTE 2: Minimum number of PRS resource repetitions among resource j and resource i. TrepPRS, LPRS ,KcombPRS are configured by higher layer parameter dl-PRS-ResourceRepetitionFactor, dl-PRS-NumSymbols and dl-PRS-CombSizeN defined in TS 37.355 [34], respectively. NOTE 3: Io is assumed to have constant EPRE across the bandwidth. NOTE 4: NR operating band groups in FR1 are as defined in clause 3.5.2. NOTE 5: Tc is the basic timing unit defined in TS 38.211 [6]. NOTE 6: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth of the smallest RB number for the corresponding SCS. NOTE 7: Δis the value of the timing error margin for the RxTx TEG, reported via nr-UE-RxTxTEG-TimingErrorMargin. Δ cannot be larger than the sum of the margins in table 10.1.25.2-5 (dependent on PRS/SRS BW) for any pair of individual UE Rx-Tx time difference measurements associated with the RxTx TEG. . |  |  |  |  |  |  |  |

The accuracy requirements in Table 10.1.25.2-2 for FR1 are valid under the following conditions:

Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

PRP|dBm according to Annex B.2.14 for a corresponding Band.

Fading propagation condition.

Table 10.1.25.2-2: UE Rx-Tx time difference measurement accuracy in FR1 in fading

| Accuracy | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | PRS Ês/Iot | Minimum PRS bandwidth | PRS SCS | PRS resource repetition (TrepPRS*LPRS/KcombPRSNote 3 | NR operating band groupsNote 2 | IoNote 4 range |  |
|  |  |  |  |  |  | Minimum IoNote 1 | Maximum Io |
| TcNote 5 | dB | RB | kHz |  |  | dBm / SCSPRS | dBm/BW |
| ± 137+ | -3 | ≥24 | 15 | ≥4 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -127 | -50 |
|  |  |  |  |  | NR_FDD_FR1_B | -126.5 |  |
|  |  |  |  |  | NR_TDD_FR1_C | -126 |  |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -125.5 |  |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -125 |  |
|  |  |  |  |  | NR_FDD_FR1_F | -124.5 |  |
|  |  |  |  |  | NR_FDD_FR1_G, NR_TDD_FR1_G | -124 |  |
|  |  |  |  |  | NR_FDD_FR1_H | -123.5 |  |
|  |  |  |  |  | NR_FDD_FR1_N | -120.5 |  |
| ± 96+ |  | ≥52 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 62+ |  | >104 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 87+ |  | ≥24 | 30 | ≥4 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -124 | -50 |
|  |  |  |  |  | NR_FDD_FR1_B | -123.5 |  |
|  |  |  |  |  | NR_TDD_FR1_C | -123 |  |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -122.5 |  |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -122 |  |
|  |  |  |  |  | NR_FDD_FR1_F | -121.5 |  |
|  |  |  |  |  | NR_FDD_FR1_G, NR_TDD_FR1_G | -121 |  |
|  |  |  |  |  | NR_FDD_FR1_H | -120.5 |  |
|  |  |  |  |  | NR_FDD_FR1_N | -117.5 |  |
| ± 68+ |  | ≥48 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 44+ |  | ≥132 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 59+ |  | ≥24 | 60 | ≥4 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -50 |
|  |  |  |  |  | NR_FDD_FR1_B | -120.5 |  |
|  |  |  |  |  | NR_TDD_FR1_C | -120 |  |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 |  |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 |  |
|  |  |  |  |  | NR_FDD_FR1_F | -118.5 |  |
|  |  |  |  |  | NR_FDD_FR1_G, NR_TDD_FR1_G | -118 |  |
|  |  |  |  |  | NR_FDD_FR1_H | -117.5 |  |
|  |  |  |  |  | NR_FDD_FR1_N | -114.5 |  |
| ± 42+ |  | ≥ 64 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 36+ |  | ≥ 132 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 180+ | -13 | ≥24 | 15 | ≥4 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 98+ |  | ≥52 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 68+ |  | >104 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 87+ |  | ≥24 | 30 | ≥4 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 85+ |  | ≥48 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 44+ |  | ≥132 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 139+ |  | ≥24 | 60 | ≥4 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 46+ |  | ≥ 64 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 30+ |  | ≥ 132 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| NOTE 1: This minimum Io condition is expressed as the average Io per RE over all REs in an OFDM symbol. NOTE 2: NR operating band groups are as defined in Section 3.5. NOTE 3: TrepPRS, LPRS ,KcombPRS are configured by higher layer parameter dl-PRS-ResourceRepetitionFactor, dl-PRS-NumSymbols and dl-PRS-CombSizeNdefined in TS 37.355 [34]. NOTE 4: The Io is defined in PRS slots. The same Io range applies to PRS and non-PRS symbols. Io levels are different in PRS and non-PRS symbols within the same slot. NOTE 5: Tc is the basic timing unit defined in TS 38.211 [6]. NOTE 6: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth of the smallest RB number for the corresponding SCS. NOTE 7: is the margin determined from Table 10.1.25.2-5. |  |  |  |  |  |  |  |

The accuracy requirements in Table 10.1.25.2-3 for FR2 are valid under the following conditions:

Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled.

PRP|dBm according to Annex B.2.14 for a corresponding Band.

AWGN propagation condition.

Table 10.1.25.2-3: UE Rx-Tx time difference measurement accuracy in FR2 in AWGN

| Accuracy | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | PRS Ês/Iot | Minimum PRS bandwidth | PRS SCS | PRS resource repetition(TrepPRS*LPRS/KcombPRSNote 3 | IoNote 4 range |  |
|  |  |  |  |  | Minimum IoNote 1 | Maximum Io |
| TcNote 5 | dB | RB | kHz |  | dBm / SCSPRS | dBm/BWChannel |
| ± 22+ | -3 | ≥24 | 60 | ≥4 | Same value as PRP in Table B.2.14-2, according to UE Power class, operating band and angle of arrival | -50 |
| ± 15+ |  | ≥64 |  | ≥1 | NOTE 6 | NOTE 6 |
| ± 7+ |  | ≥132 |  | ≥1 | NOTE 6 | NOTE 6 |
| ± 12+ |  | ≥32 | 120 | ≥4 | Same value as PRP in Table B.2.14-2, according to UE Power class, operating band and angle of arrival | -50 |
| ± 7+ |  | ≥64 |  | ≥1 | NOTE 6 | NOTE 6 |
| ± 4+ |  | ≥128 |  | ≥1 | NOTE 6 | NOTE 6 |
| ± 35+ | -13 | ≥24 | 60 | ≥4 | NOTE 6 | NOTE 6 |
| ± 15+ |  | ≥64 |  | ≥1 | NOTE 6 | NOTE 6 |
| ± 7+ |  | ≥132 |  | ≥1 | NOTE 6 | NOTE 6 |
| ± 14+ |  | ≥32 | 120 | ≥4 | NOTE 6 | NOTE 6 |
| ± 9+ |  | ≥64 |  | ≥1 | NOTE 6 | NOTE 6 |
| ± 4+ |  | ≥128 |  | ≥1 | NOTE 6 | NOTE 6 |
| NOTE 1: This minimum Io condition is expressed as the average Io per RE over all REs in an OFDM symbol. NOTE 2: NR operating band groups are as defined in Section 3.5. NOTE 3: TrepPRS, LPRS ,KcombPRS are configured by higher layer parameter dl-PRS-ResourceRepetitionFactor, dl-PRS-NumSymbols and dl-PRS-CombSizeN defined in TS 37.355 [34]. NOTE 4: The Io is defined in PRS slots. The same Io range applies to PRS and non-PRS symbols. Io levels are different in PRS and non-PRS symbols within the same slot. NOTE 5: Tc is the basic timing unit defined in TS 38.211 [6]. NOTE 6: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth of the smallest RB number for the corresponding SCS. NOTE 7: is the margin determined from Table 10.1.25.2-6. |  |  |  |  |  |  |

The accuracy requirements in Table 10.1.25.2-3a for FR2 are valid under the following conditions:

Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled.

PRP|dBm according to Annex B.2.14 for a corresponding Band

Number of measurement samples is less than 4

AWGN propagation condition.

Table 10.1.25.2-3a: UE Rx-Tx time difference measurement accuracy in FR2 in AWGN with reduced measurement samples

| Accuracy | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | PRS Ês/Iot | Minimum PRS bandwidth | PRS SCS | PRS resource repetition(TrepPRS*LPRS/KcombPRSNote 3 | IoNote 4 range |  |
|  |  |  |  |  | Minimum IoNote 1 | Maximum Io |
| TcNote 5 | dB | RB | kHz |  | dBm / SCSPRS | dBm/BWChannel |
| ± 15+ | 0 | ≥64 | 60 | ≥1 | Same value as PRP in Table B.2.14-2, according to UE Power class, operating band and angle of arrival | NOTE 6 |
| ± 7+ |  | ≥132 |  | ≥1 | NOTE 6 | NOTE 6 |
| ± 7+ |  | ≥64 | 120 | ≥1 | Same value as PRP in Table B.2.14-2, according to UE Power class, operating band and angle of arrival | NOTE 6 |
| ± 4+ |  | ≥128 |  | ≥1 | NOTE 6 | NOTE 6 |
| ± 15+ | -6 | ≥64 | 60 | ≥1 | NOTE 6 | NOTE 6 |
| ± 7+ |  | ≥132 |  | ≥1 | NOTE 6 | NOTE 6 |
| ± 9+ |  | ≥64 | 120 | ≥1 | NOTE 6 | NOTE 6 |
| ± 4+ |  | ≥128 |  | ≥1 | NOTE 6 | NOTE 6 |
| NOTE 1: This minimum Io condition is expressed as the average Io per RE over all REs in an OFDM symbol. NOTE 2: NR operating band groups are as defined in Section 3.5. NOTE 3: TrepPRS, LPRS ,KcombPRS are configured by higher layer parameter dl-PRS-ResourceRepetitionFactor, dl-PRS-NumSymbols and dl-PRS-CombSizeNdefined in TS 37.355 [34]. NOTE 4: The Io is defined in PRS slots. The same Io range applies to PRS and non-PRS symbols. Io levels are different in PRS and non-PRS symbols within the same slot. NOTE 5: Tc is the basic timing unit defined in TS 38.211 [6]. NOTE 6: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth of the smallest RB number for the corresponding SCS. NOTE 7: is the margin determined from Table 10.1.25.2-6. |  |  |  |  |  |  |

The relative accuracy requirements in Table 10.1.25.2-3b for FR2 are valid under the following conditions:

Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled.

PRP|dBm according to Annex B.2.14 for a corresponding Band

AWGN propagation condition.

the two UE Rx-Tx time difference measurements are associated with the same RxTx TEG

Table 10.1.25.2-3b: UE Rx-Tx time difference relative measurement accuracy in FR2 in AWGN with TEG reporting

| Accuracy | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | PRS Ês/Iot | PRS SCS | PRS bandwidth Note 1 | PRS resource repetition (TrepPRS*LPRS/KcombPRS) Note 2 | Io Note 3 range |  |
|  |  |  |  |  | Minimum Io | Maximum Io |
| Tc Note 4 | dB | kHz | RB |  | dBm/SCS | dBm/BWChannel |
| 35 +ΔNote 6 | (PRS Ês/Iot)j ≥-6dB (PRS Ês/Iot)i ≥-13dB | 60 | ≥ 24 | ≥ 4 | Same value as PRS_RP in Table B.2.14-2, according to UE Power class, operating band and angle of arrival | -50 |
| 24 +Δ |  |  | ≥ 64 | ≥ 1 | Note 5 | Note 5 |
| 11 +Δ |  |  | ≥ 132 | ≥ 1 | Note 5 | Note 5 |
| 24+Δ |  | 120 | ≥ 32 | ≥ 4 | Same value as PRS_RP in Table B.2.14-2, according to UE Power class, operating band and angle of arrival | -50 |
| 13 +Δ |  |  | ≥ 64 | ≥ 1 | Note 5 | Note 5 |
| 6 +Δ |  |  | ≥ 128 | ≥ 1 | Note 5 | Note 5 |
| NOTE 1: Minimum PRS bandwidth, which is minimum of the PRS bandwidths of resource j and resource i. NOTE 2: Minimum number of PRS resource repetitions among resource j and resource i. TrepPRS, LPRS ,KcombPRS are configured by higher layer parameter dl-PRS-ResourceRepetitionFactor, dl-PRS-NumSymbols and dl-PRS-CombSizeNdefined in TS 37.355 [34], respectively. NOTE 3: Io is assumed to have constant EPRE across the bandwidth. NOTE 4: Tc is the basic timing unit defined in TS 38.211 [6]. NOTE 5: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth of the smallest RB number for the corresponding SCS. NOTE 6: Δis the value of the timing error margin for the RxTx TEG, reported via nr-UE-RxTxTEG-TimingErrorMargin. Δ cannot be larger than the sum of the margins in table 10.1.25.2-6 (dependent on PRS/SRS BW) for any pair of individual UE Rx-Tx time difference measurements associated with the RxTx TEG. |  |  |  |  |  |  |

The accuracy requirements in Table 10.1.25.2-4 for FR2 are valid under the following conditions:

Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled.

PRP|dBm according to Annex B.2.14 for a corresponding Band.

Fading propagation condition.

Table 10.1.25.2-4: UE Rx-Tx time difference measurement accuracy in FR2 in fading

| Accuracy | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | PRS Ês/Iot | Minimum PRS bandwidth | PRS SCS | PRS resource repetition(TrepPRS*LPRS/KcombPRSNote 3 | IoNote 4 range |  |
|  |  |  |  |  | Minimum IoNote 1 | Maximum Io |
| TcNote 5 | dB | RB | kHz |  | dBm / SCSPRS | dBm/BWChannel |
| ± 75+ | -3 | ≥24 | 60 | ≥4 | Same value as PRP in Table B.2.14-2, according to UE Power class, operating band and angle of arrival | -50 |
| ± 72+ |  | ≥64 |  | ≥1 | NOTE 6 | NOTE 6 |
| ± 57+ |  | ≥132 |  | ≥1 | NOTE 6 | NOTE 6 |
| ± 61+ |  | ≥32 | 120 | ≥4 | Same value as PRP in Table B.2.14-2, according to UE Power class, operating band and angle of arrival | -50 |
| ± 64+ |  | ≥64 |  | ≥1 | NOTE 6 | NOTE 6 |
| ± 55+ |  | ≥128 |  | ≥1 | NOTE 6 | NOTE 6 |
| ± 92+ | -13 | ≥24 | 60 | ≥4 | NOTE 6 | NOTE 6 |
| ± 70+ |  | ≥64 |  | ≥1 | NOTE 6 | NOTE 6 |
| ± 57+ |  | ≥132 |  | ≥1 | NOTE 6 | NOTE 6 |
| ± 60+ |  | ≥32 | 120 | ≥4 | NOTE 6 | NOTE 6 |
| ± 66+ |  | ≥64 |  | ≥1 | NOTE 6 | NOTE 6 |
| ± 62+ |  | ≥128 |  | ≥1 | NOTE 6 | NOTE 6 |
| NOTE 1: This minimum Io condition is expressed as the average Io per RE over all REs in an OFDM symbol. NOTE 2: NR operating band groups are as defined in Section 3.5. NOTE 3: TrepPRS, LPRS ,KcombPRS are configured by higher layer parameter dl-PRS-ResourceRepetitionFactor, dl-PRS-NumSymbols and dl-PRS-CombSizeN defined in TS 37.355 [34]. NOTE 4: The Io is defined in PRS slots. The same Io range applies to PRS and non-PRS symbols. Io levels are different in PRS and non-PRS symbols within the same slot. NOTE 5: Tc is the basic timing unit defined in TS 38.211 [6]. NOTE 6: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth of the smallest RB number for the corresponding SCS. NOTE 7: is the margin determined from Table 10.1.25.2-6. |  |  |  |  |  |  |

Table 10.1.25.2-5: Margin for UE Rx-Tx time difference measurement accuracy in FR1

| Min(PRS BW, SRS BW) (RB) |  |  | Margin (Tc Note 1) |
| --- | --- | --- | --- |
| SCS = 15 kHz | SCS = 30 kHz | SCS = 60 kHz |  |
| ≥ 24 | N/A | N/A | 160 |
| ≥ 52 | ≥ 24 | N/A | 80 |
| ≥ 104 | ≥ 48 | ≥ 24 | 56 |
| N/A | ≥ 132 | ≥ 64 | 24 |
| N/A | N/A | ≥ 132 | 24 |
| NOTE 1: Tc is the basic timing unit defined in TS 38.211 [6]. NOTE 2: If SRS and PRS have different SCS, the margin corresponding to the smallest RS BW in MHz applies. |  |  |  |

Table 10.1.25.2-6: Margin for UE Rx-Tx time difference measurement accuracy in FR2

| Min(PRS BW, SRS BW) (MHz) |  | Margin (Tc Note 1) |
| --- | --- | --- |
| SCS = 60 kHz | SCS = 120 kHz |  |
| ≥ 24 | N/A | 76 |
| ≥ 64 | ≥ 32 | 32 |
| ≥ 132 | ≥ 64 | 24 |
| N/A | ≥ 128 | 20 |
| NOTE 1: Tc is the basic timing unit defined in TS 38.211 [6]. NOTE 2: If SRS and PRS have different SCS, the margin corresponding to the smallest RS BW in MHz applies. |  |  |

#### 10.1.25.3	Report mapping

Absolute UE Rx-Tx measurement reporting in clause 10.1.25.3.1, differential reporting for UE Rx-Tx measurement in clause 10.1.25.3.2, and additional path report mapping for UE Rx-Tx measurement in clause 10.1.25.3.3 applies, regardless of number of samples used to measure PRS, to report:

-	TEG based measurement corresponding to UE reported Rx TEG in nr-UE-Rx-TEG-ID-r17 [34],

-	gap-based UE Rx-Tx measurement,

-	gapless UE Rx-Tx measurement,

-	UE Rx-Tx in RRC_INACTIVE state.

##### 10.1.25.3.1	Absolute UE Rx-Tx Measurement Report Mapping

The reporting range for the absolute UE Rx-Tx time difference measurement (TUE Rx-Tx) is defined from -985024Tc to 985024Tc with the resolution step of 2kTc, where:

Tc is defined in TS 38.211 [6],

kmin≤k≤kmax,

kmin=2 and kmax=5, when at least one of the PRS and the SRS resources configured for TUE Rx-Tx is in FR1,

kmin=0 and kmax=5, when both PRS and SRS resources configured for TUE Rx-Tx are in FR2,

k≥ timingReportingGranularityFactor [34] configured by LMF via LPP for the UE Rx-Tx time difference measurement.

The TUE Rx-Tx report mapping for k = 0, 1, 2, 3, 4, and 5 are specified in Tables 10.1.25.3.1-1, 10.1.25.3.1-2, 10.1.25.3.1-3, 10.1.25.3.1-4, 10.1.25.3.1-5, and 10.1.25.3.1-6, respectively.

Table 10.1.25.3.1-1: Absolute UE Rx-Tx time difference measurement report mapping for k=0

| Reported Quantity Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| RX-TX_TIME_DIFFERENCE_0000 | TUE Rx-Tx < -985024 | Tc |
| RX-TX_TIME_DIFFERENCE_0001 | -985024  TUE Rx-Tx < -985023 | Tc |
| RX-TX_TIME_DIFFERENCE_0002 | -985023  TUE Rx-Tx < -985022 | Tc |
|  |  | … |
| RX-TX_TIME_DIFFERENCE_985024 | -1  TUE Rx-Tx < 0 | Tc |
| RX-TX_TIME_DIFFERENCE_985025 | 0  TUE Rx-Tx < 1 | Tc |
| … | … | … |
| RX-TX_TIME_DIFFERENCE_1970047 | 985022  TUE Rx-Tx < 985023 | Tc |
| RX-TX_TIME_DIFFERENCE_1970048 | 985023  TUE Rx-Tx < 985024 | Tc |
| RX-TX_TIME_DIFFERENCE_1970049 | 985024  TUE Rx-Tx | Tc |

Table 10.1.25.3.1-2: Absolute UE Rx-Tx time difference measurement report mapping for k=1

| Reported Quantity Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| RX-TX_TIME_DIFFERENCE_0000 | TUE Rx-Tx < -985024 | Tc |
| RX-TX_TIME_DIFFERENCE_0001 | -985024  TUE Rx-Tx < -985022 | Tc |
| RX-TX_TIME_DIFFERENCE_0002 | -985022  TUE Rx-Tx < -985020 | Tc |
|  |  | … |
| RX-TX_TIME_DIFFERENCE_492512 | -2  TUE Rx-Tx < 0 | Tc |
| RX-TX_TIME_DIFFERENCE_492513 | 0  TUE Rx-Tx < 2 | Tc |
| … | … | … |
| RX-TX_TIME_DIFFERENCE_985023 | 985020  TUE Rx-Tx < 985022 | Tc |
| RX-TX_TIME_DIFFERENCE_985024 | 985022  TUE Rx-Tx < 985024 | Tc |
| RX-TX_TIME_DIFFERENCE_985025 | 985024  TUE Rx-Tx | Tc |

Table 10.1.25.3.1-3: Absolute UE Rx-Tx time difference measurement report mapping for k=2

| Reported Quantity Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| RX-TX_TIME_DIFFERENCE_0000 | TUE Rx-Tx < -985024 | Tc |
| RX-TX_TIME_DIFFERENCE_0001 | -985024  TUE Rx-Tx < -985020 | Tc |
| RX-TX_TIME_DIFFERENCE_0002 | -985020  TUE Rx-Tx < -985016 | Tc |
|  |  | … |
| RX-TX_TIME_DIFFERENCE_246256 | -4  TUE Rx-Tx < 0 | Tc |
| RX-TX_TIME_DIFFERENCE_246257 | 0  TUE Rx-Tx < 4 | Tc |
| … | … | … |
| RX-TX_TIME_DIFFERENCE_492511 | 985016  TUE Rx-Tx < 985020 | Tc |
| RX-TX_TIME_DIFFERENCE_492512 | 985020  TUE Rx-Tx < 985024 | Tc |
| RX-TX_TIME_DIFFERENCE_492513 | 985024  TUE Rx-Tx | Tc |

Table 10.1.25.3.1-4: Absolute UE Rx-Tx time difference measurement report mapping for k=3

| Reported Quantity Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| RX-TX_TIME_DIFFERENCE_0000 | TUE Rx-Tx < -985024 | Tc |
| RX-TX_TIME_DIFFERENCE_0001 | -985024  TUE Rx-Tx < -985016 | Tc |
| RX-TX_TIME_DIFFERENCE_0002 | -985016  TUE Rx-Tx < -985008 | Tc |
|  |  | … |
| RX-TX_TIME_DIFFERENCE_123128 | -8  TUE Rx-Tx < 0 | Tc |
| RX-TX_TIME_DIFFERENCE_123129 | 0  TUE Rx-Tx < 8 | Tc |
| … | … | … |
| RX-TX_TIME_DIFFERENCE_246255 | 985008  TUE Rx-Tx < 985016 | Tc |
| RX-TX_TIME_DIFFERENCE_246256 | 985016  TUE Rx-Tx < 985024 | Tc |
| RX-TX_TIME_DIFFERENCE_246257 | 985024  TUE Rx-Tx | Tc |

Table 10.1.25.3.1-5: Absolute UE Rx-Tx time difference measurement report mapping for k=4

| Reported Quantity Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| RX-TX_TIME_DIFFERENCE_0000 | TUE Rx-Tx < -985024 | Tc |
| RX-TX_TIME_DIFFERENCE_0001 | -985024  TUE Rx-Tx < -985008 | Tc |
| RX-TX_TIME_DIFFERENCE_0002 | -985008  TUE Rx-Tx < -984992 | Tc |
|  |  | … |
| RX-TX_TIME_DIFFERENCE_61564 | -16  TUE Rx-Tx < 0 | Tc |
| RX-TX_TIME_DIFFERENCE_61565 | 0  TUE Rx-Tx < 16 | Tc |
| … | … | … |
| RX-TX_TIME_DIFFERENCE_123127 | 984992  TUE Rx-Tx < 985008 | Tc |
| RX-TX_TIME_DIFFERENCE_123128 | 985008  TUE Rx-Tx < 985024 | Tc |
| RX-TX_TIME_DIFFERENCE_123129 | 985024  TUE Rx-Tx | Tc |

Table 10.1.25.3.1-6: Absolute UE Rx-Tx time difference measurement report mapping for k=5

| Reported Quantity Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| RX-TX_TIME_DIFFERENCE_0000 | TUE Rx-Tx < -985024 | Tc |
| RX-TX_TIME_DIFFERENCE_0001 | -985024  TUE Rx-Tx < -984992 | Tc |
| RX-TX_TIME_DIFFERENCE_0002 | -984992  TUE Rx-Tx < -984960 | Tc |
|  |  | … |
| RX-TX_TIME_DIFFERENCE_30782 | -32  TUE Rx-Tx < 0 | Tc |
| RX-TX_TIME_DIFFERENCE_30783 | 0  TUE Rx-Tx < 32 | Tc |
| … | … | … |
| RX-TX_TIME_DIFFERENCE_61563 | 984960  TUE Rx-Tx < 984992 | Tc |
| RX-TX_TIME_DIFFERENCE_61564 | 984992  TUE Rx-Tx < 985024 | Tc |
| RX-TX_TIME_DIFFERENCE_61565 | 985024  TUE Rx-Tx | Tc |

##### 10.1.25.3.2	Differential UE Rx-Tx Measurement Report Mapping

The reporting range for differential UE Rx-Tx time difference measurement (TUE Rx-Tx) is defined from 0 up to 8191Tc where:

TUE Rx-Tx = TUE Rx-Tx1 - TUE Rx-Tx2; where:

TUE Rx-Tx1 > TUE Rx-Tx2,

TUE Rx-Tx1 is the first absolute UE Rx-Tx time difference measurement,

TUE Rx-Tx1 is the second absolute UE Rx-Tx time difference measurement,

Tc is defined in TS 38.211 [6],

kmin≤k≤kmax,

kmin=2 and kmax=5, when at least one of the PRS and the SRS resources configured for TUE Rx-Tx is in FR1,

kmin=0 and kmax=5, when all the PRS and SRS resources configured for TUE Rx-Tx are in FR2,

k≥ timingReportingGranularityFactor [34] configured by LMF via LPP for the UE Rx-Tx time difference measurement.

The TUE Rx-Tx report mapping for k = 0, 1, 2, 3, 4, and 5 are specified in Tables 10.1.25.3.2-1, 10.1.25.3.2-2, 10.1.25.3.2-3, 10.1.25.3.2-4, 10.1.25.3.2-5, and 10.1.25.3.2-6, respectively.

Table 10.1.25.3.2-1: Differential UE Rx-Tx time difference measurement report mapping for k=0

| Reported Quantity Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| DIFF_RX-TX_TIME_DIFFERENCE_0000 | 0  TUE Rx-Tx < 1 | Tc |
| DIFF_RX-TX_TIME_DIFFERENCE_0001 | 1  TUE Rx-Tx < 2 | Tc |
| DIFF_RX-TX_TIME_DIFFERENCE_0002 | 2  TUE Rx-Tx < 3 | Tc |
|  |  | … |
| DIFF_RX-TX_TIME_DIFFERENCE_8189 | 8189  TUE Rx-Tx < 8190 | Tc |
| DIFF_RX-TX_TIME_DIFFERENCE_8190 | 8190  TUE Rx-Tx < 8191 | Tc |
| DIFF_RX-TX_TIME_DIFFERENCE_8191 | 8191  TUE Rx-Tx | Tc |

Table 10.1.25.3.2-2: Differential UE Rx-Tx time difference measurement report mapping for k=1

| Reported Quantity Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| DIFF_RX-TX_TIME_DIFFERENCE_0000 | 0  TUE Rx-Tx < 2 | Tc |
| DIFF_RX-TX_TIME_DIFFERENCE_0001 | 2  TUE Rx-Tx < 4 | Tc |
| DIFF_RX-TX_TIME_DIFFERENCE_0002 | 4  TUE Rx-Tx < 6 | Tc |
|  |  | … |
| DIFF_RX-TX_TIME_DIFFERENCE_4093 | 8186  TUE Rx-Tx < 8188 | Tc |
| DIFF_RX-TX_TIME_DIFFERENCE_4094 | 8188  TUE Rx-Tx < 8190 | Tc |
| DIFF_RX-TX_TIME_DIFFERENCE_4095 | 8190  TUE Rx-Tx | Tc |

Table 10.1.25.3.2-3: Differential UE Rx-Tx time difference measurement report mapping for k=2

| Reported Quantity Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| DIFF_RX-TX_TIME_DIFFERENCE_0000 | 0  TUE Rx-Tx < 4 | Tc |
| DIFF_RX-TX_TIME_DIFFERENCE_0001 | 4  TUE Rx-Tx < 8 | Tc |
| DIFF_RX-TX_TIME_DIFFERENCE_0002 | 8  TUE Rx-Tx < 12 | Tc |
|  |  | … |
| DIFF_RX-TX_TIME_DIFFERENCE_2045 | 8180  TUE Rx-Tx < 8184 | Tc |
| DIFF_RX-TX_TIME_DIFFERENCE_2046 | 8184  TUE Rx-Tx < 8188 | Tc |
| DIFF_RX-TX_TIME_DIFFERENCE_2047 | 8188  TUE Rx-Tx | Tc |

Table 10.1.25.3.2-4: Differential UE Rx-Tx time difference measurement report mapping for k=3

| Reported Quantity Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| DIFF_RX-TX_TIME_DIFFERENCE_0000 | 0  TUE Rx-Tx < 8 | Tc |
| DIFF_RX-TX_TIME_DIFFERENCE_0001 | 8  TUE Rx-Tx < 16 | Tc |
| DIFF_RX-TX_TIME_DIFFERENCE_0002 | 16  TUE Rx-Tx < 24 | Tc |
|  |  | … |
| DIFF_RX-TX_TIME_DIFFERENCE_1021 | 8168  TUE Rx-Tx < 8176 | Tc |
| DIFF_RX-TX_TIME_DIFFERENCE_1022 | 8176  TUE Rx-Tx < 8184 | Tc |
| DIFF_RX-TX_TIME_DIFFERENCE_1023 | 8184  TUE Rx-Tx | Tc |

Table 10.1.25.3.2-5: Differential UE Rx-Tx time difference measurement report mapping for k=4

| Reported Quantity Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| DIFF_RX-TX_TIME_DIFFERENCE_0000 | 0  TUE Rx-Tx < 16 | Tc |
| DIFF_RX-TX_TIME_DIFFERENCE_0001 | 16  TUE Rx-Tx < 32 | Tc |
| DIFF_RX-TX_TIME_DIFFERENCE_0002 | 32  TUE Rx-Tx < 48 | Tc |
|  |  | … |
| DIFF_RX-TX_TIME_DIFFERENCE_509 | 8144  TUE Rx-Tx < 8160 | Tc |
| DIFF_RX-TX_TIME_DIFFERENCE_510 | 8160  TUE Rx-Tx < 8176 | Tc |
| DIFF_RX-TX_TIME_DIFFERENCE_511 | 8176  TUE Rx-Tx | Tc |

Table 10.1.25.3.2-6: Differential UE Rx-Tx time difference measurement report mapping for k=5

| Reported Quantity Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| DIFF_RX-TX_TIME_DIFFERENCE_0000 | 0  TUE Rx-Tx < 32 | Tc |
| DIFF_RX-TX_TIME_DIFFERENCE_0001 | 32  TUE Rx-Tx < 64 | Tc |
| DIFF_RX-TX_TIME_DIFFERENCE_0002 | 64  TUE Rx-Tx < 96 | Tc |
|  |  | … |
| DIFF_RX-TX_TIME_DIFFERENCE_253 | 8096  TUE Rx-Tx < 8128 | Tc |
| DIFF_RX-TX_TIME_DIFFERENCE_254 | 8128  TUE Rx-Tx < 8160 | Tc |
| DIFF_RX-TX_TIME_DIFFERENCE_255 | 8160  TUE Rx-Tx | Tc |

##### 10.1.25.3.3	Additional Path Report Mapping for UE Rx-Tx Time Difference

The reporting range for the additional path reporting for an UE Rx-Tx time difference measurement is defined up to the range from -8175Tc to 8175Tc with the resolution step of 2kTc, where

Tc is defined in TS 38.211 [6],

kmin≤k≤kmax,

kmin=2 and kmax=5, when at least one of the PRS resource and SRS resource configured for the UE Rx-Tx time difference measurement is in FR1,

kmin=0 and kmax=5, when both of the PRS resource and SRS resource configured for the UE Rx-Tx time difference measurement is in FR2,

k≥ timingReportingGranularityFactor [34] configured by LMF via LPP for the UE Rx-Tx time difference measurement.

The UE can report the timing of up to two additional paths with respect to the path timing determining the UE Rx-Tx time difference measurement.

The UE capable of  additionalPathsExtSupport-r17 can report the timing of up to its supported number of additional paths with respect to the path timing determining the UE Rx-Tx measurement.

The report mappings for different k values are specified in Tables 10.1.25.3.3-1  10.1.25.3.3-6.

Table 10.1.25.3.3-1: Report mapping for k=0

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_00000 | path < -8175 | Tc |
| path_00001 | -8175 path < -8174 | Tc |
| path_00002 | -8174 path < -8173 | Tc |
|  |  | … |
| path_08175 | -1 path < 0 | Tc |
| path_08176 | 0 path < 1 | Tc |
| … | … | … |
| path_ 16349 | 8173 path < 8174 | Tc |
| path_ 16350 | 8174 path < 8175 | Tc |
| path_ 16351 | 8175 path | Tc |

Table 10.1.25.3.3-2: Report mapping for k=1

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_0000 | path < -8175 | Tc |
| path_0001 | -8175 path < -8173 | Tc |
| path_0002 | -8173 path < -8171 | Tc |
|  |  | … |
| path_4088 | -1 path < 1 | Tc |
| … | … | … |
| path_8174 | 8171 path < 8173 | Tc |
| path_8175 | 8173 path < 8175 | Tc |
| path_8176 | 8175 path | Tc |

Table 10.1.25.3.3-3: Report mapping for k=2

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_0000 | path < -8174 | Tc |
| path_0001 | -8174 path < -8170 | Tc |
| path_0002 | -8170 path < -8166 | Tc |
|  |  | … |
| path_2044 | -2 path < 2 | Tc |
| … | … | … |
| path_4086 | 8166 path < 8170 | Tc |
| path_4087 | 8170 path < 8174 | Tc |
| path_4088 | 8174 path | Tc |

Table 10.1.25.3.3-4: Report mapping for k=3

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_0000 | path < -8172 | Tc |
| path_0001 | -8172 path < -8164 | Tc |
| path_0002 | -8164 path < -8156 | Tc |
|  |  | … |
| path_1022 | -4 path < 4 | Tc |
| … | … | … |
| path_2042 | 8156 path < 8164 | Tc |
| path_2043 | 8164 path < 8172 | Tc |
| path_2044 | 8172 path | Tc |

Table 10.1.25.3.3-5: Report mapping for k=4

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_0000 | path < -8168 | Tc |
| path_0001 | -8168 path < -8152 | Tc |
| path_0002 | -8152 path < -8136 | Tc |
|  |  | … |
| path_511 | -8 path < 8 | Tc |
| … | … | … |
| path_1020 | 8136 path < 8152 | Tc |
| path_1021 | 8152 path < 8168 | Tc |
| path_1022 | 8168 path | Tc |

Table 10.1.25.3.3-6: Report mapping for k=5

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_000 | path < -8160 | Tc |
| path_001 | -8160 path < -8128 | Tc |
| path_002 | -8128 path < -8096 | Tc |
|  |  | … |
| path_256 | 0 path < 32 | Tc |
| … | … | … |
| path_509 | 8096 path < 8128 | Tc |
| path_510 | 8128 path < 8160 | Tc |
| path_511 | 8160 path | Tc |

### 10.1.25C	UE Rx-Tx Time Difference Measurements in Satellite Accesss

#### 10.1.25C.1	Introduction

The requirements in Clause 10.1.25C shall apply, provided the UE has received nr-Multi-RTT-RequestLocationInformation message from LMF via LPP [31] requesting the UE to report one or more UE Rx-Tx time difference measurements defined in TS 38.215 [4]. The requirements in Clause 10.1.25C shall apply:

-	when UE is in RRC_CONNECTED state and the measurement is performed with MG or without MG.

#### 10.1.25C.2	Measurement Accuracy Requirements

The UE Rx-Tx time difference measurement accuracy requirements in this clause shall not apply, if:

-	NTA_offset defined in Table 7.1.2-2 changes during the UE Rx-Tx measurement period or

-	if the uplink transmission timing changes during the UE Rx-Tx measurement period due to the network-configured Timing Advance.

The UE Rx-Tx time difference measurement accuracy requirements in this clause shall apply provided that:

-	The UE transmits SRS within [-160, 160] msec of at least one DL PRS resource of each of the TRPs corresponding to the serving cell in the assistance data.

If the uplink transmission timing changes during the UE Rx-Tx measurement period due to the autonomous timing adjustment defined in clause 7.1C.2 then:

-	UE Rx-Tx measurement accuracy requirements shall apply for a cell, which is also the downlink reference cell (defined in section 7.1C.1) for SRS transmission even if the uplink transmission timing changes during the UE Rx-Tx measurement period due to autonomous adjustment.

The accuracy requirements in Table 10.1.25C.2-1 for FR1-NTN are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-5 [18] for reference sensitivity are fulfilled.

-	PRP|dBm according to Annex B.2.X for a corresponding Band.

-	AWGN propagation condition.

Table 10.1.25C.2-1: UE Rx-Tx time difference measurement accuracy in FR1 in AWGN with reduced measurement samples

| Accuracy | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | PRS Ês/Iot | Minimum PRS bandwidth | PRS SCS | PRS resource repetition (TrepPRS*LPRS/KcombPRSNote 3 | NR operating band groupsNote 2 | IoNote 4 range |  |
|  |  |  |  |  |  | Minimum IoNote 1 | Maximum Io |
| TcNote 5 | dB | RB | kHz |  |  | dBm / SCSPRS | dBm/BW |
| ± 59+ | 0 | ≥52 | 15 | ≥1 | NR_FDD_SAB_FR1_A | -121 | -50 |
| ± 30+6 |  | >104 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 30+ |  | ≥48 | 30 | ≥1 | NR_FDD_SAB_FR1_A | -118 | -50 |
| ± 15+ |  | ≥132 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 15+ |  | ≥64 | 60 | ≥1 | NR_FDD_SAB_FR1_A | -115 | -50 |
| ± 7+ |  | ≥132 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 75+ | -6 | ≥52 | 15 | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 37+ |  | >104 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 39+ |  | ≥48 | 30 | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 16+ |  | ≥132 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 16+ |  | ≥64 | 60 | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 8+ |  | ≥132 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| NOTE 1: This minimum Io condition is expressed as the average Io per RE over all REs in an OFDM symbol. NOTE 2: NR operating band groups are as defined in Section 3.5. NOTE 3: TrepPRS, LPRS ,KcombPRS are configured by higher layer parameter dl-PRS-ResourceRepetitionFactor, dl-PRS-NumSymbols and dl-PRS-CombSizeN defined in TS 37.355 [34]. NOTE 4: The Io is defined in PRS slots. The same Io range applies to PRS and non-PRS symbols. Io levels are different in PRS and non-PRS symbols within the same slot. NOTE 5: Tc is the basic timing unit defined in TS 38.211 [6]. NOTE 6: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth of the smallest RB number for the corresponding SCS. NOTE 7: is the margin determined from Table 10.1.25C.2-2. |  |  |  |  |  |  |  |

Table 10.1.25C.2-2: Margin for UE Rx-Tx time difference measurement accuracy in FR1-NTN

| Min(PRS BW, SRS BW) (RB) |  |  | Margin (Tc Note 1) |
| --- | --- | --- | --- |
| SCS = 15 kHz | SCS = 30 kHz | SCS = 60 kHz |  |
| ≥ 24 | N/A | N/A | 160 |
| ≥ 52 | ≥ 24 | N/A | 80 |
| ≥ 104 | ≥ 48 | ≥ 24 | 56 |
| N/A | ≥ 132 | ≥ 64 | 24 |
| N/A | N/A | ≥ 132 | 24 |
| NOTE 1: Tc is the basic timing unit defined in TS 38.211 [6]. NOTE 2: If SRS and PRS have different SCS, the margin corresponding to the smallest RS BW in MHz applies. |  |  |  |

#### 10.1.25C.3	Report mapping

The report mapping provided in clause 10.1.25.3 is applicable for NTN.

### 10.1.26	FR2 P-MPR report

The FR2 P-MPR report mapping is defined by this clause.

#### 10.1.26.1	Report mapping

Table 10.1.26.1-1 defines the FR2 P-MPR report mapping.

Table 10.1.26.1-1 Mapping of FR2 P-MPR

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| P-MPR_00 | 3  PMP-R < 6 | dB |
| P-MPR_01 | 6  PMP-R < 9 | dB |
| P-MPR_02 | 9  PMP-R < 12 | dB |
| P-MPR_03 | PMP-R 12 | dB |

### 10.1.27	L1-SINR accuracy requirements for FR1

#### 10.1.27.1	L1-SINR accuracy requirements with CSI-RS based CMR and no dedicated IMR configured

##### 10.1.27.1.1	Absolute Accuracy

Unless otherwise specified, the requirements for absolute accuracy of CSI-RS based L1-SINR in this clause apply to all CSI-RS resources configured as CMR and no dedicated resource configured as IMR of the serving cell configured for L1-SINR measurement.

The accuracy requirements in Table 10.1.27.1.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for L1-SINR measurements are fulfilled according to Annex B.2.8.1 for a corresponding Band for each relevant CSI-RS based CMR.

-	The bandwidth of CSI-RS as CMR is 48 PRBs and the density is 3.

-	AWGN radio propagation conditions.

The performance with larger bandwidth of CSI-RS as CMR is equal to or better than the accuracy requirements in Table 10.1.27.1.1-1.

Table 10.1.27.1.1-1: L1-SINR absolute accuracy for CSI-RS based CMR only in FR1

| Accuracy |  | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | CSI-RS CMR Ês/Iot | Io Note 1 range |  |  |  |  |  |
|  |  |  | NR operating band groups Note 2 | Minimum Io |  |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSCSI-RS |  |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSCSI-RS = 15 kHz | SCSCSI-RS = 30 kHz | SCSCSI-RS = 60 kHz |  |  |
|  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | -115 | N/A | -50 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | -114.5 | N/A | -50 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | -114 | N/A | -50 |
| ±5.5 | ±6.5 | -3 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | -113.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | -113 | N/A | -50 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | -112.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | -112 | N/A | -50 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | -111.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_N | -114.5 | -111.5 | -108.5 | N/A | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups in FR1 are as defined in clause 3.5.2. NOTE 3: The requirements apply for CSI-RS CMR Ês/Iot ≤ 6 dB with SCS 15kHz or 30kHz under NR high speed scenarios. |  |  |  |  |  |  |  |  |

##### 10.1.27.1.2	Relative Accuracy

The relative accuracy of CSI-RS based L1-SINR is defined as the L1-SINR measured from one CSI-RS compared to the largest measured value of L1-SINR among all CSI-RS resources of the serving cell.

The accuracy requirements in Table 10.1.27.1.2-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for L1-SINR measurements are fulfilled according to Annex B.2.8.1 for a corresponding Band for each relevant CSI-RS based CMR.

-	The bandwidth of CSI-RS is 48 PRBs and the density is 3.

-	AWGN radio propagation conditions.

The performance with larger bandwidth of CSI-RS as CMR is equal to or better than the accuracy requirements in Table 10.1.27.1.2-1.

Table 10.1.27.1.2-1: L1-SINR relative accuracy for CSI-RS based CMR only in FR1

| Accuracy |  | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | CSI-RS CMR Ês/Iot Note 2 | Io Note 1 range |  |  |  |  |  |
|  |  |  | NR operating band groups Note 3 | Minimum Io |  |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSCSI-RS |  |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSCSI-RS = 15 kHz | SCSCSI-RS = 30 kHz | SCSCSI-RS = 60 kHz |  |  |
|  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | -115 | N/A | -50 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | -114.5 | N/A | -50 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | -114 | N/A | -50 |
| ±4.5 | ±5.5 | -3 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | -113.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | -113 | N/A | -50 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | -112.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | -112 | N/A | -50 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | -111.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_N | -114.5 | -111.5 | -108.5 | N/A | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The parameter CSI-RS CMR Ês/Iot is the minimum CMR CMR Ês/Iot of the pair of CSI-RS resources to which the requirement applies. NOTE 3: NR operating band groups in FR1 are as defined in clause 3.5.2. NOTE 4: The requirements apply for CSI-RS CMR Ês/Iot ≤ 6 dB with SCS 15kHz or 30kHz under NR high speed scenarios. |  |  |  |  |  |  |  |  |

#### 10.1.27.2	L1-SINR accuracy requirements with SSB based CMR and dedicated IMR configured

##### 10.1.27.2.1	Absolute Accuracy

Unless otherwise specified, the requirements for absolute accuracy of SSB based L1-SINR in this clause apply to all SSBs configured as CMR and dedicated resources configured as IMR of the serving cell configured for L1-SINR measurement.

The accuracy requirements are defined in Table 10.1.27.2.1-1 for SSB based CMR and NZP-IMR and in Table 10.1.27.2.1-2 for SSB based CMR and ZP-IMR.

The accuracy requirements in Tables 10.1.27.2.1-1 and 10.1.27.2.1-2 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for L1-SINR measurements are fulfilled according to Annex B.2.8.2 for a corresponding Band for each relevant SSB based CMR and IMR.

-	The bandwidth of NZP-IMR and ZP-IMR is 48 PRBs and the density is 3.

-	AWGN radio propagation conditions.

The performance with larger bandwidth of NZP-IMR and ZP-IMR is equal to or better than the accuracy requirements in Tables 10.1.27.2.1-1 and 10.1.27.2.1-2.

Table 10.1.27.2.1-1: L1-SINR absolute accuracy for SSB based CMR and NZP-IMR in FR1

| Accuracy |  | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB- CMR Ês/Iot | NZP-IMR Ês/Iot | Io Note 1 range |  |  |  |  |
|  |  |  |  | NR operating band groups Note 2 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
|  |  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | N/A | -50 |
|  |  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | N/A | -50 |
|  |  |  |  | NR_TDD_FR1_C | -120 | -117 | N/A | -50 |
| ±4.0 | ±5.0 | 0 | 0 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | N/A | -50 |
|  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | N/A | -50 |
|  |  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | N/A | -50 |
|  |  |  |  | NR_FDD_FR1_G | -118 | -115 | N/A | -50 |
|  |  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | N/A | -50 |
|  |  |  |  | NR_FDD_FR1_N | -114.5 | -111.5 | N/A | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups in FR1 are as defined in clause 3.5.2. NOTE 3: The requirements apply for SSB-CMR Ês/Iot ≤ 6 dB with SCS 15kHz or 30kHz under NR high speed scenarios. |  |  |  |  |  |  |  |  |

Table 10.1.27.2.1-2: L1-SINR absolute accuracy for SSB based CMR and ZP-IMR in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB- CMR Ês/Iot | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 2 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
|  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | N/A | -50 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | N/A | -50 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | N/A | -50 |
| ±4.5 | ±5.5 | -3 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | N/A | -50 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | N/A | -50 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_N | -114.5 | -111.5 | N/A | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups in FR1 are as defined in clause 3.5.2. NOTE 3: The requirements apply for SSB Ês/Iot ≤ 6 dB with SCS 15kHz or 30kHz under NR high speed scenarios. |  |  |  |  |  |  |  |

##### 10.1.27.2.2	Relative Accuracy

The relative accuracy of SSB based L1-SINR is defined as the L1-SINR measured from one SSB configured as CMR and one IMR configured as IMR compared to the largest measured value of L1-SINR among all SSBs and IMRs of the serving cell.

The accuracy requirements are defined in Table 10.1.27.2.2-1 for SSB based CMR and NZP-IMR and in Table 10.1.27.2.2-2 for SSB based CMR and ZP-IMR.

The accuracy requirements in Tables 10.1.27.2.2-1 and 10.1.27.2.2-2 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for L1-SINR measurements are fulfilled according to Annex B.2.8.2 for a corresponding Band for each relevant SSB based CMR and IMR.

-	The bandwidth of NZP-IMR and ZP-IMR is 48 PRBs and the density is 3.

-	AWGN radio propagation conditions.

The performance with larger bandwidth of NZP-IMR and ZP-IMR is equal to or better than the accuracy requirements in Tables 10.1.27.2.2-1 and 10.1.27.2.2-2.

Table 10.1.27.2.2-1: L1-SINR relative accuracy for SSB based CMR and NZP-IMR in FR1

| Accuracy |  | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB- CMR Ês/Iot Note 2 | NZP-IMR Ês/Iot | Io Note 1 range |  |  |  |  |
|  |  |  |  | NR operating band groups Note 3 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
|  |  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | N/A | -50 |
|  |  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | N/A | -50 |
|  |  |  |  | NR_TDD_FR1_C | -120 | -117 | N/A | -50 |
| ±3.0 | ±4.0 | 0 | 0 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | N/A | -50 |
|  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | N/A | -50 |
|  |  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | N/A | -50 |
|  |  |  |  | NR_FDD_FR1_G | -118 | -115 | N/A | -50 |
|  |  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | N/A | -50 |
|  |  |  |  | NR_FDD_FR1_N | -114.5 | -111.5 | N/A | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The parameter SSB CMR Ês/Iot is the minimum SSB CMR Ês/Iot of the pair of SSBs to which the requirement applies. NOTE 3: NR operating band groups in FR1 are as defined in clause 3.5.2. NOTE 4: The requirements apply for SSB-CMR Ês/Iot ≤ 6 dB with SCS 15kHz or 30kHz under NR high speed scenarios. |  |  |  |  |  |  |  |  |

Table 10.1.27.2.2-2: L1-SINR relative accuracy for SSB based CMR and ZP-IMR in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB- CMR Ês/Iot Note 2 | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 3 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
|  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | N/A | -50 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | N/A | -50 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | N/A | -50 |
| ±3.5 | ±4.5 | -3 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | N/A | -50 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | N/A | -50 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_N | -114.5 | -111.5 | N/A | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The parameter SSB CMR Ês/Iot is the minimum SSB CMR Ês/Iot of the pair of SSBs to which the requirement applies. NOTE 3: NR operating band groups in FR1 are as defined in clause 3.5.2. NOTE 4: The requirements apply for SSB Ês/Iot ≤ 6 dB with SCS 15kHz or 30kHz under NR high speed scenarios. |  |  |  |  |  |  |  |

#### 10.1.27.3	L1-SINR accuracy requirements with CSI-RS based CMR and dedicated IMR configured

##### 10.1.27.3.1	Absolute Accuracy

Unless otherwise specified, the requirements for absolute accuracy of CSI-RS based L1-SINR in this clause apply to all CSI-RS resources configured as CMR and dedicated resources configured as IMR of the serving cell configured for L1-SINR measurement.

The accuracy requirements are defined in Table 10.1.27.3.1-1 for CSI-RS based CMR and NZP-IMR and in Table 10.1.27.3.1-2 for CSI-RS based CMR and ZP-IMR.

The accuracy requirements in Tables 10.1.27.3.1-1 and 10.1.27.3.1-2 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for L1-SINR measurements are fulfilled according to Annex B.2.8.3 for a corresponding Band for each relevant CSI-RS based CMR and IMR.

-	The bandwidth of CSI-RS as CMR, NZP-IMR and ZP-IMR is 48 PRBs and the density is 3.

-	AWGN radio propagation conditions.

The performance with larger bandwidth of CSI-RS as CMR, NZP-IMR and ZP-IMR is equal to or better than the accuracy requirements in Tables 10.1.27.3.1-1 and 10.1.27.3.1-2.

Table 10.1.27.3.1-1: L1-SINR absolute accuracy for CSI-RS based CMR and NZP-IMR in FR1

| Accuracy |  | Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | CSI-RS CMR Ês/Iot | NZP-IMR Ês/Iot | Io Note 1 range |  |  |  |  |  |
|  |  |  |  | NR operating band groups Note 2 | Minimum Io |  |  |  | Maximum Io |
| dB | dB | dB | dB |  | dBm / SCSCSI-RS |  |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  |  | SCSCSI-RS = 15 kHz | SCSCSI-RS = 30 kHz | SCSCSI-RS = 60 kHz |  |  |
|  |  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | -115 | N/A | -50 |
|  |  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | -114.5 | N/A | -50 |
|  |  |  |  | NR_TDD_FR1_C | -120 | -117 | -114 | N/A | -50 |
| ±4.0 | ±5.0 | 0 | 0 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | -113.5 | N/A | -50 |
|  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | -113 | N/A | -50 |
|  |  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | -112.5 | N/A | -50 |
|  |  |  |  | NR_FDD_FR1_G | -118 | -115 | -112 | N/A | -50 |
|  |  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | -111.5 | N/A | -50 |
|  |  |  |  | NR_FDD_FR1_N | -114.5 | -111.5 | -108.5 | N/A | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups in FR1 are as defined in clause 3.5.2. NOTE 3: The requirements apply for CSI-RS CMR Ês/Iot ≤ 6 dB with SCS 15kHz or 30kHz under NR high speed scenarios. |  |  |  |  |  |  |  |  |  |

Table 10.1.27.3.1-2: L1-SINR absolute accuracy for CSI-RS based CMR and ZP-IMR in FR1

| Accuracy |  | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | CSI-RS CMR Ês/Iot | Io Note 1 range |  |  |  |  |  |
|  |  |  | NR operating band groups Note 2 | Minimum Io |  |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSCSI-RS |  |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSCSI-RS = 15 kHz | SCSCSI-RS = 30 kHz | SCSCSI-RS = 60 kHz |  |  |
|  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | -115 | N/A | -50 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | -114.5 | N/A | -50 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | -114 | N/A | -50 |
| ±4.5 | ±5.5 | -3 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | -113.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | -113 | N/A | -50 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | -112.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | -112 | N/A | -50 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | -111.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_N | -114.5 | -111.5 | -108.5 | N/A | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups in FR1 are as defined in clause 3.5.2. NOTE 3: The requirements apply for CSI-RS CMR Ês/Iot ≤ 6 dB with SCS 15kHz or 30kHz under NR high speed scenarios. |  |  |  |  |  |  |  |  |

##### 10.1.27.3.2	Relative Accuracy

The relative accuracy of CSI-RS based L1-SINR is defined as the L1-SINR measured from one CSI-RS configured as CMR and one IMR configured as IMR compared to the largest measured value of L1-SINR among all CSI-RS and IMR resources of the serving cell.

The accuracy requirements are defined in Table 10.1.27.3.2-1 for CSI-RS based CMR and NZP-IMR and in Table 10.1.27.3.2-2 for CSI-RS based CMR and ZP-IMR.

The accuracy requirements in Tables 10.1.27.3.2-1 and 10.1.27.3.2-2 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for L1-SINR measurements are fulfilled according to Annex B.2.8.3 for a corresponding Band for each relevant CSI-RS based CMR and IMR.

-	The bandwidth of CSI-RS as CMR, NZP-IMR and ZP-IMR is 48 PRBs and the density is 3.

-	AWGN radio propagation conditions.

The performance with larger bandwidth of CSI-RS as CMR, NZP-IMR and ZP-IMR is equal to or better than the accuracy requirements in Tables 10.1.27.3.2-1 and 10.1.27.3.2-2.

Table 10.1.27.3.2-1: L1-SINR relative accuracy for CSI-RS based CMR and NZP-IMR in FR1

| Accuracy |  | Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | CSI-RS CMR Ês/Iot Note 2 | NZP-IMR Ês/Iot | Io Note 1 range |  |  |  |  |  |
|  |  |  |  | NR operating band groups Note 3 | Minimum Io |  |  |  | Maximum Io |
| dB | dB | dB | dB |  | dBm / SCSCSI-RS |  |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  |  | SCSCSI-RS = 15 kHz | SCSCSI-RS = 30 kHz | SCSCSI-RS = 60 kHz |  |  |
|  |  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | -115 | N/A | -50 |
|  |  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | -114.5 | N/A | -50 |
|  |  |  |  | NR_TDD_FR1_C | -120 | -117 | -114 | N/A | -50 |
| ±3.0 | ±4.0 | 0 | 0 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | -113.5 | N/A | -50 |
|  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | -113 | N/A | -50 |
|  |  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | -112.5 | N/A | -50 |
|  |  |  |  | NR_FDD_FR1_G | -118 | -115 | -112 | N/A | -50 |
|  |  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | -111.5 | N/A | -50 |
|  |  |  |  | NR_FDD_FR1_N | -114.5 | -111.5 | -108.5 | N/A | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The parameter CSI-RS CMR Ês/Iot is the minimum CMR CMR Ês/Iot of the pair of CSI-RS resources to which the requirement applies. NOTE 3: NR operating band groups in FR1 are as defined in clause 3.5.2. NOTE 4: The requirements apply for CSI-RS CMR Ês/Iot ≤ 6 dB with SCS 15kHz or 30kHz under NR high speed scenarios. |  |  |  |  |  |  |  |  |  |

Table 10.1.27.3.2-2: L1-SINR relative accuracy for CSI-RS based CMR and ZP-IMR in FR1

| Accuracy |  | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | CSI-RS CMR Ês/Iot Note 2 | Io Note 1 range |  |  |  |  |  |
|  |  |  | NR operating band groups Note 3 | Minimum Io |  |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSCSI-RS |  |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSCSI-RS = 15 kHz | SCSCSI-RS = 30 kHz | SCSCSI-RS = 60 kHz |  |  |
|  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | -115 | N/A | -50 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | -114.5 | N/A | -50 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | -114 | N/A | -50 |
| ±3.5 | ±4.5 | -3 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | -113.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | -113 | N/A | -50 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | -112.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | -112 | N/A | -50 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | -111.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_N | -114.5 | -111.5 | -108.5 | N/A | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The parameter CSI-RS CMR Ês/Iot is the minimum CMR CMR Ês/Iot of the pair of CSI-RS resources to which the requirement applies. NOTE 3: NR operating band groups in FR1 are as defined in clause 3.5.2. NOTE 4: The requirements apply for CSI-RS CMR Ês/Iot ≤ 6 dB with SCS 15kHz or 30kHz under NR high speed scenarios. |  |  |  |  |  |  |  |  |

### 10.1.28	L1-SINR accuracy requirements for FR2

#### 10.1.28.1	L1-SINR accuracy requirements with CSI-RS based CMR and no dedicated IMR configured

##### 10.1.28.1.1	Absolute Accuracy

Unless otherwise specified, the requirements for absolute accuracy of CSI-RS based L1-SINR in this clause apply to all CSI-RS resources configured as CMR and no dedicated resource configured as IMR of the serving cell configured for L1-SINR measurement.

The accuracy requirements in Table 10.1.28.1.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled.

-	Conditions for L1-SINR measurements are fulfilled according to Annex B.2.8.1 for a corresponding Band for each relevant CSI-RS based CMR.

-	The bandwidth of CSI-RS as CMR is 48 PRBs and the density is 3.

-	The measured signals are in the directions covered by the percentile EIS spherical coverage of the UE, defined in clause 7.3.4 of TS 38.101-2 [19].

-	AWGN radio propagation conditions.

The performance with larger bandwidth of CSI-RS as CMR is equal to or better than the accuracy requirements in Table 10.1.28.1.1-1.

Table 10.1.28.1.1-1: L1-SINR absolute accuracy for CSI-RS based CMR only in FR2

| Accuracy |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | CSI-RS CMR Ês/Iot Note 3 | Io Note 1 range |  |  |  |
|  |  |  | Minimum Io |  |  | Maximum Io |
| dB | dB | dB | dBm / SCSCSI-RS Note 2 |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  | SCSCSI-RS = 60kHz | SCSCSI-RS = 120kHz |  |  |
| ±5.5 | ±6.5 | ≥-3 | Same value as CSI-RS_RP in Table in B.2.8.1, according to UE Power class, operating band and angle of arrival |  | N/A | -50 |
| NOTE 1: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. NOTE 2: Values based on Refsens and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. NOTE 3: In the test cases, the CSI-RS CMR Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. |  |  |  |  |  |  |

##### 10.1.28.1.2	Relative Accuracy

The relative accuracy of CSI-RS based L1-SINR is defined as the L1-SINR measured from one CSI-RS compared to the largest measured value of L1-SINR among all CSI-RS resources of the serving cell.

The accuracy requirements in Table 10.1.28.1.2-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled.

-	Conditions for L1-SINR measurements are fulfilled according to Annex B.2.8.1 for a corresponding Band for each relevant CSI-RS based CMR.

-	The bandwidth of CSI-RS as CMR is 48 PRBs and the density is 3.

-	The measured signals are in the directions covered by the percentile EIS spherical coverage of the UE, defined in clause 7.3.4 of TS 38.101-2 [19].

-	AWGN radio propagation conditions.

The performance with larger bandwidth of CSI-RS as CMR is equal to or better than the accuracy requirements in Table 10.1.28.1.2-1.

Table 10.1.28.1.2-1: L1-SINR relative accuracy for CSI-RS based CMR only in FR2

| Accuracy |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | CSI-RS CMR Ês/Iot Note 2, Note 4 | Io Note 1 range |  |  |  |
|  |  |  | Minimum Io |  |  | Maximum Io |
| dB | dB | dB | dBm / SCSCSI-RS Note 3 |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  | SCSCSI-RS = 60kHz | SCSCSI-RS = 120kHz |  |  |
| ±4.5 | ±5.5 | ≥-3 | Same value as CSI-RS_RP in Table in B.2.8.1, according to UE Power class, operating band and angle of arrival |  | N/A | -50 |
| NOTE 1: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. NOTE 2: The parameter CSI-RS CMR Ês/Iot is the minimum CSI-RS CMR Ês/Iot of the pair of CSI-RS resources to which the requirement applies. NOTE 3: Values based on Refsens and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. NOTE 4: In the test cases, the CSI-RS CMR Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. |  |  |  |  |  |  |

#### 10.1.28.2	L1-SINR accuracy requirements with SSB based CMR and dedicated IMR configured

##### 10.1.28.2.1	Absolute Accuracy

Unless otherwise specified, the requirements for absolute accuracy of SSB based L1-SINR in this clause apply to all SSBs configured as CMR and dedicated resources configured as IMR of the serving cell configured for L1-SINR measurement.

The accuracy requirements are defined in Table 10.1.28.2.1-1 for SSB based CMR and NZP-IMR and in Table 10.1.28.2.1-2 for SSB based CMR and ZP-IMR.

The accuracy requirements in Tables 10.1.28.2.1-1 and 10.1.28.2.1-2 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled.

-	Conditions for L1-SINR measurements are fulfilled according to Annex B.2.8.2 for a corresponding Band for each relevant SSB based CMR and IMR.

-	The bandwidth of NZP-IMR and ZP-IMR is 48 PRBs and the density is 3.

-	The measured signals are in the directions covered by the percentile EIS spherical coverage of the UE, defined in clause 7.3.4 of TS 38.101-2 [19].

-	AWGN radio propagation conditions.

-	SSB based CMR and IMR in the test come from the same direction.

The performance with larger bandwidth of NZP-IMR and ZP-IMR is equal to or better than the accuracy requirements in Tables 10.1.28.2.1-1 and 10.1.28.2.1-2.

Table 10.1.28.2.1-1: L1-SINR absolute accuracy for SSB based CMR and NZP-IMR in FR2

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB CMR Ês/Iot Note 3 | NZP-IMR Ês/Iot Note 3 | Io Note 1 range |  |  |  |
|  |  |  |  | Minimum Io |  |  | Maximum Io |
| dB | dB | dB | dB | dBm / SCSSSB Note 2 |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 120kHz | SCSSSB = 240kHz |  |  |
| ±4.0 | ±5.0 | ≥0 | ≥0 | Same value as SSB_RP in Table in B.2.8.2, according to UE Power class, operating band and angle of arrival |  | N/A | -50 |
| NOTE 1: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. NOTE 2: Values based on Refsens and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. NOTE 3: In the test cases, the SSB Ês/Iot, NZP-IMR Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. |  |  |  |  |  |  |  |

Table 10.1.28.2.1-2: L1-SINR absolute accuracy for SSB based CMR and ZP-IMR in FR2

| Accuracy |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB CMR Ês/Iot Note 3 | Io Note 1 range |  |  |  |
|  |  |  | Minimum Io |  |  | Maximum Io |
| dB | dB | dB | dBm / SCSSSB Note 2 |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  | SCSSSB = 120kHz | SCSSSB = 240kHz |  |  |
| ±4.5 | ±5.5 | ≥-3 | Same value as SSB_RP in Table in B.2.8.2, according to UE Power class, operating band and angle of arrival |  | N/A | -50 |
| NOTE 1: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. NOTE 2: Values based on Refsens and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. NOTE 3: In the test cases, the SSB CMR Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. |  |  |  |  |  |  |

##### 10.1.28.2.2	Relative Accuracy

The relative accuracy of SSB based L1-SINR is defined as the L1-SINR measured from one SSB configured as CMR and one IMR configured as IMR compared to the largest measured value of L1-SINR among all SSB based CMRs and IMRs of the serving cell.

The accuracy requirements are defined in Table 10.1.28.2.2-1 for SSB based CMR and NZP-IMR and in Table 10.1.28.2.2-2 for SSB based CMR and ZP-IMR.

The accuracy requirements in Tables 10.1.28.2.2-1 and 10.1.28.2.2-2 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled.

-	Conditions for L1-SINR measurements are fulfilled according to Annex B.2.8.2 for a corresponding Band for each relevant SSB based CMR and IMR.

-	The bandwidth of NZP-IMR and ZP-IMR is 48 PRBs and the density is 3.

-	The measured signals are in the directions covered by the percentile EIS spherical coverage of the UE, defined in clause 7.3.4 of TS 38.101-2 [19].

-	AWGN radio propagation conditions.

-	SSB based CMR and IMR in the test come from the same direction.

The performance with larger bandwidth of NZP-IMR and ZP-IMR is equal to or better than the accuracy requirements in Tables 10.1.28.2.2-1 and 10.1.28.2.2-2.

Table 10.1.28.2.2-1: L1-SINR relative accuracy for SSB based CMR and NZP-IMR in FR2

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB CMR Ês/Iot Note 2, Note 4 | NZP-IMR Ês/Iot Note 4 | Io Note 1 range |  |  |  |
|  |  |  |  | Minimum Io |  |  | Maximum Io |
| dB | dB | dB | dB | dBm / SCSSSB Note 3 |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 120kHz | SCSSSB = 240kHz |  |  |
| ±3.0 | ±4.0 | ≥0 | ≥0 | Same value as SSB_RP in Table in B.2.8.2, according to UE Power class, operating band and angle of arrival |  | N/A | -50 |
| NOTE 1: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. NOTE 2: The parameter SSB CMR Ês/Iot is the minimum SSB CMR Ês/Iot of the pair of SSBs to which the requirement applies. NOTE 3: Values based on Refsens and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. NOTE 4: In the test cases, the SSB CMR Ês/Iot, NZP-IMR Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. |  |  |  |  |  |  |  |

Table 10.1.28.2.2-2: L1-SINR relative accuracy for SSB based CMR and ZP-IMR in FR2

| Accuracy |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB CMR Ês/Iot Note 2, Note 4 | Io Note 1 range |  |  |  |
|  |  |  | Minimum Io |  |  | Maximum Io |
| dB | dB | dB | dBm / SCSSSB Note 3 |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  | SCSSSB = 120kHz | SCSSSB = 240kHz |  |  |
| ±3.5 | ±4.5 | ≥-3 | Same value as SSB_RP in Table in B.2.8.2, according to UE Power class, operating band and angle of arrival |  | N/A | -50 |
| NOTE 1: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. NOTE 2: The parameter SSB CMR Ês/Iot is the minimum SSB CMR Ês/Iot of the pair of SSBs to which the requirement applies. NOTE 3: Values based on Refsens and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. NOTE 4: In the test cases, the SSB CMR Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. |  |  |  |  |  |  |

#### 10.1.28.3	L1-SINR accuracy requirements with CSI-RS based CMR and dedicated IMR configured

##### 10.1.28.3.1	Absolute Accuracy

Unless otherwise specified, the requirements for absolute accuracy of CSI-RS based L1-SINR in this clause apply to all CSI-RS resources as CMR and dedicated resources configured as IMR of the serving cell configured for L1-SINR measurement.

The accuracy requirements are defined in Table 10.1.28.3.1-1 for CSI-RS based CMR and NZP-IMR and in Table 10.1.28.3.1-2 for CSI-RS based CMR and ZP-IMR.

The accuracy requirements in Tables 10.1.28.3.1-1 and 10.1.28.3.1-2 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled.

-	Conditions for L1-SINR measurements are fulfilled according to Annex B.2.8.3 for a corresponding Band for each relevant CSI-RS based CMR and IMR.

-	The bandwidth of CSI-RS as CMR, NZP-IMR and ZP-IMR is 48 PRBs and the density is 3.

-	The measured signals are in the directions covered by the percentile EIS spherical coverage of the UE, defined in clause 7.3.4 of TS 38.101-2 [19].

-	AWGN radio propagation conditions.

-	CSI-RS based CMR and IMR in the test come from the same direction.

The performance with larger bandwidth of CSI-RS as CMR, NZP-IMR and ZP-IMR is equal to or better than the accuracy requirements in Tables 10.1.28.3.1-1 and 10.1.28.3.1-2.

Table 10.1.28.3.1-1: L1-SINR absolute accuracy for CSI-RS based CMR and NZP-IMR in FR2

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | CSI-RS CMR Ês/Iot Note 3 | NZP-IMR Ês/Iot Note 3 | Io Note 1 range |  |  |  |
|  |  |  |  | Minimum Io |  |  | Maximum Io |
| dB | dB | dB | dB | dBm / SCSCSI-RS Note 2 |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSCSI-RS = 60kHz | SCSCSI-RS = 120kHz |  |  |
| ±4.0 | ±5.0 | ≥0 | ≥0 | Same value as CSI-RS_RP in Table in B.2.8.3, according to UE Power class, operating band and angle of arrival |  | N/A | -50 |
| NOTE 1: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. NOTE 2: Values based on Refsens and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. NOTE 3: In the test cases, the CSI-RS Ês/Iot, NZP-IMR Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. |  |  |  |  |  |  |  |

Table 10.1.28.3.1-2: L1-SINR absolute accuracy for CSI-RS based CMR and ZP-IMR in FR2

| Accuracy |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | CSI-RS CMR Ês/Iot Note 3 | Io Note 1 range |  |  |  |
|  |  |  | Minimum Io |  |  | Maximum Io |
| dB | dB | dB | dBm / SCSCSI-RS Note 2 |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  | SCSCSI-RS = 60kHz | SCSCSI-RS = 120kHz |  |  |
| ±4.5 | ±5.5 | ≥-3 | Same value as CSI-RS_RP in Table in B.2.8.3, according to UE Power class, operating band and angle of arrival |  | N/A | -50 |
| NOTE 1: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. NOTE 2: Values based on Refsens and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. NOTE 3: In the test cases, the CSI-RS Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. |  |  |  |  |  |  |

##### 10.1.28.3.2	Relative Accuracy

The relative accuracy of CSI-RS based L1-SINR is defined as the L1-SINR measured from one CSI-RS configured as CMR and one IMR configured as IMR compared to the largest measured value of L1-SINR among all CSI-RS based CMRs and IMRs of the serving cell.

The accuracy requirements are defined in Table 10.1.28.3.2-1 for CSI-RS based CMR and NZP-IMR and in Table 10.1.28.3.2-2 for CSI-RS based CMR and ZP-IMR.

The accuracy requirements in Tables 10.1.28.3.2-1 and 10.1.28.3.2-2 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled.

-	Conditions for L1-SINR measurements are fulfilled according to Annex B.2.8.3 for a corresponding Band for each relevant CSI-RS based CMR and IMR.

-	The bandwidth of CSI-RS as CMR, NZP-IMR and ZP-IMR is 48 PRBs and the density is 3.

-	The measured signals are in the directions covered by the percentile EIS spherical coverage of the UE, defined in clause 7.3.4 of TS 38.101-2 [19].

-	AWGN radio propagation conditions.

-	CSI-RS based CMR and IMR in the test come from the same direction.

The performance with larger bandwidth of CSI-RS as CMR, NZP-IMR and ZP-IMR is equal to or better than the accuracy requirements in Tables 10.1.28.3.2-1 and 10.1.28.3.2-2.

Table 10.1.28.3.2-1: L1-SINR relative accuracy for CSI-RS based CMR and NZP-IMR in FR2

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | CSI-RS CMR Ês/Iot Note 2, Note 4 | NZP-IMR Ês/Iot Note 4 | Io Note 1 range |  |  |  |
|  |  |  |  | Minimum Io |  |  | Maximum Io |
| dB | dB | dB | dB | dBm / SCSCSI-RS Note 3 |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSCSI-RS = 60kHz | SCSCSI-RS = 120kHz |  |  |
| ±3.0 | ±4.0 | ≥0 | ≥0 | Same value as CSI-RS_RP in Table in B.2.8.3, according to UE Power class, operating band and angle of arrival |  | N/A | -50 |
| NOTE 1: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. NOTE 2: The parameter CSI-RS CMR Ês/Iot is the minimum CSI-RS CMR Ês/Iot of the pair of CSI-RS resources to which the requirement applies. NOTE 3: Values based on Refsens and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. NOTE 4: In the test cases, the CSI-RS CMR Ês/Iot, NZP-IMR Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. |  |  |  |  |  |  |  |

Table 10.1.28.3.2-2: L1-SINR relative accuracy for CSI-RS based CMR and ZP-IMR in FR2

| Accuracy |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | CSI-RS CMR Ês/Iot Note 2, Note 4 | Io Note 1 range |  |  |  |
|  |  |  | Minimum Io |  |  | Maximum Io |
| dB | dB | dB | dBm / SCSCSI-RS Note 3 |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  | SCSCSI-RS = 60kHz | SCSCSI-RS = 120kHz |  |  |
| ±3.5 | ±4.5 | ≥-3 | Same value as CSI-RS_RP in Table in B.2.8.3, according to UE Power class, operating band and angle of arrival |  | N/A | -50 |
| NOTE 1: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. NOTE 2: The parameter CSI-RS CMR Ês/Iot is the minimum CSI-RS CMR Ês/Iot of the pair of CSI-RS resources to which the requirement applies. NOTE 3: Values based on Refsens and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. NOTE 4: In the test cases, the CSI-RS CMR Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. |  |  |  |  |  |  |

### 10.1.29	Intra-frequency RSRQ accuracy requirements under CCA

#### 10.1.29.1	Intra-frequency SS-RSRQ accuracy requirements in FR1

##### 10.1.29.1.1	Absolute SS-RSRQ Accuracy

Unless otherwise specified, the requirements for absolute accuracy of SS-RSRQ in this clause apply to a cell on the same frequency as that of the serving cell under CCA.

The accuracy requirements in Table 10.1.29.1.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3F of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for intra-frequency measurements are fulfilled according to Annex B.2.9 for a corresponding Band for each relevant SSB.

Table 10.1.29.1.1-1: SS-RSRQ intra-frequency absolute accuracy under CCA

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 3 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
| 2.5 | 4 | -3 | NR_CCA_FR1_I | -117 | -114 | N/A | -50 |
|  |  |  | NR_CCA_FR1_J | -116.5 | -113.5 |  |  |
| 3.5 | 4 | -6 | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 3: NR operating band groups are as defined in clause 3.5.2. |  |  |  |  |  |  |  |

### 10.1.30	Inter-frequency RSRQ accuracy requirements under CCA

#### 10.1.30.1	Inter-frequency SS-RSRQ accuracy requirements in FR1

##### 10.1.30.1.1	Aboslute Accuracy of SS-RSRQ

The requirements for absolute accuracy of SS-RSRQ in this clause apply to a cell on a frequency under CCA that has different carrier frequency from the serving cell.

The accuracy requirements in Table 10.1.30.1.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3F of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for inter-frequency measurements are fulfilled according to Annex B.2.10 for a corresponding Band for each relevant SSB.

Table 10.1.30.1.1-1: SS-RSRQ inter-frequency absolute accuracy under CCA

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 3 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
| 2.5 | 4 | -3 | NR_CCA_FR1_I | -117 | -114 | N/A | -50 |
|  |  |  | NR_CCA_FR1_J | -116.5 | -113.5 |  |  |
| 3.5 | 4 | -6 | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 3: NR operating band groups are as defined in clause 3.5.2. |  |  |  |  |  |  |  |

##### 10.1.30.1.2	Relative Accuracy of SS-RSRQ

The relative accuracy of SS-RSRQ in inter-frequency case is defined as the RSRQ measured from one cell on a frequency compared to the RSRP measured from another cell on a different frequency, with at least one of the two frequencies being under CCA.

The accuracy requirements in Table 10.1.30.1.2-1 are valid under the following conditions:

-	Conditions defined in clause 7.3F of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for inter-frequency measurements are fulfilled according to Annex B.2.10 for a corresponding Band for each relevant SSB.

-	|SSB_RP1dBm - SSB_RP2dBm| ≤ 27 dB

-	|Channel 1_Io -Channel 2_Io |  20 dB

Table 10.1.30.1.2-1: SS-RSRQ inter-frequency relative accuracy under CCA

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 1 range |  |  |  |  |
|  |  | Note 2 | NR operating band groups Note 4 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
| 3 | 4 | -3 | NR_CCA_FR1_I | -117 | -114 | N/A | -50 |
|  |  |  | NR_CCA_FR1_J | -116.5 | -113.5 |  |  |
| 4 | 4 | -6 | Note 3 | Note 3 | Note 3 | Note 3 | Note 3 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The parameter SSB Ês/Iot is the minimum SSB Ês/Iot of the pair of cells to which the requirement applies. NOTE 3: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 4: NR operating band groups are as defined in clause 3.5.2. |  |  |  |  |  |  |  |

### 10.1.31	Intra-frequency SINR accuracy requirements under CCA

#### 10.1.31.1	Intra-frequency SS-SINR accuracy requirements in FR1

##### 10.1.31.1.1	Absolute SS-SINR Accuracy

Unless otherwise specified, the requirements for absolute accuracy of SS-SINR in this clause apply to a cell on the same frequency as that of the serving cell under CCA.

The accuracy requirements in Table 10.1.31.1.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3F of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for intra-frequency measurements are fulfilled according to Annex B.2.9 for a corresponding Band.

Table 10.1.31.1.1-1: SS-SINR intra-frequency absolute accuracy under CCA

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 1 range |  |  |  |  |
|  |  | Note 3 | NR operating band groups Note 4 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
| 3.0 | 4 | -3 | NR_CCA_FR1_I | -117 | -114 | N/A | -50 |
|  |  |  | NR_CCA_FR1_J | -116.5 | -113.5 |  |  |
| 3.5 | 4 | -6 | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 3: The requirements apply for SSB Ês/Iot ≤ 25 dB. NOTE 4: NR operating band groups are as defined in clause 3.5.2. |  |  |  |  |  |  |  |

### 10.1.32	Inter-frequency SINR accuracy requirements under CCA

#### 10.1.32.1	Inter-frequency SS-SINR accuracy requirements in FR1

##### 10.1.32.1.1	Aboslute Accuracy of SS-SINR

The requirements for absolute accuracy of SS-SINR in this clause apply to a cell on a frequency under CCA that has different carrier frequency from the serving cell.

The accuracy requirements in Table 10.1.32.1.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3F of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for inter-frequency measurements are fulfilled according to Annex B.2.10 for a corresponding Band.

Table 10.1.32.1.1-1: SS-SINR inter-frequency absolute accuracy under CCA

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot Note 3 | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 4 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
| 3.0 | 4 | -3 | NR_CCA_FR1_I | -117 | -114 | N/A | -50 |
|  |  |  | NR_CCA_FR1_J | 116.5 | -113.5 |  |  |
| 3.5 | 4 | -6 | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 3: The requirements apply for SSB Ês/Iot ≤ 25 dB. NOTE 4: NR operating band groups are as defined in clause 3.5.2. |  |  |  |  |  |  |  |

##### 10.1.32.1.2	Relative Accuracy of SS-SINR

The relative accuracy of SS-SINR in inter frequency case is defined as the SS-SINR measured from one cell on a frequency compared to the SS-SINR measured from another cell on a different frequency, with at least one of the two frequencies being under CCA.

The accuracy requirements in Table 10.1.32.1.2-1 are valid under the following conditions:

-	Conditions defined in clause 7.3F of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for inter-frequency measurements are fulfilled according to Annex B.2.10 for a corresponding Band.

-	SSB_RP1dBm - SSB_RP2dBm| ≤ 27 dB

-	|Channel 1_Io -Channel 2_Io |  20 dB

Table 10.1.32.1.2-1: SS-SINR inter-frequency relative accuracy under CCA

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 1 range |  |  |  |  |
|  |  | Note 2,4 | NR operating band groups Note 5 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 120 kHz | SCSSSB = 240 kHz |  |  |
| 3.5 | 4 | -3 | NR_CCA_FR1_I | -117 | -114 | N/A | -50 |
|  |  |  | NR_CCA_FR1_J | -116.5 | -113.5 |  |  |
| 4 | 4 | -6 | Note 3 | Note 3 | Note 3 | Note 3 | Note 3 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The parameter SSB Ês/Iot is the minimum SSB Ês/Iot of the pair of cells to which the requirement applies. NOTE 3: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 4: The requirements apply for SSB Ês/Iot ≤ 25 dB. NOTE 5: NR operating band groups are as defined in clause 3.5.2. |  |  |  |  |  |  |  |

### 10.1.33	L1-RSRP accuracy requirements under CCA

#### 10.1.33.1	SSB based L1-RSRP accuracy requirements in FR1

##### 10.1.33.1.1	Absolute Accuracy

Unless otherwise specified, the requirements for absolute accuracy of SSB based L1-RSRP in this clause apply to all SSBs of the serving cell configured for L1-RSRP measurement under CCA.

The accuracy requirements in Table 10.1.33.1.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3F of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for L1-RSRP measurements are fulfilled according to Annex B.2.10.1 for a corresponding Band for each relevant SSB.

Table 10.1.33.1.1-1: SSB based L1-RSRP absolute accuracy under CCA

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 2 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
| ±5.0 | ±9.5 | -3 | NR_CCA_FR1_I | -117 | -114 | N/A | -70 |
|  |  |  | NR_CCA_FR1_J | -116.5 | -113.5 |  |  |
| ±8.5 | ±11.5 | -3 | NR_CCA_FR1_I | N/A | N/A | -70 | -50 |
|  |  |  | NR_CCA_FR1_J |  |  |  |  |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups are as defined in clause 3.5.2. |  |  |  |  |  |  |  |

##### 10.1.33.1.2	Relative Accuracy

The relative accuracy of SSB based L1-RSRP is defined as the L1-RSRP measured from one SSB compared to the largest measured value of L1-RSRP among all SSBs of the serving cell under CCA.

The accuracy requirements in Table 10.1.33.1.2-1 are valid under the following conditions:

-	Conditions defined in clause 7.3F of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for L1-RSRP measurements are fulfilled according to Annex B.2.10.1 for a corresponding Band for each relevant SSB.

Table 10.1.33.1.2-1: SSB based L1-RSRP relative accuracy under CCA

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot Note 2 | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 3 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
| ±3 | ±4 | -3 | NR_CCA_FR1_I | -117 | -114 | N/A | -50 |
|  |  |  | NR_CCA_FR1_J | -116.5 | -113.5 |  |  |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The parameter SSB Ês/Iot is the minimum SSB Ês/Iot of the pair of SSBs to which the requirement applies. NOTE 3: NR operating band groups are as defined in clause 3.5.2. |  |  |  |  |  |  |  |

### 10.1.34	RSSI measurements under CCA

#### 10.1.34.1	Intra-frequency absolute RSSI measurement accuracy requirements in FR1

The accuracy requirements for intra-frequency RSSI measurements on a carrier frequency under CCA are specified in Table 10.1.34.1-1. The requirements apply for any configured RSSI measDuration [2], provided that:

-	All symbols duing each RSSI measurement duration are available for RSSI sampling within the same reporting interval.

The intra-frequency RSSI measurement bandwidth is the channel bandwidth defined in Clause 4 of TS 37.213 [33], where the channel has the center frequency configured by ARFCN-valueNR.

Table 10.1.34.1-1: Intra-frequency RSSI accuracy under CCA

| Accuracy |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Io Note 1 range |  |  |  |  |
|  |  | NR operating band groups Note 2 | Minimum Io |  |  | Maximum Io |
| dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
| 3.5 | 6.5 | NR_CCA_FR1_I | -117 | -114 | N/A | -70 |
|  |  | NR_CCA_FR1_J | -116.5 | -113.5 |  |  |
| 5.5 | 8.5 | NR_CCA_FR1_I | N/A | N/A | -70 | -50 |
|  |  | NR_CCA_FR1_J |  |  |  |  |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups are as defined in clause 3.5.2. |  |  |  |  |  |  |

#### 10.1.34.2	Inter-frequency absolute RSSI measurement accuracy requirements in FR1

The accuracy requirements for inter-frequency RSSI measurements on a carrier frequency under CCA are the same as specified in clause 10.1.34.1.

The inter-frequency RSSI measurement bandwidth is the channel bandwidth defined in Clause 4 of TS 37.213 [33], where the channel has the center frequency configured by ARFCN-valueNR.

#### 10.1.34.3	RSSI measurement report mapping

The reporting range of RSSI measurement is defined from -100 dBm to -25 dBm with 1 dBm resolution.

The mapping of the measured quantity is defined in Table 10.1.34.3-1. The range in the signalling may be larger than the guaranteed accuracy range, provided that the following condition is met:

the RSSI measurement bandwidth is the channel bandwidth defined in Clause 4 of TS 37.213 [33], where the channel has the center frequency configured by ARFCN-valueNR.

Table 10.1.34.3-1: RSSI measurement report mapping

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| RSSI_00 | RSSI < -100 | dBm |
| RSSI_01 | -100 RSSI < -99 | dBm |
| RSSI_02 | -99 RSSI < -98 | dBm |
| … | … | … |
| RSSI_74 | -27 RSSI < -26 | dBm |
| RSSI_75 | -26 RSSI < -25 | dBm |
| RSSI_76 | -25 RSSI | dBm |

### 10.1.35	Channel occupancy measurements under CCA

#### 10.1.35.1	Intra-frequency channel occupancy measurement accuracy requirements in FR1

The UE shall be able to correctly evaluate the intra-frequency channel occupancy configured according to TS 38.331 [2], provided that the following conditions are met:

-	All symbols during each RSSI measurement duration are available for RSSI sampling within the same reporting interval,

-	RSSI at the UE receiver meets the following condition with respect to the configured channelOccupancyThreshold [2]:

-	RSSI at the UE receiver is below channelOccupancyThreshold-, or

-	RSSI at the UE receiver is above channelOccupancyThreshold+,

-	where  is the applicable RSSI measurement accuracy value from the RSSI measurement accuracy requirements specified in clause 10.1.34.1.

The channel occupancy measurement bandwidth is the same as the RSSI measurement bandwidth in Clause 10.1.34.1.

#### 10.1.35.2	Inter-frequency channel occupancy measurement accuracy requirements in FR1

The UE shall be able to correctly evaluate the inter-frequency channel occupancy configured according to TS 38.331 [2], provided that the following conditions are met:

-	All symbols during each RSSI measurement duration are available for RSSI sampling within the same reporting interval,

-	RSSI at the UE receiver meets the following condition with respect to the configured channelOccupancyThreshold [2]:

-	RSSI at the UE receiver is below channelOccupancyThreshold-, or

-	RSSI at the UE receiver is above channelOccupancyThreshold+,

-	where  is the applicable RSSI measurement accuracy value from the RSSI measurement accuracy requirements specified in clause 10.1.34.2.

The channel occupancy measurement bandwidth is the same as the RSSI measurement bandwidth in Clause 10.1.34.2.

### 10.1.36	Intra-frequency RSRP accuracy requirements under CCA

#### 10.1.36.1	Intra-frequency SS-RSRP accuracy requirements in FR1

##### 10.1.36.1.1	Absolute SS-RSRP Accuracy

Unless otherwise specified, the requirements for absolute accuracy of SS-RSRP in this clause apply to a cell on the same frequency as that of the serving cell under CCA.

The accuracy requirements in Table 10.1.36.1.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3F of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for intra-frequency measurements are fulfilled according to Annex B.2.8 for a corresponding Band for each relevant SSB.

Table 10.1.36.1.1-1: SS-RSRP intra-frequency absolute accuracy

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 2 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
| 4.5 | 9 | -6 | NR_CCA_FR1_I | -117 | -114 | N/A | -70 |
|  |  |  | NR_CCA_FR1_J | -116.5 | -113.5 |  |  |
| 8 | 11 | -6 | NR_CCA_FR1_I | N/A | N/A | -70 | -50 |
|  |  |  | NR_CCA_FR1_J |  |  |  |  |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups are as defined in clause 3.5.2. |  |  |  |  |  |  |  |

##### 10.1.36.1.2	Relative SS-RSRP Accuracy

The relative accuracy of SS-RSRP is defined as the SS-RSRP measured from one cell compared to the SS-RSRP measured from another cell on the same frequency, or between any two SS-RSRP levels measured on the same cell under CCA.

The accuracy requirements in Table 10.1.36.1.2-1 are valid under the following conditions:

-	Conditions defined in clause 7.3F of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for intra-frequency measurements are fulfilled according to Annex B.2.8 for a corresponding Band for each relevant SSB.

Table 10.1.36.1.2-1: SS-RSRP intra-frequency relative accuracy under CCA

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot Note 2 | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 4 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
| 2 | 3 | -3 | NR_CCA_FR1_I | -117 | -114 | N/A | -50 |
|  |  |  | NR_CCA_FR1_J | -116.5 | -113.5 |  |  |
| 3 | 3 | -6 | Note 3 | Note 3 | Note 3 | N/A | Note 3 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The parameter SSB Ês/Iot is the minimum SSB Ês/Iot of the pair of cells to which the requirement applies. NOTE 3: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 4: NR operating band groups are as defined in clause 3.5.2. |  |  |  |  |  |  |  |

### 10.1.37	Inter-frequency RSRP accuracy requirements under CCA

#### 10.1.37.1	Inter-frequency SS-RSRP accuracy requirements in FR1

##### 10.1.37.1.1	Absolute Accuracy of SS-RSRP

The requirements for absolute accuracy of SS-RSRP in this clause apply to a cell on a frequency under CCA that has different carrier frequency from the serving cell.

The accuracy requirements in Table 10.1.37.1.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3F of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for inter-frequency measurements are fulfilled according to Annex B.2.9 for a corresponding Band for each relevant SSB.

Table 10.1.37.1.1-1: SS-RSRP inter-frequency absolute accuracy under CCA

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 2 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
| 4.5 | 9 | -6 | NR_CCA_FR1_I | -117 | -114 | N/A | -70 |
|  |  |  | NR_CCA_FR1_J | -116.5 | -113.5 |  |  |
| 8 | 11 | -6 | NR_CCA_FR1_I | N/A | N/A | -70 | -50 |
|  |  |  | NR_CCA_FR1_J |  |  |  |  |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups are as defined in clause 3.5.2. |  |  |  |  |  |  |  |

##### 10.1.37.1.2	Relative Accuracy of SS-RSRP

The relative accuracy of SS-RSRP in inter frequency case is defined as the RSRP measured from one cell on a frequency compared to the RSRP measured from another cell on a different frequency, with at least one of the two frequencies being under CCA.

The accuracy requirements in Table 10.1.37.1.2-1 are valid under the following conditions:

-	Conditions defined in clause 7.3F of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for inter-frequency measurements are fulfilled according to Annex B.2.9 for a corresponding Band for each relevant SSB.

-	|SSB_RP1dBm - SSB_RP2dBm| ≤ 27 dB

-	|Channel 1_Io -Channel 2_Io |  20 dB

Table 10.1.37.1.2-1: SS-RSRP inter-frequency relative accuracy under CCA

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot Note 2 | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 3 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
| 4.5 | 6 | -6 | NR_CCA_FR1_I | -117 | -114 | N/A | -50 |
|  |  |  | NR_CCA_FR1_J | -116.5 | -113.5 |  |  |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The parameter SSB Ês/Iot is the minimum SSB Ês/Iot of the pair of cells to which the requirement applies. NOTE 3: NR operating band groups are as defined in clause 3.5.2. |  |  |  |  |  |  |  |

### 10.1.38	PRS-RSRPP Measurements

#### 10.1.38.1	Introduction

The requirements in Clause 10.1.38.2 shall apply, provided the UE has received nr-DL-AoD-RequestLocationInformation message from LMF via LPP [34] requesting the UE to report one or more DL PRS-RSRPP measurements defined in TS 38.215 [4]. The requirements in Clause 10.1.38 shall apply:

-	when UE is in RRC_CONNECTED state,

-	when UE is in RRC_INACTIVE state.

The requirements in Clause 10.1.38.2 apply for the first path PRS-RSRP measurement.

#### 10.1.38.2	Measurement Accuracy Requirements

##### 10.1.38.2.1	Absolute PRS RSRPP accuracy

The absolute accuracy requirements for PRS-RSRPP measurement for FR1 defined in Table 10.1.38.2.1-1 and Table 10.1.38.2.1-3 are valid under the following conditions:

-	Conditions defined in 38.101-1 Clause 7.3 for reference sensitivity are fulfilled.

-	PRP 1,2|dBm according to Annex B.2.14 for a corresponding Band

The absolute accuracy requirements for PRS-RSRPP measurement for FR2 defined in Table 10.1.38.2.1-2 and Table 10.1.38.2.1-4 are valid under the following conditions:

-	Conditions defined in 38.101-2 Clause 7.3 for reference sensitivity are fulfilled.

-	PRP 1,2|dBm according to Annex B.2.14 for a corresponding Band

The absolute accuracy requirements for PRS-RSRPP measurement defined in Table 10.1.38.2.1-1 and Table 10.1.38.2.1-2 apply for the UE not supporting supportedDL-PRS-ProcessingSamples [34] or LMF does not indicate UE to perform positioning measurements with reduced number of samples.

The absolute accuracy requirements for PRS-RSRPP measurement defined in Table 10.1.38.2.1-3 and Table 10.1.38.2.1-4 apply for the UE supporting supportedDL-PRS-ProcessingSamples [34].

NOTE: The requriements in this clause are derived based on:

two-tap channel defined in 38.101-4 Annex B.2.4 (a = 1, τd=0.45 µs and fD=5 Hz),

the difference between the estimated PRS-RSRPP compared to the ideal PRS-RSRPP defined as

RSRPPp∝kHkexpj2πDpkNIFFT2

where:

Hk is the effective channel frequency response (over REs occupied by PRS) measured without receiver noise.

Dp is the exact delay of the p-th path in the channel model.

Table 10.1.38.2.1-1: PRS-RSRPP absolute accuracy for FR1

| Accuracy |  | Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | PRS Ês/Iot | PRS BW | Repetition factor (TrepPRS*LPRS/KcombPRS) | Io Note 7 range |  |  |  |  |
|  |  |  |  |  | NR operating band groups Note 8 | Minimum Io Note 1 dBm / SCSPRS |  |  | Maximum Io |
| dB | dB | dB | PRB | - |  | dBm / SCSPRS |  |  | dBm/BWChannel |
|  |  |  |  |  |  | dBm/15kHz Note 6 | dBm/30kHz Note 6 | dBm/60kHz Note 6 |  |
| ±4.1 | ±8.6 | ≥-3 | ≥24 | All | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -127 | -124 | -121 | -50 |
|  |  |  |  |  | NR_FDD_FR1_B | -126.5 | -123.5 | -120.5 | -50 |
|  |  |  |  |  | NR_TDD_FR1_C | -126 | -123 | -120 | -50 |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -125.5 | -122.5 | -119.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -125 | -122 | -119 | -50 |
|  |  |  |  |  | NR_FDD_FR1_F | -124.5 | -121.5 | -118.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_G | -124 | -121 | -118 | -50 |
|  |  |  |  |  | NR_FDD_FR1_H | -123.5 | -120.5 | -117.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_N | -120.5 | -117.5 | -114.5 | -50 |
|  |  |  |  |  | Note 4 |  |  |  |  |
|  |  |  |  |  | Note 4 |  |  |  |  |
| ±5.8 | ±10.3 | ≥-13 | 24 ≤ BW ≤ 52 | All | Note 4 |  |  |  |  |
| ±4.9 | ±9.4 |  | BW > 52 | All | Note 4 |  |  |  |  |
| NOTE 1: This minimum Io condition is expressed as the average Io per RE over all REs in an OFDM symbol. NOTE 2: Void. NOTE 3: PRS bandwidth is as indicated in prs-Bandwidth in the DL-TDOA or DL-AoD assistance data defined in [34]. NOTE 4: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth ≥ 24 RB. NOTE 5: The serving cell, the reference cell, and the measured neighbour cell i are on the same carrier frequency. NOTE 6: The condition level is increased by ∆>0, when applicable, as described in Sections B.3.2 and B.3.3. NOTE 7: The Io is defined in PRS positioning subframes. The same Io range applies to PRS and non-PRS symbols. Io levels are different in PRS and non-PRS symbols within the same subframe. NOTE 8: NR operating band groups are as defined in Section 3.5.2. |  |  |  |  |  |  |  |  |  |

Table 10.1.38.2.1-2: PRS-RSRPP absolute accuracy for FR2

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | PRS Ês/Iot | PRS BW | Repetition factor (TrepPRS*LPRS/KcombPRS) | Io Note 7 range |  |  |
|  |  |  |  |  | Minimum Io Note 1 dBm / SCSPRS |  | Maximum Io |
| dB | dB | dB | PRB | - | dBm / SCSPRS |  | dBm/BWChannel |
|  |  |  |  |  | dBm/120kHz Note 6 | dBm/60kHz Note 6 |  |
| ±6.0 | ±9.0 | ≥-3 | ≥24 | All | Same value as PRP in Table B.2.14-2, according to UE Power class, operating band and angle of arrival |  | -50 |
|  |  |  |  |  | Note 4 |  |  |
|  |  |  |  |  | Note 4 |  |  |
| ±7.3 | ±10.3 | ≥-13 | 24 ≤ BW ≤ 64 | All | Note 4 |  |  |
| ±6.2 | ±9.2 |  | BW >64 | All | Note 4 |  |  |
| NOTE 1: This minimum Io condition is expressed as the average Io per RE over all REs in an OFDM symbol. NOTE 2: Void. NOTE 3: PRS bandwidth is as indicated in prs-Bandwidth in the DL-TDOA or DL-AoD assistance data defined in [34]. NOTE 4: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth ≥ 24 RB. NOTE 5: The serving cell, the reference cell, and the measured neighbour cell i are on the same carrier frequency. NOTE 6: The condition level is increased by ∆>0, when applicable, as described in Sections B.3.2 and B.3.3. NOTE 7: The Io is defined in PRS positioning subframes. The same Io range applies to PRS and non-PRS symbols. Io levels are different in PRS and non-PRS symbols within the same subframe. NOTE 8: NR operating band groups are as defined in Section 3.5.2. |  |  |  |  |  |  |  |

Table 10.1.38.2.1-3: PRS-RSRPP absolute accuracy for FR1 for reduced number of samples

| Accuracy |  | Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | PRS Ês/Iot | PRS BW | Repetition factor (TrepPRS*LPRS/KcombPRS) | Io Note 7 range |  |  |  |  |
|  |  |  |  |  | NR operating band groups Note 8 | Minimum Io Note 1 dBm / SCSPRS |  |  | Maximum Io |
| dB | dB | dB | PRB | - |  | dBm / SCSPRS |  |  | dBm/BWChannel |
|  |  |  |  |  |  | dBm/15kHz Note 6 | dBm/30kHz Note 6 | dBm/60kHz Note 6 |  |
| ±3.9 | ±8.4 | ≥0 | ≥48 | All | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -127 | -124 | -121 | -50 |
|  |  |  |  |  | NR_FDD_FR1_B | -126.5 | -123.5 | -120.5 | -50 |
|  |  |  |  |  | NR_TDD_FR1_C | -126 | -123 | -120 | -50 |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -125.5 | -122.5 | -119.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -125 | -122 | -119 | -50 |
|  |  |  |  |  | NR_FDD_FR1_F | -124.5 | -121.5 | -118.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_G | -124 | -121 | -118 | -50 |
|  |  |  |  |  | NR_FDD_FR1_H | -123.5 | -120.5 | -117.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_N | -120.5 | -117.5 | -114.5 | -50 |
|  |  |  |  |  | Note 4 |  |  |  |  |
|  |  |  |  |  | Note 4 |  |  |  |  |
| ±4.2 | ±8.7 | ≥-6 | 48 ≤ BW ≤ 52 | All | Note 4 |  |  |  |  |
| ±4.1 | ±8.6 |  | BW >52 | All | Note 4 |  |  |  |  |
| NOTE 1: This minimum Io condition is expressed as the average Io per RE over all REs in an OFDM symbol. NOTE 2: Void. NOTE 3: PRS bandwidth is as indicated in prs-Bandwidth in the DL-TDOA or DL-AoD assistance data defined in [34]. NOTE 4: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth ≥ 48 RB. NOTE 5: The serving cell, the reference cell, and the measured neighbour cell i are on the same carrier frequency. NOTE 6: The condition level is increased by ∆>0, when applicable, as described in Sections B.3.2 and B.3.3. NOTE 7: The Io is defined in PRS positioning subframes. The same Io range applies to PRS and non-PRS symbols. Io levels are different in PRS and non-PRS symbols within the same subframe. NOTE 8: NR operating band groups are as defined in Section 3.5.2. |  |  |  |  |  |  |  |  |  |

Table 10.1.38.2.1-4: PRS-RSRPP absolute accuracy for FR2 for reduced number of samples

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | PRS Ês/Iot | PRS BW | Repetition factor (TrepPRS*LPRS/KcombPRS) | Io Note 7 range |  |  |
|  |  |  |  |  | Minimum Io Note 1 dBm / SCSPRS |  | Maximum Io |
| dB | dB | dB | PRB | - | dBm / SCSPRS |  | dBm/BWChannel |
|  |  |  |  |  | dBm/120kHz Note 6 | dBm/60kHz Note 6 |  |
| ± 5.9 | ± 8.9 | ≥0 | ≥48 | All | Same value as PRP in Table B.2.14-2, according to UE Power class, operating band and angle of arrival |  | -50 |
|  |  |  |  |  | Note 4 |  |  |
|  |  |  |  |  | Note 4 |  |  |
| ±5.6 | ±8.6 | ≥-6 | 48 ≤ BW ≤ 64 | All | Note 4 |  |  |
| ±5.4 | ±8.4 |  | BW >64 | All | Note 4 |  |  |
| NOTE 1: This minimum Io condition is expressed as the average Io per RE over all REs in an OFDM symbol. NOTE 2: Void. NOTE 3: PRS bandwidth is as indicated in prs-Bandwidth in the DL-TDOA or DL-AoD assistance data defined in [34]. NOTE 4: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth ≥ 48 RB. NOTE 5: The serving cell, the reference cell, and the measured neighbour cell i are on the same carrier frequency. NOTE 6: The condition level is increased by ∆>0, when applicable, as described in Sections B.3.2 and B.3.3. NOTE 7: The Io is defined in PRS positioning subframes. The same Io range applies to PRS and non-PRS symbols. Io levels are different in PRS and non-PRS symbols within the same subframe. NOTE 8: NR operating band groups are as defined in Section 3.5.2. |  |  |  |  |  |  |  |

#### 10.1.38.3	Report mapping

##### 10.1.38.3.1	Absolute PRS-RSRPP Measurement Report Mapping

The reporting range of absolute PRS-RSRPP measurement is defined from -156 dBm to -31 dBm with 1 dB resolution.

The mapping of measured quantity is defined in Table 10.1.38.3.1-1. The range in the signalling may be larger than the guaranteed accuracy range.

The UE capable of additionalPathsExtSupport-r17 can report the PRS-RSRPP measurement of up to its supported number of additional paths.

Table 10.1.38.3.1-1: Measurement report mapping for PRS-RSRPP

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| PRS_RSRPP_0 | PRS-RSRPP<-156 | dBm |
| PRS_RSRPP_1 | -156≤PRS-RSRPP<-155 | dBm |
| PRS_RSRPP_2 | -155≤PRS-RSRPP<-154 | dBm |
| PRS_RSRPP_3 | -154≤PRS-RSRPP<-153 | dBm |
| PRS_RSRPP_4 | -153≤PRS-RSRPP<-152 | dBm |
| PRS_RSRPP_5 | -152≤PRS-RSRPP<-151 | dBm |
| PRS_RSRPP_6 | -151≤PRS-RSRPP<-150 | dBm |
| PRS_RSRPP_7 | -150≤PRS-RSRPP<-149 | dBm |
| PRS_RSRPP_8 | -149≤PRS-RSRPP<-148 | dBm |
| PRS_RSRPP_9 | -148≤PRS-RSRPP<-147 | dBm |
| PRS_RSRPP_10 | -147≤PRS-RSRPP<-146 | dBm |
| PRS_RSRPP_11 | -146≤PRS-RSRPP<-145 | dBm |
| PRS_RSRPP_12 | -145≤PRS-RSRPP<-144 | dBm |
| PRS_RSRPP_13 | -144≤PRS-RSRPP<-143 | dBm |
| PRS_RSRPP_14 | -143≤PRS-RSRPP<-142 | dBm |
| PRS_RSRPP_15 | -142≤PRS-RSRPP<-141 | dBm |
| PRS_RSRPP_16 | -141≤PRS-RSRPP<-140 | dBm |
| PRS_RSRPP_17 | -140≤PRS-RSRPP<-139 | dBm |
| PRS_RSRPP_18 | -139≤PRS-RSRPP<-138 | dBm |
| … | … | … |
| PRS_RSRPP_111 | -46≤PRS-RSRPP<-45 | dBm |
| PRS_RSRPP_112 | -45≤PRS-RSRPP<-44 | dBm |
| PRS_RSRPP_113 | -44≤PRS-RSRPP<-43 | dBm |
| PRS_RSRPP_114 | -43≤PRS-RSRPP<-42 | dBm |
| PRS_RSRPP_115 | -42≤PRS-RSRPP<-41 | dBm |
| PRS_RSRPP_116 | -41≤PRS-RSRPP<-40 | dBm |
| PRS_RSRPP_117 | -40≤PRS-RSRPP<-39 | dBm |
| PRS_RSRPP_118 | -39≤PRS-RSRPP<-38 | dBm |
| PRS_RSRPP_119 | -38≤PRS-RSRPP<-37 | dBm |
| PRS_RSRPP_120 | -37≤PRS-RSRPP<-36 | dBm |
| PRS_RSRPP_121 | -36≤PRS-RSRPP<-35 | dBm |
| PRS_RSRPP_122 | -35≤PRS-RSRPP<-34 | dBm |
| PRS_RSRPP_123 | -34≤PRS-RSRPP<-33 | dBm |
| PRS_RSRPP_124 | -33≤PRS-RSRPP<-32 | dBm |
| PRS_RSRPP_125 | -32≤PRS-RSRPP<-31 | dBm |
| PRS_RSRPP_126 | -31≤PRS-RSRPP | dBm |

##### 10.1.38.3.2	Differential Report Mapping for PRS-RSRPP Measurement

The reporting range of differential PRS-RSRPP is defined from -30 dB to 30 dB with 1 dB resolution.

The mapping of measured quantity is defined in Table 10.1.38.3.2-1. The range in the signalling may be larger than the guaranteed accuracy range.

For differential reporting, PRS-RSRPP is reported as the difference in dB with respect to the first reported PRS-RSRPP.

Table 10.1.38.3.2-1: Measurement report mapping for differential PRS-RSRPP

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| DIFFRSRPP_0 | -30≥ΔRSRPP | dB |
| DIFFRSRPP_1 | -29≥ΔRSRPP>-30 | dB |
| DIFFRSRPP_2 | -28≥ΔRSRPP>-29 | dB |
| DIFFRSRPP_3 | -27≥ΔRSRPP>-28 | dB |
| DIFFRSRPP_4 | -26≥ΔRSRPP>-27 | dB |
| DIFFRSRPP_5 | -25≥ΔRSRPP>-26 | dB |
| DIFFRSRPP_6 | -24≥ΔRSRPP>-25 | dB |
| DIFFRSRPP_7 | -23≥ΔRSRPP>-24 | dB |
| DIFFRSRPP_8 | -22≥ΔRSRPP>-23 | dB |
| DIFFRSRPP_9 | -21≥ΔRSRPP>-22 | dB |
| DIFFRSRPP_10 | -20≥ΔRSRPP>-21 | dB |
| DIFFRSRPP_11 | -19≥ΔRSRPP>-20 | dB |
| DIFFRSRPP_12 | -18≥ΔRSRPP>-19 | dB |
| DIFFRSRPP_13 | -17≥ΔRSRPP>-18 | dB |
| DIFFRSRPP_14 | -16≥ΔRSRPP>-17 | dB |
| … | … | … |
| DIFFRSRPP_25 | -5≥ΔRSRPP>-6 | dB |
| DIFFRSRPP_26 | -4≥ΔRSRPP>-5 | dB |
| DIFFRSRPP_27 | -3≥ΔRSRPP>-4 | dB |
| DIFFRSRPP_28 | -2≥ΔRSRPP>-3 | dB |
| DIFFRSRPP_29 | -1≥ΔRSRPP>-2 | dB |
| DIFFRSRPP_30 | 0≥ΔRSRPP>-1 | dB |
| DIFFRSRPP_31 | 1≥ΔRSRPP>0 | dB |
| DIFFRSRPP_32 | 2≥ΔRSRPP>1 | dB |
| DIFFRSRPP_33 | 3≥ΔRSRPP>2 | dB |
| DIFFRSRPP_34 | 4≥ΔRSRPP>3 | dB |
| DIFFRSRPP_35 | 5≥ΔRSRPP>4 | dB |
| DIFFRSRPP_36 | 6≥ΔRSRPP>5 | dB |
| … | … | … |
| DIFFRSRPP_47 | 17≥ΔRSRPP>16 | dB |
| DIFFRSRPP_48 | 18≥ΔRSRPP>17 | dB |
| DIFFRSRPP_49 | 19≥ΔRSRPP>18 | dB |
| DIFFRSRPP_50 | 20≥ΔRSRPP>19 | dB |
| DIFFRSRPP_51 | 21≥ΔRSRPP>20 | dB |
| DIFFRSRPP_52 | 22≥ΔRSRPP>21 | dB |
| DIFFRSRPP_53 | 23≥ΔRSRPP>-22 | dB |
| DIFFRSRPP_54 | 24≥ΔRSRPP>23 | dB |
| DIFFRSRPP_55 | 25≥ΔRSRPP>24 | dB |
| DIFFRSRPP_56 | 26≥ΔRSRPP>25 | dB |
| DIFFRSRPP_57 | 27≥ΔRSRPP>26 | dB |
| DIFFRSRPP_58 | 28≥ΔRSRPP>27 | dB |
| DIFFRSRPP_59 | 29≥ΔRSRPP>28 | dB |
| DIFFRSRPP_60 | 30≥ΔRSRPP>29 | dB |
| DIFFRSRPP_61 | ΔRSRPP>30 | dB |

### 10.1.39	UE Rx-Tx time difference measurements for RTT-based PDC

#### 10.1.39.1	Void

#### 10.1.39.2		Measurement Accuracy Requirements for PRS

The error in the reported value of UE Rx-Tx time difference measurement, including both the measurement error and the reporting quantization error, shall be within the accuracy requirements specified in this clause.

The UE Rx-Tx time difference measurement accuracy requirements in this clause shall not apply, if:

-	NTA_offset defined in Table 7.1.2-2 changes during the UE Rx-Tx measurement period or

-	if the uplink transmission timing changes during the UE Rx-Tx measurement period due to the network-configured Timing Advance.

The UE Rx-Tx time difference measurement accuracy requirements in this clause shall apply provided that:

-	The UE transmits SRS within [-160, 160] msec of at least one PDC DL PRS resource from the serving cell (PCell).

When a serving cell change occurs during the UE Rx-Tx measurement period, the UE Rx-Tx time difference measurement accuracy requirements in this clause shall apply provided that the serving cell change does not impact SRS configuration for the UE Rx-Tx measurement.

The accuracy requirements in Table 10.1.39.2-1 for FR1 are valid under the following conditions:

Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

PRP|dBm according to Annex  B.2.14 for a corresponding Band.

AWGN propagation condition.

Table 10.1.39.2-1: UE Rx-Tx time difference measurement accuracy in FR1 in AWGN

| Accuracy | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | PRS Ês/Iot | Minimum PRS bandwidth | PRS SCS | PRS resource repetition (TrepPRS*LPRS/KcombPRSNote 3 | NR operating band groupsNote 2 | IoNote 4 range |  |
|  |  |  |  |  |  | Minimum IoNote 1 | Maximum Io |
| TcNote 5 | dB | RB | kHz |  |  | dBm / SCSPRS | dBm/BW |
| ± (78++ℇ) | -3 | ≥24 | 15 | ≥4 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -50 |
|  |  |  |  |  | NR_FDD_FR1_B | -120.5 |  |
|  |  |  |  |  | NR_TDD_FR1_C | -120 |  |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 |  |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 |  |
|  |  |  |  |  | NR_FDD_FR1_F | -118.5 |  |
|  |  |  |  |  | NR_FDD_FR1_G | -118 |  |
|  |  |  |  |  | NR_FDD_FR1_H | -117.5 |  |
|  |  |  |  |  | NR_FDD_FR1_N | -114.5 |  |
| ± (59++ℇ) |  | ≥52 |  | ≥1 | Note 6 | NOTE 6 | NOTE 6 |
| ± (30++ℇ) |  | >104 |  | ≥1 | Note 6 | NOTE 6 | NOTE 6 |
| ± (57++ℇ) |  | ≥24 | 30 | ≥4 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -118 | -50 |
|  |  |  |  |  | NR_FDD_FR1_B | -117.5 |  |
|  |  |  |  |  | NR_TDD_FR1_C | -117 |  |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -116.5 |  |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -116 |  |
|  |  |  |  |  | NR_FDD_FR1_F | -115.5 |  |
|  |  |  |  |  | NR_FDD_FR1_G | -115 |  |
|  |  |  |  |  | NR_FDD_FR1_H | -114.5 |  |
|  |  |  |  |  | NR_FDD_FR1_N | -111.5 |  |
| ± (30++ℇ) |  | ≥48 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± (15++ℇ) |  | ≥132 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± (29++ℇ) |  | ≥24 | 60 | ≥4 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -115 | -50 |
|  |  |  |  |  | NR_FDD_FR1_B | -114.5 |  |
|  |  |  |  |  | NR_TDD_FR1_C | -114 |  |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -113.5 |  |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -113 |  |
|  |  |  |  |  | NR_FDD_FR1_F | -113.5 |  |
|  |  |  |  |  | NR_FDD_FR1_G | -113 |  |
|  |  |  |  |  | NR_FDD_FR1_H | -111.5 |  |
|  |  |  |  |  | NR_FDD_FR1_N | -108.5 |  |
| ± (15++ℇ) |  | ≥64 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± (7++ℇ) |  | ≥132 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| NOTE 1: This minimum Io condition is expressed as the average Io per RE over all REs in an OFDM symbol. NOTE 2: NR operating band groups are as defined in Section 3.5. NOTE 3: TrepPRS, LPRS ,KcombPRS are configured by higher layer parameter dl-PRS-ResourceRepetitionFactor, dl-PRS-NumSymbols and dl-PRS-CombSizeN defined in TS 37.355 [34]. NOTE 4: The Io is defined in PRS slots. The same Io range applies to PRS and non-PRS symbols. Io levels are different in PRS and non-PRS symbols within the same slot. NOTE 5: Tc is the basic timing unit defined in TS 38.211 [6]. NOTE 6: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth of the smallest RB number for the corresponding SCS. NOTE 7: is the margin determined from Table 10.1.39.2-3. NOTE 8: ℇ is the margin for reporting quantitization error and ℇ=16 Tc. |  |  |  |  |  |  |  |

The accuracy requirements in Table 10.1.39.2-2 for FR2 are valid under the following conditions:

Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled.

PRP|dBm according to Annex B.2.14 for a corresponding Band.

AWGN propagation condition.

Table 10.1.39.2-2: UE Rx-Tx time difference measurement accuracy in FR2 in AWGN

| Accuracy | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | PRS Ês/Iot | Minimum PRS bandwidth | PRS SCS | PRS resource repetition(TrepPRS*LPRS/KcombPRSNote 3 | IoNote 4 range |  |
|  |  |  |  |  | Minimum IoNote 1 | Maximum Io |
| TcNote 5 | dB | RB | kHz |  | dBm / SCSPRS | dBm/BWChannel |
| ± (22++ℇ) | -3 | ≥24 | 60 | ≥1 | Same value as PRP in Table B.2.14-2, according to UE Power class, operating band and angle of arrival | -50 |
| ± (15++ℇ) |  | ≥64 |  | ≥1 | NOTE 6 | NOTE 6 |
| ± (7++ℇ) |  | ≥132 |  | ≥1 | NOTE 6 | NOTE 6 |
| ± (12++ℇ) |  | ≥32 | 120 | ≥1 | Same value as PRP in Table B.2.14-2, according to UE Power class, operating band and angle of arrival | -50 |
| ± (7++ℇ) |  | ≥64 |  | ≥1 | NOTE 6 | NOTE 6 |
| ± (4++ℇ) |  | ≥128 |  | ≥1 | NOTE 6 | NOTE 6 |
| NOTE 1: This minimum Io condition is expressed as the average Io per RE over all REs in an OFDM symbol. NOTE 2: NR operating band groups are as defined in Section 3.5. NOTE 3: TrepPRS, LPRS ,KcombPRS are configured by higher layer parameter dl-PRS-ResourceRepetitionFactor, dl-PRS-NumSymbols and dl-PRS-CombSizeNdefined in TS 37.355 [34]. NOTE 4: The Io is defined in PRS slots. The same Io range applies to PRS and non-PRS symbols. Io levels are different in PRS and non-PRS symbols within the same slot. NOTE 5: Tc is the basic timing unit defined in TS 38.211 [6]. NOTE 6: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth of the smallest RB number for the corresponding SCS. NOTE 7: is the margin determined from Table 10.1.39.2-4. NOTE 8: ℇ is the margin for reporting quantitization error and ℇ=16 Tc. |  |  |  |  |  |  |

Table 10.1.39.2-3: Margin for UE Rx-Tx time difference measurement accuracy in FR1

| Min(PRS BW, SRS BW) (RB) |  |  | Margin (Tc Note 1) |
| --- | --- | --- | --- |
| SCS = 15 kHz | SCS = 30 kHz | SCS = 60 kHz |  |
| ≥ 24 | N/A | N/A | 160 |
| ≥ 52 | ≥ 24 | N/A | 80 |
| ≥ 104 | ≥ 48 | ≥ 24 | 56 |
| N/A | ≥ 132 | ≥ 64 | 24 |
| N/A | N/A | ≥ 132 | 24 |
| NOTE 1: Tc is the basic timing unit defined in TS 38.211 [6]. NOTE 2: If SRS and PRS have different SCS, the margin corresponding to the smallest RS BW in MHz applies. |  |  |  |

Table 10.1.39.2-4: Margin for UE Rx-Tx time difference measurement accuracy in FR2

| Min(PRS BW, SRS BW) (MHz) |  | Margin (Tc Note 1) |
| --- | --- | --- |
| SCS = 60 kHz | SCS = 120 kHz |  |
| ≥ 24 | N/A | 76 |
| ≥ 64 | ≥ 32 | 32 |
| ≥ 132 | ≥ 64 | 24 |
| N/A | ≥ 128 | 20 |
| NOTE 1: Tc is the basic timing unit defined in TS 38.211 [6]. NOTE 2: If SRS and PRS have different SCS, the margin corresponding to the smallest RS BW in MHz applies. |  |  |

#### 10.1.39.3		Measurement Accuracy Requirements for TRS

The error in the reported value of UE Rx-Tx time difference measurement, including both the measurement error and the reporting quantization error, shall be within the accuracy requirements specified in this clause.

The UE Rx-Tx time difference measurement accuracy requirements in this clause shall not apply, if:

-	NTA_offset defined in Table 7.1.2-2 changes during the UE Rx-Tx measurement period or

-	if the uplink transmission timing changes during the UE Rx-Tx measurement period due to the network-configured Timing Advance.

The UE Rx-Tx time difference measurement accuracy requirements in this clause shall apply provided that:

-	The UE transmits SRS within [-160, 160] msec of at least one PDC TRS resource from the serving cell (PCell).

When a serving cell change occurs during the UE Rx-Tx measurement period, the UE Rx-Tx time difference measurement accuracy requirements in this clause shall apply provided that the serving cell change does not impact SRS configuration for the UE Rx-Tx measurement.

The accuracy requirements in Table 10.1.39.3-1 for FR1 are valid under the following conditions:

Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

PRP|dBm according to Annex B.2.13 for a corresponding Band.

AWGN propagation condition.

Table 10.1.39.3-1: UE Rx-Tx time difference measurement accuracy in FR1 in AWGN

| Accuracy | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | TRS Ês/Iot | Minimum TRS bandwidth | TRS SCS | NR operating band groupsNote 2 | IoNote 3 range |  |
|  |  |  |  |  | Minimum IoNote 1 | Maximum Io |
| TcNote 4 | dB | RB | kHz |  | dBm / SCSTRS | dBm/BW |
| ± (116++ℇ) | -3 | ≥24 | 15 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -50 |
|  |  |  |  | NR_FDD_FR1_B | -120.5 |  |
|  |  |  |  | NR_TDD_FR1_C | -120 |  |
|  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 |  |
|  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 |  |
|  |  |  |  | NR_FDD_FR1_F | -118.5 |  |
|  |  |  |  | NR_FDD_FR1_G | -118 |  |
|  |  |  |  | NR_FDD_FR1_H | -117.5 |  |
|  |  |  |  | NR_FDD_FR1_N | -114.5 |  |
| ±(60++ℇ) |  | ≥52 |  | Note 5 | NOTE 5 | NOTE 5 |
| ±(29++ℇ) |  | >104 |  | Note 5 | NOTE 5 | NOTE 5 |
| ±(56++ℇ) |  | ≥24 | 30 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -118 | -50 |
|  |  |  |  | NR_FDD_FR1_B | -117.5 |  |
|  |  |  |  | NR_TDD_FR1_C | -117 |  |
|  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -116.5 |  |
|  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -116 |  |
|  |  |  |  | NR_FDD_FR1_F | -115.5 |  |
|  |  |  |  | NR_FDD_FR1_G | -115 |  |
|  |  |  |  | NR_FDD_FR1_H | -114.5 |  |
|  |  |  |  | NR_FDD_FR1_N | -111.5 |  |
| ±(29++ℇ) |  | ≥48 |  | NOTE 5 | NOTE 5 | NOTE 5 |
| ±(15++ℇ) |  | ≥132 |  | NOTE 5 | NOTE 5 | NOTE 5 |
| ±(29++ℇ) |  | ≥24 | 60 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -115 | -50 |
|  |  |  |  | NR_FDD_FR1_B | -114.5 |  |
|  |  |  |  | NR_TDD_FR1_C | -114 |  |
|  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -113.5 |  |
|  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -113 |  |
|  |  |  |  | NR_FDD_FR1_F | -113.5 |  |
|  |  |  |  | NR_FDD_FR1_G | -113 |  |
|  |  |  |  | NR_FDD_FR1_H | -111.5 |  |
|  |  |  |  | NR_FDD_FR1_N | -108.5 |  |
| ±(14++ℇ) |  | ≥64 |  | NOTE 5 | NOTE 5 | NOTE 5 |
| ±(7++ℇ) |  | ≥132 |  | NOTE 5 | NOTE 5 | NOTE 5 |
| NOTE 1: This minimum Io condition is expressed as the average Io per RE over all REs in an OFDM symbol. NOTE 2: NR operating band groups are as defined in Section 3.5. NOTE 3: The Io is defined in TRS slots. The same Io range applies to TRS and non-TRS symbols. Io levels are different in TRS and non-TRS symbols within the same slot. NOTE 4: Tc is the basic timing unit defined in TS 38.211 [6]. NOTE 5: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the TRS bandwidth of the smallest RB number for the corresponding SCS. NOTE 6: is the margin determined from Table 10.1.39.3-3. NOTE 7: ℇ is the margin for reporting quantitization error and ℇ=16 Tc. |  |  |  |  |  |  |

The accuracy requirements in Table 10.1.39.3-2 for FR2 are valid under the following conditions:

Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled.

PRP|dBm according to Annex B.2.13 for a corresponding Band.

AWGN propagation condition.

Table 10.1.39.3-2: UE Rx-Tx time difference measurement accuracy in FR2 in AWGN

| Accuracy | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | TRS Ês/Iot | Minimum TRS bandwidth | TRS SCS | IoNote 3 range |  |
|  |  |  |  | Minimum IoNote 1 | Maximum Io |
| TcNote 4 | dB | RB | kHz | dBm / SCSTRS | dBm/BWChannel |
| ±(29++ℇ) | -3 | ≥24 | 60 | Same value as PRP in Table B.2.14-2, according to UE Power class, operating band and angle of arrival | -50 |
| ±(14++ℇ) |  | ≥64 |  | NOTE 5 | NOTE 5 |
| ±(7++ℇ) |  | ≥132 |  | NOTE 5 | NOTE 5 |
| ±(15++ℇ) |  | ≥32 | 120 | Same value as PRP in Table B.2.14-2, according to UE Power class, operating band and angle of arrival | -50 |
| ±(7++ℇ) |  | ≥64 |  | NOTE 5 | NOTE 5 |
| ±(4++ℇ) |  | ≥128 |  | NOTE 5 | NOTE 5 |
| NOTE 1: This minimum Io condition is expressed as the average Io per RE over all REs in an OFDM symbol. NOTE 2: NR operating band groups are as defined in Section 3.5. NOTE 3: The Io is defined in TRS slots. The same Io range applies to TRS and non-TRS symbols. Io levels are different in TRS and non-TRS symbols within the same slot. NOTE 4: Tc is the basic timing unit defined in TS 38.211 [6]. NOTE 5: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the TRS bandwidth of the smallest RB number for the corresponding SCS. NOTE 6: is the margin determined from Table 10.1.39.3-4. NOTE 7: ℇ is the margin for reporting quantitization error and ℇ=16 Tc. |  |  |  |  |  |

Table 10.1.39.3-3: Margin for UE Rx-Tx time difference measurement accuracy in FR1

| Min(TRS BW, SRS BW) (RB) |  |  | Margin (Tc Note 1) |
| --- | --- | --- | --- |
| SCS = 15 kHz | SCS = 30 kHz | SCS = 60 kHz |  |
| ≥ 24 | N/A | N/A | 160 |
| ≥ 52 | ≥ 24 | N/A | 80 |
| ≥ 104 | ≥ 48 | ≥ 24 | 56 |
| N/A | ≥ 132 | ≥ 64 | 24 |
| N/A | N/A | ≥ 132 | 24 |
| NOTE 1: Tc is the basic timing unit defined in TS 38.211 [6]. NOTE 2: If SRS and TRS have different SCS, the margin corresponding to the smallest RS BW in MHz applies. |  |  |  |

Table 10.1.39.3-4: Margin for UE Rx-Tx time difference measurement accuracy in FR2

| Min(TRS BW, SRS BW) (MHz) |  | Margin (Tc Note 1) |
| --- | --- | --- |
| SCS = 60 kHz | SCS = 120 kHz |  |
| ≥ 24 | N/A | 76 |
| ≥ 64 | ≥ 32 | 32 |
| ≥ 132 | ≥ 64 | 24 |
| N/A | ≥ 128 | 20 |
| NOTE 1: Tc is the basic timing unit defined in TS 38.211 [6]. NOTE 2: If SRS and TRS have different SCS, the margin corresponding to the smallest RS BW in MHz applies. |  |  |

### 10.1.40	Void

### 10.1.41	FR1 DPC report

The FR1 DPC report mapping is defined in this clause.

#### 10.1.41.1	Report mapping

Table 10.1.41.1-1 defines the FR1 DPC report mapping.

Table 10.1.41.1-1 Mapping of FR1 DPC

| Reported value | Reported quantity value | Unit |
| --- | --- | --- |
| DPC_00 | 0 | dB |
| DPC_03 | 3 | dB |
| DPC_06 | 6 | dB |

### 10.1.42	TDCP Measurement Report Mapping

The reporting range of TDCP amplitude is defined from 0 to 1. The reporting range of TDCP phase is 0 to 2p. The mapping of measured quantity is defined in Tables 10.1.42-1, 10.1.42-2. The range in the signalling may be larger than the guaranteed accuracy range.

Table 10.1.42-1: TDCP amplitude measurement report mapping

| Reported value | Measured quantity value |
| --- | --- |
| TDCP_0 | 1- 11282 < TDCP ≤1 |
| TDCP_1 | 1-1128 < TDCP ≤ 1- 11282 |
| TDCP_2 | 1-1642 < TDCP ≤ 1- 1128 |
| TDCP_3 | 1-164 < TDCP ≤ 1- 1642 |
| TDCP_4 | 1-1322 < TDCP ≤ 1- 164 |
| TDCP_5 | 1-132 < TDCP ≤ 1- 1322 |
| TDCP_6 | 1-1162 < TDCP ≤ 1- 132 |
| TDCP_7 | 1-116 < TDCP ≤ 1- 1162 |
| TDCP_8 | 1-182 < TDCP ≤ 1- 116 |
| TDCP_9 | 1-18 < TDCP ≤ 1- 182 |
| TDCP_10 | 1-142 < TDCP ≤ 1- 18 |
| TDCP_11 | 1-14 < TDCP ≤ 1- 142 |
| TDCP_12 | 1-122 < TDCP ≤ 1- 14 |
| TDCP_13 | 1-12 < TDCP ≤ 1- 122 |
| TDCP_14 | 1-12 < TDCP ≤ 1- 12 |
| TDCP_15 | 0 ≤ TDCP ≤ 1- 12 |

Table 10.1.42-2: TDCP phase measurement report mapping

| Reported value | Measured quantity value |
| --- | --- |
| TDCP_PH_0 | 0 ≤ TDCP_PH < 2π16 |
| TDCP_PH_1 | 2π16≤ TDCP_PH < 2π*216 |
| TDCP_PH_2 | 2π*216 ≤ TDCP_PH < 2π*316 |
| TDCP_PH_3 | 2π*316 ≤ TDCP_PH < 2π*416 |
| TDCP_PH_4 | 2π*416 ≤ TDCP_PH < 2π*516 |
| TDCP_PH_5 | 2π*516≤ TDCP_PH < 2π*616 |
| TDCP_PH_6 | 2π*616 ≤ TDCP_PH < 2π*716 |
| TDCP_PH_7 | 2π*716 ≤ TDCP_PH < 2π*816 |
| TDCP_PH_8 | 2π*816 ≤ TDCP_PH < 2π*916 |
| TDCP_PH_9 | 2π*916 ≤ TDCP_PH < 2π*1016 |
| TDCP_PH_10 | 2π*1016 ≤ TDCP_PH < 2π*1116 |
| TDCP_PH_11 | 2π*1116≤ TDCP_PH < 2π*1216 |
| TDCP_PH_12 | 2π*1216 ≤ TDCP_PH < 2π*1316 |
| TDCP_PH_13 | 2π*1316 ≤ TDCP_PH < 2π*1416 |
| TDCP_PH_14 | 2π*1416 ≤ TDCP_PH < 2π*1516 |
| TDCP_PH_15 | 2π*1516 ≤ TDCP_PH ≤ 2π |

## 10.1A	NR measurements for RedCap

### 10.1A.1	Introduction

The requirements in this section are applicable for RedCap UE as follows:

-	intra-frequency requirements apply for PCell measurements in SA,

-	inter-frequency requirements apply for non-serving cell measurements on NR carrier frequencies.

-	inter-frequency requirements apply for measurements from one cell on a frequency compared to the measurement from another cell on a different frequency.

The accuracy requirements in this clause are applicable for AWGN radio propagation conditions. The accuracy requirements of RSRP, RSRQ amd SINR are applicable provided that reference SSB is not changed during measurement period.

### 10.1A.2	Intra-frequency RSRP accuracy requirements for FR1

#### 10.1A.2.1	Intra-frequency SS-RSRP accuracy requirements

##### 10.1A.2.1.1	Absolute SS-RSRP Accuracy

The accuracy requirements in clause 10.1.2.1.1 shall apply when RedCap UE is capable of 2Rx. When UE is only required to support 1RX, the absolute accuracy requirements in Table 10.1A.2.1.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3I of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for intra-frequency measurements are fulfilled according to Annex B.2.15 for a corresponding Band for each relevant SSB.

Table 10.1A.2.1.1-1: SS-RSRP Intra frequency absolute accuracy for 1Rx RedCap UE in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 2 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
|  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | N/A | -70 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | N/A | -70 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | N/A | -70 |
| 5.5 | 10 | -6 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | N/A | -70 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | N/A | -70 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_N | -114.5 | -111.5 | N/A | -70 |
| 9 | 12 | -6 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A, NR_FDD_FR1_B, NR_TDD_FR1_C, NR_FDD_FR1_D, NR_TDD_FR1_D, NR_FDD_FR1_E, NR_TDD_FR1_E, NR_FDD_FR1_F, NR_FDD_FR1_G, NR_FDD_FR1_H, NR_FDD_FR1_N | N/A | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |

##### 10.1A.2.1.2	Relative SS-RSRP Accuracy

The accuracy requirements in clause 10.1.2.1.2 shall apply when RedCap UE is capable of 2Rx. When UE is only required to support 1RX, the absolute accuracy requirements in Table 10.1A.2.1.2-1 are valid under the following conditions:

-	Conditions defined in clause 7.3I of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for intra-frequency measurements are fulfilled according to Annex B.2.15 for a corresponding Band for each relevant SSB.

Table 10.1A.2.1.2-1: SS-RSRP Intra frequency relative accuracy for 1Rx RedCap UE in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot Note 2 | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 4 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
|  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | N/A | -50 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | N/A | -50 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | N/A | -50 |
| 3 | 4 | -3 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | N/A | -50 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | N/A | -50 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_N | -114.5 | -111.5 | N/A | -50 |
| 4 | 4 | -6 | Note 3 | Note 3 | Note 3 | N/A | Note 3 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The parameter SSB Ês/Iot is the minimum SSB Ês/Iot of the pair of cells to which the requirement applies. NOTE 3: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 4: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |

### 10.1A.3	Intra-frequency RSRP accuracy requirements for FR2

#### 10.1A.3.1	Intra-frequency SS-RSRP accuracy requirements

##### 10.1A.3.1.1	Absolute SS-RSRP Accuracy

The accuracy requirements in clause 10.1.3.1.1 shall apply.

##### 10.1A.3.1.2	Relative SS-RSRP Accuracy

The accuracy requirements in clause 10.1.3.1.2 shall apply.

### 10.1A.4	Inter-frequency RSRP accuracy requirements for FR1

#### 10.1A.4.1	Inter-frequency SS-RSRP accuracy requirements

##### 10.1A.4.1.1	Absolute Accuracy of SS-RSRP in FR1

The accuracy requirements in clause 10.1.4.1.1 shall apply when RedCap UE is capable of 2Rx. When UE is only required to support 1RX, the absolute accuracy requirements in Table 10.1A.4.1.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3I of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for inter-frequency measurements are fulfilled according to Annex B.2.16 for a corresponding Band for each relevant SSB.

Table 10.1A.4.1.1-1: SS-RSRP Inter frequency Absolute accuracy for 1Rx RedCap UE in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot Note 2 | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 3 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
|  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | N/A | -70 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | N/A | -70 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | N/A | -70 |
| 5.5 | 10 | -6 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | N/A | -70 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | N/A | -70 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_N | -114.5 | -111.5 | N/A | -70 |
| 9 | 12 | -6 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A, NR_FDD_FR1_B, NR_TDD_FR1_C, NR_FDD_FR1_D, NR_TDD_FR1_D, NR_FDD_FR1_E, NR_TDD_FR1_E, NR_FDD_FR1_F, NR_FDD_FR1_G, NR_FDD_FR1_H | N/A | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: Void NOTE 3: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |

##### 10.1A.4.1.2	Relative Accuracy of SS-RSRP in FR1

The accuracy requirements in clause 10.1.4.1.2 shall apply when RedCap UE is capable of 2Rx. When UE is only required to support 1RX, the absolute accuracy requirements in Table 10.1A.4.1.2-1 are valid under the following conditions:

-	Conditions defined in clause 7.3I of TS 38.101-1 [18] Clause 7.3 for reference sensitivity are fulfilled.

-	Conditions for inter-frequency measurements are fulfilled according to Annex B.2.16 for a corresponding Band for each relevant SSB.

-	|SSB_RP1dBm - SSB_RP2dBm| ≤ 27 dB

-	|Channel 1_Io -Channel 2_Io |  20 dB

Table 10.1A.4.1.2-1: SS-RSRP Inter frequency relative accuracy for 1Rx RedCap UE in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot Note 2 | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 3 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
|  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | N/A | -50 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | N/A | -50 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | N/A | -50 |
| 5.5 | 7 | -6 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | N/A | -50 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | N/A | -50 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_N | -114.5 | -111.5 | N/A | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The parameter SSB Ês/Iot is the minimum SSB Ês/Iot of the pair of cells to which the requirement applies. NOTE 3: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |

### 10.1A.5	Inter-frequency RSRP accuracy requirements for FR2

#### 10.1A.5.1	Inter-frequency SS-RSRP accuracy requirements

##### 10.1A.5.1.1	Absolute SS-RSRP Accuracy

The accuracy requirements in clause 10.1.5.1.1 shall apply.

##### 10.1A.5.1.2	Relative SS-RSRP Accuracy

The accuracy requirements in clause 10.1.5.1.2 shall apply.

### 10.1A.6	Intra-frequency RSRQ accuracy requirements for FR1

#### 10.1A.6.1	Intra-frequency SS-RSRQ accuracy requirements in FR1

##### 10.1A.6.1.1	Absolute SS-RSRQ Accuracy in FR1

The accuracy requirements in clause 10.1.7.1.1 shall apply when RedCap UE is capable of 2Rx. When UE is only required to support 1RX, the absolute accuracy requirements in Table 10.1A.6.1.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3I of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for intra-frequency measurements are fulfilled according to Annex B.2.15 for a corresponding Band for each relevant SSB.

Table 10.1A.6.1.1-1: SS-RSRQ Intra frequency absolute accuracy for 1Rx RedCap UE in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 3 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
|  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | N/A | -50 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | N/A | -50 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | N/A | -50 |
| 3.5 | 5 | -3 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | N/A | -50 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | N/A | -50 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_N | -114.5 | -111.5 | N/A | -50 |
| 4.5 | 5 | -6 | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 3: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |

### 10.1A.7	Intra-frequency RSRQ accuracy requirements for FR2

#### 10.1A.7.1	Intra-frequency SS-RSRQ accuracy requirements in FR2

##### 10.1A.7.1.1	Absolute SS-RSRQ Accuracy in FR2

The accuracy requirements in clause 10.1.8.1.1 shall apply.

### 10.1A.8	Inter-frequency RSRQ accuracy requirements for FR1

#### 10.1A.8.1	Inter-frequency SS-RSRQ accuracy requirements in FR1

##### 10.1A.8.1.1	Absolute Accuracy of SS-RSRQ in FR1

The accuracy requirements in clause 10.1.9.1.1 shall apply when RedCap UE is capable of 2Rx. When UE is only required to support 1RX, the absolute accuracy requirements in Table 10.1A.8.1.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3I of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for inter-frequency measurements are fulfilled according to Annex B.2.16 for a corresponding Band for each relevant SSB.

Table 10.1A.8.1.1-1: SS-RSRQ Inter frequency absolute accuracy for 1Rx RedCap UE in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 3 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
|  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | N/A | -50 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | N/A | -50 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | N/A | -50 |
| 3.5 | 5 | -3 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | N/A | -50 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | N/A | -50 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_N | -114.5 | -111.5 | N/A | -50 |
| 4.5 | 5 | -6 | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 3: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |

##### 10.1A.8.1.2	Relative Accuracy of SS-RSRQ in FR1

The accuracy requirements in clause 10.1.9.1.2 shall apply when RedCap UE is capable of 2Rx. When UE is only required to support 1RX, the absolute accuracy requirements in Table 10.1A.8.1.2-1 are valid under the following conditions:

-	Conditions defined in clause 7.3I of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for inter-frequency measurements are fulfilled according to Annex B.2.16 for a corresponding Band for each relevant SSB.

-	|SSB_RP1dBm - SSB_RP2dBm| ≤ 27 dB

-	|Channel 1_Io -Channel 2_Io |  20 dB

Table 10.1A.8.1.2-1: SS-RSRQ Inter frequency relative accuracy for 1Rx RedCap UE in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 1 range |  |  |  |  |
|  |  | Note 2 | NR operating band groups Note 4 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
|  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | N/A | -50 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | N/A | -50 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | N/A | -50 |
| 4 | 5 | -3 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | N/A | -50 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | N/A | -50 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_N | -114.5 | -111.5 | N/A | -50 |
| 5 | 5 | -6 | Note 3 | Note 3 | Note 3 | Note 3 | Note 3 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The parameter SSB Ês/Iot is the minimum SSB Ês/Iot of the pair of cells to which the requirement applies. NOTE 3: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 4: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |

### 10.1A.9	Inter-frequency RSRQ accuracy requirements for FR2

#### 10.1A.9.1	Inter-frequency SS-RSRQ accuracy requirements in FR2

##### 10.1A.9.1.1	Absolute Accuracy of SS-RSRQ in FR2

The accuracy requirements in clause 10.1.10.1.1 shall apply.

##### 10.1A.9.1.2	Relative Accuracy of SS-RSRQ in FR2

The accuracy requirements in clause 10.1.10.1.2 shall apply.

### 10.1A.10	 Intra-frequency SINR accuracy requirements for FR1

#### 10.1A.10.1	Intra-frequency SS-SINR accuracy requirements in FR1

##### 10.1A.10.1.1	Absolute SS-SINR Accuracy in FR1

The accuracy requirements in clause 10.1.12.1.1 shall apply when RedCap UE is capable of 2Rx. When UE is only required to support 1RX, the absolute accuracy requirements in Table 10.1A.10.1.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3I of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for intra-frequency measurements are fulfilled according to Annex B.2.15 for a corresponding Band.

Table 10.1A.10.1.1-1: SS-SINR Intra frequency absolute accuracy for 1Rx RedCap UE in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 1 range |  |  |  |  |
|  |  | Note 3 | NR operating band groups Note 4 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
|  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | N/A | -50 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | N/A | -50 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | N/A | -50 |
| 3.0 | 4 | -3 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | N/A | -50 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | N/A | -50 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_N | -114.5 | -111.5 | N/A | -50 |
| 3.5 | 4 | -6 | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 3: The requirements apply for SSB Ês/Iot ≤ 25 dB under non-HST scenarios. NOTE 4: NR operating band groups in FR1 are as defined in clause 3.5.2. NOTE 5: The requirements apply for SSB Ês/Iot ≤5 dB with SCS 15kHz or 30kHz under NR high speed scenarios. |  |  |  |  |  |  |  |

### 10.1A.11	Intra-frequency SINR accuracy requirements for FR2

#### 10.1A.11.1	Intra-frequency SS-SINR accuracy requirements in FR2

##### 10.1A.11.1.1	Absolute SS-SINR Accuracy in FR2

The accuracy requirements in clause 10.1.13.1.1 shall apply.

### 10.1A.12 	Inter-frequency SINR accuracy requirements for FR1

#### 10.1A.12.1	Inter-frequency SS-SINR accuracy requirements in FR1

##### 10.1A.12.1.1	Aboslute Accuracy of SS-SINR in FR1

The accuracy requirements in clause 10.1.14.1.1 shall apply when RedCap UE is capable of 2Rx. When UE is only required to support 1RX, the absolute accuracy requirements in Table 10.1A.12.1.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3I of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for inter-frequency measurements are fulfilled according to Annex B.2.16 for a corresponding Band.

Table 10.1A.12.1.1-1: SS-SINR Inter frequency absolute accuracy for 1Rx RedCap UE in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot Note 3 | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 4 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
|  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | N/A | -50 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | N/A | -50 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | N/A | -50 |
| 4.0 | 5 | -3 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | N/A | -50 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | N/A | -50 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_N | -114.5 | -111.5 | N/A | -50 |
| 4.5 | 5 | -6 | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 3: The requirements apply for SSB Ês/Iot ≤ 25 dB. NOTE 4: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |

##### 10.1A.12.1.2	Relative Accuracy of SS-SINR in FR1

The accuracy requirements in clause 10.1.14.1.2 shall apply when RedCap UE is capable of 2Rx. When UE is only required to support 1RX, the absolute accuracy requirements in Table 10.1A.12.1.2-1 are valid under the following conditions:

-	Conditions defined in clause 7.3I of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for inter-frequency measurements are fulfilled according to Annex B.2.16 for a corresponding Band.

-	|SSB_RP1dBm - SSB_RP2dBm| ≤ 27 dB

-	| Channel 1_Io -Channel 2_Io |  20 dB

Table 10.1A.12.1.2-1: SS-SINR Inter frequency relative accuracy for 1Rx RedCap UE in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 1 range |  |  |  |  |
|  |  | Note 2,4 | NR operating band groups Note 5 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 120 kHz | SCSSSB = 240 kHz |  |  |
|  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | N/A | -50 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | N/A | -50 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | N/A | -50 |
| 4.5 | 5 | -3 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | N/A | -50 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | N/A | -50 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_N | -114.5 | -111.5 | N/A | -50 |
| 5 | 5 | -6 | Note 3 | Note 3 | Note 3 | Note 3 | Note 3 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The parameter SSB Ês/Iot is the minimum SSB Ês/Iot of the pair of cells to which the requirement applies. NOTE 3: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 4: The requirements apply for SSB Ês/Iot ≤ 25 dB. NOTE 5: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |

### 10.1A.13	 Inter-frequency SINR accuracy requirements for FR2

#### 10.1A.13.1	Inter-frequency SS-SINR accuracy requirements in FR2

##### 10.1A.13.1.1	Aboslute Accuracy of SS-SINR in FR2

The accuracy requirements in clause 10.1.15.1.1 shall apply.

##### 10.1A.13.1.2	Relative Accuracy of SS-SINR in FR2

The accuracy requirements in clause 10.1.15.1.1 shall apply.

### 10.1A.14	 L1-RSRP accuracy requirements for FR1

#### 10.1A.14.1	SSB based L1-RSRP accuracy requirements

##### 10.1A.14.1.1	Absolute Accuracy

The accuracy requirements in clause 10.1.19.1.1 shall apply when RedCap UE is capable of 2Rx. When UE is only required to support 1RX, the absolute accuracy requirements in Table 10.1A.14.1.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3I of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for L1-RSRP measurements are fulfilled according to Annex B.2.4.1 for a corresponding Band for each relevant SSB.

Table 10.1A.14.1.1-1: SSB based L1-RSRP absolute accuracy for 1Rx RedCap UE in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 2 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
|  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | N/A | -70 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | N/A | -70 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | N/A | -70 |
| ±8.0 | ±12.5 | -3 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | N/A | -70 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | N/A | -70 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_N | -114.5 | -111.5 | N/A | -70 |
| ±11.5 | ±14.5 | -3 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A, NR_FDD_FR1_B, NR_TDD_FR1_C, NR_FDD_FR1_D, NR_TDD_FR1_D, NR_FDD_FR1_E, NR_TDD_FR1_E, NR_FDD_FR1_F, NR_FDD_FR1_G, NR_FDD_FR1_H, NR_FDD_FR1_N | N/A | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |

##### 10.1A.14.1.2	Relative Accuracy

The accuracy requirements in clause 10.1.19.1.2 shall apply when RedCap UE is capable of 2Rx. When UE is only required to support 1RX, the absolute accuracy requirements in Table 10.1A.14.1.2-1 are valid under the following conditions:

-	Conditions defined in clause 7.3I of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for L1-RSRP measurements are fulfilled according to Annex B.2.4.1 for a corresponding Band for each relevant SSB.

Table 10.1A.14.1.2-1: SSB based L1-RSRP relative accuracy for 1Rx RedCap UE in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot Note 2 | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 4 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
|  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | N/A | -50 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | N/A | -50 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | N/A | -50 |
| ±6 | ±7 | -3 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | N/A | -50 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | N/A | -50 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_N | -114.5 | -111.5 | N/A | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The parameter SSB Ês/Iot is the minimum SSB Ês/Iot of the pair of SSBs to which the requirement applies. NOTE 3: Void NOTE 4: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |

#### 10.1A.14.2	CSI-RS based L1-RSRP accuracy requirements

##### 10.1A.14.2.1	Absolute Accuracy

The accuracy requirements in clause 10.1.19.2.2 shall apply when RedCap UE is capable of 2Rx. When UE is only required to support 1RX, the absolute accuracy requirements in Table 10.1A.14.2.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3I of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for L1-RSRP measurements are fulfilled according to Annex B.2.4.2 for a corresponding Band for each relevant CSI-RS.

-	The bandwidth of CSI-RS is 48 PRBs and the density is 3.

The performance with larger bandwidth of CSI-RS is equal to or better than the accuracy requirements in Table 10.1A.19.2.1-1.

Table 10.1A.14.2.1-1: CSI-RS based L1-RSRP absolute accuracy for 1Rx RedCap UE in FR1

| Accuracy |  | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | CSI-RS Ês/Iot | Io Note 1 range |  |  |  |  |  |
|  |  |  | NR operating band groups Note 2 | Minimum Io |  |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSCSI-RS |  |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSCSI-RS = 15 kHz | SCSCSI-RS = 30 kHz | SCSCSI-RS = 60 kHz |  |  |
|  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | -115 | N/A | -70 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | -114.5 | N/A | -70 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | -114 | N/A | -70 |
| ±8.0 | ±12.5 | -3 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | -113.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | -113 | N/A | -70 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | -112.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | -112 | N/A | -70 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | -111.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_N | -114.5 | -111.5 | -108.5 | N/A | -70 |
| ±11.5 | ±14.5 | -3 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A, NR_FDD_FR1_B, NR_TDD_FR1_C, NR_FDD_FR1_D, NR_TDD_FR1_D, NR_FDD_FR1_E, NR_TDD_FR1_E, NR_FDD_FR1_F, NR_FDD_FR1_G, NR_FDD_FR1_H, NR_FDD_FR1_N | N/A | N/A | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |  |

##### 10.1A.14.2.2	Relative Accuracy

The accuracy requirements in clause 10.1.19.2.2 shall apply when RedCap UE is capable of 2Rx. When UE is only required to support 1RX, the absolute accuracy requirements in Table 10.1A.14.2.2-1 are valid under the following conditions:

-	Conditions defined in clause 7.3I of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for L1-RSRP measurements are fulfilled according to Annex B.2.4.2 for a corresponding Band for each relevant CSI-RS.

-	The bandwidth of CSI-RS is 48 PRBs and the density is 3.

The performance with larger bandwidth of CSI-RS is equal to or better than the accuracy requirements in Table 10.1A.14.2.2-1.

Table 10.1A.14.2.2-1: CSI-RS based L1-RSRP relative accuracy for 1Rx RedCap UE in FR1

| Accuracy |  | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | CSI-RS Ês/Iot Note 2 | Io Note 1 range |  |  |  |  |  |
|  |  |  | NR operating band groups Note 4 | Minimum Io |  |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSCSI-RS |  |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSCSI-RS = 15 kHz | SCSCSI-RS = 30 kHz | SCSCSI-RS = 60 kHz |  |  |
|  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | -115 | N/A | -50 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | -114.5 | N/A | -50 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | -114 | N/A | -50 |
| ±6 | ±7 | -3 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | -113.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | -113 | N/A | -50 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | -112.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | -112 | N/A | -50 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | -111.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_N | -114.5 | -111.5 | -108.5 | N/A | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The parameter CSI-RS Ês/Iot is the minimum CSI-RS Ês/Iot of the pair of CSI-RS resources to which the requirement applies. NOTE 3: Void NOTE 4: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |  |

### 10.1A.15	 L1-RSRP accuracy requirements for FR2

#### 10.1A.15.1	SSB based L1-RSRP accuracy requirements

##### 10.1A.15.1.1	Absolute Accuracy

The accuracy requirements in clause 10.1.20.1.1 shall apply.

##### 10.1A.15.1.2	Relative Accuracy

The accuracy requirements in clause 10.1.20.1.1 shall apply.

#### 10.1A.15.2	CSI-RS based L1-RSRP accuracy requirements

##### 10.1A.15.2.1	Absolute Accuracy

The accuracy requirements in clause 10.1.20.1.1 shall apply.

##### 10.1A.15.2.2	Relative Accuracy

The accuracy requirements in clause 10.1.20.1.1 shall apply.

## 10.2	E-UTRAN measurements

### 10.2.1	Introduction

Accuracy requirements for measurements on E-UTRAN carrier frequencies are specified in clause 10.2 and apply for UE in SA or NR-DC or NE-DC operation mode, unless otherwise specified.

Unless otherwise specified, the requirements in clause 10.2 are applicable for a UE:

-	in RRC_CONNECTED state

-	performing measurements with appropriate measurement gaps according to clause 9.1.2.

-	that is synchronised to the cell that is measured.

The reported measurement result after layer 1 filtering shall be an estimate of the average value of the measured quantity over the measurement period. The reference point for the measurement result after layer 1 filtering is referred to as point B in the measurement model described in TS 36.300 [24].

The accuracy requirements of E-UTRA measurements in this clause are valid for the reported measurement result after layer 1 filtering. The accuracy requirements are verified from the measurement report at point D in the measurement model having the layer 3 filtering disabled.

If the UE needs measurement gaps to perform the inter-RAT NR ─ E-UTRAN FDD and NR ─ E-UTRAN TDD measurements, the relevant measurement procedure and measurement gap patterns stated in clause 9.1.2 shall apply.

### 10.2.2	E-UTRAN RSRP measurements

NOTE:	This measurement is for handover between NR and E-UTRAN.

The measurement period of E-UTRA RSRP in RRC_CONNECTED state is specified in clause 9.4.2 and 9.4.3.

The accuracy requirements of E-UTRA RSRP measurements in RRC_CONNECTED state and the corresponding side conditions shall be the same as the inter-frequency RSRP Accuracy Requirements in clause 9.1.3 of TS 36.133 [15].

The reporting range and mapping specified for RSRP measurements in clause 9.1.4 of TS 36.133 [15] shall apply.

### 10.2.3	E-UTRAN RSRQ measurements

NOTE:	This measurement is for handover between NR and E-UTRAN.

The measurement period of E-UTRA RSRQ in RRC_CONNECTED state is specified in clause 9.4.2 and 9.4.3.

The accuracy requirements of E-UTRA RSRQ measurements in RRC_CONNECTED state and the corresponding side conditions shall be the same as the inter-frequency RSRQ Accuracy Requirements in clause 9.1.6 of TS 36.133 [15].

The requirements for accuracy of E-UTRA RSRQ measurements in RRC_CONNECTED state and the corresponding side conditions shall be the same as the inter-frequency RSRQ Accuracy Requirements in clause 9.1.6 of TS 36.133 [15].

The reporting range and mapping specified for RSRQ measurements in clause 9.1.7 of TS 36.133 [15] shall apply.

### 10.2.4	E-UTRAN RSTD measurements

The requirements in this clause are valid for UE supporting this capability.

The measurement period is specified in clauses 9.4.4.1 and 9.4.4.2 for inter-RAT NR ─ E-UTRAN FDD and inter-RAT NR ─ E-UTRAN TDD RSTD measurements, respectively.

The accuracy requirements and the corresponding side conditions shall be the same as the inter-frequency measurement accuracy requirements for RSTD measurements in RRC_CONNECTED in clause 9.1.10.2 of TS 36.133 [15].

If the UE needs measurement gaps to perform the inter-RAT NR ─ E-UTRAN FDD and NR ─ E-UTRAN TDD RSTD measurements, the relevant measurement procedure and measurement gap patterns stated in clause 9.1.2 shall apply.

The reporting range and mapping for the inter-RAT NR ─ E-UTRAN FDD and NR ─ E-UTRAN TDD RSTD measurements is the same as specified for RSTD measurements in TS 36.133 [15, clauses 9.1.10.3 and 9.1.10.4].

### 10.2.5	E-UTRAN RS-SINR measurements

NOTE:	This measurement is for handover between NR and E-UTRAN.

The measurement period of E-UTRA RS-SINR in RRC_CONNECTED state is specified in clause 9.4.2 and 9.4.3.

The accuracy requirements of E-UTRA RS-SINR measurements in RRC_CONNECTED state and the corresponding side conditions shall be the same as the inter-frequency RS-SINR Accuracy Requirements in clause 9.1.17.3 of TS 36.133 [15].

The reporting range and mapping for E-UTRA RS-SINR measurements shall be the same as specified for RS-SINR measurements in clause 9.1.17.1 of TS 36.133 [15].

### 10.2.6	E-UTRAN RSRP measurements for CA/DC Idle Mode Measurements

NOTE:	This measurement is for CA/DC Idle Mode measurements between NR and E-UTRAN.

The requirements in this clause are applicable for a UE:

-	in state RRC_IDLE or RRC INACTIVE

-	that is synchronised to the cell that is measured.

The requirements are for absolute accuracy of E-UTRA RSRP.

The measurement period of E-UTRA RSRP in RRC_IDLE and RRC INACTIVE states are specified in clause 4.4.2.

The accuracy requirements of E-UTRA RSRP measurements in RRC_IDLE and RRC INACTIVE states and the corresponding side conditions shall be as the inter-frequency RSRP Accuracy Requirements in clause 9.1.3B.2 of TS 36.133 [15].

The reporting range and mapping specified for RSRP measurements in clause 9.1.4 of TS 36.133 [15] shall apply.

### 10.2.7	E-UTRAN RSRQ measurements for CA/DC Idle Mode Measurements

NOTE:	This measurement is for CA/DC Idle Mode measurements between NR and E-UTRAN.

The requirements in this clause are applicable for a UE:

-	in state RRC_IDLE or RRC INACTIVE

-	that is synchronised to the cell that is measured.

The requirements are for absolute accuracy of E-UTRA RSRQ.

The measurement period of E-UTRA RSRQ in RRC_IDLE and RRC INACTIVE states are specified in clause 4.4.2.

The accuracy requirements of E-UTRA RSRQ measurements in RRC_IDLE and RRC INACTIVE states and the corresponding side conditions shall be as the inter-frequency RSRQ Accuracy Requirements in clause 9.1.6B.2 of TS 36.133 [15].

The reporting range and mapping specified for RSRQ measurements in clause 9.1.7 of TS 36.133 [15] shall apply.

## 10.2A	E-UTRAN measurements for RedCap

### 10.2A.1	Introduction

Accuracy requirements for measurements on E-UTRAN carrier frequencies are specified in clause 10.2A and apply for RedCap UE in SA operation mode, unless otherwise specified.

Unless otherwise specified, the requirements in clause 10.2A are applicable for a UE:

-	in RRC_CONNECTED state

-	performing measurements with appropriate measurement gaps according to clause 9.1A.2.

-	that is synchronised to the cell that is measured.

The reported measurement result after layer 1 filtering shall be an estimate of the average value of the measured quantity over the measurement period. The reference point for the measurement result after layer 1 filtering is referred to as point B in the measurement model described in TS 36.300 [24].

The accuracy requirements of E-UTRA measurements in this clause are valid for the reported measurement result after layer 1 filtering. The accuracy requirements are verified from the measurement report at point D in the measurement model having the layer 3 filtering disabled.

If the UE needs measurement gaps to perform the inter-RAT NR ─ E-UTRAN FDD and NR ─ E-UTRAN TDD measurements, the relevant measurement procedure and measurement gap patterns stated in clause 9.1A.2 shall apply.

### 10.2A.2	E-UTRAN RSRP measurements

NOTE:	This measurement is for handover between NR and E-UTRAN.

The measurement period of E-UTRA RSRP in RRC_CONNECTED state is specified in clause 9.4A.2 and 9.4A.3.

For 2Rx RedCap,

-	The accuracy requirements of E-UTRA RSRP measurements in RRC_CONNECTED state and the corresponding side conditions shall be the same as the inter-frequency RSRP accuracy requirements for UE other than Cat.1bis in clause 9.1.3 of TS 36.133 [15].

For 1Rx RedCap,

-	The accuracy requirements of E-UTRA RSRP measurements in RRC_CONNECTED state and the corresponding side conditions shall be the same as the inter-frequency RSRP accuracy requirements for UE Cat.1bis in clause 9.1.3 of TS 36.133 [15].

The reporting range and mapping specified for RSRP measurements in clause 9.1.4 of TS 36.133 [15] shall apply.

### 10.2A.3	E-UTRAN RSRQ measurements

NOTE:	This measurement is for handover between NR and E-UTRAN.

The measurement period of E-UTRA RSRQ in RRC_CONNECTED state is specified in clause 9.4A.2 and 9.4A.3.

For 2Rx RedCap,

-	The accuracy requirements of E-UTRA RSRQ measurements in RRC_CONNECTED state and the corresponding side conditions shall be the same as the inter-frequency RSRQ accuracy requirements for UE other than Cat.1bis in clause 9.1.6 of TS 36.133 [15].

For 1Rx RedCap,

-	The accuracy requirements of E-UTRA RSRQ measurements in RRC_CONNECTED state and the corresponding side conditions shall be the same as the inter-frequency RSRQ accuracy requirements for UE Cat.1bis in clause 9.1.6 of TS 36.133 [15].

The requirements for accuracy of E-UTRA RSRQ measurements in RRC_CONNECTED state and the corresponding side conditions shall be the same as the inter-frequency RSRQ Accuracy Requirements in clause 9.1.6 of TS 36.133 [15].

The reporting range and mapping specified for RSRQ measurements in clause 9.1.7 of TS 36.133 [15] shall apply.

### 10.2A.4	E-UTRAN RS-SINR measurements

NOTE:	This measurement is for handover between NR and E-UTRAN.

The measurement period of E-UTRA RS-SINR in RRC_CONNECTED state is specified in clause 9.4A.2 and 9.4A.3.

For 2Rx RedCap,

-	The accuracy requirements of E-UTRA RS-SINR measurements in RRC_CONNECTED state and the corresponding side conditions shall be the same as the inter-frequency RS-SINR accuracy requirements in clause 9.1.17.3 of TS 36.133 [15].

The reporting range and mapping for E-UTRA RS-SINR measurements shall be the same as specified for RS-SINR measurements in clause 9.1.17.1 of TS 36.133 [15].

## 10.3	UTRAN FDD Measurements

The requirements in this clause are applicable for a UE:

-	in state RRC_CONNECTED

-	performing measurements according to clause 9.4.6 with appropriate measurement gaps

-	that is synchronised to the cell that is measured.

The reported measurement result after layer 1 filtering shall be an estimate of the average value of the measured quantity over the measurement period. The reference point for the measurement result after layer 1 filtering is referred to as point B in the measurement model described in TS 25.302 [30].

The accuracy requirements in this clause are valid for the reported measurement result after layer 1 filtering. The accuracy requirements are verified from the measurement report at point D in the measurement model having the layer 3 filtering disabled.

### 10.3.1	UTRAN FDD CPICH RSCP

NOTE:	This measurement is for handover between E-UTRAN and UTRAN FDD.

The requirements in this clause are valid for terminals supporting this capability.

The measurement period for RRC_CONNECTED state is specified in clause 9.4.6.

In RRC_CONNECTED state the accuracy requirements shall meet the absolute accuracy requirements in table 10.3.1-1, under the following conditions:

-	CPICH Ec/Io condition for a detectable cell is as specified in clause 9.4.6;

-	SCH_Ec/Io condition for a detectable cell is as specified in clause 9.4.6.

Table 10.3.1-1: UTRAN FDD CPICH_RSCP absolute accuracy

| Accuracy |  | Conditions |  |  |
| --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Io range |  |  |
|  |  | UTRA operating bands | Minimum Io | Maximum Io |
| dB | dB |  | dBm/3.84 MHz | dBm/3.84 MHz |
|  |  | Band I, IV, VI, X XI, XIX and XXI | -94 | -70 |
|  |  | Band IX | -93 | -70 |
| 6 | 9 | Band II, V and VII | -92 | -70 |
|  |  | Band III, VIII, XII, XIII, XIV , XX and XXII | -91 | -70 |
|  |  | Band XXV, XXVI Note 1 | -90.5 | -70 |
| 8 | 11 | Note 2 | -70 | -50 |
| NOTE 1: For Band XXVI, the condition has the minimum Io of -92 dBm/3.84 MHz when the carrier frequency of the assigned UTRA channel is within 869-894 MHz for the UE which supports both Band V and Band XXVI operating frequencies. NOTE 2: The same bands apply for this requirement as for the corresponding highest accuracy requirement. |  |  |  |  |

If the UE, in RRC_CONNECTED state, needs measurement gaps to perform UTRAN FDD measurements, the relevant UTRAN FDD measurement procedure and measurement gap pattern stated in clause 9.4.6 shall apply.

The reporting range and mapping specified for FDD CPICH RSCP in TS 25.133 [29] shall apply.

### 10.3.2	UTRAN FDD CPICH Ec/No

NOTE:	This measurement is for handover between E-UTRAN and UTRAN FDD.

The requirements in this clause are valid for terminals supporting this capability.

The measurement period for RRC_CONNECTED state is specified in clause 9.4.6.

In RRC_CONNECTED state the accuracy requirements shall be the same as the inter-frequency measurement accuracy requirements for FDD CPICH Ec/No in TS 25.133 [29].

If the UE, in RRC_CONNECTED state, needs measurement gaps to perform UTRAN FDD measurements, the UTRAN FDD measurement procedure and measurement gap pattern stated in clause 9.4.6 shall apply.

The reporting range and mapping specified for FDD CPICH Ec/No in TS 25.133 [29] shall apply.

## 10.4	V2X measurements

### 10.4.1	Introduction

The requirements in this section are applicable for a UE capable of V2X sidelink communication.

The accuracy requirements in this clause are:

-	applicable for AWGN radio propagation conditions,

-	assume independent interference (noise) at each receiver antenna port.

### 10.4.2	Intra-frequency PSBCH-RSRP accuracy requirements for FR1

#### 10.4.2.1	PSBCH-RSRP Absolute Accuracy

The requirements for absolute accuracy of PSBCH-RSRP in this clause apply to a V2X synchronization source on the same frequency as that of the own V2X UE performing the measurement in FR1.

The accuracy requirements in Table 10.4.2.1-1 are valid under the following conditions:

-	Demodulation reference signals are transmitted from one port.

-	Conditions defined in Clause 7.3E of TS38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for PSBCH-RSRP measurements are fulfilled according to Annex B.4.2 for a corresponding Band for each relevant PSBCH-DMRS.

Table 10.4.2.1-1: Intra-frequency PSBCH-RSRP absolute accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot Note 3 | Io Note 1 range |  |  |  |  |  |
|  |  |  | NR V2X operating band groups Note 2 | Minimum Io |  |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSL |  |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSL = 15 kHz | SCSSL = 30 kHz | SCSSL = 60 kHz |  |  |
| 4.5 | 9 | -6 | NR_TDD_FR1_B | -120.5 | -117.5 | -114.5 | N/A | -70 |
|  |  |  | NR_TDD_FR1_J | -116.5 | -113.5 | -110.5 | N/A | -70 |
| 8 | 11 | -6 | NR_TDD_FR1_B, NR_TDD_FR1_J | N/A | N/A | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR V2X operating band groups in FR1 are as defined in clause 3.5.2. NOTE 3: Ês/Iot for a SyncRef UE is the Ês/Iot of PSBCH-DMRS. |  |  |  |  |  |  |  |  |

#### 10.4.2.2	PSBCH-RSRP Relative Accuracy

The relative accuracy of PSBCH-RSRP is defined as the PSBCH-RSRP measured from one V2X synchronization source compared to the PSBCH-RSRP measured from another V2X synchronization source on the same frequency in FR1.

The accuracy requirements in Table 10.4.2.2-1 are valid under the following conditions:

-	Demodulation reference signals are transmitted from one port.

-	Conditions defined in Clause 7.3E of TS38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for PSBCH-RSRP accuracy measurements are fulfilled according to Annex B.4.2 for a corresponding Band for each relevant PSBCH-DMRS.

Table 10.4.2.2-1: Intra-frequency PSBCH-RSRP relative accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot Note 3 | Io Note 1 range |  |  |  |  |  |
|  |  |  | NR V2X operating band groups Note 2 | Minimum Io |  |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSL |  |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSL = 15 kHz | SCSSL = 30 kHz | SCSSL = 60 kHz |  |  |
| 2 | 3 | -3 | NR_TDD_FR1_B | -120.5 | -117.5 | -114.5 | N/A | -50 |
|  |  |  | NR_TDD_FR1_J | -116.5 | -113.5 | -110.5 | N/A | -50 |
| 3 | 3 | -6 | Note 4 | Note 4 | Note 4 | Note 4 | N/A | Note 4 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR V2X operating band groups in FR1 are as defined in clause 3.5.2. NOTE 3: Ês/Iot for a SyncRef UE is the Ês/Iot of PSBCH-DMRS. NOTE 4: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. |  |  |  |  |  |  |  |  |

### 10.4.2A	Intra-frequency PSBCH-RSRP accuracy requirements for FR1 under CCA

#### 10.4.2A.1	PSBCH-RSRP Absolute Accuracy

The requirements for absolute accuracy of PSBCH-RSRP in this clause apply to a sidelink synchronization source on the same frequency as that of the own sidelink UE performing the measurement in FR1 under CCA.

The accuracy requirements in Table 10.4.2A.1-1 are valid under the following conditions:

-	Demodulation reference signals are transmitted from one port.

-	Conditions defined in Clause 7.3E of TS38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for PSBCH-RSRP measurements are fulfilled according to Annex B.4.2 for a corresponding Band for each relevant PSBCH-DMRS.

Table 10.4.2A.1-1: Intra-frequency PSBCH-RSRP absolute accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot Note 3 | Io Note 1 range |  |  |  |  |
|  |  |  | NR sidelink operating band groups Note 2 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSL |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSL = 15 kHz | SCSSL = 30 kHz |  |  |
| 4.5 | 9 | -6 | NR_CCA_FR1_I | -117 | -114 | N/A | -70 |
|  |  |  | NR_CCA_FR1_J | -116.5 | -113.5 | N/A | -70 |
| 8 | 11 | -6 | NR_CCA_FR1_I, NR_CCA_FR1_J | N/A | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR sidelink operating band groups in FR1 are as defined in clause 3.5.2. NOTE 3: Ês/Iot for a SyncRef UE is the Ês/Iot of PSBCH-DMRS. |  |  |  |  |  |  |  |

#### 10.4.2A.2	PSBCH-RSRP Relative Accuracy

The relative accuracy of PSBCH-RSRP is defined as the PSBCH-RSRP measured from one sidelink synchronization source compared to the PSBCH-RSRP measured from another sidelink synchronization source on the same frequency in FR1 under CCA.

The accuracy requirements in Table 10.4.2A.2-1 are valid under the following conditions:

-	Demodulation reference signals are transmitted from one port.

-	Conditions defined in Clause 7.3F of TS38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for PSBCH-RSRP accuracy measurements are fulfilled according to Annex B.4.2 for a corresponding Band for each relevant PSBCH-DMRS.

-	The same number of S-SSB repetitions on frequency domain is configured between two sidelink synchronization sources.

Table 10.4.2A.2-1: Intra-frequency PSBCH-RSRP relative accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot Note 3 | Io Note 1 range |  |  |  |  |
|  |  |  | NR sidelink operating band groups Note 2 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSL |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSL = 15 kHz | SCSSL = 30 kHz |  |  |
| 2 | 3 | -3 | NR_CCA_FR1_I | -117 | -114 | N/A | -50 |
|  |  |  | NR_CCA_FR1_J | -116.5 | -113.5 | N/A | -50 |
| 3 | 3 | -6 | Note 4 | Note 4 | Note 4 | N/A | Note 4 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR sidelink operating band groups in FR1 are as defined in clause 3.5.2. NOTE 3: Ês/Iot for a SyncRef UE is the Ês/Iot of PSBCH-DMRS. NOTE 4: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. |  |  |  |  |  |  |  |

### 10.4.3	Intra-Frequency SL-RSSI Measurement Accuracy Requirements for FR1

#### 10.4.3.1	Absolute SL-RSSI Accuracy

The intra-frequency SL-RSSI requirements are specified in Table 10.4.3.1-1. The requirements apply for measurement period of 1slot and for any configured measurement bandwidth larger than 10 RBs, provided that:

-	All symbols duing each RSSI measurement duration are available for RSSI sampling within the same measurement interval.

Table 10.4.3.1-1: Intra-frequency SL-RSSI absolute accuracy

| Accuracy |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Io Note 1 range |  |  |  |  |
|  |  | NR V2X operating band groups Note 2 | Minimum Io |  |  | Maximum Io |
| dB | dB |  | dBm/SCSSL |  |  | dBm/BWChannel |
|  |  |  | SCSSL = 15kHz | SCSSL = 30kHz | SCSSL = 60kHz |  |
| 2.5 | 5.5 | NR_TDD_FR1_B | -120.5 | -117.5 | -114.5 | -50 |
|  |  | NR_TDD_FR1_J | -116.5 | -113.5 | -110.5 | -50 |
| 4.5 | 7.5 | Note 3 | Note 3 | Note 3 | Note 3 | Note 3 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR V2X operating band groups are as defined in Section 3.5 for the corresponding NR operating bands. NOTE 3: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. |  |  |  |  |  |  |

### 10.4.3A	Intra-Frequency SL-RSSI Measurement Accuracy Requirements for FR1 under CCA

#### 10.4.3A.1	Absolute SL-RSSI Accuracy

The intra-frequency SL-RSSI requirements are specified in Table 10.4.3A.1-1 under CCA. The requirements apply for measurement period of 1slot and for any configured measurement bandwidth larger than 10 RBs, provided that:

-	All symbols during each RSSI measurement duration according to indication of first or second starting symbol within a slot are available for RSSI sampling within the same measurement interval.

Table 10.4.3A.1-1: Intra-frequency SL-RSSI absolute accuracy

| Accuracy |  | Conditions |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Io Note 1 range |  |  |  |
|  |  | NR sidelink operating band groups Note 2 | Minimum Io |  | Maximum Io |
| dB | dB |  | dBm/SCSSL |  | dBm/BWChannel |
|  |  |  | SCSSL = 15kHz | SCSSL = 30kHz |  |
| 2.5 | 5.5 | NR_CCA_FR1_I | -117 | -114 | -50 |
|  |  | NR_CCA_FR1_J | -116.5 | -113.5 | -50 |
| 4.5 | 7.5 | Note 3 | Note 3 | Note 3 | Note 3 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR sidelink operating band groups are as defined in Section 3.5 for the corresponding NR operating bands. NOTE 3: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. |  |  |  |  |  |

### 10.4.4	Intra-Frequency L1 SL-RSRP Measurement Accuracy Requirements for FR1

#### 10.4.4.1	Absolute L1 SL-RSRP Accuracy

The requirements for absolute accuracy of L1 SL-RSRP in this clause apply to a UE performing PSCCH-RSRP and/or PSSCH-RSRP measurements on the same frequency as used by operating V2X sidelink communication.

The accuracy requirements in Table 10.4.4.1-1 are valid under the following conditions:

-	Demodulation reference signals for PSCCH and/or PSSCH are transmitted from one port.

-	Conditions defined in clause 7.3E of TS38.101-1 [18] for reference sensitivity are fulfilled.

-	PSCCH-RSRP|dBm and/or PSSCH-RSRP|dBm according to Annex B.4.4 for a corresponding Band are fulfilled.

Table 10.4.4.1-1: Intra-frequency L1 SL-RSRP absolute accuracy for UE capable of V2X sidelink communication

| Accuracy |  | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot Note 3 | Io Note 1 range |  |  |  |  |  |
|  |  |  | NR V2X operating band groups Note 2 | Minimum Io |  |  |  | Maximum Io |
| dB | dB | dB |  | dBm/SCS |  |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCS = 15kHz | SCS = 30kHz | SCS = 60kHz |  |  |
| 4.5 | 9 | 0 dB | NR_TDD_FR1_B | -120.5 | -117.5 | -114.5 | N/A | -70 |
|  |  |  | NR_TDD_FR1_J | -116.5 | -113.5 | -110.5 | N/A | -70 |
| 8 | 11 | 0 dB | NR_TDD_FR1_B NR_TDD_FR1_J | N/A | N/A | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR V2X operating band groups are as defined in Section 3.5 for the corresponding NR operating bands. NOTE 3: The parameter Ês/Iot is the Ês/Iot of PSCCH-DMRS and/or PSSCH-DMRS. |  |  |  |  |  |  |  |  |

### 10.4.4A	Intra-Frequency L1 SL-RSRP Measurement Accuracy Requirements for FR1 under CCA

#### 10.4.4A.1	Absolute L1 SL-RSRP Accuracy

The requirements for absolute accuracy of L1 SL-RSRP in this clause apply to a UE performing PSCCH-RSRP and/or PSSCH-RSRP measurements on the same frequency as used by operating sidelink communication under CCA.

The accuracy requirements in Table 10.4.4A.1-1 are valid under the following conditions:

-	Demodulation reference signals for PSCCH and/or PSSCH are transmitted from one port.

-	Conditions defined in clause 7.3F of TS38.101-1 [18] for reference sensitivity are fulfilled.

-	PSCCH-RSRP|dBm and/or PSSCH-RSRP|dBm according to Annex B.4.4 for a corresponding Band are fulfilled.

Table 10.4.4A.1-1: Intra-frequency L1 SL-RSRP absolute accuracy for UE capable of sidelink communication

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot Note 3 | Io Note 1 range |  |  |  |  |
|  |  |  | NR sidelink operating band groups Note 2 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm/SCS |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCS = 15kHz | SCS = 30kHz |  |  |
| 4.5 | 9 | 0 dB | NR_CCA_FR1_I | -117 | -114 | N/A | -70 |
|  |  |  | NR_CCA_FR1_J | -116.5 | -113.5 | N/A | -70 |
| 8 | 11 | 0 dB | NR_CCA_FR1_I NR_CCA_FR1_J | N/A | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR sidelink operating band groups are as defined in Section 3.5 for the corresponding NR operating bands. NOTE 3: The parameter Ês/Iot is the Ês/Iot of PSCCH-DMRS and/or PSSCH-DMRS. |  |  |  |  |  |  |  |

### 10.4.5	Intra-Frequency Discovery Signal Measurement Accuracy Requirements

The requirements in this clause are applicable for a remote sidelink UE in U2N relay scenario provided that the remote UE:

-	is out of coverage on the frequency used for sidelink, and

-	that is synchronised to the sidelink relay UE that is measured.

The requirements in this clause are applicable for a remote sidelink UE in multipath relay scenario provided that the remote UE:

-	is synchronised to the sidelink relay UE that is measured and

-	is in-coverage on the frequency used for sidelink if both the direct path and the sidelink on the indirect path are on the same frequency

-	is out of co coverage on the frequency used for sidelink if the direct path and the sidelink on the indirect path are on different frequencies.

#### 10.4.5.1	Absolute Discovery Signal Measurement Accuracy

The requirements for absolute accuracy of discovery signal measurement in this clause apply to a sidelink UE performing SD-RSRP measurements for direct to indirect path swich or SL-RSRP measurements for indirect to direct path switch on the same frequency as used by the sidelink relay UE transmitting the relay Discovery message.

The accuracy requirements in Table 10.4.5.1-1 are valid under the following conditions:

-	Demodulation reference signals for PSCCH and/or PSSCH are transmitted from one port.

-	Conditions defined in clause 7.3E of TS38.101-1 [18] for reference sensitivity are fulfilled.

-	PSCCH-RSRP|dBm and/or PSSCH-RSRP|dBm according to Annex B.4.4 for a corresponding Band are fulfilled.

Table 10.4.5.1-1: Intra-frequency discovery signal measurement absolute accuracy for a remote sidelink UE [2] capable of sidelink Communication and sidelink Discovery and configured by upper layers for relay operation.

| Accuracy |  | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot Note 3 | Io Note 1 range |  |  |  |  |  |
|  |  |  | NR V2X operating band groups Note 2 | Minimum Io |  |  |  | Maximum Io |
| dB | dB | dB |  | dBm/SCS |  |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCS = 15kHz | SCS = 30kHz | SCS = 60kHz |  |  |
| 4.5 | 9 | 0 dB | NR_TDD_FR1_B | -120.5 | -117.5 | -114.5 | N/A | -70 |
|  |  |  | NR_TDD_FR1_J | -116.5 | -113.5 | -110.5 | N/A | -70 |
| 8 | 11 | 0 dB | NR_TDD_FR1_B NR_TDD_FR1_J | N/A | N/A | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR sidelink operating band groups are as defined in Section 3.5 for the corresponding NR operating bands. NOTE 3: The parameter Ês/Iot is the Ês/Iot of PSCCH-DMRS and/or PSSCH-DMRS. |  |  |  |  |  |  |  |  |

# 11	Void

# 12	V2X Requirements

## 12.1	Introduction

This clause contains the requirements for the UE capable of V2X sidelink communication when the UE is out of coverage on the carrier used for V2X sidelink operation, as defined in TS 38.304 [1]. The requirements apply when the UE is:

-	in any cell selection state, or any of the below applies:

-	configured for V2X SL operation on a V2X carrier which is dedicated to only V2X SL operation and configured with only a PCell on WAN carrier,

-	configured for SL operation on a sidelink carrier with CCA and configured with only a PCell on WAN carrier,

-	configured for inter-band con-current V2X operation,

-	configured for intra-band con-current V2X operation with different carriers,

-	configured in co-channel coexistence for LTE SL and NR SL operation with same carrier.

Any cell selection state refers to a UE that is out of network coverage and is not associated with a serving cell on any carrier as defined in TS 38.304 [1].

When a UE in RRC_CONNECTED state is performing transmissions and/or reception for V2X sidelink communication, the UE shall meet all the requirements specified in Clause 9 assuming that UE has a dedicated RX/TX chain for V2X sidelink communication. Otherwise, the UE may interrup the V2X sidelink communication in order to meet the measurement requirements specified in Clause 9.

This clause also contains the requirements for the UE capable of V2X sidelink communication when the UE is in coverage on the carrier used for V2X sidelink operation, as defined in TS 38.304 [1]. The requirements apply when the UE is:

-	configured for intra-band con-current NR V2X cooperation with same carrier.

-	configured in co-channel coexistence for LTE SL and NR SL operation with same carrier，

For UE capable of Public Safety sidelink communication and/or other commercial sidelink commnunication, unless explicitly stated, V2X requirements apply.

For sidelink communication in unlicensed spectrum,

-	the term SyncRef UE subject to CCA is not available at the UE refers to when all the candidate S-SSB positions monitored in every S-SSB period are not available during the last 1280 ms. Otherwise, the SyncRef UE subject to CCA is considered as available at the UE.

-	the term S-SSB period subject to CCA is not available at the UE refers to the S-SSB period in which all the candidate S-SSB positions are not available. Otherwise, the S-SSB period subject to CCA is considered as available at the UE.

## 12.2	UE Transmit Timing

### 12.2.1	Introduction

This clause contains requirements of transmission timing for V2X sidelink communication when:

-	GNSS is used as the synchronization reference source;

-	NR Cell is used as the synchronization reference source;

-	E-UTRAN Cell is used as the synchronization reference source;

-	SyncRef UE is used as the synchronization reference source.

The requirements for 60 kHz SCS of sidelink signal defined in this caluse do not apply to the sidelink communication in unlicensed spectrum.

### 12.2.2	GNSS as synchronization reference source

The requirements in this subclause are applicable when the reference timing used by the UE for V2X sidelink communication is derived from GNSS.

The sidelink transmissions takes place  before the subframe starting boundary as defined in TS 38.331 [2], where  = 0 and=0.

The transmission timing error for sidelink transmissions shall be less than or equal to Te where the timing error limit value Te is defined in Table 12.2.2-1.

Table 12.2.2-1: Te Timing Error Limit

| Frequency Range of sidelink | Te_ |
| --- | --- |
| FR1 | 12*64*Tc |
| Note 1: Tc is the basic timing unit defined in TS 38.211 [6]. |  |

### 12.2.3	NR Cell as synchronization reference source

The requirements in this subclause are applicable when the reference timing used for sidelink transmissions is a NR serving cell on a non-V2X sidelink carrier or a V2X sidelink carrier.

The sidelink transmissions takes place  before the reception of the first detected path (in time) of the corresponding downlink frame from the reference cell, where  = 0. If uplink transmission and sidelink transmission are in the same band,  is defined in Table 7.1.2-2, otherwise  is 0.

The transmission timing error for sidelink transmissions shall be less than or equal to Te where the timing error limit value Te is defined in Table 12.2.3-1.

Table 12.2.3-1: Te Timing Error Limit

| Frequency Range of sidelink | SCS of SSB signals ( kHz) | SCS of sidelink signals (kHz) | Te |
| --- | --- | --- | --- |
| FR1 | 15 | 15 | 14*64*Tc |
|  |  | 30 | 12*64*Tc |
|  |  | 60 | 12*64*Tc |
|  | 30 | 15 | 10*64*Tc |
|  |  | 30 | 10*64*Tc |
|  |  | 60 | 9*64*Tc |
| Note 1: Tc is the basic timing unit defined in TS 38.211 [6]. |  |  |  |

### 12.2.4	E-URTAN Cell as synchronization reference source

The requirements in this subclause are applicable when the reference timing used for sidelink transmissions is an E-UTRAN serving cell on a non-V2X sidelink carrier.

The sidelink transmissions takes place  before the reception of the first detected path (in time) of the corresponding E-UTRAN downlink frame from the reference cell, where  = 0 and=0.

The transmission timing error for sidelink transmissions shall be less than or equal to Te where the timing error limit value Te is defined in Table 12.2.4-1.

Table 12.2.4-1: Te Timing Error Limit

| Frequency Range of sidelink | E-UTRAN downlink bandwidth (MHz) | Te_ |
| --- | --- | --- |
| FR1 | ≥3 | 14*64*Tc |
| Note 1: Tc is the basic timing unit defined in TS 38.211 [6]. |  |  |

### 12.2.5	SyncRef UE as synchronization reference source

The requirements in this subclause are applicable when the reference timing used for deriving sidelink transmission is from SyncRef UE transmitting sidelink synchronization signals.

The sidelink transmissions takes place  before the reception of the first detected path (in time) of the corresponding timing reference frame from the SyncRef UE, where  = 0 and=0.

The transmission timing error for sidelink transmissions shall be less than or equal to Te where the timing error limit value Te is defined in Table 12.2.5-1.

Table 12.2.5-1: Te Timing Error Limit

| Frequency Range of sidelink | SCS of sidelink signals (kHz) | Te |
| --- | --- | --- |
| FR1 | 15 | 12*64*Tc |
|  | 30 | 8*64*Tc |
|  | 60 | 5*64*Tc |
| Note 1: Tc is the basic timing unit defined in TS 38.211 [6]. |  |  |

If the UE uses SyncRefUE on a carrier frequency subject to CCA for deriving the UE transmit timing, then the UE shall meet all the transmit timing requirements defined in clause 12.2.5: The transmission timing error for sidelink transmissions shall be less than or equal to Te where the timing error limit value Te is defined in Table 12.2.5-1 provided that at least one S-SSB is available at the UE during the last 160 ms.

## 12.3	Initiation/Cease of SLSS Transmissions

### 12.3.1	Introduction

The requirements in this subclause are applicable to the UE capable of V2X sidelink communication when:

-	GNSS is used as the synchronization reference source;

-	NR Cell is used as the synchronization reference source;

-	E-UTRAN Cell is used as the synchronization reference source;

-	SyncRef UE is used as the synchronization reference source.

#### 12.3.1.1	Initiation/Cease of SLSS transmissions with NR cell as synchronization reference source

The requirements apply when the NR Cell is used as synchronization reference source and when the UE is

-	out of coverage on the V2X NR sidelink carrier and in coverage with a serving cell on a NR non-V2X sidelink carrier, or

-	 in coverage with a serving cell on a NR V2X sidelink carrier,

and when the conditions for SLSS transmissions specified in TS 38.331 [2] are met; networkControlledSyncTx is not configured; and syncTxThreshIC is included in SystemInformationBlockType12. The UE shall be capable of measuring the RSRP of the cell used as synchronization reference source to evaluate to initiate/cease SLSS transmissions within Tevaluate,SLSS

where,

-	Tevaluate,SLSS is as specified in Table 12.3.1.1-1 when UE performs SSB based measurements without measurement gaps.

-	Tevaluate,SLSS is as specified in Table 12.3.1.1-2 when UE performs SSB based measurements with measurement gaps.

Table 12.3.1.1-1: Tevaluate,SLSS for measurements without measurement gaps when NR cell is used as synchronization reference source (FR1)

| DRX cycle in NR cell | Tevaluate,SLSS |
| --- | --- |
| No DRX | max(400ms, ceil( 2 x 5 x Kp) x SMTC period)Note 1 |
| DRX cycle≤ 320ms | max(400ms, ceil(1.5 x 2 x 5 x Kp) x max(SMTC period, DRX cycle)) |
| DRX cycle>320ms | ceil( 7 x Kp ) x DRX cycle |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Table 12.3.1.1-2: Tevaluate,SLSS for measurements with measurement gaps when NR cell is used as synchronization reference source (FR1)

| DRX cycle in NR cell | Tevaluate,SLSS |
| --- | --- |
| No DRX | max(400ms, 2 x 5 x max(MGRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 320ms | max(400ms, ceil(2 x 1.5x 5) x max(MGRP, SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | 7 x max(MGRP, DRX cycle) x CSSFintra |

If higher layer filtering is configured, an additional delay in evaluation to initiate/cease SLSS transmissions can be expected.

For the NR cell as synchronization reference source:

-	SS-RSRP related side conditions given in clauses 10.1.2 for FR1, respectively, for a corresponding Band,

-	SS-RSRQ related side conditions given in clauses 10.1.7 for FR1, respectively, for a corresponding Band,

-	SS-SINR related side conditions given in clauses 10.1.12 for FR1, respectively, for a corresponding Band,

-	SSB_RP and SSB Ês/Iot according to Annex B.2.2 for a corresponding Band.

#### 12.3.1.2	Initiation/Cease of SLSS transmissions with E-UTRAN cell as synchronization reference source

The requirements apply when the E-UTRAN Cell is used as synchronization reference source and when the UE is

-	out of coverage on the V2X NR sidelink carrier and in coverage with a serving cell on a LTE non-V2X sidelink carrier,

and when the conditions for SLSS transmissions specified in TS 36.331 [16] are met; networkControlledSyncTx is not configured; and syncTxThreshIC is included in SystemInformationBlockType28. The UE shall be capable of measuring the RSRP of the cell used as synchronization reference source to evaluate to initiate/cease SLSS transmissions within Tevaluate,SLSS

where,

-	Tevaluate,SLSS = 0.4 seconds when UE is not configured with DRX.

-	Tevaluate,SLSS = as specified in Table 12.3.1.2-1 when UE is configured with DRX.

Table 12.3.1.2-1: Tevaluate,SLSS when E-UTRAN cell is used as synchronization reference source

| DRX cycle length in E-UTRAN cell[s] | Tevaluate,SLSS [s] (number of DRX cycles) |
| --- | --- |
| ≤0.04 | 0.4 (Note 1) |
| 0.04<DRX-cycle≤2.56 | Note 2 (6) |
| Note1: Number of DRX cycles depends upon the DRX cycle in use Note2: Time depends upon the DRX cycles in use |  |

If higher layer filtering is configured, an additional delay in evaluation to initiate/cease SLSS transmissions can be expected.

For the cell as synchronization reference source:

-	RSRP related side conditions given in TS 36.133 [15] Clauses 9.1.2.1 and 9.1.2.2 and RSRQ related side conditions given in TS 36.133 [15] Clause 9.1.5.1 for a corresponding Band are fulfilled,

-	SCH_RP and SCH Ês/Iot according to TS 36.133 [15] Annex B.2.1 for a corresponding Band are fulfilled.

#### 12.3.1.3	Initiation/Cease of SLSS transmissions with GNSS as synchronization reference source

The requirements apply when GNSS is used as synchronization reference source and when the UE is

-	out of coverage on the V2X sidelink carrier and in coverage with a serving cell on a non-V2X sidelink carrier, or

-	in coverage with a serving cell on a NR V2X sidelink carrier,

and when the conditions for SLSS transmissions specified in TS 38.331 [2] are met; networkControlledSyncTx is not configured; and syncTxThreshIC is included in SystemInformationBlockType12 in a NR cell.

When the conditions for SLSS transmissions specified in TS 36.331 [16] are met; networkControlledSyncTx is not configured; and syncTxThreshIC is included in SystemInformationBlockType28 in a E-UTRAN cell.

The requirements in Clause 12.3.1.1 shall apply if the serving cell is a NR cell.

The requirements in Clause 12.3.1.2 shall apply if the serving cell is a E-UTRAN cell.

#### 12.3.1.4	Initiation/Cease of SLSS transmissions with SyncRef UE as synchronization reference source

The requirements apply when SyncRef UE is used as synchronization reference source and when the UE is

-	in any cell selection state, or

-	out of coverage on the V2X sidelink carrier and is associated with a serving cell on a non-V2X sidelink carrier, or

-	in coverage with a serving cell on a NR V2X sidelink carrier,

and when the conditions for SLSS transmissions specified in TS 38.331 [2] are met and when SyncRef UE is used as synchronization reference source and if syncTxThreshOoC is included in the preconfigured V2X parameters.

The UE shall be capable of measuring the PSBCH-RSRP of the selected SyncRef UE used as synchronization reference source and evaluate it to initiate/cease SLSS transmissions within Tevaluate,SLSS, as shown in Table 12.3.1.4-1.

Table 12.3.1.4-1: Tevaluate,SLSS when SyncRef UE is used as synchronization reference source

| SL-DRX cycleNote 1 [ms] | Tevaluate,SLSS [ms] |
| --- | --- |
| No SL-DRX | 4 x S-SSB periods |
| SL-DRX cycle ≤ 160 | 4 x S-SSB periods |
| SL-DRX cycle > 160 | 4 x SL-DRX cycle |
| Note 1: If multiple SL-DRX cycles are configured for SL UE, the SL-DRX cycle in the requirement is the shortest one. When the shortest SL-DRX cycle UE used changes, the requirements do not apply to the time of transition. |  |

If higher layer filtering for PSBCH-RSRP measurements is pre-configured, an additional delay in evaluation to initiate/cease SLSS transmissions can be expected.

For the selected SyncRef UE as defined in TS 38.331 [2] used to derive transmission timing for V2X sidelink communication:

-	PSBCH-RSRP related side conditions given in Clause 12.4 for a corresponding Band are fulfilled,

-	V2X S-SSB_RP and S-SSB Ês/Iot according to Annex B. 4 for a corresponding Band are fulfilled.

## 12.3A	Initiation/Cease of SLSS Transmissions with CCA

### 12.3A.1	Introduction

The requirements in this subclause are applicable to the UE capable of sidelink communication in unlicensed spectrum when:

-	GNSS is used as the synchronization reference source;

-	NR Cell is used as the synchronization reference source;

-	E-UTRAN Cell is used as the synchronization reference source;

-	SyncRef UE is used as the synchronization reference source on a carrier frequency subject to CCA.

#### 12.3A.1.1	Initiation/Cease of SLSS transmissions with NR cell as synchronization reference source

The requirements defined in subclause 12.3.1.1 apply when the NR Cell is used as synchronization reference source and when the UE is

-	out of coverage on the NR sidelink carrier and in coverage with a serving cell on a NR non- sidelink carrier.

#### 12.3A.1.2	Initiation/Cease of SLSS transmissions with E-UTRAN cell as synchronization reference source

The requirements defined in subclause 12.3.1.2 apply when the E-UTRAN Cell is used as synchronization reference source and when the UE is

-	out of coverage on the NR sidelink carrier and in coverage with a serving cell on a LTE non-sidelink carrier.

#### 12.3A.1.3	Initiation/Cease of SLSS transmissions with GNSS as synchronization reference source

The requirements defined in subclause 12.3.1.3 apply when GNSS is used as synchronization reference source and when the UE is

-	out of coverage on the sidelink carrier and in coverage with a serving cell on a non-sidelink carrier.

#### 12.3A.1.4	Initiation/Cease of SLSS transmissions with SyncRef UE as synchronization reference source

The requirements apply when SyncRef UE is used as synchronization reference source and when the UE is

-	in any cell selection state, or

-	out of coverage on the sidelink carrier and is associated with a serving cell on a non-sidelink carrier.

and when the conditions for SLSS transmissions specified in TS 38.331[2] are met and when SyncRef UE is used as synchronization reference source and if syncTxThreshOoC is included in the preconfigured sidelink parameters.

The UE shall be capable of measuring the PSBCH-RSRP of the selected SyncRef UE used as synchronization reference source and evaluate it to initiate/cease SLSS transmissions within Tevaluate,SLSS_CCA, as shown in Table 12.3A.1.4-1 when the SyncRef UE is transmitting S-SSB on a carrier frequency subject to CCA.

Table 12.3A.1.4-1: Tevaluate,SLSS_CCA when SyncRef UE is transmitting S-SSB on a carrier subject to CCA and is used as synchronization reference source

| SL-DRX cycleNote 1 [ms] | Tevaluate,SLSS_CCA [ms] |
| --- | --- |
| No SL-DRX | (4 + LSLSS) x S-SSB periods |
| SL-DRX cycle ≤ 160 | (4 + LSLSS) x S-SSB periods |
| SL-DRX cycle > 160 | (4 + LSLSS) x SL-DRX cycle |
| Note 1: If multiple SL-DRX cycles are configured for SL UE, the SL-DRX cycle in the requirement is the shortest of all the configured SL-DRX cycles. When the shortest SL-DRX cycle UE used changes, the requirements do not apply to the time of transition. Note 2: LSLSS is the number of unavailable S-SSB period during Tevaluate,SLSS_CCA due to the CCA failures; where LSLSS ≤ LSLSS, max and LSLSS, max = 4. |  |

The UE shall initiate the procedure for selection/reselection of different synchronization reference source as defined in TS 38.331 [2] when the requirements cannot be met due to that LSLSS exceeding LSLSS, max during Tevaluate,SLSS_CCA.

If higher layer filtering for PSBCH-RSRP measurements is pre-configured, an additional delay in evaluation to initiate/cease SLSS transmissions can be expected.

For the selected SyncRef UE as defined in TS 38.331 [2] used to derive transmission timing for sidelink communication:

-	PSBCH-RSRP related side conditions given in Clause 12.4 for a corresponding Band are fulfilled,

-	sidelink S-SSB_RP and S-SSB Ês/Iot according to Annex B. 4 for a corresponding Band are fulfilled.

## 12.4	Selection / Reselection of V2X Synchronization Reference Source

The requirements defined in this clause do not apply to the UEs that do not support transmission and reception of SLSS.

A SyncRef UE is considered to be detectable when

-	PSBCH-RSRP related side conditions given in Clause 10 are fulfilled for a corresponding Band,

-	S-SSB_RP and S-SSB Ês/Iot according to Annex B.4.3 for a corresponding Band are fulfilled.

When GNSS synchronization reference source is configured as the highest priority and

-	UE is synchronized to GNSS directly,

-	UE shall not drop any V2X SLSS and data transmission for the purpose of selection/reselection to the SyncRef UE.

-	UE is synchronized to a SyncRef UE that is synchronized to GNSS directly or in-directly,

-	UE shall not drop any V2X data transmission for the purpose of selection/reselection to the SyncRef UE. The UE shall be able to identify newly detectable intra-frequency SyncRef UE within Tdetect,SyncRef UE_V2X seconds if the SyncRef UE meets the selection / reselection criterion defined in TS 38.331 [2]. Tdetect,SyncRef UE_V2X is defined as 1.6 seconds at S-SSB Ês/Iot ≥ 0 dB, provided that the UE is allowed to drop a maximum of 30% of its SLSS transmissions during Tdetect,SyncRef UE_V2X for the purpose of selection / reselection to the SyncRef UE.

-	in other case:

-	When UE is in non-SL-DRX

-	The UE shall be able to identify newly detectable intra-frequency SyncRef UE within Tdetect,SyncRef UE_V2X seconds if the SyncRef UE meets the selection / reselection criterion defined in TS 38.331 [2]. Tdetect,SyncRef UE_V2X is defined as 8 seconds at S-SSB Ês/Iot ≥ 0 dB, provided that the UE is allowed to drop a maximum of 6 % of its V2X data and SLSS transmissions during Tdetect,SyncRef UE_V2X for the purpose of selection / reselection to the SyncRef UE.

-	UE is allowed to drop up to 2 slots of its V2X data reception per PSBCH monitoring occasion and overall drop rate shall not exceed 0.3% of its V2X data reception during Tdetect,SyncRef UE_V2X for the purpose of selection / reselection to the SyncRef UE.

-	When UE is in SL-DRX

-	UE shall be able to identify newly detectable intra-frequency SyncRef UE within Tdetect,SyncRef UE_V2X seconds if the SyncRef UE meets the selection / reselection criterion defined in TS 38.331 [2]. Tdetect,SyncRef UE_V2X is defined as 8 seconds at S-SSB Ês/Iot ≥ 0 dB, provided that the V2X UE is allowed to drop a maximum of 6 % of its V2X data and SLSS transmissions for the purpose of selection / reselection to the SyncRef UE.

-	UE is allowed to drop up to 2 slots of its V2X data reception per PSBCH monitoring occasion and UE is allowed to drop at most an aggregated window of 24ms of its V2X data reception during Tdetect,SyncRef UE_V2X for the purpose of selection / reselection to the SyncRef UE.

-	The UE is allowed to extend Tdetect,SyncRef UE_V2X to max(4 x 50 SL-DRX cycle length, 8s) when the following conditions are satisfied over an evaluation period Tevaluate,SLSS in clause 12.3.1.1 if an NR cell is used as synchronization reference source, or Tevaluate,SLSS in clause 12.3.1.2 if an E-UTRA cell is used as synchronization reference source, or Tevaluate,SLSS in clause 12.3.1.4 if an SLSS is used as synchronization reference source. If multiple SL-DRX cycles are configured, the SL-DRX cycle length is the longest one.

-	SS-RSRP is larger than syncTxThreshOoC.

When serving cell/PCell synchronization reference source is configured as the highest priority,

-	When UE is in non-SL-DRX

-	UE shall be able to identify newly detectable intra-frequency SyncRef UE within Tdetect,SyncRef UE_V2X seconds if the SyncRef UE meets the selection / reselection criterion defined in TS 38.331 [2].  Tdetect,SyncRef UE_V2X is defined as 8  seconds at SCH Es/Iot ≥ 0 dB, provided that the V2X UE is allowed to drop a maximum of 6 % of its V2X data and SLSS transmissions for the purpose of selection / reselection to the SyncRef UE.

-	UE is allowed to drop up to 2 slots of its V2X data reception per PSBCH monitoring occasion and overall drop rate shall not exceed 0.3% of its V2X data reception during Tdetect,SyncRef UE_V2X for the purpose of selection / reselection to the SyncRef UE.

-	When UE is in SL-DRX

-	The UE shall be able to identify newly detectable intra-frequency SyncRef UE within Tdetect,SyncRef UE_V2X seconds if the SyncRef UE meets the selection / reselection criterion defined in TS 38.331 [2]. Tdetect,SyncRef UE_V2X is defined as 8 seconds at SCH Es/Iot ≥ 0 dB, provided that the UE is allowed to drop its V2X data and SLSS transmissions at most in an aggregated window of 480ms during Tdetect,SyncRef UE_V2X for the purpose of selection / reselection to the SyncRef UE.

-	UE is allowed to drop up to 2 slots of its V2X data reception per PSBCH monitoring occasion and UE is allowed to drop at most an aggregated window of 24ms of its V2X data reception during Tdetect,SyncRef UE_V2X for the purpose of selection / reselection to the SyncRef UE.

-	The UE is allowed to extend Tdetect,SyncRef UE_V2X to max(4 x 50 SL-DRX cycle length, 8s) when the following conditions are satisfied over an evaluation period Tevaluate,SLSS in clause 12.3.1.1 if an NR cell is used as synchronization reference source, or Tevaluate,SLSS in clause 12.3.1.2 if an E-UTRA cell is used as synchronization reference source, or Tevaluate,SLSS in clause 12.3.1.4 if an SLSS is used as synchronization reference source. If multiple SL-DRX cycles are configured, the SL-DRX cycle length is the longest one.

-	SS-RSRP is larger than syncTxThreshOoC.

UE shall be capable of performing PSBCH-RSRP measurements for 3 identified intra-frequency SyncRef UE with the measurement period of Tmeasure,PSBCH-RSRP in Table 12.4-1. It is assumed that the SyncRef UE do not drop or delay any SLSS transmission within the measurement period. Otherwise, the measurement period may be extended.

Table 12.4-1: PSBCH-RSRP measurement period for intra-frequency SyncRef UE

| SL-DRX cycleNote 1 [ms] | Tmeasure,PSBCH-RSRP [ms] |
| --- | --- |
| No SL-DRX | 320 |
| SL-DRX cycle ≤ 160ms | 320 |
| SL-DRX cycle > 160ms | 2 x SL-DRX cycle |
| Note 1: If multiple SL-DRX cycles are configured, the SL-DRX cycle is the shortest one. |  |

When UE is synchronized to GNSS directly, before selection / reselection of the new synchronization reference source UE shall evaluate the GNSS synchronization source reliability for at least 20 seconds before changing the synchronization reference from GNSS to another synchronization reference source. UE shall be always synchronized to GNSS directly during the evaluation of GNSS synchronization source reliability.

## 12.4A	Selection / Reselection of Sidelink Synchronization Reference Source with CCA

The requirements defined in this clause do not apply to the UEs that do not support transmission and reception of SLSS.

A SyncRef UE is considered to be detectable when

-	PSBCH-RSRP related side conditions given in Clause 10 are fulfilled for a corresponding Band,

-	S-SSB_RP and S-SSB Ês/Iot according to Annex B.4.3 for a corresponding Band are fulfilled.

When GNSS synchronization reference source is configured as the highest priority and

-	UE is synchronized to GNSS directly,

-	UE shall not drop any sidelink SLSS and data transmission for the purpose of selection/reselection to the SyncRef UE.

-	UE is synchronized to a SyncRef UE that is synchronized to GNSS directly or in-directly,

-	UE shall not drop any sidelink data transmission for the purpose of selection/reselection to the SyncRef UE. The UE shall be able to identify newly detectable intra-frequency SyncRef UE within Tdetect,SyncRef UE_V2X_CCA seconds if the SyncRef UE meets the selection/reselection criterion defined in TS 38.331 [2] and all S-SSB periods selected for SyncRefUE identification are available during the Tdetect,SyncRef UE_V2X_CCA seconds. Tdetect,SyncRef UE_V2X_CCA is defined as 1.6 seconds at S-SSB Ês/Iot ≥ 0 dB, provided that the UE is allowed to drop a maximum of 30% of its SLSS transmissions during Tdetect,SyncRef UE_V2X_CCA for the purpose of selection/reselection to the SyncRef UE.

For other cases

-	When UE is in non-SL-DRX

-	The UE shall be able to identify newly detectable intra-frequency SyncRef UE within Tdetect,SyncRef UE_V2X_CCA seconds if the SyncRef UE meets the selection/reselection criterion defined in TS 38.331 [2] and all S-SSB periods selected for SyncRefUE identification are available during the Tdetect,SyncRef UE_V2X_CCA seconds. Tdetect,SyncRef UE_V2X_CCA is defined as 8 seconds at S-SSB Ês/Iot ≥ 0 dB, provided that the UE is allowed to drop its sidelink data and SLSS transmissions at most in an aggregated window of 480ms during Tdetect,SyncRef UE_V2X_CCA for the purpose of selection/reselection to the SyncRef UE. Only if UE additionally drops a maximum of 30% of its SLSS transmission, the UE shall be able to identify newly detectable intra-frequency SyncRef UE within T’detect,SyncRef UE_V2X_CCA seconds, when all S-SSB periods selected for SyncRefUE identification are available during the T’detect,SyncRef UE_V2X_CCA seconds. T’detect,SyncRef UE_V2X_CCA is defined as 1.6 seconds at S-SSB Ês/Iot ≥ 0 dB.

-	UE is allowed to drop up to 2 slots of its sidelink data reception per PSBCH monitoring occasion and overall drop rate shall not exceed 0.3% of its sidelink data reception during Tdetect,SyncRef UE_V2X_CCA for the purpose of selection/reselection to the SyncRef UE.

-	When UE is in SL-DRX

-	UE shall be able to identify newly detectable intra-frequency SyncRef UE within Tdetect,SyncRef UE_V2X_CCA seconds if the SyncRef UE meets the selection/reselection criterion defined in TS 38.331 [2] and all S-SSB periods selected for SyncRefUE identification are available during the Tdetect,SyncRef UE_V2X_CCA seconds. Tdetect,SyncRef UE_V2X_CCA is defined as 8 seconds at S-SSB Ês/Iot ≥ 0 dB, provided that the sidelink UE is allowed to drop a maximum of 6 % of its sidelink data and SLSS transmissions for the purpose of selection/reselection to the SyncRef UE. Only if UE additionally drops a maximum of 30% of its SLSS transmission, the UE shall be able to identify newly detectable intra-frequency SyncRef UE within T’detect,SyncRef UE_V2X_CCA seconds, when all S-SSB periods selected for SyncRefUE identification are available during the T’detect,SyncRef UE_V2X_CCA seconds. T’detect,SyncRef UE_V2X_CCA is defined as 1.6 seconds at S-SSB Ês/Iot ≥ 0 dB.

-	UE is allowed to drop up to 2 slots of its sidelink data reception per PSBCH monitoring occasion and UE is allowed to drop at most an aggregated window of 24ms of its sidelink data reception during Tdetect,SyncRef UE_V2X_CCA for the purpose of selection/reselection to the SyncRef UE.

-	The UE is allowed to extend Tdetect,SyncRef UE_V2X_CCA to max(4 x 50 SL-DRX cycle length, 8s) when the following conditions are satisfied over an evaluation period Tevaluate,SLSS in clause 12.3.1.1 if an NR cell is used as synchronization reference source, or Tevaluate,SLSS in clause 12.3.1.2 if an E-UTRA cell is used as synchronization reference source, or Tevaluate,SLSS_CCA in clause 12.3A.1.4 if an SLSS is used as synchronization reference source. If multiple SL-DRX cycles are configured, the SL-DRX cycle length is the longest one.

-	SS-RSRP is larger than syncTxThreshOoC.

When serving cell/PCell synchronization reference source is configured as the highest priority,

-	When UE is in non-SL-DRX

-	UE shall be able to identify newly detectable intra-frequency SyncRef UE within Tdetect,SyncRef UE_V2X_CCA seconds if the SyncRef UE meets the selection/reselection criterion defined in TS 38.331 [2] and all S-SSB periods selected for SyncRefUE identification are available during the Tdetect,SyncRef UE_V2X_CCA seconds. Tdetect,SyncRef UE_V2X_CCA is defined as 8 seconds at SCH Es/Iot ≥ 0 dB, provided that the sidelink UE is allowed to drop a maximum of 6 % of its sidelink data and SLSS transmissions for the purpose of selection/reselection to the SyncRef UE. When GNSS is not available and only if UE additionally drops a maximum of 30% of its SLSS transmission, the UE shall be able to identify newly detectable intra-frequency SyncRef UE within T’detect,SyncRef UE_V2X_CCA seconds, when all S-SSB periods selected for SyncRefUE identification are available during the T’detect,SyncRef UE_V2X_CCA seconds. T’detect,SyncRef UE_V2X_CCA is defined as 1.6 seconds at S-SSB Ês/Iot ≥ 0 dB.

-	UE is allowed to drop up to 2 slots of its sidelink data reception per PSBCH monitoring occasion and overall drop rate shall not exceed 0.3% of its sidelink data reception during Tdetect,SyncRef UE_V2X_CCA for the purpose of selection/reselection to the SyncRef UE.

-	When UE is in SL-DRX

-	The UE shall be able to identify newly detectable intra-frequency SyncRef UE within Tdetect,SyncRef UE_V2X_CCA seconds if the SyncRef UE meets the selection/reselection criterion defined in TS 38.331 [2] and all S-SSB periods selected for SyncRefUE identification are available during the Tdetect,SyncRef UE_V2X_CCA seconds. Tdetect,SyncRef UE_V2X_CCA is defined as 8 seconds at SCH Es/Iot ≥ 0 dB, provided that the UE is allowed to drop its sidelink data and SLSS transmissions at most in an aggregated window of 480ms during Tdetect,SyncRef UE_V2X_CCA for the purpose of selection/reselection to the SyncRef UE. When GNSS is not available and only if UE additionally drops a maximum of 30% of its SLSS transmission, the UE shall be able to identify newly detectable intra-frequency SyncRef UE within T’detect,SyncRef UE_V2X_CCA seconds, when all S-SSB periods selected for SyncRefUE identification are available during the T’detect,SyncRef UE_V2X_CCA seconds. T’detect,SyncRef UE_V2X_CCA is defined as 1.6 seconds at S-SSB Ês/Iot ≥ 0 dB.

UE is allowed to drop up to 2 slots of its sidelink data reception per PSBCH monitoring occasion and UE is allowed to drop at most an aggregated window of 24ms of its sidelink data reception during Tdetect,SyncRef UE_V2X_CCA for the purpose of selection/reselection to the SyncRef UE.

-	The UE is allowed to extend Tdetect,SyncRef UE_V2X_CCA to max(4 x 50 SL-DRX cycle length, 8s) when the following conditions are satisfied over an evaluation period Tevaluate,SLSS in clause 12.3.1.1 if an NR cell is used as synchronization reference source, or Tevaluate,SLSS in clause 12.3.1.2 if an E-UTRA cell is used as synchronization reference source, or Tevaluate,SLSS_CCA in clause 12.3A.1.4 if an SLSS is used as synchronization reference source. If multiple SL-DRX cycles are configured, the SL-DRX cycle length is the longest one.

-	SS-RSRP is larger than syncTxThreshOoC.

The UE shall be capable of performing PSBCH-RSRP measurements for 3 identified intra-frequency SyncRef UE with the measurement period of Tmeasure,PSBCH-RSRP_CCA in Table 12.4A-1. It is assumed that the SyncRef UE do not drop or delay any SLSS transmission within the measurement period. Otherwise, the measurement period may be extended.

Table 12.4A-1: PSBCH-RSRP measurement period for intra-frequency SyncRef UE

| SL-DRX cycleNote 1 [ms] | Tmeasure,PSBCH-RSRP_CCANote 3 [ms] |
| --- | --- |
| No SL-DRX | (2 + LPSBCH)*160 |
| SL-DRX cycle ≤ 160 | (2 + LPSBCH)*160 |
| SL-DRX cycle > 160 | (2 + LPSBCH)*SL-DRX cycle |
| Note 1: If multiple SL-DRX cycles are configured, the SL-DRX cycle is the shortest one. Note 2: LPSBCH is the number of S-SSB periods in which the SLSS is not available due to CCA failures during Tmeasure,PSBCH-RSRP_CCA, where LPSBCH ≤ LPSBCH,max and LPSBCH,max = 2. Note 3: Upon LPSBCH exceeding LPSBCH,max, the UE shall not reselect to the measured candidate SyncRef UE based on the measurement result. |  |

When UE is synchronized to GNSS directly, before selection/reselection of the new synchronization reference source UE shall evaluate the GNSS synchronization source reliability for at least 20 seconds before changing the synchronization reference from GNSS to another synchronization reference source. UE shall be always synchronized to GNSS directly during the evaluation of GNSS synchronization source reliability.

## 12.5	L1 SL-RSRP measurements

### 12.5.1	Introduction

This clause contains the measurement requirements related to resource reselection and resource pre-emption of the UE capable of V2X sidelink communication.

### 12.5.2	SL-RSRP measurements

The UE physical layer shall be capable of performing the L1 SL-RSRP measurements on the carrier operating V2X sidelink communication for determining the subset of resources to be excluded in PSSCH resource selection in sidelink transmission mode 2 based on network configuration or pre-configuration. The L1 SL-RSRP measurement period corresponds to one slot and the measurement shall meet the L1 SL-RSRP measurement accuracy requirement in Clause 10. After resource (re-)selection procedure, re-evaluation is performed on the reserved resources by L1 SL-RSRP measurements before transmission of SCI with reservation when the conditions specified in TS 38.214 [26] are satisfied.

When the pre-emption mechanism is enabled for the resource pool that UE is monitoring and selecting resource from, after UE selects from the resource not excluded based on L1 SL-RSRP measurement procedure, the UE shall be capable of triggering reselection of already signalled resource(s) as a resource reservation when the conditions specified in TS 38.214 [26] are satisfied.

When partial sensing mechanism is enabled for the resource pool that UE is monitoring and selecting resource from, the UE shall be capable of performing the L1 SL-RSRP measurements on the sensing periods specified in TS 38.214 [26]. When SL-DRX is enabled, the UE shall be capable of performing the L1 SL-RSRP measurements and select resource during SL-DRX active time as specified in TS 38.214 [26].

## 12.6	Congestion Control measurements

The UE shall be capable of estimating the channel busy ratio for one or more transmission pools indicated by higher layers in TS 38.331 [2], based on SL-RSSI measurements provided by the physical layer.

When no sidelink transmissions occur, the UE physical layer shall perform a single-shot SL-RSSI measurement for each sub-channel included in all the slots configured as transmission pools.

The SL-RSSI measurement performed according to this clause shall meet the SL-RSSI measurement accuracy requirements defined in Clause 10.

The UE shall perform channel busy ratio (CBR) measurement based on SL-RSSI measurements as described in TS 38.215 [4].

## 12.7	Interruption

### 12.7.1	Interruptions to WAN due to V2X Sidelink Communication

This clause contains the requirements related to the interruptions on the PCell/serving cell due to V2X sidelink communication.

A UE capable of V2X sidelink communication may indicate its interest (initiation or termination) in V2X sidelink communication to the connected gNodeB using IE SidelinkUEInformationNR in TS 38.331 [2].

The UE is allowed an interruption of up to the duration shown in table 12.7.1-1 on the PCell/serving cell during the RRC reconfiguration procedure that includes the V2X sidelink communication configuration message SL-ConfigDedicatedNR in TS 38.331 [2] (setup and release). This interruption is for both uplink and downlink of the PCell/serving cell.

Table 12.7.1-1: Interruption length at V2X RRC reconfiguration

|  | NR Slot length (ms) | Interruption length (number of slots) |
| --- | --- | --- |
| 0 | 1 | 2 |
| 1 | 0.5 | 3 |
| 2 | 0.25 | 5 |
| 3 | 0.125 | 9 |

### 12.7.2	V2X Sidelink Communication Dropping due to synchronization source change

This clause contains the requirements related to the interruptions on the V2X sidelink communication due to synchronization source change.

For NR V2X UE not supporting gNB/eNB as synchronization reference source, UE is allowed to drop LTE and NR V2X SL transmission or reception for up to 1ms when synchronization source is changed, where the drop of LTE V2X SL transmission or reception applies only to in-device coexistence scenario in TS 38.213 [3]:

-	From GNSS

-	to syncRef UE that is synchronized to GNSS directly/in-directly

-	to syncRef UE that has the lowest priority

-	From syncRef UE that is synchronized to GNSS directly/in-directly

-	to GNSS

-	to syncRef UE that has the lowest priority

-	From syncRef UE that has the lowest priority

-	to GNSS

-	to syncRef UE that is synchronized to GNSS directly/in-directly

-	to syncRef UE that has the lowest priority

For NR V2X UE supporting gNB/eNB as synchronization reference source, UE is allowed to drop LTE and NR V2X SL transmission or reception for up to 1ms when synchronization source is changed, where the drop of  LTE V2X SL transmission or reception applies only to in-device coexistence scenario in TS 38.213 [3]:

-	From GNSS

-	to syncRef UE that is synchronized to GNSS directly/in-directly

-	to gNB/eNB

-	to syncRef UE that is synchronized to gNB/eNB directly

-	to syncRef UE that is synchronized to gNB/eNB in-directly

-	to syncRef UE that has the lowest priority

-	From syncRef UE that is synchronized to GNSS directly/in-directly

-	to GNSS

-	to gNB/eNB

-	to syncRef UE that is synchronized to gNB/eNB directly

-	to syncRef UE that is synchronized to gNB/eNB in-directly

-	to syncRef UE that has the lowest priority

-	From gNB or eNB

-	to GNSS

-	to syncRef UE that is synchronized to GNSS directly/in-directly

-	to eNB or gNB

-	to syncRef UE that is synchronized to gNB or eNB directly

-	to syncRef UE that is synchronized to gNB or eNB in-directly

-	to syncRef UE that has the lowest priority

-	From syncRef UE that is synchronized to gNB/eNB directly

-	to GNSS

-	to syncRef UE that is synchronized to GNSS directly/in-directly

-	to gNB/eNB

-	to syncRef UE that is synchronized to gNB/eNB directly

-	to syncRef UE that is synchronized to gNB/eNB in-directly

-	to syncRef UE that has the lowest priority

-	From syncRef UE that is synchronized to gNB/eNB in-directly

-	to GNSS

-	to syncRef UE that is synchronized to GNSS directly/in-directly

-	to gNB/eNB

-	to syncRef UE that is synchronized to gNB/eNB directly

-	to syncRef UE that is synchronized to gNB/eNB in-directly

-	to syncRef UE that has the lowest priority

-	From syncRef UE that has the lowest priority

-	to GNSS

-	to syncRef UE that is synchronized to GNSS directly

-	to syncRef UE that is synchronized to GNSS in-directly

-	to gNB/eNB

-	to syncRef UE that is synchronized to gNB/eNB directly

-	to syncRef UE that is synchronized to gNB/eNB in-directly

-	to syncRef UE that has the lowest priority

UE is allowed to interruption any V2X sidelink signals including PSSCH, PSCCH, PSBCH, PSFCH and SLSS signals.

### 12.7.3	Interruptions to WAN due to switching between E-UTRA V2X Sidelink and NR V2X Sidelink

This sub-clause contains the requirements related to the interruptions on the PCell/serving cell due to switching between E-UTRA V2X sidelink and NR V2X sidelink transmissions on a dedicated carrier. It is applicable for UE capable of both NR V2X sidelink and E-UTRA V2X sidelink transmissions in TDM-ed manner.

When a UE capable of switching between E-UTRA V2X sidelink and NR V2X sidelink, the UE is allowed an interruption of up to the duration shown in table 12.7.3-1 on the PCell/serving cell during the E-UTRA V2X sidelink and NR V2X sidelink switch.

This interruption is for both uplink and downlink of the PCell/serving cell.

Table 12.7.3-1: Interruption length due to switching between E-UTRA V2X and NR V2X

|  | Slot length (ms) | Interruption length (number of slots) |
| --- | --- | --- |
| 0 | 1 | 2 |
| 1 | 0.5 | 2 |
| 2 | 0.25 | 2 |
| 3 | 0.125 | 3 |

### 12.7.4	Interruptions to WAN at transitions between active and non-active during SL-DRX

Interruption on PCell/serving cell if configured due to V2X transitions between active and non-active during SL-DRX are allowed with up to 1% probability of missed ACK/NACK when the configured SL-DRX cycle is less than 640 ms, and 0.625% probability of missed ACK/NACK is allowed when the configured SL-DRX cycle is 640 ms or longer. When multiple SL-DRX cycles are configured, the shortest SL-DRX cycle is applied. Each interruption shall not exceed X slot as defined in table 12.7.4-1.

Table 12.7.4-1: Interruption length X at transition between active and non-active during SL-DRX

|  | NR Slot | Interruption length X (slots) |  |
| --- | --- | --- | --- |
|  | length (ms) | Sync | Async |
| 0 | 1 | 1 | 2 |
| 1 | 0.5 | 1 | 2 |
| 2 | 0.25 | 3 |  |

For SL-DRX active to inactive state transition, when the UE is in non-DRX or DRX on WAN and V2X is in sidelink resource allocation mode 2, the interruptions in this clause shall not apply when one of the following conditions is met:

-	While receiving paging,

-	While receiving system information.

In addition, for SL-DRX active to inactive state transition, when the UE is in non-DRX or DRX on WAN and V2X is in sidelink resource allocation mode 2 and SL DRX cycle is less than 320 ms, the interruptions in this clause shall not apply when one of the following conditions is met:

-	T310 timer is running for RLF on PCell

-	performing candidate beam detection on PCell/serving cell as specfied in section 8.5.5. and 8.5.6

During the U2N relay operation as defined in clause 5.8.14 of TS 38.331 [2], the interruption requirements defined in this clause apply only to the sidelink relay UE.

During the multipath relay operation [2], the interruption requirements defined in this clause apply to the SL remote UE or the SL relay UE.

### 12.7.5	Interruptions to V2X sidelink at transitions between active and non-active during DRX

Interruption on V2X sidelink if configured due to PCell transitions between active and non-active during DRX are allowed with up to 1% probability of missed ACK/NACK when the configured DRX cycle is less than 640 ms, and 0.625% probability of missed ACK/NACK is allowed when the configured DRX cycle is 640 ms or longer. It is only applied when HARQ process on V2X sidelink is supported. Each interruption shall not exceed X slot as defined in table 12.7.5-1.

Table 12.7.5-1: Interruption length X at transition between active and non-active during DRX

|  | NR V2X Slot | Interruption length X (slots) |  |
| --- | --- | --- | --- |
|  | length (ms) | Sync | Async |
| 0 | 1 | 1 | 2 |
| 1 | 0.5 | 1 | 2 |
| 2 | 0.25 | 3 |  |

### 12.7.6	Interruptions to V2X sidelink due to Active BWP switching Requirement

This clause contains the requirements related to the interruptions on the V2X sidelink due to BWP switch in FDM based intra-band concurrent V2X operation.

The requirements in clause 8.2.2.2.5 shall apply. The interrupted X slot is defined in Table 12.7.6-1.

Table 12.7.6-1: Interruption length X

|  | NR V2X Slot | Interruption length X (slots) |
| --- | --- | --- |
|  | length (ms) |  |
| 0 | 1 | 1 |
| 1 | 0.5 | 1 |
| 2 | 0.25 | 3 |

No sidelink communication happens during BWP switching delay period for TDM based intra-band concurrent operation.

### 12.7.7	Interruptions to WAN due to SyncRef UE detection and/or Sensing during SL DRX off duration

This sub-clause contains the requirements related to the interruptions on the PCell/serving cell due to SyncRef UE detection and/or Sensing during SL DRX off duration.

The requirements in clause 12.7.4 shall apply.

### 12.7.8	Interruptions at NR sidelink discovery configuration

This clause contains the requirements related to the interruptions on the PCell/serving cell due to NR sidelink discovery.

A UE capable of NR sidelink discovery may indicate its interest (initiation or termination) in NR sidelink discovery to the connected gNodeB using IE SidelinkUEInformationNR in TS 38.331 [2].

The UE is allowed an interruption of up to the duration shown in Table 12.7.8-1 on the PCell/serving cell during the RRC reconfiguration procedure that includes the NR sidelink discovery configuration message sl-DiscConfig in TS 38.331 [2] (setup and release). This interruption is for both uplink and downlink of the PCell/serving cell.

12.7.8-1: Interruption length at NR sidelink discovery configuration

|  | NR Slot length (ms) | Interruption length (number of slots) |
| --- | --- | --- |
| 0 | 1 | 2 |
| 1 | 0.5 | 3 |
| 2 | 0.25 | 5 |
| 3 | 0.125 | 9 |

### 12.7.9	Interruptions to WAN due to sidelink carrier addition/release

This sub-clause contains the requirements related to the interruptions on the PCell/serving cell due to sidelink component carrier addition/release. It is applicable for UE is in sidelink resource allocation mode 2.

A UE capable of V2X sidelink communication may indicate its interest (initiation or termination) in V2X sidelink communication to the connected gNodeB using IE SidelinkUEInformationNR in TS 38.331 [2].

The UE is allowed an interruption of up to the duration shown in table 12.7.9-1 on the PCell/serving cell when any number of sidelink component carriers is added or released. This interruption is for both uplink and downlink of the PCell/serving cell.

Table 12.7.9-1: Interruption length due to sidelink component carrier addition/release

|  | NR Slot length (ms) | Interruption length (number of slots) |
| --- | --- | --- |
| 0 | 1 | 2 |
| 1 | 0.5 | 3 |
| 2 | 0.25 | 5 |
| 3 | 0.125 | 9 |

For sidelink component carrier addition/release, when the UE is in RRC_IDLE/RRC_INACTIVE mode, the interruptions in this clause shall not apply when one of the following conditions is met:

-	While the UE is receiving paging,

-	While the UE is receiving system information.

## 12.8	Reliability of GNSS signal

This clause contains requirements regarding reliability of GNSS signal for the UE capable of  V2X sidelink communication under the following additional condition:

-	The UE is configured or pre-configured with parameters for enabling the UE to acquire the GNSS synchronization.

If UE considers GNSS is a reliable synchronization reference, the UE shall meet timing accuracy requirement as specified in 12.2 and frequency accuracy requirement as specified in 6.4E of TS 38.101-1 [18]. Otherwise, the UE shall be capable to select another synchronization reference source.

## 12.9	Scheduling availability

### 12.9.1	Scheduling availability of UE switching between E-UTRA sidelink and NR sidelink

This clause contains the restrictions on the scheduling availability for V2X sidelink due to switching between E-UTRA V2X sidelink and NR V2X sidelink transmission on a dedicated carrier. For the NR V2X sidelink, the assumed number of configured symbols in a slot is 14.

When switch from E-UTRA V2X sidelink to NR V2X sidelink occurs in NR slot ‘n’,

-	UE is not expected to transmit or receive on NR V2X sidelink on the slot ‘n’.

When switch from NR V2X sidelink to E-UTRA V2X sidelink occurs in NR slot ‘n-1’,

-	UE is not expected to transmit or receive on NR V2X sidelink on the slot ‘n-1’.

When switch from NR V2X sidelink to E-UTRA V2X sidelink occurs in E-UTRA subframe ‘n’,

-	UE is not expected to transmit or receive on E-UTRA V2X sidelink on the subframe ‘n’.

When switch from E-UTRA V2X sidelink to NR V2X sidelink occurs in E-UTRA subframe ‘n-1’,

-	UE is not expected to transmit or receive E-UTRA on V2X sidelink on the subframe ‘n-1’.

### 12.9.2	Scheduling availability of UE switching between Uu uplink  and V2X sidelink

This clause contains the restrictions on the scheduling availability for V2X sidelink due to switching between Uu uplink and V2X sidelink. For NR V2X sidelink, the assumed number of configured symbols in a slot is 14.

When switch from Uu uplink slot to V2X sidelink slot occurs in sidelink slot ‘n’,

-	UE is not expected to transmit or receive on V2X sidelink on the sidelink slot ‘n’.

When switch from V2X sidelink slot to Uu uplink slot occurs in sidelink slot ‘n-1’,

-	UE is not expected to transmit or receive on V2X sidelink on the sidelink slot ‘n-1’.

When switch from V2X sidelink slot to Uu uplink slot occurs in Uu slot ‘n’,

-	UE is not expected to transmit uplink or receive downlink on the Uu slot ‘n’.

When switch from Uu uplink slot to V2X sidelink slot occurs in Uu slot ‘n-1’,

-	UE is not expected to transmit uplink or receive downlink on the Uu slot ‘n-1’.

-	UE is not expected to transmit uplink or receive downlink on the Uu slot ‘n-1’.

## 12.10	Selection / Reselection of relay UE

### 12.10.1	Introduction

This section contains the requirements related to selection and reselection of relay UE.

The requirements apply for the selection and reselection of candidate relay UEs that are transmitting relay discovery signals within the resource pool as configured for the remote UE. The requirements are applicable to the following UE capabilities in TS 38.331 [2]:

-	a remote UE that communicates with the network via a UE-to-network (U2N) relay UE or

-	a remote UE that communicates with another via a UE-to-UE (U2U) relay UE.

### 12.10.2	Selection / Reselection of relay UE

For a remote UE configured by upper layer for relay operation, the remote UE shall search for candidate relay UEs for selection and/or reselection every discovery period which is determined by resource reservation period or SPS transmission periodicity configured by network.

If the remote UE has a selected sidelink relay UE, then the remote UE shall measure the SD-RSRP or SL-RSRP of the selected relay once in every four discovery periods and evaluate if it meets the relay selection criterion as defined in TS 38.331 [2] (clause 5.8.15.3 for U2N relay and clause 5.8.17.3 for U2U relay).

The remote UE shall measure SD-RSRP or SL-RSRP of the candidate relay UEs every Tmeasure, SL_Relay_Intra for relay UEs that are detected and measured according to the measurement rules.

For intra-frequency relay UEs that are detected, but that has not been selected or reselected to, the remote UE shall be capable of evaluating that the intra-frequency relay UE has met selection or reselection criterion defined in TS 38.331 [2] (clause 5.8.15.3 for U2N relay and clause 5.8.17.3 for U2U relay) within Tevaluate, SL_Relay_Intra as specified in table 12.10.2-1.

The minimum requirements are required to meet when the selected and candidate relay UEs are transmitting relay discovery message every discovery period.

Table 12.10.2-1: Tmeasure, SL_Relay_Intra and Tevaluate, SL_Relay_Intra

| Discovery Period [s] | Tmeasure,SL_Relay_Intra [s] (number of discovery periods) | Tevaluate, SL_Relay_Intra [s] (number of discovery periods) |
| --- | --- | --- |
| 0.04≤Discovery period≤10.24 | Note 1 (4) | Note 1 (16) |
| Note 1: Time depends upon the discovery period which is resource reservation period (in mode 2) or SPS transmission periodicity (in mode 1). Note 2: SL-RSRP or SD-RSRP can be derived from PSCCH-DMRS and/or PSSCH-DMRS. |  |  |

## 12.11	Component Carrier Addition and Release Delay for Sidelink Carrier Aggregation

Requirements in this clause are applicable to UE supporting NR sidelink carrier aggregation.

For UE configured in sidelink resource allocation mode 2, the delay within which the UE shall accomplish the NR sidelink component carrier addition/release is up to UE implementation.

## 12.12	Selection / Reselection of Synchronization Reference Source for NR SL Carrier Aggregation

Requirements in this clause are applicable to UE supporting NR sidelink carrier aggregation.

When the UE is synchronized to a SyncRef UE in a carrier and required only to search other SyncRef UEs in the synchronized carrier, the UE shall be able to identify a newly detectable NR SL SyncRef UE within Tdetect,SyncRef UE_V2X if the SyncRef UE meets the selection/reselection criterion defined in TS 38.331 [2]. The UE shall be capable of performing PSBCH-RSRP measurements for 3 identified NR SL SyncRef UE with the measurement period of Tmeasure,PSBCH-RSRP in Table 12.4-1.

When the synchronization reference source for NR sidelink carrier aggregation is lost and UE has to search SyncRef UE on the aggregated carriers which are configured as synchronization carrier, the UE shall be able to identify a newly detectable NR SL SyncRef UE within N×Tdetect,SyncRef UE_V2X if the SyncRef UE meets the selection/reselection criterion defined in TS 38.331 [2]. The UE shall be capable of performing PSBCH-RSRP measurements for 3 identified NR SL SyncRef UE per carrier with the measurement period of N×Tmeasure,PSBCH-RSRP. N is the number of aggregated carriers configured as synchronization carrier.

It is assumed that the identified NR SL SyncRef UE does not drop or delay any SLSS transmission within the measurement period. Otherwise, the measurement period may be extended.

When GNSS synchronization reference source is configured as the highest priority and

-	UE is synchronized to a SyncRef UE that is synchronized to GNSS directly or in-directly,

-	The value of Tdetect,SyncRef UE_V2X is as 1.6 seconds at S-SSB Es/Iot ≥0 dB, provided that the UE is allowed to drop a maximum of 30% of its SLSS transmissions on each carrier operating NR SL sidelink communication during Tdetect,SyncRef UE_V2X for the purpose of selection / reselection to the SyncRef UE.

-	in other case:

-	When UE is in non-SL-DRX

-	The value of Tdetect,SyncRef UE_V2X is as 8 seconds at S-SSB Es/Iot ≥0 dB, provided that the UE is allowed to drop a maximum of 6%  of its SLSS transmissions on each carrier operating SL sidelink communication during Tdetect,SyncRef UE_V2X for the purpose of selection/reselection to the SyncRef UE.

-	UE is allowed to drop up to 2 slots of its SL data reception on each carrier operating SL sidelink communication per PSBCH monitoring occasion and overall drop rate shall not exceed 0.3% of its SL data reception during Tdetect,SyncRef UE_V2X for the purpose of selection/reselection to the SyncRef UE.

-	When UE is in SL-DRX

-	The value of Tdetect,SyncRef UE_V2X is as 8 seconds at S-SSB Es/Iot ≥0 dB, provided that the UE is allowed to drop a maximum of 6%  of its SLSS transmissions on each carrier operating SL sidelink communication during Tdetect,SyncRef UE_V2X for the purpose of selection/reselection to the SyncRef UE.

-	UE is allowed to drop up to 2 slots of its SL data reception on each carrier operating SL sidelink communication per PSBCH monitoring occasion and UE is allowed to drop at most an aggregated window of 24 ms of its SL data reception during Tdetect,SyncRef UE_V2X for the purpose of selection/reselection to the SyncRef UE.

-	The UE is allowed to extend Tdetect,SyncRef UE_V2X to max(4×50 SL-DRX cycle length, 8s) when the following conditions are satisfied over an evaluation period Tevaluate,SLSS in clause 12.3.1.1 if an NR cell is used as synchronization reference source, or Tevaluate,SLSS in clause 12.3.1.2 if an E-UTRA cell is used as synchronization reference source, or Tevaluate,SLSS in clause 12.3.1.4 if an SLSS is used as synchronization reference source. If multiple SL-DRX cycles are configured, the SL-DRX cycle length is the longest one.

-	SS-RSRP is larger than syncTxThreshOoC.

When serving cell/PCell synchronization reference source is configured as the highest priority,

-	When UE is in non-SL-DRX

-	The value of Tdetect,SyncRef UE_V2X is as 8 seconds at S-SSB Es/Iot ≥0 dB, provided that the UE is allowed to drop a maximum of 6%  of its SLSS transmissions on each carrier operating NR sidelink communication during Tdetect,SyncRef UE_V2X for the purpose of selection/reselection to the SyncRef UE.

-	UE is allowed to drop up to 2 slots of its SL data reception on each carrier operating NR SL sidelink communication per PSBCH monitoring occasion and overall drop rate shall not exceed 0.3% of its SL data reception during Tdetect,SyncRef UE_V2X for the purpose of selection/reselection to the SyncRef UE.

-	When UE is in SL-DRX

-	The value of Tdetect,SyncRef UE_V2X is as 8 seconds at S-SSB Es/Iot ≥0 dB, provided that the UE is allowed to drop a maximum of 6%  of its SLSS transmissions on each carrier operating NR SL sidelink communication during Tdetect,SyncRef UE_V2X at most in an aggregated window of 480 ms during Tdetect,SyncRef UE_V2X for the purpose of selection/reselection to the SyncRef UE.

-	UE is allowed to drop up to 2 slots of its SL data reception per PSBCH monitoring occasion and UE is allowed to drop at most an aggregated window of 24 ms of its SL data reception during Tdetect,SyncRef UE_V2X for the purpose of selection/reselection to the SyncRef UE.

-	The UE is allowed to extend Tdetect,SyncRef UE_V2X to max(4×50 SL-DRX cycle length, 8s) when the following conditions are satisfied over an evaluation period Tevaluate,SLSS in clause 12.3.1.1 if an NR cell is used as synchronization reference source, or Tevaluate,SLSS in clause 12.3.1.2 if an E-UTRA cell is used as synchronization reference source, or Tevaluate,SLSS in clause 12.3.1.4 if an SLSS is used as synchronization reference source. If multiple SL-DRX cycles are configured, the SL-DRX cycle length is the longest one.

-	SS-RSRP is larger than syncTxThreshOoC.

# 12A	NR Sidelink Measurements for Positioning

## 12A.1	Introduction

Clause 12A contains requirements for UE capable of V2X or 5G ProSe operation, which is also capable of performing SL positioning measurements defined in TS 38.215 [4], including SL RSTD, SL PRS-RSRP, SL Rx-Tx time difference, SL PRS-RSRPP measurements, SL AoA, and SL RTOA, provided that:

- 	The SL-PRS are received on NR PC5 interface within a single sidelink BWP on a single carrier,

-	The UE is in any cell selection state or the UE is inside NG-RAN coverage while configured for SL positioning operation on a sidelink carrier, which is dedicated to only sidelink operation, and configured with only a PCell on WAN carrier, and

-	The measuring UE is the location target UE or an anchor UE, and

-	The UE is not required to monitor PSCCH, which is associated with SL-PRS in the same slot, outside the SL-DRX active time.

Any cell selection state refers to a UE that is out of network coverage and is not associated with a serving cell on any carrier as defined in TS 38.304 [1].

When a UE in RRC_CONNECTED state is performing transmissions and/or reception for SL positioning operation, the UE shall meet all the requirements specified in Clause 9 assuming that UE has a dedicated RX/TX chain for the sidelink operation. Otherwise, the UE may interrup the SL positioning measurements in order to meet the measurement requirements specified in Clause 9.

When a UE in RRC_CONNECTED state is performing transmissions and/or reception for SL positioning operation, the UE shall meet all relevant requirements related to its WAN operation, assuming that UE has a dedicated RX/TX chain for the sidelink operation. Otherwise, the UE may interrup the SL positioning measurements or SL-PRS transmissions in order to meet the measurement requirements related to its WAN operation.

Prior to performing SL-PRS based measurements, the target UE may need to perform the discovery procedure to discover anchor UEs according to TS 38.305 [22].

## 12A.2	SL RSTD measurements

### 12A.2.1	Introduction

The requirements in clause 12A.2 apply for SL RSTD measurements of the first and additional paths.

The requirements in clause 12A.2 shall apply provided the UE has received a RequestLocationInformation message from LMF or another UE via SLPP [37] requesting the UE to measure and report SL RSTD measurements defined in TS 38.215 [4] based on SL-PRS.

### 12A.2.2	Requirements Applicability

The requirements in clause 12A.2 apply for periodic, aperiodic, and triggered RSTD measurements, provided:

-	SL RSTD related side conditions given in clause 10.4A.2.2 for FR1 are fulfilled, for a corresponding Band.

### 12A.2.3	Measurement Capability

UE SL RSTD measurement capability is as indicated by the UE in:

SL-TDOA-ProvideCapabilities, according to TS 38.355 [37].

### 12A.2.4	Measurement Reporting Requirements

The measurement reporting delay is defined as the time between the moment when the measurement report is triggered and the moment when the UE starts to transmit the measurement report over the air interface.

For UE reporting to LMF, this requirement assumes that the measurement report is not delayed by other SLPP signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH where TTIDCCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PUSCH with subframe or slot or subslot duration.

For UE reporting to another UE, this requirement assumes that the measurement report is not delayed by other SLPP signalling on the STCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the transmitted STCH. The delay uncertainty is: 2 x TTISTCH where TTISTCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PSSCH with subframe or slot or subslot duration.

This measurement reporting delay excludes any delay caused by no SL resources for UE to send the measurement report.

The reported SL RSTD measurement values contained in measurement reports shall be based on the measurement report mapping requirements specified in clause 10.4A.2.1.

The SL RSTD measurements performed and reported according to this section shall meet the SL RSTD measurement accuracy requirements in clause 10.4A.2.2, for each measured SL-PRS resource.

### 12A.2.5	Measurements Period Requirements

When the physical layer receives the last of SL-TDOA-ProvideAssistanceData and SL-TDOA-RequestLocationInformation from LMF or another UE via SLPP [37], the UE shall be able to perform multiple SL RSTD measurements based on SL-PRS from one or more other SL UEs (up to the UE capability specified in Clause 12A.2.3), with each SL RSTD measurement based on  SL-PRS from the reference UE and SL-PRS from another anchor UE, as defined in TS 38.215 [4]. The SL RSTD measurement shall be performed during the measurement period TSL RSTD,Total defined as:

TSL RSTD,Total=s=1STSL RSTD,effect,s  ,

where

S is the number of samples per measured link, defined below:

S = 1 for SL-PRS bandwidth>48 PRBs,

S = 4 for SL-PRS bandwidth≤48 PRBs, and

for each SL-PRS sample s of the target measured link, which is received within a slot where the UE receives SCI and the associated SL-PRS within its capabilities [Components 2 and 3 of FG 41-1-1], TSL RSTD,effect,s is defined as:

TSL RSTD,effect,s=ts+1-ts , for s<S, where ts+1 and ts are the beginning of the first slot  of SL-PRS sample s+1 and SL-PRS sample s, respectively,

TSL RSTD,effect,s=Tdur,s+ΔSLproc , for s=S,

Tdur,s is the duration of the slot carrying SL-PRS sample s of the SL RSTD measurement,

ΔSLproc is the processing time given by the UE capability in [Components 4 of FG 41-1-1].

[A UE may drop one or more SL PRS measurement samples if the number of active slots and number of active resources per slot for the ongoing SL PRS measurement exceed the UE capabilities in [FG 41-1-1]. For a single-sample measurement, the whole measurement may not be performed.]

If the synchronization reference source changes during TSL RSTD,Total at the measuring UE, while the UE is performing the SL RSTD measurement, then the UE shall restart the SL RSTD measurement after the synchronization reference source change and shall send the measurement report during a measurement period, which can be longer than TSL RSTD,Total.

The requirements in this clause do not apply, when the synchronization reference source changes during TSL RSTD,Total at the UE transmitting SL-PRS for the SL-RSTD measurement.

The requirements in this clause apply, provided that no SL-PRS symbols are dropped due to, e.g., selection or reselection of synchronization reference source according to clause 12.4 during the measurement period TSL RSTD,Total. Otherwise, the measurement period can be longer.

The requirements in this clause apply, provided that the reception of slots containing SL-PRS is not interrupted during the measurement period TSL RSTD,Total. Otherwise, if the reception of the slots containing SL-PRS is interrupted, the measurement period can be longer.

## 12A.3	SL PRS-RSRP measurements

### 12A.3.1	Introduction

The requirements in clause 12A.3 apply for SL PRS-RSRP measurements and for SL PRS-RSRP path measurements of the first and additional paths.

The requirements in clause 12A.3 shall apply provided the UE has received a RequestLocationInformation message from LMF or another UE via SLPP [37] requesting the UE to measure and report SL PRS-RSRP measurements defined in TS 38.215 [4] based on SL-PRS.

### 12A.3.2	Requirements Applicability

The requirements in clause 12A.3 apply for periodic, aperiodic, and triggered SL PRS-RSRP measurements, provided:

-	SL PRS-RSRP related side conditions given in clause 10.4A.3.2 for FR1 are fulfilled, for a corresponding Band.

### 12A.3.3	Measurement Capability

UE SL PRS-RSRP measurement capability is as indicated by the UE in:

-	SL-TDOA-ProvideCapabilities, SL-RTT-ProvideCapabilities, SL-AOA-ProvideCapabilities, or SL-TOA-ProvideCapabilities, according to TS 38.355 [37].

### 12A.3.4	Measurement Reporting Requirements

The measurement reporting delay is defined as the time between the moment when the measurement report is triggered and the moment when the UE starts to transmit the measurement report over the air interface.

For UE reporting to LMF, this requirement assumes that the measurement report is not delayed by other SLPP signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH where TTIDCCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PUSCH with subframe or slot or subslot duration.

For UE reporting to another UE, this requirement assumes that the measurement report is not delayed by other SLPP signalling on the STCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the transmitted STCH. The delay uncertainty is: 2 x TTISTCH where TTISTCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PSSCH with subframe or slot or subslot duration.

This measurement reporting delay excludes any delay caused by no SL resources for UE to send the measurement report.

The reported SL PRS-RSRP measurement values contained in measurement reports shall be based on the measurement report mapping requirements specified in clause 10.4A.3.1.

The SL PRS-RSRP measurements performed and reported according to this section shall meet the SL PRS-RSRP measurement accuracy requirements in clause 10.4A.3.2, for each measured SL-PRS resource.

### 12A.3.5	Measurements Period Requirements

When the physical layer receives the last of:

-	SL-TDOA-ProvideAssistanceData and SL-TDOA-RequestLocationInformation, or

-	SL-AOA-ProvideAssistanceData and SL-AOA-RequestLocationInformation, or

-	SL-TOA-ProvideAssistanceData and SL-TOA-RequestLocationInformation, or

-	SL-RTT-ProvideAssistanceData and SL-RTT-RequestLocationInformation,

from LMF or another UE via SLPP [37], the UE shall be able to perform multiple SL PRS-RSRP measurements based on SL-PRS from one or more other SL UEs (up to the UE capability specified in Clause 12A.3.3), as defined in TS 38.215 [4]. The SL PRS-RSRP measurement shall be performed during the measurement period TSL PRS-RSRP,Total defined as:

TSL PRS-RSRP,Total=s=1STSL PRS-RSRP,effect,s  ,

where

S is the number of samples per measured link, defined below:

S = 1 for SL-PRS bandwidth>48 PRBs,

S = 4 for SL-PRS bandwidth≤48 PRBs,

For each SL-PRS sample s of the target measured link, which is received within a slot where the UE receives SCI and the associated SL-PRS within its capabilities [Components 2 and 3 of FG 41-1-1], TSL PRS-RSRP,effect,s is defined as:

TSL PRS-RSRP,effect,s=ts+1-ts , for s<S, where ts+1 and ts are the beginning of the first slot of SL-PRS sample s+1 and SL-PRS samples s, respectively,

TSL PRS-RSRP,effect,s=Tdur,s+ΔSLproc , for s=S, is the time for completing the last sample of the SL PRS-RSRP measurement,

Tdur,s is the duration of the slot carrying SL-PRS sample s of the SL PRS-RSRP measurement,

ΔSLproc is the processing time given by the UE capability in [Components 4 of FG 41-1-1].

If the synchronization reference source changes during TSL PRS-RSRP,Total at the measuring UE, while the measuring UE is performing the SL PRS-RSRP measurement, then the UE shall continue performing the SL PRS-RSRP measurement after the synchronization reference source change, while meeting the measurement period TSL PRS-RSRP,Total defined in this clause and the accuracy requirements in clause 10.4A.3.2.

The requirements in this clause do not apply, when the synchronization reference source changes during TSL PRS-RSRP, Total at the UE transmitting SL-PRS for the SL PRS-RSRP measurement.

The requirements in this clause apply, provided that no SL-PRS symbols are dropped due to, e.g., selection or reselection of synchronization reference source according to clause 12.4 during the measurement period TSL PRS-RSRP,Total. Otherwise, the measurement period can be longer.

The requirements in this clause apply, provided that the reception of slots containing SL-PRS is not interrupted during the measurement period TSL PRS-RSRP,Total. Otherwise, if the reception of the slots containing SL-PRS is interrupted, the measurement period can be longer.

## 12A.4	SL Rx-Tx measurements

### 12A.4.1	Introduction

The requirements in clause 12A.4 apply for SL Rx-Tx measurements of the first and additional paths.

The requirements in clause 12A.4 shall apply provided the UE has received SL-RTT-RequestLocationInformation from LMF or another UE via SLPP [37] requesting the UE to measure and report SL Rx-Tx time difference measurements defined in TS 38.215 [4] based on SL-PRS.

### 12A.4.2	Requirements Applicability

The requirements in clause 12A.4 apply for periodic, aperiodic, and triggered SL Rx-Tx time difference measurements, provided:

-	SL Rx-Tx time difference related side conditions given in clause 10.4A.4.2 for FR1 are met for a corresponding Band.

-	The actual time difference between the corresponding SL-PRS transmission and reception used to derive the measurement is no larger than 160 ms.

### 12A.4.3	Measurement Capability

SL Rx-Tx time difference measurement capability is as indicated by the UE in SL-RTT-ProvideCapabilities according to TS 38.355 [37].

### 12A.4.4	Measurement Reporting Requirements

The measurement reporting delay is defined as the time between the moment when the measurement report is triggered and the moment when the UE starts to transmit the measurement report over the air interface.

For UE report to LMF, this requirement assumes that the measurement report is not delayed by other SLPP signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH where TTIDCCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PSSCH with subframe or slot or subslot duration.

For UE report to another UE, this requirement assumes that the measurement report is not delayed by other SLPP signalling on the STCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the sidelink STCH. The delay uncertainty is: 2 x TTISTCH where TTISTCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PSSCH with subframe or slot or subslot duration.

The measurement reporting delay excludes any delay caused by no SL resources for UE to send the measurement report.

The reported SL Rx-Tx time difference measurement values contained in measurement reports shall be based on the measurement report mapping requirements specified in clause 10.4A.4.1.

The SL Rx-Tx time difference measurements performed and reported according to this section shall meet the SL Rx-Tx time difference measurement accuracy requirements in clause 10.4A.4.2, for each measured SL-PRS resource.

### 12A.4.5	Measurement Period Requirements

When the physical layer receives SL-RTT-ProvideAssistanceData message from SL-RTT-RequestLocationInformation message from LMF or another UE via SLPP [37], the UE shall be able to perform multiple SL Rx-Tx time difference measurements based on SL-PRS from one or more other SL UEs (up to the UE capability specified in Clause 12A.4.3), as defined in TS 38.215 [4]. For each individual SL-PRS resource measured by a UE, the SL Rx-Tx time difference measurement is performed during TSL Rx-Tx,total defined as:

TSL Rx-Tx,total=s=1STSL Rx-Tx,effect,s+Tuncertain ,

where,

S is the number of samples for a single SL Rx-Tx measurement defined below:

S = 1 for SL-PRS bandwidth > 48 PRBs,

S = 4 for SL-PRS bandwidth≤48 PRBs.

For SL-PRS sample s, which is received within a slot where the UE receives SCI and the associated SL-PRS is within its capabilities [Components 2 and 3 of FG 41-1-1], TSL Rx-Tx,effect,s is defined as:

TSL Rx-Tx,effect,s=ts+1-ts, for s<S, where ts and ts+1 are the start of the s-th and (s+1)-th slot of SL-PRS samples s and SL-PRS samples s+1, respectively.

TSL Rx-Tx,effect,s=Tdur,s+ΔSLproc , for s = S,

Tdur,s is the duration of the slot carrying SL-PRS sample s of SL Rx-Tx measurement.

ΔSLprocis the processing time as indicated via capability [component 4 of FG 41-1-1] of the UE performing the SL Rx-Tx time difference measurement.

Tuncertain is defined as below:

If the UE reports the transmission timestamp of a SL PRS as defined in TS 38.215 [4], and the SL PRS transmission occurs after the SL PRS reception used to derive the measurement, Tuncertain is the additional time delay from the SL PRS reception until the actual SL PRS transmission.

Otherwise, Tuncertain=0.

[A UE may drop one or more SL PRS measurement samples if the number of active slots and number of active resources per slot for the ongoing SL PRS measurement exceed the UE capabilities in [FG 41-1-1].

- For a single-sample measurement, the whole measurement may not be performed.]

If the synchronization reference source changes during TSL Rx-Tx,total at the measuring UE, while the measuring UE is performing the SL Rx-Tx time difference measurement, then the measuring UE shall restart the SL Rx-Tx time difference measurement and shall send the measurement report during a measurement period, which can be longer than TSL Rx-Tx,total.

The requirements in this clause do not apply, when the synchronization reference source changes during TSL Rx-Tx,total at the UE transmitting SL-PRS for the SL Rx-Tx measurement.

The requirements in this clause apply, provided that no SL-PRS symbols are dropped due to, e.g., selection or reselection of synchronization reference source according to clause 12.4 during the measurement period TSL Rx-Tx,total. Otherwise, the measurement period can be longer.

The requirements in this clause apply, provided that the reception of slots containing SL-PRS is not interrupted during the measurement period TSL Rx-Tx,total. Otherwise, if the reception of the slots containing SL-PRS is interrupted, the measurement period can be longer.

## 12A.5	SL PRS-RSRPP measurements

### 12A.5.1	Introduction

The requirements in clause 12A.5 shall apply provided the UE has received SL-TDOA-RequestLocationInformation or SL-AOA-RequestLocationInformation or SL-TOA-RequestLocationInformation or SL-RTT-RequestLocationInformation from LMF or another UE via SLPP requesting the UE to measure and report SL PRS-RSRPP measurements defined in TS 38.215 [4].

### 12A.5.2	Requirements Applicability

The requirements in clause 12A.5 apply for periodic and triggered SL PRS-RSRPP measurements, provided:

-	SL PRS-RSRPP related side conditions given in clause 10.4A.5.2  for FR1 are met for a corresponding Band.

### 12A.5.3	Measurement Capability

SL PRS-RSRPP measurement capability is as indicated by the UE in SL-TDOA-ProvideCapabilities, SL-RTT-ProvideCapabilities, SL-AOA-ProvideCapabilities, or SL-TOA-ProvideCapabilities according to TS 38.355 [37].

### 12A.5.4	Measurement Reporting Requirements

The measurement reporting delay is defined as the time between the moment when the measurement report is triggered and the moment when the UE starts to transmit the measurement report over the air interface.

For UE report to LMF, this requirement assumes that the measurement report is not delayed by other SLPP signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH where TTIDCCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PSSCH with subframe or slot or subslot duration.

For UE report to another UE, this requirement assumes that the measurement report is not delayed by other SLPP signalling on the STCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the sidelink STCH. The delay uncertainty is: 2 x TTISTCH where TTISTCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PSSCH with subframe or slot or subslot duration.

This measurement reporting delay excludes any delay caused by no SL resources for UE to send the measurement report.

The reported SL PRS-RSRPP measurement values contained in measurement reports shall be based on the measurement report mapping requirements specified in clauses 10.4A.5.1.

The SL PRS-RSRPP measurements performed and reported according to this section shall meet the SL PRS-RSRPP measurement accuracy requirements in clause 10.4A.5.2, for each measured SL-PRS resource.

### 12A.5.5	Measurement Period Requirements

When the physical layer receives

-	SL-TDOA-ProvideAssistanceData message and SL-TDOA-RequestLocationInformation message, or

-	SL-AOA-ProvideAssistanceData message and SL-AOA-RequestLocationInformation message, or

-	SL-TOA-ProvideAssistanceData message and SL-TOA-RequestLocationInformation message, or

-	SL-RTT-ProvideAssistanceData message and SL-RTT-RequestLocationInformation message,

from LMF or another UE via SLPP, the measurement period requirements for SL PRS-RSRP defined in 12A.3.5 are reused for PRS-RSRPP measurement.

## 12A.6	SL AoA measurements

### 12A.6.1	Introduction

The requirements in clause 12A.6 apply for SL AoA measurements of the first and additional paths.

The requirements in clause 12A.6.5 shall apply for azimuth angle of arrival (A-AoA) and zenith angle of arrival (Z-AoA) first path measurement, provided the UE has received SL-AoA-RequestLocationInformation from LMF or another UE via SLPP [37] requesting the UE to measure and report SL AoA measurements defined in TS 38.215 [4] based on SL-PRS.

### 12A.6.2	Requirements Applicability

The requirements in clause 12A.6 apply for periodic, aperiodic, and triggered SL AoA measurements, provided:

-	Conditions defined in clause 7.3E of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	SL AoA related side conditions given in clause [TBD] for FR1 are met for a corresponding Band.

### 12A.6.3	Measurement Capability

SL AoA measurement capability is as indicated by the UE in:

SL-AoA-ProvideCapabilities according to TS 38.355 [37].

### 12A.6.4	Measurement Reporting Requirements

The measurement reporting delay is defined as the time between the moment when the measurement report is triggered and the moment when the UE starts to transmit the measurement report over the air interface.

For UE reporting to LMF, this requirement assumes that the measurement report is not delayed by other SLPP signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH where TTIDCCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PUSCH with subframe or slot or subslot duration.

For UE reporting to another UE, this requirement assumes that the measurement report is not delayed by other SLPP signalling on the STCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the sidelink STCH. The delay uncertainty is: 2 x TTISTCH where TTISTCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PSSCH with subframe or slot or subslot duration.

The measurement reporting delay excludes any delay caused by no SL resources or no SL-PRS resources for UE to send the measurement report.

The reported SL AoA measurement values contained in measurement reports shall be based on the measurement report mapping requirements specified in clauses 10.4A.6.1.

### 12A.6.5	Measurement Period Requirements

When the physical layer receives the last of SL-AoA-ProvideAssistanceData message from SL-AoA-RequestLocationInformation message from LMF or another UE via SLPP, the UE shall be able to measure multiple SL AoA measurements based on SL-PRS from one or more other SL UEs (up to the UE capapbility specified in 12A.6.3), as defined in TS 38.215 [4]. The SL AoA measurement shall be performedduring the measurement period TSL AoA,total defined as:

TSL AoA,total=s=1STSL AoA, effect,s ,

where,

S is the number of samples for the SL AoA measurement, defined as below:

S = 1 for SL-PRS bandwidth > 48 PRBs,

S = 4 for SL-PRS bandwidth ≤ 48 PRBs,

For each SL-PRS sample s, which is received within a slot where the UE receives SCI and the associated SL-PRS is within its capabilities [Components 2 and 3 of FG 41-1-1]. TSL AoA,effect,s is defined as below,

TSL AoA,effect,s=ts+1-ts  for s < S, where ts and ts+1 are the beginning of the slots of SL-PRS sample s and SL-PRS sample s+1, respectively.

TSL AoA,effect,s=Tdur,s+ΔSLproc for s = S,

Tdur,s is the duration of slot carrying SL-PRS sample s of the SL AoA measurement,

ΔSLproc is the processing time indicated via UE capability in [component 4 of FG 41-1-1] of the UE performing SL AoA measurement.

[A UE may drop one or more SL PRS measurement samples if the number of active slots and number of active resources per slot for the ongoing SL PRS measurement exceed the UE capabilities in [FG 41-1-1].

-	For a single-sample measurement, the whole measurement may not be performed.]

If the synchronization reference source of the measuring UE changes during TSL AoA,total, while the UE is performing the SL AoA measurements, then the measuring UE shall continue performing the SL AoA measurement after the synchronization reference source change, while meeting the requirements in this clause.

The requirements in this clause do not apply, when the synchronization reference source changes during TSL AoA,total at the UE transmitting SL-PRS for the SL AoA measurement.

The requirements in this clause apply provided that no SL-PRS symbols for the SL AoA measurement are dropped due to e.g., the selection or reselection of synchronization reference source according to clause 12.4 during the measurement period. Otherwise, the measurement period can be extended.

The requirements in this clause, apply provided that reception of slots containing SL-PRS for the SL AoA measurement is not interrupted e.g., due to network coverage change. Otherwise, if the reception of the slots containing SL-PRS is interrupted, the measurement period can be extended.

## 12A.7	SL RTOA measurements

### 12A.7.1	Introduction

The requirements in clause 12A.7 apply for SL RTOA measurements of the first and additional paths.

The requirements in clause 12A.7.5 shall apply provided the UE has received SL-RTOA-RequestLocationInformation from LMF or another UE via SLPP requesting the UE to measure and report SL RTOA measurements defined in TS 38.215 [4].

### 12A.7.2	Requirements Applicability

The requirements in clause 12A.7 apply for periodic, aperiodic, and triggered SL RTOA measurements, provided:

-	Conditions defined in clause 7.3E of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	SL RTOA related side conditions given in clause B.4A.1 for FR1 are met for a corresponding Band.

### 12A.7.3	Measurement Capability

SL RTOA measurement capability is as indicated by the UE in SL-RTOA-ProvideCapabilities according to TS 38.355 [37].

### 12A.7.4	Measurement Reporting Requirements

The measurement reporting delay is defined as the time between the moment when the measurement report is triggered and the moment when the UE starts to transmit the measurement report over the air interface.

For UE reporting to LMF, this requirement assumes that the measurement report is not delayed by other SLPP signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH where TTIDCCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PUSCH with subframe or slot or subslot duration.

For UE reporting to another UE, this requirement assumes that the measurement report is not delayed by other SLPP signalling on the STCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the sidelink STCH. The delay uncertainty is: 2 x TTISTCH where TTISTCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PSSCH with subframe or slot or subslot duration.

The measurement reporting delay excludes any delay caused by no SL resources or no SL-PRS resources for UE to send the measurement report.

The reported SL RTOA measurement values contained in measurement reports shall be based on the measurement report mapping requirements specified in clauses 10.4A.7.1.

### 12A.7.5	Measurement Period Requirements

When the physical layer receives the last of SL-RTOA-ProvideAssistanceData message from SL-RTOA-RequestLocationInformation message from LMF or another UE via SLPP, the UE shall be able to measure multiple SL RTOA measurements based on SL-PRS from one or more other SL UEs (up to the UE capapbility specified in 12A.7.3), as defined in TS 38.215 [4]. The SL RTOA measurement shall be performed during TSL RTOA,total defined as:

TSL RTOA,total=s=1STSL RTOA, effect,s ,

where,

S is the number of samples for the SL RTOA measurement, defined as below:

S = 1 for SL-PRS bandwidth > 48 PRBs,

S = 4 for SL-PRS bandwidth ≤ 48 PRBs,

For SL-PRS sample s, which is received within a slot where the UE receives SCI and the associated SL-PRS is within its capabilities [Components 2 and 3 of FG 41-1-1]. TSL RTOA,effect,s is defined as below,

TSL RTOA,effect,s=ts+1-ts, for s<S, where ts and ts+1 are the beginning of the slots of SL-PRS sample s and SL-PRS sample s+1, respectively,

TSL RTOA,effect,s=Tdur,s+ΔSLproc for s = S,

Tdur,s is the duration of the slot carrying SL-PRS sample s of the SL RTOA measurement.

ΔSLprocis the processing time indicated via capability in [component 4 of FG 41-1-1] of the UE performing SL RTOA measurement.

If the synchronization reference source of the measuring UE changes during TSL RTOA,total, while the UE is performing the SL RTOA measurements, then the measuring UE shall restart the SL RTOA measurement after the synchronization reference source change and shall send the measurement report during a measurement period, which can be longer than TSL RTOA,total.

The requirements in this clause do not apply, when the synchronization reference source changes during TSL RTOA,Total at the UE transmitting SL-PRS for the SL RTOA measurement.

The requirements in this clause apply provided that no SL-PRS symbols for the SL RTOA measurement that are dropped due to e.g., the selection or reselection of synchronization reference source according to clause 12.4 during the measurement period. Otherwise, the measurement period can be extended.The requirements in this clause apply, provided that reception of slots containing SL-PRS for the SL RTOA measurement is not interrupted due to network coverage change. Otherwise, if the reception of the slots containing SL-PRS is interrupted, the measurement period can be longer

Annex A (normative): Test Cases

# A.1	Purpose of annex

The purpose of annex A is to define test cases and the relevant configurations for verifying various types of RRM requirements with NR cells, including SA NR, EN-DC, and NE-DC deployments as well as standalone E-UTRA deployments.

# A.2	Requirement classification for statistical testing

Requirements in this specification are either expressed as absolute requirements with a single value stating the requirement or expressed as a success rate. There are no provisions for the statistical variations that will occur when the parameter is tested.

Annex A outlines the tests in more detail and lists the test parameters needed. The test will result in an outcome of a test variable value for the device under test (DUT) inside or outside the test limit. Overall, the probability of a "good" DUT being inside the test limit(s) and the probability of a "bad" DUT being outside the test limit(s) should be as high as possible. For this reason, when selecting the test variable and the test limit(s), the statistical nature of the test is accounted for.

The statistical nature depends on the type of requirement. Some have large statistical variations, while others are not statistical in nature at all. When testing a parameter with a statistical nature, a confidence level is set. This establishes the probability that a DUT passing the test actually meets the requirements and determines how many times a test has to be repeated and what the pass and fail criteria are. Those aspects are not covered by TS 38.133. The details of the tests on how many times to run it and how to establish confidence in the tests are described in TS 38.533 [5]. This Annex establishes the variable to be used in the test and whether it can be viewed as statistical in nature or not.

## A.2.1	Types of requirements in TS 38.133

### A.2.1.1	Time and delay requirements on UE higher layer actions

A very large part of the RRM requirements are delay requirements:

-	In RRC_IDLE state mobility (clause A.6.1 and A.7.1) there is cell re-selection delay.

-	In RRC_CONNECTED state mobility (clauses A.4.3, A.4.6, A.5.3, A.5.6, A.6.3, A.6.6, A.7.3 and A.7.6) there is handover delay, cell search delay and measurement reporting delay.

-	In RRC Connection Control (clauses A.4.3.2, A.5.3.2, A.6.3.2 and A.7.3.2) there is RRC re-establishment delay.

All have in common that the UE is required to perform an action observable in higher layers (e.g. camp on the correct cell) within a certain time after a specific event (e.g. when a new strong pilot or reference signal appears). The delay time is statistical in nature for several reasons, among others that several of the measurements are performed by the UE in a fading radio environment.

The variations make a strict limit unsuitable for a test. Instead there is a condition set for a correct action by the UE, e.g. that the UE shall camp on the correct cell within X seconds. Then the rate of correct events is observed during repeated tests and a limit is set on the rate of correct events, usually 90% correct events are required. How the limit is applied in the test depends on the confidence required, further detailed are in TS 38.533 [5].

### A.2.1.2	Measurements of power levels, relative powers and time

A very large number of requirements are on measurements that the UE performs:

-	In RRC_CONNECTED state mobility (clauses A.4.3, A.5.3, A.6.3 and A.7.3) there are measurement reports.

-	In Measurement Performance Requirements (clauses A.4.7, A.5.7, A.6.7 and A.7.7) there are requirements for all type of measurements.

The accuracy requirements on measurements are expressed in this specification as a fixed limit (e.g. +/-X dB), but the measurement error will have a distribution that is not easily confined in fixed limits. Assuming a Gaussian distribution of the error, the limits will have to be set at +/-3.29 if the probability of failing a "good DUT" in a single test is to be kept at 0.1%. It is more reasonable to set the limit tighter and test the DUT by counting the rate of measurements that are within the limits, in a way similar to the requirements on delay.

### A.2.1.3	Implementation requirements

A few requirements are strict actions the UE should take or capabilities the UE should have, without any allowance for deviations. These requirements are absolute and should be tested as such. Examples are:

-	"Event triggered report rate" in RRC_CONNECTED state mobility (clauses A.4.3, A.4.6, A.5.3, A.5.6, A.6.3, A.6.6, A.7.3 and A.7.6),

-	"Correct behaviour at time-out" in RRC connection control (clauses A.4.3.2, A.5.3.2, A.6.3.2 and A.7.3.2).

### A.2.1.4	Physical layer timing requirements

There are requirements on Timing (clauses A.4.4, A.5.4, A.6.4 and A.7.4). There are both absolute and relative limits on timing accuracy depending upon the type of requirement. Examples are:

-	Initial Transmit Timing (clauses A.4.4.1, A.5.4.1, A.6.4.1 and A.7.4.1) has an absolute limit on timing accuracy.

-	Timing Advance (clauses A.4.4.2, A.5.4.2, A.6.4.2 and A.7.4.2) has a relative limit on timing accuracy.

### A.2.1.5	Requirements under CCA

A few requirements include CCA failures in DL and or UL. Considering that the CCA model is of statistical nature, requirements that include CCA failures are always considered of statistical nature.

# A.3	RRM test configurations

## A.3.1	Reference measurement channels

### A.3.1.1	PDSCH

#### A.3.1.1.1	FDD

Table A.3.1.1.1-1: PDSCH Reference Measurement Channels for SCS=15kHz

| Parameter | Unit | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | SR.1.1 FDD | SR.1.2 FDD |  |  |  |  |  |
| Channel bandwidth | MHz | Defined in test case | Defined in test case by the parameter NRB,c |  |  |  |  |  |
| Number of transmitter antennas |  | 1 | 1 |  |  |  |  |  |
| Allocated resource blocks for PDSCH Note 1 |  | 24 | Defined by test case |  |  |  |  |  |
| Allocated slots per Radio Frame |  | 10 | 10 |  |  |  |  |  |
| Radio frame containing SSB | slots | Note 5 | Note 5 |  |  |  |  |  |
| Radio frame not containing SSB | slots | 10 | 10 |  |  |  |  |  |
| MCS index |  | 4 | 4 |  |  |  |  |  |
| Modulation |  | QPSK | QPSK |  |  |  |  |  |
| Target Coding Rate |  | 1/3 | 1/3 |  |  |  |  |  |
| Number of control symbols |  | 2 | 2 |  |  |  |  |  |
| PDSCH mapping type |  | Type A | Type A |  |  |  |  |  |
| Information Bit Payload |  |  |  |  |  |  |  |  |
| For slots with RMSI Note 2 | bits | 1608 | 1608 |  |  |  |  |  |
| For slots without RMSI | bits | 1864 | 1864 |  |  |  |  |  |
| Number of Code Blocks per slot |  | 1 | 1 |  |  |  |  |  |
| Binary Channel Bits Per slot |  |  |  |  |  |  |  |  |
| For slots with RMSI Note 2,4 | bits | 5184 | 5184 |  |  |  |  |  |
| For slots without RMSI Note 6 | bits | 6048 | 6048 |  |  |  |  |  |
| Note 1: Allocated outside the SMTC duration in time and in resource blocks which do not overlap with the resource blocks allocated for SS/PBCH block. Note 2: PDSCH is scheduled on the slots with RMSI. Note 3: If necessary the information bit payload size can be adjusted to facilitate the test implementation. The payload sizes are defined in TS 38.213 [3]. Note 4: Derived based on the PDSCH DMRS assumption: dmrs-TypeA-Position=2, dmrs-Type=1, dmrs-AdditonalPositions=2, maxLength=1, Antenna port index: 1000, and Number of PDSCH DMRS CDM group(s) without data: 2. Note 5: PDSCH is not scheduled in slots containing SSB according to the SSB configuration used in the test. SSB configurations are defined in clause A.3.10. Note 6: Derived based on the PDSCH DMRS assumption: dmrs-TypeA-Position=2, dmrs-Type=1, dmrs-AdditonalPositions=2, maxLength=1, Antenna port index: 1000, and Number of PDSCH DMRS CDM group(s) without data: 1. Note 7: When DRX is configured, PDCCH can be scheduled both for downlink assignment and/or UL grant only during (10 ms - drx-InactivityTimer) from timing when drx-onDurationTimer starts, unless otherwise specified in the test case. |  |  |  |  |  |  |  |  |

#### A.3.1.1.2	TDD

Table A.3.1.1.2-1: PDSCH Reference Measurement Channels for SCS=15kHz

| Parameter | Unit | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | SR.1.1 TDD | SR.1.2 TDD |  |  |  |  |  |
| Channel bandwidth | MHz | Defined in test case | Defined in test case |  |  |  |  |  |
| Number of transmitter antennas |  | 1 | 1 |  |  |  |  |  |
| Allocated resource blocks for PDSCH Note 1 |  | 24 | 24 |  |  |  |  |  |
| Allocated slots per Radio Frame |  |  |  |  |  |  |  |  |
| Radio frame containing SSB | slots | Note 5 | Note 5 |  |  |  |  |  |
| Radio frame not containing SSB | slots | 4 | 6 |  |  |  |  |  |
| MCS table |  | 64QAM | 64QAM |  |  |  |  |  |
| MCS index |  | 4 | 4 |  |  |  |  |  |
| Modulation |  | QPSK | QPSK |  |  |  |  |  |
| Target Coding Rate |  | 1/3 | 1/3 |  |  |  |  |  |
| Number of control symbols |  | 2 | 2 |  |  |  |  |  |
| PDSCH mapping type |  | Type A | Type A |  |  |  |  |  |
| Information Bit Payload |  |  |  |  |  |  |  |  |
| For slots with RMSI Note 2 | bits | 1608 | 1608 |  |  |  |  |  |
| For slots without RMSI | bits | 1864 | 1864 |  |  |  |  |  |
| For special slots | bits | N/A | 1128 |  |  |  |  |  |
| Number of Code Blocks per slot |  | 1 | 1 |  |  |  |  |  |
| Binary Channel Bits Per slot |  |  |  |  |  |  |  |  |
| For slots with RMSI Note 2,4 | bits | 5184 | 5184 |  |  |  |  |  |
| For slots without RMSI Note 6 | bits | 6048 | 6048 |  |  |  |  |  |
| For special slots Note 6 | bits | - | 3744 |  |  |  |  |  |
| Note 1: Allocated outside the SMTC duration in time and in resource blocks which do not overlap with the resource blocks allocated for SS/PBCH block. Note 2: PDSCH is scheduled on the slots with RMSI. Note 3: If necessary the information bit payload size can be adjusted to facilitate the test implementation. The payload sizes are defined in TS 38.213 [3]. Note 4: Derived based on the PDSCH DMRS assumption: dmrs-TypeA-Position=2, dmrs-Type=1, dmrs-AdditonalPositions=2, maxLength=1, Antenna port index: 1000, and Number of PDSCH DMRS CDM group(s) without data: 2. Note 5: PDSCH is not scheduled in slots containing SSB according to the SSB configuration used in the test. SSB configurations are defined in clause A.3.10. Note 6: Derived based on the PDSCH DMRS assumption: dmrs-TypeA-Position=2, dmrs-Type=1, dmrs-AdditonalPositions=2, maxLength=1, Antenna port index: 1000, and Number of PDSCH DMRS CDM group(s) without data: 1. Note 7: When DRX is configured, PDCCH can be scheduled both for downlink assignment and/or UL grant only during (10 ms - drx-InactivityTimer) from timing when drx-onDurationTimer starts, unless otherwise specified in the test case. |  |  |  |  |  |  |  |  |

Table A.3.1.1.2-2: PDSCH Reference Measurement Channels for SCS=30kHz

| Parameter | Unit | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | SR.2.1 TDD |  |  |  |  |  |  |
| Channel bandwidth | MHz | Defined in test case |  |  |  |  |  |  |
| Number of transmitter antennas |  | 1 |  |  |  |  |  |  |
| Allocated resource blocks for PDSCH Note 1 |  | 24 |  |  |  |  |  |  |
| Allocated slots per Radio Frame |  |  |  |  |  |  |  |  |
| Radio frame containing SSB | slots | Note 5 |  |  |  |  |  |  |
| Radio frame not containing SSB | slots | 10 |  |  |  |  |  |  |
| MCS table |  | 64QAM |  |  |  |  |  |  |
| MCS index |  | 4 |  |  |  |  |  |  |
| Modulation |  | QPSK |  |  |  |  |  |  |
| Target Coding Rate |  | 1/3 |  |  |  |  |  |  |
| Number of control symbols |  | 2 |  |  |  |  |  |  |
| PDSCH mapping type |  | Type A |  |  |  |  |  |  |
| Information Bit Payload |  |  |  |  |  |  |  |  |
| For slots with RMSI Note 2 | bits | 1608 |  |  |  |  |  |  |
| For slots without RMSI | bits | 1864 |  |  |  |  |  |  |
| Number of Code Blocks per slot |  | 1 |  |  |  |  |  |  |
| Binary Channel Bits Per slot |  |  |  |  |  |  |  |  |
| For slots with RMSI Note 2,4 | bits | 6048 |  |  |  |  |  |  |
| Note 1: Allocated outside the SMTC duration in time and in resource blocks which do not overlap with the resource blocks allocated for SS/PBCH block. Note 2: PDSCH is scheduled on the slots with RMSI. Note 3: If necessary the information bit payload size can be adjusted to facilitate the test implementation. The payload sizes are defined in TS 38.213 [3]. Note 4: Derived based on the PDSCH DMRS assumption: dmrs-TypeA-Position=2, dmrs-Type=1, dmrs-AdditonalPositions=2, maxLength=1, Antenna port index: 1000, and Number of PDSCH DMRS CDM group(s) without data: 2. Note 5: PDSCH is not scheduled in slots containing SSB according to the SSB configuration used in the test. SSB configurations are defined in clause A.3.10. Note 6: Derived based on the PDSCH DMRS assumption: dmrs-TypeA-Position=2, dmrs-Type=1, dmrs-AdditonalPositions=2, maxLength=1, Antenna port index: 1000, and Number of PDSCH DMRS CDM group(s) without data: 1. Note 7: When DRX is configured, PDCCH can be scheduled both for downlink assignment and/or UL grant only during (10 ms - drx-InactivityTimer) from timing when drx-onDurationTimer starts, unless otherwise specified in the test case. |  |  |  |  |  |  |  |  |

Table A.3.1.1.2-3: PDSCH Reference Measurement Channels for SCS=120kHz

| Parameter | Unit | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | SR.3.1 TDD | SR.3.2 TDD | SR.3.3 TDD |  |  |  |  |
| Channel bandwidth | MHz | 100 | 100 | 100 |  |  |  |  |
| Number of transmitter antennas |  | 1 | 1 | 1 |  |  |  |  |
| Allocated resource blocks for PDSCH |  | 24 Note 1 | 24Note 7 | 48Note 7 |  |  |  |  |
| Allocated slots per Radio Frame |  |  |  |  |  |  |  |  |
| Radio frame containing SSB | slots | Note 5 | Note 5 | Note 5 |  |  |  |  |
| Radio frame not containing SSB | slots | 48 | 48 | 48 |  |  |  |  |
| MCS table |  | 64QAM | 64QAM | 64QAM |  |  |  |  |
| MCS index |  | 4 | 4 | 4 |  |  |  |  |
| Modulation |  | QPSK | QPSK | QPSK |  |  |  |  |
| Target Coding Rate |  | 1/3 | 1/3 | 1/3 |  |  |  |  |
| Number of control symbols |  | 2 | 2 | 2 |  |  |  |  |
| PDSCH mapping type |  | Type A | Type A | Type A |  |  |  |  |
| Information Bit Payload |  |  |  |  |  |  |  |  |
| For slots with RMSI | bits | 1608 | 1608 | 3104 |  |  |  |  |
| For slots without RMSI | bits | 1864 | 1864 | 3624 |  |  |  |  |
| Number of Code Blocks per slot |  | 1 | 1 | 1 |  |  |  |  |
| Binary Channel Bits Per slot |  |  |  |  |  |  |  |  |
| For slots with RMSI Note 4 | bits | 5184 | 5184 | 10368 |  |  |  |  |
| For slots without RMSI Note 6 | bits | 6048 | 6048 | 12096 |  |  |  |  |
| Note 1: Allocated in resource blocks which do not overlap with the resource blocks allocated for SS/PBCH block Note 2: Void Note 3: If necessary the information bit payload size can be adjusted to facilitate the test implementation. The payload sizes are defined in TS 38.213 [3]. Note 4: Derived based on the PDSCH DMRS assumption: dmrs-TypeA-Position=2, dmrs-Type=1, dmrs-AdditonalPositions=2, maxLength=1, Antenna port index: 1000, and Number of PDSCH DMRS CDM group(s) without data: 2. Note 5: PDSCH is not scheduled in slots containing SSB according to the SSB configuration used in the test. SSB configurations are defined in clause A.3.10. Note 6: Derived based on the PDSCH DMRS assumption: dmrs-TypeA-Position=2, dmrs-Type=1, dmrs-AdditonalPositions=2, maxLength=1, Antenna port index: 1000, and Number of PDSCH DMRS CDM group(s) without data: 1. Note 7: Allocated in the same resource blocks as the CORESET. Note 8: When DRX is configured, PDSCH is scheduled only while drx-onDurationTimer is running, unless otherwise specified in the test case. |  |  |  |  |  |  |  |  |

### A.3.1.2	CORESET for RMSI scheduling

#### A.3.1.2.1	FDD

Table A.3.1.2.1-1: RMSI CORESET Reference Channel for FDD with SCS=15KHz

| Parameter | Unit | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | CR.1.1 FDD | CR.1.2 FDD | CR.1.3 FDD | CR.1.4 FDD |  |  |  |
| Channel bandwidth | MHz | Defined in test case | Defined in test case | Defined in test case | Defined in test case |  |  |  |
| Subcarrier spacing for RMSI CORESET | kHz | 15 | 15 | 15 | 15 |  |  |  |
| Allocated resource blocks for RMSI CORESET Note 7 |  | 24 | 12 | 15 | 20 |  |  |  |
| Subcarrier spacing for SSB | kHz | 15 | 15 | 15 | 15 |  |  |  |
| SSB and RMSI CORESET multiplexing configuration Note 7 |  | Pattern 1 | Pattern 1 | Pattern 1 | Pattern 1 |  |  |  |
| Offset between SSB and RMSI CORESET Note 3, 7 | RB | 0 (Note8) | 0 (Note8) | 0 (Note8) | 0 (Note8) |  |  |  |
| Configuration of PDCCH monitoring occasions for RMSI CORESET Note 4 |  | Index 4 | Index 4 | Index 4 | Index 4 |  |  |  |
| Number of transmitter antennas |  | 1 | 1 | 1 | 1 |  |  |  |
| Duration of RMSI CORESET Note 7 | symbols | 2 | 2 | 2 | 2 |  |  |  |
| DCI Format Note 1 |  | Note 2 | Note 2 | Note 2 | Note 2 |  |  |  |
| Aggregation level | CCE | 8 | 4 | 8 | 8 |  |  |  |
| DMRS precoder granularity |  | 6 | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size |  |  |  |
| REG bundle size |  | 6 | 6 | 6 | 6 |  |  |  |
| Mapping from REG to CCE |  | Distributed | Distributed | Non-Distributed | Distributed |  |  |  |
| Cell ID |  | Note 5 | Note 5 | Note 5 | Note 5 |  |  |  |
| Payload (without CRC) | bits | Note 6 | Note 6 | Note 6 | Note 6 |  |  |  |
| Note 1: DCI formats are defined in TS 38.212 [8]. Note 2: DCI format shall depend upon the test configuration. Note 3: The offset is defined with respect to the subcarrier spacing of the CORESET from the smallest RB index of RMSI CORESET to the smallest RB index of the common RB overlapping with the first RB of the SS/PBCH block. Note 4: The configuration of PDCCH monitoring occasions for RMSI CORESET is defined in Table 13-11 in TS 38.213 [3]. Note 5: Cell ID shall depend upon the test configuration. Note 6: Payload size shall depend upon the test configuration. Note 7: The configuration of set of resource blocks and slot symbols of control resource set for Type0-PDCCH search space corresponds to index 0 in Table 13-1 in TS 38.213 [3] Note 8: Other values can be used to align with GSCN [13] as long as SSB does not overlap the RMC. |  |  |  |  |  |  |  |  |

#### A.3.1.2.2	TDD

Table A.3.1.2.2-1: RMSI CORESET Reference Channel for TDD with SCS=15KHz

| Parameter | Unit | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | CR.1.1 TDD |  |  |  |  |  |  |
| Channel bandwidth | MHz | Defined in test case |  |  |  |  |  |  |
| Subcarrier spacing | kHz | 15 |  |  |  |  |  |  |
| Allocated resource blocks for RMSI CORESET Note 7 |  | 24 |  |  |  |  |  |  |
| SSB and RMSI CORESET multiplexing configuration Note 7 |  | Pattern 1 |  |  |  |  |  |  |
| Offset between SSB and RMSI CORESET Note 3, 7 | RB | 0 (Note 8) |  |  |  |  |  |  |
| Configuration of PDCCH monitoring occasions for RMSI CORESET Note 4 |  | Index 4 |  |  |  |  |  |  |
| Number of transmitter antennas |  | 1 |  |  |  |  |  |  |
| Duration of RMSI CORESET Note 7 | symbols | 2 |  |  |  |  |  |  |
| DCI Format Note 1 |  | Note 2 |  |  |  |  |  |  |
| Aggregation level | CCE | 8 |  |  |  |  |  |  |
| DMRS precoder granularity |  | 6 |  |  |  |  |  |  |
| REG bundle size |  | 6 |  |  |  |  |  |  |
| Mapping from REG to CCE |  | Distributed |  |  |  |  |  |  |
| Cell ID |  | Note 5 |  |  |  |  |  |  |
| Payload (without CRC) | bits | Note 6 |  |  |  |  |  |  |
| Note 1: DCI formats are defined in TS 38.212 [8]. Note 2: DCI format shall depend upon the test configuration. Note 3: The offset is defined with respect to the subcarrier spacing of the CORESET from the smallest RB index of RMSI CORESET to the smallest RB index of the common RB overlapping with the first RB of the SS/PBCH block. Note 4: The configuration of PDCCH monitoring occasions for RMSI CORESET is defined in Table 13-11 in TS 38.213 [3]. Note 5: Cell ID shall depend upon the test configuration. Note 6: Payload size shall depend upon the test configuration. Note 7: The configuration of set of resource blocks and slot symbols of control resource set for Type0-PDCCH search space corresponds to index 0 in Table 13-1 in TS 38.213 [3]. Note 8: Other values can be used to align with GSCN [13] as long as SSB does not overlap the RMC. |  |  |  |  |  |  |  |  |

Table A.3.1.2.2-2: RMSI CORESET Reference Channel for TDD with SCS=30KHz

| Parameter | Unit | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | CR.2.1 TDD |  |  |  |  |  |  |
| Channel bandwidth | MHz | Defined in test case |  |  |  |  |  |  |
| Subcarrier spacing | kHz | 30 |  |  |  |  |  |  |
| Allocated resource blocks for RMSI CORESET Note 7 |  | 24 |  |  |  |  |  |  |
| SSB and RMSI CORESET multiplexing configuration Note 7 |  | Pattern 1 |  |  |  |  |  |  |
| Offset between SSB and RMSI CORESET Note 3, 7 | RB | 0 (Note 8) |  |  |  |  |  |  |
| Configuration of PDCCH monitoring occasions for RMSI CORESET Note 4 |  | Index 4 |  |  |  |  |  |  |
| Number of transmitter antennas |  | 1 |  |  |  |  |  |  |
| Duration of RMSI CORESET Note 7 | symbols | 2 |  |  |  |  |  |  |
| DCI Format Note 1 |  | Note 2 |  |  |  |  |  |  |
| Aggregation level | CCE | 8 |  |  |  |  |  |  |
| DMRS precoder granularity |  | 6 |  |  |  |  |  |  |
| REG bundle size |  | 6 |  |  |  |  |  |  |
| Mapping from REG to CCE |  | Distributed |  |  |  |  |  |  |
| Cell ID |  | Note 5 |  |  |  |  |  |  |
| Payload (without CRC) | bits | Note 6 |  |  |  |  |  |  |
| Note 1: DCI formats are defined in TS 38.212 [8]. Note 2: DCI format shall depend upon the test configuration. Note 3: The offset is defined with respect to the subcarrier spacing of the CORESET from the smallest RB index of RMSI CORESET to the smallest RB index of the common RB overlapping with the first RB of the SS/PBCH block. Note 4: The configuration of PDCCH monitoring occasions for RMSI CORESET is defined in Table 13-11 in TS 38.213 [3]. Note 5: Cell ID shall depend upon the test configuration. Note 6: Payload size shall depend upon the test configuration. Note 7: The configuration of set of resource blocks and slot symbols of control resource set for Type0-PDCCH search space corresponds to index 0 in Table 13-6 in TS 38.213 [3]. Note 8: Other values can be used to align with GSCN [13] as long as SSB does not overlap the RMC. |  |  |  |  |  |  |  |  |

Table A.3.1.2.2-3: RMSI CORESET Reference Channel for TDD with SCS=120KHz

| Parameter | Unit | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | CR.3.1 TDD | CR.3.2 TDD |  |  |  |  |  |
| Channel bandwidth | MHz | 100 | 100 |  |  |  |  |  |
| Subcarrier spacing | kHz | 120 | 120 |  |  |  |  |  |
| Allocated resource blocks for RMSI CORESET |  | 24 Note 7 | 48 Note 9 |  |  |  |  |  |
| SSB and RMSI CORESET multiplexing configuration |  | Pattern 1 Note 7 | Pattern 1 Note 9 |  |  |  |  |  |
| Offset between SSB and RMSI CORESET Note 3 | RB | 0 (Note 8) Note 7 | 0 (Note 8) Note 9 |  |  |  |  |  |
| Configuration of PDCCH monitoring occasions for RMSI CORESET Note 4 |  | Index 4 | Index 4 |  |  |  |  |  |
| Number of transmitter antennas |  | 1 | 1 |  |  |  |  |  |
| Duration of RMSI CORESET | symbols | 2 Note 7 | 2 Note 9 |  |  |  |  |  |
| DCI Format Note 1 |  | Note 2 | Note 2 |  |  |  |  |  |
| Aggregation level | CCE | 8 | 8 |  |  |  |  |  |
| DMRS precoder granularity |  | 6 | 6 |  |  |  |  |  |
| REG bundle size |  | 6 | 6 |  |  |  |  |  |
| Mapping from REG to CCE |  | Distributed | Distributed |  |  |  |  |  |
| Cell ID |  | Note 5 | Note 5 |  |  |  |  |  |
| Payload (without CRC) | bits | Note 6 | Note 6 |  |  |  |  |  |
| Note 1: DCI formats are defined in TS 38.212 [8]. Note 2: DCI format shall depend upon the test configuration. Note 3: The offset is defined with respect to the subcarrier spacing of the CORESET from the smallest RB index of RMSI CORESET to the smallest RB index of the common RB overlapping with the first RB of the SS/PBCH block. Note 4: The configuration of PDCCH monitoring occasions for RMSI CORESET is defined in Table 13-12 in TS 38.213 [3]. Note 5: Cell ID shall depend upon the test configuration. Note 6: Payload size shall depend upon the test configuration. Note 7: The configuration of set of resource blocks and slot symbols of control resource set for Type0-PDCCH search space corresponds to index 0 in Table 13-8 in TS 38.213 [3]. Note 8: Other values can be used to align with GSCN [13] as long as SSB does not overlap the RMC. Note 9: The configuration of set of resource blocks and slot symbols of control resource set for Type0-PDCCH search space corresponds to index 2 in Table 13-10 in TS 38.213 [3]. |  |  |  |  |  |  |  |  |

### A.3.1.3	CORESET for RMC scheduling

#### A.3.1.3.1	FDD

Table A.3.1.3.1-1: Control Channel RMC for FDD with SCS=15KHz

| Parameter | Unit | Value |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | CCR.1.1 FDD | CCR.1.2 FDD | CCR.1.3 FDD | CCR.1.4 FDD | CCR.1.5 FDD | CCR.1.6 FDD | CCR.1.7 FDD | CCR.1.8 FDD |  |  |
| Channel bandwidth | MHz | Defined in test case | Defined in test case | Defined in test case | Defined in test case | 10 | 3MHz | 3MHz | 5MHz |  |  |
| Subcarrier spacing | kHz | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 |  |  |
| Allocated resource blocks for CORESET Note 3 |  | 24 | 18 | 24 | 18 | 24 | 12 | 12 | 18 |  |  |
| Number of transmitter antennas |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |  |  |
| Duration of CORESET | symbols | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |  |  |
| monitoringSymbolsWithinSlot |  | 1000000 0000000 | 1000000 0000000 | 1000000 0000000 | 1000000 0000000 | 0010000 0000000 | 1000000 0000000 | 1000000 0000000 | 1000000 0000000 |  |  |
| REG bundle size |  | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 |  |  |
| DMRS precoder granularity |  | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size |  |  |
| CCE to REG mapping |  | Interleaved | Interleaved | Interleaved | Interleaved | Interleaved | Interleaved | Non-Interleaved | Interleaved |  |  |
| Interleave n_shift |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  |  |
| Interleave size |  | 2 | 2 | 2 | 2 | 2 | 2 | N/A | 2 |  |  |
| Beamforming Pre-Coder |  | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |  |  |
| Aggregation level | CCE | 4 | 2 | 8 | 4 | 4 | 4 | 8 | 8 |  |  |
| DCI formats |  | Note 1 | Note 1 | Note 1 | Note 1 | Note 1 | 1-0 | 1-0 | 1-0 |  |  |
| Payload size (without CRC) | bits | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 |  |  |
| Note 1: DCI format shall depend upon the test configuration. Note 2: Payload size shall depend upon the test configuration. Note 3: Allocated in the resource blocks where the associated RMC is scheduled. |  |  |  |  |  |  |  |  |  |  |  |

Table A.3.1.3.1-2: Control Channel RMC for FDD with SCS=120KHz

| Parameter | Unit | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | CCR.2.1 FDD |  |  |  |  |  |  |
| Channel bandwidth | MHz | 100 |  |  |  |  |  |  |
| Subcarrier spacing | kHz | 120 |  |  |  |  |  |  |
| Allocated resource blocks for CORESET Note 3 |  | 24 |  |  |  |  |  |  |
| Number of transmitter antennas |  | 1 |  |  |  |  |  |  |
| Duration of CORESET | symbols | 1 |  |  |  |  |  |  |
| monitoringSlotPeriodicityAndOffset Note 4 |  | sl160 0 |  |  |  |  |  |  |
| monitoringSymbolsWithinSlot |  | 1100000 0000000 |  |  |  |  |  |  |
| REG bundle size |  | 6 |  |  |  |  |  |  |
| DMRS precoder granularity |  | Same as REG bundle size |  |  |  |  |  |  |
| CCE to REG mapping |  | Interleaved |  |  |  |  |  |  |
| Interleave n_shift |  | 0 |  |  |  |  |  |  |
| Interleave size |  | 2 |  |  |  |  |  |  |
| Beamforming Pre-Coder |  | N/A |  |  |  |  |  |  |
| Aggregation level | CCE | 4 |  |  |  |  |  |  |
| DCI formats |  | Note 1 |  |  |  |  |  |  |
| Payload size (without CRC) | bits | Note 2 |  |  |  |  |  |  |
| Note 1: DCI format shall depend upon the test configuration. Note 2: Payload size shall depend upon the test configuration. Note 3: Allocated in the resource blocks where the associated RMC is scheduled. Note 4: monitoringSlotPeriodicityAndOffet is set to “sl1 0” if it is specifically stated that cell(s) configured with one of the control channel RMCs above shall transmit PDCCHs continuously. |  |  |  |  |  |  |  |  |

#### A.3.1.3.2	TDD

Table A.3.1.3.2-1: Control Channel RMC for TDD with SCS=15KHz

| Parameter | Unit | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | CCR.1.1 TDD | CCR.1.2 TDD | CCR.1.3 TDD | CCR.1.4 TDD | CCR.1.5 TDD |  |  |
| Channel bandwidth | MHz | Defined in test case | Defined in test case | Defined in test case | Defined in test case | 10 |  |  |
| Subcarrier spacing | kHz | 15 | 15 | 15 | 15 | 15 |  |  |
| Allocated resource blocks for CORESET Note 3 |  | 24 | 18 | 24 | 18 | 18 |  |  |
| Number of transmitter antennas |  | 1 | 1 | 1 | 1 | 1 |  |  |
| Duration of CORESET | symbols | 2 | 2 | 2 | 2 | 2 |  |  |
| monitoringSymbolsWithinSlot |  | 1000000 0000000 | 1000000 0000000 | 1000000 0000000 | 1000000 0000000 | 0010000 0000000 |  |  |
| REG bundle size |  | 6 | 6 | 6 | 6 | 6 |  |  |
| DMRS precoder granularity |  | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size |  |  |
| CCE to REG mapping |  | Interleaved | Interleaved | Interleaved | Interleaved | Interleaved |  |  |
| Interleave n_shift |  | 0 | 0 | 0 | 0 | 0 |  |  |
| Interleave size |  | 2 | 2 | 2 | 2 | 2 |  |  |
| Beamforming Pre-Coder |  | N/A | N/A | N/A | N/A | N/A |  |  |
| Aggregation level | CCE | 4 | 2 | 8 | 4 | 4 |  |  |
| DCI formats |  | Note 1 | Note 1 | Note 1 | Note 1 | Note 1 |  |  |
| Payload size (without CRC) | bits | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 |  |  |
| Note 1: DCI format shall depend upon the test configuration. Note 2: Payload size shall depend upon the test configuration. Note 3: Allocated in the resource blocks where the associated RMC is scheduled. |  |  |  |  |  |  |  |  |

Table A.3.1.3.2-2: Control Channel RMC for TDD with SCS=30KHz

| Parameter | Unit | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | CCR.2.1 TDD | CCR.2.2 TDD | CCR.2.3 TDD | CCR.2.4 TDD |  |  |  |
| Channel bandwidth | MHz | Defined in test case | Defined in test case | Defined in test case | Defined in test case |  |  |  |
| Subcarrier spacing | kHz | 30 | 30 | 30 | 30 |  |  |  |
| Allocated resource blocks for CORESET Note 3 |  | 24 | 24 | 18 | 18 |  |  |  |
| Number of transmitter antennas |  | 1 | 1 | 1 | 1 |  |  |  |
| Duration of CORESET | symbols | 2 | 2 | 2 | 2 |  |  |  |
| REG bundle size |  | 6 | 6 | 6 | 6 |  |  |  |
| DMRS precoder granularity |  | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size |  |  |  |
| CCE to REG mapping |  | Interleaved | Interleaved | Interleaved | Interleaved |  |  |  |
| Interleave n_shift |  | 0 | 0 | 0 | 0 |  |  |  |
| Interleave size |  | 2 | 2 | 2 | 2 |  |  |  |
| Beamforming Pre-Coder |  | N/A | N/A | N/A | N/A |  |  |  |
| Aggregation level | CCE | 4 | 8 | 4 | 2 |  |  |  |
| DCI formats |  | Note 1 | Note 1 | Note 1 | Note 1 |  |  |  |
| Payload size (without CRC) | bits | Note 2 | Note 2 | Note 2 | Note 2 |  |  |  |
| Note 1: DCI format shall depend upon the test configuration. Note 2: Payload size shall depend upon the test configuration. Note 3: Allocated in the same resource blocks where the associated RMC is scheduled. |  |  |  |  |  |  |  |  |

Table A.3.1.3.2-3: Control Channel RMC for TDD with SCS=120KHz

| Parameter | Unit | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | CCR.3.1 TDD | CCR.3.2 TDD | CCR.3.3 TDD | CCR.3.4 TDD | CCR.3.5 TDD | CCR.3.6 TDD | CCR.3.7 TDD |
| Channel bandwidth | MHz | 100 | 100 | 100 | 100 | 100 | 100 | 100 |
| Subcarrier spacing | kHz | 120 | 120 | 120 | 120 | 120 | 120 | 120 |
| Allocated resource blocks for CORESET Note 3 |  | 24 | 24 | 24 | 24 | 24 | 24 | 48 |
| Number of transmitter antennas |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| monitoringSlotPeriodicityAndOffset Note 4 |  | sl160 0 | sl160 0 | sl160 80 | sl160 0 | sl160 0 | sl160 80 | sl160 0 |
| monitoringSymbolsWithinSlot |  | 1100000 0000000 | 0011000 0000000 | 1100000 0000000 | 1000000 0000000 | 0010000 0000000 | 1000000 0000000 | 1100000 0000000 |
| Duration of CORESET | slot | 1 | 1 | 1 | 2 | 2 | 2 | 1 |
| REG bundle size |  | 6 | 6 | 6 | 6 | 6 | 6 | 6 |
| DMRS precoder granularity |  | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size |
| CCE to REG mapping |  | Interleaved | Interleaved | Interleaved | Interleaved | Interleaved | Interleaved | Interleaved |
| Interleave n_shift |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Interleave size |  | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| Beamforming Pre-Coder |  | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| Aggregation level | CCE | 4 | 4 | 4 | 8 | 8 | 8 | 4 |
| DCI formats |  | Note 1 | Note 1 | Note 1 | Note 1 | Note 1 | Note 1 | Note 1 |
| Payload size (without CRC) | bits | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 |
| Note 1: DCI format shall depend upon the test configuration. Note 2: Payload size shall depend upon the test configuration. Note 3: Allocated in the same resource blocks where the associated PDSCH RMC is scheduled. Note 4: monitoringSlotPeriodicityAndOffet is set to “sl1 0” if it is specifically stated that cell(s) configured with one of the control channel RMCs above shall transmit PDCCHs continuously. |  |  |  |  |  |  |  |  |

### A.3.1.4	TDD UL/DL configuration

Table A.3.1.4-1: TDD UL/DL configuration for SCS=15kHz

| Parameter | Unit | Value |  |  |
| --- | --- | --- | --- | --- |
| Reference channel |  | TDDConf.1.1 | TDDConf.1.2 |  |
| referenceSubcarrierSpacing | kHz | 15 | 15 |  |
| TDD UL/DL pattern 1 Note 2 |  | ‘DSUU’ S=’10DL:2GP:2UL’ | ‘DSUU’ S=’ 6DL: 2GP: 6UL’ |  |
| dl-UL-TransmissionPeriodicity | ms | 4 | 4 |  |
| nrofDownlinkSlots |  | 1 | 1 |  |
| nrofDownlinkSymbols |  | 10 | 6 |  |
| nrofUplinkSlot |  | 2 | 2 |  |
| nrofUplinkSymbols |  | 2 | 6 |  |
| TDD UL/DL pattern 2 Note 2 |  | ‘D’ | ‘D’ |  |
| dl-UL-TransmissionPeriodicity | ms | 1 | 1 |  |
| nrofDownlinkSlots |  | 1 | 1 |  |
| nrofDownlinkSymbols |  | 0 | 0 |  |
| nrofUplinkSlot |  | 0 | 0 |  |
| nrofUplinkSymbols |  | 0 | 0 |  |
| Note 1: As specified in TS 38.213 [3] and TS 38.331 [2]. Note 2: For information |  |  |  |  |

Table A.3.1.4-2: TDD UL/DL configuration for SCS=30kHz

| Parameter | Unit | Value |  |  |
| --- | --- | --- | --- | --- |
| Reference channel |  | TDDConf.2.1 | TDDConf.2.2 | TDDConf.2.3 |
| referenceSubcarrierSpacing | kHz | 30 | 30 | 30 |
| TDD UL/DL pattern 1 Note 2 |  | ‘3D1S4U’ S=’6DL:4GP:4UL’ | ‘1D1S2U’ S=’11DL: 1GP:2UL’ | ‘3D1S4U’ S=’4DL:4GP:6UL’ |
| dl-UL-TransmissionPeriodicity | ms | 4 | 2 | 4 |
| nrofDownlinkSlots |  | 3 | 1 | 3 |
| nrofDownlinkSymbols |  | 6 | 11 | 4 |
| nrofUplinkSlot |  | 4 | 2 | 4 |
| nrofUplinkSymbols |  | 4 | 2 | 6 |
| TDD UL/DL pattern 2 Note 2 |  | ‘DD’ | Not configured | ‘DD’ |
| dl-UL-TransmissionPeriodicity | ms | 1 | Not configured | 1 |
| nrofDownlinkSlots |  | 2 | Not configured | 2 |
| nrofDownlinkSymbols |  | 0 | Not configured | 0 |
| nrofUplinkSlot |  | 0 | Not configured | 0 |
| nrofUplinkSymbols |  | 0 | Not configured | 0 |
| Note 1: As specified in TS 38.213 [3] and TS 38.331 [2]. Note 2: For information |  |  |  |  |

Table A.3.1.4-3: TDD UL/DL configuration for SCS=120kHz

| Parameter | Unit | Value |  |  |
| --- | --- | --- | --- | --- |
| Reference channel |  | TDDConf.3.1 | TDDConf.3.2 |  |
| referenceSubcarrierSpacing | kHz | 120 | 120 |  |
| TDD UL/DL pattern 1 Note 2 |  | ‘DDDSU’ S=’10DL:2GP:2UL’ | ‘DDDSU’ S=’ 6DL: 2GP: 6UL’ |  |
| dl-UL-TransmissionPeriodicity | ms | 0.625 | 0.625 |  |
| nrofDownlinkSlots |  | 3 | 3 |  |
| nrofDownlinkSymbols |  | 10 | 6 |  |
| nrofUplinkSlot |  | 1 | 1 |  |
| nrofUplinkSymbols |  | 2 | 6 |  |
| TDD UL/DL pattern 2 Note 2 |  | Not configured | Not configured |  |
| dl-UL-TransmissionPeriodicity | ms | Not configured | Not configured |  |
| nrofDownlinkSlots |  | Not configured | Not configured |  |
| nrofDownlinkSymbols |  | Not configured | Not configured |  |
| nrofUplinkSlot |  | Not configured | Not configured |  |
| nrofUplinkSymbols |  | Not configured | Not configured |  |
| Note 1: As specified in TS 38.213 [3] and TS 38.331 [2]. Note 2: For information |  |  |  |  |

## A.3.1A	Reference measurement channels under CCA

### A.3.1A.1	PDSCH

#### A.3.1A.1.1	TDD

Table A.3.1A.1.1-1: PDSCH Reference Measurement Channels for SCS=30kHz

| Parameter | Unit | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | SR.1.1 CCA |  |  |  |  |  |  |
| Channel bandwidth | MHz | 40 |  |  |  |  |  |  |
| Number of transmitter antennas |  | 1 |  |  |  |  |  |  |
| Allocated resource blocks for PDSCH Note 1 |  | 24 |  |  |  |  |  |  |
| Allocated slots per Radio Frame |  |  |  |  |  |  |  |  |
| Radio frame containing SSB | slots | Note 5 |  |  |  |  |  |  |
| Radio frame not containing SSB | slots | Note 7 |  |  |  |  |  |  |
| MCS table |  | 64QAM |  |  |  |  |  |  |
| MCS index |  | 4 |  |  |  |  |  |  |
| Modulation |  | QPSK |  |  |  |  |  |  |
| Target Coding Rate |  | 1/3 |  |  |  |  |  |  |
| Number of control symbols |  | 2 |  |  |  |  |  |  |
| PDSCH mapping type |  | Type A |  |  |  |  |  |  |
| Information Bit Payload |  |  |  |  |  |  |  |  |
| For slots with RMSI Note 2 | bits | 1608 |  |  |  |  |  |  |
| For slots without RMSI | bits | 1864 |  |  |  |  |  |  |
| Number of Code Blocks per slot |  | 1 |  |  |  |  |  |  |
| Binary Channel Bits Per slot |  |  |  |  |  |  |  |  |
| For slots with RMSI Note 2, Note 4 | bits | 5184 |  |  |  |  |  |  |
| For slots without RMSI Note 6 | bits | 6048 |  |  |  |  |  |  |
| Note 1: Allocated outside the discovery burst transmission window in time and in resource blocks which do not overlap with the resource blocks allocated for SS/PBCH block. Note 2: PDSCH is scheduled on the slots with RMSI. Note 3: If necessary the information bit payload size can be adjusted to facilitate the test implementation. The payload sizes are defined in TS 38.213 [3]. Note 4: Derived based on the PDSCH DMRS assumption: dmrs-TypeA-Position=2, dmrs-Type=1, dmrs-AdditonalPositions=2, maxLength=1, Antenna port index: 1000, and Number of PDSCH DMRS CDM group(s) without data: 2. Note 5: PDSCH is not scheduled in slots containing SSB according to the SSB configuration used in the test. SSB configurations are defined in clause A.3.10A. Note 6: Derived based on the PDSCH DMRS assumption: dmrs-TypeA-Position=2, dmrs-Type=1, dmrs-AdditonalPositions=2, maxLength=1, Antenna port index: 1000, and Number of PDSCH DMRS CDM group(s) without data: 1. Note 7: PDSCH is transmitted during the RMC burst as specified in A.3.1A.5. |  |  |  |  |  |  |  |  |

### A.3.1A.2	CORESET for RMSI scheduling

#### A.3.1A.2.1	TDD

Table A.3.1A.2.1-1: RMSI CORESET Reference Channel for SCS=30KHz

| Parameter | Unit | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | CR.1.1 CCA |  |  |  |  |  |  |
| Channel bandwidth | MHz | 40 |  |  |  |  |  |  |
| Subcarrier spacing | kHz | 30 |  |  |  |  |  |  |
| Allocated resource blocks for RMSI CORESET Note 7 |  | 48 |  |  |  |  |  |  |
| SSB and RMSI CORESET multiplexing configuration Note 7 |  | Pattern 1 |  |  |  |  |  |  |
| Offset between SSB and RMSI CORESET Note 3, 7 | RB | 0 (Note 8) |  |  |  |  |  |  |
| Configuration of PDCCH monitoring occasions for RMSI CORESET Note 4 |  | Index 0 |  |  |  |  |  |  |
| Number of transmitter antennas |  | 1 |  |  |  |  |  |  |
| Duration of RMSI CORESET Note 7 | symbols | 2 |  |  |  |  |  |  |
| DCI Format Note 1 |  | Note 2 |  |  |  |  |  |  |
| Aggregation level | CCE | 8 |  |  |  |  |  |  |
| DMRS precoder granularity |  | 6 |  |  |  |  |  |  |
| REG bundle size |  | 6 |  |  |  |  |  |  |
| Mapping from REG to CCE |  | Distributed |  |  |  |  |  |  |
| Cell ID |  | Note 5 |  |  |  |  |  |  |
| Payload (without CRC) | bits | Note 6 |  |  |  |  |  |  |
| Note 1: DCI formats are defined in TS 38.212 [8]. Note 2: DCI format shall depend upon the test configuration. Note 3: The offset is defined with respect to the subcarrier spacing of the CORESET from the smallest RB index of RMSI CORESET to the smallest RB index of the common RB overlapping with the first RB of the SS/PBCH block. Note 4: The configuration of PDCCH monitoring occasions for RMSI CORESET is defined in Table 13-11 in TS 38.213 [3]. Note 5: Cell ID shall depend upon the test configuration. Note 6: Payload size shall depend upon the test configuration. Note 7: The configuration of set of resource blocks and slot symbols of control resource set for Type0-PDCCH search space corresponds to index 4 in Table 13-4A in TS 38.213 [3]. Note 8: Other values can be used to align with GSCN [13] as long as SSB does not overlap the RMC. |  |  |  |  |  |  |  |  |

### A.3.1A.3	CORESET for RMC scheduling

#### A.3.1A.3.1	TDD

Table A.3.1A.3.1-1: Control Channel RMC with SCS=30KHz

| Parameter | Unit | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | CCR.1.1 CCA | CCR.1.2 CCA | CCR.1.3 CCA |  |  |  |  |
| Channel bandwidth | MHz | 40 | 40 | 40 |  |  |  |  |
| Subcarrier spacing | kHz | 30 | 30 | 30 |  |  |  |  |
| Allocated resource blocks for CORESET Note 3 |  | 24 | 24 | 18 |  |  |  |  |
| Number of transmitter antennas |  | 1 | 1 | 1 |  |  |  |  |
| Duration of CORESET | symbols | 2 | 2 | 2 |  |  |  |  |
| REG bundle size |  | 6 | 6 | 6 |  |  |  |  |
| DMRS precoder granularity |  | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size |  |  |  |  |
| CCE to REG mapping |  | Interleaved | Interleaved | Interleaved |  |  |  |  |
| Interleave n_shift |  | 0 | 0 | 0 |  |  |  |  |
| Interleave size |  | 2 | 2 | 2 |  |  |  |  |
| Beamforming Pre-Coder |  | N/A | N/A | N/A |  |  |  |  |
| Aggregation level | CCE | 4 | 8 | 4 |  |  |  |  |
| DCI formats |  | Note 1 | Note 1 | Note 1 |  |  |  |  |
| Payload size (without CRC) | bits | Note 2 | Note 2 | Note 2 |  |  |  |  |
| Note 1: DCI format shall depend upon the test configuration. Note 2: Payload size shall depend upon the test configuration. Note 3: Allocated in the same resource blocks where the associated RMC is scheduled. |  |  |  |  |  |  |  |  |

### A.3.1A.4	TDD UL/DL configuration

Table A.3.1A.4-1: TDD UL/DL configuration for SCS=30kHz

| Parameter | Unit | Value |  |  |
| --- | --- | --- | --- | --- |
| Reference channel |  | TDDConf.1.1 CCA |  |  |
| referenceSubcarrierSpacing | kHz | N/A |  |  |
| TDD UL/DL pattern 1 Note 2, Note 3 |  | ‘3D1S4U’ S=’6DL:4GP:4UL’ |  |  |
| dl-UL-TransmissionPeriodicity | ms | N/A |  |  |
| nrofDownlinkSlots |  | N/A |  |  |
| nrofDownlinkSymbols |  | N/A |  |  |
| nrofUplinkSlot |  | N/A |  |  |
| nrofUplinkSymbols |  | N/A |  |  |
| TDD UL/DL pattern 2 Note 2, Note 3 |  | ‘DD’ |  |  |
| dl-UL-TransmissionPeriodicity | ms | N/A |  |  |
| nrofDownlinkSlots |  | N/A |  |  |
| nrofDownlinkSymbols |  | N/A |  |  |
| nrofUplinkSlot |  | N/A |  |  |
| nrofUplinkSymbols |  | N/A |  |  |
| Note 1: As specified in TS 38.213 [3] and TS 38.331 [2]. Note 2: Do not configure tdd-UL-DL-ConfigurationCommon using RRC configuration Note 3: The UE will be scheduled via DCI according to the TDD pattern defined in the table. |  |  |  |  |

### A.3.1A.5	RMC burst transmission model

RMC not conveying RMSI is scheduled during the RMC burst. The length of the transmission burst in slots is defined as N. The burst transmission format is determined according to the steps below:

1.	Select N randomly from a given set of the number of slots S1 = {1,3,5} with equal probability as the total length of RMC burst transmission format.

2.	A uniform random variable from 0 to 1 is generated. If the random variable is less than PCCA_DL, a burst of N fully occupied slots is transmitted. Otherwise, the RMC burst transmission is muted and the muting duration is the same as the number N of slots for determined burst format.

RMC burst transmission is scheduled outside discovery burst transmission window. If transmission occurred in the previous slot, transmission is muted for a duration of one slot. Additionally, if the start time of the candidate RMC burst transmission is within 5 slots of the start of the discovery burst transmission window, RMC transmission is not performed.A.3.2	OFDMA channel noise generator (OCNG).

### A.3.2.1	Generic OFDMA Channel Noise Generator (OCNG)

The OCNG pattern is used in a test for modelling allocations of unused resources in the channel bandwidth to virtual UEs (which are not under test). The OCNG pattern comprises PDCCH and PDSCH transmissions to the virtual UEs. For the cells operating under CCA, OCNG is transmitted only in slots with RMC burst transmission and is not transmitted during the slots for which CCA attempt is unsuccessful or during DBT windows.

#### A.3.2.1.1	OCNG pattern 1: Generic OCNG pattern for all unused REs

Table A.3.2.1.1-1: OP.1: Generic OCNG pattern for all unused REs

| OCNG Parameters | Control Region | Data Region |
| --- | --- | --- |
| Resource allocation | Unused REs (Note 1) | Unused REs (Note 2) |
| Channel | PDCCH | PDSCH |
| Contents | Virtual UE IDs | Uncorrelated pseudo random QPSK modulated data |
| Antenna transmission scheme | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| Subcarrier spacing | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| Aggregation level | Same as used in PDCCH RMC | N/A |
| Code rate | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| Transmit Power | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| CP length | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| Note 1: REs not used in the active CORESETs where PDCCH is scheduled for the UE under test. Note 2: REs not allocated to any physical channels, CORESET, SSB or any other reference signal within the channel bandwidth of the cell, confined to BWoccupied where specified in the test case. |  |  |

#### A.3.2.1.2	OCNG pattern 2: Generic OCNG pattern for all unused REs for 2AoA setup

Table A.3.2.1.2-2: OP.2: Generic OCNG pattern for all unused REs for 2AoA setup

| OCNG Parameters | Control Region | Data Region |
| --- | --- | --- |
| Probe | Transmitting the serving beam |  |
| Resource allocation | Unused REs (Note 1) in the symbols where SSB/CSI-RS are not transmitted from both the serving beam probe and non-serving beam probe. | Unused REs (Note 2) in the symbols where SSB/CSI-RS are not transmitted from both the serving beam probe and non-serving beam probe. |
| Channel | PDCCH | PDSCH |
| Contents | Virtual UE IDs | Uncorrelated pseudo random QPSK modulated data |
| Antenna transmission scheme | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| Subcarrier spacing | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| Aggregation level | Same as used in PDCCH RMC | N/A |
| Code rate | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| Transmit Power | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| CP length | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| Note 1: REs not used in the active CORESETs where PDCCH is scheduled for the UE under test. Note 2: REs not allocated to any physical channels, CORESET, SSB or any other reference signal within the channel bandwidth of the cell, confined to BWoccupied where specified in the test case. Note 3: No OCNG is transmitted from the probe transmitting non-serving beam. |  |  |

#### A.3.2.1.3	OCNG pattern 3: Generic OCNG pattern for unused REs in the same bandwidth as CORESET

Table A.3.2.1.3-1: OP.3: Generic OCNG pattern for unused REs in the same BW as CORESET

| OCNG Parameters | Control Region | Data Region |
| --- | --- | --- |
| Resource allocation | Unused REs (Note 1) | Unused REs (Note 2) |
| Channel | PDCCH | PDSCH |
| Contents | Virtual UE IDs | Uncorrelated pseudo random QPSK modulated data |
| Antenna transmission scheme | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| Subcarrier spacing | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| Aggregation level | Same as used in PDCCH RMC | N/A |
| Code rate | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| Transmit Power | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| CP length | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| Note 1: REs not used in the active CORESETs where PDCCH is scheduled for the UE under test. REs for OCNG shall not be allocated outside the allocated bandwidth of the CORESET of the serving cell. Note 2: REs not allocated to any physical channels, CORESET, SSB or any other reference signal within the allocated bandwidth of the CORESET of the serving cell. REs for OCNG shall not be allocated outside the allocated bandwidth of the CORESET of the serving cell. |  |  |

#### A.3.2.1.4	OCNG pattern 4: Generic OCNG pattern for all unused REs outside SSB slot(s)

Table A.3.2.1.4-1: OP.4: Generic OCNG pattern for all unused REs outside SSB slot(s)

| OCNG Parameters | Control Region | Data Region |
| --- | --- | --- |
| Resource allocation | Unused REs (Note 1) | Unused REs (Note 2) |
| Channel | PDCCH | PDSCH |
| Contents | Virtual UE IDs | Uncorrelated pseudo random QPSK modulated data |
| Antenna transmission scheme | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| Subcarrier spacing | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| Aggregation level | Same as used in PDCCH RMC | N/A |
| Code rate | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| Transmit Power | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| CP length | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| Note 1: REs not used in the active CORESETs where PDCCH is scheduled for the UE under test. REs for OCNG shall not be allocated in the slot(s) containing SSB of the respective cell. Note 2: REs not allocated to any physical channels, CORESET, SSB or any other reference signal within the channel bandwidth of the cell. REs for OCNG shall not be allocated in the slot(s) containing SSB of the respective cell. |  |  |

#### A.3.2.1.5	OCNG pattern 5: Generic OCNG pattern for unused REs in the same bandwidth as CORESET for 2AoA setup

Table A.3.2.1.5-1: OP.5: Generic OCNG pattern for unused REs in the same BW as CORESET for 2AoA setup

| OCNG Parameters | Control Region | Data Region |
| --- | --- | --- |
| Probe | Transmitting the serving beam |  |
| Resource allocation | Unused REs (Note 1) in the symbols where SSB/CSI-RS are not transmitted from both the serving beam probe and non-serving beam probe. | Unused REs (Note 2) in the symbols where SSB/CSI-RS are not transmitted from both the serving beam probe and non-serving beam probe. |
| Channel | PDCCH | PDSCH |
| Contents | Virtual UE IDs | Uncorrelated pseudo random QPSK modulated data |
| Antenna transmission scheme | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| Subcarrier spacing | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| Aggregation level | Same as used in PDCCH RMC | N/A |
| Code rate | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| Transmit Power | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| CP length | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| Note 1: REs not used in the active CORESETs where PDCCH is scheduled for the UE under test. REs for OCNG shall not be allocated outside the allocated bandwidth of the CORESET of the serving cell. Note 2: REs not allocated to any physical channels, CORESET, SSB or any other reference signal within the allocated bandwidth of the CORESET of the serving cell. REs for OCNG shall not be allocated outside the allocated bandwidth of the CORESET of the serving cell. Note 3: No OCNG is transmitted from the probe transmitting non-serving beam. |  |  |

### A.3.2.2	Void

## A.3.3	Reference DRX configurations

### A.3.3.1	DRX Configuration 1: DRX cycle = 40 ms and TAT = 500 ms

Table A.3.3.1-1: DRX.1: DRX cycle = 40 ms and time alignment timer (TAT) = 500 ms

| Field | Value |
| --- | --- |
| drx-onDurationTimer | 1 ms |
| drx-InactivityTimer | 1 ms |
| drx-RetransmissionTimerDL | 1 slot |
| drx-RetransmissionTimerUL | 1 slot |
| drx-LongCycleStartOffset | 40 ms |
| shortDRX | disable |
| TimeAlignmentTimer | 500 ms |
| Note: This DRX configuration is applicable for NR serving cell. The DRX cycle and time alignment timer parameters are specified in clause 6.3.2 in TS 38.331 [2] |  |

### A.3.3.2	DRX Configuration 2: DRX cycle = 640 ms and TAT = 500 ms

Table A.3.3.2-1: DRX.2: DRX cycle = 640 ms and time alignment timer (TAT) = 500 ms

| Field | Value |
| --- | --- |
| drx-onDurationTimer | 1 ms |
| drx-InactivityTimer | 1 ms |
| drx-RetransmissionTimerDL | 1 slot |
| drx-RetransmissionTimerUL | 1 slot |
| drx-LongCycleStartOffset | 640 ms |
| shortDRX | disable |
| TimeAlignmentTimer | 500 ms |
| Note: This DRX configuration is applicable for NR serving cell. The DRX cycle and time alignment timer parameters are specified in clause 6.3.2 in TS 38.331 [2] |  |

### A.3.3.3	DRX Configuration 3: DRX cycle = 40 ms and TAT = Infinity

Table A.3.3.3-1: DRX.3: DRX cycle = 40 ms and time alignment timer (TAT) = Infinity

| Field | Value |
| --- | --- |
| drx-onDurationTimer | 6 ms |
| drx-InactivityTimer | 1 ms |
| drx-RetransmissionTimerDL | 1 slot |
| drx-RetransmissionTimerUL | 1 slot |
| drx-LongCycleStartOffset | 40 ms |
| shortDRX | disable |
| TimeAlignmentTimer | Infinity |
| Note: This DRX configuration is applicable for NR serving cell. The DRX cycle and time alignment timer parameters are specified in clause 6.3.2 in TS 38.331 [2] |  |

### A.3.3.4	DRX Configuration 4: DRX cycle = 160 ms and TAT = Infinity

Table A.3.3.4-1: DRX.4: DRX cycle = 160 ms and time alignment timer (TAT) = Infinity

| Field | Value |
| --- | --- |
| drx-onDurationTimer | psf2 |
| drx-InactivityTimer | psf2 |
| drx-RetransmissionTimer | Psf16 |
| longDRX-CycleStartOffset | sf160, 0 |
| shortDRX | disable |
| TimeAlignmentTimer | Infinity |
| Note: This DRX configuration is applicable for E-UTRA serving cell. For further information see clause 6.3.2 in TS 36.331 [16]. |  |

### A.3.3.5	DRX Configuration 5: DRX cycle = 320 ms and TAT = Infinity

Table A.3.3.5-1: DRX.5: DRX cycle = 320 ms and time alignment timer (TAT) = Infinity

| Field | Value |
| --- | --- |
| drx-onDurationTimer | psf6 |
| drx-InactivityTimer | psf1920 |
| drx-RetransmissionTimer | psf16 |
| longDRX-CycleStartOffset | sf320, 0 |
| shortDRX | disable |
| TimeAlignmentTimer | Infinity |
| Note: This DRX configuration is applicable for E-UTRA serving cell. For further information see clause 6.3.2 in TS 36.331 [16]. |  |

### A.3.3.6	DRX Configuration 6: DRX cycle = 320 ms and TAT = 500 ms

Table A.3.3.6-1: DRX.6: DRX cycle = 320 ms and time alignment timer (TAT) = 500 ms

| Field | Value |
| --- | --- |
| drx-onDurationTimer | 1 ms |
| drx-InactivityTimer | 1 ms |
| drx-RetransmissionTimerDL | 1 slot |
| drx-RetransmissionTimerUL | 1 slot |
| drx-LongCycleStartOffset | 320 ms |
| shortDRX | disable |
| TimeAlignmentTimer | 500 ms |
| Note: This DRX configuration is applicable for NR serving cell. The DRX cycle and time alignment timer parameters are specified in clause 6.3.2 in TS 38.331 [2] |  |

### A.3.3.7	DRX Configuration 7: DRX cycle = 640 ms and TAT = Infinity

Table A.3.3.7-1: DRX.7: DRX cycle = 640 ms and time alignment timer (TAT) = Infinity

| Field | Value |
| --- | --- |
| drx-onDurationTimer | 6 ms |
| drx-InactivityTimer | 1 ms |
| drx-RetransmissionTimerDL | 1 slot |
| drx-RetransmissionTimerUL | 1 slot |
| drx-LongCycleStartOffset | 640 ms |
| shortDRX | disable |
| TimeAlignmentTimer | Infinity |
| Note: This DRX configuration is applicable for NR serving cell. The DRX cycle and time alignment timer parameters are specified in clause 6.3.2 in TS 38.331 [2] |  |

### A.3.3.8	DRX Configuration 8: DRX cycle = 320 ms and TAT = Infinity

Table A.3.3.8-1: DRX.8: DRX cycle = 320 ms and time alignment timer (TAT) = Infinity

| Field | Value |
| --- | --- |
| drx-onDurationTimer | 6 ms |
| drx-InactivityTimer | 1 ms |
| drx-RetransmissionTimerDL | 1 slot |
| drx-RetransmissionTimerUL | 1 slot |
| drx-LongCycleStartOffset | 320 ms |
| shortDRX | disable |
| TimeAlignmentTimer | Infinity |
| Note: This DRX configuration is applicable for NR serving cell. The DRX cycle and time alignment timer parameters are specified in clause 6.3.2 in TS 38.331 [2] |  |

### A.3.3.9	DRX Configuration 9: DRX cycle = 40 ms and TAT = 500 ms

Table A.3.3.9-1: DRX.9: DRX cycle = 40 ms and time alignment timer (TAT) = 500 ms

| Field | Value |
| --- | --- |
| drx-onDurationTimer | psf2 |
| drx-InactivityTimer | psf2 |
| drx-RetransmissionTimer | psf16 |
| longDRX-CycleStartOffset | sf40, 0 |
| shortDRX | disable |
| TimeAlignmentTimer | 500 ms |
| Note: This DRX configuration is applicable for E-UTRA serving cell. For further information see clause 6.3.2 in TS 36.331 [16]. |  |

### A.3.3.10	DRX Configuration 10: DRX cycle = 640 ms and TAT = 500 ms

Table A.3.3.10-1: DRX.10: DRX cycle = 640 ms and time alignment timer (TAT) = 500 ms

| Field | Value |
| --- | --- |
| drx-onDurationTimer | psf6 |
| drx-InactivityTimer | psf2 |
| drx-RetransmissionTimer | psf16 |
| longDRX-CycleStartOffset | sf640, 0 |
| shortDRX | disable |
| TimeAlignmentTimer | 500 ms |
| Note: This DRX configuration is applicable for E-UTRA serving cell. For further information see clause 6.3.2 in TS 36.331 [16]. |  |

### A.3.3.11	DRX Configuration 11: DRX cycle = 20 ms and TAT = Infinity

Table A.3.3.11-1: DRX.11: DRX cycle = 20 ms and time alignment timer (TAT) = Infinity

| Field | Value |
| --- | --- |
| drx-onDurationTimer | 6 ms |
| drx-InactivityTimer | 1 ms |
| drx-RetransmissionTimerDL | 1 slot |
| drx-RetransmissionTimerUL | 1 slot |
| drx-LongCycleStartOffset | 20 ms |
| shortDRX | disable |
| TimeAlignmentTimer | Infinity |
| Note: This DRX configuration is applicable for NR serving cell. The DRX cycle and time alignment timer parameters are specified in clause 6.3.2 in TS 38.331 [2] |  |

### A.3.3.12	DRX Configuration 12: DRX cycle = 640 ms and TAT = Infinity

Table A.3.3.12-1: DRX.12: DRX cycle = 640 ms and time alignment timer (TAT) = Infinity

| Field | Value |
| --- | --- |
| drx-onDurationTimer | psf6 |
| drx-InactivityTimer | psf2 |
| drx-RetransmissionTimer | psf16 |
| longDRX-CycleStartOffset | sf640, 0 |
| shortDRX | disable |
| TimeAlignmentTimer | Infinity |
| Note: This DRX configuration is applicable for E-UTRA serving cell. For further information see clause 6.3.2 in TS 36.331 [16]. |  |

### A.3.3.13	DRX Configuration X1: DRX cycle = 80 ms and TAT = Infinity

Table A.3.3.13-1: DRX.X1: DRX cycle = 80 ms and time alignment timer (TAT) = Infinity

| Field | Value |
| --- | --- |
| drx-onDurationTimer | 6 ms |
| drx-InactivityTimer | 1 ms |
| drx-RetransmissionTimerDL | 1 slot |
| drx-RetransmissionTimerUL | 1 slot |
| drx-LongCycleStartOffset | 80 ms |
| shortDRX | disable |
| TimeAlignmentTimer | Infinity |
| Note: This DRX configuration is applicable for NR serving cell. The DRX cycle and time alignment timer parameters are specified in clause 6.3.2 in TS 38.331 [2] |  |

### A.3.3.14	DRX Configuration 14: DRX cycle = 160 ms and TAT = Infinity

Table A.3.3.13-1: DRX.14: DRX cycle = 160 ms and time alignment timer (TAT) = Infinity

| Field | Value |
| --- | --- |
| drx-onDurationTimer | 6 ms |
| drx-InactivityTimer | 1 ms |
| drx-RetransmissionTimerDL | 1 slot |
| drx-RetransmissionTimerUL | 1 slot |
| drx-LongCycleStartOffset | 160 ms |
| shortDRX | disable |
| TimeAlignmentTimer | Infinity |
| Note: This DRX configuration is applicable for NR serving cell. The DRX cycle and time alignment timer parameters are specified in clause 6.3.2 in TS 38.331 [2] |  |

## A.3.4	Test Cases with Different Channel Bandwidths

### A.3.4.1	Test Cases with Different E-UTRA Channel Bandwidths

#### A.3.4.1.1	Introduction

In Annex A test cases involving E-UTRA cell(s) may be defined with different E-UTRA channel bandwidths to verify the same type of RRM requirement.

#### A.3.4.1.2	Principle of testing

If multiple test cases involving E-UTRA cell(s) are defined with different E-UTRA channel bandwidths to verify the same type of RRM requirement that is E-UTRA channel bandwidth independent, then the UE needs to be tested with only one channel bandwidth in each E-UTRA cell and with the same bandwidth in all the E-UTRA cells used in the test case.

## A.3.5	Test Cases for Synchronous and Asynchronous DC Operations

### A.3.5.1	EN-DC Test Cases for Synchronous and Asynchronous EN-DC Operations

#### A.3.5.1.1	Introduction

This clause defines a principle which is applicable to test cases verifying RRM requirements for EN-DC operation in synchronous and asynchronous scenarios.

In Annex A test cases may be defined in both synchronous EN-DC and asynchronous EN-DC scenarios to verify the same type of RRM requirement.

#### A.3.5.1.2	Principle of Testing

If EN-DC test cases are defined in both synchronous and asynchronous EN-DC scenarios to verify the same type of RRM requirement then the UE capable of both synchronous and asynchronous EN-DC operations needs to be tested with one of the tests in either synchronous or asynchronous EN-DC scenarios.

## A.3.6	Antenna configurations

### A.3.6.1	Antenna configurations for FR1

Unless otherwise specified, NR FDD or NR TDD cells in all RRM Test cases in AWGN propagation condition are configured with Antenna Configuration 1x2.

#### A.3.6.1.1	Antenna connection for 4 Rx capable UEs

##### A.3.6.1.1.1	Introduction

All tests in clause A.4 and A.6 are specified for UEs supporting 2RX. In this clause, the antenna connection method for applying 2RX tests to UEs supporting 4RX antenna ports is specified. No tests are currently specified in clause A.4 or A.6 which are applicable only to 4RX antenna ports, so 4RX capable UEs are always tested by reusing tests which were originally specified for 2RX UEs.

##### A.3.6.1.1.2	Principle of testing

A.3.6.1.1.2.1	Single carrier tests

For 4RX capable UEs supporting at least one band where 2RX is supported and 4RX is not supported, the, all single carrier tests specified in clause A.4 and A.6 except those in A.4.7 and A.6.7 shall be tested on any band where 2RX is supported and 4RX is not supported with the antenna connection specified in A.3.6.1.1.2.4. For single carrier tests specified in clause A.4.7 or A.6.7, all tests shall be tested with the antenna connection specified in A.3.6.1.1.2.4 for bands where 2RX is supported and 4RX is not supported, and the antenna connection specified in A.3.6.1.1.2.5 for bands where 4RX is supported.

For 4RX capable UEs which do not support any band where 2RX is supported and 4RX is not supported, all tests specified in clauses A.4 and A.6 shall be tested using the antenna connection specified in clause A.3.6.1.1.2.5. For radio link monitoring tests, the SNR levels are modified according to table A.3.6.1.1.2.1-1 and table A.3.6.1.1.2.1-2. For beam failure detection and link recovery tests, the SNR levels are modified according to table A.3.6.1.1.2.1-3.

Table A.3.6.1.1.2.1-1: Modified parameters for RLM out of sync testing with 4 RX antenna connection

| Test case | SNR during T3 (dB) |  |  |  |
| --- | --- | --- | --- | --- |
|  | Test 1 | Test 2 | Test 3 | Test 4 |
| A.4.5.1.1 | -18 | N/A | N/A | N/A |
| A.4.5.1.3 | -18 | N/A | N/A | N/A |
| A.4.5.1.5 | -18 | N/A | N/A | N/A |
| A.4.5.1.7 | -18 | N/A | N/A | N/A |
| A.5.5.1.1 | -18 | N/A | N/A | N/A |
| A.5.5.1.3 | -18 | N/A | N/A | N/A |
| A.5.5.1.5 | -18 | N/A | N/A | N/A |
| A.5.5.1.7 | -18 | N/A | N/A | N/A |
| A.6.5.1.1 | -18 | N/A | N/A | N/A |
| A.6.5.1.3 | -18 | N/A | N/A | N/A |
| A.6.5.1.5 | -18 | N/A | N/A | N/A |
| A.6.5.1.7 | -18 | N/A | N/A | N/A |
| A.7.5.1.1 | -18 | N/A | N/A | N/A |
| A.7.5.1.3 | -18 | N/A | N/A | N/A |
| A.7.5.1.5 | -18 | N/A | N/A | N/A |
| A.7.5.1.7 | -18 | N/A | N/A | N/A |

Table A.3.6.1.1.2.1-2: Modified parameters for RLM in sync single carrier testing with 4 RX antenna connection

| Test case | SNR during T3 (dB) |  | SNR during T4 (dB) |  |
| --- | --- | --- | --- | --- |
|  | Test 1 | Test 2 | Test 1 | Test 2 |
| A.4.5.1.2 | -18 | N/A | -8 | N/A |
| A.4.5.1.4 | -18 | N/A | -8 | N/A |
| A.4.5.1.6 | -18 | N/A | -8 | N/A |
| A.4.5.1.8 | -18 | N/A | -8 | N/A |
| A.5.5.1.2 | -18 | N/A | -8 | N/A |
| A.5.5.1.4 | -18 | N/A | -8 | N/A |
| A.5.5.1.6 | -18 | N/A | -8 | N/A |
| A.5.5.1.8 | -18 | N/A | -8 | N/A |
| A.6.5.1.2 | -18 | N/A | -8 | N/A |
| A.6.5.1.4 | -18 | N/A | -8 | N/A |
| A.6.5.1.6 | -18 | N/A | -8 | N/A |
| A.6.5.1.8 | -18 | N/A | -8 | N/A |
| A.7.5.1.2 | -18 | N/A | -8 | N/A |
| A.7.5.1.4 | -18 | N/A | -8 | N/A |
| A.7.5.1.6 | -18 | N/A | -8 | N/A |
| A.7.5.1.8 | -18 | N/A | -8 | N/A |

Table A.3.6.1.1.2.1-3: Modified parameters for Beam Failure Detection and Link Recovery testing with 4 RX antenna connection

| Test case | SNR for RS in set q0 during T3, T4 and T5 (dB) |
| --- | --- |
|  | Test 1 |
| A.4.5.5.1 | -15 |
| A.4.5.5.2 | -15 |
| A.4.5.5.3 | -15 |
| A.4.5.5.4 | -15 |
| A.4.5.5.5 | -15 |
| A.4.5.5.6 | -15 |
| A.5.5.5.1 | -15 |
| A.5.5.5.2 | -15 |
| A.5.5.5.3 | -15 |
| A.5.5.5.4 | -15 |
| A.5.5.5.5 | -15 |
| A.5.5.5.6 | -15 |
| A.5.5.5.7 | -15 |
| A.6.5.5.1 | -15 |
| A.6.5.5.2 | -15 |
| A.6.5.5.3 | -15 |
| A.6.5.5.4 | -15 |
| A.6.5.5.5 | -15 |
| A.6.5.5.6 | -15 |
| A.7.5.5.1 | -15 |
| A.7.5.5.2 | -15 |
| A.7.5.5.3 | -15 |
| A.7.5.5.4 | -15 |
| A.7.5.5.5 | -15 |
| A.7.5.5.6 | -15 |
| A.7.5.5.7 | -15 |

A.3.6.1.1.2.2	Carrier aggregation tests

All carrier aggregation tests are performed using the antenna connection in clause A.3.6.1.1.2.4 for the PCell antenna connection if the PCell is on a band where 2RX is supported and 4RX is not supported, or using the antenna connection in A.3.6.1.1.2.5 for the PCell antenna connection if the PCell is on a band where 4RX is supported.

All carrier aggregation tests are performed using the antenna connection in clause A.3.6.1.1.2.4 for the SCell antenna connection if an SCell is on band where 2RX is supported and 4RX is not supported, or using the antenna connection in A.3.6.1.1.2.5 for the SCell antenna connection if an SCell is on a band where 4RX is supported.

A.3.6.1.1.2.3	EN-DC tests

All EN-DC tests are performed using the antenna connection in clause A.3.6.1.1.2.6 for the PCell antenna connection if the PCell is on a band where 2RX is supported and 4RX is not supported, or using the antenna connection in A.3.6.1.1.2.7 for the PCell antenna connection if the PCell is on a band where 4RX is supported.

All EN-DC tests are performed using the antenna connection in clause A.3.6.1.1.2.4 for the PSCell or SCell antenna connection if an SCell is on band where 2RX is supported and 4RX is not supported, or using antenna connection in A.3.6.1.1.2.5 for the SCell antenna connection if an SCell or PSCell is on a band where 4RX is supported.

A.3.6.1.1.2.4	Antenna connection for bands where 2RX is supported

For bands where 2RX is supported and 4RX is not supported, it is left to the UE declaration and antenna port configuration to decide which 2 of the 4 Rx ports are connected with data source from system simulator. The remaining 2 RX ports shall be connected with zero input. No test parameters or requirements are modified.

A.3.6.1.1.2.5	Antenna connection for bands where 4RX is supported

For bands where 4RX is supported, all 4 RX antennas are connected with data source from system simulator. The system simulator shall provide independent noise and fading (low correlation) for each antenna port. Except for the modifications to radio link monitoring thresholds and beam failure detection thresholds described in clauses A.3.6.1.1.2.1, no test parameters or requirements are modified.

A.3.6.1.1.2.6	EN-DC LTE Antenna connection for bands where 2RX is supported

For E-UTRAN bands where 2RX is supported and 4RX is not supported, it is left to the UE declaration and antenna port configuration to decide which 2 of the 4 Rx ports are connected with data source from system simulator. The remaining 2 RX ports shall be connected with zero input. No test parameters or requirements are modified.

A.3.6.1.1.2.7	EN-DC LTE Antenna connection for bands where 4RX is supported

For bands E-UTRAN where 4RX is supported, all 4 RX antennas are connected with data source from system simulator. The system simulator shall provide independent noise and fading (low correlation) for each antenna port. Except for the modifications to radio link monitoring thresholds described in clauses A.3.8.1.2.1 and A.3.8.1.2.2 of TS 36.133 [15], no test parameters or requirements are modified.

#### A.3.6.1.2	Antenna connection for 8 Rx capable UEs

##### A.3.6.1.2.1	Introduction

All tests in clause A.4 and A.6 are specified for UEs supporting 2RX. In this clause, the antenna connection method for applying 2RX tests or 4RX tests to UEs supporting 8RX antenna ports is specified. No tests are currently specified in clause A.4 or A.6 which are applicable only to 8RX antenna ports, so 8RX capable UEs are always tested by reusing tests which were originally specified for 2RX UEs or 4Rx UEs.

##### A.3.6.1.2.2	Principle of testing

A.3.6.1.2.2.1	Single carrier tests

For 8RX capable UEs supporting at least one band where 2RX is supported and either 4RX or 8RX is not supported, all single carrier tests specified in clause A.4 and A.6 except those in A.4.7 and A.6.7 shall be tested on any band where 2RX is supported and 8RX is not supported with the antenna connection specified in A.3.6.1.2.2.4. For single carrier tests specified in clause A.4.7 or A.6.7, all tests shall be tested with the antenna connection specified in A.3.6.1.2.2.4 for bands where 2RX is supported and 8RX is not supported, and the antenna connection specified in A.3.6.1.2.2.6 for bands where 8RX is supported.

For 8RX capable UEs supporting at least one band where 4RX is supported and 8RX is not supported but without supporting any band where only 2RX is supported, all single carrier tests specified in clause A.4 and A.6 except those in A.4.7 and A.6.7 shall be tested on any band where 4RX is supported and 8RX is not supported with the antenna connection specified in A.3.6.1.2.2.5. For single carrier tests specified in clause A.4.7 or A.6.7, all tests shall be tested with the antenna connection specified in A.3.6.1.2.2.5 for bands where 4RX is supported and 8RX is not supported, and the antenna connection specified in A.3.6.1.2.2.6 for bands where 8RX is supported. For radio link monitoring tests, the SNR levels are modified according to table A.3.6.1.1.2.1-1 and table A.3.6.1.1.2.1-2. For beam failure detection and link recovery tests, the SNR levels are modified according to table A.3.6.1.1.2.1-3.

For 8RX capable UEs which do not support any band where 8RX is not supported, all tests specified in clauses A.4 and A.6 shall be tested using the antenna connection specified in clause A.3.6.1.2.2.6. For radio link monitoring tests, the SNR levels are modified according to table A.3.6.1.1.2.1-1 and table A.3.6.1.1.2.1-2. For beam failure detection and link recovery tests, the SNR levels are modified according to table A.3.6.1.1.2.1-3.

A.3.6.1.2.2.2	Carrier aggregation tests

All carrier aggregation tests are performed using the antenna connection in clause A.3.6.1.2.2.4 for the PCell antenna connection if the PCell is on a band where 2RX is supported and either 4RX or 8RX is not supported, or using the antenna connection in A.3.6.1.2.2.5 for the PCell antenna connection if the PCell is on a band where 4RX is supported and 8RX is not supported, or using the antenna connection in A.3.6.1.2.2.6 for the PCell antenna connection if the PCell is on a band where 8RX is supported.

All carrier aggregation tests are performed using the antenna connection in clause A.3.6.1.2.2.4 for the SCell antenna connection if an SCell is on band where 2RX is supported and either 4RX or 8RX is not supported, or using the antenna connection in A.3.6.1.2.2.5 for the SCell antenna connection if an SCell is on a band where 4RX is supported and 8RX is not supported, or using the antenna connection in A.3.6.1.2.2.6 for the SCell antenna connection if an SCell is on a band where 8RX is supported.

A.3.6.1.2.2.3	EN-DC tests

All EN-DC tests are performed using the antenna connection in clause A.3.6.1.2.2.7 for the PCell antenna connection if the PCell is on a band where 2RX is supported and either 4RX or 8RX is not supported, or using the antenna connection in A.3.6.1.2.2.8 for the PCell antenna connection if the PCell is on a band where 4RX is supported and 8RX is not supported, or using the antenna connection in A.3.6.1.2.2.9 for the PCell antenna connection if the PCell is on a band where 8RX is supported.

All EN-DC tests are performed using the antenna connection in clause A.3.6.1.2.2.4 for the PSCell or SCell antenna connection if an SCell or PSCell is on band where 2RX is supported and either 4RX or 8RX is not supported, or using antenna connection in A.3.6.1.2.2.5 for the PSCell or SCell antenna connection if an SCell or PSCell is on a band where 4RX is supported and 8RX is not supported, or using antenna connection in A.3.6.1.2.2.6 for the PSCell or SCell antenna connection if an SCell or PSCell is on a band where 8RX is supported.

A.3.6.1.2.2.4	Antenna connection for bands where 2RX is supported

For bands where 2RX is supported and either 4RX or 8RX is not supported, it is left to the UE declaration and antenna port configuration to decide which 2 of the 8 Rx ports are connected with data source from system simulator. The remaining 6 RX ports shall be connected with zero input. No test parameters or requirements are modified.

A.3.6.1.2.2.5	Antenna connection for bands where 4RX is supported

For bands where 4RX is supported and 8RX is not supported, it is left to the UE declaration and antenna port configuration to decide which 4 of the 8 Rx ports are connected with data source from system simulator. The remaining 4 RX ports shall be connected with zero input. Except for the modifications to radio link monitoring thresholds and beam failure detection thresholds described in clauses A.3.6.1.1.2.1, no test parameters or requirements are modified.

A.3.6.1.2.2.6	Antenna connection for bands where 8RX is supported

For bands where 8RX is supported, all 8 RX antennas are connected with data source from system simulator. The system simulator shall provide independent noise and fading (low correlation) for each antenna port. Except for the modifications to radio link monitoring thresholds described in clauses A.3.6.1.1.2.1, no test parameters or requirements are modified.

A.3.6.1.2.2.7	EN-DC LTE Antenna connection for bands where 2RX is supported

For E-UTRAN bands where 2RX is supported and 4RX or 8RX is not supported, it is left to the UE declaration and antenna port configuration to decide which 2 of the 8 Rx ports are connected with data source from system simulator. The remaining 6 RX ports shall be connected with zero input. No test parameters or requirements are modified.

A.3.6.1.2.2.8	EN-DC LTE Antenna connection for bands where 4RX is supported

For bands E-UTRAN where 4RX is supported and 8RX is not supported, it is left to the UE declaration and antenna port configuration to decide which 4 of the 8 Rx ports are connected with data source from system simulator. The remaining 4 RX ports shall be connected with zero input. Except for the modifications to radio link monitoring thresholds described in clauses A.3.8.1.2.1 and A.3.8.1.2.2 of TS 36.133 [15], no test parameters or requirements are modified.

A.3.6.1.2.2.9	EN-DC LTE Antenna connection for bands where 8RX is supported

For bands E-UTRAN where 8RX is supported, all 8 RX antennas are connected with data source from system simulator. The system simulator shall provide independent noise and fading (low correlation) for each antenna port. Except for the modifications to radio link monitoring thresholds described in clauses A.3.8.1.2.1 and A.3.8.1.2.2 of TS 36.133 [15], no test parameters or requirements are modified.

### A.3.6.2	Antenna configurations for FR2

Unless otherwise specified, the default Downlink Antenna Configuration for NR FR2 cells is 1x2.

In case of Downlink Antenna Configuration 2x2 for NR FR2 cells, unless otherwise specified, the downlink signal is transmitted over the two polarizations (V and H) of the dual polarized antenna of the test equipment.

In both cases, the downlink signal is received assuming 2 UE baseband receivers. As the UE is tested following the Blackbox Approach with regard to the UE Rx antennas, the exact UE Rx antenna configuration is not relevant for the test configuration and has no impact on the test implementation.

## A.3.6A	Antenna configurations with unlicensed bands

### A.3.6A.1	Antenna configurations for FR1

Unless otherwise specified, NR unlicensed cells in all RRM Test cases in AWGN propagation condition are configured with Antenna Configuration 1x2.

#### A.3.6A.1.1	Antenna connection for 4 Rx capable UEs

##### A.3.6A.1.1.1	Introduction

All tests in clause A.13, A.10, A.11, and A.12 are specified for UEs supporting 2RX. In this clause, the antenna connection method for applying 2RX tests to UEs supporting 4RX antenna ports is specified. No tests are currently specified in clause A.13, A.10, A.11 or A.12 which are applicable only to 4RX antenna ports, so 4RX capable UEs are always tested by reusing tests which were originally specified for 2RX UEs.

##### A.3.6A.1.1.2	Principle of testing

A.3.6A.1.1.2.1	Single carrier tests

For 4RX capable UEs supporting at least one 2RX band, the, all single carrier tests specified in clause A.13. A.10, A.11 and A.12 except those in A.13.4, A.10.5, A.11.6 and A.12.5 shall be tested on any band where 2RX is supported with the antenna connection specified in A.3.6A.1.1.2.4. For single carrier tests specified in clause A.13.4, A.10.5, A.11.6 or A.12.5, all tests shall be tested with the antenna connection specified in A.3.6A.1.1.2.4 for bands where 2RX is supported, and the antenna connection specified in A.3.6A.1.1.2.5 for bands where 4RX is supported.

For 4RX capable UEs which do not support any 2RX band, all tests specified in clauses A.13, A.10, A.11 and A.12 shall be tested using the antenna connection specified in clause A.3.6A.1.1.2.5. For radio link monitoring tests, the SNR levels are modified according to table A.3.6A.1.1.2.1-1 and table A.3.6A.1.1.2.1-2

Table A.3.6A.1.1.2.1-1: Modified parameters for RLM out of sync testing with 4 RX antenna connection

| Test case | SNR during T3 (dB) |  |  |  |
| --- | --- | --- | --- | --- |
|  | Test 1 | Test 2 | Test 3 | Test 4 |
| A.10.3.1.2 | -18 | N/A | N/A | N/A |
| A.11.4.1.2 | -18 | N/A | N/A | N/A |

Table A.3.6A.1.1.2.1-2: Modified parameters for RLM in sync single carrier testing with 4 RX antenna connection

| Test case | SNR during T3 (dB) |  | SNR during T4 (dB) |  |
| --- | --- | --- | --- | --- |
|  | Test 1 | Test 2 | Test 1 | Test 2 |
| A.10.3.1.3 | -18 | N/A | -8 | N/A |
| A.11.4.1.3 | -18 | N/A | -8 | N/A |

Table A.3.6A.1.1.2.1-3: Modified parameters for Beam Failure Detection and Link Recovery testing with 4 RX antenna connection

| Test case | SNR for RS in set q0 during T3, T4 and T5 (dB) |  |
| --- | --- | --- |
|  | Test 1 | Test 2 |
| A.10.3.4.1 | -15 | N/A |
| A.10.3.4.2 | -15 | N/A |
| A.11.4.4.1 | -15 | N/A |
| A.11.4.4.2 | -15 | N/A |

A.3.6A.1.1.2.2	Carrier aggregation tests

All carrier aggregation tests are performed using the antenna connection in clause A.3.6A.1.1.2.4 for the PCell antenna connection if the PCell is on a band where 2RX is supported or the antenna connection in A.3.6A.1.1.2.5 for the PCell antenna connection if the PCell is on a band where 4RX is supported.

All carrier aggregation tests are performed using the antenna connection in clause A.3.6A.1.1.2.4 for the SCell antenna connection if an SCell is on band where 2RX is supported or the testing procedure in A.3.6A.1.1.2.5 for the SCell antenna connection if an SCell is on a band where 4RX is supported.

A.3.6A.1.1.2.3	EN-DC tests

All carrier aggregation tests are performed using the antenna connection in clause A.3.6A.1.1.2.6 for the PCell antenna connection if the PCell is on a band where 2RX is supported or the antenna connection in A.3.6A.1.1.2.7 for the PCell antenna connection if the PCell is on a band where 4RX is supported.

All carrier aggregation tests are performed using the antenna connection in clause A.3.6A.1.1.2.4 for the PSCell or SCell antenna connection if an SCell is on band where 2RX is supported or the testing procedure in A.3.6A.1.1.2.5 for the SCell antenna connection if an SCell or PSCell is on a band where 4RX is supported.

A.3.6A.1.1.2.4	Antenna connection for bands where 2RX is supported

For bands where 2RX is supported, it is left to the UE declaration and AP configuration to decide which 2 of the 4 Rx ports are connected with data source from system simulator. The remaining 2 Rx ports shall be connected with zero input. No test parameters or requirements are modified.

A.3.6A.1.1.2.5	Antenna connection for bands where 4RX is supported

For bands where 4RX is supported, all 4 RX antennas are connected with data source from system simulator. The system simulator shall provide independent noise and fading (low correlation) for each antenna port. Except for the modifications to radio link monitoring thresholds described in clauses A.3.6A.1.1.2.1 and A.3.6A.1.1.2.2, no test parameters or requirements are modified.

A.3.6A.1.1.2.6	EN-DC LTE Antenna connection for bands where 2RX is supported

For bands where LTE 2RX is supported, it is left to the UE declaration and AP configuration to decide which 2 of the 4 Rx ports are connected with data source from system simulator. The remaining 2 Rx ports shall be connected with zero input. No test parameters or requirements are modified.

A.3.6A.1.1.2.7	EN-DC LTE Antenna connection for bands where 4RX is supported

For bands where LTE 4RX is supported, all 4 RX antennas are connected with data source from system simulator. The system simulator shall provide independent noise and fading (low correlation) for each antenna port. Except for the modifications to radio link monitoring thresholds described in clauses A.3.8.1.2.1 and A.3.8.1.2.2 of TS 36.133 [15], no test parameters or requirements are modified.

## A.3.7	EN-DC test setup

### A.3.7.1	Introduction

### A.3.7.2	E-UTRAN Serving Cell Parameters

#### A.3.7.2.1	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR1

Table A.3.7.2.1-1 defines cell specific test parameters for E-UTRAN cell which can be used in EN-DC test cases or in any test case comprising at least one E-UTRA serving cell with all NR cells in FR1. Unless otherwise stated within the test, all measurements in Annex A.4 and A.5 are performed only on the NR carrier. The E-UTRA serving cell shall configured to not interfere with NR operation and the E-UTRA serving cell signal power shall not be critical to the test purpose.

Table A.3.7.2.1-1: E-UTRAN cell specific test parameters for tests with all NR cells in FR1

| Parameter | Unit | E-UTRAN Cell |
| --- | --- | --- |
| Duplex mode |  | FDD or TDD |
| TDD special subframe configurationNote1 |  | 6 |
| TDD uplink-downlink configurationNote1 |  | 1 |
| BWchannel |  | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  | 5 MHz: R.7 FDD 10 MHz: R.3 FDD 20 MHz: R.6 FDD 5 MHz: R.4 TDD 10 MHz: R.0 TDD 20 MHz: R.3 TDD |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 |  | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |
| OCNG PatternsNote2 |  | 5 MHz: OP.20 FDD 10 MHz: OP.10 FDD 20 MHz: OP.17 FDD 5 MHz: OP.9 TDD 10 MHz: OP.1 TDD 20 MHz: OP.7 TDD |
| PBCH_RA | dB |  |
| PBCH_RB | dB |  |
| PSS_RA | dB |  |
| SSS_RA | dB |  |
| PCFICH_RB | dB |  |
| PHICH_RA | dB |  |
| PHICH_RB | dB | 0 |
| PDCCH_RA | dB |  |
| PDCCH_RB | dB |  |
| PDSCH_RA | dB |  |
| PDSCH_RB | dB |  |
| OCNG_RANote3 | dB |  |
| OCNG_RBNote3 | dB |  |
| NocNote4 | dBm/15 kHz | -104 |
| Ês/Noc | dB | 17 |
| Ês/Iot | dB | 17 |
| RSRP Note5 | dBm/15 kHz | -87 |
| SCH_RP Note5 | dBm/15 kHz | -87 |
| Io Note5 | dBm/Ch BW | -59.13+10log(NRB,c /50) |
| Propagation Condition |  | AWGN |
| Antenna Configuration |  | 1x2 |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211. Note 2: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 respectively. Note 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 5: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |

#### A.3.7.2.2	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR2

Table A.3.7.2.2-1 defines cell specific test parameters for E-UTRAN cell which can be used in EN-DC test cases or in any test case comprising at least one E-UTRA serving cell with one or more NR cells in FR2.

Table A.3.7.2.2-1: E-UTRAN cell specific test parameters for tests with one or more NR cells in FR2

| Parameter | Unit | E-UTRAN Cell |
| --- | --- | --- |
| Duplex mode |  | FDD or TDD |
| TDD special subframe configurationNote1 |  | 6 |
| TDD uplink-downlink configurationNote1 |  | 1 |
| BWchannel | MHz | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  | 5 MHz: R.7 FDD 10 MHz: R.3 FDD 20 MHz: R.6 FDD 5 MHz: R.4 TDD 10 MHz: R.0 TDD 20 MHz: R.3 TDD |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 |  | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |
| OCNG PatternsNote2 |  | 5 MHz: OP.20 FDD 10 MHz: OP.10 FDD 20 MHz: OP.17 FDD 5 MHz: OP.9 TDD 10 MHz: OP.1 TDD 20 MHz: OP.7 TDD |
| PBCH_RA | dB |  |
| PBCH_RB | dB |  |
| PSS_RA | dB |  |
| SSS_RA | dB |  |
| PCFICH_RB | dB |  |
| PHICH_RA | dB |  |
| PHICH_RB | dB | 0 |
| PDCCH_RA | dB |  |
| PDCCH_RB | dB |  |
| PDSCH_RA | dB |  |
| PDSCH_RB | dB |  |
| OCNG_RANote3 | dB |  |
| OCNG_RBNote3 | dB |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211. Note 2: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 respectively. Note 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: The E-UTRA signal is required only to ensure the E-UTRA link to the DUT in the EN-DC operation. The Test System shall provide a stable and noise-free E-UTRA signal without need of precise propagation modelling, path loss and polarization control. Further details of the E-UTRA signal configuration are not defined as part of the cell specific test parameters, since the E-UTRA link is not under performance verification and shall not affect the test result unless otherwise specifically stated in the test case. |  |  |

## A.3.7A	NR FR1-FR2 test setup

Some Test cases in clause A.7 have NR cells in both FR1 and FR2. Unless otherwise stated within the test, the NR FR1 Cell signal is required only to provide a link to the UE under test. The Test System shall provide a stable and noise-free NR FR1 signal without need of precise propagation modelling, path loss and polarization control. Further details of the NR FR1 signal configuration are not defined as part of the cell specific test parameters, since the NR FR1 link is not under performance verification and shall not affect the test result unless otherwise specifically stated in the test case.

## A.3.7B	EN-DC test setup with unlicensed bands

### A.3.7B.1	Introduction

### A.3.7B.2	E-UTRAN Serving Cell Parameters

#### A.3.7B.2.1	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) under CCA in FR1

Table A.3.7A.2.1-1 defines cell specific test parameters for E-UTRAN cell which can be used in EN-DC test cases or in any test case comprising at least one E-UTRA serving cell with all NR cells under CCA in FR1. Unless otherwise stated within the test, all measurements in Annex A.4 and A.5 are performed only on the unlicensed NR carrier. The E-UTRA serving cell shall configured to not interfere with NR operation and the E-UTRA serving cell signal power shall not be critical to the test purpose.

Table A.3.7B.2.1-1: E-UTRAN cell specific test parameters for tests with all NR cells user CCA in FR1

| Parameter | Unit | E-UTRAN Cell |
| --- | --- | --- |
| Duplex mode |  | FDD or TDD |
| TDD special subframe configurationNote1 |  | 6 |
| TDD uplink-downlink configurationNote1 |  | 1 |
| BWchannel |  | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  | 5 MHz: R.7 FDD 10 MHz: R.3 FDD 20 MHz: R.6 FDD 5 MHz: R.4 TDD 10 MHz: R.0 TDD 20 MHz: R.3 TDD |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 |  | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |
| OCNG PatternsNote2 |  | 5 MHz: OP.20 FDD 10 MHz: OP.10 FDD 20 MHz: OP.17 FDD 5 MHz: OP.9 TDD 10 MHz: OP.1 TDD 20 MHz: OP.7 TDD |
| PBCH_RA | dB |  |
| PBCH_RB | dB |  |
| PSS_RA | dB |  |
| SSS_RA | dB |  |
| PCFICH_RB | dB |  |
| PHICH_RA | dB |  |
| PHICH_RB | dB | 0 |
| PDCCH_RA | dB |  |
| PDCCH_RB | dB |  |
| PDSCH_RA | dB |  |
| PDSCH_RB | dB |  |
| OCNG_RANote3 | dB |  |
| OCNG_RBNote3 | dB |  |
| NocNote4 | dBm/15 kHz | -104 |
| Ês/Noc | dB | 17 |
| Ês/Iot | dB | 17 |
| RSRP Note5 | dBm/15 kHz | -87 |
| SCH_RP Note5 | dBm/15 kHz | -87 |
| Io Note5 | dBm/Ch BW | -59.13+10log(NRB,c /50) |
| Propagation Condition |  | AWGN |
| Antenna Configuration |  | 1x2 |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211. Note 2: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 respectively. Note 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 5: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |

## A.3.7C	LTE-FR1/FR2 test setup

Some Test cases in clause A.5 have LTE and FR2 NR cells. Unless otherwise stated within the test, the LTE Cell signal is required only to provide a link to the UE under test. The Test System shall provide a stable and noise-free LTE signal without need of precise propagation modelling, path loss and polarization control. Further details of the LTE signal configuration are not defined as part of the cell specific test parameters, since the LTE link is not under performance verification and shall not affect the test result unless otherwise specifically stated in the test case.

## A.3.7D	NE-DC test setup

### A.3.7D.1	Introduction

### A.3.7D.2	E-UTRAN Serving Cell Parameters

#### A.3.7D.2.1	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR1

The parameters are same as as specified in clause A.3.7.2.1.

#### A.3.7D.2.2	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR2

The parameters are same as as specified in clause A.3.7.2.2.

## A.3.8	PRACH configurations

### A.3.8.1	Introduction

This clause provides the typical PRACH configurations used for RRM test cases defined in Annex A. To note that for other parameters not listed in this clause, either it can be derived from the set up of each test or it is subjected to RAN5 specifications.

### A.3.8.2	PRACH configurations in FR1

#### A.3.8.2.1	FR1 PRACH configuration 1

FR1 PRACH configuration 1 in this clause provides the typical PRACH configuration for SSB-based contention based random access in FR1.

Table A.3.8.2.1-1: Parameters for FR1 PRACH configuration 1

| Field | Value | Comment |
| --- | --- | --- |
| prach-ConfigurationIndex | 102 | 10ms PRACH periodicity, and other detailed configuration defined in table 6.3.3.2-2 and table 6.3.3.2-3 in TS 38.211 [6]. |
| msg1-SubcarrierSpacing | Same as UL carrier SCS |  |
| totalNumberOfRA-Preambles | 48 | Total number of preambles used for contention based and contention free random access |
| numberOfRA-PreamblesGroupA | 48 | No group B. |
| prach-RootSequenceIndex | 0 | Logic sequence index = 0, resulting in root sequence = 1. |
| ssb-perRACH-OccasionAndCB-PreamblesPerSSB | oneFourth, n48 | OneFourth: 1 SSB associated with 4 RACH occasions n48: 48 contention based preambles per SSB |
| msg1-FDM | One | One PRACH transmission occasions FDMed in one time instance. |
| rsrp-ThresholdSSB | RSRP_51 | The actual value of the threshold is -105dBm, as defined in TS 38.331 [2]. |
| ra-ContentionResolutionTimer | sf48 | 48 sub-frames |
| powerRampingStep | dB2 |  |
| preambleReceivedTargetPower | dBm-120 |  |
| preambleTransMax | n6 | Max number of RA preamble transmission performed before declaring a failure is 6 |
| ra-ResponseWindow | sl10 | 10 slots |
| zeroCorrelationZoneConfig | 11 | N-CS configuration, NCS = 23 |
| Backoff Parameter Index | 2 | 20ms, as defined in table 7.2-1 in TS 38.321 [7]. |
| Note: For further information see clause 6.3.2 in TS 38.331 [2]. |  |  |

#### A.3.8.2.2	FR1 PRACH configuration 2

FR1 PRACH configuration 2 in this clause provides the typical PRACH configuration for SSB based non-contention based random access in FR1.

Table A.3.8.2.2-1: Parameters for FR1 PRACH configuration 2

| Field | Value | Comment |
| --- | --- | --- |
| prach-ConfigurationIndex | 102 | 10ms PRACH periodicity, and other detailed configuration defined in table 6.3.3.2-2 and table 6.3.3.2-3 in TS 38.211 [6]. |
| msg1-SubcarrierSpacing | Same as UL carrier SCS |  |
| totalNumberOfRA-Preambles | 48 | Total number of preambles used for contention based and contention free random access |
| numberOfRA-PreamblesGroupA | 48 | No group B. |
| prach-RootSequenceIndex | 0 | Logic sequence index = 0, resulting in root sequence = 1. |
| ssb-perRACH-Occasion | oneFourth | OneFourth: 1 SSB associated with 4 RACH occasions |
| msg1-FDM | One | One PRACH transmission occasions FDMed in one time instance. |
| powerRampingStep | dB2 |  |
| preambleReceivedTargetPower | dBm-120 |  |
| preambleTransMax | n6 | Max number of RA preamble transmission performed before declaring a failure is 6 |
| ra-ResponseWindow | sl10 | 10 slots |
| zeroCorrelationZoneConfig | 11 | N-CS configuration, NCS = 23 |
| Backoff Parameter Index | 2 | 20ms, as defined in table 7.2-1 in TS 38.321 [7]. |
| ssb-ResourceList | ra-PreambleIndex = 50 | Associated with SSB index 0. UE doesn’t use ssb-ResourceList and BFR-SSB-Resource IEs at the same time. UE doesn’t use this field if is transmitting CFRA to convey BFR. |
| BFR-SSB-Resource | ra-PreambleIndex = 50 | Associated with SSB index 0. UE doesn’t use ssb-ResourceList and BFR-SSB-Resource IEs at the same time. UE uses this field only if is transmitting CFRA to convey BFR |
| ra-ssb-OccasionMaskIndex | 1 | PRACH occasion index 1 is allowed |
| rsrp-ThresholdSSB | RSRP_51 | The actual value of the threshold is -105dBm, as defined in TS 38.331 [2]. |
| Note: For further information see clause 6.3.2 in TS 38.331 [2]. |  |  |

#### A.3.8.2.3	FR1 PRACH configuration 3

FR1 PRACH configuration 3 in this clause provides the typical PRACH configuration for CSI-RS based non-contention based random access in FR1.

Table A.3.8.2.3-1: Parameters for FR1 PRACH configuration 3

| Field | Value | Comment |
| --- | --- | --- |
| prach-ConfigurationIndex | 102 | 10ms PRACH periodicity, and other detailed configuration defined in table 6.3.3.2-2 and table 6.3.3.2-3 in TS 38.211 [6]. |
| msg1-SubcarrierSpacing | Same as UL carrier SCS |  |
| totalNumberOfRA-Preambles | 48 | Total number of preambles used for contention based and contention free random access |
| numberOfRA-PreamblesGroupA | 48 | No group B. |
| prach-RootSequenceIndex | 0 | Logic sequence index = 0, resulting in root sequence = 1. |
| ssb-perRACH-Occasion | oneFourth | OneFourth: 1 SSB associated with 4 RACH occasions |
| msg1-FDM | One | One PRACH transmission occasions FDMed in one time instance. |
| powerRampingStep | dB2 |  |
| preambleReceivedTargetPower | dBm-120 |  |
| preambleTransMax | n6 | Max number of RA preamble transmission performed before declaring a failure is 6 |
| ra-ResponseWindow | sl10 | 10 slots |
| zeroCorrelationZoneConfig | 11 | N-CS configuration, NCS = 23 |
| Backoff Parameter Index | 2 | 20ms, as defined in table 7.2-1 in TS 38.321 [7]. |
| csirs-ResourceList | ra-PreambleIndex = 50 | Associated with CSI-RS configured |
| ra-OccasionList | 1 | RA occasions allowed corresponding to CSI-RS |
| rsrp-ThresholdCSI-RS | RSRP_51 | The actual value of the threshold is -105dBm, as defined in TS 38.331 [2]. |
| Note: For further information see clause 6.3.2 in TS 38.331 [2]. |  |  |

#### A.3.8.2.4	FR1 PRACH configuration 4

FR1 PRACH configuration 4 in this clause provides the PRACH configuration for CSI-RS based non-contention based random access in FR1 to convey BFR.

Table A.3.8.2.4-1: Parameters for FR1 PRACH configuration 4

| Field | Value | Comment |
| --- | --- | --- |
| prach-ConfigurationIndex | 102 | 10ms PRACH periodicity, and other detailed configuration defined in table 6.3.3.2-2 and table 6.3.3.2-3 in TS 38.211 [6]. |
| totalNumberOfRA-Preambles | 48 | Total number of preambles used for contention based and contention free random access |
| numberOfRA-PreamblesGroupA | 48 | No group B. |
| prach-RootSequenceIndex | 0 | Logic sequence index = 0, resulting in root sequence = 1. |
| ssb-perRACH-Occasion | oneFourth | OneFourth: 1 SSB associated with 4 RACH occasions |
| msg1-FDM | One | One PRACH transmission occasions FDMed in one time instance. |
| powerRampingStep | dB2 |  |
| preambleReceivedTargetPower | dBm-120 |  |
| preambleTransMax | n200 | Max number of RA preamble transmission performed before declaring a failure is 200 |
| ra-ResponseWindow | sl1 | 1 slot |
| zeroCorrelationZoneConfig | 11 | N-CS configuration, NCS = 93 |
| Backoff Parameter Index | 2 | 20ms, as defined in table 7.2-1 in TS 38.321 [7]. |
| BFR-CSIRS-Resource | ra-PreambleIndex = 50 | Associated with CSI-RS configured |
| ra-OccasionList | 1 | RA occasions allowed corresponding to CSI-RS |
| rsrp-ThresholdSSB | RSRP_51 | The actual value of the threshold is -105dBm, as defined in TS 38.331 [2]. |
| Note: For further information see clause 6.3.2 in TS 38.331 [2]. |  |  |

#### A.3.8.2.5	FR1 PRACH configuration 5

FR1 PRACH configuration 5 in this clause provides the typical PRACH configuration for LTM early UL synchronization on candidate cell in FR1.

Table A.3.8.2.5-1: Parameters for FR1 PRACH configuration 5

| Field | Value | Comment |
| --- | --- | --- |
| prach-ConfigurationIndex | 102 | 10ms PRACH periodicity, and other detailed configuration defined in table 6.3.3.2-2 and table 6.3.3.2-3 in TS 38.211 [6]. |
| msg1-FDM | One | One PRACH transmission occasions FDMed in one time instance. |
| msg1-FrequencyStart | 0 |  |
| zeroCorrelationZoneConfig | 11 | N-CS configuration, NCS = 23 |
| preambleReceivedTargetPower | dBm-120 |  |
| preambleTransMax | n6 |  |
| powerRampingStep | dB2 |  |
| ssb-perRACH-Occasion | oneFourth | OneFourth: 1 SSB associated with 4 RACH occasions |
| prach-RootSequenceIndex | 0 | Logic sequence index = 0, resulting in root sequence = 1. |
| ltm-prach-SubcarrierSpacing | Same as UL carrier SCS |  |
| Note: For further information see clause 6.3.2 in TS 38.331 [2]. |  |  |

#### A.3.8.2.6	FR1 PRACH configuration 6

FR1 PRACH configuration 6 in this clause provides the typical PRACH configuration for SSB-based contention free random access in FR1.

Table A.3.8.2.6-1: Parameters for FR1 PRACH configuration 6

| Field | Value | Comment |
| --- | --- | --- |
| prach-ConfigurationIndex | 102 | 10ms PRACH periodicity, and other detailed configuration defined in table 6.3.3.2-2 and table 6.3.3.2-3 in TS 38.211 [6]. |
| msg1-SubcarrierSpacing | Same as UL carrier SCS |  |
| totalNumberOfRA-Preambles | 48 | Total number of preambles used for contention based and contention free random access |
| numberOfRA-PreamblesGroupA | 48 | No group B. |
| prach-RootSequenceIndex | 0 | Logic sequence index = 0, resulting in root sequence = 1. |
| ssb-perRACH-OccasionAndCB-PreamblesPerSSB | oneFourth, n16 | OneFourth: 1 SSB associated with 4 RACH occasions n16: 16 contention based preambles per SSB |
| msg1-FDM | One | One PRACH transmission occasions FDMed in one time instance. |
| rsrp-ThresholdSSB | RSRP_51 | The actual value of the threshold is -105dBm, as defined in TS 38.331 [2]. |
| ra-ContentionResolutionTimer | sf48 | 48 sub-frames |
| powerRampingStep | dB2 |  |
| preambleReceivedTargetPower | dBm-120 |  |
| preambleTransMax | n6 | Max number of RA preamble transmission performed before declaring a failure is 6 |
| ra-ResponseWindow | sl10 | 10 slots |
| zeroCorrelationZoneConfig | 11 | N-CS configuration, NCS = 23 |
| Backoff Parameter Index | 2 | 20ms, as defined in table 7.2-1 in TS 38.321 [7]. |
| Note: For further information see clause 6.3.2 in TS 38.331 [2]. |  |  |

### A.3.8.3	PRACH configurations in FR2

#### A.3.8.3.1	FR2 PRACH configuration 1

FR2 PRACH configuration 1 in this clause provides the typical PRACH configuration for SSB-based contention based random access in FR2.

Table A.3.8.3.1-1: Parameters for FR2 PRACH configuration 1

| Field | Value | Comment |
| --- | --- | --- |
| prach-ConfigurationIndex | 190 | Preamble Format C2, with 10ms PRACH periodicity, and other detailed configuration defined in table 6.3.3.2-4 in TS 38.211 [6]. |
| msg1-SubcarrierSpacing | Same as UL carrier SCS |  |
| totalNumberOfRA-Preambles | 48 | Total number of preambles used for contention based and contention free random access |
| numberOfRA-PreamblesGroupA | 48 | No group B. |
| prach-RootSequenceIndex | 0 | Logic sequence index = 0, resulting in root sequence = 1. |
| ssb-perRACH-OccasionAndCB-PreamblesPerSSB | oneFourth, n48 | OneFourth: 1 SSB associated with 4 RACH occasions n48: 48 contention based preambles per SSB |
| msg1-FDM | One | One PRACH transmission occasions FDMed in one time instance. |
| rsrp-ThresholdSSB | RSRP_51 | The actual value of the threshold is -105dBm, as defined in TS 38.331 [2]. |
| ra-ContentionResolutionTimer | sf48 | 48 sub-frames |
| powerRampingStep | dB2 |  |
| preambleReceivedTargetPower | dBm-120 |  |
| preambleTransMax | n6 | Max number of RA preamble transmission performed before declaring a failure is 6 |
| ra-ResponseWindow | sl10 | 10 slots |
| zeroCorrelationZoneConfig | 11 | N-CS configuration, NCS = 23 |
| Backoff Parameter Index | 2 | 20 ms, as defined in table 7.2-1 in TS 38.321 [7]. |
| Note: For further information see clause 6.3.2 in TS 38.331 [2]. |  |  |

#### A.3.8.3.2	FR2 PRACH configuration 2

FR2 PRACH configuration 2 in this clause provides the typical PRACH configuration for SSB based non-contention based random access in FR2.

Table A.3.8.3.2-1: Parameters for FR2 PRACH configuration 2

| Field | Value | Comment |
| --- | --- | --- |
| prach-ConfigurationIndex | 190 | Preamble Format C2, with 10ms PRACH periodicity, and other detailed configuration defined in table 6.3.3.2-4 in TS 38.211 [6]. |
| msg1-SubcarrierSpacing | Same as UL carrier SCS |  |
| totalNumberOfRA-Preambles | 48 | Total number of preambles used for contention based and contention free random access |
| numberOfRA-PreamblesGroupA | 48 | No group B. |
| prach-RootSequenceIndex | 0 | Logic sequence index = 0, resulting in root sequence = 1. |
| ssb-perRACH-Occasion | oneFourth | OneFourth: 1 SSB associated with 4 RACH occasions |
| msg1-FDM | One | One PRACH transmission occasions FDMed in one time instance. |
| powerRampingStep | dB2 |  |
| preambleReceivedTargetPower | dBm-120 |  |
| preambleTransMax | n6 | Max number of RA preamble transmission performed before declaring a failure is 6 |
| ra-ResponseWindow | sl10 | 10 slots |
| zeroCorrelationZoneConfig | 11 | N-CS configuration, NCS = 23 |
| Backoff Parameter Index | 2 | 20 ms, as defined in table 7.2-1 in TS 38.321 [7]. |
| ssb-ResourceList | ra-PreambleIndex = 50 | Associated with SSB index 0. UE doesn’t use ssb-ResourceList and BFR-SSB-Resource IEs at the same time. UE doesn’t use this field if is transmitting CFRA to convey BFR. |
| BFR-SSB-Resource | ra-PreambleIndex = 50 | Associated with SSB index 0. UE doesn’t use ssb-ResourceList and BFR-SSB-Resource IEs at the same time. UE uses this field only if is transmitting CFRA to convey BFR |
| ra-ssb-OccasionMaskIndex | 1 | PRACH occasion index 1 is allowed |
| rsrp-ThresholdSSB | RSRP_51 | The actual value of the threshold is -105dBm, as defined in TS 38.331 [2]. |
| Note: For further information see clause 6.3.2 in TS 38.331 [2]. |  |  |

#### A.3.8.3.3	FR2 PRACH configuration 3

FR2 PRACH configuration 3 in this clause provides the typical PRACH configuration for CSI-RS based non-contention based random access in FR2.

| Table A.3.8.3.3-1: Parameters for FR2 PRACH configuration 3Field | Value | Comment |
| --- | --- | --- |
| prach-ConfigurationIndex | 190 | Preamble Format C2, with 10ms PRACH periodicity, and other detailed configuration defined in table 6.3.3.2-4 in TS 38.211 [6]. |
| msg1-SubcarrierSpacing | Same as UL carrier SCS |  |
| totalNumberOfRA-Preambles | 48 | Total number of preambles used for contention based and contention free random acces |
| numberOfRA-PreamblesGroupA | 48 | No group B. |
| prach-RootSequenceIndex | 0 | Logic sequence index = 0, resulting in root sequence = 1. |
| ssb-perRACH-Occasion | oneFourth | OneFourth: 1 SSB associated with 4 RACH occasions |
| msg1-FDM | One | One PRACH transmission occasions FDMed in one time instance. |
| powerRampingStep | dB2 |  |
| preambleReceivedTargetPower | dBm-120 |  |
| preambleTransMax | n6 | Max number of RA preamble transmission performed before declaring a failure is 6 |
| ra-ResponseWindow | sl10 | 10 slots |
| zeroCorrelationZoneConfig | 11 | N-CS configuration, NCS = 23 |
| Backoff Parameter Index | 2 | 20 ms, as defined in table 7.2-1 in TS 38.321 [7]. |
| csirs-ResourceList | ra-PreambleIndex = 50 | Associated with CSI-RS configured |
| ra-OccasionList | 1 | RA occasions allowed corresponding to CSI-RS |
| rsrp-ThresholdCSI-RS | RSRP_51 | The actual value of the threshold is -105dBm, as defined in TS 38.331 [2]. |
| Note: For further information see clause 6.3.2 in TS 38.331 [2]. |  |  |

#### A.3.8.3.4	FR2 PRACH configuration 4

FR2 PRACH configuration 4 in this clause provides the PRACH configuration for CSI-RS based non-contention based random access in FR2 to convey BFR.

Table A.3.8.3.4-1: Parameters for FR2 PRACH configuration 4

| Field | Value | Comment |
| --- | --- | --- |
| prach-ConfigurationIndex | 190 | Preamble Format C2, with 10ms PRACH periodicity, and other detailed configuration defined in table 6.3.3.2-4 in TS 38.211 [6]. |
| msg1-SubcarrierSpacing | Same as UL carrier SCS |  |
| totalNumberOfRA-Preambles | 48 | Total number of preambles used for contention based and contention free random access |
| numberOfRA-PreamblesGroupA | 48 | No group B. |
| prach-RootSequenceIndex | 0 | Logic sequence index = 0, resulting in root sequence = 1. |
| ssb-perRACH-Occasion | oneFourth | OneFourth: 1 SSB associated with 4 RACH occasions |
| msg1-FDM | One | One PRACH transmission occasions FDMed in one time instance. |
| powerRampingStep | dB2 |  |
| preambleReceivedTargetPower | dBm-120 |  |
| preambleTransMax | n200 | Max number of RA preamble transmission performed before declaring a failure is 200. |
| ra-ResponseWindow | sl40 | 40 slots |
| zeroCorrelationZoneConfig | 11 | N-CS configuration, NCS = 23 |
| Backoff Parameter Index | 2 | 20 ms, as defined in table 7.2-1 in TS 38.321 [7]. |
| BFR-CSIRS-Resource | ra-PreambleIndex = 50 | Associated with CSI-RS configured |
| ra-OccasionList | 1 | RA occasions allowed corresponding to CSI-RS |
| rsrp-ThresholdSSB | RSRP_51 | The actual value of the threshold is -105dBm, as defined in TS 38.331 [2]. |
| Note: For further information see clause 6.3.2 in TS 38.331 [2]. |  |  |

#### A.3.8.3.5	FR2 PRACH configuration 5

FR2 PRACH configuration 5 in this clause provides the typical PRACH configuration for LTM early UL synchronization on candidate cell in FR2.

Table A.3.8.3.5-1: Parameters for FR2 PRACH configuration 5

| Field | Value | Comment |
| --- | --- | --- |
| prach-ConfigurationIndex | 190 | Preamble Format C2, with 10ms PRACH periodicity, and other detailed configuration defined in table 6.3.3.2-4 in TS 38.211 [6] |
| msg1-FDM | One | One PRACH transmission occasions FDMed in one time instance. |
| msg1-FrequencyStart | 0 |  |
| zeroCorrelationZoneConfig | 11 | N-CS configuration, NCS = 23 |
| preambleReceivedTargetPower | dBm-120 |  |
| preambleTransMax | n6 |  |
| powerRampingStep | dB2 |  |
| ssb-perRACH-Occasion | oneFourth | OneFourth: 1 SSB associated with 4 RACH occasions |
| prach-RootSequenceIndex | 0 | Logic sequence index = 0, resulting in root sequence = 1. |
| ltm-prach-SubcarrierSpacing | Same as UL carrier SCS |  |
| Note: For further information see clause 6.3.2 in TS 38.331 [2]. |  |  |

#### A.3.8.3.6	FR2 PRACH configuration 6

FR2 PRACH configuration 6 in this clause provides the typical PRACH configuration for SSB-based contention free random access in FR2.

Table A.3.8.3.6-1: Parameters for FR2 PRACH configuration 6

| Field | Value | Comment |
| --- | --- | --- |
| prach-ConfigurationIndex | 190 | Preamble Format C2, with 10ms PRACH periodicity, and other detailed configuration defined in table 6.3.3.2-4 in TS 38.211 [6]. |
| msg1-SubcarrierSpacing | Same as UL carrier SCS |  |
| totalNumberOfRA-Preambles | 48 | Total number of preambles used for contention based and contention free random access |
| numberOfRA-PreamblesGroupA | 48 | No group B. |
| prach-RootSequenceIndex | 0 | Logic sequence index = 0, resulting in root sequence = 1. |
| ssb-perRACH-OccasionAndCB-PreamblesPerSSB | oneFourth, n16 | OneFourth: 1 SSB associated with 4 RACH occasions n16: 16 contention based preambles per SSB |
| msg1-FDM | One | One PRACH transmission occasions FDMed in one time instance. |
| rsrp-ThresholdSSB | RSRP_51 | The actual value of the threshold is -105dBm, as defined in TS 38.331 [2]. |
| ra-ContentionResolutionTimer | sf48 | 48 sub-frames |
| powerRampingStep | dB2 |  |
| preambleReceivedTargetPower | dBm-120 |  |
| preambleTransMax | n6 | Max number of RA preamble transmission performed before declaring a failure is 6 |
| ra-ResponseWindow | sl10 | 10 slots |
| zeroCorrelationZoneConfig | 11 | N-CS configuration, NCS = 23 |
| Backoff Parameter Index | 2 | 20 ms, as defined in table 7.2-1 in TS 38.321 [7]. |
| Note: For further information see clause 6.3.2 in TS 38.331 [2]. |  |  |

## A.3.8A	PRACH configurations under CCA

### A.3.8A.1	Introduction

This clause provides the typical PRACH configurations used for RRM test cases defined in Annex A. To note that for other parameters not listed in this clause, either it can be derived from the set up of each test or it is subjected to RAN5 specifications.

### A.3.8A.2	PRACH configurations in FR1

#### A.3.8A.2.1	FR1 PRACH configuration 1 under CCA

FR1 PRACH configuration 1 under CCA in this clause provides the typical PRACH configuration for SSB-based contention based random access in FR1.

Table A.3.8A.2.1-1: Parameters for FR1 PRACH configuration 1 under CCA

| Field | Value | Comment |
| --- | --- | --- |
| prach-ConfigurationIndex | 102 | 10ms PRACH periodicity, and other detailed configuration defined in table 6.3.3.2-2 and table 6.3.3.2-3 in TS 38.211 [6]. |
| msg1-SubcarrierSpacing | Same as UL carrier SCS |  |
| totalNumberOfRA-Preambles | 48 | Total number of preambles used for contention based and contention free random access |
| numberOfRA-PreamblesGroupA | 48 | No group B. |
| prach-RootSequenceIndex | 0 | Logic sequence index = 0, resulting in root sequence = 1. |
| ssb-perRACH-OccasionAndCB-PreamblesPerSSB | oneFourth, n48 | OneFourth: 1 SSB associated with 4 RACH occasions n48: 48 contention based preambles per SSB |
| msg1-FDM | One | One PRACH transmission occasions FDMed in one time instance. |
| rsrp-ThresholdSSB | RSRP_51 | The actual value of the threshold is -105dBm, as defined in TS 38.331 [2]. |
| ra-ContentionResolutionTimer | sf48 | 48 sub-frames |
| powerRampingStep | dB2 |  |
| preambleReceivedTargetPower | dBm-114 | Increased by 6 dB compared with FR1 PRACH configuration 1 for random access test with UL CCA failures. |
| preambleTransMax | n20 | Max number of RA preamble transmission performed before declaring a failure is 20 to account for CCA failures |
| ra-ResponseWindow | Sl20 | 20 slots |
| zeroCorrelationZoneConfig | 11 | N-CS configuration, NCS = 23 |
| Backoff Parameter Index | 2 | 20ms, as defined in table 7.2-1 in TS 38.321 [7]. |
| Note: For further information see clause 6.3.2 in TS 38.331 [2]. |  |  |

#### A.3.8A.2.2	FR1 PRACH configuration 2 under CCA

FR1 PRACH configuration 2 under CCA in this clause provides the typical PRACH configuration for SSB based non-contention based random access in FR1.

Table A.3.8A.2.2-1: Parameters for FR1 PRACH configuration 2 under CCA

| Field | Value | Comment |
| --- | --- | --- |
| prach-ConfigurationIndex | 102 | 10ms PRACH periodicity, and other detailed configuration defined in table 6.3.3.2-2 and table 6.3.3.2-3 in TS 38.211 [6]. |
| msg1-SubcarrierSpacing | Same as UL carrier SCS |  |
| totalNumberOfRA-Preambles | 48 | Total number of preambles used for contention based and contention free random access |
| numberOfRA-PreamblesGroupA | 48 | No group B. |
| prach-RootSequenceIndex | 0 | Logic sequence index = 0, resulting in root sequence = 1. |
| ssb-perRACH-Occasion | oneFourth | OneFourth: 1 SSB associated with 4 RACH occasions |
| msg1-FDM | One | One PRACH transmission occasions FDMed in one time instance. |
| powerRampingStep | dB2 |  |
| preambleReceivedTargetPower | dBm-114 | Increased by 6 dB compared with FR1 PRACH configuration 2 for random access test with UL CCA failures. |
| preambleTransMax | n20 | Max number of RA preamble transmission performed before declaring a failure is 20 to account for CCA failures |
| ra-ResponseWindow | sl20 | 20 slots |
| zeroCorrelationZoneConfig | 11 | N-CS configuration, NCS = 23 |
| Backoff Parameter Index | 2 | 20ms, as defined in table 7.2-1 in TS 38.321 [7]. |
| ssb-ResourceList | ra-PreambleIndex = 50 | Associated with SSB index 0. UE doesn’t use ssb-ResourceList and BFR-SSB-Resource IEs at the same time. UE doesn’t use this field if is transmitting CFRA to convey BFR. |
| BFR-SSB-Resource | ra-PreambleIndex = 50 | Associated with SSB index 0. UE doesn’t use ssb-ResourceList and BFR-SSB-Resource IEs at the same time. UE uses this field only if is transmitting CFRA to convey BFR |
| ra-ssb-OccasionMaskIndex | 1 | PRACH occasion index 1 is allowed |
| rsrp-ThresholdSSB | RSRP_51 | The actual value of the threshold is -105dBm, as defined in TS 38.331 [2]. |
| Note: For further information see clause 6.3.2 in TS 38.331 [2]. |  |  |

## A.3.9	BWP configurations

### A.3.9.1	Introduction

This clause provides the typical BWP configurations used for RRM test cases defined in Annex A. For downlink BWP, both initial BWP and dedicated BWP configurations are specified in clause A.3.9.2 and for uplink BWP, both initial BWP and dedicated BWP configurations are specified in clause A.3.9.3. To note that for other parameters not listed in this clause, either it can be derived from the set up of each test or it is subjected to RAN5 specifications.

### A.3.9.2	Downlink BWP configurations

#### A.3.9.2.1	Initial BWP

Table A.3.9.2.1-1: Downlink BWP patterns for initial BWP configuration

| BWP Parameters | Unit | Values |  |  |
| --- | --- | --- | --- | --- |
| Reference BWP |  | DLBWP.0.1 | DLBWP.0.2 |  |
| Starting PRB index |  | 0 | RBc Note 1 |  |
| Bandwidth | RB | Same as RF channel defined in each test | same as RMSI CORESET (CORESET #0) defined in each test |  |
| Note 1: RBc is the lowest PRB index to guarantee the BWP including CORESET #0 which is defined in Clause A.3.1.2. |  |  |  |  |

#### A.3.9.2.2	Dedicated BWP

Table A.3.9.2.2-1: Downlink BWP patterns for dedicated BWP configuration

| BWP Parameters | Unit | Values |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference BWP |  | DLBWP.1.1 | DLBWP.1.2 | DLBWP.1.3 | DLBWP.1.4 | DLBWP.1.5 | DLBWP.1.6 | DLBWP.1.7 |
| Starting PRB index |  | 0 | RBb Note 1 | RBa Note 2 | 0 | RBb Note 1 | RBa Note 2 | RBc Note 3 |
| Bandwidth | RB | Same as RF channel defined in each test | 25 for SSB SCS = 15KHz, 51 for SSB SCS = 30KHz, 32 for SSB SCS = 120KHz 48 for SSB SCS = 240KHz | 25 for SSB SCS = 15KHz, 51 for SSB SCS = 30KHz, 32 for SSB SCS = 120KHz 48 for SSB SCS = 240KHz | 24 for SSB SCS = 120KHz 24 for SSB SCS = 240KHz | 24 for SSB SCS = 120KHz 24 for SSB SCS = 240KHz | 24 for SSB SCS = 120KHz 24 for SSB SCS = 240KHz | 25 for SSB SCS = 15KHz, 51 for SSB SCS = 30KHz, 32 for SSB SCS = 120KHz 48 for SSB SCS = 240KHz |
| Note 1: RBb is the lowest PRB index to guarantee the BWP not fully overlapped with SSB PRB index (RBJ, RBJ+1,.…, RBJ+19) which is defined in Clause A.3.10. Note 2: RBa is the lowest PRB index to guarantee the BWP including SSB PRB index (RBJ, RBJ+1,.…, RBJ+19) which is defined in Clause A.3.10. Note 3: RBc is the PRB index having an offset of 6 PRBs with respect to the start of SSB PRB index (RBJ, RBJ+1,.…, RBJ+19) which is defined in Clause A.3.10. |  |  |  |  |  |  |  |  |

### A.3.9.3	Uplink BWP configurations

#### A.3.9.3.1	Initial BWP

Table A.3.9.3.1-1: Uplink BWP patterns for initial BWP configuration

| BWP Parameters | Unit | Values |  |  |
| --- | --- | --- | --- | --- |
| Reference BWP |  | ULBWP.0.1 | ULBWP.0.2 |  |
| Starting PRB index |  | 0 | RBc Note 1 |  |
| Bandwidth | RB | Same as RF channel defined in each test | same as RMSI CORESET (CORESET #0) defined in each test |  |
| Note 1: RBc is same as RBc for DLBWP.0.2 as defined in Table A.3.9.2.1-1. |  |  |  |  |

#### A.3.9.3.2	Dedicated BWP

Table A.3.9.3.2-1: Uplink BWP patterns for dedicated BWP configuration

| BWP Parameters | Unit | Values |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference BWP |  | ULBWP.1.1 | ULBWP.1.2 | ULBWP.1.3 | ULBWP.1.4 | ULBWP.1.5 | ULBWP.1.6 | ULBWP.1.7 |
| Starting PRB index |  | 0 | RBb Note 1 | RBa Note 2 | 0 | RBb Note 1 | RBa Note 2 | RBc Note 3 |
| Bandwidth | RB | Same as RF channel defined in each test | 25 for SSB SCS = 15KHz, 51 for SSB SCS = 30KHz, 32 for SSB SCS = 120KHz 48 for SSB SCS = 240KHz | 25 for SSB SCS = 15KHz, 51 for SSB SCS = 30KHz, 32 for SSB SCS = 120KHz 48 for SSB SCS = 240KHz | 24 for SSB SCS = 120KHz 24 for SSB SCS = 240KHz | 24 for SSB SCS = 120KHz 24 for SSB SCS = 240KHz | 24 for SSB SCS = 120KHz 24 for SSB SCS = 240KHz | 25 for SSB SCS = 15KHz, 51 for SSB SCS = 30KHz, 32 for SSB SCS = 120KHz 48 for SSB SCS = 240KHz |
| Note 1: RBb is same as RBb for DLBWP.1.2 as defined in Table A.3.9.2.2-1. Note 2: RBa is same as RBa for DLBWP.1.3 as defined in Table A.3.9.2.2-1. Note 3: RBc is same as RBc for DLBWP.1.7 as defined in Table A.3.9.2.2-1 |  |  |  |  |  |  |  |  |

## A.3.9A	BWP configurations for RedCap

### A.3.9A.1	Introduction

This clause provides the typical BWP configurations used for RedCap RRM test cases defined in Annex A. For downlink BWP, RedCap dedicated BWP configurations are specified in clause A.3.9A.2 and for uplink BWP, RedCap dedicated BWP configurations are specified in clause A.3.9A.3. To note that for other parameters not listed in this clause, either it can be derived from the set up of each test or it is subjected to RAN5 specifications.

### A.3.9A.2	Downlink BWP configurations

#### A.3.9A.2.1	Dedicated BWP

Table A.3.9A.2.2-1: Downlink BWP patterns for RedCap dedicated BWP configuration

| BWP Parameters | Unit | Values |  |  |
| --- | --- | --- | --- | --- |
| Reference BWP |  | DLBWP.1.1 RedCap | DLBWP.1.2 RedCap | DLBWP.1.3 RedCap |
| Starting PRB index |  | RBa Note 1 | RBb Note 2 | RBc Note 3 |
| Bandwidth | RB | 25 for SSB SCS = 15KHz, 25 for SSB SCS = 30KHz, 32 for SSB SCS = 120KHz 16 for SSB SCS = 240KHz | 25 for SSB SCS = 15KHz, 25 for SSB SCS = 30KHz, 32 for SSB SCS = 120KHz 16 for SSB SCS = 240KHz | 25 for SSB SCS = 15KHz, 25 for SSB SCS = 30KHz, 32 for SSB SCS = 120KHz 16 for SSB SCS = 240KHz |
| Note 1: RBa is the lowest PRB index to guarantee the BWP including SSB PRB index (RBJ, RBJ+1,.…, RBJ+19) which is defined in Clause A.3.10. Note 2: RBb is the lowest PRB index to guarantee the BWP not fully overlapped with SSB PRB index (RBJ, RBJ+1,.…, RBJ+19) which is defined in Clause A.3.10. Note 3: RBc is the lowest PRB index to guarantee the BWP including SSB PRB index (RBJ, RBJ+1,.…, RBJ+19) which is defined in Clause A.3.10. |  |  |  |  |

### A.3.9A.3	Uplink BWP configurations

#### A.3.9A.3.1	Dedicated BWP

Table A.3.9A.3.1-1: Uplink BWP patterns for RedCap dedicated BWP configuration

| BWP Parameters | Unit | Values |  |  |
| --- | --- | --- | --- | --- |
| Reference BWP |  | ULBWP.1.1 RedCap | ULBWP.1.2 RedCap | ULBWP.1.3 RedCap |
| Starting PRB index |  | RBa Note 1 | RBb Note 2 | RBc Note 3 |
| Bandwidth | RB | 25 for SSB SCS = 15KHz, 25 for SSB SCS = 30KHz, 32 for SSB SCS = 120KHz 16 for SSB SCS = 240KHz | 25 for SSB SCS = 15KHz, 25 for SSB SCS = 30KHz, 32 for SSB SCS = 120KHz 16 for SSB SCS = 240KHz | 25 for SSB SCS = 15KHz, 25 for SSB SCS = 30KHz, 32 for SSB SCS = 120KHz 16 for SSB SCS = 240KHz |
| Note 1: RBa is the same as RBa for DLBWP.1.1 RedCap. Note 2: RBb is the same as RBb for DLBWP.1.2 RedCap. Note 3: RBc is the same as RBc for DLBWP.1.3 RedCap. |  |  |  |  |

## A.3.10	SSB Configurations

### A.3.10.1	SSB Configurations for FR1

#### A.3.10.1.1	SSB pattern 1 in FR1: SSB allocation for SSB SCS=15 kHz in 10 MHz

Table A.3.10.1.1-1: SSB.1 FR1: SSB Pattern 1 for SSB SCS=15 kHz in 10 MHz channel

| SSB Parameters | Values |
| --- | --- |
| Channel bandwidth | 10 MHz |
| SSB SCS | 15 kHz |
| SSB periodicity (TSSB) | 20 ms |
| Number of SSBs per SS-burst | 1 |
| SS/PBCH block index | 0 |
| Symbol numbers containing SSB Note 2 | 2-5 |
| Slot numbers containing SSB Note 2 | 0 |
| SFN containing SSB | SFN mod (max(TSSB,10ms)/10ms) = 0 |
| RB numbers containing SSB within channel BW | (RBJ, RBJ+1,.…, RBJ+19)Note 1 |
| Note 1: RBs containing SSB can be configured in any frequency location within the cell bandwidth according to the allowed synchronization raster defined in TS 38.104 [13]. Note 2: These values have been derived from other parameters for information purposes (as per TS 38.213 [3]). They are not settable parameters themselves. |  |

#### A.3.10.1.2	SSB pattern 2 in FR1: SSB allocation for SSB SCS=30 kHz in 40 MHz

Table A.3.10.1.2-1: SSB.2 FR1: SSB Pattern 2 for SSB SCS=30 kHz in 40 MHz channel

| SSB Parameters | Values |
| --- | --- |
| Channel bandwidth | 40 MHz |
| SSB SCS | 30 kHz |
| SSB periodicity (TSSB) | 20 ms |
| Number of SSBs per SS-burst | 1 |
| SS/PBCH block index | 0 |
| Symbol numbers containing SSB Note 3 | 4-7 or 2-5 Note 2 |
| Slot numbers containing SSB Note 3 | 0 |
| SFN containing SSB | SFN mod (max(TSSB,10ms)/10ms) = 0 |
| RB numbers containing SSB within channel BW | (RBJ, RBJ+1,.…, RBJ+19)Note 1 |
| Note 1: RBs containing SSB can be configured in any frequency location within the cell bandwidth according to the allowed synchronization raster defined in TS 38.104 [13]. Note 2: Symbols 4-7 is chosen, if the SSB pattern Case B should be used for the current band as indicated by Table 5.4.3.3-1 of TS 38.104 [13]; Otherwise, symbol 2-5 is chosen. Note 3: These values have been derived from other parameters for information purposes (as per TS 38.213 [3]). They are not settable parameters themselves |  |

#### A.3.10.1.3	SSB pattern 3 in FR1: SSB allocation for SSB SCS=15 kHz in 10 MHz

Table A.3.10.1.3-1: SSB.3 FR1: SSB Pattern 3 for SSB SCS=15 kHz in 10 MHz channel

| SSB Parameters | Values |  |
| --- | --- | --- |
| Channel bandwidth | 10 MHz |  |
| SSB SCS | 15 kHz |  |
| SSB periodicity (TSSB) | 20 ms |  |
| Number of SSBs per SS-burst | 2 |  |
| SS/PBCH block index | 0 | 1 |
| Symbol numbers containing SSB Note 2 | 2-5 | 8-11 |
| Slot numbers containing SSB Note 2 | 0 | 0 |
| SFN containing SSB | SFN mod (max(TSSB,10ms)/10ms) = 0 |  |
| RB numbers containing SSB within channel BW | (RBJ, RBJ+1,.…, RBJ+19)Note 1 |  |
| Note 1: RBs containing SSB can be configured in any frequency location within the cell bandwidth according to the allowed synchronization raster defined in TS 38.104 [13]. Note 2: These values have been derived from other parameters for information purposes (as per TS 38.213 [3]). They are not settable parameters themselves. |  |  |

#### A.3.10.1.4	SSB pattern 4 in FR1: SSB allocation for SSB SCS=30 kHz in 40 MHz

Table A.3.10.1.4-1: SSB.4 FR1: SSB Pattern 4 for SSB SCS=30 kHz in 40 MHz channel

| SSB Parameters | Values |  |
| --- | --- | --- |
| Channel bandwidth | 40 MHz |  |
| SSB SCS | 30 kHz |  |
| SSB periodicity (TSSB) | 20 ms |  |
| Number of SSBs per SS-burst | 2 |  |
| SS/PBCH block index | 0 | 1 |
| Symbol numbers containing SSB Note 3 | 4-7 or 2-5 Note 2 | 8-11 |
| Slot numbers containing SSB Note 3 | 0 | 0 |
| SFN containing SSB | SFN mod (max(TSSB,10ms)/10ms) = 0 |  |
| RB numbers containing SSB within channel BW | (RBJ, RBJ+1,.…, RBJ+19)Note 1 |  |
| Note 1: RBs containing SSB can be configured in any frequency location within the cell bandwidth according to the allowed synchronization raster defined in TS 38.104 [13]. Note 2: Symbols 4-7 is chosen, if the SSB pattern Case B should be used for the current band as indicated by Table 5.4.3.3-1 of TS 38.104 [13]; Otherwise, symbol 2-5 is chosen. Note 3: These values have been derived from other parameters for information purposes (as per TS 38.213 [3]). They are not settable parameters themselves. |  |  |

#### A.3.10.1.5	SSB pattern 5 in FR1: SSB allocation for SSB SCS=15 kHz starting from odd SFN in 10 MHz

Table A.3.10.1.5-1: SSB.5 FR1: SSB Pattern 5 for SSB SCS=15 kHz in 10 MHz channel

| SSB Parameters | Values |
| --- | --- |
| Channel bandwidth | 10 MHz |
| SSB SCS | 15 kHz |
| SSB periodicity (TSSB) | 20 ms |
| Number of SSBs per SS-burst | 1 |
| SS/PBCH block index | 0 |
| Symbol numbers containing SSB Note 2 | 2-5 |
| Slot numbers containing SSB Note 2 | 0 |
| SFN containing SSB | SFN mod (max(TSSB,10ms)/10ms) = 1 |
| RB numbers containing SSB within channel BW | (RBJ, RBJ+1,.…, RBJ+19)Note 1 |
| Note 1: RBs containing SSB can be configured in any frequency location within the cell bandwidth according to the allowed synchronization raster defined in TS 38.104 [13]. Note 2: These values have been derived from other parameters for information purposes (as per TS 38.213 [3]). They are not settable parameters themselves. |  |

#### A.3.10.1.6	SSB pattern 6 in FR1: SSB allocation for SSB SCS=30 kHz starting from odd SFN in 40 MHz

Table A.3.10.1.6-1: SSB.6 FR1: SSB Pattern 6 for SSB SCS=30 kHz in 40 MHz channel

| SSB Parameters | Values |
| --- | --- |
| Channel bandwidth | 40 MHz |
| SSB SCS | 30 kHz |
| SSB periodicity (TSSB) | 20 ms |
| Number of SSBs per SS-burst | 1 |
| SS/PBCH block index | 0 |
| Symbol numbers containing SSB Note 3 | 4-7 or 2-5 Note 2 |
| Slot numbers containing SSB Note 3 | 0 |
| SFN containing SSB | SFN mod (max(TSSB,10ms)/10ms) = 1 |
| RB numbers containing SSB within channel BW | (RBJ, RBJ+1,.…, RBJ+19)Note 1 |
| Note 1: RBs containing SSB can be configured in any frequency location within the cell bandwidth according to the allowed synchronization raster defined in TS 38.104 [13]. Note 2: Symbols 4-7 is chosen, if the SSB pattern Case B should be used for the current band as indicated by Table 5.4.3.3-1 of TS 38.104 [13]; Otherwise, symbol 2-5 is chosen. Note 3: These values have been derived from other parameters for information purposes (as per TS 38.213 [3]). They are not settable parameters themselves. |  |

#### A.3.10.1.7	SSB pattern 7 in FR1: SSB allocation for SSB SCS=15 kHz in 10 MHz

Table A.3.10.1.7-1: SSB.7 FR1: SSB Pattern 7 for SSB SCS=15 kHz in 10 MHz channel

| SSB Parameters | Values |  |
| --- | --- | --- |
| Channel bandwidth | 10 MHz |  |
| SSB SCS | 15 kHz |  |
| SSB periodicity (TSSB) | 20 ms |  |
| Number of SSBs per SS-burst | 2 |  |
| SS/PBCH block index | 2 | 3 |
| Symbol numbers containing SSB Note 2 | 2-5 | 8-11 |
| Slot numbers containing SSB Note 2 | 1 | 1 |
| SFN containing SSB | SFN mod (max(TSSB,10ms)/10ms) = 0 |  |
| RB numbers containing SSB within channel BW | (RBJ, RBJ+1,.…, RBJ+19)Note 1 |  |
| Note 1: RBs containing SSB can be configured in any frequency location within the cell bandwidth according to the allowed synchronization raster defined in TS 38.104 [13]. Note 2: These values have been derived from other parameters for information purposes (as per TS 38.213 [3]). They are not settable parameters themselves. |  |  |

#### A.3.10.1.8	SSB pattern 8 in FR1: SSB allocation for SSB SCS=30 kHz in 40 MHz

Table A.3.10.1.8-1: SSB.8 FR1: SSB Pattern 8 for SSB SCS=30 kHz in 40 MHz channel

| SSB Parameters | Values |  |
| --- | --- | --- |
| Channel bandwidth | 40 MHz |  |
| SSB SCS | 30 kHz |  |
| SSB periodicity (TSSB) | 20 ms |  |
| Number of SSBs per SS-burst | 2 |  |
| SS/PBCH block index | 2 | 3 |
| Symbol numbers containing SSB Note 3 | 4-7 or 2-5 Note 2 | 8-11 |
| Slot numbers containing SSB Note 3 | 1 | 1 |
| SFN containing SSB | SFN mod (max(TSSB,10ms)/10ms) = 0 |  |
| RB numbers containing SSB within channel BW | (RBJ, RBJ+1,.…, RBJ+19)Note 1 |  |
| Note 1: RBs containing SSB can be configured in any frequency location within the cell bandwidth according to the allowed synchronization raster defined in TS 38.104 [13]. Note 2: Symbols 4-7 is chosen, if the SSB pattern Case B should be used for the current band as indicated by Table 5.4.3.3-1 of TS 38.104 [13]; Otherwise, symbol 2-5 is chosen. Note 3: These values have been derived from other parameters for information purposes (as per TS 38.213 [3]). They are not settable parameters themselves. |  |  |

#### A.3.10.1.9	SSB pattern 9 in FR1: SSB allocation for SSB SCS=15 kHz in 10 MHz

Table A.3.10.1.9-1: SSB.9 FR1: SSB Pattern 9 for SSB SCS=15 kHz in 10 MHz channel

| SSB Parameters | Values |
| --- | --- |
| Channel bandwidth | 10 MHz |
| SSB SCS | 15 kHz |
| SSB periodicity (TSSB) | 80 ms |
| Number of SSBs per SS-burst | 1 |
| SS/PBCH block index | 0 |
| Symbol numbers containing SSB Note 2 | 2-5 |
| Slot numbers containing SSB Note 2 | 0 |
| SFN containing SSB | SFN mod (max(TSSB,10ms)/10ms) = 0 |
| RB numbers containing SSB within channel BW | (RBJ, RBJ+1,.…, RBJ+19)Note 1 |
| Note 1: RBs containing SSB can be configured in any frequency location within the associated bandwidth part except the RBs for allowed synchronization raster defined in TS 38.104 [13]. Note 2: These values have been derived from other parameters for information purposes (as per TS 38.213 [3]). They are not settable parameters themselves. |  |

#### A.3.10.1.10	SSB pattern 10 in FR1: SSB allocation for SSB SCS=30 kHz in 40 MHz

Table A.3.10.1.10-1: SSB.10 FR1: SSB Pattern 10 for SSB SCS=30 kHz in 40 MHz channel

| SSB Parameters | Values |
| --- | --- |
| Channel bandwidth | 40 MHz |
| SSB SCS | 30 kHz |
| SSB periodicity (TSSB) | 80 ms |
| Number of SSBs per SS-burst | 1 |
| SS/PBCH block index | 0 |
| Symbol numbers containing SSB Note 3 | 4-7 or 2-5 Note 2 |
| Slot numbers containing SSB Note 3 | 0 |
| SFN containing SSB | SFN mod (max(TSSB,10ms)/10ms) = 0 |
| RB numbers containing SSB within channel BW | (RBJ, RBJ+1,.…, RBJ+19)Note 1 |
| Note 1: RBs containing SSB can be configured in any frequency location within the associated bandwidth part except the RBs for allowed synchronization raster defined in TS 38.104 [13]. Note 2: Symbols 4-7 is chosen, if the SSB pattern Case B should be used for the current band as indicated by Table 5.4.3.3-1 of TS 38.104 [13]; Otherwise, symbol 2-5 is chosen. Note 3: These values have been derived from other parameters for information purposes (as per TS 38.213 [3]). They are not settable parameters themselves |  |

#### A.3.10.1.11	SSB pattern 11 in FR1: SSB allocation for SSB SCS=15 kHz in 10 MHz

Table A.3.10.1.11-1: SSB.11 FR1: SSB Pattern 11 for SSB SCS=15 kHz in 10 MHz channel

| SSB Parameters | Values |  |
| --- | --- | --- |
| Channel bandwidth | 10 MHz |  |
| SSB SCS | 15 kHz |  |
| SSB periodicity (TSSB) | 80 ms |  |
| Number of SSBs per SS-burst | 2 |  |
| SS/PBCH block index | 0 | 1 |
| Symbol numbers containing SSB Note 2 | 2-5 | 8-11 |
| Slot numbers containing SSB Note 2 | 0 | 0 |
| SFN containing SSB | SFN mod (max(TSSB,10ms)/10ms) = 0 |  |
| RB numbers containing SSB within channel BW | (RBJ, RBJ+1,.…, RBJ+19)Note 1 |  |
| Note 1: RBs containing SSB can be configured in any frequency location within the associated bandwidth part except the RBs for allowed synchronization raster defined in TS 38.104 [13]. Note 2: These values have been derived from other parameters for information purposes (as per TS 38.213 [3]). They are not settable parameters themselves. |  |  |

#### A.3.10.1.12	SSB pattern 12 in FR1: SSB allocation for SSB SCS=30 kHz in 40 MHz

Table A.3.10.1.12-1: SSB.12 FR1: SSB Pattern 12 for SSB SCS=30 kHz in 20 MHz channel

| SSB Parameters | Values |  |
| --- | --- | --- |
| Channel bandwidth | 20 MHz |  |
| SSB SCS | 30 kHz |  |
| SSB periodicity (TSSB) | 80 ms |  |
| Number of SSBs per SS-burst | 2 |  |
| SS/PBCH block index | 0 | 1 |
| Symbol numbers containing SSB Note 3 | 4-7 or 2-5 Note 2 | 8-11 |
| Slot numbers containing SSB Note 3 | 0 | 0 |
| SFN containing SSB | SFN mod (max(TSSB,10ms)/10ms) = 0 |  |
| RB numbers containing SSB within channel BW | (RBJ, RBJ+1,.…, RBJ+19)Note 1 |  |
| Note 1: RBs containing SSB can be configured in any frequency location within the associated bandwidth part except the RBs for allowed synchronization raster defined in TS 38.104 [13]. Note 2: Symbols 4-7 is chosen, if the SSB pattern Case B should be used for the current band as indicated by Table 5.4.3.3-1 of TS 38.104 [13]; Otherwise, symbol 2-5 is chosen. Note 3: These values have been derived from other parameters for information purposes (as per TS 38.213 [3]). They are not settable parameters themselves. |  |  |

#### A.3.10.1.13	SSB pattern 13 in FR1: SSB allocation for SSB SCS=15 kHz in 3 MHz

Table A.3.10.1.13-1: SSB.13 FR1: SSB Pattern 1 for SSB SCS=15 kHz in 3 MHz channel

| SSB Parameters | Values |
| --- | --- |
| Channel bandwidth | 3MHz |
| SSB SCS | 15 kHz |
| SSB periodicity (TSSB) | 20 ms |
| Number of SSBs per SS-burst | 1 |
| SS/PBCH block index | 0 |
| Symbol numbers containing SSB Note 2 | 2-5 |
| Slot numbers containing SSB Note 2 | 0 |
| SFN containing SSB | SFN mod (max(TSSB,10ms)/10ms) = 0 |
| RB numbers containing SSB within channel BW | (RBJ, RBJ+1,.…, RBJ+11)Note 1 |
| Note 1: RBs containing SSB can be configured in any frequency location within the cell bandwidth according to the allowed synchronization raster defined in TS 38.104 [13]. Note 2: These values have been derived from other parameters for information purposes (as per TS 38.213 [3]). They are not settable parameters themselves. |  |

### A.3.10.2	SSB Configurations for FR2

#### A.3.10.2.1	SSB pattern 1 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz

Table A.3.10.2.1-1: SSB.1 FR2: SSB Pattern 1 for SSB SCS = 120 kHz in 100 MHz channel with 2 SSBs per SS-burst

| SSB Parameters | Values |  |
| --- | --- | --- |
| Channel bandwidth | 100 MHz |  |
| SSB SCS | 120 kHz |  |
| SSB periodicity (TSSB) | 20 ms |  |
| Number of SSBs per SS-burst | 2 |  |
| SS/PBCH block index | 0 | 1 |
| Symbol numbers containing SSBs Note 2 | 4-7 | 8-11 |
| Slot numbers containing SSB Note 2 | 0 | 0 |
| SFN containing SSB | SFN mod (max(TSSB,10ms)/10ms) = 0 |  |
| RB numbers containing SSBs within channel BW | (RBJ, RBJ+1,.…, RBJ+19)Note 1 |  |
| Note 1: RBs containing SSB can be configured in any frequency location within the cell bandwidth according to the allowed synchronization raster defined in TS 38.104 [13]. Note 2: These values have been derived from other parameters for information purposes (as per TS 38.213 [3]). They are not settable parameters themselves. |  |  |

#### A.3.10.2.2	SSB pattern 2 in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz

Table A.3.10.2.2-1: SSB.2 FR2: SSB Pattern 2 for SSB SCS = 240 kHz in 100 MHz channel with 2 SSBs per SS-burst

| SSB Parameters | Values |  |
| --- | --- | --- |
| Channel bandwidth | 100 MHz |  |
| SSB SCS | 240 kHz |  |
| SSB periodicity (TSSB) | 20 ms |  |
| Number of SSBs per SS-burst | 2 |  |
| SS/PBCH block index | 0 | 1 |
| Symbol numbers containing SSBs Note 2 | 8-11 | 12-13, 0-1 |
| Slot numbers containing SSB Note 2 | 0 | 0, 1 |
| SFN containing SSB | SFN mod (max(TSSB,10ms)/10ms) = 0 |  |
| RB numbers containing SSBs within channel BW | (RBJ, RBJ+1,.…, RBJ+39)Note 1 |  |
| Note 1: RBs containing SSB can be configured in any frequency location within the cell bandwidth according to the allowed synchronization raster defined in TS 38.104 [13]. Note 2: These values have been derived from other parameters for information purposes (as per TS 38.213 [3]). They are not settable parameters themselves. |  |  |

#### A.3.10.2.3	SSB pattern 3 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz

Table A.3.10.2.3-1: SSB.3 FR2: SSB Pattern 3 for SSB SCS = 120 kHz in 100 MHz channel with 1 SSB per SS-burst

| SSB Parameters | Values |
| --- | --- |
| Channel bandwidth | 100 MHz |
| SSB SCS | 120 kHz |
| SSB periodicity (TSSB) | 20 ms |
| Number of SSBs per SS-burst | 1 |
| SS/PBCH block index | 0 |
| Symbol numbers containing SSBs Note 2 | 4-7 |
| Slot numbers containing SSB Note 2 | 0 |
| SFN containing SSB | SFN mod (max(TSSB,10ms)/10ms) = 0 |
| RB numbers containing SSBs within channel BW | (RBJ, RBJ+1,.…, RBJ+19)Note 1 |
| Note 1: RBs containing SSB can be configured in any frequency location within the cell bandwidth according to the allowed synchronization raster defined in TS 38.104 [13]. Note 2: These values have been derived from other parameters for information purposes (as per TS 38.213 [3]). They are not settable parameters themselves. |  |

#### A.3.10.2.4	SSB pattern 4 in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz

Table A.3.10.2.4-1: SSB.4 FR2: SSB Pattern 4 for SSB SCS = 240 kHz in 100 MHz channel with 1 SSB per SS-burst

| SSB Parameters | Values |
| --- | --- |
| Channel bandwidth | 100 MHz |
| SSB SCS | 240 kHz |
| SSB periodicity (TSSB) | 20 ms |
| Number of SSBs per SS-burst | 1 |
| SS/PBCH block index | 0 |
| Symbol numbers containing SSBs Note 2 | 8-11 |
| Slot numbers containing SSB Note 2 | 0 |
| SFN containing SSB | SFN mod (max(TSSB,10ms)/10ms) = 0 |
| RB numbers containing SSBs within channel BW | (RBJ, RBJ+1,.…, RBJ+39)Note 1 |
| Note 1: RBs containing SSB can be configured in any frequency location within the cell bandwidth according to the allowed synchronization raster defined in TS 38.104 [13]. Note 2: These values have been derived from other parameters for information purposes (as per TS 38.213 [3]). They are not settable parameters themselves. |  |

#### A.3.10.2.5	SSB pattern 5 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz

Table A.3.10.2.5-1: SSB.5 FR2: SSB Pattern 5 for SSB SCS = 120 kHz in 100 MHz channel with 2 SSBs per SS-burst

| SSB Parameters | Values |  |
| --- | --- | --- |
| Channel bandwidth | 100 MHz |  |
| SSB SCS | 120 kHz |  |
| SSB periodicity (TSSB) | 20 ms |  |
| Number of SSBs per SS-burst | 2 |  |
| SS/PBCH block index | 2 | 3 |
| Symbol numbers containing SSBs Note 2 | 2-5 | 6-9 |
| Slot numbers containing SSB Note 2 | 1 | 1 |
| SFN containing SSB | SFN mod (max(TSSB,10ms)/10ms) = 0 |  |
| RB numbers containing SSBs within channel BW | (RBJ, RBJ+1,.…, RBJ+19)Note 1 |  |
| Note 1: RBs containing SSB can be configured in any frequency location within the cell bandwidth according to the allowed synchronization raster defined in TS 38.104 [13]. Note 2: These values have been derived from other parameters for information purposes (as per TS 38.213 [3]). They are not settable parameters themselves. |  |  |

#### A.3.10.2.6	SSB pattern 6 in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz

Table A.3.10.2.6-1: SSB.6 FR2: SSB Pattern 6 for SSB SCS = 240 kHz in 100 MHz channel with 2 SSBs per SS-burst

| SSB Parameters | Values |  |
| --- | --- | --- |
| Channel bandwidth | 100 MHz |  |
| SSB SCS | 240 kHz |  |
| SSB periodicity (TSSB) | 20 ms |  |
| Number of SSBs per SS-burst | 2 |  |
| SS/PBCH block index | 2 | 3 |
| Symbol numbers containing SSBs Note 2 | 2-5 | 6-9 |
| Slot numbers containing SSB Note 2 | 1 | 1 |
| SFN containing SSB | SFN mod (max(TSSB,10ms)/10ms) = 0 |  |
| RB numbers containing SSBs within channel BW | (RBJ, RBJ+1,.…, RBJ+39)Note 1 |  |
| Note 1: RBs containing SSB can be configured in any frequency location within the cell bandwidth according to the allowed synchronization raster defined in TS 38.104 [13]. Note 2: These values have been derived from other parameters for information purposes (as per TS 38.213 [3]). They are not settable parameters themselves. |  |  |

#### A.3.10.2.7	SSB pattern 7 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz

Table A.3.10.2.7-1: SSB.7 FR2: SSB Pattern 7 for SSB SCS = 120 kHz in 100 MHz channel with 1 SSB per SS-burst

| SSB Parameters | Values |
| --- | --- |
| Channel bandwidth | 100 MHz |
| SSB SCS | 120 kHz |
| SSB periodicity (TSSB) | 20 ms |
| Number of SSBs per SS-burst | 1 |
| SS/PBCH block index | 1 |
| Symbol numbers containing SSBs Note 2 | 8-11 |
| Slot numbers containing SSB Note 2 | 0 |
| SFN containing SSB | SFN mod (max(TSSB,10ms)/10ms) = 0 |
| RB numbers containing SSBs within channel BW | (RBJ, RBJ+1,.…, RBJ+19)Note 1 |
| Note 1: RBs containing SSB can be configured in any frequency location within the cell bandwidth according to the allowed synchronization raster defined in TS 38.104 [13]. Note 2: These values have been derived from other parameters for information purposes (as per TS 38.213 [3]). They are not settable parameters themselves. |  |

#### A.3.10.2.8	SSB pattern 8 in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz

Table A.3.10.2.8-1: SSB.8 FR2: SSB Pattern 8 for SSB SCS = 240 kHz in 100 MHz channel with 1 SSB per SS-burst

| SSB Parameters | Values |  |
| --- | --- | --- |
| Channel bandwidth | 100 MHz |  |
| SSB SCS | 240 kHz |  |
| SSB periodicity (TSSB) | 20 ms |  |
| Number of SSBs per SS-burst | 1 |  |
| SS/PBCH block index | 1 |  |
| Symbol numbers containing SSBs Note 2 | 12-13 | 0-1 |
| Slot numbers containing SSB Note 2 | 0 | 1 |
| SFN containing SSB | SFN mod (max(TSSB,10ms)/10ms) = 0 |  |
| RB numbers containing SSBs within channel BW | (RBJ, RBJ+1,.…, RBJ+39)Note 1 |  |
| Note 1: RBs containing SSB can be configured in any frequency location within the cell bandwidth according to the allowed synchronization raster defined in TS 38.104 [13]. Note 2: These values have been derived from other parameters for information purposes (as per TS 38.213 [3]). They are not settable parameters themselves. |  |  |

#### A.3.10.2.9	SSB pattern 9 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz

Table A.3.10.2.9-1: SSB.9 FR2: SSB Pattern 9 for SSB SCS = 120 kHz in 100 MHz channel with 2 SSBs per SS-burst

| SSB Parameters | Values |  |
| --- | --- | --- |
| Channel bandwidth | 100 MHz |  |
| SSB SCS | 120 kHz |  |
| SSB periodicity (TSSB) | 20 ms |  |
| Number of SSBs per SS-burst | 2 |  |
| SS/PBCH block index | 2 | 3 |
| Symbol numbers containing SSBs Note 2 | 2-5 | 6-9 |
| Slot numbers containing SSB Note 2 | 1 | 1 |
| SFN containing SSB | SFN mod (max(TSSB,10ms)/10ms) = 0 |  |
| RB numbers containing SSBs within channel BW | (RBJ, RBJ+1,.…, RBJ+19)Note 1 |  |
| Note 1: RBs containing SSB can be configured in any frequency location within the cell bandwidth according to the allowed synchronization raster defined in TS 38.104 [13]. Note 2: These values have been derived from other parameters for information purposes (as per TS 38.213 [3]). They are not settable parameters themselves. |  |  |

#### A.3.10.2.10	SSB pattern 10 in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz

Table A.3.10.2.10-1: SSB.10 FR2: SSB Pattern 10 for SSB SCS = 240 kHz in 100 MHz channel with 2 SSBs per SS-burst

| SSB Parameters | Values |  |
| --- | --- | --- |
| Channel bandwidth | 100 MHz |  |
| SSB SCS | 240 kHz |  |
| SSB periodicity (TSSB) | 20 ms |  |
| Number of SSBs per SS-burst | 2 |  |
| SS/PBCH block index | 2 | 3 |
| Symbol numbers containing SSBs Note 2 | 4-7 | 8-11 |
| Slot numbers containing SSB Note 2 | 1 | 1 |
| SFN containing SSB | SFN mod (max(TSSB,10ms)/10ms) = 0 |  |
| RB numbers containing SSBs within channel BW | (RBJ, RBJ+1,.…, RBJ+39)Note 1 |  |
| Note 1: RBs containing SSB can be configured in any frequency location within the cell bandwidth according to the allowed synchronization raster defined in TS 38.104 [13]. Note 2: These values have been derived from other parameters for information purposes (as per TS 38.213 [3]). They are not settable parameters themselves. |  |  |

#### A.3.10.2.11	SSB pattern 11 in FR2: SSB allocation for SSB SCS=480 kHz in 400 MHz

Table A.3.10.2.11-1: SSB.11 FR2: SSB Pattern 9 for SSB SCS = 480 kHz in 400 MHz channel with 2 SSBs per SS-burst

| SSB Parameters | Values |  |
| --- | --- | --- |
| Channel bandwidth | 400 MHz |  |
| SSB SCS | 480 kHz |  |
| SSB periodicity (TSSB) | 20 ms |  |
| Number of SSBs per SS-burst | 2 |  |
| SS/PBCH block index | 0 | 1 |
| Symbol numbers containing SSBs Note 2 | 2-5 | 9-12 |
| Slot numbers containing SSB Note 2 | 0 | 0 |
| SFN containing SSB | SFN mod (max(TSSB,10ms)/10ms) = 0 |  |
| RB numbers containing SSBs within channel BW | (RBJ, RBJ+1,.…, RBJ+19)Note 1 |  |
| Note 1: RBs containing SSB can be configured in any frequency location within the cell bandwidth according to the allowed synchronization raster defined in TS 38.104 [13]. Note 2: These values have been derived from other parameters for information purposes (as per TS 38.213 [3]). They are not settable parameters themselves. |  |  |

#### A.3.10.2.12	SSB pattern 12 in FR2: SSB allocation for SSB SCS=960 kHz in 400 MHz

Table A.3.10.2.12-1: SSB.12 FR2: SSB Pattern 10 for SSB SCS = 960 kHz in 400 MHz channel with 2 SSBs per SS-burst

| SSB Parameters | Values |  |
| --- | --- | --- |
| Channel bandwidth | 400 MHz |  |
| SSB SCS | 960 kHz |  |
| SSB periodicity (TSSB) | 20 ms |  |
| Number of SSBs per SS-burst | 2 |  |
| SS/PBCH block index | 0 | 1 |
| Symbol numbers containing SSBs Note 2 | 2-5 | 9-12 |
| Slot numbers containing SSB Note 2 | 0 | 0 |
| SFN containing SSB | SFN mod (max(TSSB,10ms)/10ms) = 0 |  |
| RB numbers containing SSBs within channel BW | (RBJ, RBJ+1,.…, RBJ+39)Note 1 |  |
| Note 1: RBs containing SSB can be configured in any frequency location within the cell bandwidth according to the allowed synchronization raster defined in TS 38.104 [13]. Note 2: These values have been derived from other parameters for information purposes (as per TS 38.213 [3]). They are not settable parameters themselves. |  |  |

#### A.3.10.2.13	SSB pattern 13 in FR2: SSB allocation for SSB SCS=480 kHz in 400 MHz

Table A.3.10.2.13-1: SSB.13 FR2: SSB Pattern 11 for SSB SCS = 480 kHz in 400 MHz channel with 1 SSB per SS-burst

| SSB Parameters | Values |
| --- | --- |
| Channel bandwidth | 400 MHz |
| SSB SCS | 480 kHz |
| SSB periodicity (TSSB) | 20 ms |
| Number of SSBs per SS-burst | 1 |
| SS/PBCH block index | 0 |
| Symbol numbers containing SSBs Note 2 | 2-5 |
| Slot numbers containing SSB Note 2 | 0 |
| SFN containing SSB | SFN mod (max(TSSB,10ms)/10ms) = 0 |
| RB numbers containing SSBs within channel BW | (RBJ, RBJ+1,.…, RBJ+19)Note 1 |
| Note 1: RBs containing SSB can be configured in any frequency location within the cell bandwidth according to the allowed synchronization raster defined in TS 38.104 [13]. Note 2: These values have been derived from other parameters for information purposes (as per TS 38.213 [3]). They are not settable parameters themselves. |  |

#### A.3.10.2.14	SSB pattern 14 in FR2: SSB allocation for SSB SCS=960 kHz in 400 MHz

Table A.3.10.2.14-1: SSB.14 FR2: SSB Pattern 12 for SSB SCS = 960 kHz in 400 MHz channel with 1 SSB per SS-burst

| SSB Parameters | Values |
| --- | --- |
| Channel bandwidth | 400 MHz |
| SSB SCS | 960 kHz |
| SSB periodicity (TSSB) | 20 ms |
| Number of SSBs per SS-burst | 1 |
| SS/PBCH block index | 0 |
| Symbol numbers containing SSBs Note 2 | 2-5 |
| Slot numbers containing SSB Note 2 | 0 |
| SFN containing SSB | SFN mod (max(TSSB,10ms)/10ms) = 0 |
| RB numbers containing SSBs within channel BW | (RBJ, RBJ+1,.…, RBJ+39)Note 1 |
| Note 1: RBs containing SSB can be configured in any frequency location within the cell bandwidth according to the allowed synchronization raster defined in TS 38.104 [13]. Note 2: These values have been derived from other parameters for information purposes (as per TS 38.213 [3]). They are not settable parameters themselves. |  |

#### A.3.10.2.15	SSB pattern 15 in FR2: SSB allocation for SSB SCS=480 kHz in 400 MHz

Table A.3.10.2.15-1: SSB.15 FR2: SSB Pattern 13 for SSB SCS = 480 kHz in 400 MHz channel with 2 SSBs per SS-burst

| SSB Parameters | Values |  |
| --- | --- | --- |
| Channel bandwidth | 400 MHz |  |
| SSB SCS | 480 kHz |  |
| SSB periodicity (TSSB) | 20 ms |  |
| Number of SSBs per SS-burst | 2 |  |
| SS/PBCH block index | 2 | 3 |
| Symbol numbers containing SSBs Note 2 | 2-5 | 9-12 |
| Slot numbers containing SSB Note 2 | 1 | 1 |
| SFN containing SSB | SFN mod (max(TSSB,10ms)/10ms) = 0 |  |
| RB numbers containing SSBs within channel BW | (RBJ, RBJ+1,.…, RBJ+19)Note 1 |  |
| Note 1: RBs containing SSB can be configured in any frequency location within the cell bandwidth according to the allowed synchronization raster defined in TS 38.104 [13]. Note 2: These values have been derived from other parameters for information purposes (as per TS 38.213 [3]). They are not settable parameters themselves. |  |  |

#### A.3.10.2.16	SSB pattern 16 in FR2: SSB allocation for SSB SCS=960 kHz in 400 MHz

Table A.3.10.2.16-1: SSB.16 FR2: SSB Pattern 14 for SSB SCS = 960 kHz in 400 MHz channel with 2 SSBs per SS-burst

| SSB Parameters | Values |  |
| --- | --- | --- |
| Channel bandwidth | 400 MHz |  |
| SSB SCS | 960 kHz |  |
| SSB periodicity (TSSB) | 20 ms |  |
| Number of SSBs per SS-burst | 2 |  |
| SS/PBCH block index | 2 | 3 |
| Symbol numbers containing SSBs Note 2 | 2-5 | 9-12 |
| Slot numbers containing SSB Note 2 | 1 | 1 |
| SFN containing SSB | SFN mod (max(TSSB,10ms)/10ms) = 0 |  |
| RB numbers containing SSBs within channel BW | (RBJ, RBJ+1,.…, RBJ+39)Note 1 |  |
| Note 1: RBs containing SSB can be configured in any frequency location within the cell bandwidth according to the allowed synchronization raster defined in TS 38.104 [13]. Note 2: These values have been derived from other parameters for information purposes (as per TS 38.213 [3]). They are not settable parameters themselves. |  |  |

#### A.3.10.2.17	SSB pattern 17 in FR2: SSB allocation for SSB SCS=480 kHz in 400 MHz

Table A.3.10.2.17-1: SSB.17 FR2: SSB Pattern 15 for SSB SCS = 480 kHz in 400 MHz channel with 1 SSB per SS-burst

| SSB Parameters | Values |
| --- | --- |
| Channel bandwidth | 400 MHz |
| SSB SCS | 480 kHz |
| SSB periodicity (TSSB) | 20 ms |
| Number of SSBs per SS-burst | 1 |
| SS/PBCH block index | 1 |
| Symbol numbers containing SSBs Note 2 | 9-12 |
| Slot numbers containing SSB Note 2 | 0 |
| SFN containing SSB | SFN mod (max(TSSB,10ms)/10ms) = 0 |
| RB numbers containing SSBs within channel BW | (RBJ, RBJ+1,.…, RBJ+19)Note 1 |
| Note 1: RBs containing SSB can be configured in any frequency location within the cell bandwidth according to the allowed synchronization raster defined in TS 38.104 [13]. Note 2: These values have been derived from other parameters for information purposes (as per TS 38.213 [3]). They are not settable parameters themselves. |  |

#### A.3.10.2.18	SSB pattern 18 in FR2: SSB allocation for SSB SCS=960 kHz in 400 MHz

Table A.3.10.2.18-1: SSB.18 FR2: SSB Pattern 16 for SSB SCS = 960 kHz in 400 MHz channel with 1 SSB per SS-burst

| SSB Parameters | Values |
| --- | --- |
| Channel bandwidth | 400 MHz |
| SSB SCS | 960 kHz |
| SSB periodicity (TSSB) | 20 ms |
| Number of SSBs per SS-burst | 1 |
| SS/PBCH block index | 1 |
| Symbol numbers containing SSBs Note 2 | 9-12 |
| Slot numbers containing SSB Note 2 | 0 |
| SFN containing SSB | SFN mod (max(TSSB,10ms)/10ms) = 0 |
| RB numbers containing SSBs within channel BW | (RBJ, RBJ+1,.…, RBJ+39)Note 1 |
| Note 1: RBs containing SSB can be configured in any frequency location within the cell bandwidth according to the allowed synchronization raster defined in TS 38.104 [13]. Note 2: These values have been derived from other parameters for information purposes (as per TS 38.213 [3]). They are not settable parameters themselves. |  |

#### A.3.10.2.19	SSB pattern 19 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz

Table A.3.10B.2.2-1: SSB.19 FR2: SSB Pattern 17 for SSB SCS = 120 kHz in 100 MHz channel with 1 SSB per SS-burst

| SSB Parameters | Values |
| --- | --- |
| Channel bandwidth | 100 MHz |
| SSB SCS | 120 kHz |
| SSB periodicity (TSSB) | 40 ms |
| Number of SSBs per SS-burst | 1 |
| SS/PBCH block index | 0 |
| Symbol numbers containing SSBs Note 2 | 4-7 |
| Slot numbers containing SSB Note 2 | 0 |
| SFN containing SSB | SFN mod (max(TSSB,10ms)/10ms) = 0 |
| RB numbers containing SSBs within channel BW | (RBJ, RBJ+1,.…, RBJ+19)Note 1 |
| Note 1: RBs containing SSB can be configured in any frequency location within the cell bandwidth according to the RBs for allowed synchronization raster defined in TS 38.104 [13]. Note 2: These values have been derived from other parameters for information purposes (as per TS 38.213 [3]). They are not settable parameters themselves. |  |

#### A.3.10.2.20	SSB pattern 20 in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz

Table A.3.10.2.20-1: SSB.20 FR2: SSB Pattern 18 for SSB SCS = 240 kHz in 100 MHz channel with 1 SSB per SS-burst

| SSB Parameters | Values |
| --- | --- |
| Channel bandwidth | 100 MHz |
| SSB SCS | 240 kHz |
| SSB periodicity (TSSB) | 40 ms |
| Number of SSBs per SS-burst | 1 |
| SS/PBCH block index | 0 |
| Symbol numbers containing SSBs Note 2 | 8-11 |
| Slot numbers containing SSB Note 2 | 0 |
| SFN containing SSB | SFN mod (max(TSSB,10ms)/10ms) = 0 |
| RB numbers containing SSBs within channel BW | (RBJ, RBJ+1,.…, RBJ+19)Note 1 |
| Note 1: RBs containing SSB can be configured in any frequency location within the cell bandwidth according to the RBs for allowed synchronization raster defined in TS 38.104 [13]. Note 2: These values have been derived from other parameters for information purposes (as per TS 38.213 [3]). They are not settable parameters themselves. |  |

#### A.3.10.2.21	SSB pattern 21 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz

Table A.3.10.2.21-1: SSB.21 FR2: SSB Pattern 19 for SSB SCS = 120 kHz in 100 MHz channel with 2 SSBs per SS-burst

| SSB Parameters | Values |  |
| --- | --- | --- |
| Channel bandwidth | 100 MHz |  |
| SSB SCS | 120 kHz |  |
| SSB periodicity (TSSB) | 80 ms |  |
| Number of SSBs per SS-burst | 2 |  |
| SS/PBCH block index | 0 | 1 |
| Symbol numbers containing SSBs Note 2 | 4-7 | 8-11 |
| Slot numbers containing SSB Note 2 | 0 | 0 |
| SFN containing SSB | SFN mod (max(TSSB,10ms)/10ms) = 0 |  |
| RB numbers containing SSBs within channel BW | (RBJ, RBJ+1,.…, RBJ+19)Note 1 |  |
| Note 1: RBs containing SSB can be configured in any frequency location within the associated bandwidth part except the RBs for allowed synchronization raster defined in TS 38.104 [13]. Note 2: These values have been derived from other parameters for information purposes (as per TS 38.213 [3]). They are not settable parameters themselves. |  |  |

#### A.3.10.2.22	SSB pattern 22 in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz

Table A.3.10.2.22-1: SSB.22 FR2: SSB Pattern 20 for SSB SCS = 240 kHz in 100 MHz channel with 2 SSBs per SS-burst

| SSB Parameters | Values |  |
| --- | --- | --- |
| Channel bandwidth | 100 MHz |  |
| SSB SCS | 240 kHz |  |
| SSB periodicity (TSSB) | 80 ms |  |
| Number of SSBs per SS-burst | 2 |  |
| SS/PBCH block index | 0 | 1 |
| Symbol numbers containing SSBs Note 2 | 8-11 | 12-13, 0-1 |
| Slot numbers containing SSB Note 2 | 0 | 0, 1 |
| SFN containing SSB | SFN mod (max(TSSB,10ms)/10ms) = 0 |  |
| RB numbers containing SSBs within channel BW | (RBJ, RBJ+1,.…, RBJ+39)Note 1 |  |
| Note 1: RBs containing SSB can be configured in any frequency location within the associated bandwidth part except the RBs for allowed synchronization raster defined in TS 38.104 [13]. Note 2: These values have been derived from other parameters for information purposes (as per TS 38.213 [3]). They are not settable parameters themselves. |  |  |

#### A.3.10.2.23	SSB pattern 23 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz

Table A.3.10.2.23-1: SSB.23 FR2: SSB Pattern 21 for SSB SCS = 120 kHz in 100 MHz channel with 1 SSB per SS-burst

| SSB Parameters | Values |
| --- | --- |
| Channel bandwidth | 100 MHz |
| SSB SCS | 120 kHz |
| SSB periodicity (TSSB) | 40 ms |
| Number of SSBs per SS-burst | 1 |
| SS/PBCH block index | 1 |
| Symbol numbers containing SSBs Note 2 | 8-11 |
| Slot numbers containing SSB Note 2 | 0 |
| SFN containing SSB | SFN mod (max(TSSB,10ms)/10ms) = 0 |
| RB numbers containing SSBs within channel BW | (RBJ, RBJ+1,.…, RBJ+19)Note 1 |
| Note 1: RBs containing SSB can be configured in any frequency location within the cell bandwidth according to the allowed synchronization raster defined in TS 38.104 [13]. Note 2: These values have been derived from other parameters for information purposes (as per TS 38.213 [3]). They are not settable parameters themselves. |  |

#### A.3.10.2.24	SSB pattern 24 in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz

Table A.3.10.2.24-1: SSB.24 FR2: SSB Pattern 22 for SSB SCS = 240 kHz in 100 MHz channel with 1 SSB per SS-burst

| SSB Parameters | Values |  |
| --- | --- | --- |
| Channel bandwidth | 100 MHz |  |
| SSB SCS | 240 kHz |  |
| SSB periodicity (TSSB) | 40 ms |  |
| Number of SSBs per SS-burst | 1 |  |
| SS/PBCH block index | 1 |  |
| Symbol numbers containing SSBs Note 2 | 12-13 | 0-1 |
| Slot numbers containing SSB Note 2 | 0 | 1 |
| SFN containing SSB | SFN mod (max(TSSB,10ms)/10ms) = 0 |  |
| RB numbers containing SSBs within channel BW | (RBJ, RBJ+1,.…, RBJ+39)Note 1 |  |
| Note 1: RBs containing SSB can be configured in any frequency location within the cell bandwidth according to the allowed synchronization raster defined in TS 38.104 [13]. Note 2: These values have been derived from other parameters for information purposes (as per TS 38.213 [3]). They are not settable parameters themselves. |  |  |

#### A.3.10.2.25	SSB pattern 25 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz

Table A.3.10.2.25-1: SSB.25 FR2: SSB Pattern 17 for SSB SCS = 120 kHz in 100 MHz channel with 3 SSBs per SS-burst

| SSB Parameters | Values |  |  |  |
| --- | --- | --- | --- | --- |
| Channel bandwidth | 100 MHz |  |  |  |
| SSB SCS | 120 kHz |  |  |  |
| SSB periodicity (TSSB) | 20 ms |  |  |  |
| Number of SSBs per SS-burst | 3 |  |  |  |
| SS/PBCH block index | 0 | 1 | 2 | 3 |
| Symbol numbers containing SSBs Note 2 | 4-7 | 8-11 | 2-5 | 6-9 |
| Slot numbers containing SSB Note 2 | 0 | 0 | 1 | 1 |
| SFN containing SSB | SFN mod (max(TSSB,10ms)/10ms) = 0 |  |  |  |
| RB numbers containing SSBs within channel BW | (RBJ, RBJ+1,.…, RBJ+19)Note 1 |  |  |  |
| Note 1: RBs containing SSB can be configured in any frequency location within the cell bandwidth according to the allowed synchronization raster defined in TS 38.104 [13]. Note 2: These values have been derived from other parameters for information purposes (as per TS 38.213 [3]). They are not settable parameters themselves. |  |  |  |  |

## A.3.10A	SSB Configurations under CCA

### A.3.10A.1	SSB Configurations under CCA for FR1

#### A.3.10A.1.1	SSB pattern 1 under CCA for semi-static channel access: SSB allocation for SSB SCS=30kHz in 40MHz

Table A.3.10A.1.1-1: SSB.1 CCA: SSB Pattern 1 for SSB SCS=30 kHz in 40 MHz channel

| SSB Parameters | Values |
| --- | --- |
| Channel bandwidth | 40 MHz |
| SSB SCS | 30 kHz |
| SSB periodicity (TSSB) | 20 ms |
| Number of SSB indexes per SS-burst (NSSBQCL) | 1 |
| Number of SS/PBCH block candidates per SSB index | 1 |
| SS/PBCH block candidate position | 0 |
| SS/PBCH block index | 0 |
| Symbol numbers containing SSB Note 2 | 2-5 |
| Slot numbers containing SSB Note 2 | 0 |
| SFN containing SSB | SFN mod (max(TSSB,10ms)/10ms) = 0 |
| RB numbers containing SSB within channel BW | (RBJ, RBJ+1,.…, RBJ+19)Note 1 |
| Note 1: RBs containing SSB can be configured in any frequency location within the cell bandwidth according to the allowed synchronization raster defined in TS 38.104 [13]. Note 2: These values have been derived from other parameters for information purposes (as per TS 38.213 [3]). They are not settable parameters themselves |  |

#### A.3.10A.1.2	SSB pattern 2 under CCA for dynamic channel access: SSB allocation for SSB SCS=30kHz in 40MHz

Table A.3.10A.1.2-1: SSB.2 CCA: SSB Pattern 2 for SSB SCS=30 kHz in 40 MHz channel

| SSB Parameters | Values |  |
| --- | --- | --- |
| Channel bandwidth | 40 MHz |  |
| SSB SCS | 30 kHz |  |
| SSB periodicity (TSSB) | 20 ms |  |
| Number of SSB indexes per SS-burst (NSSBQCL) | 1 |  |
| Number of SS/PBCH block candidates per SSB index | 2 |  |
| SS/PBCH block candidate position | 0 | 2 |
| SS/PBCH block index | 0 | 0 |
| Symbol numbers containing SSB Note 2 | 2-5 | 2-5 |
| Slot numbers containing SSB Note 2 | 0 | 1 |
| SFN containing SSB | SFN mod (max(TSSB,10ms)/10ms) = 0 |  |
| RB numbers containing SSB within channel BW | (RBJ, RBJ+1,.…, RBJ+19)Note 1 |  |
| Note 1: RBs containing SSB can be configured in any frequency location within the cell bandwidth according to the allowed synchronization raster defined in TS 38.104 [13]. Note 2: These values have been derived from other parameters for information purposes (as per TS 38.213 [3]). They are not settable parameters themselves |  |  |

#### A.3.10A.1.3	SSB pattern 3 under CCA for semi-static channel access: SSB allocation for SSB SCS=30 kHz in 40 MHz

Table A.3.10.1.3-1: SSB.3 CCA: SSB Pattern 3 for SSB SCS=30 kHz in 40 MHz channel

| SSB Parameters | Values |  |
| --- | --- | --- |
| Channel bandwidth | 40 MHz |  |
| SSB SCS | 30 kHz |  |
| SSB periodicity (TSSB) | 20 ms |  |
| Number of SSB indexes per SS-burst (NSSBQCL) | 2 |  |
| Number of SS/PBCH block candidates per SSB index | 1 |  |
| SS/PBCH block candidate position | 0 | 1 |
| SS/PBCH block index | 0 | 1 |
| Symbol numbers containing SSB Note 2 | 2-5 | 8-11 |
| Slot numbers containing SSB Note 2 | 0 | 0 |
| SFN containing SSB | SFN mod (max(TSSB,10ms)/10ms) = 0 |  |
| RB numbers containing SSB within channel BW | (RBJ, RBJ+1,.…, RBJ+19)Note 1 |  |
| Note 1: RBs containing SSB can be configured in any frequency location within the cell bandwidth according to the allowed synchronization raster defined in TS 38.104 [13]. Note 2: These values have been derived from other parameters for information purposes (as per TS 38.213 [3]). They are not settable parameters themselves. |  |  |

#### A.3.10A.1.4	SSB pattern 4 under CCA for dynamic channel access: SSB allocation for SSB SCS=30 kHz in 40 MHz

Table A.3.10.1.4-1: SSB.4 CCA: SSB Pattern 4 for SSB SCS=30 kHz in 40 MHz channel

| SSB Parameters | Values |  |  |  |
| --- | --- | --- | --- | --- |
| Channel bandwidth | 40 MHz |  |  |  |
| SSB SCS | 30 kHz |  |  |  |
| SSB periodicity (TSSB) | 20 ms |  |  |  |
| Number of SSB indexes per SS-burst (NSSBQCL) | 2 |  |  |  |
| Number of SS/PBCH block candidates per SSB index | 2 |  |  |  |
| SS/PBCH block candidate position | 0 | 2 | 1 | 3 |
| SS/PBCH block index | 0 | 0 | 1 | 1 |
| Symbol numbers containing SSB Note 2 | 2-5 | 2-5 | 8-11 | 8-11 |
| Slot numbers containing SSB Note 2 | 0 | 1 | 0 | 1 |
| SFN containing SSB | SFN mod (max(TSSB,10ms)/10ms) = 0 |  |  |  |
| RB numbers containing SSB within channel BW | (RBJ, RBJ+1,.…, RBJ+19)Note 1 |  |  |  |
| Note 1: RBs containing SSB can be configured in any frequency location within the cell bandwidth according to the allowed synchronization raster defined in TS 38.104 [13]. Note 2: These values have been derived from other parameters for information purposes (as per TS 38.213 [3]). They are not settable parameters themselves. |  |  |  |  |

## A.3.10B	SSB Configurations for RedCap

### A.3.10B.1	SSB Configurations for FR1

#### A.3.10B.1.1	SSB pattern 1 for RedCap in FR1: SSB allocation for SSB SCS=30 kHz in 20 MHz

Table A.3.10B.1.1-1: SSB.1 RedCap FR1: SSB Pattern 1 for SSB SCS=30 kHz in 20 MHz channel

| SSB Parameters | Values |
| --- | --- |
| Channel bandwidth | 20 MHz |
| SSB SCS | 30 kHz |
| SSB periodicity (TSSB) | 20 ms |
| Number of SSBs per SS-burst | 1 |
| SS/PBCH block index | 0 |
| Symbol numbers containing SSB Note 3 | 4-7 or 2-5 Note 2 |
| Slot numbers containing SSB Note 3 | 0 |
| SFN containing SSB | SFN mod (max(TSSB,10ms)/10ms) = 0 |
| RB numbers containing SSB within channel BW | (RBJ, RBJ+1,.…, RBJ+19)Note 1 |
| Note 1: RBs containing SSB can be configured in any frequency location within the cell bandwidth according to the allowed synchronization raster defined in TS 38.104 [13]. Note 2: Symbols 4-7 is chosen, if the SSB pattern Case B should be used for the current band as indicated by Table 5.4.3.3-1 of TS 38.104 [13]; Otherwise, symbol 2-5 is chosen. Note 3: These values have been derived from other parameters for information purposes (as per TS 38.213 [3]). They are not settable parameters themselves |  |

#### A.3.10B.1.2	SSB pattern 2 for RedCap in FR1: SSB allocation for SSB SCS=30 kHz in 20 MHz

Table A.3.10B.1.2-1: SSB.2 RedCap FR1: SSB Pattern 2 for SSB SCS=30 kHz in 20 MHz channel

| SSB Parameters | Values |  |
| --- | --- | --- |
| Channel bandwidth | 20 MHz |  |
| SSB SCS | 30 kHz |  |
| SSB periodicity (TSSB) | 20 ms |  |
| Number of SSBs per SS-burst | 2 |  |
| SS/PBCH block index | 0 | 1 |
| Symbol numbers containing SSB Note 3 | 4-7 or 2-5 Note 2 | 8-11 |
| Slot numbers containing SSB Note 3 | 0 | 0 |
| SFN containing SSB | SFN mod (max(TSSB,10ms)/10ms) = 0 |  |
| RB numbers containing SSB within channel BW | (RBJ, RBJ+1,.…, RBJ+19)Note 1 |  |
| Note 1: RBs containing SSB can be configured in any frequency location within the cell bandwidth according to the allowed synchronization raster defined in TS 38.104 [13]. Note 2: Symbols 4-7 is chosen, if the SSB pattern Case B should be used for the current band as indicated by Table 5.4.3.3-1 of TS 38.104 [13]; Otherwise, symbol 2-5 is chosen. Note 3: These values have been derived from other parameters for information purposes (as per TS 38.213 [3]). They are not settable parameters themselves. |  |  |

#### A.3.10B.1.3	SSB pattern 3 for RedCap in FR1: SSB allocation for SSB SCS=30 kHz starting from odd SFN in 20 MHz

Table A.3.10B.1.3-1: SSB.3 RedCap FR1: SSB Pattern 3 for SSB SCS=30 kHz in 20 MHz channel

| SSB Parameters | Values |
| --- | --- |
| Channel bandwidth | 20 MHz |
| SSB SCS | 30 kHz |
| SSB periodicity (TSSB) | 20 ms |
| Number of SSBs per SS-burst | 1 |
| SS/PBCH block index | 0 |
| Symbol numbers containing SSB Note 3 | 4-7 or 2-5 Note 2 |
| Slot numbers containing SSB Note 3 | 0 |
| SFN containing SSB | SFN mod (max(TSSB,10ms)/10ms) = 1 |
| RB numbers containing SSB within channel BW | (RBJ, RBJ+1,.…, RBJ+19)Note 1 |
| Note 1: RBs containing SSB can be configured in any frequency location within the cell bandwidth according to the allowed synchronization raster defined in TS 38.104 [13]. Note 2: Symbols 4-7 is chosen, if the SSB pattern Case B should be used for the current band as indicated by Table 5.4.3.3-1 of TS 38.104 [13]; Otherwise, symbol 2-5 is chosen. Note 3: These values have been derived from other parameters for information purposes (as per TS 38.213 [3]). They are not settable parameters themselves. |  |

#### A.3.10B.1.4	SSB pattern 4 for RedCap in FR1: SSB allocation for SSB SCS=15 kHz in 10 MHz

Table A.3.10B.1.4-1: SSB.4 RedCap FR1: SSB Pattern 4 for SSB SCS=15 kHz in 10 MHz channel

| SSB Parameters | Values |
| --- | --- |
| Channel bandwidth | 10 MHz |
| SSB SCS | 15 kHz |
| SSB periodicity (TSSB) | 20 ms |
| Number of SSBs per SS-burst | 1 |
| SS/PBCH block index | 0 |
| Symbol numbers containing SSB Note 2 | 2-5 |
| Slot numbers containing SSB Note 2 | 0 |
| SFN containing SSB | SFN mod (max(TSSB,10ms)/10ms) = 0 |
| RB numbers containing SSB within channel BW | (RBJ, RBJ+1,.…, RBJ+19)Note 1 |
| Note 1: RBs containing SSB can be configured in any frequency location within the cell bandwidth according to the allowed synchronization raster defined in TS 38.104 [13]. Note 2: These values have been derived from other parameters for information purposes (as per TS 38.213 [3]). They are not settable parameters themselves. |  |

#### A.3.10B.1.5	SSB pattern 5 for RedCap in FR1: SSB allocation for SSB SCS=30 kHz in 20 MHz

Table A.3.10B.1.5-1: SSB.5 RedCap FR1: SSB Pattern 5 for SSB SCS=30 kHz in 20 MHz channel

| SSB Parameters | Values |
| --- | --- |
| Channel bandwidth | 20 MHz |
| SSB SCS | 30 kHz |
| SSB periodicity (TSSB) | 20 ms |
| Number of SSBs per SS-burst | 1 |
| SS/PBCH block index | 0 |
| Symbol numbers containing SSB Note 3 | 4-7 or 2-5 Note 2 |
| Slot numbers containing SSB Note 3 | 0 |
| SFN containing SSB | SFN mod (max(TSSB,10ms)/10ms) = 0 |
| RB numbers containing SSB within channel BW | (RBJ, RBJ+1,.…, RBJ+19)Note 1 |
| Note 1: RBs containing SSB can be configured in any frequency location within the cell bandwidth according to the allowed synchronization raster defined in TS 38.104 [13]. Note 2: Symbols 4-7 is chosen, if the SSB pattern Case B should be used for the current band as indicated by Table 5.4.3.3-1 of TS 38.104 [13]; Otherwise, symbol 2-5 is chosen. Note 3: These values have been derived from other parameters for information purposes (as per TS 38.213 [3]). They are not settable parameters themselves |  |

#### A.3.10B.1.6	SSB pattern 6 for RedCap in FR1: SSB allocation for SSB SCS=15 kHz in 10 MHz

Table A.3.10B.1.6-1: SSB.6 RedCap FR1: SSB Pattern 6 for SSB SCS=15 kHz in 10 MHz channel

| SSB Parameters | Values |
| --- | --- |
| Channel bandwidth | 10 MHz |
| SSB SCS | 15 kHz |
| SSB periodicity (TSSB) | 80 ms |
| Number of SSBs per SS-burst | 1 |
| SS/PBCH block index | 0 |
| Symbol numbers containing SSB Note 2 | 2-5 |
| Slot numbers containing SSB Note 2 | 0 |
| SFN containing SSB | SFN mod (max(TSSB,10ms)/10ms) = 0 |
| RB numbers containing SSB within channel BW | (RBJ, RBJ+1,.…, RBJ+19)Note 1 |
| Note 1: RBs containing SSB can be configured in any frequency location within the associated bandwidth part except the RBs for allowed synchronization raster defined in TS 38.104 [13]. Note 2: These values have been derived from other parameters for information purposes (as per TS 38.213 [3]). They are not settable parameters themselves. |  |

#### A.3.10B.1.7	SSB pattern 7 for RedCap in FR1: SSB allocation for SSB SCS=30 kHz in 20 MHz

Table A.3.10B.1.7-1: SSB.7 RedCap FR1: SSB Pattern 7 for SSB SCS=30 kHz in 40 MHz channel

| SSB Parameters | Values |
| --- | --- |
| Channel bandwidth | 20 MHz |
| SSB SCS | 30 kHz |
| SSB periodicity (TSSB) | 80 ms |
| Number of SSBs per SS-burst | 1 |
| SS/PBCH block index | 0 |
| Symbol numbers containing SSB Note 3 | 4-7 or 2-5 Note 2 |
| Slot numbers containing SSB Note 3 | 0 |
| SFN containing SSB | SFN mod (max(TSSB,10ms)/10ms) = 0 |
| RB numbers containing SSB within channel BW | (RBJ, RBJ+1,.…, RBJ+19)Note 1 |
| Note 1: RBs containing SSB can be configured in any frequency location within the associated bandwidth part except the RBs for allowed synchronization raster defined in TS 38.104 [13]. Note 2: Symbols 4-7 is chosen, if the SSB pattern Case B should be used for the current band as indicated by Table 5.4.3.3-1 of TS 38.104 [13]; Otherwise, symbol 2-5 is chosen. Note 3: These values have been derived from other parameters for information purposes (as per TS 38.213 [3]). They are not settable parameters themselves |  |

### A.3.10B.2	SSB Configurations for FR2

#### A.3.10B.2.1	SSB pattern 1 for RedCap in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz

Table A.3.10B.2.1-1: SSB.1 RedCap FR2: SSB Pattern 1 for SSB SCS = 120 kHz in 100 MHz channel with 1 SSB per SS-burst

| SSB Parameters | Values |
| --- | --- |
| Channel bandwidth | 100 MHz |
| SSB SCS | 120 kHz |
| SSB periodicity (TSSB) | 20 ms |
| Number of SSBs per SS-burst | 1 |
| SS/PBCH block index | 0 |
| Symbol numbers containing SSBs Note 2 | 4-7 |
| Slot numbers containing SSB Note 2 | 0 |
| SFN containing SSB | SFN mod (max(TSSB,10ms)/10ms) = 0 |
| RB numbers containing SSBs within channel BW | (RBJ, RBJ+1,.…, RBJ+19)Note 1 |
| Note 1: RBs containing SSB can be configured in any frequency location within the cell bandwidth according to the allowed synchronization raster defined in TS 38.104 [13]. Note 2: These values have been derived from other parameters for information purposes (as per TS 38.213 [3]). They are not settable parameters themselves. |  |

#### A.3.10B.2.2	SSB pattern 2 for RedCap in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz

Table A.3.10B.2.2-1: SSB.2 RedCap FR2: SSB Pattern 2 for SSB SCS = 120 kHz in 100 MHz channel with 1 SSB per SS-burst

| SSB Parameters | Values |
| --- | --- |
| Channel bandwidth | 100 MHz |
| SSB SCS | 120 kHz |
| SSB periodicity (TSSB) | 40 ms |
| Number of SSBs per SS-burst | 1 |
| SS/PBCH block index | 0 |
| Symbol numbers containing SSBs Note 2 | 4-7 |
| Slot numbers containing SSB Note 2 | 0 |
| SFN containing SSB | SFN mod (max(TSSB,10ms)/10ms) = 0 |
| RB numbers containing SSBs within channel BW | (RBJ, RBJ+1,.…, RBJ+19)Note 1 |
| Note 1: RBs containing SSB can be configured in any frequency location within the cell bandwidth according to the RBs for allowed synchronization raster defined in TS 38.104 [13]. Note 2: These values have been derived from other parameters for information purposes (as per TS 38.213 [3]). They are not settable parameters themselves. |  |

#### A.3.10B.2.3	SSB pattern 3 for RedCap in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz

Table A.3.10B.2.3-1: SSB.3 RedCap FR2: SSB Pattern 3 for SSB SCS = 120 kHz in 100 MHz channel with 1 SSB per SS-burst

| SSB Parameters | Values |
| --- | --- |
| Channel bandwidth | 100 MHz |
| SSB SCS | 120 kHz |
| SSB periodicity (TSSB) | 80 ms |
| Number of SSBs per SS-burst | 1 |
| SS/PBCH block index | 0 |
| Symbol numbers containing SSBs Note 2 | 4-7 |
| Slot numbers containing SSB Note 2 | 0 |
| SFN containing SSB | SFN mod (max(TSSB,10ms)/10ms) = 0 |
| RB numbers containing SSBs within channel BW | (RBJ, RBJ+1,.…, RBJ+19)Note 1 |
| Note 1: RBs containing SSB can be configured in any frequency location within the associated bandwidth part except the RBs for allowed synchronization raster defined in TS 38.104 [13]. Note 2: These values have been derived from other parameters for information purposes (as per TS 38.213 [3]). They are not settable parameters themselves. |  |

#### A.3.10B.2.4	SSB pattern 4 for RedCap in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz

Table A.3.10B.2.4-1: SSB.4 RedCap FR2: SSB Pattern 4 for SSB SCS = 240 kHz in 100 MHz channel with 1 SSB per SS-burst

| SSB Parameters | Values |
| --- | --- |
| Channel bandwidth | 100 MHz |
| SSB SCS | 240 kHz |
| SSB periodicity (TSSB) | 40 ms |
| Number of SSBs per SS-burst | 1 |
| SS/PBCH block index | 0 |
| Symbol numbers containing SSBs Note 2 | 8-11 |
| Slot numbers containing SSB Note 2 | 0 |
| SFN containing SSB | SFN mod (max(TSSB,10ms)/10ms) = 0 |
| RB numbers containing SSBs within channel BW | (RBJ, RBJ+1,.…, RBJ+19)Note 1 |
| Note 1: RBs containing SSB can be configured in any frequency location within the cell bandwidth according to the RBs for allowed synchronization raster defined in TS 38.104 [13]. Note 2: These values have been derived from other parameters for information purposes (as per TS 38.213 [3]). They are not settable parameters themselves. |  |

#### A.3.10B.2.5	SSB pattern 5 for RedCap in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz

Table A.3.10B.2.5-1: SSB.5 RedCap FR2: SSB Pattern 5 for SSB SCS = 240 kHz in 100 MHz channel with 1 SSB per SS-burst

| SSB Parameters | Values |
| --- | --- |
| Channel bandwidth | 100 MHz |
| SSB SCS | 240 kHz |
| SSB periodicity (TSSB) | 80 ms |
| Number of SSBs per SS-burst | 1 |
| SS/PBCH block index | 0 |
| Symbol numbers containing SSBs Note 2 | 8-11 |
| Slot numbers containing SSB Note 2 | 0 |
| SFN containing SSB | SFN mod (max(TSSB,10ms)/10ms) = 0 |
| RB numbers containing SSBs within channel BW | (RBJ, RBJ+1,.…, RBJ+19)Note 1 |
| Note 1: RBs containing SSB can be configured in any frequency location within the associated bandwidth part except the RBs for allowed synchronization raster defined in TS 38.104 [13]. Note 2: These values have been derived from other parameters for information purposes (as per TS 38.213 [3]). They are not settable parameters themselves. |  |

## A.3.11	SMTC Configurations

### A.3.11.1	SMTC pattern 1: SMTC period = 20 ms with SMTC duration = 1 ms

Table A.3.11.1-1: SMTC.1: SMTC Pattern 1 for SMTC period = 20 ms and duration = 1 ms

| SMTC Parameters | Values |
| --- | --- |
| SMTC periodicity | 20 ms |
| SMTC offset | 0 ms |
| SMTC duration | 1 ms |

### A.3.11.2	SMTC pattern 2: SMTC period = 20 ms with SMTC duration = 5 ms

Table A.3.11.2-1: SMTC.2: SMTC Pattern 2 for SMTC period = 20 ms and duration = 5 ms

| SMTC Parameters | Values |
| --- | --- |
| SMTC periodicity | 20 ms |
| SMTC offset | 0 ms |
| SMTC duration | 5 ms |

### A.3.11.3	SMTC pattern 3: SMTC period = 160 ms with SMTC duration = 1 ms

Table A.3.11.3-1: SMTC.3: SMTC Pattern 3 for SMTC period = 160 ms and duration = 1 ms

| SMTC Parameters | Values |
| --- | --- |
| SMTC periodicity | 160 ms |
| SMTC offset | 0 ms |
| SMTC duration | 1 ms |

### A.3.11.4	SMTC pattern 4: SMTC period = 20 ms with SMTC duration = 1 ms

Table A.3.11.4-1: SMTC.4: SMTC Pattern 4 for SMTC period = 20 ms and duration = 1 ms

| SMTC Parameters | Values |
| --- | --- |
| SMTC periodicity | 20 ms |
| SMTC offset | 10 ms |
| SMTC duration | 1 ms |

### A.3.11.5	SMTC pattern 5: SMTC period = 20 ms with SMTC duration = 5 ms

Table A.3.11.5-1: SMTC.5: SMTC Pattern 5 for SMTC period = 20 ms and duration = 5 ms

| SMTC Parameters | Values |
| --- | --- |
| SMTC periodicity | 20 ms |
| SMTC offset | 10 ms |
| SMTC duration | 5 ms |

### A.3.11.6	SMTC pattern 6: SMTC period = 20 ms with SMTC duration = 5 ms

Table A.3.11.6-1: SMTC.6: SMTC Pattern 6 for SMTC period = 20 ms and duration = 5 ms

| SMTC Parameters | Values |
| --- | --- |
| SMTC periodicity | 20 ms |
| SMTC offset | 17 ms |
| SMTC duration | 5 ms |

### A.3.11.7	SMTC pattern 7: SMTC period = 20 ms with SMTC duration = 5 ms

Table A.3.11.7-1: SMTC.7: SMTC Pattern 7 for SMTC period = 20 ms and duration = 5 ms

| SMTC Parameters | Values |
| --- | --- |
| SMTC periodicity | 20 ms |
| SMTC offset | 5 ms |
| SMTC duration | 5 ms |

### A.3.11.8	SMTC pattern 8: SMTC period = 10 ms with SMTC duration = 1 ms

Table A.3.11.8-1: SMTC.8: SMTC Pattern 8 for SMTC period = 10 ms and duration = 1 ms

| SMTC Parameters | Values |
| --- | --- |
| SMTC periodicity | 10 ms |
| SMTC offset | 0 ms |
| SMTC duration | 1 ms |

### A.3.11.9	SMTC pattern 9: SMTC period = 20 ms with SMTC duration = 1 ms

Table A.3.11.9-1: SMTC.9: SMTC Pattern 6 for SMTC period = 20 ms and duration = 1 ms

| SMTC Parameters | Values |
| --- | --- |
| SMTC periodicity | 20 ms |
| SMTC offset | 13 ms |
| SMTC duration | 1 ms |

### A.3.11.10	SMTC pattern 10: SMTC period = 80 ms with SMTC duration = 1 ms

Table A.3.11.10-1: SMTC.10: SMTC Pattern 10 for SMTC period = 80 ms and duration = 1 ms

| SMTC Parameters | Values |
| --- | --- |
| SMTC periodicity | 80 ms |
| SMTC offset | 0 ms |
| SMTC duration | 1 ms |

### A.3.11.11	SMTC pattern 11: SMTC period = 80 ms with SMTC duration = 5 ms

Table A.3.11.11-1: SMTC.11: SMTC Pattern 11 for SMTC period = 80 ms and duration = 5 ms

| SMTC Parameters | Values |
| --- | --- |
| SMTC periodicity | 80 ms |
| SMTC offset | 5 ms |
| SMTC duration | 5 ms |

## A.3.11A	SMTC Configurations for RedCap

### A.3.11A.0	Introduction

The SMTC configuration for RedCap can also be used in test case for non-RedCap.

### A.3.11A.1	SMTC pattern 1 for RedCap: SMTC period = 40 ms with SMTC duration = 1 ms

Table A.3.11A.1-1: SMTC.1 RedCap: SMTC Pattern 1 for SMTC period = 40 ms and duration = 1 ms

| SMTC Parameters | Values |
| --- | --- |
| SMTC periodicity | 40 ms |
| SMTC offset | 0 ms |
| SMTC duration | 1 ms |

### A.3.11A.2	SMTC pattern 2 for RedCap: SMTC period = 80 ms with SMTC duration = 1 ms

Table A.3.11A.2-1: SMTC.2 RedCap: SMTC Pattern 2 for SMTC period = 80 ms and duration = 1 ms

| SMTC Parameters | Values |
| --- | --- |
| SMTC periodicity | 80 ms |
| SMTC offset | 0 ms |
| SMTC duration | 1 ms |

### A.3.11A.3	SMTC pattern 3 for RedCap: SMTC period = 40 ms with SMTC duration = 1 ms

Table A.3.11A.3-1: SMTC.3 RedCap: SMTC Pattern 3 for SMTC period = 40 ms and duration = 1 ms

| SMTC Parameters | Values |
| --- | --- |
| SMTC periodicity | 40 ms |
| SMTC offset | 20 ms |
| SMTC duration | 1 ms |

### A.3.11A.4	SMTC pattern 4 for RedCap: SMTC period = 80 ms with SMTC duration = 5 ms

Table A.3.11A.4-1: SMTC.4 RedCap: SMTC Pattern 4 for SMTC period = 80 ms and duration = 5 ms

| SMTC Parameters | Values |
| --- | --- |
| SMTC periodicity | 80 ms |
| SMTC offset | 0 ms |
| SMTC duration | 5 ms |

## A.3.12	Test Cases with Different CC Configurations

### A.3.12.1 EN-DC Test Cases with Different EN-DC Configurations

#### A.3.12.1.1	Introduction

In Annex A EN-DC test cases may be defined for two component carriers (CCs) as well as for more than two CCs to verify the same RRM requirement.

#### A.3.12.1.2	Principle of testing

If multiple EN-DC test cases are defined for two CCs as well as for more than two CCs to verify the same type of RRM requirement, which depends on the number of CCs, then from the UE performance point of view the test coverage can be considered fulfilled by executing only the EN-DC test cases with the maximum number of CCs in EN-DC supported by the UE. Otherwise if the same type of RRM requirement is independent of the number of CCs then from the UE performance point of view the test coverage can be considered fulfilled by executing only the EN-DC test cases with two CCs in EN-DC supported by the UE.

### A.3.12.2	Carrier Aggregation Test Cases with Different CA Configurations

#### A.3.12.2.1	Introduction

In Annex A carrier aggregation test cases may be defined for two CCs as well as for more than two CCs to verify the same RRM requirement.

#### A.3.12.2.2	Principle of testing

If multiple carrier aggregation test cases are defined for two CCs as well as for more than two CCs to verify the same RRM requirement, which depends on the number of CCs, then from the UE performance point of view the test coverage can be considered fulfilled by executing only the CA test cases with the maximum number of CCs in CA supported by the UE. Otherwise if the same type of RRM requirement is independent of the number of CCs then from the UE performance point of view the test coverage can be considered fulfilled by executing only the CA test cases with at least two CCs in CA supported by the UE.

## A.3.13	Test Cases in SA and EN-DC Operations

### A.3.13.1	Introduction

This clause defines a principle which is applicable to test cases verifying RRM requirements in standalone (SA) or EN-DC operations.

In Annex A test cases may be defined in SA and EN-DC operations to verify the same RRM requirement.

### A.3.13.2	Principle of Testing

If test cases are defined in both SA and EN-DC operations to verify the same RRM requirement then the UE capable of both SA and EN-DC operations needs to verify that RRM requirement by performing test case(s) in either SA operation or in EN-DC operation.

If test cases are defined in both SA and EN-DC operations to verify at least one common RRM requirement then the UE capable of both SA and EN-DC operations needs to verify RRM requirements by performing test case(s) in either SA operation or in EN-DC operation provided that the performed test case(s):

-	verifies the largest number of RRM requirements and

-	verifies at least all RRM requirements covered in the test case(s), which is not performed.

## A.3.13A	Test Cases involving E-UTRA/FR1 and FR2 carriers

### A.3.13A.1	Introduction

The following applies to UE compliant to this version of the specification when undergoing tests with a mix of E-UTRA/NR FR1 and NR FR2 carriers in clauses A.5, A.7 and A.8.

### A.3.13A.2	Principle of Testing in EN-DC

For test cases in clause A.5 listed in Table A.3.13A.2-1, the following applies:

-	UE does not have to pass the test case.

Table A.3.13A.2-1: Test cases UE does not have to pass in current version of specification (EN-DC)

| Clause | Test case slogan |
| --- | --- |
| A.5.5.2.7 | E-UTRAN – NR FR2 interruptions at E-UTRA SRS carrier based switching |
| A.5.5.2.9 | E-UTRAN – NR FR2 interruptions during measurements on deactivated NR PSCell |
| A.5.5.3.2 | SCell Activation and deactivation of known SCell in FR1 for 160ms SCell measurement cycle |
| A.5.5.3.5 | SCell Activation and deactivation of SCell in FR2 |
| A.5.5.3.6 | Multiple SCell Activation and deactivation of one unknown SCell and one known SCell in FR2 |
| A.5.5.6.4.2 | E-UTRAN – NR FR1 PSCell SCell dormancy switch of two FR2 SCells outside active time |
| A.5.7.1.3 | EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR2 target cell |

### A.3.13A.3	Principle of Testing in SA

For test cases in clause A.7 listed in Table A.3.13A.3-1, the following applies:

-	UE does not have to pass the test case.

Table A.3.13A.3-1: Test cases UE does not have to pass in current version of specification (SA)

| Clause | Test case slogan |
| --- | --- |
| A.7.5.3.2 | SCell Activation and deactivation for FR1+FR2 inter-band with target SCell in FR2 |
| A.7.5.3.7 | PUCCH SCell activation and deactivation delay requirements of FR2 unknown cell with FR1 PCell |
| A.7.5.6.1.2 | NR FR1- NR FR2 DL active BWP switch of PCell with non-DRX in SA |
| A.7.5.6.4.2 | NR FR1 PCell SCell dormancy switch of two FR2 SCells outside active time |
| A.7.6.2.5 | SA event triggered reporting tests for FR2 without SSB time index detection when DRX is not used (PCell in FR1) |
| A.7.6.2.6 | SA event triggered reporting tests for FR2 without SSB time index detection when DRX is used (PCell in FR1) |
| A.7.6.2.7 | SA event triggered reporting tests for FR2 with SSB time index detection when DRX is not used (PCell in FR1) |
| A.7.6.2.8 | SA event triggered reporting tests for FR2 with SSB time index detection when DRX is used (PCell in FR1) |
| A.7.7.1.3 | SA inter-frequency measurement accuracy with FR1 serving cell and FR2 target cell |

### A.3.13A.4	Principle of Testing in E-UTRA

For test cases in clause A.8 listed in Table A.3.13A.4-1, the following applies:

-	UE does not have to pass the test case.

Table A.3.13A.4-1: Test cases UE does not have to pass in current version of specification (E-UTRA)

| Clause | Test case slogan |
| --- | --- |
| A.8.2.2.2 | E-UTRA – NR Early Measurement Reporting for NR in FR2 |
| A.8.4.2.5 | NR Inter-RAT event triggered reporting tests for FR2 without SSB time index detection when DRX is not used |
| A.8.4.2.6 | NR Inter-RAT event triggered reporting tests for FR2 without SSB time index detection when DRX is used |
| A.8.4.2.7 | NR Inter-RAT event triggered reporting tests for FR2 with SSB time index detection when DRX is not used |
| A.8.4.2.8 | NR Inter-RAT event triggered reporting tests for FR2 with SSB time index detection when DRX is used |

### A.3.13A.5	Principle of Testing in NR-DC

For test cases in clause A.6 listed in Table A.3.13A.5-1, the following applies:

-	UE does not have to pass the test case.

Table A.3.13A.5-1: Test cases UE does not have to pass in current version of specification (EN-DC)

| Clause | Test case slogan |
| --- | --- |
| A.6.3.X.2 | CHO including target MCG and target SCG in NR-DC (FR1-FR2 NR-DC to FR1-FR2 NR-DC) |
| A.5.3.X.4 | CHO including target MCG and candidate SCG in NR-DC (FR1-FR2 NR-DC to FR1-FR2 NR-DC) |
| A.6.5.X.3 | Intra-frequency CPC from FR1-FR2 NR-DC to FR1-FR2 NR-DC |
| A.6.5.X.4 | Inter-frequency CPA from FR1-FR2 NR-DC to FR1-FR2 NR-DC |

## A.3.13B		Test Cases for EN-DC and NE-DC Operations

### A.3.13B.1	Active BWP switch Test Cases for EN-DC and NE-DC Operations

#### A.3.13B.1.1	Introduction

This clause defines a principle which is applicable to test cases verifying active BWP switch requirements for EN-DC operation and NE-DC operations.

In Annex A test cases are defined for both EN-DC and NE-DC operations to verify the same type of RRM requirement.

#### A.3.13B.1.2	Principle of Testing

UE capable of both EN-DC and NE-DC operations needs to be tested with one of the tests in either EN-DC or NE-DC operations.

### A.3.13B.2	SFTD accuracy Test Cases for EN-DC and NE-DC Operations

#### A.3.13B.2.1	Introduction

This clause defines a principle which is applicable to test cases verifying SFTD accuracy requirements for EN-DC operation and NE-DC operations.

In Annex A test cases are defined for both EN-DC and NE-DC operations to verify the same type of RRM requirement.

#### A.3.13B.2.2	Principle of Testing

UE capable of both EN-DC and NE-DC operations needs to be tested with one of the tests in either EN-DC or NE-DC operations.

## A.3.14	CSI-RS configurations

### A.3.14.1	FDD

Table A.3.14.1-1: CSI-RS Reference Measurement Channels for SCS=15kHz

|  | CSI-RS.1.1 FDD | CSI-RS.1.2 FDD | CSI-RS.1.3 FDD | CSI-RS.1.4 FDD | CSI-RS.1.5 FDD | CSI-RS.1.6 FDD | CSI-RS.1.7 FDD |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Resource Type | periodic | periodic | aperiodic | aperiodic | aperiodic | periodic | periodic |
| Resource Set Config |  |  |  |  |  |  |  |
| nzp-CSI-ResourceSetId | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| repetition | n.a. | off | off | on | off | n.a. | off |
| aperiodicTriggeringOffset | n.a. | n.a. | 0 | 0 | 0 | n.a. | n.a. |
| trs-Info | n.a. | n.a. | n.a. | n.a. | n.a. | n.a. | n.a. |
| Resource Config |  |  |  |  |  |  |  |
|  |  | 0 for resource #0 | 0 for resource #0 | 0 for resource #0 | 0 for resource #0 |  | 2 for resource #0 |
|  |  |  |  | 1 for resource #1 |  |  |  |
|  |  |  |  | 2 for resource #2 |  |  |  |
|  |  |  |  | 3 for resource #3 |  |  |  |
| nzp-CSI-RS-ResourceId | 0 for resource #0 | 1 for resource #1 | 1 for resource #1 | 4 for resource #4 | 1 for resource #1 | 0 for resource #0 | 3 for resource #1 |
|  |  |  |  | 5 for resource #5 |  |  |  |
|  |  |  |  | 6 for resource #6 |  |  |  |
|  |  |  |  | 7 for resource #7 |  |  |  |
| powerControlOffset | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| powerControlOffsetSS | db0 | db0 | db0 | db0 | db0 | db0 | db0 |
| scramblingID | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Period (slots) | slot5 | slot10 | n.a. | n.a. | n.a. | slot40 | slot10 |
| Offset | 1 | 1 | n.a. | n.a. | n.a. | 1 | 1 |
| qcl-InfoPeriodicCSI-RS | TCI.State.0 | TCI.State.0 | n.a. | n.a. | n.a. | TCI.State.0 | TCI.State.0 |
|  |  | TCI.State.1 |  |  |  |  | TCI.State.1 |
| frequencyDomainAllocation | 000001 | 0001 | 0001 | 0001 | 000001 | 000001 | 0100 |
| nrofPorts | 2 | 1 | 1 | 1 | 1 | 2 | 1 |
|  |  | 6 for resource #0 | 6 for resource #0 | 0 for resource #0 | Specified in the test case for resource #0 |  | 6 for resource #0 |
|  |  |  |  | 1 for resource #1 |  |  |  |
|  |  |  |  | 2 for resource #2 |  |  |  |
|  |  |  |  | 3 for resource #3 |  |  |  |
| firstOFDMSymbolInTimeDomain | 4 for resource #0 | 10 for resource #1 | 10 for resource #1 | 4 for resource #4 | n.a. | 5 for resource #0 | 10 for resource #1 |
|  |  |  |  | 5 for resource #5 |  |  |  |
|  |  |  |  | 6 for resource #6 |  |  |  |
|  |  |  |  | 7 for resource #7 |  |  |  |
| cdm-Type | FD-CDM2 | noCDM | noCDM | noCDM | noCDM | FD-CDM2 | noCDM |
| density | 1 | 3 | 3 | 3 | 3 | 1 | 3 |
| startingRB | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| nrofRBs | 276 (Note 1) | 276 (Note 1) | 276 (Note 1) | 276 (Note 1) | 276 (Note 1) | 276 (Note 1) | 276 (Note 1) |
| Note 1: If the configured value of PRBs is larger than the width of the corresponding BWP relevant for the test case, the Test Equipment shall implement CSI-RS only in the width of that BWP. |  |  |  |  |  |  |  |

Table A.3.14.1-2: CSI-RS Reference Measurement Channels for SCS=120kHz

|  | CSI-RS.2.1 FDD |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Resource Type | periodic |  |  |  |  |  |  |
| Resource Set Config |  |  |  |  |  |  |  |
| nzp-CSI-ResourceSetId | 0 |  |  |  |  |  |  |
| repetition | n.a. |  |  |  |  |  |  |
| aperiodicTriggeringOffset | n.a. |  |  |  |  |  |  |
| trs-Info | n.a. |  |  |  |  |  |  |
| Resource Config |  |  |  |  |  |  |  |
| nzp-CSI-RS-ResourceId | 0 for resource #0 |  |  |  |  |  |  |
| powerControlOffset | 0 |  |  |  |  |  |  |
| powerControlOffsetSS | db0 |  |  |  |  |  |  |
| scramblingID | 0 |  |  |  |  |  |  |
| Period (slots) | slot40. |  |  |  |  |  |  |
| Offset | 8 |  |  |  |  |  |  |
| qcl-InfoPeriodicCSI-RS | TCL.State.0 |  |  |  |  |  |  |
| frequencyDomainAllocation | 000001 |  |  |  |  |  |  |
| nrofPorts | 2 |  |  |  |  |  |  |
| firstOFDMSymbolInTimeDomain | 5 for resource #0 |  |  |  |  |  |  |
| cdm-Type | FD-CDM2 |  |  |  |  |  |  |
| density | 1 |  |  |  |  |  |  |
| startingRB | 0 |  |  |  |  |  |  |
| nrofRBs | 276 (Note 1) |  |  |  |  |  |  |
| Note 1: If the configured value of PRBs is larger than the width of the corresponding BWP relevant for the test case, the Test Equipment shall implement CSI-RS only in the width of that BWP. |  |  |  |  |  |  |  |

### A.3.14.2	TDD

Table A.3.14.2-1: CSI-RS Reference Measurement Channels for SCS=15kHz

|  | CSI-RS.1.1 TDD | CSI-RS.1.2 TDD | CSI-RS.1.3 TDD | CSI-RS.1.4 TDD | CSI-RS.1.5 TDD | CSI-RS.1.6 TDD |
| --- | --- | --- | --- | --- | --- | --- |
| Resource Type | periodic | periodic | aperiodic | aperiodic | periodic | periodic |
| Resource Set Config |  |  |  |  |  |  |
| nzp-CSI-ResourceSetId | 0 | 0 | 0 | 0 | 0 | 0 |
| repetition | n.a. | off | off | on | n.a. | off |
| aperiodicTriggeringOffset | n.a. | n.a. | 0 | 0 | n.a. | n.a. |
| trs-Info | n.a. | n.a. | n.a. | n.a. | n.a. | n.a. |
| Resource Config |  |  |  |  |  |  |
|  |  | 0 for resource #0 | 0 for resource #0 | 0 for resource #0 |  | 2 for resource #0 |
|  |  |  |  | 1 for resource #1 |  |  |
|  |  |  |  | 2 for resource #2 |  |  |
|  |  |  |  | 3 for resource #3 |  |  |
| nzp-CSI-RS-ResourceId | 0 for resource #0 | 1 for resource #1 | 1 for resource #1 | 4 for resource #4 | 0 for resource #0 | 3 for resource #1 |
|  |  |  |  | 5 for resource #5 |  |  |
|  |  |  |  | 6 for resource #6 |  |  |
|  |  |  |  | 7 for resource #7 |  |  |
| powerControlOffset | 0 | 0 | 0 | 0 | 0 | 0 |
| powerControlOffsetSS | db0 | db0 | db0 | db0 | db0 | db0 |
| scramblingID | 0 | 0 | 0 | 0 | 0 | 0 |
| Period (slots) | slot5 | slot10 | n.a. | n.a. | slot40 | slot10 |
| Offset | 1 | 1 | n.a. | n.a. | 1 | 1 |
| qcl-InfoPeriodicCSI-RS | TCI.State.0 | TCI.State.0 | n.a. | n.a. | TCI.State.0 | TCI.State.0 |
|  |  | TCI.State.1 |  |  |  | TCI.State.1 |
| frequencyDomainAllocation | 000001 | 0001 | 0001 | 0001 | 000001 | 0100 |
| nrofPorts | 2 | 1 | 1 | 1 | 2 | 1 |
|  |  | 6 for resource #0 | 6 for resource #0 | 0 for resource #0 |  | 6 for resource #0 |
|  |  |  |  | 1 for resource #1 |  |  |
|  |  |  |  | 2 for resource #2 |  |  |
|  |  |  |  | 3 for resource #3 |  |  |
| firstOFDMSymbolInTimeDomain | 4 for resource #0 | 10 for resource #1 | 10 for resource #1 | 4 for resource #4 | 5 for resource #0 | 10 for resource #1 |
|  |  |  |  | 5 for resource #5 |  |  |
|  |  |  |  | 6 for resource #6 |  |  |
|  |  |  |  | 7 for resource #7 |  |  |
| cdm-Type | FD-CDM2 | noCDM | noCDM | noCDM | FD-CDM2 | noCDM |
| density | 1 | 3 | 3 | 3 | 1 | 3 |
| startingRB | 0 | 0 | 0 | 0 | 0 | 0 |
| nrofRBs | 276 (Note 1) | 276 (Note 1) | 276 (Note 1) | 276 (Note 1) | 276 (Note 1) | 276 (Note 1) |
| Note 1: If the configured value of PRBs is larger than the width of the corresponding BWP relevant for the test case, the Test Equipment shall implement CSI-RS only in the width of that BWP. |  |  |  |  |  |  |

Table A.3.14.2-1A: CSI-RS Reference Measurement Channels for SCS=15kHz

|  | CSI-RS.1.1A TDD | CSI-RS.1.2A TDD | CSI-RS.1.3A TDD |
| --- | --- | --- | --- |
| Resource Type | periodic | aperiodic | periodic |
| Resource Set Config |  |  |  |
| nzp-CSI-ResourceSetId | 1 | 1 | 1 |
| repetition | off | off | off |
| aperiodicTriggeringOffset | n.a. | 0 | n.a. |
| trs-Info | n.a. | n.a. | n.a. |
| Resource Config |  |  |  |
| nzp-CSI-RS-ResourceId | 12 for resource #0 | 22 for resource #0 | 14 for resource #0 |
|  | 13 for resource #1 | 23 for resource #1 | 15 for resource #1 |
| powerControlOffset | 0 | 0 | 0 |
| powerControlOffsetSS | db0 | db0 | db0 |
| scramblingID | 0 | 0 | 0 |
| Period (slots) | slot20 | n.a. | slot10 |
| Offset | 1 | n.a. | 2 |
| qcl-InfoPeriodicCSI-RS | n.a. | n.a. | n.a. |
| frequencyDomainAllocation | 0001 | 0001 | 0001 |
| nrofPorts | 1 | 1 | 1 |
| firstOFDMSymbolInTimeDomain | 6 for resource #0 | 7 for resource #0 | 6 for resource #0 |
|  | 10 for resource #1 | 11 for resource #1 | 10 for resource #1 |
| cdm-Type | noCDM | noCDM | noCDM |
| density | 3 | 3 | 3 |
| startingRB | 0 | 0 | 0 |
| nrofRBs | 276 (Note 1) | 276 (Note 1) | 276 (Note 1) |
| Note 1: If the configured value of PRBs is larger than the width of the corresponding BWP relevant for the test case, the Test Equipment shall implement CSI-RS only in the width of that BWP. |  |  |  |

Table A.3.14.2-2: CSI-RS Reference Measurement Channels for SCS=30kHz

|  | CSI-RS.2.1 TDD | CSI-RS.2.2 TDD | CSI-RS.2.3 TDD | CSI-RS.2.4 TDD | CSI-RS.2.5 TDD | CSI-RS.2.6 TDD | CSI-RS.2.7 TDD |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Resource Type | periodic | periodic | aperiodic | aperiodic | aperiodic | periodic | periodic |
| Resource Set Config |  |  |  |  |  |  |  |
| nzp-CSI-ResourceSetId | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| repetition | n.a. | off | off | on | off | n.a. | off |
| aperiodicTriggeringOffset | n.a. | n.a. | 0 | 0 | 0 | n.a. | n.a. |
| trs-Info | n.a. | n.a. | n.a. | n.a. | n.a. | n.a. | n.a. |
| Resource Config |  |  |  |  |  |  |  |
|  |  | 0 for resource #0 | 0 for resource #0 | 0 for resource #0 | 0 for resource #0 |  | 2 for resource #0 |
|  |  |  |  | 1 for resource #1 |  |  |  |
|  |  |  |  | 2 for resource #2 |  |  |  |
|  |  |  |  | 3 for resource #3 |  |  |  |
| nzp-CSI-RS-ResourceId | 0 for resource #0 | 1 for resource #1 | 1 for resource #1 | 4 for resource #4 | 1 for resource #1 | 0 for resource #0 | 3 for resource #1 |
|  |  |  |  | 5 for resource #5 |  |  |  |
|  |  |  |  | 6 for resource #6 |  |  |  |
|  |  |  |  | 7 for resource #7 |  |  |  |
| powerControlOffset | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| powerControlOffsetSS | db0 | db0 | db0 | db0 | db0 | db0 | db0 |
| scramblingID | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Period (slots) | slot10 | slot20 | n.a. | n.a. | n.a. | slot80 | slot20 |
| Offset | 2 | 2 | n.a. | n.a. | n.a. | 2 | 2 |
| qcl-InfoPeriodicCSI-RS | TCI.State.0 | TCI.State.0 | n.a. | n.a. | n.a. | TCI.State.0 | TCI.State.0 |
|  |  | TCI.State.1 |  |  |  |  | TCI.State.1 |
| frequencyDomainAllocation | 000001 | 0001 | 0001 | 0001 | 000001 | 000001 | 0100 |
| nrofPorts | 2 | 1 | 1 | 1 | 1 | 2 | 1 |
|  |  | 6 for resource #0 | 6 for resource #0 | 0 for resource #0 | Specified in the test case for resource #0 |  | 6 for resource #0 |
|  |  |  |  | 1 for resource #1 |  |  |  |
|  |  |  |  | 2 for resource #2 |  |  |  |
|  |  |  |  | 3 for resource #3 |  |  |  |
| firstOFDMSymbolInTimeDomain | 5 for resource #0 | 10 for resource #1 | 10 for resource #1 | 4 for resource #4 | n.a. | 5 for resource #0 | 10 for resource #1 |
|  |  |  |  | 5 for resource #5 |  |  |  |
|  |  |  |  | 6 for resource #6 |  |  |  |
|  |  |  |  | 7 for resource #7 |  |  |  |
| cdm-Type | FD-CDM2 | noCDM | noCDM | noCDM | noCDM | FD-CDM2 | noCDM |
| density | 1 | 3 | 3 | 3 | 3 | 1 | 3 |
| startingRB | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| nrofRBs | 276 (Note 1) | 276 (Note 1) | 276 (Note 1) | 276 (Note 1) | 276 (Note 1) | 276 (Note 1) | 276 (Note 1) |
| Note 1: If the configured value of PRBs is larger than the width of the corresponding BWP relevant for the test case, the Test Equipment shall implement CSI-RS only in the width of that BWP. |  |  |  |  |  |  |  |

Table A.3.14.2-2A: CSI-RS Reference Measurement Channels for SCS=30kHz

|  | CSI-RS.2.1A TDD | CSI-RS.2.2A TDD | CSI-RS.2.3A TDD |
| --- | --- | --- | --- |
| Resource Type | periodic | aperiodic | periodic |
| Resource Set Config |  |  |  |
| nzp-CSI-ResourceSetId | 1 | 1 | 1 |
| repetition | off | off | off |
| aperiodicTriggeringOffset | n.a. | 0 | n.a. |
| trs-Info | n.a. | n.a. | n.a. |
| Resource Config |  |  |  |
| nzp-CSI-RS-ResourceId | 12 for resource #0 | 22 for resource #0 | 14 for resource #0 |
|  | 13 for resource #1 | 23 for resource #1 | 15 for resource #1 |
| powerControlOffset | 0 | 0 | 0 |
| powerControlOffsetSS | db0 | db0 | db0 |
| scramblingID | 0 | 0 | 0 |
| Period (slots) | slot40 | n.a. | slot20 |
| Offset | 2 | n.a. | 4 |
| qcl-InfoPeriodicCSI-RS | n.a. | n.a. | n.a. |
| frequencyDomainAllocation | 0001 | 0001 | 0001 |
| nrofPorts | 1 | 1 | 1 |
| firstOFDMSymbolInTimeDomain | 6 for resource #0 | 7 for resource #0 | 6 for resource #0 |
|  | 10 for resource #1 | 11 for resource #1 | 10 for resource #1 |
| cdm-Type | noCDM | noCDM | noCDM |
| density | 3 | 3 | 3 |
| startingRB | 0 | 0 | 0 |
| nrofRBs | 276 (Note 1) | 276 (Note 1) | 276 (Note 1) |
| Note 1: If the configured value of PRBs is larger than the width of the corresponding BWP relevant for the test case, the Test Equipment shall implement CSI-RS only in the width of that BWP. |  |  |  |

Table A.3.14.2-3: CSI-RS Reference Measurement Channels for SCS=120kHz

|  | CSI-RS.3.1 TDD | CSI-RS.3.2 TDD | CSI-RS.3.3 TDD | CSI-RS.3.4 TDD | CSI-RS.3.5 TDD | CSI-RS.3.6 TDD |
| --- | --- | --- | --- | --- | --- | --- |
| Resource Type | periodic | periodic | aperiodic | aperiodic | periodic | periodic |
| Resource Set Config |  |  |  |  |  |  |
| nzp-CSI-ResourceSetId | 0 | 0 | 0 | 0 | 0 | 0 |
| repetition | n.a. | off | off | on | n.a. | off |
| aperiodicTriggeringOffset | n.a. | n.a. | 4 | 4 | n.a. | n.a. |
| trs-Info | n.a. | n.a. | n.a. | n.a. | n.a. | n.a. |
| Resource Config |  |  |  |  |  |  |
|  |  | 0 for resource #0 | 0 for resource #0 | 0 for resource #0 |  | 2 for resource #0 |
|  |  |  |  | 1 for resource #1 |  |  |
|  |  |  |  | 2 for resource #2 |  |  |
|  |  |  |  | 3 for resource #3 |  |  |
| nzp-CSI-RS-ResourceId | 0 for resource #0 | 1 for resource #1 | 1 for resource #1 | 4 for resource #4 | 0 for resource #0 | 3 for resource #1 |
|  |  |  |  | 5 for resource #5 |  |  |
|  |  |  |  | 6 for resource #6 |  |  |
|  |  |  |  | 7 for resource #7 |  |  |
| powerControlOffset | 0 | 0 | 0 | 0 | 0 | 0 |
| powerControlOffsetSS | db0 | db0 | db0 | db0 | db0 | db0 |
| scramblingID | 0 | 0 | 0 | 0 | 0 | 0 |
| Period (slots) | slot40 | slot80 | n.a. | n.a. | slot320 | slot80 |
| Offset | 8 | 16 | n.a. | n.a. | 8 | 16 |
| qcl-InfoPeriodicCSI-RS | TCI.State.0 | TCI.State.0 | n.a. | n.a. | TCI.State.0 | TCI.State.0 |
|  |  | TCI.State.1 |  |  |  | TCI.State.1 |
| frequencyDomainAllocation | 000001 | 0001 | 0001 | 0001 | 000001 | 0100 |
| nrofPorts | 2 | 1 | 1 | 1 | 1 | 1 |
|  |  | 6 for resource #0 | 6 for resource #0 | 0 for resource #0 |  | 6 for resource #0 |
|  |  |  |  | 1 for resource #1 |  |  |
|  |  |  |  | 2 for resource #2 |  |  |
|  |  |  |  | 3 for resource #3 |  |  |
| firstOFDMSymbolInTimeDomain | 5 for resource #0 | 10 for resource #1 | 10 for resource #1 | 4 for resource #4 | 5 for resource #0 | 10 for resource #1 |
|  |  |  |  | 5 for resource #5 |  |  |
|  |  |  |  | 6 for resource #6 |  |  |
|  |  |  |  | 7 for resource #7 |  |  |
| cdm-Type | FD-CDM2 | noCDM | noCDM | noCDM | FD-CDM2 | noCDM |
| density | 1 | 3 | 3 | 3 | 1 | 3 |
| startingRB | 0 | 0 | 0 | 0 | 0 | 0 |
| nrofRBs | 276 (Note 1) | 276 (Note 1) | 276 (Note 1) | 276 (Note 1) | 276 (Note 1) | 276 (Note 1) |
| Note 1: If the configured value of PRBs is larger than the width of the corresponding BWP relevant for the test case, the Test Equipment shall implement CSI-RS only in the width of that BWP. |  |  |  |  |  |  |

Table A.3.14.2-3A: CSI-RS Reference Measurement Channels for SCS=120kHz

|  | CSI-RS.3.1A TDD | CSI-RS.3.2A TDD | CSI-RS.3.3A TDD | CSI-RS.3.4A TDD | CSI-RS.3.5A TDD |
| --- | --- | --- | --- | --- | --- |
| Resource Type | periodic | aperiodic | periodic | periodic | periodic |
| Resource Set Config |  |  |  |  |  |
| nzp-CSI-ResourceSetId | 1 | 1 | 1 | 0 | 1 |
| repetition | off | off | off | off | off |
| aperiodicTriggeringOffset | n.a. | 6 | n.a. | n.a. | n.a. |
| trs-Info | n.a. | n.a. | n.a. | n.a. | n.a. |
| Resource Config |  |  |  |  |  |
| nzp-CSI-RS-ResourceId | 12 for resource #0 | 22 for resource #0 | 14 for resource #0 | 0 for resource#0 | 1 for resource #1 |
|  | 13 for resource #1 | 23 for resource #1 | 15 for resource #1 |  |  |
| powerControlOffset | 0 | 0 | 0 | 0 | 0 |
| powerControlOffsetSS | db0 | db0 | db0 | [db6] | [db6] |
| scramblingID | 0 | 0 | 0 | 0 | 0 |
| Period (slots) | slot160 | n.a. | slot80 | Slot80 | Slot80 |
| Offset | 8 | n.a. | 16 | 8 | 8 |
| qcl-InfoPeriodicCSI-RS | n.a. | n.a. | n.a. | TCI.State.0 | TCI.State.1 |
| frequencyDomainAllocation | 0001 | 0001 | 0001 | 0001 | 0010 |
| nrofPorts | 1 | 1 | 1 | 1 | 1 |
| firstOFDMSymbolInTimeDomain | 6 for resource #0 | 7 for resource #0 | 6 for resource #0 | 5 | 5 |
|  | 10 for resource #1 | 11 for resource #1 | 10 for resource #1 |  |  |
| cdm-Type | noCDM | noCDM | noCDM | noCDM | noCDM |
| density | 3 | 3 | 3 | 3 | 3 |
| startingRB | 0 | 0 | 0 | 0 | 0 |
| nrofRBs | 276 (Note 1) | 276 (Note 1) | 276 (Note 1) | 276 (Note 1) | 276 (Note 1) |
| Note 1: If the configured value of PRBs is larger than the width of the corresponding BWP relevant for the test case, the Test Equipment shall implement CSI-RS only in the width of that BWP. |  |  |  |  |  |

## A.3.15	Angle of Arrival (AoA) for FR2 RRM test cases

This clause specifies the AoA setups for FR2 RRM test cases in clause A.5 and A.7. The applicable AoA setup is defined in each test case in clause A.5 and A.7.

### A.3.15.1	Setup 1: Single AoA in Rx beam peak direction

There is only one active probe in the test. The DL signals, and noise if applicable, transmitted from the probe, are aligned to the UE Rx beam peak direction (as defined in TS 38.101-2 [19]).

### A.3.15.2	Setup 2: Single AoA in non Rx beam peak direction

#### A.3.15.2.1	Setup 2a: Single AoA in non Rx beam peak direction without change in direction

There is only one active probe in the test. The DL signals, and noise if applicable, transmitted from the probe, align to a direction (AoA) which is from the set of directions corresponding to the EIS spherical coverage percentile of the DUT as defined in clause 7.3.4 of TS 38.101-2 [19] for each UE power class. The direction (AoA) of the signals shall not be changed between test iterations.

#### A.3.15.2.2	Setup 2b: Single AoA in non Rx beam peak direction with change in direction

There is only one active probe in the test. The DL signals, and noise if applicable, transmitted from the probe, align to a direction (AoA) which is from the set of directions corresponding to the EIS spherical coverage percentile of the DUT as defined in clause 7.3.4 of TS 38.101-2 [19] for each UE power class. For UE power class 3, the direction (AoA) of the signals shall be changed for each test iteration.

### A.3.15.3	Setup 3: 2 AoAs

There are 2 active probes in the test. The DL signals, and noise if applicable, transmitted from the two active probes, align to directions (AoAs) which are from the set of directions corresponding to the EIS spherical coverage percentile of the DUT as defined in clause 7.3.4 of TS 38.101-2 [19] for each UE power class. The relative angular offset between the directions (AoAs) of the 2 active probes, shall be changed for each test iteration. The applicable set of relative angular offsets between the 2 active probes is given in Table 3.15.3-1 for each UE power class.

Table A.3.15.3-1: Set of relative angular offsets between active probes for each power class

| UE Power class | Relative angular offset between active probes |
| --- | --- |
| 1 | 30°, 60°, 90° and 120° |
| 2 | FFS |
| 3 | 30°, 60°, 90°, 120° and 150° |
| 4 | FFS |
| 5 | 30°, 60°, 90° and 120° |
| 6 | 30°, 60°, 90°, 120° and 150° |
| 7 | FFS |

### A.3.15.4	Setup 4: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak

#### A.3.15.4.1	Setup 4a: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak without change in direction

There are 2 active probes in the test. The DL signals, and noise if applicable, are transmitted from the two active probes. One probe is aligned to the UE Rx beam peak direction as defined in TS 38.101-2 [19]. The second is aligned to a direction (AoA) which is from the set of directions corresponding to the EIS spherical coverage percentile of the DUT as defined in clause 7.3.4 of TS 38.101-2 [19] for each UE power class. The direction (AoA) of the non Rx beam peak signal shall not be changed between test iterations.

#### A.3.15.4.2	Setup 4b: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak with change in direction

There are 2 active probes in the test. The DL signals, and noise if applicable, are transmitted from the two active probes. One probe is aligned to the UE Rx beam peak direction as defined in TS 38.101-2 [19]. The second is aligned to a direction (AoA) which is from the set of directions corresponding to the EIS spherical coverage percentile of the DUT as defined in clause 7.3.4 of TS 38.101-2 [19] for each UE power class.

For UE power class 3, the relative angular offset between the directions (AoAs) of the 2 active probes shall be changed for each test iteration, within the probe alignment described above. The applicable set of relative angular offsets between the 2 active probes is given in Table 3.15.3-1 for each UE power class.

#### A.3.15.4.3	Setup 4c: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak for power class 6 UE supporting simultaneous reception from multiple directions

There are 2 active probes in the test. The DL signals, and noise if applicable, are transmitted from the two active probes. One probe is aligned to the UE Rx beam peak direction as defined in TS 38.101-2 [19]. The second is aligned to a direction (AoA) which is from the set of directions corresponding to the 2AoA spherical coverage requirement for simultaneous reception from multiple directions as defined in clause 7.3K.6 of TS 38.101-2 [19] for power class 6.

For power class 6 supporting simultaneous reception from multiple directions, the angular separation between the directions (AoAs) of the 2 active probes is 150°, and the direction (AoA) of the non Rx beam peak signal shall not be changed between test iterations.

### A.3.15.5	Setup 5: 2 AoAs for simultaneous reception with QCL Type-D

There are 2 active probes in the test. The DL signals, and noise if applicable, transmitted from the two active probes.  The 2 AoAs [(AoA1 and AoA2)]  for simultaneous reception with different QCL-typeD is from the set of [qualified AoA pairs] according to the spherical coverage requirement for simultaneous reception from multiple directions as defined in clause 7.3K.3 of TS 38.101-2 [19] for power class 3 supporting simultaneous reception from multiple directions, the angular separation between the directions (AoAs) of the 2 active probes is declared from Table 7.3K.3-1 in clause 7.3K.3 of TS 38.101-2 [19] and [shall not be changed for each test iteration].

[The relative angular offset between the directions of the AoA pair is based on the UE’s declared [AoA separation and UE] orientation as defined in clause 7.3K.3 of TS 38.101-2.]

[One of the AoAs (AoA1) needs to satisfy the spherical coverage requirement in Table 7.3.4.3-1 of TS 38.101-2].

Editor’s note: FFS how single AoA RF test and 2 AoA RF test could ensure this.

### A.3.15.6	Setup 6: 3 AoAs for simultaneous reception with different QCL Type-D

There are 3 active probes in the test and the DL signals and noise are transmitted from the three active probes.

Out of the three AoA, one AoA [(AoA1)] is aligned to a direction which is from the set of directions corresponding to the EIS spherical coverage percentile of the DUT as defined in clause 7.3.4 of TS 38.101-2 [19] for UE power class 3 and other 2 AoAs [(AoA2, AoA3)] are from the set of [qualified AoA pairs] according to the spherical coverage requirement for simultaneous reception from multiple directions as defined in clause 7.3K.3 of TS 38.101-2 for power class 3 supporting simultaneous reception from multiple directions.

The relative angular offset between the directions of the AoA pair is based on the UE’s declared [AoA separation and UE] orientation as defined in clause 7.3K.3 of TS 38.101-2 and [shall not be changed for each test iteration].

### A.3.15.7	Setup 7: 3 AoAs

There are 3 active probes in the test. The DL signals, and noise if applicable, transmitted from the three active probes, align to directions (AoAs) which are from the set of directions corresponding to the EIS spherical coverage percentile of the DUT as defined in clause 7.3.4 of TS 38.101-2 [19] for each UE power class. The relative angular offset between the directions (AoAs) of the 3 active probes, shall be changed for each test iteration. Any combinations of two relative angular offsets between 2 active probes specified in Table A.3.15.3-1, is considered as a valid applicable relative angular offsets between 3 active probes for the respective power class.

## A.3.16	TCI State Configuration

### A.3.16.1	Introduction

This clause provides the configurations for TCI states towards either SSB or CSI-RS. The TCI states defined in this clause are configured in each test when applicable to indicate that certain DL signals are QCL’ed with the referenceSignal configured in the TCI states.

### A.3.16.2	TCI states

Table A.3.16.2-1: TCI States

| Parameter | TCI.State.0 | TCI.State.1 | TCI.State.2 | TCI.State.3 |
| --- | --- | --- | --- | --- |
| tci-StateId | Id0 | Id1 | Id2 | Id3 |
| qcl-Type1 | typeC | typeC | typeA | typeA |
| qcl-Type2Note1 | typeD | typeD | typeD | typeD |
| referenceSignal | SSB0 | SSB1 | Resource #4 in TRS resource set 1 Note3 | Resource #4 in TRS resource set 2 Note3 |
| Note 1: qcl-Type2 of typeD only where applicable. For RRM test cases, this will be only in FR2 Note 2: referenceSignal configurations towards which the TCI states are configured are defined in a test-specific manner. Note 3: Reference TRS resource sets are defined in A.3.17, and the applicable TRS resource set(s) are specified in each test case. When a single TRS resource set is configured in a test case, it is considered as resource set 1. |  |  |  |  |

Table A.3.16.2-2: Void

## A.3.16A	Unified TCI State Configuration

### A.3.16A.1	Introduction

This clause provides the configurations for unified TCI states towards either SSB or CSI-RS. The DLorJoint TCI states defined in this clause are configured in each test when applicable to indicate that certain DL (and UL, if joint DL/UL operation is configured) signals are QCL’ed with the referenceSignal configured in the TCI states. The UL TCI states defined in this clause are configured in each test when applicable to indicate that certain UL signals are QCL’ed with the referenceSignal configured in the TCI states.

### A.3.16A.2	DLorJoint TCI states

Table A.3.16A.2-1: DLorJoint TCI States

| Parameter | DLorJoint TCI.State.0 | DLorJoint TCI.State.1 | DLorJoint TCI.State.2 | DLorJoint TCI.State.3 | DLorJoint TCI.State.4 | DLorJoint TCI.State.5 | DLorJoint TCI.State.6 | DLorJoint TCI.State.7 | DLorJoint TCI.State.8 | DLorJoint TCI.State.9 | DLorJoint TCI.State.10 | DLorJoint TCI.State.11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| tci-StateUnifiedId | Id0 | Id1 | Id2 | Id3 | Id4 | Id5 | Id6 | Id7 | Id8 | Id9 | Id10 | Id11 |
| qcl-Type1 | typeA | typeA | typeA | typeA | typeC | typeC | typeA | typeA | typeA | typeC | typeC | typeC |
| qcl-Type2Note1 | typeD | typeD | typeD | typeD | typeD | typeD | typeD | typeD | typeD | typeD | typeD | typeD |
| referenceSignal Note2 | Resource #4 in TRS resource set 1 Note3 | Resource #4 in TRS resource set 2 Note 5 | Resource #4 in TRS resource set 1 Note3 | Resource #4 in TRS resource set 2 Note3 | SSB0 | SSB1 from the cell with different PCI | Resource #4 in TRS resource set 3 Note 6 | Resource #4 in TRS resource set 4 Note 7 | Resource #4 in TRS resource set 5 Note 8 | SSB1 | SSB2 | SSB3 |
| pathlossReferenceRS | N/A | N/A | Resource #4 in TRS resource set 1 Note3 | Resource #4 in TRS resource set 1 Note3 | N/A | N/A | Resource #4 in TRS resource set 3 Note 6 | Resource #4 in TRS resource set 4 Note 7 | Resource #4 in TRS resource set 5 Note 8 | N/A | N/A | N/A |
| additionalPCI | N/A | configured Note4 | N/A | N/A | N/A | configured Note4 | N/A | N/A | N/A | N/A | N/A | N/A |
| Note 1: qcl-Type2 of typeD only where applicable. For RRM test cases, this will be only in FR2 Note 2: referenceSignal configurations towards which the TCI states are configured are defined in a test-specific manner. Note 3: Reference TRS resource sets are defined in A.3.17, and the applicable TRS resource set(s) are specified in each test case. When a single TRS resource set is configured in a test case, it is considered as resource set 1. The TCI state of the TRS is the DLorJoint TCI.State.4. Note 4: Only one PCI than serving cell PCI is included in the additionalPCIList, and the additionalPCIIndex is configured as 0. Note 5: Reference TRS resource sets are defined in A.3.17, and the applicable TRS resource set(s) are specified in each test case. When a single TRS resource set is configured in a test case, it is considered as resource set 1. The TCI state of the TRS is the DLorJoint TCI.State.5. Note 6: Reference TRS resource sets are defined in A.3.17, and the applicable TRS resource set(s) are specified in each test case. When a single TRS resource set is configured in a test case, it is considered as resource set 1. The TCI state of the TRS is the DlorJoint TCI.State.9. Note 7: Reference TRS resource sets are defined in A.3.17, and the applicable TRS resource set(s) are specified in each test case. When a single TRS resource set is configured in a test case, it is considered as resource set 1. The TCI state of the TRS is the DlorJoint TCI.State.10. Note 7: Reference TRS resource sets are defined in A.3.17, and the applicable TRS resource set(s) are specified in each test case. When a single TRS resource set is configured in a test case, it is considered as resource set 1. The TCI state of the TRS is the DLorJoint TCI.State.11. |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.3.16A.2-2: DLorJoint TCI States for mTRP FR1

| Parameter | DLorJoint TCI.State.1.1 | DLorJoint TCI.State.1.2 | DLorJoint TCI.State.1.3 | DLorJoint TCI.State.1.4 |
| --- | --- | --- | --- | --- |
| tci-StateUnifiedId | Id1 | Id2 | Id3 | Id4 |
| qcl-Type1 | typeA | typeA | typeC | typeC |
| qcl-Type2 | N/A | N/A | N/A | N/A |
| referenceSignal Note2 | Resource #4 in TRS resource set 1 Note2 | Resource #4 in TRS resource set 2 Note3 | SSB1 | SSB2 |
| pathlossReferenceRS | N/A | N/A | N/A | N/A |
| additionalPCI | N/A | N/A | N/A | N/A |
| Note 1: referenceSignal configurations towards which the TCI states are configured are defined in a test-specific manner. Note 2: Reference TRS resource sets are defined in A.3.17, and the applicable TRS resource set(s) are specified in each test case. When a single TRS resource set is configured in a test case, it is considered as resource set 1. The TCI state of the TRS is the DLorJoint TCI.State.1.1. Note 3: Reference TRS resource sets are defined in A.3.17, and the applicable TRS resource set(s) are specified in each test case. When a single TRS resource set is configured in a test case, it is considered as resource set 1. The TCI state of the TRS is the DLorJoint TCI.State.1.2. |  |  |  |  |

### A.3.16A.3	UL TCI states

Table A.3.16A.3-1: UL TCI States

| Parameter | UL TCI.State.0 | UL TCI.State.1 | UL TCI.State.2 | UL TCI.State.3 | UL TCI.State.4 |
| --- | --- | --- | --- | --- | --- |
| ul-TCIState-Id | Id0 | Id1 | Id2 | Id3 | Id4 |
| referenceSignal Note1 | SSB0 | SSB1 | Resource #4 in TRS resource set 1 Note2 | Resource #4 in TRS resource set 2 Note2 | Resource #4 in TRS resource set 3 Note2 |
| pathlossReferenceRS | Resource #4 in TRS resource set 1 Note2 | Resource #4 in TRS resource set 1 Note2 | Resource #4 in TRS resource set 1 Note2 | Resource #4 in TRS resource set 2 Note2 | Resource #4 in TRS resource set 3 Note2 |
| additionalPCI | N/A | configured Note3 | N/A | N/A | N/A |
| Note 1: referenceSignal configurations towards which the UL TCI states are configured are defined in a test-specific manner. Note 2: Reference TRS resource sets are defined in A.3.17, and the applicable TRS resource set(s) are specified in each test case. When a single TRS resource set is configured in a test case, it is considered as resource set 1. Note 3: Only one PCI than serving cell PCI is included in the additionalPCIList, and the additionalPCIIndex is configured as 0. |  |  |  |  |  |

## A.3.16B		LTM Candidate TCI State Configuration

### A.3.16B.1	Introduction

This clause provides the configurations for TCI states of LTM candidate cell(s) towards either SSB or TRS. The LTM candidate DLorJoint TCI states defined in this clause are configured in each test when applicable to indicate that certain DL (and UL, if joint DL/UL operation is configured) signals are QCL’ed with the referenceSignal configured in the TCI states. The UL TCI states defined in this clause are configured in each test when applicable to indicate that certain UL signals are QCL’ed with the referenceSignal configured in the TCI states.

### A.3.16B.2	LTM candidate DLorJoint TCI states

Table A.3.16B.2-1: LTM candidate DLorJoint TCI States

| Parameter | Candidate DLorJoint TCI.State.0 | Candidate DLorJoint TCI.State.1 | Candidate DLorJoint TCI.State.2 | Candidate DLorJoint TCI.State.3 |
| --- | --- | --- | --- | --- |
| Candidatetci-StateId | Id0 | Id1 | Id2 | Id3 |
| qcl-Type1 | typeA | typeA | typeA | typeA |
| referenceSignal of qcl-Type 1 Note2 | SSB0 | Resource #4 in TRS resource set 1 Note3 | SSB0 | Resource #4 in TRS resource set 1 Note3 |
| qcl-Type2Note1 | typeD | typed | typeD | typeD |
| referenceSignal of qcl-Type2 Note2 | SSB0 | Resource #4 in TRS resource set 1 Note3 | SSB0 | Resource #4 in TRS resource set 1 Note3 |
| pathlossReferenceRS | SSB0 | SSB0 | N/A | N/A |
| additionalPCI | N/A | N/A | N/A | N/A |
| Note 1: qcl-Type2 of typeD only where applicable. For RRM test cases, this will be only in FR2 Note 2: referenceSignal configurations towards which the TCI states are configured are defined in a test-specific manner. Note 3: Reference TRS resource sets are defined in A.3.17, and the applicable TRS resource set(s) are specified in each test case. When a single TRS resource set is configured for a candidate cell in a test case, it is considered as resource set 1. The TCI state of the TRS is the CandidateDLorJoint TCI.State.2. |  |  |  |  |

### A.3.16B.3	LTM candidate UL TCI states

Table A.3.16B.3-1: LTM candidate UL TCI States

| Parameter | Candidate UL TCI.State.0 |
| --- | --- |
| Candidateul-TCIState-Id | Id0 |
| referenceSignal Note1 | SSB0 |
| pathlossReferenceRS | SSB0 |
| additionalPCI | N/A |
| Note 1: referenceSignal configurations towards which the UL TCI states are configured are defined in a test-specific manner. |  |

## A.3.17	Configurations of CSI-RS for tracking

### A.3.17.1	Configuration of CSI-RS for tracking for FR1

#### A.3.17.1.1	FDD

Table A.3.17.1.1-1: CSI-RS for tracking for SCS=15kHz

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | TRS.1.1 FDD |
| Bandwidth |  | BW of Active BWPNote 1 |
| SCS | kHz | 15 |
| First subcarrier index in the PRB used for CSI-RS |  | k0=0 for CSI-RS resource 1,2,3,4 |
| First OFDM symbol in the slot used for CSI-RS |  | l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 |
| Number of CSI-RS ports (X) |  | 1 for CSI-RS resource 1,2,3,4 |
| CDM Type |  | ‘No CDM’ for CSI-RS resource 1,2,3,4 |
| Density (ρ) |  | 3 for CSI-RS resource 1,2,3,4 |
| CSI-RS periodicity | slots | 20 for CSI-RS resource 1,2,3,4 |
| CSI-RS offset | slots | 10 for CSI-RS resource 1 and 2 11 for CSI-RS resource 3 and 4 |
| EPRE ratio to SSS | dB | 0Note 2 |
| TCI state |  | TCI.State.0 |
| Note 1: BW of TRS is configured same as the BW size of UE active BWP in the RRM test cases Note 2: Unless otherwise specified in the test case |  |  |

Table A.3.17.1.1-2: CSI-RS for tracking for SCS=30kHz

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | TRS.1.2 FDD |
| Bandwidth |  | BW of Active BWPNote 1 |
| SCS | kHz | 30 |
| First subcarrier index in the PRB used for CSI-RS |  | k0=0 for CSI-RS resource 1,2,3,4 |
| First OFDM symbol in the slot used for CSI-RS |  | l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 |
| Number of CSI-RS ports (X) |  | 1 for CSI-RS resource 1,2,3,4 |
| CDM Type |  | ‘No CDM’ for CSI-RS resource 1,2,3,4 |
| Density (ρ) |  | 3 for CSI-RS resource 1,2,3,4 |
| CSI-RS periodicity | slots | 40 for CSI-RS resource 1,2,3,4 |
| CSI-RS offset | slots | 20 for CSI-RS resource 1 and 2 21 for CSI-RS resource 3 and 4 |
| EPRE ratio to SSS | dB | 0Note 2 |
| TCI state |  | TCI.State.0 |
| Note 1: BW of TRS is configured same as the BW size of UE active BWP in the RRM test cases Note 2: Unless otherwise specified in the test case |  |  |

Table A.3.17.1.1-3: Aperiodic CSI-RS for tracking for SCS=15kHz

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | TRS.1.3 FDD |
| Bandwidth |  | BW of Active BWPNote 1 |
| SCS | kHz | 15 |
| First subcarrier index in the PRB used for CSI-RS |  | k0=0 for CSI-RS resource 1,2,3,4 |
| First OFDM symbol in the slot used for CSI-RS |  | l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 |
| Number of CSI-RS ports (X) |  | 1 for CSI-RS resource 1,2,3,4 |
| CDM Type |  | ‘No CDM’ for CSI-RS resource 1,2,3,4 |
| Density (ρ) |  | 3 for CSI-RS resource 1,2,3,4 |
| aperiodicTriggeringOffsetL2 | slots | 2 |
| Aperiodic CSI-RS offset | slots | 2 for CSI-RS resource 1 and 2 3 for CSI-RS resource 3 and 4 |
| EPRE ratio to SSS | dB | 0Note 2 |
| TCI state |  | TCI.State.0 |
| Note 1: BW of TRS is configured same as the BW size of UE active BWP in the RRM test cases Note 2: Unless otherwise specified in the test case |  |  |

Table A.3.17.1.1-4: Aperiodic CSI-RS for tracking for SCS=30kHz

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | TRS.1.4 FDD |
| Bandwidth |  | BW of Active BWPNote 1 |
| SCS | kHz | 30 |
| First subcarrier index in the PRB used for CSI-RS |  | k0=0 for CSI-RS resource 1,2,3,4 |
| First OFDM symbol in the slot used for CSI-RS |  | l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 |
| Number of CSI-RS ports (X) |  | 1 for CSI-RS resource 1,2,3,4 |
| CDM Type |  | ‘No CDM’ for CSI-RS resource 1,2,3,4 |
| Density (ρ) |  | 3 for CSI-RS resource 1,2,3,4 |
| aperiodicTriggeringOffsetL2 | slots | 2 |
| Aperiodic CSI-RS offset | slots | 2 for CSI-RS resource 1 and 2 3 for CSI-RS resource 3 and 4 |
| EPRE ratio to SSS | dB | 0Note 2 |
| TCI state |  | TCI.State.0 |
| Note 1: BW of TRS is configured same as the BW size of UE active BWP in the RRM test cases Note 2: Unless otherwise specified in the test case |  |  |

Table A.3.17.1.1-5: CSI-RS for tracking for SCS=15kHz Set 1

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | TRS.1.5 FDD |
| Bandwidth |  | BW of Active BWPNote 1 |
| SCS | kHz | 15 |
| First subcarrier index in the PRB used for CSI-RS |  | k0=0 for CSI-RS resource 1,2,3,4 |
| First OFDM symbol in the slot used for CSI-RS |  | l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 |
| Number of CSI-RS ports (X) |  | 1 for CSI-RS resource 1,2,3,4 |
| CDM Type |  | ‘No CDM’ for CSI-RS resource 1,2,3,4 |
| Density (ρ) |  | 3 for CSI-RS resource 1,2,3,4 |
| CSI-RS periodicity | slots | 20 for CSI-RS resource 1,2,3,4 |
| CSI-RS offset | slots | 10 for CSI-RS resource 1 and 2 11 for CSI-RS resource 3 and 4 |
| EPRE ratio to SSS | dB | 0Note 2 |
| TCI state |  | DLorJoint TCI.State.1.3 |
| Note 1: BW of TRS is configured same as the BW size of UE active BWP in the RRM test cases Note 2: Unless otherwise specified in the test case |  |  |

Table A.3.17.1.1-6: CSI-RS for tracking for SCS=15kHz Set 2

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | TRS.1.6 FDD |
| Bandwidth |  | BW of Active BWPNote 1 |
| SCS | kHz | 15 |
| First subcarrier index in the PRB used for CSI-RS |  | k0=0 for CSI-RS resource 1,2,3,4 |
| First OFDM symbol in the slot used for CSI-RS |  | l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 |
| Number of CSI-RS ports (X) |  | 1 for CSI-RS resource 1,2,3,4 |
| CDM Type |  | ‘No CDM’ for CSI-RS resource 1,2,3,4 |
| Density (ρ) |  | 3 for CSI-RS resource 1,2,3,4 |
| CSI-RS periodicity | slots | 20 for CSI-RS resource 1,2,3,4 |
| CSI-RS offset | slots | 10 for CSI-RS resource 1 and 2 11 for CSI-RS resource 3 and 4 |
| EPRE ratio to SSS | dB | 0Note 2 |
| TCI state |  | DLorJoint TCI.State.1.4 |
| Note 1: BW of TRS is configured same as the BW size of UE active BWP in the RRM test cases Note 2: Unless otherwise specified in the test case |  |  |

Table A.3.17.1.1-7: CSI-RS for tracking for SCS=15kHz

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | TRS.1.7 FDD |
| Bandwidth |  | BW of Active BWPNote 1 |
| SCS | kHz | 15 |
| First subcarrier index in the PRB used for CSI-RS |  | k0=0 for CSI-RS resource 1,2,3,4 |
| First OFDM symbol in the slot used for CSI-RS |  | l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 |
| Number of CSI-RS ports (X) |  | 1 for CSI-RS resource 1,2,3,4 |
| CDM Type |  | ‘No CDM’ for CSI-RS resource 1,2,3,4 |
| Density (ρ) |  | 3 for CSI-RS resource 1,2,3,4 |
| CSI-RS periodicity | slots | 20 for CSI-RS resource 1,2,3,4 |
| CSI-RS offset | slots | 10 for CSI-RS resource 1 and 2 11 for CSI-RS resource 3 and 4 |
| EPRE ratio to SSS | dB | 0Note 2 |
| TCI state |  | TCI.State.0 |
| Note 1: BW of TRS is configured same as the BW size of UE active BP in the RRM test cases Note 2: Unless otherwise specified in the test case |  |  |

#### A.3.17.1.2	TDD

Table A.3.17.1.2-1: CSI-RS for tracking for SCS=15kHz

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | TRS.1.1 TDD |
| Bandwidth |  | BW of Active BWPNote 1 |
| SCS | kHz | 15 |
| First subcarrier index in the PRB used for CSI-RS |  | k0=0 for CSI-RS resource 1,2,3,4 |
| First OFDM symbol in the slot used for CSI-RS |  | l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 |
| Number of CSI-RS ports (X) |  | 1 for CSI-RS resource 1,2,3,4 |
| CDM Type |  | ‘No CDM’ for CSI-RS resource 1,2,3,4 |
| Density (ρ) |  | 3 for CSI-RS resource 1,2,3,4 |
| CSI-RS periodicity | slots | 20 for CSI-RS resource 1,2,3,4 |
| CSI-RS offset | slots | 10 for CSI-RS resource 1 and 2 11 for CSI-RS resource 3 and 4 |
| EPRE ratio to SSS | dB | 0Note 2 |
| TCI state |  | TCI.State.0 |
| Note: BW of TRS is configured same as the BW size of UE active BWP in the RRM test cases |  |  |

Table A.3.17.1.2-2: CSI-RS for tracking for SCS=30kHz

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | TRS.1.2 TDD |
| Bandwidth |  | BW of Active BWPNote 1 |
| SCS | kHz | 30 |
| First subcarrier index in the PRB used for CSI-RS |  | k0=0 for CSI-RS resource 1,2,3,4 |
| First OFDM symbol in the slot used for CSI-RS |  | l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 |
| Number of CSI-RS ports (X) |  | 1 for CSI-RS resource 1,2,3,4 |
| CDM Type |  | ‘No CDM’ for CSI-RS resource 1,2,3,4 |
| Density (ρ) |  | 3 for CSI-RS resource 1,2,3,4 |
| CSI-RS periodicity | slots | 40 for CSI-RS resource 1,2,3,4 |
| CSI-RS offset | slots | 20 for CSI-RS resource 1 and 2 21 for CSI-RS resource 3 and 4 |
| EPRE ratio to SSS | dB | 0Note 2 |
| TCI state |  | TCI.State.0 |
| Note 1: BW of TRS is configured same as the BW size of UE active BWP in the RRM test cases Note 2: Unless otherwise specified in the test case |  |  |

Table A.3.17.1.2-3: Aperiodic CSI-RS for tracking for SCS=15kHz

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | TRS.1.3 TDD |
| Bandwidth |  | BW of Active BWPNote 1 |
| SCS | kHz | 15 |
| First subcarrier index in the PRB used for CSI-RS |  | k0=0 for CSI-RS resource 1,2,3,4 |
| First OFDM symbol in the slot used for CSI-RS |  | l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 |
| Number of CSI-RS ports (X) |  | 1 for CSI-RS resource 1,2,3,4 |
| CDM Type |  | ‘No CDM’ for CSI-RS resource 1,2,3,4 |
| Density (ρ) |  | 3 for CSI-RS resource 1,2,3,4 |
| aperiodicTriggeringOffsetL2 | slots | 2 |
| Aperiodic CSI-RS offset | slots | 2 for CSI-RS resource 1 and 2 3 for CSI-RS resource 3 and 4 |
| EPRE ratio to SSS | dB | 0Note 2 |
| TCI state |  | TCI.State.0 |
| Note: BW of TRS is configured same as the BW size of UE active BWP in the RRM test cases |  |  |

Table A.3.17.1.2-4: Aperiodic CSI-RS for tracking for SCS=30kHz

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | TRS.1.4 TDD |
| Bandwidth |  | BW of Active BWPNote 1 |
| SCS | kHz | 30 |
| First subcarrier index in the PRB used for CSI-RS |  | k0=0 for CSI-RS resource 1,2,3,4 |
| First OFDM symbol in the slot used for CSI-RS |  | l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 |
| Number of CSI-RS ports (X) |  | 1 for CSI-RS resource 1,2,3,4 |
| CDM Type |  | ‘No CDM’ for CSI-RS resource 1,2,3,4 |
| Density (ρ) |  | 3 for CSI-RS resource 1,2,3,4 |
| aperiodicTriggeringOffsetL2 | slots | 2 |
| Aperiodic CSI-RS offset | slots | 2 for CSI-RS resource 1 and 2 3 for CSI-RS resource 3 and 4 |
| EPRE ratio to SSS | dB | 0Note 2 |
| TCI state |  | TCI.State.0 |
| Note 1: BW of TRS is configured same as the BW size of UE active BWP in the RRM test cases Note 2: Unless otherwise specified in the test case |  |  |

Table A.3.17.1.2-5: CSI-RS for tracking for SCS=15kHz Set 1

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | TRS.1.5 TDD |
| Bandwidth |  | BW of Active BWPNote 1 |
| SCS | kHz | 15 |
| First subcarrier index in the PRB used for CSI-RS |  | k0=0 for CSI-RS resource 1,2,3,4 |
| First OFDM symbol in the slot used for CSI-RS |  | l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 |
| Number of CSI-RS ports (X) |  | 1 for CSI-RS resource 1,2,3,4 |
| CDM Type |  | ‘No CDM’ for CSI-RS resource 1,2,3,4 |
| Density (ρ) |  | 3 for CSI-RS resource 1,2,3,4 |
| CSI-RS periodicity | slots | 20 for CSI-RS resource 1,2,3,4 |
| CSI-RS offset | slots | 10 for CSI-RS resource 1 and 2 11 for CSI-RS resource 3 and 4 |
| EPRE ratio to SSS | dB | 0Note 2 |
| TCI state |  | DLorJoint TCI.State.1.3 |
| Note: BW of TRS is configured same as the BW size of UE active BWP in the RRM test cases |  |  |

Table A.3.17.1.2-6: CSI-RS for tracking for SCS=15kHz Set 2

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | TRS.1.6 TDD |
| Bandwidth |  | BW of Active BWPNote 1 |
| SCS | kHz | 15 |
| First subcarrier index in the PRB used for CSI-RS |  | k0=0 for CSI-RS resource 1,2,3,4 |
| First OFDM symbol in the slot used for CSI-RS |  | l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 |
| Number of CSI-RS ports (X) |  | 1 for CSI-RS resource 1,2,3,4 |
| CDM Type |  | ‘No CDM’ for CSI-RS resource 1,2,3,4 |
| Density (ρ) |  | 3 for CSI-RS resource 1,2,3,4 |
| CSI-RS periodicity | slots | 20 for CSI-RS resource 1,2,3,4 |
| CSI-RS offset | slots | 10 for CSI-RS resource 1 and 2 11 for CSI-RS resource 3 and 4 |
| EPRE ratio to SSS | dB | 0Note 2 |
| TCI state |  | DLorJoint TCI.State.1.4 |
| Note: BW of TRS is configured same as the BW size of UE active BWP in the RRM test cases |  |  |

Table A.3.17.1.2-7: CSI-RS for tracking for SCS=30kHz Set 1

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | TRS.1.7 TDD |
| Bandwidth |  | BW of Active BWPNote 1 |
| SCS | kHz | 30 |
| First subcarrier index in the PRB used for CSI-RS |  | k0=0 for CSI-RS resource 1,2,3,4 |
| First OFDM symbol in the slot used for CSI-RS |  | l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 |
| Number of CSI-RS ports (X) |  | 1 for CSI-RS resource 1,2,3,4 |
| CDM Type |  | ‘No CDM’ for CSI-RS resource 1,2,3,4 |
| Density (ρ) |  | 3 for CSI-RS resource 1,2,3,4 |
| CSI-RS periodicity | slots | 40 for CSI-RS resource 1,2,3,4 |
| CSI-RS offset | slots | 20 for CSI-RS resource 1 and 2 21 for CSI-RS resource 3 and 4 |
| EPRE ratio to SSS | dB | 0Note 2 |
| TCI state |  | DLorJoint TCI.State.1.3 |
| Note 1: BW of TRS is configured same as the BW size of UE active BWP in the RRM test cases Note 2: Unless otherwise specified in the test case |  |  |

Table A.3.17.1.2-8: CSI-RS for tracking for SCS=30kHz Set 2

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | TRS.1.8 TDD |
| Bandwidth |  | BW of Active BWPNote 1 |
| SCS | kHz | 30 |
| First subcarrier index in the PRB used for CSI-RS |  | k0=0 for CSI-RS resource 1,2,3,4 |
| First OFDM symbol in the slot used for CSI-RS |  | l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 |
| Number of CSI-RS ports (X) |  | 1 for CSI-RS resource 1,2,3,4 |
| CDM Type |  | ‘No CDM’ for CSI-RS resource 1,2,3,4 |
| Density (ρ) |  | 3 for CSI-RS resource 1,2,3,4 |
| CSI-RS periodicity | slots | 40 for CSI-RS resource 1,2,3,4 |
| CSI-RS offset | slots | 20 for CSI-RS resource 1 and 2 21 for CSI-RS resource 3 and 4 |
| EPRE ratio to SSS | dB | 0Note 2 |
| TCI state |  | DLorJoint TCI.State.1.3 |
| Note 1: BW of TRS is configured same as the BW size of UE active BWP in the RRM test cases Note 2: Unless otherwise specified in the test case |  |  |

Table A.3.17.1.2-9: CSI-RS for tracking for SCS=30kHz

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | TRS.1.9 TDD |
| Bandwidth |  | BW of Active BWPNote 1 |
| SCS | kHz | 30 |
| First subcarrier index in the PRB used for CSI-RS |  | k0=0 for CSI-RS resource 1,2,3,4 |
| First OFDM symbol in the slot used for CSI-RS |  | l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 |
| Number of CSI-RS ports (X) |  | 1 for CSI-RS resource 1,2,3,4 |
| CDM Type |  | ‘No CDM’ for CSI-RS resource 1,2,3,4 |
| Density (ρ) |  | 3 for CSI-RS resource 1,2,3,4 |
| CSI-RS periodicity | slots | 40 for CSI-RS resource 1,2,3,4 |
| CSI-RS offset | slots | 20 for CSI-RS resource 1 and 2 21 for CSI-RS resource 3 and 4 |
| EPRE ratio to SSS | dB | 0Note 2 |
| TCI state |  | TCI.State.0 |
| Note 1: BW of TRS is configured same as the BW size of UE active BWP in the RRM test cases Note 2: Unless otherwise specified in the test case |  |  |

### A.3.17.2	Configuration of CSI-RS for tracking for FR2

#### A.3.17.2.1	TDD

Table A.3.17.2.1-1: CSI-RS for tracking for SCS=120kHz Set 1

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | TRS.2.1 TDD |
| Bandwidth |  | BW of Active BWPNote 1,3 |
| SCS | kHz | 120 |
| First subcarrier index in the PRB used for CSI-RS |  | k0=0 for CSI-RS resource 1,2,3,4 |
| First OFDM symbol in the slot used for CSI-RS |  | l0 = 1 for CSI-RS resource 1 and 3 l0 = 5 for CSI-RS resource 2 and 4 |
| Number of CSI-RS ports (X) |  | 1 for CSI-RS resource 1,2,3,4 |
| CDM Type |  | ‘No CDM’ for CSI-RS resource 1,2,3,4 |
| Density (ρ) |  | 3 for CSI-RS resource 1,2,3,4 |
| CSI-RS periodicity | slots | 80 for CSI-RS resource 1,2,3,4 |
| CSI-RS offset | slots | 40 for CSI-RS resource 1 and 2 41 for CSI-RS resource 3 and 4 |
| EPRE ratio to SSS | dB | 0Note 2 |
| TCI state |  | TCI.State.0 |
| Note 1: BW of TRS is configured same as the BW size of UE active BWP in the RRM test cases Note 2: Unless otherwise specified in the test case Note 3: If active BWP is larger than 52RBs, BW of TRS is configured as 52RBs. Otherwise, same as active BWP size. |  |  |

Table A.3.17.2.1-2: CSI-RS for tracking for SCS=120kHz Set 2

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | TRS.2.2 TDD |
| Bandwidth |  | BW of Active BWPNote 1,3 |
| SCS | kHz | 120 |
| First subcarrier index in the PRB used for CSI-RS |  | k0=0 for CSI-RS resource 1,2,3,4 |
| First OFDM symbol in the slot used for CSI-RS |  | l0 = 2 for CSI-RS resource 1 and 3 l0 = 6 for CSI-RS resource 2 and 4 |
| Number of CSI-RS ports (X) |  | 1 for CSI-RS resource 1,2,3,4 |
| CDM Type |  | ‘No CDM’ for CSI-RS resource 1,2,3,4 |
| Density (ρ) |  | 3 for CSI-RS resource 1,2,3,4 |
| CSI-RS periodicity | slots | 80 for CSI-RS resource 1,2,3,4 |
| CSI-RS offset | slots | 40 for CSI-RS resource 1 and 2 41 for CSI-RS resource 3 and 4 |
| EPRE ratio to SSS | dB | 0Note 2 |
| TCI state |  | TCI.State.1 |
| Note 1: BW of TRS is configured same as the BW size of UE active BWP in the RRM test cases Note 2: Unless otherwise specified in the test case Note 3: If active BWP is larger than 52RBs, BW of TRS is configured as 52RBs. Otherwise, same as active BWP size. |  |  |

Table A.3.17.2.1-3: Aperiodic CSI-RS for tracking for SCS=120kHz Set 1

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | TRS.2.3 TDD |
| Bandwidth |  | BW of Active BWPNote 1,3 |
| SCS | kHz | 120 |
| First subcarrier index in the PRB used for CSI-RS |  | k0=0 for CSI-RS resource 1,2,3,4 |
| First OFDM symbol in the slot used for CSI-RS |  | l0 = 1 for CSI-RS resource 1 and 3 l0 = 5 for CSI-RS resource 2 and 4 |
| Number of CSI-RS ports (X) |  | 1 for CSI-RS resource 1,2,3,4 |
| CDM Type |  | ‘No CDM’ for CSI-RS resource 1,2,3,4 |
| Density (ρ) |  | 3 for CSI-RS resource 1,2,3,4 |
| aperiodicTriggeringOffsetL2 | slots | 2 |
| Aperiodic CSI-RS offset | slots | 2 for CSI-RS resource 1 and 2 3 for CSI-RS resource 3 and 4 |
| EPRE ratio to SSS | dB | 0Note 2 |
| TCI state |  | TCI.State.0 |
| Note 1: BW of TRS is configured same as the BW size of UE active BWP in the RRM test cases Note 2: Unless otherwise specified in the test case Note 3: If active BWP is larger than 52RBs, BW of TRS is configured as 52RBs. Otherwise, same as active BWP size. |  |  |

Table A.3.17.2.1-4: CSI-RS for tracking for SCS=120kHz Set 3

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | TRS.2.4 TDD |
| Bandwidth |  | BW of Active BWPNote 1,3 |
| SCS | kHz | 120 |
| First subcarrier index in the PRB used for CSI-RS |  | k0=0 for CSI-RS resource 1,2,3,4 |
| First OFDM symbol in the slot used for CSI-RS |  | l0 = 3 for CSI-RS resource 1 and 3 l0 = 7 for CSI-RS resource 2 and 4 |
| Number of CSI-RS ports (X) |  | 1 for CSI-RS resource 1,2,3,4 |
| CDM Type |  | ‘No CDM’ for CSI-RS resource 1,2,3,4 |
| Density (ρ) |  | 3 for CSI-RS resource 1,2,3,4 |
| CSI-RS periodicity | slots | 80 for CSI-RS resource 1,2,3,4 |
| CSI-RS offset | slots | 40 for CSI-RS resource 1 and 2 41 for CSI-RS resource 3 and 4 |
| EPRE ratio to SSS | dB | 0Note 2 |
| TCI state |  | TCI.State.2 |
| Note 1: BW of TRS is configured same as the BW size of UE active BWP in the RRM test cases Note 2: Unless otherwise specified in the test case Note 3: If active BWP is larger than 52RBs, BW of TRS is configured as 52RBs. Otherwise, same as active BWP size. |  |  |

Table A.3.17.2.1-5: CSI-RS for tracking for SCS=120kHz Set 4

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | TRS.2.5 TDD |
| Bandwidth |  | BW of Active BWPNote 1,3 |
| SCS | kHz | 120 |
| First subcarrier index in the PRB used for CSI-RS |  | k0=0 for CSI-RS resource 1,2,3,4 |
| First OFDM symbol in the slot used for CSI-RS |  | l0 = 4 for CSI-RS resource 1 and 3 l0 = 8 for CSI-RS resource 2 and 4 |
| Number of CSI-RS ports (X) |  | 1 for CSI-RS resource 1,2,3,4 |
| CDM Type |  | ‘No CDM’ for CSI-RS resource 1,2,3,4 |
| Density (ρ) |  | 3 for CSI-RS resource 1,2,3,4 |
| CSI-RS periodicity | slots | 80 for CSI-RS resource 1,2,3,4 |
| CSI-RS offset | slots | 40 for CSI-RS resource 1 and 2 41 for CSI-RS resource 3 and 4 |
| EPRE ratio to SSS | dB | 0Note 2 |
| TCI state |  | TCI.State.3 |
| Note 1: BW of TRS is configured same as the BW size of UE active BWP in the RRM test cases Note 2: Unless otherwise specified in the test case Note 3: If active BWP is larger than 52RBs, BW of TRS is configured as 52RBs. Otherwise, same as active BWP size. |  |  |

#### A.3.17.2.2	FDD

Table A.3.17.2.2-1: CSI-RS for tracking for SCS=120kHz Set 1

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | TRS.2.1 FDD |
| Bandwidth |  | BW of Active BWPNote 1,3 |
| SCS | kHz | 120 |
| First subcarrier index in the PRB used for CSI-RS |  | k0=0 for CSI-RS resource 1,2,3,4 |
| First OFDM symbol in the slot used for CSI-RS |  | l0 = 1 for CSI-RS resource 1 and 3 l0 = 5 for CSI-RS resource 2 and 4 |
| Number of CSI-RS ports (X) |  | 1 for CSI-RS resource 1,2,3,4 |
| CDM Type |  | ‘No CDM’ for CSI-RS resource 1,2,3,4 |
| Density (ρ) |  | 3 for CSI-RS resource 1,2,3,4 |
| CSI-RS periodicity | slots | 80 for CSI-RS resource 1,2,3,4 |
| CSI-RS offset | slots | 40 for CSI-RS resource 1 and 2 41 for CSI-RS resource 3 and 4 |
| EPRE ratio to SSS | dB | 0Note 2 |
| TCI state |  | TCI.State.0 |
| Note 1: BW of TRS is configured same as the BW size of UE active BWP in the RRM test cases Note 2: Unless otherwise specified in the test case Note 3: If active BWP is larger than 52RBs, BW of TRS is configured as 52RBs. Otherwise, same as active BWP size. |  |  |

## A.3.18	Additional definitions related to OTA testing for FR2 RRM test cases

### A.3.18.1	Introduction

FR2 RRM test cases are performed over the air (OTA). This clause provides additional definitions and clarifications on the OTA measurements and metrics defined or refered in the test cases.

### A.3.18.2	PRACH Power Measurement

PRACH power is measured as EIRP(Link=Link angle, Meas=Link angle) as defined in clause 3.1 of TS 38.101-2 [19].

## A.3.19	Test applicability for DAPS handover

### A.3.19.1	Introduction

In Annex A test cases for DAPS handover may be defined with cells in on same or different carrier frequency to verify intra-frequency, intra-band inter-frequency and inter-band inter-frequency DAPS handover RRM requirements, respectively.

### A.3.19.2	Principle of testing

To verify intra-frequency DAPS handover requirements

-	The UE capable of intra-frequency asynchronous DAPS handover on any band needs to be tested only in asynchronous scenario.

-	The UE not capable of intra-frequency asynchronous DAPS handover on any band but capable of synchronous DAPS handover on some band needs to be tested only in synchronous scenario.

To verify intra-band inter-frequency DAPS handover requirements

-	The UE capable of intra-band inter-frequency asynchronous DAPS handover on any band needs to be tested only in asynchronous scenario.

-	The UE not capable of intra-band inter-frequency asynchronous DAPS handover on any band but capable of intra-band inter-frequency synchronous DAPS handover on some band needs to be tested only in synchronous scenario.

To verify inter-band inter-frequency DAPS handover requirements

-	The UE capable of inter-band inter-frequency asynchronous DAPS handover on any band combination needs to be tested only in asynchronous scenario.

-	The UE not capable of inter-band inter-frequency asynchronous DAPS handover on any band combination but capable of inter-band inter-frequency synchronous DAPS handover on some band combination needs to be tested only in synchronous scenario.

## A.3.20	MsgA configurations

### A.3.20.1	Introduction

This clause provides the typical PRACH and PUSCH configurations for MsgA used for RRM test cases defined in Annex A. To note that for other parameters not listed in this clause, either it can be derived from the set up of each test or it is subjected to RAN5 specifications.

### A.3.20.2	MsgA configurations in FR1

#### A.3.20.2.1	FR1 MsgA configuration 1

FR1 MsgA configuration 1 in this clause provides the typical MsgA configuration for SSB-based contention based random access for 2-step RA type in FR1.

Table A.3.20.2.1-1: Parameters for FR1 MsgA configuration 1

| Field | Value | Comment |
| --- | --- | --- |
| msgA-prach-ConfigurationIndex | 102 | 10ms PRACH periodicity, and other detailed configuration defined in table 6.3.3.2-2 and table 6.3.3.2-3 in TS 38.211 [6]. |
| msgA-SubcarrierSpacing | Same as UL carrier SCS |  |
| msgA-totalNumberOfRA-Preambles | 48 | Total number of preambles used for contention based and contention free random access |
| numberOfRA-PreamblesGroupA | 48 | No group B. |
| msgA-PRACH-RootSequenceIndex | 0 | Logic sequence index = 0, resulting in root sequence = 1. |
| msgA-SSB-perRACH-OccasionAndCB-PreamblesPerSSB | oneFourth, n48 | OneFourth: 1 SSB associated with 4 RACH occasions n48: 48 contention based preambles per SSB |
| msgA-RO-FDM | One | One PRACH transmission occasions FDMed in one time instance. |
| ra-ContentionResolutionTimer | sf48 | 48 sub-frames |
| msgA-PreamblePowerRampingStep | dB2 |  |
| msgA-PreambleReceivedTargetPower | dBm-120 |  |
| preambleTransMax | n6 | Max number of RA preamble transmission performed before declaring a failure is 6 |
| msgB-ResponseWindow | sl10 | 10 slots |
| msgA-ZeroCorrelationZoneConfig | 11 | N-CS configuration, NCS = 23 |
| Backoff Parameter Index | 2 | 20ms, as defined in table 7.2-1 in TS 38.321 [7]. |
| msgA-MCS | 1 | MCS index for MsgA PUSCH |
| nrofSlotsMsgA-PUSCH | 1 | Number of slots containing one or multiple PUSCH occasions |
| nrofMsgA-PO-PerSlot | 1 | Number of time domain PUSCH occasions in each slot |
| msgA-PUSCH-TimeDomainOffset | 1 | A single time offset with respect to the start of each PRACH slot, counted as the number of slots |
| PUSCH start symbol | 0 |  |
| PUSCH allocation length | 14 |  |
| mappingTypeMsgA-PUSCH | typeA |  |
| nrofPRBs-PerMsgA-PO | 2 | Number of RBs per PUSCH occasion |
| nrofMsgA-PO-FDM | One | The number of MsgA PUSCH occasions FDMed in one time instance |
| msgA-DMRS-AdditionalPosition | pos1 | Position for additional DM-RS |
| msgA-PUSCH-NrofPorts | 1 | Configure 1 port per CDM group |
| msgA-DeltaPreamble | 3 | Power offset of msgA PUSCH relative to the preamble received target power |
| msgA-Alpha | alpha1 | Alpha value for MsgA PUSCH. Set 1 |
| deltaMCS | Disabled | Whether to apply delta MCS |
| Note: For further information see clause 6.3.2 in TS 38.331 [2]. |  |  |

#### A.3.20.2.2	FR1 MsgA configuration 2

FR1 PRACH configuration 2 in this clause provides the typical MsgA configuration for SSB based non-contention based random access for 2-step RA type in FR1.

Table A.3.20.2.2-1: Parameters for FR1 MsgA configuration 2

| Field | Value | Comment |
| --- | --- | --- |
| msgA-prach-ConfigurationIndex | 102 | 10ms PRACH periodicity, and other detailed configuration defined in table 6.3.3.2-2 and table 6.3.3.2-3 in TS 38.211 [6]. |
| msgA-SubcarrierSpacing | Same as UL carrier SCS |  |
| msgA-totalNumberOfRA-Preambles | 48 | Total number of preambles used for contention based and contention free random access |
| numberOfRA-PreamblesGroupA | 48 | No group B. |
| msgA-PRACH-RootSequenceIndex | 0 | Logic sequence index = 0, resulting in root sequence = 1. |
| ssb-perRACH-Occasion | oneFourth | OneFourth: 1 SSB associated with 4 RACH occasions |
| msgA-RO-FDM | One | One PRACH transmission occasions FDMed in one time instance. |
| msgA-PreamblePowerRampingStep | dB2 |  |
| msgA-PreambleReceivedTargetPower | dBm-120 |  |
| preambleTransMax | n6 | Max number of RA preamble transmission performed before declaring a failure is 6 |
| msgB-ResponseWindow | sl10 | 10 slots |
| msgA-ZeroCorrelationZoneConfig | 11 | N-CS configuration, NCS = 23 |
| Backoff Parameter Index | 2 | 20ms, as defined in table 7.2-1 in TS 38.321 [7]. |
| ssb-ResourceList | ra-PreambleIndex = 50 | Associated with SSB index 0. UE doesn’t use ssb-ResourceList and BFR-SSB-Resource IEs at the same time. UE doesn’t use this field if is transmitting CFRA to convey BFR. |
| BFR-SSB-Resource | ra-PreambleIndex = 50 | Associated with SSB index 0. UE doesn’t use ssb-ResourceList and BFR-SSB-Resource IEs at the same time. UE uses this field only if is transmitting CFRA to convey BFR |
| ra-ssb-OccasionMaskIndex | 1 | PRACH occasion index 1 is allowed |
| msgA-MCS | 1 | MCS index for MsgA PUSCH |
| nrofSlotsMsgA-PUSCH | 1 | Number of slots containing one or multiple PUSCH occasions |
| nrofMsgA-PO-PerSlot | 1 | Number of time domain PUSCH occasions in each slot |
| msgA-PUSCH-TimeDomainOffset | 1 | A single time offset with respect to the start of each PRACH slot, counted as the number of slots |
| PUSCH start symbol | 0 |  |
| PUSCH allocation length | 14 |  |
| mappingTypeMsgA-PUSCH | typeA |  |
| nrofPRBs-PerMsgA-PO | 2 | Number of RBs per PUSCH occasion |
| nrofMsgA-PO-FDM | One | The number of MsgA PUSCH occasions FDMed in one time instance |
| msgA-DMRS-AdditionalPosition | pos1 | Position for additional DM-RS |
| msgA-PUSCH-NrofPorts | 1 | Configure 1 port per CDM group |
| msgA-DeltaPreamble | 3 | Power offset of msgA PUSCH relative to the preamble received target power |
| msgA-Alpha | alpha1 | Alpha value for MsgA PUSCH. Set 1 |
| deltaMCS | Disabled | Whether to apply delta MCS |
| Note: For further information see clause 6.3.2 in TS 38.331 [2]. |  |  |

### A.3.20.3	MsgA configurations in FR2

#### A.3.20.3.1	FR2 MsgA configuration 1

FR2 MsgA configuration 1 in this clause provides the typical MsgA configuration for SSB-based contention based random access for 2-step RA type in FR2.

Table A.3.20.3.1-1: Parameters for FR2 MsgA configuration 1

| Field | Value | Comment |
| --- | --- | --- |
| msgA-prach-ConfigurationIndex | 190 | Preamble Format C2, with 10ms PRACH periodicity, and other detailed configurations defined in table 6.3.3.2-4 in TS 38.211 [6]. |
| msgA-SubcarrierSpacing | Same as UL carrier SCS |  |
| msgA-totalNumberOfRA-Preambles | 48 | Total number of preambles used for contention based and contention free random access |
| numberOfRA-PreamblesGroupA | 48 | No group B. |
| msgA-PRACH-RootSequenceIndex | 0 | Logic sequence index = 0, resulting in root sequence = 1. |
| msgA-SSB-perRACH-OccasionAndCB-PreamblesPerSSB | oneFourth, n48 | OneFourth: 1 SSB associated with 4 RACH occasions n48: 48 contention-based preambles per SSB |
| msgA-RO-FDM | One | One PRACH transmission occasions FDMed in one time instance. |
| ra-ContentionResolutionTimer | sf48 | 48 sub-frames |
| msgA-PreamblePowerRampingStep | dB2 |  |
| msgA-PreambleReceivedTargetPower | dBm-120 |  |
| preambleTransMax | n6 | Max number of RA preamble transmission performed before declaring a failure is 6 |
| msgB-ResponseWindow | sl10 | 10 slots |
| msgA-ZeroCorrelationZoneConfig | 11 | N-CS configuration, NCS = 23 |
| Backoff Parameter Index | 2 | 20 ms, as defined in table 7.2-1 in TS 38.321 [7]. |
| msgA-MCS | 1 | MCS index for MsgA PUSCH |
| nrofSlotsMsgA-PUSCH | 1 | Number of slots containing one or multiple PUSCH occasions |
| nrofMsgA-PO-PerSlot | 1 | Number of time domain PUSCH occasions in each slot |
| msgA-PUSCH-TimeDomainOffset | 1 | A single time offset with respect to the start of each PRACH slot, counted as the number of slots |
| PUSCH start symbol | 0 |  |
| PUSCH allocation length | 10 |  |
| mappingTypeMsgA-PUSCH | typeA |  |
| nrofPRBs-PerMsgA-PO | 2 | Number of RBs per PUSCH occasion |
| nrofMsgA-PO-FDM | One | The number of MsgA PUSCH occasions FDMed in one time instance |
| msgA-DMRS-AdditionalPosition | pos1 | Position for additional DM-RS |
| msgA-PUSCH-NrofPorts | 1 | Configure 1 port per CDM group |
| msgA-DeltaPreamble | 3 | Power offset of msgA PUSCH relative to the preamble received target power |
| msgA-Alpha | alpha1 | Alpha value for MsgA PUSCH. Set 1 |
| deltaMCS | Disabled | Whether to apply delta MCS |
| Note: For further information see clause 6.3.2 in TS 38.331 [2]. |  |  |

#### A.3.20.3.2	FR2 MsgA configuration 2

FR2 MsgA configuration 2 in this clause provides the typical MsgA configuration for SSB based non-contention based random access for 2-step RA type in FR2.

Table A.3.20.3.2-1: Parameters for FR2 MsgA configuration 2

| Field | Value | Comment |
| --- | --- | --- |
| msgA-prach-ConfigurationIndex | 190 | Preamble Format C2, with 10ms PRACH periodicity, and other detailed configurations defined in table 6.3.3.2-4 in TS 38.211 [6]. |
| msgA-SubcarrierSpacing | Same as UL carrier SCS |  |
| totalNumberOfRA-Preambles | 48 | Total number of preambles used for contention based and contention free random access |
| numberOfRA-PreamblesGroupA | 48 | No group B. |
| msgA-PRACH-RootSequenceIndex | 0 | Logic sequence index = 0, resulting in root sequence = 1. |
| ssb-perRACH-Occasion | oneFourth | OneFourth: 1 SSB associated with 4 RACH occasions |
| msgA-RO-FDM | One | One PRACH transmission occasions FDMed in one time instance. |
| msgA-PreamblePowerRampingStep | dB2 |  |
| msgA-PreambleReceivedTargetPower | dBm-120 |  |
| preambleTransMax | n6 | Max number of RA preamble transmission performed before declaring a failure is 6 |
| msgB-ResponseWindow | sl10 | 10 slots |
| msgA-ZeroCorrelationZoneConfig | 11 | N-CS configuration, NCS = 23 |
| Backoff Parameter Index | 2 | 20 ms, as defined in table 7.2-1 in TS 38.321 [7]. |
| ssb-ResourceList | ra-PreambleIndex = 50 | Associated with SSB index 0. UE doesn’t use ssb-ResourceList and BFR-SSB-Resource IEs at the same time. UE doesn’t use this field if is transmitting CFRA to convey BFR. |
| BFR-SSB-Resource | ra-PreambleIndex = 50 | Associated with SSB index 0. UE doesn’t use ssb-ResourceList and BFR-SSB-Resource IEs at the same time. UE uses this field only if is transmitting CFRA to convey BFR |
| ra-ssb-OccasionMaskIndex | 1 | PRACH occasion index 1 is allowed |
| msgA-MCS | 1 | MCS index for MsgA PUSCH |
| nrofSlotsMsgA-PUSCH | 1 | Number of slots containing one or multiple PUSCH occasions |
| nrofMsgA-PO-PerSlot | 1 | Number of time domain PUSCH occasions in each slot |
| msgA-PUSCH-TimeDomainOffset | 1 | A single time offset with respect to the start of each PRACH slot, counted as the number of slots |
| PUSCH start symbol | 0 |  |
| PUSCH allocation length | 10 |  |
| mappingTypeMsgA-PUSCH | typeA |  |
| nrofPRBs-PerMsgA-PO | 2 | Number of RBs per PUSCH occasion |
| nrofMsgA-PO-FDM | One | The number of MsgA PUSCH occasions FDMed in one time instance |
| msgA-DMRS-AdditionalPosition | pos1 | Position for additional DM-RS |
| msgA-PUSCH-NrofPorts | 1 | Configure 1 port per CDM group |
| msgA-DeltaPreamble | 3 | Power offset of msgA PUSCH relative to the preamble received target power |
| msgA-Alpha | alpha1 | Alpha value for MsgA PUSCH. Set 1 |
| deltaMCS | Disabled | Whether to apply delta MCS |
| Note: For further information see clause 6.3.2 in TS 38.331 [2]. |  |  |

## A.3.20A	MsgA configurations under CCA

### A.3.20A.1	Introduction

This clause provides the typical PRACH and PUSCH configurations for MsgA used for RRM test cases defined in Annex A. To note that for other parameters not listed in this clause, either it can be derived from the set up of each test or it is subjected to RAN5 specifications.

### A.3.20A.2	MsgA configurations in FR1

#### A.3.20A.2.1	FR1 MsgA configuration 1 under CCA

FR1 MsgA configuration 1 under CCA in this clause provides the typical MsgA configuration for SSB-based contention based random access for 2-step RA type in FR1.

Table A.3.20A.2.1-1: Parameters for FR1 MsgA configuration 1 under CCA

| Field | Value | Comment |
| --- | --- | --- |
| msgA-prach-ConfigurationIndex | 102 | 10ms PRACH periodicity, and other detailed configuration defined in table 6.3.3.2-2 and table 6.3.3.2-3 in TS 38.211 [6]. |
| msgA-SubcarrierSpacing | Same as UL carrier SCS |  |
| msgA-totalNumberOfRA-Preambles | 48 | Total number of preambles used for contention based and contention free random access |
| numberOfRA-PreamblesGroupA | 48 | No group B. |
| msgA-PRACH-RootSequenceIndex | 0 | Logic sequence index = 0, resulting in root sequence = 1. |
| msgA-SSB-perRACH-OccasionAndCB-PreamblesPerSSB | oneFourth, n48 | OneFourth: 1 SSB associated with 4 RACH occasions n48: 48 contention based preambles per SSB |
| msgA-RO-FDM | One | One PRACH transmission occasions FDMed in one time instance. |
| ra-ContentionResolutionTimer | sf48 | 48 sub-frames |
| msgA-PreamblePowerRampingStep | dB2 |  |
| msgA-PreambleReceivedTargetPower | dBm-114 | Increased by 6 dB compared with FR1 MsgA configuration 1 for random access test with UL CCA failures. |
| preambleTransMax | n20 | Max number of RA preamble transmission performed before declaring a failure is 20 to account for CCA failures |
| msgB-ResponseWindow | sl20 | 20 slots |
| msgA-ZeroCorrelationZoneConfig | 11 | N-CS configuration, NCS = 23 |
| Backoff Parameter Index | 2 | 20ms, as defined in table 7.2-1 in TS 38.321 [7]. |
| msgA-MCS | 1 | MCS index for MsgA PUSCH |
| nrofSlotsMsgA-PUSCH | 1 | Number of slots containing one or multiple PUSCH occasions |
| nrofMsgA-PO-PerSlot | 1 | Number of time domain PUSCH occasions in each slot |
| msgA-PUSCH-TimeDomainOffset | 1 | A single time offset with respect to the start of each PRACH slot, counted as the number of slots |
| PUSCH start symbol | 0 |  |
| PUSCH allocation length | 14 |  |
| mappingTypeMsgA-PUSCH | typeA |  |
| nrofPRBs-PerMsgA-PO | 2 | Number of RBs per PUSCH occasion |
| nrofMsgA-PO-FDM | One | The number of MsgA PUSCH occasions FDMed in one time instance |
| msgA-DMRS-AdditionalPosition | pos1 | Position for additional DM-RS |
| msgA-PUSCH-NrofPorts | 1 | Configure 1 port per CDM group |
| msgA-DeltaPreamble | 3 | Power offset of msgA PUSCH relative to the preamble received target power |
| msgA-Alpha | alpha1 | Alpha value for MsgA PUSCH. Set 1 |
| deltaMCS | Disabled | Whether to apply delta MCS |
| Note: For further information see clause 6.3.2 in TS 38.331 [2]. |  |  |

#### A.3.20A.2.2	FR1 MsgA configuration 2 under CCA

FR1 PRACH configuration 2 under CCA in this clause provides the typical MsgA configuration for SSB based non-contention based random access for 2-step RA type in FR1.

Table A.3.20A.2.2-1: Parameters for FR1 MsgA configuration 2 under CCA

| Field | Value | Comment |
| --- | --- | --- |
| msgA-prach-ConfigurationIndex | 102 | 10ms PRACH periodicity, and other detailed configuration defined in table 6.3.3.2-2 and table 6.3.3.2-3 in TS 38.211 [6]. |
| msgA-SubcarrierSpacing | Same as UL carrier SCS |  |
| msgA-totalNumberOfRA-Preambles | 48 | Total number of preambles used for contention based and contention free random access |
| numberOfRA-PreamblesGroupA | 48 | No group B. |
| msgA-PRACH-RootSequenceIndex | 0 | Logic sequence index = 0, resulting in root sequence = 1. |
| ssb-perRACH-Occasion | oneFourth | OneFourth: 1 SSB associated with 4 RACH occasions |
| msgA-RO-FDM | One | One PRACH transmission occasions FDMed in one time instance. |
| msgA-PreamblePowerRampingStep | dB2 |  |
| msgA-PreambleReceivedTargetPower | dBm-114 | Increased by 6 dB compared with FR1 MsgA configuration 2 for random access test with UL CCA failures. |
| preambleTransMax | n20 | Max number of RA preamble transmission performed before declaring a failure is 20 to account for CCA failures |
| msgB-ResponseWindow | sl20 | 20 slots |
| msgA-ZeroCorrelationZoneConfig | 11 | N-CS configuration, NCS = 23 |
| Backoff Parameter Index | 2 | 20ms, as defined in table 7.2-1 in TS 38.321 [7]. |
| ssb-ResourceList | ra-PreambleIndex = 50 | Associated with SSB index 0. UE doesn’t use ssb-ResourceList and BFR-SSB-Resource IEs at the same time. UE doesn’t use this field if is transmitting CFRA to convey BFR. |
| BFR-SSB-Resource | ra-PreambleIndex = 50 | Associated with SSB index 0. UE doesn’t use ssb-ResourceList and BFR-SSB-Resource IEs at the same time. UE uses this field only if is transmitting CFRA to convey BFR |
| ra-ssb-OccasionMaskIndex | 1 | PRACH occasion index 1 is allowed |
| msgA-MCS | 1 | MCS index for MsgA PUSCH |
| nrofSlotsMsgA-PUSCH | 1 | Number of slots containing one or multiple PUSCH occasions |
| nrofMsgA-PO-PerSlot | 1 | Number of time domain PUSCH occasions in each slot |
| msgA-PUSCH-TimeDomainOffset | 1 | A single time offset with respect to the start of each PRACH slot, counted as the number of slots |
| PUSCH start symbol | 0 |  |
| PUSCH allocation length | 14 |  |
| mappingTypeMsgA-PUSCH | typeA |  |
| nrofPRBs-PerMsgA-PO | 2 | Number of RBs per PUSCH occasion |
| nrofMsgA-PO-FDM | One | The number of MsgA PUSCH occasions FDMed in one time instance |
| msgA-DMRS-AdditionalPosition | pos1 | Position for additional DM-RS |
| msgA-PUSCH-NrofPorts | 1 | Configure 1 port per CDM group |
| msgA-DeltaPreamble | 3 | Power offset of msgA PUSCH relative to the preamble received target power |
| msgA-Alpha | alpha1 | Alpha value for MsgA PUSCH. Set 1 |
| deltaMCS | Disabled | Whether to apply delta MCS |
| Note: For further information see clause 6.3.2 in TS 38.331 [2]. |  |  |

## A.3.21	V2X sidelink communication

### A.3.21.1	Introduction

This clause also defines the principle and the reference configurations that are applicable to test cases verifying RRM core requirements for V2X sidelink communication.

### A.3.21.2	Reference resource pool configurations for V2X Sidelink Communication

Table A.3.21.2-1: V2X sidelink SL-BWP configuration for NR

| Field | Value | Comment |
| --- | --- | --- |
| SL-BWP-ConfigCommon-r16 |  |  |
| sl-BWP-Generic-r16 |  |  |
| sl-LengthSymbols-r16 | sym14 | All 14 symbols in a slot without S-SSB are used for sidelink |
| sl-StartSymbol-r16 | sym0 | Symbol #0 is the starting symbol used for sidelink in a slot without S-SSB |
| sl-BWP-PoolConfigCommon-r16 |  |  |
| sl-RxPool-r16 |  | Indicates the resource pool for reception on the configured BWP. 1 entry |
| SL-ResourcePool-r16[1] | Set according to Table A.3.21.2-2 | Entry 1 |
| sl-TxPoolSelectedNormal-r16 |  | Indicates the resources pool for mode 2 sidelink communication on the configured BWP. 1 entry |
| SL-ResourcePoolConfig-r16[1] |  | Entry 1 |
| sl-TxPoolExceptional-r16 | Not present |  |

Table A.3.21.2-2: V2X sidelink resource pool configuration for NR

| Field | Value | Comment |
| --- | --- | --- |
| SL-ResourcePool-r16 |  |  |
| sl-PSCCH-Config-r16 | Set according to Table A.3.21.3-1 |  |
| sl-PSSCH-Config-r16 | Set according to Table A.3.21.3-2 |  |
| sl-PSFCH-Config-r16 | Not present |  |
| sl-SyncAllowed-r16 |  | Indicates the allowed synchronization reference(s) which is (are) allowed to use the configured resource pool. |
| gnss-Sync-r16 | true |  |
| gnbEnb-Sync-r16 | true |  |
| ue-Sync-r16 | true |  |
| sl-SubchannelSize-r16 | n10 | Subchannel bandwidth is 10 RB |
| sl-StartRB-Subchannel-r16 | 0 | The offset of lowest RB index of the subchannel with the lowest index in the resource pool with respect to the lowest RB index of a SL BWP |
| sl-NumSubchannel-r16 | 1 | Number of subchannels in resource pool |
| sl-UE-SelectedConfigRP-r16 |  |  |
| sl-Thres-RSRP-List-r16 | Set according to the specific test configuration | Indicates a list of 64 thresholds, and the threshold should be selected based on the priority in the decoded SCI and the priority in the SCI to be transmitted. A resource is excluded if it is indicated or reserved by a decoded SCI and PSSCH RSRP in the associated data resource is above a threshold. |
| sl-MultiReserveResource-r16 | Not present |  |
| sl-MaxNumPerReserve-r16 | n2 | At most 2 PSCCH/PSSCH resources can be reserved by a single SCI. |
| sl-SensingWindow-r16 | ms100 | Length of resource sensing window specified in TS 38.214 [26] subclause 8.1.4. which is 100ms. |
| sl-SelectionWindowList-r16 |  | Parameter that determines the end of the selection window for each priority level 8 entries |
| SL-SelectionWindowConfig-r16[k] |  | entry k (k=1..8) |
| sl-Priority-r16 | k | for priority level = k |
| sl-SelectionWindow-r16 | n20 | Length of resource selection window specified in TS 38.214 [26] subclause 8.1.4. which is 20∙2μslots, where µ=0,1,2,3 refers to SCS 15,30,60,120 kHz respectively |
| sl-ResourceReservePeriodList-r16 | Not present |  |
| sl-RS-ForSensing-r16 | pssch | PSSCH-RSRP measurement is used in the sensing operation. |
| sl-RxParametersNcell | Not present |  |
| sl-ZoneConfigMCR-List-r16 | Not present |  |
| sl-PreemptionEnable-r16 | enabled |  |
| sl-MinMaxMCS-List-r16 |  | 1 entry |
| SL-MinMaxMCS-Config-r16[1] |  | Entry 1 |
| sl-MCS-Table-r16 | qam64 | TS 38.214 [26] Table 5.1.3.1-1 is the MCS table used in the resource pool. |
| sl-TimeResource-r16 | 1111111111 1111111111 | Every slot in a period of 20 slots during a SFN or DFN cycle can be used for sidelink |
| SL-TxPercentageList-r16 |  |  |
| SL-TxPercentageConfig-r16 |  |  |
| sl-TxPercentage-r16 | p20 | Indicates the portion of candidate single-slot PSSCH resources over the toal resources. Value p20 corresponds to 20%, and so on. |

Table A.3.21.2-3: V2X sidelink UE autonomous resource selection configuration for NR

| Field | Value | Comment |
| --- | --- | --- |
| SL-UE-SelectedConfig-r16 |  |  |
| sl-PSSCH-TxConfigList-r16 |  | 1 entry |
| SL-PSSCH-TxConfig-r16[1] |  | Entry 1 |
| sl-TypeTxSync-r16 | Not present | When this filed is absent, the configuration is applicable for all synchronization reference types. |
| sl-ThresUE-Speed-r16 | kmph200 | UE shall apply the parameters in sl-ParametersAboveThres-r16 if UE absolute speed is higher than 200 km/h, otherwise UE shall apply the parameters in sl-ParametersBelowThres-r16 |
| sl-ParametersAboveThres-r16 |  |  |
| sl-MinMCS-PSSCH-r16 | 0 | The minimum MCS index value can be used for PSSCH transmission. |
| sl-MaxMCS-PSSCH-r16 | 15 | The maximum MCS index value can be used for PSSCH transmission. |
| sl-MinSubChannelNumPSSCH-r16 | 1 | The minimum number of subchannels can be used for PSSCH transmission. |
| sl-MaxSubchannelNumPSSCH-r16 | 1 | The maximum number of subchannels can be used for PSSCH transmission. |
| sl-MaxTxTransNumPSSCH-r16 | 1 | The maximum transmission number for PSSCH (including new transmission and retransmission). |
| sl-MaxTxPower-r16 | Not present | Not applicable |
| sl-ParametersBelowThres-r16 |  |  |
| sl-MinMCS-PSSCH-r16 | 4 | Same as above |
| sl-MaxMCS-PSSCH-r16 | 25 | Same as above |
| sl-MinSubChannelNumPSSCH-r16 | 1 | Same as above |
| sl-MaxSubchannelNumPSSCH-r16 | 1 | Same as above |
| sl-MaxTxTransNumPSSCH-r16 | 1 | Same as above |
| sl-MaxTxPower-r16 | Not present | Same as above |
| sl-ProbResourceKeep-r16 | v0dot8 | The probability of UE keeping current resource is 80% when the resource reselection counter reaches 0 (see TS 38.321 [7]). |
| sl-ReselectAfter-r16 | n1 | Resource reselection is triggered after 1 sidelink transmission is skipped (see TS 38.321 [7]). |

### A.3.21.3	Reference measurement channels for V2X Sidelink Communication

Table A.3.21.3-1: PSCCH Reference Measurement Channels

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Reference channel |  |  | CC.1A HD |
| Channel bandwidth |  | MHz | Note2 |
| Number of PSCCH symbols per slot |  |  | 2 |
| Number of PSCCH RB |  |  | 10 |
| Modulation |  |  | QPSK |
| Information Bit Payload (without CRC) |  | Bits | 26 |
| Information Bit | Number of DMRS ports |  | 0 (1 port) |
|  | Priority |  | As set by higher layers |
|  | Resource reservation period |  | N/A |
|  | Modulation and coding scheme |  | Set as the PSSCH MCS specified in the test |
|  | DMRS pattern |  | 0 (2 DMRS) |
|  | 2nd stage SCI format |  | 00 (SCI format 2-A) |
|  | Beta offset indicator |  | Set as specified in the test |
|  | Frequency resource assignment |  | Set as per PSSCH RB allocation specific in the test |
|  | Time resource assignment |  | Set as per PSSCH slot allocation specific in the test |
|  | Reserved bits |  | Set all these bits to 0 |
| Transport block CRC |  | Bits | 24 |
| Binary Channel Bits (see Note 1) |  | Bits | 360 |
| Note 1: Binary channel bits calculated under assumption of 2 CP-OFDM symbols per subframe. Note 2: Channel bandwidth depends on test configuration. |  |  |  |

Table A.3.21.3-2: PSSCH Reference Measurement Channels

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | CD.1A HD |
| Sidelink transmission mode |  | 2 |
| Channel bandwidth | MHz | Note1 |
| Allocated PSSCH resource blocks |  | 10 |
| Number of PSSCH symbols per slot |  | 10 |
| Modulation |  | QPSK |
| Target Code Rate |  | 1/3 |
| Information Bit Payload (Transport block size) | Bits | 672 |
| Transport block CRC | Bits | 24 |
| Number of PSSCH HARQ retransmissions |  | 0 |
| Binary Channel Bits | Bits | 2160 |
| Note 1: Channel bandwidth depends on test configuration. Note 2: 2nd state SCI and PSFCH are not allocated per slot. |  |  |

### A.3.21.4	Reference SL-DRX configurations

#### A.3.21.4.1	SL-DRX Configuration 1: SL-DRX cycle = 40 ms

Table A.3.21.4.1-1: SL-DRX.1: SL-DRX cycle = 40 ms

| Field | Value |
| --- | --- |
| sl-DRX-GC-BC-OnDurationTimer | 2 ms |
| sl-DRX-GC-InactivityTimer | 2 ms |
| sl-DRX-GC-RetransmissionTimer | 16 slot |
| sl-DRX-GC-BC-CyclelongDRX | 40 ms |
| Note: This SL-DRX configuration is applicable for V2X sidelink communication. For further information see clause 6.3.5 in TS 38.331 [2]. |  |

#### A.3.21.4.2	SL-DRX Configuration 2: SL-DRX cycle = 320 ms

Table A.3.21.4.2-1: SL-DRX.2: SL-DRX cycle = 320 ms

| Field | Value |
| --- | --- |
| sl-DRX-GC-BC-OnDurationTimer | 2 ms |
| sl-DRX-GC-InactivityTimer | 2 ms |
| sl-DRX-GC-RetransmissionTimer | 16 slot |
| sl-DRX-GC-BC-CyclelongDRX | 320 ms |
| Note: This SL-DRX configuration is applicable for V2X sidelink communication. For further information see clause 6.3.5 in TS 38.331 [2]. |  |

#### A.3.21.4.3	SL-DRX Configuration 3: SL-DRX cycle = 640 ms

Table A.3.21.4.3-1: SL-DRX.3: SL-DRX cycle = 640 ms

| Field | Value |
| --- | --- |
| sl-DRX-GC-BC-OnDurationTimer | 2 ms |
| sl-DRX-GC-InactivityTimer | 2 ms |
| sl-DRX-GC-RetransmissionTimer | 16 slot |
| sl-DRX-GC-BC-CyclelongDRX | 640 ms |
| Note: This SL-DRX configuration is applicable for V2X sidelink communication. For further information see clause 6.3.5 in TS 38.331 [2]. |  |

## A.3.22	CSI-IM configurations

### A.3.22.1	FDD

Table A.3.22.1-1: CSI-IM Reference Measurement Channels for SCS=15kHz

|  | CSI-IM.1.1 FDD | CSI-IM.1.2 FDD | CSI-IM.1.3 FDD |
| --- | --- | --- | --- |
| Resource Type | Periodic | aperiodic | periodic |
| Resource Set Config |  |  |  |
| csi-IM-ResourceSetId | 0 | 0 | 0 |
| Resource Config |  |  |  |
| csi-IM-ResourceId | 0 for resource #0 | 10 for resource #0 | 2 for resource #0 |
|  | 1 for resource #1 | 11 for resource #1 | 3 for resource #1 |
| csi-IM-ResourceElementPattern | pattern1 | pattern1 | pattern1 |
| subcarrierLocation-p1 | s0 | s0 | s0 |
| symbolLocation-p1 | 6 for resource #0 | 7 for resource #0 | 6 for resource #0 |
|  | 10 for resource #1 | 11 for resource #1 | 10 for resource #1 |
| Period (slots) | slot20 | n.a. | slot10 |
| Offset | 1 | n.a. | 2 |
| startingRB | 0 | 0 | 0 |
| nrofRBs | 276 (Note 1) | 276 (Note 1) | 276 (Note 1) |
| Note 1: If the configured value of PRBs is larger than the width of the corresponding BWP relevant for the test case, the test Equipment shall implement CSI-RS only in the width of that BWP. |  |  |  |

### A.3.22.2	TDD

Table A.3.22.2-1: CSI-IM Reference Measurement Channels for SCS=15kHz

|  | CSI-IM.1.1 TDD | CSI-IM.1.2 TDD | CSI-IM.1.3 TDD |
| --- | --- | --- | --- |
| Resource Type | periodic | aperiodic | periodic |
| Resource Set Config |  |  |  |
| csi-IM-ResourceSetId | 0 | 0 | 0 |
| Resource Config |  |  |  |
| csi-IM-ResourceId | 0 for resource #0 | 10 for resource #0 | 2 for resource #0 |
|  | 1 for resource #1 | 11 for resource #1 | 3 for resource #1 |
| csi-IM-ResourceElementPattern | pattern1 | pattern1 | pattern1 |
| subcarrierLocation-p1 | s0 | s0 | s0 |
| symbolLocation-p1 | 6 for resource #0 | 7 for resource #0 | 6 for resource #0 |
|  | 10 for resource #1 | 11 for resource #1 | 10 for resource #1 |
| Period (slots) | slot20 | n.a. | slot10 |
| Offset | 1 | n.a. | 2 |
| startingRB | 0 | 0 | 0 |
| nrofRBs | 276 (Note 1) | 276 (Note 1) | 276 (Note 1) |
| Note 1: If the configured value of PRBs is larger than the width of the corresponding BWP relevant for the test case, the test Equipment shall implement CSI-RS only in the width of that BWP. |  |  |  |

Table A.3.22.2-2: CSI-IM Reference Measurement Channels for SCS=30kHz

|  | CSI-IM.2.1 TDD | CSI-IM.2.2 TDD | CSI-IM.2.3 TDD |
| --- | --- | --- | --- |
| Resource Type | periodic | aperiodic | periodic |
| Resource Set Config |  |  |  |
| csi-IM-ResourceSetId | 0 | 0 | 0 |
| Resource Config |  |  |  |
| csi-IM-ResourceId | 0 for resource #0 | 10 for resource #0 | 2 for resource #0 |
|  | 1 for resource #1 | 11 for resource #1 | 3 for resource #1 |
| csi-IM-ResourceElementPattern | pattern1 | pattern1 | pattern1 |
| subcarrierLocation-p1 | s0 | s0 | s0 |
| symbolLocation-p1 | 6 for resource #0 | 7 for resource #0 | 6 for resource #0 |
|  | 10 for resource #1 | 11 for resource #1 | 10 for resource #1 |
| Period (slots) | slot40 | n.a. | slot40 |
| Offset | 2 | n.a. | 4 |
| startingRB | 0 | 0 | 0 |
| nrofRBs | 276 (Note 1) | 276 (Note 1) | 276 (Note 1) |
| Note 1: If the configured value of PRBs is larger than the width of the corresponding BWP relevant for the test case, the test Equipment shall implement CSI-RS only in the width of that BWP. |  |  |  |

Table A.3.22.2-3: CSI-RS Reference Measurement Channels for SCS=120kHz

|  | CSI-IM.3.1 TDD | CSI-IM.3.2 TDD | CSI-IM.3.3 TDD |
| --- | --- | --- | --- |
| Resource Type | periodic | aperiodic | periodic |
| Resource Set Config |  |  |  |
| csi-IM-ResourceSetId | 0 | 0 | 0 |
| Resource Config |  |  |  |
| csi-IM-ResourceId | 0 for resource #0 | 10 for resource #0 | 2 for resource #0 |
|  | 1 for resource #1 | 11 for resource #1 | 3 for resource #1 |
| csi-IM-ResourceElementPattern | pattern1 | pattern1 | pattern1 |
| subcarrierLocation-p1 | s0 | s0 | s0 |
| symbolLocation-p1 | 6 for resource #0 | 7 for resource #0 | 6 for resource #0 |
|  | 10 for resource #1 | 11 for resource #1 | 10 for resource #1 |
| Period (slots) | slot160 | n.a. | slot80 |
| Offset | 8 | n.a. | 16 |
| startingRB | 0 | 0 | 0 |
| nrofRBs | 276 (Note 1) | 276 (Note 1) | 276 (Note 1) |
| Note 1: If the configured value of PRBs is larger than the width of the corresponding BWP relevant for the test case, the test Equipment shall implement CSI-RS only in the width of that BWP. |  |  |  |

## A.3.23	Spatial Relation Configuration

### A.3.23.1	Introduction

This clause provides the configurations for spatial relation towards either SSB or CSI-RS. The spatial relation defined in this clause are configured in each test when applicable to indicate spatial setting for certain UL signals with the referenceSignal configured in the spatial relation.

### A.3.23.2	Spatial Relation

Table A.3.23.2-1: PUCCH Spatial Relation

| Parameter | PUCCH.SRI.0 | PUCCH.SRI.1 |
| --- | --- | --- |
| pucch-SpatialRelationInfoId | Id0 | Id1 |
| referenceSignal | SSB0 | SSB1 |
| PUCCH-PathlossReferenceRS | SSB0 | SSB1 |
| Note 1: referenceSignal configurations towards which the spatial relation are configured in a test-specific manner. |  |  |

Table A.3.23.2-2: SRS Spatial Relation

| Parameter | SRS.SRI0 | SRS.SRI1 |
| --- | --- | --- |
| srs-SpatialRelationInfoId | Id0 | Id1 |
| referenceSignal | SSB0 | SSB1 |
| Note 1: referenceSignal configurations towards which the spatial relation are configured in a test-specific manner. |  |  |

## A.3.24	SRS configuration

Table A.3.24-1: Sounding Reference Symbol Configuration for SCS=15kHz

|  | SRS.1 TDD | SRS.4 TDD | PDC-SRS.1 | POS-SRS.1 |  |
| --- | --- | --- | --- | --- | --- |
| Field | Value |  |  |  | Comment |
| c-SRS | 12 | 12 | Same as NRB,c in the test case | Same as NRB,c in the test case |  |
| b-SRS | 0 | 0 | 0 | n.a. |  |
| b-hop | 0 | 0 | 0 | n.a. | Frequency hopping is disabled |
| groupOrSequenceHopping | neither | neither | neither | neither | No group or sequence hopping |
| freqDomainPosition | 0 | 0 | 0 | 0 | Frequency domain position of SRS |
| freqDomainShift | 0 | 0 | 0 | 0 |  |
| pathlossReferenceRS ssb-Index | 0 | 0 | 0 | 0 | SSB #0 is used for SRS path loss estimation |
| usage | antennaSwitching | antennaSwitching | usagePDC-r17 | n.a. |  |
| startPositionNote 1 | 1 | 1 | 5 | 5 | resourceMapping setting |
| nrofSymbols Note 1 | 1 | 1 | 4 | 4 |  |
| repetitionFactor | n1 | n1 | n.a. | n.a. | without repetition. |
| transmissionComb | n2 | n2 | n4 | n4 |  |
| combOffset-n2 | 0 | 0 | 0 | 0 | transmissionComb setting |
| cyclicShift-n2 | 0 | 0 | 0 | 0 |  |
| nrofSRS-Ports Note 1 | port1 | port1 | port1 | port1 | Number of antenna ports used for SRS transmission |
| resourceType | Periodic | Aperiodic | Periodic | Periodic |  |
| periodicityAndOffset-p Note 1 | sl40, 1 | N/A | sl160, 20 | sl160, 20 | SRS transmission periodicity |
| Note 1: For test cases in A.4.5.2.11, A.4.5.2.12, A.6.5.2.3, A.6.5.2.4, the startPosition, nrofSymbols, nrofSRS-Ports, periodicityAndOffset-p are specified in each test cases. |  |  |  |  |  |

Table A.3.24-2: Sounding Reference Symbol Configuration for SCS=30kHz

|  | SRS.2 TDD | SRS.5 TDD | PDC-SRS.2 | POS-SRS.2 |  |
| --- | --- | --- | --- | --- | --- |
| Field | Value |  |  |  | Comment |
| c-SRS | 24 | 24 | Same as NRB,c in the test case | Same as NRB,c in the test case |  |
| b-SRS | 0 | 0 | 0 | n.a. |  |
| b-hop | 0 | 0 | 0 | n.a. | Frequency hopping is disabled |
| groupOrSequenceHopping | neither | neither | neither | neither | No group or sequence hopping |
| freqDomainPosition | 0 | 0 | 0 | 0 | Frequency domain position of SRS |
| freqDomainShift | 0 | 0 | 0 | 0 |  |
| pathlossReferenceRS ssb-Index | 0 | 0 | 0 | 0 | SSB #0 is used for SRS path loss estimation |
| usage | antennaSwitching | antennaSwitching | usagePDC-r17 | n.a. |  |
| startPosition Note 1 | 3 | 3 | 5 | 5 | resourceMapping setting |
| nrofSymbols Note 1 | 1 | 1 | 4 | 4 | SRS symbols belong to the same SRS resource. |
| repetitionFactor | n1 | n1 | n.a. | n.a. | without repetition. |
| transmissionComb | n2 | n2 | n4 | n4 |  |
| combOffset-n2 | 0 | 0 | 0 | 0 | transmissionComb setting |
| cyclicShift-n2 | 0 | 0 | 0 | 0 |  |
| nrofSRS-Ports Note 1 | port1 | port1 | port1 | port1 | Number of antenna ports used for SRS resource transmission |
| resourceType | Periodic | Aperiodic | Periodic | Periodic |  |
| periodicityAndOffset-p Note 1 | sl80, 3 | N/A | Sl320, 40 | Sl320, 40 | SRS transmission periodicity |
| Note 1: For test cases in A.4.5.2.11, A.4.5.2.12, A.6.5.2.3, A.6.5.2.4, the startPosition, nrofSymbols, nrofSRS-Ports, periodicityAndOffset-p are specified in each test cases. |  |  |  |  |  |

Table A.3.24-3: Sounding Reference Symbol Configuration for SCS=120kHz

|  | SRS.3 TDD | SRS.6 TDD | PDC-SRS.3 | POS-SRS.3 |  |
| --- | --- | --- | --- | --- | --- |
| Field | Value |  |  |  | Comment |
| c-SRS | 17 | 17 | Same as NRB,c in the test case | Same as NRB,c in the test case |  |
| b-SRS | 0 | 0 | 0 | n.a. |  |
| b-hop | 0 | 0 | 0 | n.a. | Frequency hopping is disabled |
| groupOrSequenceHopping | neither | neither | neither | neither | No group or sequence hopping |
| freqDomainPosition | 0 | 0 | 0 | 0 | Frequency domain position of SRS |
| freqDomainShift | 0 | 0 | 0 | 0 |  |
| pathlossReferenceRS ssb-Index | 0 | 0 | 0 | 0 | SSB #0 is used for SRS path loss estimation |
| usage | antennaSwitching | antennaSwitching | usagePDC-r17 | n.a. |  |
| startPosition | 1 | 1 | 5 | 5 | resourceMapping setting |
| nrofSymbols | 1 | 1 | 4 | 4 | SRS symbols belong to the same SRS resource. |
| repetitionFactor | n1 | n1 | n1 | n.a. | without repetition. |
| transmissionComb | n2 | n2 | n4 | n4 |  |
| combOffset-n2 | 0 | 0 | 0 | 0 | transmissionComb setting |
| cyclicShift-n2 | 0 | 0 | 0 | 0 |  |
| nrofSRS-Ports | port1 | port1 | port1 | port1 | Number of antenna ports used for SRS resource transmission |
| resourceType | Periodic | Aperiodic | Periodic | Periodic |  |
| periodicityAndOffset-p | sl320, 3 | N/A | Sl1280, 160 | Sl1280, 160 | SRS transmission periodicity |

## A.3.25	Channel bandwidth (CBW) configurations

### A.3.25.1	DL UE specific CBW

Table A.3.25.1-1: DL CBW patterns for UE specific CBW configuration

| BWP Parameters | Unit | Values |  |
| --- | --- | --- | --- |
| Reference CBW |  | DLCBW.1.1 | DLCBW.1.2 |
| OffsetToCarrier | RB | RBy Note 2 | RBx Note 1 |
| carrierBandwidth | RB | Same as RF channel defined in each test | Same as RF channel defined in each test |
| Note 1: RBx is offset in frequency domain between Point A (lowest subcarrier of common RB 0) and the lowest usable subcarrier on this carrier. Note that RBx has to be within the CBW of BS. Note 2: RBy is offset in frequency domain between Point A (lowest subcarrier of common RB 0) and the lowest usable subcarrier on this carrier. Note that RBy has to be within the CBW of BS and different with RBx, and CBW should include SSB and CORESET#0. |  |  |  |

### A.3.25.2	UL UE specific CBW

Table A.3.25.2-1: UL CBW patterns for UE specific CBW configuration

| BWP Parameters | Unit | Values |  |
| --- | --- | --- | --- |
| Reference CBW |  | ULCBW.1.1 | ULCBW.1.2 |
| OffsetToCarrier | RB | RBy Note 2 | RBx Note 1 |
| carrierBandwidth | RB | Same as RF channel defined in each test | Same as RF channel defined in each test |
| Note 1: RBx is offset in frequency domain between Point A (lowest subcarrier of common RB 0) and the lowest usable subcarrier on this carrier. Note that RBx has to be within the CBW of BS. Note 2: RBy is offset in frequency domain between Point A (lowest subcarrier of common RB 0) and the lowest usable subcarrier on this carrier. Note that RBy has to be within the CBW of BS and different with RBx. |  |  |  |

## A.3.26	CCA model

### A.3.26.1	Introduction

The CCA model is used in some RRM test cases with at least one cell on a carrier frequency with CCA. The intention with the CCA model is to emulate in the test equipment the behaviour of a gNB or UE which performs channel measurement to check that the channel is clear prior to performing one or more downlink, uplink or sidelink transmissions.

### A.3.26.2	CCA model for operation on a carrier frequency with CCA in FR1

#### A.3.26.2.1	DL CCA model

The same DL CCA model is applicable regardless of whether DRX cycle is used or not with the following differences:

-	The counter, lCCA, is used to monitor the number of unavailable DBT samples withing an evaluation window, WCCA_DL. DBT samples outside of the evaluation window WCCA_DL are discarded.

If DRX cycle is not used then prior to each DBT window, the test equipment shall determine whether the DL CCA attempt is successful (i.e., the corresponding signals have to be transmitted), based on probability PCCA_DL of successful DL CCA configured in the corresponding test case. If DRX cycle is not used, then the DL CCA model shall increment the counter lCCA for every unavailable DBT sample due to DL CCA failure.

-	If DRX cycle is used, then the DL CCA model shall increment the counter, lCCA, once per DRX cycle for a DRX cycle if the first DBT sample in that DRX cycle is unavailable due to DL CCA failure. DL CCA failures in a DRX cycle are determined as follows:

-	The test system in the first DBT window of each DRX cycle determines whether the DL CCA attempt is successful or not using the principle as follows:

-	If the DL CCA is successful then the test system shall transmit in all DBT windows within that DRX cycle.

-	If the DL CCA is not successful then the test system shall not transmit in any of the DBT windows within that DRX cycle. In this case lCCA is increased by 1.

-	The parameters, LCCA_DL, LCCA_UL, WCCA_DL and WCCA_UL can be used as in non-DRX tests.

If the CCA attempt is successful for a transmission, then the test equipment shall transmit also other remaining transmissions, according to the configuration, within the same DBT window.

If the CCA attempt is not successful for a transmission within the DBT window, the test equipment shall determine whether the CCA attempt is successful for the next configured transmission, based on probability PCCA_DL.

The probability can be different in different time intervals Ti during a test case. One probability value (per cell) applies at any time point during a test; one or more probability values can be configured in the entire test, one value PCCA_DL per time interval Ti where i≥1, and the multiple time intervals (when i>1) do not overlap (e.g., PCCA_DL=1.0 in T1 and PCCA_DL=0.75 in T2)..

For semi-static channel access configuration, a single value PCCA_DL is used to configure the probability of CCA success in different time intervals Ti during a test realization.  An additional limit LCCA_DL is used to determine the maximum number of unavailable DBT samples within an evaluation window WCCA_DL. If the number of unavailable DBT samples on the last WCCA_DL DBT samples is larger or equal to LCCA_DL, the CCA attempt is considered successful for transmission.

For dynamic channel access configuration, the parameters PCCA_DL_1 and PCCA_DL_2 are used to configure the probability of CCA success on the first and second SSB candidate positions, respectively, in different time intervals Ti during a test realization. An additional limit LCCA_DL is used to determine the maximum number of unavailable DBT samples within an evaluation window WCCA_DL. If the number of unavailable DBT samples on the last WCCA_DL DBT samples is larger or equal to LCCA_DL, the CCA attempt is considered successful for transmission.

For semi-static channel access configuration or for dynamic channel access configuration where one candidate SSB position is modeled, prior to each discovery burst transmission window within a time interval Ti of the test, the test equipment shall:

1	Generate a uniform random variable p1 from the range [0, 1] for the first candidate position.

2 	Transmit the discovery burst based on p1 in the first candidate position. If p1 ≤ PCCA_DL, the discovery burst is transmitted at the first candidate SSB location; else if lCCA is larger than or equal to LCCA_DL, the discovery burst is transmitted at the first candidate SSB location, otherwise the discovery burst is muted. If DRX cycle is used, then the decision whether the discover burst is muted or not is repeated for the rest of the DRX cycle.

For dynamic channel access configuration where two candidate SSB positions are modelled, prior to each discovery burst transmission window within a time interval Ti of the test, the test equipment shall:

1 	Generate a uniform random variable p1 from the range [0, 1] for the first candidate position.

2 	Transmit the discovery burst based on p1 in the first candidate position: if p1 ≤ PCCA_DL1, the discovery burst is transmitted at first candidate SSB location, else the test equipment shall:

a 	Generate a uniform random variable p2 from the range [0, 1] for the second candidate SSB position.

b 	Transmit the discovery burst based on p2 in the second candidate position.  If p2 ≤ PCCA_DL2, the discovery burst is transmitted at the second candidate SSB location; else if lCCA is larger than or equal to LCCA_DL, the discovery burst is transmitted at the second candidate SSB location, otherwise the discovery burst is muted. If DRX cycle is used, then the decision whether the discover burst is muted or not is repeated for the rest of the DRX cycle.

The above steps are repeated for each discovery burst transmission window in each time interval Ti of the test. The limit LCCA_DL and window WCCA_DL is a configuration parameter for each test case.

In many test cases, the requirement under a test depends on the number of configured SSB transmissions which are not available during the test due to CCA failure, so the test equipment shall track how many such signal occasions are not transmitted in DL during the test period.

#### A.3.26.2.2	UL CCA model

For UL CCA, the modelling approach is based on probability PCCA_UL of successful CCA. Probability PCCA_UL is configured in the corresponding test case, based on a set SCCA_UL of possible values including 75 % and 87% as typical values for dynamic and semi-static channel access configurations, 0% to model consistent UL CCA failures, and 100% to model no UL CCA failures.

Consistent UL CCA failures are modelled by configuring a low value for PCCA_UL, e.g., PCCA_UL = 0%.

In the same time interval Ti during the same test case, PCCA_UL can be different from PCCA_DL.

The probability can be different in different time intervals Ti during a test case. One probability value applies at any time point during a test; one or more probability values can be configured in the entire test, one value PCCA_UL per time interval Ti where I ≥ 1, and the multiple time intervals (when I > 1) do not overlap (e.g., PCCA_UL = 1.0 in T1 and PCCA_UL = 0.75 in T2).

TCCA µs prior to each UL transmission burst in the test, the test equipment (TE) shall generate a uniform random variable p from the range [0, 1]. If p>PCCA_UL, the TE transmits an OCNG noise pattern with an energy level X per LBT measurement BW, within the UE BW scheduled/configured for the UL transmission for at-least TCCA µs. Where TCCA µs is energy detection time for accessing the uplink channel as defined in section 5.1.1 of TS 37.106 [36]. Where:

-	X is 9 dB above the energy detection threshold defined in section 5.1.1 of TS 37.106 [36].

-	TCCA is the channel sensing period depending on CCA category for the next UL transmission.

The TE shall count the number of UL CCA failures, and no further UL CCA failures are modeled if the number of failures exceeds the limit LCCA_UL within a window WCCA_UL. For each UL CCA failure generated by the model, the TE shall monitor the corresponding UL resource for the desired UL signal, and based on when and/or whether the TE received the desired UL signal, it deems the test case to pass or fail.

In many cases, the requirement under a test depends on the number of configured signal occasions which are not available during the test, so the test equipment shall track how many such signal occasions are not transmitted in UL during the test period.

### A.3.26.3	CCA model for operation on a carrier frequency with CCA in FR2-2

#### A.3.26.3.1	DL CCA model

For DL CCA, the modelling approach is based on probability PCCA_DL of successful CCA.

If the CCA attempt is successful for a transmission, then the test equipment shall transmit also other remaining transmissions, according to the configuration, within the same DBT window.

If the CCA attempt is not successful for a transmission within the DBT window, the test equipment shall determine whether the CCA attempt is successful for the next configured transmission, based on probability PCCA_DL.

To decide whether the CCA attempt for one SSB/SMTC occasion within one SSB/SMTC occasion group, where one SSB/SMTC occasion group consists of 12 consecutive SSB/SMTC occasions is successful or not, TE shall:

1 - Generate a uniform random variable p from the range [0, 1].

2 - If p > PCCA_DL,

-	TE picks one SSB/SMTC occasion out of a group of 12 consecutive SSB/SMTC occasions based on a fixed pattern, where one SSB/SMTC occasion is equivalent to one SSB burst Set.

-	TE models CCA failure in this SSB/SMTC occasion. Note that other 11 SSB/SMTC occasions shall be transmitted by the TE.

-	Whole SSB/SMTC occasion group is considered as unavailable to the UE.

2 - If p ≤ PCCA_DL,

-	TE transmit 12 consecutive SSB/SMTC occasions.

-	Whole SSB/SMTC occasion group is considered as available to the UE.

In many test cases, the requirement under a test depends on the number of configured SSB transmissions which are not available during the test due to CCA failure, so the test equipment shall track how many such signal occasions are not transmitted in DL during the test period.

#### A.3.26.3.2	UL CCA model

For UL CCA, the modelling approach is based on probability PCCA_UL of successful CCA. Probability PCCA_UL is configured in the corresponding test case.

Consistent UL CCA failures are modelled by configuring a low value for PCCA_UL, e.g., PCCA_UL = 0%.

In the same time interval Ti during the same test case, PCCA_UL can be different from PCCA_DL.

The probability can be different in different time intervals Ti during a test case. One probability value applies at any time point during a test; one or more probability values can be configured in the entire test, one value PCCA_UL per time interval Ti where I ≥ 1, and the multiple time intervals (when I > 1) do not overlap (e.g., PCCA_UL = 1.0 in T1 and PCCA_UL = 0.75 in T2).

TCCA µs prior to each UL transmission burst in the test, the test equipment (TE) shall generate a uniform random variable p from the range [0, 1]. If p>PCCA_UL, the TE transmits an OCNG noise pattern with an energy level X within the UE BW scheduled/configured for the UL transmission for at-least TCCA µs. Where TCCA µs is energy detection time for accessing the uplink channel as defined in section 5.1.1 of TS 37.106 [36]. Where:

-	X is 3 dB above the energy detection threshold defined in section 5.1.1 of TS 37.106 [36].

-	TCCA is the channel sensing period depending on CCA category for the next UL transmission.

In many cases, the requirement under a test depends on the number of configured signal occasions which are not available during the test, so the test equipment shall track how many such signal occasions are not transmitted in UL during the test period.

### A.3.26.4	CCA model for operation on a sidelink carrier frequency with CCA

#### A.3.26.4.1	CCA model for SyncRef UE

For the SyncRef UE transmitting S-SSB in sidelink carrier frequency with CCA, the modelling approach is based on probability PCCA_SL_SyncRefUE of successful CCA.

If the CCA attempt is successful for a S-SSB transmission, then the test equipment shall transmit S-SSB on S-SSB transmission occasion, according to the configuration.

If the CCA attempt is not successful for a S-SSB transmission, the test equipment shall determine whether the CCA attempt is successful for the next S-SSB occasion, based on probability PCCA_SL_SyncRefUE.

A counter, denoted by lCCA, tracks the number of unavailable S-SSB periods. In the SL CCA model, the counter lCCA is incremented when CCA failures led to an unavailable S-SSB period.

The probability can be different in different time intervals Ti during a test case. One probability value applies at any time point during a test; one or more probability values can be configured in the entire test, one value PCCA_SL_SyncRefUE per time interval Ti where i ≥ 1, and the multiple time intervals (when i > 1) do not overlap (e.g., PCCA_SL_SyncRefUE = 1.0 in T1 and PCCA_SL_ SyncRefUE = 0.75 in T2).

The parameters P CCA_SL_SyncRefUE_1 and PCCA_SL_SyncRefUE_2 are used to configure the probability of CCA success on the first and second S-SSB candidate occasions, respectively, in different time intervals Ti during a test realization. An additional limit LCCA_SL determines the maximum number of unavailable S-SSB periods. If lCCA ≥LCCA_SL, the CCA attempt is considered successful for S-SSB transmissions.

Two candidate SSB occasions are modelled within each S-SSB period. To decide whether the CCA attempt for the two S-SSB occasions within each of S-SSB period is successful, TE shall:

1 - Generate a uniform random variable p1 from the range [0, 1] for the first candidate S-SSB occasion.

2 - Transmit the S-SSB based on p1 in the first candidate occasion:

-	if p1 ≤ PCCA_SL_SyncRefUE_1, the S-SSB is transmitted in first candidate SSB occasion and this S-SSB period is considered as available to the DUT,

-	else TE shall generate a uniform random variable p2 from the range [0, 1] for the second candidate SSB occasion

-	If p2 ≤ PCCA_SL_SyncRefUE_2 or if lCCA ≥ LCCA_SL, the S-SSB is transmitted in the second candidate SSB occasion;

-	else this S-SSB period is muted.

The above steps are repeated for each S-SSB periods in each time interval Ti of the test. The limit LCCA_SL is a configuration parameter for each test case.

In many test cases, the requirement under a test depends on the counter lCCA, so the test equipment shall track how many such S-SSB periods are not transmitted by TE during the test period.

## A.3.27	Test Cases with at Least One Cell on a Carrier Frequency with CCA

Editor’s note: This clause will include applicability rules for the corresponding test cases.

### A.3.27.1	Introduction

### A.3.27.2		NR Standalone Tests with NR SCell under CCA and All Other NR Cells in FR1

Editor’s note: This clause will include applicability rules for the corresponding test cases.

### A.3.27.3	EN-DC Tests with NR PSCell under CCA and Other NR Cells in FR1

Editor’s note: This clause will include applicability rules for the corresponding test cases.

### A.3.27.4	NR Standalone Tests with NR PCell under CCA and Other NR Cells in FR1

Editor’s note: This clause will include applicability rules for the corresponding test cases.

### A.3.27.5	E-UTRA Standalone Tests with at Least One NR Cell under CCA

Editor’s note: This clause will include applicability rules for the corresponding test cases.

## A.3.28	Discovery Burst Transmission Window configuration under CCA

### A.3.28.1	DBT Window pattern 1: DBT Window period = 20 ms with DBT Window duration = 1 ms

Table A.3.28.1-1: DBT.1: DBT Window Pattern 1 for DBT Window period = 20 ms and duration = 1 ms

| SMTC Parameters | Values |
| --- | --- |
| Discovery burst transmission window periodicity | 20 ms |
| Discovery burst transmission window offset | 0 ms |
| Discovery burst transmission window duration | 1 ms |

## A.3.29	Testing principles for UE capable of only NR bands with shared spectrum access

### A.3.29.1	Introduction

In annex A test cases are defined involving one or more NR cells operating on NR band(s) with shared spectrum channel access. The NR bands with shared spectrum channel access are defined in clause 5.2 of TS 38-101-1 [18].

### A.3.29.2	Principle of testing for UE capable of EN-DC with only NR bands with shared spectrum access

In Annex A, test cases in table A.3.29.2-1 are defined for UE capable of EN-DC with only NR band(s) with shared spectrum access and are not required for UE supporting also other NR band(s) (i.e. band with no shared spectrum access). The EN-DC configurations are defined in clause of 5.5B of TS 38.101-3 [20].

Table A.3.29.2-1: Test cases applicable to UE supporting EN-DC with only NR bands with shared spectrum access

| Test category | Section | Test case |
| --- | --- | --- |
| Active BWP switching | A.10.3.5.2.1 | E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC |
|  | A.10.3.5.2.2 | E-UTRAN – NR PSCell FR1 DL active BWP switch with FR1 SCell in non-DRX in synchronous EN-DC |
|  | A.10.3.5.3.1 | E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC |

### A.3.29.3	Principle of testing for UE capable of SA operation with only NR bands with shared spectrum access

In Annex A, test cases in table A.3.29.3-1 are defined for UE capable of NR SA operation with only NR band(s) with shared spectrum access and are not required for UE supporting also other NR band(s) (i.e. band with no shared spectrum access).

Table A.3.29.3-1: Test cases applicable to UE supporting SA operation with only NR bands with shared spectrum access

| Test category | Section | Test case |
| --- | --- | --- |
| Active BWP switching | A.11.4.5.2.1 | NR FR1- NR FR1 DL active BWP switch of PCell with non-DRX in SA |
|  | A.11.4.5.2.2 | NR FR1 DL active BWP switch with non-DRX in SA |
|  | A.11.4.5.3.1 | NR FR1 DL active BWP switch of Cell with non-DRX in SA |

## A.3.30	CSI-RS configurations for RRM

### A.3.30.1	FDD

Table A.3.30.1-1: CSI-RS RRM Reference Measurement Channels for SCS=15kHz

|  | CSI-RS.RRM.FR1.1 FDD |
| --- | --- |
| CSI-RS-ResourceConfigMobility |  |
| subcarrierSpacing, kHz | 15 |
| CSI-RS-CellMobility |  |
| cellIdnote1 | 0 |
| nrofPRBs | 48 |
| startPRB | 0 |
| density | 3 |
| CSI-RS-Resource-Mobility |  |
| csi-RS-Index | 0 |
| slotConfig: ms20 note2 | slot1 |
| associatedSSB | True |
| ssb-Index note3 | 0 |
| isQuasiColocated | True |
| firstOFDMSymbolInTimeDomain note4 | 10 |
| sequenceGenerationConfig | 0 |
| Others |  |
| nrofPorts | 1 |
| CDM Type | NoCDM |
| EPRE ratio to SSS, dB | 0 |
| Note1: unless specified otherwise Note2: unless specified otherwise Note3: assume the same SS/PBCH block index of the corresponding cell in the test case Note4: unless specified otherwise |  |

### A.3.30.2	TDD

Table A.3.30.2-1: CSI-RS RRM Reference Measurement Channels for SCS=15kHz

|  | CSI-RS.RRM.FR1.1 TDD |
| --- | --- |
| CSI-RS-ResourceConfigMobility |  |
| subcarrierSpacing, kHz | 15 |
| CSI-RS-CellMobility |  |
| cellIdnote1 | 0 |
| nrofPRBs | 48 |
| startPRB | 0 |
| density | 3 |
| CSI-RS-Resource-Mobility |  |
| csi-RS-Index | 0 |
| slotConfig: ms20 note2 | slot1 |
| associatedSSB | True |
| ssb-Index note3 | 0 |
| isQuasiColocated | True |
| firstOFDMSymbolInTimeDomain note4 | 10 |
| sequenceGenerationConfig | 0 |
| Others |  |
| nrofPorts | 1 |
| CDM Type | NoCDM |
| EPRE ratio to SSS, dB | 0 |
| Note1: unless specified otherwise Note2: unless specified otherwise Note3: assume the same SS/PBCH block index of the corresponding cell in the test case Note4: unless specified otherwise |  |

Table A.3.30.2-2: CSI-RS RRM Reference Measurement Channels for SCS=30kHz

|  | CSI-RS.RRM.FR1.2 TDD |
| --- | --- |
| CSI-RS-ResourceConfigMobility |  |
| subcarrierSpacing, kHz | 30 |
| CSI-RS-CellMobility |  |
| cellIdnote1 | 0 |
| nrofPRBs | 48 |
| startPRB | 0 |
| density | 3 |
| CSI-RS-Resource-Mobility |  |
| csi-RS-Index | 0 |
| slotConfig: ms20 note2 | slot1 |
| associatedSSB | True |
| ssb-Index note3 | 0 |
| isQuasiColocated | True |
| firstOFDMSymbolInTimeDomain note4 | 10 |
| sequenceGenerationConfig | 0 |
| Others |  |
| nrofPorts | 1 |
| CDM Type | NoCDM |
| EPRE ratio to SSS, dB | 0 |
| Note1: unless specified otherwise Note2: unless specified otherwise Note3: assume the same SS/PBCH block index of the corresponding cell in the test case Note4: unless specified otherwise |  |

Table A.3.30.2-3: CSI-RS RRM Reference Measurement Channels for SCS=120kHz

|  | CSI-RS.RRM.FR2.1 TDD |
| --- | --- |
| CSI-RS-ResourceConfigMobility |  |
| subcarrierSpacing, kHz | 120 |
| CSI-RS-CellMobility |  |
| cellIdnote1 | 0 |
| nrofPRBs | 48 |
| startPRB | 0 |
| density | 3 |
| CSI-RS-Resource-Mobility |  |
| csi-RS-Index | 0 |
| slotConfig: ms20 note2 | slot1 |
| associatedSSB | True |
| ssb-Index note3 | 0 |
| isQuasiColocated | True |
| firstOFDMSymbolInTimeDomain note4 | 10 |
| sequenceGenerationConfig | 0 |
| Others |  |
| nrofPorts | 1 |
| CDM Type | NoCDM |
| EPRE ratio to SSS, dB | 0 |
| Note1: unless specified otherwise Note2: unless specified otherwise Note3: assume the same SS/PBCH block index of the corresponding cell in the test case Note4: unless specified otherwise |  |

## A.3.31	PRS Configurations

### A.3.31.1.	PRS Configurations for FR1

#### A.3.31.1.1.	PRS pattern 1 in FR1: SCS=15 KHz

Table A.3.31.1.1-1: PRS.1 FR1: PRS Pattern 1 for SSB SCS=15 KHz

| PRS Parameters | Values |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel | PRS.1.1 FR1 | PRS.1.2 FR1 | PRS.1.3 FR1 |  | PRS.1.4 FR1 |  |
| Resource index in resource set | 0 | 0 | 0 | 1 | 0 | 1 |
| PRS periodicity | 160ms |  |  |  |  |  |
| PRS Resource set slot offset Note 1 | 10 ms |  |  |  |  |  |
| PRS Resource slot offset (slot) Note 1 | 0 | 4 | 0 |  | 4 |  |
| PRS RE offset Note 1 | 0 |  | 0 | 1 | 0 | 1 |
| SCS | 15kHz |  |  |  |  |  |
| PRS comb size | 2 | 4 | 2 |  | 4 |  |
| Number of PRS symbol | 4 | 4 | 4 |  | 4 |  |
| Repetion factor | 2 | 1 | 2 |  | 1 |  |
| PRS resource time gap (slot) | 1 | 1 | 1 |  | 1 |  |
| RB numbers containing PRS within channel BW Note 1 | 0-23 | 0-103 | 0-23 |  | 0-103 |  |
| PRS Start PRB | 0 |  |  |  |  |  |
| Note 1: Unless otherwise specified in the test case |  |  |  |  |  |  |

#### A.3.31.1.2.	PRS pattern 2 in FR1: SCS=30 KHz

Table A.3.31.1.2-1: PRS.2 FR1: PRS Pattern 2 for SCS=30 KHz

| PRS Parameters | Values |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel | PRS.2.1 FR1 | PRS.2.2 FR1 | PRS.2.3 FR1 |  | PRS.2.4 FR1 |  |
| Resource index in resource set | 0 | 0 | 0 | 1 | 0 | 1 |
| PRS periodicity | 160ms |  |  |  |  |  |
| PRS Resource set slot offset Note 1 | 10 ms |  |  |  |  |  |
| PRS Resource slot offset (slot) Note 1 | 0 | 4 | 0 |  | 4 |  |
| PRS RE offset Note 1 | 0 |  | 0 | 1 | 0 | 1 |
| SCS | 30kHz |  |  |  |  |  |
| PRS comb size | 2 | 4 | 2 |  | 4 |  |
| Number of PRS symbol | 4 | 4 | 4 |  | 4 |  |
| Repetion factor | 2 | 1 | 2 |  | 1 |  |
| PRS resource time gap (slot) | 1 | 1 | 1 |  | 1 |  |
| RB numbers containing PRS within channel BW Note 1 | 0-23 | 0-131 | 0-23 |  | 0-131 |  |
| PRS Start PRB | 0 |  |  |  |  |  |
| Note 1: Unless otherwise specified in the test case |  |  |  |  |  |  |

### A.3.31.2.	PRS Configurations for FR2

#### A.3.31.2.1.	PRS pattern 1 in FR2: SCS=120 KHz

Table A.3.31.2.1-1: PRS.1 FR2: PRS Pattern 1 for SCS=120 KHz

| PRS Parameters | Values |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel | PRS.1.1 FR2 | PRS.1.2 FR2 | PRS.1.3 FR2 |  | PRS.1.4 FR2 |  |
| Resource index in resource set | 0 | 0 | 0 | 1 | 0 | 1 |
| PRS periodicity | 160ms |  |  |  |  |  |
| PRS Resource set slot offset Note 1 | 10 ms |  |  |  |  |  |
| PRS Resource slot offset (slot) Note 1 | 0 | 4 | 0 |  | 4 |  |
| PRS RE offset Note 1 | 0 |  | 0 | 1 | 0 | 1 |
| SCS | 120kHz |  |  |  |  |  |
| PRS comb size | 2 | 4 | 2 |  | 4 |  |
| Number of PRS symbol | 4 | 4 | 4 |  | 4 |  |
| Repetion factor | 2 | 1 | 2 |  | 1 |  |
| PRS resource time gap (slot) | 1 | 1 | 1 |  | 1 |  |
| RB numbers containing PRS within channel BW Note 1 | 0-31 | 0-127 | 0-31 |  | 0-127 |  |
| PRS Start PRB | 0 |  |  |  |  |  |
| Note 1: Unless otherwise specified in the test case |  |  |  |  |  |  |

## A.3.32	NR sidelink discovery

### A.3.32.1	Introduction

This clause also defines the principle and the reference configurations that are applicable to test cases verifying RRM core requirements for NR sidelink discovery.

### A.3.32.2	Reference resource pool configurations for NR Sidelink Discovery

Table A.3.32.2-1: SL-BWP configuration for NR sidelink discovery

| Field | Value | Comment |
| --- | --- | --- |
| SL-BWP-ConfigCommon-r16 |  |  |
| sl-BWP-Generic-r16 |  |  |
| sl-LengthSymbols-r16 | sym14 | All 14 symbols in a slot without S-SSB are used for sidelink |
| sl-StartSymbol-r16 | sym0 | Symbol #0 is the starting symbol used for sidelink in a slot without S-SSB |
| sl-BWP-DiscPoolConfigCommon-r17 |  |  |
| sl-DiscRxPool-r17 |  | Indicates the resource pool for reception on the configured BWP. 1 entry |
| SL-ResourcePool-r16[1] | Set according to Table A.3.21.2-2 | Entry 1 |
| sl-DiscTxPoolSelected-r17 |  | Indicates the resources pool for mode 2 sidelink communication on the configured BWP. 1 entry |
| SL-ResourcePoolConfig-r16[1] |  | Entry 1 |

### A.3.32.3	Principle of Testing

The UE capable of both V2X sidelink communication and NR sidelink discovery does not have to pass the test for interruption at NR sidelink discovery configuration defined in clause 9.1.6.1, if this UE has already passed the test case for interruption due to V2X sidelink communication defined in clause 9.1.6.1.

## A.3.33	PRS Processing Window (PPW) configurations

Table A.3.33-1: Reference PPW configuration

| PPW Parameters | Unit | Values |
| --- | --- | --- |
| Reference PPW |  | PPW.1 |
| Periodicity | slot | Note 1 |
| Offset | slot | Note 2 |
| Length | ms | 10 |
| Type |  | Same as the value reported in prsProcessingType-r17 |
| Priority |  | st1 |
| Note 1: Same as PRS resource set periodicity as used in the test case. Note 2: Same as PRS resource with smallest offset as used in the test case. |  |  |

## A.3.34	Testing principles for test cases related to PRS measurements

### A.3.34.1	Introduction

In annex A test cases are defined for verifying various type of PRS measurement and accuracy requirements.

### A.3.34.2	Test cases in RRC_INACTIVE state

In Annex A, PRS measurement test cases are defined with 4 samples and with reduced number of samples in RRC_INACTIVE state. The testing principle for these test cases is as follows:

-	A UE capable of supportedDL-PRS-ProcessingSamples-RRC-Inactive [34] is only required to pass the test cases with reduced number of samples.

-	A UE not capable of supportedDL-PRS-ProcessingSamples-RRC-Inactive [34] is required to pass the test cases with 4 samples.

In Annex A, PRS measurement delay test cases are defined for both PRS-RSRP and PRS-RSRPP measurements in RRC_INACTIVE state. The testing principle for these test cases is as follows:

-	A UE capable of both PRS-RSRP and PRS-RSRPP measurements is required to pass either PRS-RSRP measurement delay test or PRS-RSRPP measurement delay test.

In Annex A, PRS measurement delay test cases are defined for both RSTD and UE Rx-Tx time difference measurements in RRC_INACTIVE state. The testing principle for these test cases is as follows:

-	A UE capable of both RSTD and UE Rx-Tx time difference measurements is required to pass either RSTD measurement delay test or UE Rx-Tx time difference measurement delay test.

### A.3.34.3	Test cases for PRS measurements with gaps in RRC_CONNECTED state

In Annex A, PRS measurement test cases are defined with 4 samples and with reduced number of samples with measurement gaps in RRC_CONNECTED state. The testing principle for these test cases is as follows:

-	A UE capable of supportedDL-PRS-ProcessingSamples [34] is only required to pass the test cases with reduced number of samples.

-	A UE not capable of supportedDL-PRS-ProcessingSamples [34] is required to pass the test cases with 4 samples.

In Annex A, PRS measurement delay test cases are defined for both PRS-RSRP and PRS-RSRPP measurements with measurement gaps in RRC_CONNECTED state. The testing principle for these test cases is as follows:

-	A UE capable of both PRS-RSRP and PRS-RSRPP measurements is required to pass either PRS-RSRP measurement delay test or PRS-RSRPP measurement delay test.

### A.3.34.4	Test cases for PRS measurements without gaps in RRC_CONNECTED state

In Annex A, PRS measurement test cases are defined with 4 samples and with reduced number of samples without measurement gaps in RRC_CONNECTED state. The testing principle for these test cases is as follows:

-	A UE capable of supportedDL-PRS-ProcessingSamples [34] is only required to pass the test cases with reduced number of samples.

-	A UE not capable of supportedDL-PRS-ProcessingSamples [34] is required to pass the test case with 4 samples.

In Annex A, PRS measurement delay test cases are defined for both PRS-RSRP and PRS-RSRPP measurements without measurement gaps in RRC_CONNECTED state. The testing principle for these test cases is as follows:

-	A UE capable of both PRS-RSRP and PRS-RSRPP measurements is required to pass either PRS-RSRP measurement delay test or PRS-RSRPP measurement delay test.

In Annex A, PRS measurement delay test cases are defined for both RSTD and UE Rx-Tx time difference measurements without measurement gaps in RRC_CONNECTED state. The testing principle for these test cases is as follows:

-	A UE capable of both RSTD and UE Rx-Tx time difference measurements is required to pass either RSTD measurement delay test or UE Rx-Tx time difference measurement delay test.

## A.3.35	Testing principle for RedCap UE

### A.3.35.1	Introduction

This clause defines a principle which is applicable to test cases verifying RRM requirements for RedCap UE.

### A.3.35.2	Principle of testing for FR1

For RedCap UEs supporting 1 Rx branch, all single carrier tests specified in clause A.16 and A.18 except for tests defined for 2 Rx and/or FR2 shall be tested on any band.

For RedCap UEs supporting 2Rx branches, all single carrier tests specified in clause A.16 and A.18 except for tests defined for 1 Rx and/or FR2 shall be tested on any band.

### A.3.35.3	Principle of testing for FR2

For RedCap UEs, all single carrier tests specified in clause A.17 and A.18 except for tests defined for FR1 shall be tested on any band.

## A.3.36	Testing related to Satellite access

## A.3.36.1	Introduction

### A.3.36.1	Introduction

In annex A test cases are defined for verifying various type of RRM requirements related to satellite access.

### A.3.36.2	Principle of testing GSO and NGSO scenarios

In Annex A, RRM test cases related to satellite access are defined for both GSO and NGSO. The testing principle for these test cases is as follows:

-	A UE capable of GSO only is required to pass the test cases with GSO.

-	A UE capable of NGSO only is required to pass the test cases with NGSO.

-	A UE capable of both GSO and NGSO is required to pass the test cases with NGSO only.

Support of GSO and NGSO scenario is indicated via ntn-ScenarioSupport-r17.

### A.3.36.2	Principle of testing different RRM requirements

In Annex A, RRM test cases related to satellite access are defined for all applicable RRM requirements. The testing principle for these test cases is as follows:

-	A UE capable of NTN only is required to pass all the test cases defined in clause A.14.

-	A UE capable of both TN and NTN is required to pass the test cases for NTN specific requirements in Table A.3.36.2-1.

Table A.3.36.2-1: Test cases for NTN specific requirements

| Clause | Test case slogan |
| --- | --- |
| A.14.1.2 | Cell reselection to FR1 intra-frequency NR cell for UE configured with [capability for enhanced requirements] |
| A.14.1.3 | Time-based cell reselection to FR1 intra-frequency NR cell |
| A.14.1.4 | Location-based cell reselection to FR1 intra-frequency NR cell |
| A.14.1.7 | Cell reselection to FR1 inter-frequency NR cell for UE configured with [capability for enhanced requirements] |
| A.14.1.8 | Time-based Cell reselection to FR1 inter-frequency NR satellite access case |
| A.14.1.9 | Location-based Cell reselection to FR1 inter-frequency NR satellite access case |
| A.14.2.1.3 | Intra-frequency SAN time-based conditional Handover from FR1 to FR1 |
| A.14.2.1.4 | Inter-frequency SAN time-based conditional Handover from FR1 to FR1 |
| A.14.2.1.5 | Intra-frequency SAN distance-based conditional Handover from FR1 to FR1 |
| A.14.2.1.6 | Inter-frequency SAN distance-based conditional Handover from FR1 to FR1 |
| A.14.3.1.1 | NR UE Transmit Timing Test for FR1 |
| A.14.5.1.1 | SA event triggered reporting tests without gap under non-DRX |
| A.14.5.1.2 | SA event triggered reporting tests without gap under DRX |
| A.14.5.1.3 | SA event triggered reporting tests without gap under non-DRX with SSB index reading |
| A.14.5.1.4 | SA event triggered reporting tests with single measurement gap under non-DRX for satellite access |
| A.14.5.1.5 | SA event triggered reporting tests with FNO concurrent gaps under DRX for satellite access |
| A.14.5.1.6 | SA event triggered reporting tests with PPO concurrent gaps under non-DRX with SSB index reading for satellite access |
| A.14.5.2.1 | Event triggered reporting test without gap under non-DRX |
| A.14.5.2.2 | Event triggered reporting tests without gap under DRX |
| A.14.6.3.1 | SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell |
| A.14.6.3.2 | SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell |
| A.14.6.4.1 | SSB based L1-RSRP measurement |
| A.14.6.4.2 | CSI-RS based L1-RSRP measurement on resource set with repetition off |

### A.3.36.3	Principle of testing different ephemeris formats

Satellite access RRM test cases are defined such that satellite ephemeris information is sent to UE in each test case, according to Tables 14.0.3-1 and 14.0.3-2.

Table A.3.36.3-1: Test cases configuring EphemerisInfo as PositionVelocity

| Functional Area | Test Case |
| --- | --- |
| RRC_IDLE state mobility | A.14.1.3 |
|  | A.14.1.4 |
|  | A.14.1.5 |
|  | A.14.1.6 |
|  | A.14.1.9 |
|  | A.14.1.10 |
| Handover | A.14.2.1.3 |
|  | A.14.2.1.4 |
|  | A.14.2.1.5 |
|  | A.14.2.1.6 |
| Timing | A.14.3.1 |
| Radio Link Monitoring | A.14.4.1.1 |
|  | A.14.4.1.2 |
|  | A.14.4.1.5 |
|  | A.14.4.1.6 |
| BFD and LR procedures | A.14.4.2.1 |
|  | A.14.4.2.3 |
|  | A.14.4.2.5 |
| Active BWP switch | A.14.4.3.1 |
| UE specific CBW change | A.14.4.4.1 |
| PL-RS switching delay | A.14.4.5.1 |
| Intra-frequency measurements | A.14.5.1.1 |
|  | A.14.5.1.3 |
|  | A.14.5.1.5 |
| Inter-frequency measurements | A.14.5.2.1 |
|  | A.14.5.2.3 |
|  | A.14.5.2.7 |
| L1-RSRP measurements | A.14.5.3.1 |
|  | A.14.5.3.3 |
| SS-RSRP accuracy | A.14.6.1.1 |
| SS-RSRQ accuracy | A.14.6.2.1 |
| SS-SINR accuracy | A.14.6.3.1 |
| L1-RSRP accuracy | A.14.6.4.1 |

Table A.3.36.3-2: Test cases configuring EphemerisInfo as Orbital

| Functional Area | Test Case |
| --- | --- |
| RRC_IDLE state mobility | A.14.1.1 |
|  | A.14.1.2 |
|  | A.14.1.5 |
|  | A.14.1.6 |
| Handover | A.14.2.1.1 |
|  | A.14.2.1.2 |
| RRC Connection Mobiliy Control | A.14.2.2.1 |
|  | A.14.2.2.2 |
|  | A.14.2.2.3 |
| Timing | A.14.3.2 |
| Radio Link Monitoring | A.14.4.1.3 |
|  | A.14.4.1.4 |
|  | A.14.4.1.7 |
|  | A.14.4.1.8 |
| BFD and LR procedures | A.14.4.2.2 |
|  | A.14.4.2.4 |
|  | A.14.4.2.6 |
| Active BWP switch | A.14.4.3.2 |
| Intra-frequency measurements | A.14.5.1.2 |
|  | A.14.5.1.4 |
|  | A.14.5.1.6 |
| Inter-frequency measurements | A.14.5.2.2 |
|  | A.14.5.2.4 |
|  | A.14.5.2.6 |
|  | A.14.5.2.8 |
| L1-RSRP measurements | A.14.5.3.2 |
|  | A.14.5.3.4 |
| SS-RSRP accuracy | A.14.6.1.2 |
| SS-RSRQ accuracy | A.14.6.2.2 |
| SS-SINR accuracy | A.14.6.3.2 |
| L1-RSRP accuracy | A.14.6.4.2 |

### A.3.36.4	General setup for SIB19

The general parameters for SIB19 setup is specified in Table A.3.36.4-1.

Table A.3.36.2-1: Test cases for NTN specific requirements

| Parameter | Unit | Test 1 |
| --- | --- | --- |
| Interval between adjacent epoch time | s | LEO: 2.56 GEO: 10.24 |
| ntn-UlSyncValidityDuration | s | LEO: 5 GEO: 900 |
| cellSpecificKoffset | slot | LEO: 14 GEO: 258 |
| kmac | slot | Not configured |
| ta-Common |  | 0 |
| ta-CommonDrift |  | 0 |
| ta-CommonDriftVariant |  | 0 |
| ntn-PolarizationDL |  | linear |
| ntn-PolarizationUL |  | linear |
| ephemerisInfo |  | Detailed ephemeris information is provided in TS 38.508-1 [38] |
| ta-Report |  | Not configured |

### A.3.36.5	Satellite specific parameters configuration

#### A.3.36.5.1	Satellite specific configuration for serving cell

Table A.3.36.5.1-1: Satellite specific configuration pattern 1 for serving cell in GSO scenario

| Parameter | SSC.1 |
| --- | --- |
| Interval between adjacent epoch time | 10.24s |
| ntn-UlSyncValidityDuration | 900s |
| cellSpecificKoffset | 258 slots |
| ta-Common | 0 |
| ta-CommonDrift | 0 |
| ta-CommonDriftVariant | 0 |
| ntn-PolarizationDL | linear |
| ntn-PolarizationUL | linear |
| ephemerisInfo | Detailed ephemeris information is provided in TS 38.508-1 [38] |

Table A.3.36.5.1-2: Satellite specific configuration pattern 2 for serving cell in NGSO scenario

| Parameter | SSC.2 |
| --- | --- |
| Interval between adjacent epoch time | 2.56s |
| ntn-UlSyncValidityDuration | 5s |
| cellSpecificKoffset | 14 slots |
| ta-Common | 0 |
| ta-CommonDrift | 0 |
| ta-CommonDriftVariant | 0 |
| ntn-PolarizationDL | linear |
| ntn-PolarizationUL | linear |
| ephemerisInfo | Detailed ephemeris information is provided in TS 38.508-1 [38] |

#### A.3.36.5.2	Satellite specific configuration for neighbour cell

Table A.3.36.5.2-1: Satellite specific configuration pattern 1 for neighbour cell in GSO scenario

| Parameter | NSC.1 |
| --- | --- |
| Interval between adjacent epoch time | 10.24s |
| ntn-UlSyncValidityDuration | 900s |
| cellSpecificKoffset | 258 slots |
| ta-Common | 0 |
| ta-CommonDrift | 0 |
| ta-CommonDriftVariant | 0 |
| ntn-PolarizationDL | linear |
| ntn-PolarizationUL | linear |
| ephemerisInfo | Detailed ephemeris information is provided in TS 38.508-1 [38] |

Table A.3.36.5.2-2: Satellite specific configuration pattern 2 for neighbour cell in NGSO scenario

| Parameter | NSC.2 |
| --- | --- |
| Interval between adjacent epoch time | 2.56s |
| ntn-UlSyncValidityDuration | 5s |
| cellSpecificKoffset | 14 slots |
| ta-Common | 0 |
| ta-CommonDrift | 0 |
| ta-CommonDriftVariant | 0 |
| ntn-PolarizationDL | linear |
| ntn-PolarizationUL | linear |
| ephemerisInfo | Detailed ephemeris information is provided in TS 38.508-1 [38] |

## A.3.37	Reference Cell DTX configurations

### A.3.37.1	Cell DTX Configuration 1: Cell DTX cycle = 640 ms and TAT = Infinity

Table A.3.37.1-1: DTX.1: Cell DTX cycle = 640 ms and time alignment timer (TAT) = Infinity

| Field | Value |
| --- | --- |
| cellDTXDRX-onDurationTimer | 1 ms |
| cellDTXDRX-CycleStartOffset | 640 ms |
| cellDTXDRXconfigType-r18 | dtx |
| cellDTXDRXactivationStatus | activated |
| Note: This Cell DTX configuration is applicable for NR serving cell. The Cell DTX cycle and time alignment timer parameters are specified in clause 6.3.2 in TS 38.331 [2]. |  |