3GPP TSG-RAN WG4 Meeting #112	R4-2413400

Maastricht, Netherlands, 19th -23rd August, 2024

Source:	Qualcomm Incorporated

Title:	Intra-band contiguous and non-contiguous UL CA with PC1.5

Agenda item:	8.1.1.1.1

Document for:	Discussion

# 1.	Introduction

In RAN4#111 companies were requested to measure/simulate MPR/AMPR numbers for several intra-band contiguous and non-contiguous scenarios. In this paper we present details on the MPR measurements done to date on contiguous CC with contiguous RBs

# 2. 	Discussion

In RAN4#111 a WF was created that stated the following [1]:

- Evaluate MPR/A-MPR numbers for contiguous CA first and then work on non-contiguous CA MPR/A-MPR afterwards.

- The intra-band PC1.5 ULCA R19 work focusses on UE implementing with two 26dBm PAs. Architectures requiring a 29dBm PA are not specified but can be implemented by fulfilling the requirements based on two 26dBm PAs.

- For intra-band contiguous ULCA w/ and w/o UL MIMO, the Architecture#2 is prioritized for MPR/A-MPR evaluation.

  - If time is allowed, architecture #1 can be evaluated.

| Architecture | Description | Indicated capability | Whether can support UL MIMO | Applicable cases |
| --- | --- | --- | --- | --- |
| #1 | 2x26 dBm PA + 2 LO with 100MHz BW | dualPA-Architecture | No | Contiguous CA |
| #2 | 2x26 dBm PA + 1 LO with 200MHz BW | TxD | Yes | Contiguous CA |

In this paper we describe our preliminary measurements for architecture#2 described in the above table for contiguous CA with contiguous RBs. The MPR for band n41C was measured using a two 26 dBm PA setup with an antenna-to-antenna isolation of 10 dB. The contiguous CCs with contiguous RBs scenario has RBs allocated in one CC exclusively, or in both CCs with the RB allocation being contiguous across the CC1 and CC2 RB boundary. Numerous RB allocations belonging to this scenario were tested for MPR performance. Also, MPR measurements were made for various CA BWs. For the cases where RBs were present in both CCs their PSDs were adjusted to be equal. Currently, more measurements are required to verify more RB allocations and more bands before a final MPR table can be generated.

# Conclusion

In this paper we present details on the MPR measurements done to date on contiguous CC with contiguous RBs. We have made numerous measurements on n41C for numerous BWs however more MPR measurements have to be made on various RB allocations and additional bands before a final MPR table can be generated.

# References

[1] R4-2410565, WF on higher power UE, May 20th – May 24th, 2024

[2] TS38.101-1, User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone, V18.6.0

[3] R4-2204225, 1CC fall-back MPR for NC UL CA with 1LO architecture, February 21st – March 3rd, 2022