3GPP TSG-RAN WG4 Meeting #112             	R4-2413444

Maastricht, Netherlands, 19th-23th, Aug, 2024

Agenda item:	8.16.3

Source:	Samsung

Title:	Initial discussion on BS demodulation requirement with MMSE-IRC receiver

Document for:	Discussion

- Introduction

In the RAN#103, the Rel-19 WI on demodulation performance requirements enhancement was approved with the following objective related with BS demodulation requirements [1]:

- Objective of Performance part WI - Define FR1 PUSCH performance requirements with inter-cell interference in homogeneous and heterogenous deployment based on the following assumption - Receiver type: MMSE-IRC - Scenarios: - FDD synchronous deployments - TDD synchronous deployments with aligned UL: DL configurations among cells - Slot-based transmission and aligned SCS among cells - Reuse LTE based interference profile in TR 36.884 as a starting point. Other interference profiles are not precluded

According to the Rel-19 schedule, this meeting is the 1st meeting to discuss the performance part of this WI. In this contribution, the view on BS demodulation with MMSE-IRC are provided.

# 2	General

## 2.1	Scenario

BS receiver with MMSE-IRC can efficiently suppress the uplink interference from UEs in the neighboring cells and thus improve uplink coverage and capability. In addition, the widely use of HPUE makes UL co-channel interference from the neighboring cell UE server that that in LTE system. According to the WID, similar as LTE, both homogeneous and heterogenous deployment are considered [2]. From receiver processing perspective, there is not difference foreseen. While considering the test coverage with different deployment and interference profile condition, it is fine to introduce PUSCH requirement with considering both homogeneous and heterogeneous network scenarios.

Proposal 1:  Define PUSCH requirement with MMSE-IRC receiver for both homogeneous and heterogeneous network scenario.

Regarding duplex mode, the requirements for both FDD and TDD with different UL/DL configurations were specified. And the same TDD requirements are applicable to different UL-DL configuration. Therefore, the same approach can be applied for PUSCH requirement with MMSE-IRC receiver.

Proposal 2:  Define PUSCH requirement with MMSE-IRC receiver for both FDD and TDD. The same TDD requirement are applicable to different UL-DL configuration.

In LTE BS demodulation requirement with MMSE-IRC, both synchronous and asynchronous scenarios were under discussion.   According to the WID, only synchronous scenario was considered.

Proposal 3:  Define PUSCH requirement with only synchronous scenario.

## 2.2	Target and interference channels

In Rel-15, for the target PUSCH, both 1Tx and 2Tx UE were considered. And in Rel-18, 4Tx with legacy DMRS and enhanced DMRS were considered. 4Tx is mainly targeting for FWA UE scenario. With large number of Tx with more layers, excepting for inter-cell interference, inter-layer has also impacted on the performance with MMSE-IRC receiver, the gain of MMSE-IRC may be limited compared with MMSE receiver.

During the SI phase of LTE, 1Tx UE scenario is prioritized and 2Tx UE scenario is not covered.  In real-deployment, most of commercial UE is 1Tx UE. Similarly, we prefer to prioritize the 1Tx UE scenario, deprioritize the 2Tx UE and not to cover 4Tx UE for both target PUSCH and interference UE.

Proposal 4:  For target PUSCH and interference PUSCH, prioritize the 1Tx UE scenario. Deprioritize 2Tx UE scenario, and not cover 4Tx UE scenario.

Regarding number of interference UE modeling, up to two interferer UE explicitly modelling. In the LTE stage, based on evaluation, the additional MMSE-IRC gain over MMSE is not significant under 2Rx condition in case the number of interferer number is increased from 1 to 2, considering the limited Rx number to handle the interference. Therefore, the feasible of number interferer UE modeling for MMSE-IRC should be further evaluated.

Observation 1:  Number of interferer UE modeling depending on number of Rx number. Further evaluation the feasible of number interferer UE modeling before specifying the PUSCH requirement with MMSE-IRC receiver.

## 2.3	Interference Profile

Regarding the interference Profile, as indicated in the WID, the LTE interference profiles can be used as a starting point, which was also derived based on system level simulator for both homogeneous and heterogeneous deployment scenarios. From MMSE-IRC receiver processing, there will be no different, the LTE interference profiles can be used for verifying the gain of MMSE-IRC compared with MMSE receiver. While comparing LTE, the deployment for NR will be different, the system-level simulation assumptions might need to be updated, especially large bandwidth are available in FR1 30KHz SCS, meanwhile the HPUEs are widely deployed, which may result in large interference. Therefore, applying the interference profile derived in LTE deployment for NR evaluation might be not proper. In case to derive the new interference profile, the system-level simulation work should be conducted. The further simulator calibration to derive the interference profile is needed. We are also open to further whether to derive the interference profile based on NR deployment.

Proposal 5:  LTE interference profiles can be used for verifying the gain of MMSE-IRC compared with MMSE receiver as a starting point. FFS to derive the interference profile based on system level simulation with NR deployment for homogeneous and heterogeneous scenarios

## 2.4	Baseline Receiver

According to TS 36.884, the MMSE-IRC receiver weight matrix is usually defined as

where and denote the estimated channel matrix and covariance matrix, respectively.

To obtain the MMSE-IRC receiver weight matrix, the covariance matrix including the sources of inter-cell interference needs to be estimated. The covariance matrix is estimated at DM-RS REs by following equations:

, ,

where the DM-RS symbols are used to estimate the covariance matrix, and Nsp is the number of sampling DM-RS REs.

The estimation of interference covariance matrix is performed at per PRB and per TTI basis.

Proposal 6:  Apply the DM-RS of severed targeting UE’s PUSCH for interference covariance matrix estimation. The baseline assumption for estimation of interference covariance matrix is performed at per PRB and per TTI basis.

## 2.5	Technical Report

According to WID, there is no study phase mentioned in this WI. While, as mentioned above, for interference profile should be derived based on system level simulator if the current LTE profile is not proper. And also, the number of interfered UE explicitly modeling should be evaluated. Therefore, the related evaluation and analysis results, it may be better to capture into the technical report, similar as the LTE BS performance requirement with MMSE-IRC receiver. If needed, the related WID should be updated to reflect the corresponding TR.

Proposal 7: FFS on Technical Report needed to capture interference profile modeling and number of interfered UE explicitly modeling evaluation and analysis results.

# 3	Simulation Assumption for evaluation

In this section, the initial simulation assumptions are provided for evaluation.

Antenna Configuration for Both Target UE and interference UE

In Rel-15, PUSCH requirement with 2Rx/4Rx/8Rx were specified. The gain of MMSE-IRC compared with MMSE is more significantly due to more diversity for interference mitigation. Therefore, it was necessary to evaluate the performance with different Rx number under different interfered UE modeling scenario.

Number of Tx port for Both Target UE and interference UE

Regarding the number of Tx port, as mentioned, most of commercial UE should be 1Tx UE. Therefore, 1 Tx for target UE and interference UE should be prioritized for evaluation

Proposal 8: Consider 1Tx and 2/4/8Rx antenna configuration for target UE and interference UE to evaluate the performance gain of MMSE-IRC

Channel Model

To check the per-PRB interference covariance estimation, it is better to choose the channel with high delay spread for the interferers. In Rel-15, several channel models are specified to cover different delay spread and doppler spread, including TDLA30-10, TDLB400-100 and TDLC300-100, where TDLC300-100 can be considered as a starting point. In LTE BS performance with MMSE-IRC receiver, different channel models were applied for target UE and interfered UE. For NR PDSCH with MMSE-IRC, the same channel was applied for both target UE and interfered UE.  Therefore, whether to differentiate the channel model used for target UE and interfered UE should be further discussed and evaluated.

Proposal 9: FFS to differentiate the channel model used for target UE and interfered UE for performance evaluation with MMSE-IRC receiver.

SCS and Channel bandwidth

Regarding the SCS and channel bandwidth, since the Rel-15 network has already deployed in the commercial market, it is more proper to choose the typical SCS and BW applied in current network. Therefore, 15KHz SCS with 10MHz, and 30KHz SCS with 40MHz can be chosen for initial performance evaluation. While considering the test coverage and manufacture declaration, additional channel bandwidth with minimum or maximum for each SCS can be considered for requirement definition.

Proposal 10: Consider 15KHz SCS, 10MHz CBW and 30KHz SCS, 40MHz CBW for initial performance evaluation with MMSE-IRC receiver.

MCS

Regarding the MCS, MCS 16 for interfered UE can be considered for initial evaluation. For target UE, MCS 2, MCS 16 and MCS 20 can be considered as candidate for initial evaluation

Proposal 11: Consider MCS 16 for interfered UE, MCS 2, MCS 16 and MCS 20 as candidate for initial performance evaluation with MMSE-IRC receiver.

Waveform

Since our preference is to prioritize 1Tx UE scenario, both CP-OFDM and DFT-s-OFDM are feasible. For performance evaluation purpose, we can prioritize CP-OFDM firstly.

Proposal 12: Consider CP-OFDM for initial performance evaluation with MMSE-IRC receiver.

Others

For other simulation assumption, the existing parameter for PUSCH requirement for target UE and interfered UEs.

Proposal 13: Reusing the existing test parameter for target UE and interfered UEs for performance evaluation with MMSE-IRC receiver.

# 4	Conclusion

In this contribution, the view on the BS demodulation requirement with MMSE-IRC receiver.

Proposal 1:  Define PUSCH requirement with MMSE-IRC receiver for both homogeneous and heterogeneous network scenario.

Proposal 2:  Define PUSCH requirement with MMSE-IRC receiver for both FDD and TDD. The same TDD requirement are applicable to different UL-DL configuration.

Proposal 3:  Define PUSCH requirement with only synchronous scenario.

Proposal 4:  For target PUSCH and interference PUSCH, prioritize the 1Tx UE scenario. Deprioritize 2Tx UE scenario, and not cover 4Tx UE scenario.

Observation 1:  Number of interferer UE modeling depending on number of Rx number. Further evaluation the feasible of number interferer UE modeling before specifying the PUSCH requirement with MMSE-IRC receiver.

Proposal 5:  LTE interference profiles can be used for verifying the gain of MMSE-IRC compared with MMSE receiver as a starting point. FFS to derive the interference profile based on system level simulation with NR deployment for homogeneous and heterogeneous scenarios

Proposal 6:  Apply the DM-RS of severed targeting UE’s PUSCH for interference covariance matrix estimation. The baseline assumption for estimation of interference covariance matrix is performed at per PRB and per TTI basis.

Proposal 7: FFS on Technical Report needed to capture interference profile modeling and number of interfered UE explicitly modeling evaluation and analysis results.

Proposal 8: Consider 1Tx and 2/4/8Rx antenna configuration for target UE and interference UE to evaluate the performance gain of MMSE-IRC

Proposal 9: FFS to differentiate the channel model used for target UE and interfered UE for performance evaluation with MMSE-IRC receiver.

Proposal 10: Consider 15KHz SCS, 10MHz CBW and 30KHz SCS, 40MHz CBW for initial performance evaluation with MMSE-IRC receiver.

Proposal 11: Consider MCS 16 for interfered UE, MCS 2, MCS 16 and MCS 20 as candidate for initial performance evaluation with MMSE-IRC receiver.

Proposal 12: Consider CP-OFDM for initial performance evaluation with MMSE-IRC receiver.

Proposal 13: Reusing the existing test parameter for target UE and interfered UEs for performance evaluation with MMSE-IRC receiver.

# References

RP-240831, NR demodulation performance: Phase 5

TS 36.884