3GPP TSG-RAN WG4 Meeting #112	R4-2413453

Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:	8.24.2

Source: 	Qualcomm Incorporated

Title: 		RRM core requirements for XR enhancements

Document for:	Discussion

# Introduction

For release 19, a new work item on XR enhancements (Phase 3) for NR has been agreed where the following WID [1] objective is expected to have impact on RRM requirements:

- Specify enhancements to enable transmission/reception in gaps/restrictions that are caused by RRM measurements (from inter-frequency RRM measurement gaps, or intra-frequency measurements, or other scheduling restrictions etc). [RAN1, RAN2, RAN4] - Specify the corresponding measurement gap and scheduling restriction to enable the identified enhancements with RRM performance impact taken into consideration, work being triggered by LS. [RAN4]

RAN1 has been discussing mechanisms to enable transmission/reception in gaps/restrictions that are caused by RRM measurements and have identified the following potential solutions:

Alt. 1: Dynamic indication to enable Tx/Rx in particular gap(s)/restriction(s) that are caused by RRM measurements

Alt. 3: Semi-static solution to enable TX/RX in gaps/restrictions that are caused by RRM measurements.

Irrespective of the scheme that is going to be eventually adopted by RAN1/RAN2, it’s quite evident that RRM measurements are going to be impacted. In this contribution, we provide an initial analysis of the RRM requirements that are expected to be impacted as well as views on the UAI aspects of the MG deactivation.

# Impact on RRM requirements due to MG deactivation

It’s quite well understood that, in most cases, measurement gaps (MG) used for RRM measurements take higher priority over data transmission to ensure that the UE connection is maintained. While such measurements are quite important for the UE for mobility purposes, the UE’s inability to transmit/receive data during the MGs has a major impact on the QoS for delay critical traffic such as XR where the data arrives in periodic bursts. Since these periodic bursts have a non-integer periodicity of 16.67ms, their collision with MGs cannot be avoided. The current measurement rules provide a higher priority to MGs, causing a delay in scheduling XR traffic, which is not desirable for XR applications which are highly delay sensitive. To address this issue, one of the main objective of this WI is to enable transmission/reception in gaps/restrictions that are caused by RRM measurements (from inter-frequency RRM measurement gaps, or intra-frequency measurements, or other scheduling restrictions etc.)

Most of the RRM measurement requirements specified in TS 38.133, such as PSS/SSS detection and SSB index identification period and measurement period, are specified as multiple of measurement gap repetition period (MGRP). Deactivation of any MG during these periods would result in UE not having sufficient measurement samples to meet the performance requirements related to delay and accuracy. One simple way to handle this would be to extend the delay requirements based on the number of deactivated MGs. A similar methodology was also followed during NR-U, where the delays were extended based on the number of LBT failures. In our opinion, from RRM requirements perspective, MG deactivation for XR can be handled in a similar manner.

Observation 1: RAN1/RAN2 are discussing schemes to enable transmission/reception in gaps/restrictions that are caused by RRM measurements.

Observation 2: Irrespective of the MG deactivation scheme adopted by RAN1/RAN2, RRM measurement requirements related to cell identification and measurements with MGs are going to be impacted.

Proposal 1: RAN4 to consider extending the L3 requirements with MGs such as cell identification and measurements delays when one or more MGs are deactivated by the network.

- The exact extension may depend on further RAN1/RAN2 discussion and agreements.

The next thing to consider is the type of MGs. RAN4 considers multiple measurement gaps such as per UE or per FR MGs, where one or more gaps can be configured as pre-configured MG or NCSG or MUSIM gaps. In some cases, multiple MGs can be activated concurrently. It’s not clear from RAN1/RAN2 discussion whether they consider all type of MGs for deactivation. Furthermore, these multiple types of measurement gaps were introduced by RAN4 after extensive discussion and RAN1/RAN2 may not have full expertise in making decisions related to these aspects of MGs. So, we think, RAN4 should also discuss how to handle different type of MGs and their concurrent operation.

Observation 3: RAN4 requirements are specified for different type of measurement gaps such as pre-MG, NCSG, MUSIM gaps etc., along with their concurrent operation.

Proposal 2: RAN4 to discuss the applicability of MG deactivation to different type of measurement gaps such as pre-MG, NCSG, MUSIM gaps etc., along with their concurrent operation.

# UAI aspects

The next issue to discuss is related to the UE assistance information (UAI) for measurement gap deactivation.

During RAN1#117, RAN1 discussed whether or not to introduce new UE assistance information for solution(s) to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements. Multiple UAI related to measurements occasions and related to channel conditions, traffic, UE mobility etc were discussed and reached a conclusion that RAN1 does not further discuss new UE assistance information related to channel conditions, traffic, UE mobility. However, there was no consensus in RAN1 to support UE assistance information related to measurements occasions. Finally, RAN1 sent an LS [2], asking RAN4 to decide whether or not to introduce any UE assistance information related to measurement occasions.

Although the motivation behind deactivating MGs is quite clear, but the importance of MGs also needs to be considered. In some cases, the network may deactivate excessive number of measurement gaps leading to very long cell detection/identification and measurement periods. Since, it’s up-to UE implementation which MG occasion it uses for each frequency layer/cell to be measured, RAN4 requirements will have to extended for all frequency layers to be measured. Some UEs may not want to track every cell for an extended period of time as it may lead to use of stale measurement samples which may degrade performance accuracy as well as higher memory requirements to store samples for a longer period. Consequentially, extremely long measurement periods will have a negative impact on UE mobility performance. So, we think it’d be beneficial for the UE to transmit some UAI to assist network in configuring reasonable number of MG deactivations to ensure minimum impact on RRM performance. UAI such as number of consecutive MGs that can be skipped, maximum number of MGs that can be skipped etc are a good starting point for the discussion. In addition to that, UE may also indicate what type of MG (per UE/per FR/pre-MG/MUSIM gaps etc.) can be deactivated by the network.

Observation 4: Deactivation of excessive number of (consecutive or non-consecutive) measurement gaps by the network may have negative impact on RRM performance.

Proposal 3: RAN4 to consider introducing UAI to indicate one or more of the following aspects for MG deactivation

- Maximum number of consecutive MGs that can be skipped

- maximum number of MGs that can be skipped

- MG type that can be skipped

# Conclusion

Observation 1: RAN1/RAN2 are discussing schemes to enable transmission/reception in gaps/restrictions that are caused by RRM measurements.

Observation 2: Irrespective of the MG deactivation scheme adopted by RAN1/RAN2, RRM measurement requirements related to cell identification and measurements with MGs are going to be impacted.

Proposal 1: RAN4 to consider extending the L3 requirements with MGs such as cell identification and measurements delays when one or more MGs are deactivated by the network.

- The exact extension may depend on further RAN1/RAN2 discussion and agreements.

Observation 3: RAN4 requirements are specified for different type of measurement gaps such as pre-MG, NCSG, MUSIM gaps etc., along with their concurrent operation.

Proposal 2: RAN4 to discuss the applicability of MG deactivation to different type of measurement gaps such as pre-MG, NCSG, MUSIM gaps etc., along with their concurrent operation.

Observation 4: Deactivation of excessive number of (consecutive or non-consecutive) measurement gaps by the network may have negative impact on RRM performance.

Proposal 3: RAN4 to consider introducing UAI to indicate one or more of the following aspects for MG deactivation

- Maximum number of consecutive MGs that can be skipped

- maximum number of MGs that can be skipped

- MG type that can be skipped

# References

[1] 	RP-240791, “Revised WID on XR (eXtended Reality) for NR Phase 3”, Nokia, RAN#103

[2] R4-2411007, “LS on UE assistance information”, Nokia, RAN4#112