3GPP TSG-RAN WG4 Meeting #112	R4-2413454

Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:	8.19.3

Source: 	Qualcomm Incorporated

Title: 		RRM core requirements for SBFD

Document for:	Discussion

# Introduction

For release 19, a new work item on Evolution of NR duplex operation: Sub-band full duplex (SBFD) has been agreed where the main objective is to specify the sub-band full duplex operation. In addition to that, enhancements to CLI handling scheme is another WID [1] objective that is expected to have impact on RRM requirements:

- Specify enhancements for CLI handling [RAN1, RAN2, RAN3, RAN4] - Information exchange among gNBs, including [RAN3] - Semi-static cell-specific SBFD time and frequency location configuration - Measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS - Strongest DL beam information - CLI-mitigation request - UL resource muting for PUSCH, including [RAN1, RAN2, RAN4] - Indication/determination of UL resource muting for PUSCH based on semi-static configuration, assuming comb-2 for both DFT-S-OFDM and CP-OFDM in each allocated PRB and up to 2 symbols in time domain - PUSCH resource mapping, i.e., rate-matching around the muted REs - UCI resource determination in symbols with muted REs - L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, including [RAN1, RAN2, RAN4] - Periodic, semi-persistent, or aperiodic measurement resource (set), i.e., SRS-RSRP resource or CLI-RSSI resource, and configuration/determination of ‘typeD’ QCL assumptions for the CLI measurement resource - At least aperiodic reporting - New report quantities, e.g., L1-SRS-RSRP, L1-CLI-RSSI and/or measurement resource indices - UCI bits generation - Priority rules for multiple CSI reporting - CLI measurement accuracy requirement - Note: Without dedicated optimization for dynamic/flexible TDD. - Specify applicable RRM core requirements for CLI handling mechanisms [RAN4] - Specify other RRM core requirements for SBFD operation, if identified [RAN4]

# L1 based UE-to-UE CLI measurement requirements

It’s quite clear that RAN4 needs to define new L1 based UE-to-UE CLI measurement requirements. While RAN1 is discussing the details on the whole CLI measurement mechanism including signaling, measurement resources, QCL determination, reporting mechanism etc., RAN4 can start initial discussion to identify what new RRM requirements need to be defined. For instance, L3 CLI measurements are applicable only when SRS-RSRP and/or CLI-RSSI measurement resource is fully confined within BW of DL active BWP. However, during SBFD symbols/slots, RAN4 to discuss whether the UE can only receive in the DL sub-band or in UL sub-band as well, which is subset of the DL BWP.

Proposal 1: RAN4 to start discussing L1 UE-to-UE CLI measurement requirements for RRM.

Proposal 2: RAN4 to discuss the measurement bandwidth for L1 CLI measurements, e.g., DL subband.

Furthermore, when more than one DL sub-bands are configured, RAN4 should also discuss whether the measurement quantities can be filtered/combined across DL subbands.

Proposal 3: When more than one DL sub-bands are configured, RAN4 to discuss whether the measurement quantities can be filtered/combined across DL subbands.

In addition to this, RAN4 also need to specify the measurement delays and accuracy requirements, scheduling and measurement restrictions etc.

# SBFD operation aspects

From SBFD operation perspective, RAN4 can start identifying impact to existing requirements and if any new requirements are needed to support RRM operation. Since RAN1 agreed that SSBs shall be preserved, we don’t see much impact to SSB based measurements. However, CSI-RS based measurements may be impacted because of reduced number of PRBs either in a single sub-band or across two sub-bands. Filtering CSI-RS measurements across sub-bands may be challenging, so performing measurements and reporting per sub-band is desirable.

Proposal 4: RAN4 to start identifying impact of SBFD operation to RRM requirements such as CSI-RS based measurements.

Another issue may be frequent DL-UL/UL-DL switching during the SBFD slots. This may have negative impact on system performance as the switching delays will affect the throughputs as well as UE complexity. To avoid this, RAN4 should consider putting limits on the number of Tx/Rx switching within the SBFD slots.

Proposal 5: RAN4 to consider putting limits on the maximum number of DL/UL switching during SBFD slots within the SBFD periodicity.

# Conclusion

Proposal 1: RAN4 to start discussing L1 UE-to-UE CLI measurement requirements for RRM.

Proposal 2: RAN4 to discuss the measurement bandwidth for L1 CLI measurements, e.g., DL subband.

Proposal 3: When more than one DL sub-bands are configured, RAN4 to discuss whether the measurement quantities can be filtered/combined across DL subbands.

Proposal 4: RAN4 to start identifying impact of SBFD operation to RRM requirements such as CSI-RS based measurements.

Proposal 5: RAN4 to consider putting limits on the maximum number of DL/UL switching during SBFD slots within the SBFD periodicity

# References

[1] 	RP-241614, “Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, Huawei, RAN#104