3GPP TSG-RAN WG4 Meeting # 112	R4- 2413455

# Maastricht, Netherlands, 19th – 23th August, 2024

Source:	LG Electronics

Title:	Discussion on Ambient IoT Device RF requirement impact

Agenda item:	8.20.3.2

Document for:	Discussion

Introduction

In 3GPP RAN #111, WF[1] was approved. In this paper, we provide our views on Ambient IoT Device RF requirement impact Discussion

Discussion

Tx requirement

The following three structures have been discussion in RAN1 until the last meeting.

- Device 1: ~1 µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, neither DL nor UL amplification in the device. The device’s UL transmission is backscattered on a carrier wave provided externally.

- Device 2a: ≤ a few hundred µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, both DL and/or UL amplification in the device. The device’s UL transmission is backscattered on a carrier wave provided externally.

- Device 2b: ≤ a few hundred µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, both DL and/or UL amplification in the device. The device’s UL transmission is generated internally by the device.

Each device is characterized by a very small power consumption (less than 1uW) and no DL UL amplification in the case of device1. Device2a has a relatively large power consumption of a few hundred units compared to device1 and has DL and/or UL amplification. Device1 and device2a send D2R signals by backscattering. Device2b has a similar power consumption limit to device2a with a few hundred uW and also has UL and/or UL amplification. The difference with Device 2a is that it can generate UL transmission inside the device. The following figures show the structure of each device.

[Device 1] [Device 2a]

Since there is no amplification for any UL and DL, device1 cannot have a transmit output power that is flexible according to the active change of PA, unlike the existing NR RF requirement. In addition, since there is no device that can generate power internally, the output power is affected by the power coming from the outside, so it is dependent on the power of the ambient IoT reader. In addition, there is also a path loss as this power goes through the device, so it will be necessary to clarify the characteristics of each device to analyze the level at which the D2R signal satisfies the reference sensitivity of the reader. In the case of device1, it will be necessary to clarify the internal device characteristics such as loss because it is a passive device.

Observation 1: The transmit output power of device1 is dependent on the ambient-IoT reader power.

Proposal 1: For device1, It is necessary to clarify insertion loss of each block.

Also, in the case of device1, there are no devices inside the device that can transmit any ULs and DLs, so parameters such as power control and transmit off power, which were previously discussed in WF, do not need to be discussed.

Proposal 2: For device1, there is no need to define Transmit OFF power and power control requirement.

In the case of device2, unlike device1, there is a reflection amplifier, and it is discussed that the amplification gain will be about 10 to 20dB. Depending on whether it is taken in the form of a reflection amplifier gain range or a power class, the transmit output power requirement will be specified. And currently, in the case of device2a, there are still blocks under discussion. Blcoks such as Large Frequency shifter and RF energy harvester are under discussion, and those blocks may be eliminated after evaluating the feasibility in RAN1. Therefore, in the case of device2, we need to proceed with the discussion after the above structures are finalized.

Proposal 3: For device2a, it is necessary to wait conclusion for device structure from RAN1

For the output RF spectrum proposed by WF[1], Adjacent channel leakage ratio (ACLR) will be defined based on the co-existence simulation results. And spurious emission should meet regulatory requirement.

Proposal 4: Adjacent channel leakage ratio (ACLR) will be defined based on the co-existence simulation results. And spurious emission should meet regulatory requirement.

Rx requirement

The ACS and ASCS will be defined based on co-existence simulation.

Proposal 5: For Rx requirement, The ACS and ASCS will be defined based on co-existence simulation.

- 2. Conclusion

In this contribution, we want to share some views on Ambient IoT Device RF requirement impact and proposals are made as following:

Observation 1: The transmit output power of device1 is dependent on the ambient-IoT reader power.

Proposal 1: For device1, It is necessary to clarify insertion loss of each block.

Proposal 2: For device1, there is no need to define Transmit OFF power and power control requirement.

Proposal 3: For device2a, it is necessary to wait conclusion for device structure from RAN1

Proposal 4: Adjacent channel leakage ratio (ACLR) will be defined based on the co-existence simulation results. And spurious emission should meet regulatory requirement.

Proposal 5: For Rx requirement, The ACS and ASCS will be defined based on co-existence simulation.

- 3. Reference

[1] R4-2410597, “WF on impacts of A-IoT on RF requirements”