3GPP TSG-RAN WG4 Meeting # 112	R4- 2413456

# Maastricht, Netherlands, 19th – 23th August, 2024

Source:	LG Electronics

Title:	Discussion on MPR applicability for FR1 intra-band UL CA

Agenda item:	8.1.1.2.2

Document for:	Discussion

Introduction

In 3GPP RAN #111, WF[2] was approved.

In this paper, we provide our views on MPR applicability for FR1 intra-band UL CA.

Discussion

MPR applicability for FR1 intra-band UL contiguous CA

The WF[2] was approved for suggestions as follows.

Way forward: - For PC3 and PC2 contiguous UL CA use the corresponding single CC MPR when only 1 CC is activated. - FFS: MPR defined in Table 6.2.2-1 applies for UE power class 3 CA bandwidth classes B and C. - FFS: MPR defined in Table 6.2D.2-1 applies for power class 2 CA bandwidth classes B and C when TxD capability is indicated. - FFS: MPR defined in Table 6.2.2-2 applies for power class 2 CA bandwidth classes B and C when TxD capability is absent. - FFS whether spurious emissions/ACLR/SEM are kept for aggregated CBW for CA

All of the above issues seem to stem from the question of whether it is necessary to follow the CA MPR table when using only 1CC in a structure that is both PC3 and PC2 contiguous UL CA. Since the existing defined MPR table considers the MPR table assuming 2CC as currently configured, We think this is a validation that a structure that is a contiguous UL CA can fall back to a structure for single carrier when assuming 1CC.

- For PC3 and PC2 contiguous UL CA use the corresponding single CC MPR when only 1 CC is activated. - FFS: MPR defined in Table 6.2.2-1 applies for UE power class 3 CA bandwidth classes B and C.

For the above issue, currently, there are three structures for MPR table defined by contiguous UL CA. UL CA have 1PA+1LO structure that does not support TxD or dualPA-Architecture or both. In this case, the above issue is a structure that does not include the following two structures, TxD or dualPA-Architecture, and has only 1 LO and 1PA. In this case, it is possible to modify the structure to 1CC. So MPR defined in Table 6.2.2-1 could applies for UE power class 3 CA bandwidth classes B and C.

Proposal 1: MPR defined in Table 6.2.2-1 could applies for UE power class 3 CA bandwidth classes B and C.

- FFS: MPR defined in Table 6.2D.2-1 applies for power class 2 CA bandwidth classes B and C when TxD capability is indicated.

The TxD structure is currently characterized by having a 2PA+1LO structure. The way to fall back to the structure of single carrier operation from that structure is to shift the LO frequency. If it shift the LO frequency from the center of each CC to one side, it can fall back to the structure of single carrier operation. So MPR defined in Table 6.2D.2-1 also could applies for power class 2 CA bandwidth classes B and C when TxD capability is indicated.

Proposal 2: MPR defined in Table 6.2D.2-1 could applies for power class 2 CA bandwidth classes B and C when TxD capability is indicated.

- FFS: MPR defined in Table 6.2.2-2 applies for power class 2 CA bandwidth classes B and C when TxD capability is absent.

This is the same discussion as the first issue. MPR defined in Table 6.2.2-2 could applies for power class 2 CA bandwidth classes B and C when TxD capability is absent.

Proposal 3: MPR defined in Table 6.2.2-2 could applies for power class 2 CA bandwidth classes B and C when TxD capability is absent.

- 2. Conclusion

In this contribution, we want to share some views on power domain enhancement and proposals are made as following:

Proposal 1: MPR defined in Table 6.2.2-1 could applies for UE power class 3 CA bandwidth classes B and C.

Proposal 2: MPR defined in Table 6.2D.2-1 could applies for power class 2 CA bandwidth classes B and C when TxD capability is indicated

Proposal 3: MPR defined in Table 6.2.2-2 could applies for power class 2 CA bandwidth classes B and C when TxD capability is absent.

- 3. Reference

[1] R4-240828, “New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4”

[2] R4-2410749, “WF on power domain enhancements”