3GPP TSG-RAN WG4 Meeting # 112	R4- 2413457

# Maastricht, Netherlands, 19th – 23th August, 2024

Source:	LG Electronics

Title:	Discussion on Power domain enhancements for single carrier

Agenda item:	8.1.1.2.1

Document for:	Discussion

Introduction

In 3GPP RAN #111, WF[2] was approved.

In this paper, we provide our views on power domain enhancements for single carrier.

Discussion

Consideration on relaxiation of requirements for identified scenarios

The WF[2] was approved for the scenario as follows.

1.1 Consideration on relaxation of requirements for identified scenarios Agreement in main session: - No relaxation of level of spurious emission requirements - FFS on whether to change the spurious emission boundary

the legacy NR spurious emission mask was represented in the 38.101-1. The following table was shown.

Table 6.5.3.1-1: Boundary between NR out of band and general spurious emission domain Channel bandwidth; OOB boundary FOOB (MHz). 3; 6. 5, 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100; BWChannel + 5. Table 6.5.3.1-2: Requirement for general spurious emissions limits Frequency Range; Maximum Level; Measurement bandwidth; NOTE. 9 kHz ≤ f < 150 kHz; -36 dBm; 1 kHz. 150 kHz ≤ f < 30 MHz; -36 dBm; 10 kHz. 30 MHz ≤ f < 1000 MHz; -36 dBm; 100 kHz. 1 GHz ≤ f < 12.75 GHz; -30 dBm; 1 MHz; 4. -25 dBm; 1 MHz; 3. 12.75 GHz ≤ f < 5th harmonic of the upper frequency edge of the UL operating band in GHz; -30 dBm; 1 MHz; 1. 12.75 GHz < f < 26 GHz; -30 dBm; 1 MHz; 2. NOTE 1: Applies for Band for which the upper frequency edge of the UL Band is greater than 2.55 GHz and less than or equal to 5.2 GHz NOTE 2: Applies for Band that the upper frequency edge of the UL Band more than 5.2 GHz NOTE 3: Applies for Band n41, CA configurations including Band n41, and EN-DC configurations that include n41 specified in clause 5.2B of TS 38.101-3 [3] when NS_04 is signalled. NOTE 4: Does not apply for Band n41, CA configurations including Band n41, and EN-DC configurations that include n41 specified in subclause 5.2B of TS 38.101-3 [3] when NS_04 is signalled..

Spurious emission mask was defined according to Table 6.5.3.1-2 as shown in Figure 1.

Figure 1. Spurious emission mask

Generally, Spurious emission boundary was defined by reffering to the regulations of international and local regulation such as ITU, ETSI and FCC. In case of ITU, spurious emission boundary was decided that the boundary is 250% away from centre frequency. As shown in follow table.

<ITU-R SM.329-10>

when we compared with 3GPP requirement. 3GPP requirement is little bit tighter than ITU regulation. Example is shown as Figure 2, if the centre frequency is 3GHz and channel bandwidth is 30MHz. the boundary of OOB is 3.015GHz+5MHz so the boundary of OOB is 3.035GHz in 3GPP requirement. but conversely if according to ITU regulation. As a result, the boundary of OOB is 3.075GHz.

Figure 2. Example of spurious emission boundary with ITU regulation and 3GPP requirement

there is a room to relax boundary of OOB. When it is compared between ITU-R regulation and 3GPP requirement

Proposal 1: when it is compared with ITU-R regulation, there is a room to relax boundary of OOB.

Mechanisms for enabling MPR reduction and/or power boosting

The WF was approved for mechanism as follows.

1.2. Mechanisms for enabling MPR reduction and/or power boosting - Study the approach to convert full RB allocation in UE CBW to “inner RB allocation” with an extended UE BW, e.g. each side of the UE BW is equal to ½ of the UE BW, inside a larger BS CBW as starting point with consideration of the following aspects - The edge of the extended UE BW, i.e. the UE CBW plus the shifted frequency symmetrically at each side of the UE CBW, should be inside BS CBW or at least aligned with the BS CBW edge - ① FFS feasibility of case where extended UE CBW edge exceeds the BS CBW edge, i.e. gap between edges of UE CBW and BS CBW < 1/2 UE CBW - ② FFS whether IBE is used between edges of UE CBW and BS CBW or between edges of UE CBW and extended UE CBW - ③ FFS ACLR and SEM are applicable from the edge of extended UE CBW or edge of BS CBW, i.e. the start point of ΔfOOB - ④ FFS integral region and boundary of OOBE is based on extended UE CBW or UE CBW

Suggested mechanism is used for applying inner MPR to assigned channel bandwidth. As a result, MPR of transmission BW that Redcap UE use will be reduced. Although actual channel bandwidth is narrow but OOB requirement of extended channel bandwidth is applied. We thought it is good method to simply reduce MPR. But it need to consider capability for guaranting new mechanism.

Proposal 2: Need to consider capability for extended UE CBW procedure.

① FFS feasibility of case where extended UE CBW edge exceeds the BS CBW edge, i.e. gap between edges of UE CBW and BS CBW < 1/2 UE CBW

Regarding issue ①, in our understanding, in case of gap between edges of UE CBW and BS CBW < 1/2 UE CBW have feasibility. Because guard band was applied with inband emission requirement. Inband emission is tigher than OOB requirement and spurious emission mask. So in feasility aspect, extended UE CBW edge should excced the BS CBW edge upto 1/2 UE CBW.

Proposal 3: In feasilbity aspect, extended UE CBW edge should excced the BS CBW edge upto 1/2 UE CBW.

② FFS whether IBE is used between edges of UE CBW and BS CBW or between edges of UE CBW and extended UE CBW

In case of issue ②, we thought that it don’t need to define new Inband emission requirement. So IBE should apply between edge of UE CBW and extended UE CBW

Proposal 4: IBE should use between edge of UE CBW and extended UE CBW

③ FFS ACLR and SEM are applicable from the edge of extended UE CBW or edge of BS CBW, i.e. the start point of ΔfOOB

Regarding issue ③,  similar with issue ②, it don’t need to define new Inband emission requirement. ACLR and SEM are applicable the edge of extended UE CBW

Proposal 5: ACLR and SEM are applicable the edge of extended UE CBW

- 2. Conclusion

In this contribution, we want to share some views on power domain enhancement and proposals are made as following:

Proposal 1: There is a room to relax boundary of OOB. when it is compared with ITU-R regulation.

Proposal 2: Need to consider capability for extended UE CBW procedure.

Proposal 3: In feasilbity aspect, extended UE CBW edge should excced the BS CBW edge upto 1/2 UE CBW.

Proposal 4: IBE should use between edge of UE CBW and extended UE CBW

Proposal 5: ACLR and SEM are applicable the edge of extended UE CBW

- 3. Reference

[1] R4-240828, “New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4”

[2] R4-2410749, “WF on power domain enhancements”