3GPP TSG-RAN WG4 Meeting #112	R4-2413458

# Maastricht, Netherlands, August 19th – August 23rd, 2024

# Title: 	Ku Band Channel Allocation for Legacy Satellite System

# Source:	SKY Perfect JSAT

# Type:	Discussion

# Document for:	Discussion

# Agenda Item:	8.9.3 System parameters

Release	Rel-19

- Introduction

The transponder used in the “Networking-RAN architecture with transparent satellite” performs operations such as power amplification on a per-satellite-channel basis. Filters are applied to each satellite channel, and guard bands are necessary between the satellite channels. In recent years, the softwarization of satellite transponders has advanced, leading to the emergence of Software Defined Satellites (SDSs). Although there are some limitations with SDSs, the satellite channel allocation is flexible, allowing it to be adjusted to meet NR specifications. Conversely, legacy satellites are composed of analogue components, resulting in fixed satellite channel allocations that cannot be freely changed.  However, satellite lifetime is generally quite long, more than15 years, and replacement from legacy satellites into SDSs will take time and during its transition time legacy satellites will be main capacity for Ku-band NTN, therefore, it is preferable to utilize both SDSs and legacy satellites for 5G NR services in order to widely deploy terminals in a timely manner to both legacy satellites and SDSs and various GEO operators who does and does not have plan to launch SDSs. This document provides the perspective on how to allocate and utilize NR channels on legacy satellite system.

- Discussion

General Structure of Satellite Transponder

Figure 2.1-1 shows the typical structure of an SDS transponder. The RF signal is received at the satellite antenna, amplified by a low noise amplifier (LNA), down-converted to an intermediate frequency (IF), converted from analogue to digital and processed by the Digital Transparent Processor. It is then converted back from digital to analogue, up-converted to the downlink frequency, amplified by a high-power amplifier (HPA) and transmitted back to earth from the satellite antenna. The bandwidth of the satellite channels can be flexibly controlled by a digital channelizer to fit the NR channel bandwidths.

Figure 2.1-1 General Structure of SDS’s Transponder

Figure 2.1-2 shows the structure of a legacy satellite’s transponder. RF signals received at the satellite antenna are amplified by a low noise amplifier (LNA), down-converted to a downlink frequency, divided by an input multiplexer (IMUX) to each channel, amplified by a high power amplifier (HPA), combined by an output multiplexer (OMUX), and transmitted from the satellite antenna. The bandwidth of the satellite channels depends on the filter characteristics of the IMUX and OMUX, and the bandwidth is fixed.

Figure 2.1-2 General Structure of Legacy Satellite’s Transponder

Legacy Satellite Channel Allocation

An example of satellite channel allocation of a legacy GEO satellite is shown in Figure 2.2-1 and Figure 2.2-2. Figure 2.2-1 illustrates an example of frequency allocation for 32 channels with a bandwidth of 27 MHz. Figure 2.2-2 illustrates an example of frequency allocation for 12 channels with a bandwidth of 36 MHz and 16 channels with a bandwidth of 27 MHz. Guard bands are provided to prevent interference between satellite channels and 27 MHz channels are allocated at 30 MHz spacing and 36 MHz channels are allocated at 40 MHz spacing respectively. Legacy GEO satellites typically use linear polarization. Isolation is achieved using horizontal and vertical polarizations, allowing the simultaneous use of both polarizations. In horizontal and vertical polarizations, the satellite channel allocation is offset by half of the satellite channel spacing.

Figure 2.2-1 An Example of Satellite Channel Allocation for GEO Satellite (27 MHz x 32 Channels)

Figure 2.2-2 An Example of Satellite Channel Allocation for GEO Satellite (36 MHz x 12 Channels & 27MHz x 16 Channels)

The satellite channel bandwidth of existing legacy satellites is not limited to 27 MHz. Figure 2.2-3 shows examples of various legacy satellite channel bandwidths. Examples of satellite channel spacings for legacy satellites include 30 MHz, 40 MHz, 60 MHz, 80 MHz, and 120MHz. The guard bands between satellite channels are fixed and cannot be utilized. The guard band for each satellite channel spacing varies depending on the implementation. For the example satellite channel spacings shown, the guard bands are 3 MHz, 4 MHz, 6 MHz, 8 MHz, and 12 MHz, with corresponding satellite channel bandwidths of 27 MHz, 36 MHz, 54 MHz, 72 MHz, and 108 MHz respectively.

Figure 2.2-3 An Example of Satellite Channel Spacing and Satellite Channel Bandwidth for Legacy GEO Satellite

NR Channel Definition of FR1 and FR2

The Ku-band NR channel bandwidth options will be determined by considering the existing NR channel bandwidth options for FR1 and FR2. Tables 2.3-1 and 2.3-2 below contain the existing FR1 channel bandwidths and minimum guard bands for reference. [1].

Table 2.3-1: Maximum transmission bandwidth configuration NRB

| SCS (kHz) | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB |
| 15 | 15 | 25 | 52 | 79 | 106 | 133 | 160 | 188 | 216 | 242 | 270 | N/A | N/A | N/A | N/A | N/A |
| 30 | N/A | 11 | 24 | 38 | 51 | 65 | 78 | 92 | 106 | 119 | 133 | 162 | 189 | 217 | 245 | 273 |
| 60 | N/A | N/A | 11 | 18 | 24 | 31 | 38 | 44 | 51 | 58 | 65 | 79 | 93 | 107 | 121 | 135 |

Table 2.3-2: Minimum guardband for each UE channel bandwidth and SCS (kHz)

| SCS (kHz) | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15 | 142.5 | 242.5 | 312.5 | 382.5 | 452.5 | 522.5 | 592.5 | 572.5 | 552.5 | 712.5 | 692.5 | N/A | N/A | N/A | N/A | N/A |
| 30 | N/A | 505 | 665 | 645 | 805 | 785 | 945 | 925 | 905 | 1065 | 1045 | 825 | 965 | 925 | 885 | 845 |
| 60 | N/A | N/A | 1010 | 990 | 1330 | 1310 | 1290 | 1630 | 1610 | 1590 | 1570 | 1530 | 1490 | 1450 | 1410 | 1370 |

Similarly, tables 2.3-3 and 2.3-4 below contain the FR2 channel bandwidths and minimum guard bands [2].

Table 2.3-3: Maximum transmission bandwidth configuration NRB

| SCS (kHz) | 50 MHz | 100 MHz | 200 MHz | 400 MHz | 800 MHz | 1600 MHz | 2000 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB | NRB | NRB |
| 60 | 66 | 132 | 264 | N/A | N/A | N/A | N/A |
| 120 | 32 | 66 | 132 | 264 | N/A | N/A | N/A |
| 4801 | N/A | N/A | N/A | 66 | 124 | 248 | N/A |
| 9601 | N/A | N/A | N/A | 33 | 62 | 124 | 148 |
| Note 1: This SCS is optional in this release of the specification. |  |  |  |  |  |  |  |

Table 2.3-4: Minimum guardband for each UE channel bandwidth and SCS (kHz)

| SCS (kHz) | 50 MHz | 100 MHz | 200 MHz | 400 MHz | 800 MHz | 1600 MHz | 2000 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 60 | 1210 | 2450 | 4930 | N/A | N/A | N/A | N/A |
| 120 | 1900 | 2420 | 4900 | 9860 | N/A | N/A | N/A |
| 480 | N/A | N/A | N/A | 9680 | 42640 | 85520 | N/A |
| 960 | N/A | N/A | N/A | 9440 | 42400 | 85280 | 147040 |

NR Channel Allocation for Legacy Satellites

    - Using FR1 Channels

Using a Smaller NR Channel Bandwidth than the Legacy Satellite’s Channel Bandwidth

By using FR1 channels, it is possible to fit the NR channel bandwidth within the legacy satellite channel spacing. An example of using only one polarization and allocating an NR channel within a legacy satellite channel is shown in Figure 2.4.1.1-1. A smaller NR channel bandwidth than the satellite channel bandwidth is selected. In this example, the satellite channel spacing is 40MHz, and the NR channel bandwidth is 35MHz.

Figure 2.4.1.1-1 (FR1) Using a Smaller NR Channel Bandwidth than the legacy Satellite’s Channel Bandwidth

This method can be used without impacting on RAN4 defined requirements, but there is an issue in effectively utilizing the legacy satellite channel bandwidth.

Observation 1: By using the FR1 channel definitions, it is possible to utilize NR channels with a bandwidth slightly smaller than the legacy satellite channel bandwidth.

Observation 2: Utilizing FR1 channels with a bandwidth smaller than the satellite channel bandwidth is possible without impacting RAN4 requirements.

Issue 1: By using FR1 channels with a bandwidth smaller than the legacy satellite channel bandwidth, satellite bandwidth is underutilized.

Using the Same NR Channel Bandwidth as the Legacy Satellite’s Channel Spacing

An example of using an NR channel bandwidth that matches the satellite channel spacing is shown in Figure 2.4.1.2-1. In this example, both the legacy satellite channel spacing and the NR channel bandwidth are 40MHz.

Figure 2.4.1.2-1 (FR1) Using the same NR Channel Bandwidth as Satellite’s Channel Spacing

The guard band of the satellite and the NR channel are different. If the satellite and NR channels have the same channel spacing, the satellite will have a larger guard band. Table 2.4.1.2-1 shows the Maximum Transmission Bandwidth Configuration NRB and Minimum Guard band for an NR channel with a 30kHz SCS, as well as the NRB value and actual guard band calculated to match the legacy satellite guard band.

Table 2.4.1.2-1 NRB to align the guard band with the legacy satellite's guard band

| Channel Bandwidth | 3 | 40 | 60 | 80 |
| --- | --- | --- | --- | --- |
|  | MHz | MHz | MHz | MHz |
| Maximum transmission bandwidth configuration NRB | 78 | 106 | 162 | 217 |
| Minimum Guard band (kHz) | 945 | 905 | 825 | 925 |
| Transponder Guard band (kHz) | 1500 | 2000 | 3000 | 4000 |
| Transmission Bandwidth Configuration NRB | 74 | 99 | 149 | 199 |
| Actual Guard band ( > Transponder Guard band)(kHz) | 1665 | 2165 | 3165 | 4165 |

The legacy satellite’s channel bandwidth is not aligned with the existing NR channel bandwidths. TR 38.844 studied how to utilize an irregular NR channel bandwidth by deploying the “larger channel bandwidth” method. As methods for using an NR channel with a larger bandwidth than the available bandwidth, the document describes using BWPs or scheduling only the usable PRBs.

Regarding the method of using BWPs, TR 38.844 states that “BWPs of sizes different from the already defined NR channel bandwidths are not currently tested within the RAN4 defined requirements.” Therefore, using BWPs that exactly match the available legacy satellite bandwidth (in this example, a channel bandwidth between 40 MHz and 35 MHz) requires a study by RAN4.

Observation 3: By using the definition of FR1 channels, it is possible to use an NR channel bandwidth that matches the legacy satellite channel spacing.

Issue 2: Using BWPs that exactly match the available legacy satellite bandwidth requires a study by RAN4 because BWPs of sizes different from the already defined NR channel bandwidths are not currently tested within the RAN4 defined requirements.

Issue 3: Using NR channel bandwidths that have already been tested within RAN4 defined requirements does not allow for efficient utilization of legacy satellite bandwidth.

To effectively utilize satellite bandwidth, the method of allocating only the usable bandwidth through scheduling is more efficient. The method of allocating PRBs through scheduling to exclude the satellite guard bands raises concerns about the use of masks defined for the NR channel bandwidth. However, since the satellite channel spacing in this example ensures the NR channel bandwidth, there is no need to consider additional impacts on adjacent NR channels. Therefore, there should be no impact on the RAN4 defined requirements, but JSAT would like to hear the RAN4 view on whether there are any issues with this approach.

Observation 4: By allocating only the PRBs that can be used through scheduling, the legacy satellite bandwidth can be utilized efficiently with no impact on existing RAN4 requirements.

Proposal 1: RAN4 to provide input on whether there are any issues or requirement impacts for scheduling fewer PRBs as a method for avoiding legacy satellite guard bands.

Considering Both Polarizations

An example of using both HV polarizations as NR channels is shown in Figure 2.4.1.3-1. In this example, the satellite channel spacing is 40MHz, and the NR channel bandwidth is also 40MHz.

Figure 2.4.1.3-1 (FR1) Using Both Polarization and Using the same NR Channel Bandwidth as Satellite’s Channel Spacing

Due to the offset in satellite channel allocation between H and V polarizations, the NR channel boundaries fall within the satellite channels. In the example shown in Figure 2.4.1.3-1, the NR channel boundary in the UE to gNB direction is located within S-04, and in the gNB to UE direction, it is located within S-04 and S-05. Although these NR channel boundaries are within the usable bandwidth of the satellite, they cannot be used because the boundaries are the guard bands for the NR channels.

Issue 4: When using both polarizations, the NR channel boundaries are placed within the usable satellite bandwidth, which makes it inefficient to fully utilize the satellite bandwidth.

Within the service link NR-02 channel in the gNB to UE direction, there are three regions that can be used as satellite frequencies: S-04 and S-05 in H polarization, and S-02 in V polarization. To avoid using the satellite guard bands, one can either use BWPs as described above or allocate only the usable bandwidth as PRBs through scheduling. The issues associated with these methods are similar to those in the case of single polarization.

    - Using FR2 Channels

Using FR2 NR Channels Without Overlap

By using FR2 channels, the NR channel bandwidths are larger than many of the legacy satellite channel spacings. Figure 2.4.2.1-1 shows an example of applying an FR2 channel with the closest channel bandwidth to the satellite channel spacing without overlapping any of the NR channels. In this example, the legacy satellite channel spacing is 40MHz, and the FR2 NR channel bandwidth is 50MHz.

Figure 2.4.2.1-1 (FR2) Using a Closest NR Channel Bandwidth to the Legacy Satellite’s Channel Bandwidth Without Overlap

When the satellite channel spacing is smaller than the smallest FR2 channel bandwidth of 50 MHz, and the FR2 channel that is closest to the satellite channel spacing is used to allocate NR channels without overlap, the NR channel boundaries are placed within the usable satellite bandwidth. This makes it inefficient to fully utilize the legacy satellite bandwidth since the usable bandwidth within the NR channels is that which excludes the legacy satellite guard bands. To avoid using the legacy satellite guard bands, one can either use BWPs or allocate only the usable bandwidth as PRBs through scheduling. This is the same issue described through section 2.4.1 for FR1.

Issue 5: When the legacy satellite channel spacing is smaller than the smallest FR2 channel bandwidth of 50 MHz, and the FR2 channel that is closest to the satellite channel spacing is used to allocate NR channels without overlap, the NR channel boundaries are placed within the usable satellite bandwidth, which makes it inefficient to fully utilize the legacy satellite bandwidth.

Overlapping NR Channels

Figure 2.4.2.2-1 shows an example of applying an FR2 channel with the closest channel bandwidth to the satellite channel spacing and overlapping the allocation of the NR channels. As with the non-overlapping case above, the satellite channel spacing is 40 MHz and the NR channel bandwidth is 50 MHz.

Figure 2.4.2.2-1 (FR2) Using One Polarization and Overlapping NR Channel Allocation

In this example, the NR channel bandwidth is 50 MHz, but due to BWPs or resource allocation through scheduling only the legacy satellite channel bandwidth of 36 MHz is used.

Overlapping carrier configurations like these may have some RAN4 issues. RF requirements (e.g., spectrum emission mask) are defined for a carrier, not the utilized bandwidth, so these overlapping configurations will not satisfy the RAN4 requirements. Furthermore, the same issues described in section 2.4.1 occur because only the usable bandwidth is scheduled either via BWPs or PRB allocation.

Issue 5: Applying an FR2 channel with the closest channel bandwidth to the satellite channel spacing with overlapping allocation of the NR channels will not satisfy the RAN4 requirements.

Comparison of FR1 and FR2 NR Channel Allocations for Legacy GEO Satellites in the Ku Band

The smaller channel bandwidths provided by FR1 allocations are more suitable than FR2 for accommodating the legacy GEO satellite channel spacings of 30 MHz, 40 MHz, 60 MHz, 80 MHz, and 120MHz. The closest FR1 bandwidth can be used with simple scheduling adjustments made to accommodate the larger guard bands of the legacy GEO satellite channels with little to no RAN4 impact. For FR2, the larger channel bandwidths make it difficult to efficiently utilise the smaller legacy bandwidth allocations, since the NR channel boundaries fall within the usable satellite bandwidth, and in the case that the NR channels are overlapped there may be substantial RAN4 impact. Furthermore, when comparing FR1 and FR2 in a general sense, it is fair to say that FR1 has a more mature industry status considering the standardization work was completed earlier and it has been more widely implemented and deployed. Therefore, to ensure legacy GEO satellite operators can meet their deployment requirements with minimal impact to RAN4, RAN4 to prioritize the channel definitions of FR1 over these of FR2.

Observation 5: When comparing FR1 and FR2 in a general sense, it is fair to say that FR1 has a more mature industry status considering the standardization work was completed earlier and it has been more widely implemented and deployed.

Proposal 2: To ensure legacy GEO satellite operators can meet their deployment requirements with minimal impact to RAN4, RAN4 to prioritize the channel definitions of FR1 over these of FR2.

- Conclusions

Observation 1: By using the FR1 channel definitions, it is possible to utilize channels with a bandwidth slightly smaller than the legacy satellite channel bandwidth.

Observation 2: Utilizing FR1 channels with a bandwidth smaller than the satellite channel bandwidth is possible without impacting RAN4 requirements.

Observation 3: By using the definition of FR1 channels, it is possible to use an NR channel bandwidth that matches the legacy satellite channel spacing.

Observation 4: By allocating only the PRBs that can be used through scheduling, the legacy satellite bandwidth can be utilized efficiently with no impact on existing RAN4 requirements.

Observation 5: When comparing FR1 and FR2 in a general sense, it is fair to say that FR1 has a more mature industry status considering the standardization work was completed earlier and it has been more widely implemented and deployed.

Issue 1: By using FR1 channels with a bandwidth smaller than the legacy satellite channel bandwidth, satellite bandwidth is underutilized.

Issue 2: Using BWPs that exactly match the available legacy satellite bandwidth requires a study by RAN4 because BWPs of sizes different from the already defined NR channel bandwidths are not currently tested within the RAN4 defined requirements.

Issue 3: Using NR channel bandwidths that have already been tested within RAN4 defined requirements does not allow for efficient utilization of legacy satellite bandwidth.

Issue 4: When using both polarizations, the NR channel boundaries are placed within the usable satellite bandwidth, which makes it inefficient to fully utilize the satellite bandwidth.

Issue 5: When the legacy satellite channel spacing is smaller than the smallest FR2 channel bandwidth of 50 MHz, and the FR2 channel that is closest to the satellite channel spacing is used to allocate NR channels without overlap, the NR channel boundaries are placed within the usable satellite bandwidth, which makes it inefficient to fully utilize the legacy satellite bandwidth.

Issue 6: Applying an FR2 channel with the closest channel bandwidth to the satellite channel spacing with overlapping allocation of the NR channels will not satisfy the RAN4 requirements.

Proposal 1: RAN4 to provide input on whether there are any issues or requirement impacts for scheduling fewer PRBs as a method for avoiding legacy satellite guard bands.

Proposal 2: To ensure legacy GEO satellite operators can meet their deployment requirements with minimal impact to RAN4, RAN4 to prioritize the channel definitions of FR1 over these of FR2.

- References

- TS 38.101-1

- TS 38.101-2