3GPP TSG-RAN WG4 Meeting #112	R4-2413462

# Maastricht, Netherlands, Aug 19 – Aug 23, 2024

Source:	Nokia

Title:	RRM core maintenance for NR Carrier Phase Positioning

Agenda item:	6.1.1.2

Document for:	Discussion

Introduction

The specification of NR carrier phase positioning (NR CPP) is one of the objectives in the Rel-18 WID on expanded and improved positioning [1].

This contribution lists discussion on some open issues with regard to impacts to RRM core requirements for NR CPP [2].

Discussion

RAN4 agreements and open issues on core requirements

RAN4 agreements and list of open issues from RAN4 #111 are captured below [2]. More details are available in [3].

## Sub-topic 2-1 Carrier Phase Positioning Core Requirements Maintenance (agenda 7.12.1.2) ### Issue 2-1-1: Clarification on the PRS measurement period requirements for DL RSCP/DL RSCPD Agreements: For UE configured for CPP measurement with legacy measurement with multiple PFLs, when LMF does not configure measurement time window(s) for a PFL - UE performs DL RSCPD/RSCP measurement on a single PFL that is common between the reference TRP and the target TRP. - Existing requirements for RSTD/UE Rx-Tx without time window apply. For UE configured for CPP measurement with legacy measurement with multiple PFLs, when UE is configured with time window and does not support FG 41-2-3: - UE performs DL RSCPD/RSCP measurement on the indicated PFL by the network. - Existing legacy requirements without time window apply for both CP measurements and legacy measurements. For UE configured for CPP measurement with legacy measurement with multiple PFLs, when UE is configured with time window and support FG 41-2-3, but does not support FG 41-2-8/41-2-9: - Existing legacy requirements without time window apply for legacy measurements. - CPP measurement requirements apply for CP measurements for the PRS resources associated to the time window have occasions only within the measurement time window. For UE configured for CPP measurement with legacy measurement with multiple PFLs, when UE is configured with time window and support FG 41-2-3 and FG 41-2-8/41-2-9: - CPP measurement requirements apply for both legacy measurements and CP measurements. FFS: Whether and/or how to define the measurement period requirements for DL RSCPD/RSCP measurements when aperiodic time window is configured. ### Issue 2-1-2: Measurement period requirements for DL RSCP/DL RSCPD with multiple PFLs configured Proposals: - Option 1: - When multiple PFLs are configured in the assistance data and DL RSCPD is requested with RSTD, the measurement period is given by TRSCPD with RSTD,Total=TRSCPD with RSTD,j+i=1,i≠jLTRSTD,i+ L-1*maxTeffect,i - where j is the index of the indicated PFL, if provided by the LFM, otherwise the PFL j is selected by UE implementation. - If an aperiodic time window is configured, the start of the measurement period coincides with the start of the time window. - When multiple PFLs are configured in the assistance data and DL RSCP is requested with UE Rx-Tx, the measurement period is given by TRSCP with UERxTx, Total=TRSCP with UERxTx, j+i=1,i≠jLTUERxTx,i+ L-1*maxTeffect,i - where j is the index of the indicated PFL, if provided by the LFM, otherwise the PFL j is selected by UE implementation. - If an aperiodic time window is configured, the start of the measurement period coincides with the start of the time window. - Option 2: - When multiple PFLs are configured where RSCPD is supposed to be measured in PFL j, support option 1 with the following updates (taking RSCPD measurement as the example) - For UE supporting both FG 41-2-3 and FG 42-2-8，TRSCPD with RSTD, j is the measurement period for both RSCPD and RSTD in PFL j by taking time window into account. - For UE supporting FG 41-2-3 only, TRSCPD with RSTD, j=max(TRSCPD, j,TRSTD, j), where TRSCPD, j is the measurement period for RSCPD in PFL j by taking time window into account, and T RSTD, j is the legacy measurement period for RSTD in PFL j without time window. - For UE not supporting FG 41-2-2 or FG 41-2-8, TRSCPD with RSTD, j=TRSTD, j. ### Issue 2-1-3: The impact of carrier frequency offset Proposals: - Option 1: - The accuracy requirements for CP measurement apply provided that the two PRS resources for calculating RSCPD or relative RSCP are located in the same set of symbols after accounting for expected RSTD. - Option 2: - The carrier phase definition should be clarified to indicate the carrier phase is defined at a specific location within the slot. - Define a common reference time and refer the DL-PRS carrier phase measurements to this reference time by subtracting the phase rotation due to the carrier frequency offset in the time interval between the DL-PRS and the reference time for the carrier phase measurement. - Define the referred carrier phase difference as the difference between the referred carrier phase measurements. - Define the same common reference time for the UE and the PRU. - The UE and the PRU report either the referred carrier phase measurements or the carrier phase difference measurements computed using the referred carrier phase measurements. - Option 3: - RAN4 not to define enhancements to mitigate impact of carrier frequency offset on CPP measurements in Rel-18. - RAN4 to define an applicability condition for DL RSCPD accuracy requirements based on the time proximity between the target and reference PRS resources used to calculate the DL RSCPD measurement. - DL RSCPD accuracy requirements are defined and apply when the target and reference PRS resources are measured in the same time slot. FFS the maximum time separation between the resources within a slot. - RSCPD accuracy derived from RAN4 simulations applies assuming zero carrier frequency error at the UE and TRPs. ### Issue 2-1-4: Update measurement period requirements for RSTD, UE Rx-Tx and PRE-RSRP(P) with time window configured Background: Companies mention that the time window can be configured for legacy measurements with RSCPD/RSCP measurements not requested. Hence the measurement period requirements for legacy measurements need to be updated. The issue has been addressed in RAN4#108bis meeting: Agreements from RAN4#108bis meeting: - It is RAN4 understanding that legacy UE positioning measurements performed within time window(s) without CPP configured by LMF is not within the scope of the Rel-18 WI. ### Issue 2-1-5: UE behaviour due to AGC adjustment Agreements: - RAN4 not to define UE behaviour related to AGC adjustment.

  - Measurement period requirements

Requirements with multiple PFLs being configured

Regarding issue 2-1-2, our preference is to go along option 1 in case of multiple configured PFLs, that is to define the measurement period for DL RSCPD with RSTD measurement or DL RSCP with UE Rx-Tx measurement, respectively, for the indicated PFL for CPP complemented by the RSTD / UE Rx-Tx measurement for the remaining PFLs. According to whether the UE supports measurements in time windows or not, the measurement time per PFL is determined.

This yields to following proposal.

RAN4 to base the measurement period definition for multiple configured PFLs on option 1 of issue 2-1-2 in R4-2410193 and define the measurement time per PFL based on the UE capability to support time window for CP / legacy measurements.

Requirements with aperiodic time window

Regarding the FFS in issue 2-1-1, i.e. whether and/or how to define the measurement period requirements for DL RSCPD/RSCP measurements when aperiodic time window is configured, our preference is to define measurement period requirements, as RAN2 has defined this window type under the name “single shot window”. Therefore, we support that the measurement period is equal to the length of the single shot window, i.e. it will start with the configured start of the window (indicated by the slot offset) and cover the duration of the time window, as specified in TS 37.355.

This yields to following proposal.

RAN4 to specify measurement requirements for aperiodic time window, defined in the LPP spec, and to set the measurement period equal to the duration of the time window.

Impact of CFO

Regarding issue 2-1-3, the impact of CFO has been discussed for a number of meetings. Yet, there is no evidence in our view that this impact is negligible. According to our preliminary investigation in [4], which is based on residual CFO assumptions in TR 38.859, as investigated by RAN1 during the study of NR CPP, we observe a non-negligible impact to the measurement accuracy at target UE, which may be mitigated to some degree by double differential carrier phase estimation at LMF. It is recommended that RAN4 studies this impact in greater detail based on common simulation assumptions, as it is yet unclear, whether this impairment can be accommodated in the impairment margin, e.g. in order to evaluate the DL RSCPD measurement accuracy at target UE.

This yields to following proposal.

RAN4 to further investigate, based on common simulation assumptions, how to reduce the impact of CFO to absolute DL RSCPD / relative DL RSCP measurement accuracy.

Conclusion

Impacts to RRM core requirements for NR carrier phase positioning (NR CPP) being one of the objectives in the Rel-18 WID on expanded and improved positioning [1], are investigated in this contribution related to open core aspects in [2].

The following proposals are made.

- RAN4 to base the measurement period definition for multiple configured PFLs on option 1 of issue 2-1-2 in R4-2410193 and define the measurement time per PFL based on the UE capability to support time window for CP / legacy measurements.

RAN4 to specify measurement requirements for aperiodic time window, defined in the LPP spec, and to set the measurement period equal to the duration of the time window.

RAN4 to further investigate, based on common simulation assumptions, how to reduce the impact of CFO to absolute DL RSCPD / relative DL RSCP measurement accuracy.

- References

- RP-233382, “Revised WID on Expanded and Improved NR positioning”, 3GPP TSG RAN#102

- R4-2410193, “WF on SL positioning and carrier phase positioning”, CATT

- R4-2408010, “Topic summary for [111][213] NR_pos_enh2_part2”, Moderator (CATT)

- R4-2413313, “Simulation results for DL RSCPD and DL RSCP”, Nokia