3GPP TSG-RAN4 Meeting #112	R4-2413473

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.181 | CR | 0032 | rev | 1 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_NTN_enh-Perf) CR on PUSCH demodulation requirements for 38.181 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_enh-Perf |  |  |  |  |  | Date: |  |  | 2024-08-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Performance requirements not defined for PUSCH with DMRS bundling requirements |  |  |  |  |  |  |  |  |
| Summary of change: |  | Inclusion of some results from R4-2408978 in square brackets All of the requirements will require revision based on simulation updates during RAN4#112, therefore this CR will require revision during meeting. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirements will not be defined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.5, 11.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR .... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from R4-2411039 |  |  |  |  |  |  |  |  |

# <Start of Change 1>

### 8.2.5	Performance requirements for PUSCH with DMRS bundling

#### 8.2.5.1	Definition and applicability

The performance requirement of PUSCH with DMRS bundling is determined by a minimum required throughput for a given SNR. The required throughput is expressed as a fraction of maximum throughput for the FRCs listed in annex A. The performance requirements assume HARQ re-transmissions.

Which specific test(s) are applicable to SAN is based on the test applicability rules defined in clause [8.1.2.9].

#### 8.2.5.2	Minimum Requirement

The minimum requirement is in TS 38.108 [2] clause [8.2.5].

#### 8.2.5.3	Test Purpose

The test shall verify the receiver's ability to achieve throughput under multipath fading propagation conditions for a given SNR with DMRS bundling.

#### 8.2.5.4	Method of test

##### 8.2.5.4.1	Initial Conditions

Test environment:	Normal, see annex [B.2].

RF channels to be tested for single carrier:	M; see clause [4.9.1].

RF channels to be tested for carrier aggregation: MBW Channel CA; see clause [4.9.1].

##### 8.2.5.4.2	Procedure

1)	Connect the SAN tester generating the wanted signal, multipath fading simulators and AWGN generators to all SAN TAB connectors for diversity reception via a combining network as shown in annex D.6 for SAN type 1-H.

2)	Adjust the AWGN generator, according to the channel bandwidth, defined in table 8.2.5.4.2-1.

Table 8.2.5.4.2-1: AWGN power level at the SAN input

| Sub-carrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- |
| 15kHz | 5 | -86.5 dBm / 4.5MHz |
| 30kHz | 10 | -83.6 dBm / 8.64MHz |
| NOTE: The AWGN power level contains an AWGN offset of 16dB by default. If needed for test purposes, the AWGN level can be reduced from the default by any value in the range 0dB to 16dB. Changing the AWGN level does not impact the validity of the test, as it reduces the effective base band SNR level. |  |  |

3)	The characteristics of the wanted signal shall be configured according to the corresponding UL reference measurement channel defined in annex A and the test parameters in table 8.2.5.4.2-2.

Table 8.2.5.4.2-2: Test parameters for testing PUSCH with DMRS bundling

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence (Note 1) | 0,0,0,0 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port(s) | {0} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain resource assignment | PUSCH mapping type | A, B |
|  | Start symbol | 0 |
|  | Allocation length | 14 |
|  | PUSCH aggregation factor | n4 for 15kHz SCS n8 for 30kHz SCS |
| pusch-TimeDomainWindowLength |  | 4 for 15kHz SCS 8 for 30kHz SCS |
| Frequency domain resource assignment | RB assignment | 6 RBs in the middle of the channel bandwidth |
|  | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
| NOTE 1: The effective RV sequence is {0, 2, 3, 1} with slot aggregation. |  |  |

4)	The multipath fading emulators shall be configured according to the corresponding channel model defined in annex G.2.

5)	Adjust the equipment so that required SNR specified in table 8.2.5.5-1 to table 8.2.5.5-4 is achieved at the SAN input.

6)	For each of the reference channels in table 8.2.5.5-1 to table 8.2.5.5-4 applicable for the base station, measure the throughput.

#### 8.2.5.5	Test Requirement

The throughput measured according to clause 8.2.5.4.2 shall not be below the limits for the SNR levels specified in table 8.2.5.5-1 to table 8.2.5.5-4.

Table 8.2.5.5-1: Test requirements for PUSCH with 70% of maximum throughput, Type A, 5 MHz channel bandwidth, FR1-NTN, 15 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (annex G) | Fraction of maximum throughput | FRC (annex G) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | Normal | NTN-TDLA100-200 Low | 70 % | G-FR1-NTN-A3-7 | pos1 | [-0.1] |
|  | 2 | Normal | NTN-TDLA100-200 Low | 70 % | G-FR1-NTN-A3-7 | pos1 | [-3.6] |

Table 8.2.5.5-2: Test requirements for PUSCH with 70% of maximum throughput, Type A, 10 MHz channel bandwidth, FR1-NTN, 30 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (annex G) | Fraction of maximum throughput | FRC (annex G) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | Normal | NTN-TDLA100-200 Low | 70 % | G-FR1-NTN-A3-8 | pos1 | [-2.9] |
|  | 2 | Normal | NTN-TDLA100-200 Low | 70 % | G-FR1-NTN-A3-8 | pos1 | [-6.1] |

Table 8.2.5.5-3: Test requirements for PUSCH with 70% of maximum throughput, Type B, 5 MHz channel bandwidth, FR1-NTN, 15 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (annex G) | Fraction of maximum throughput | FRC (annex G) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | Normal | NTN-TDLA100-200 Low | 70 % | G-FR1-NTN-A3-7 | pos1 | [-0.0] |
|  | 2 | Normal | NTN-TDLA100-200 Low | 70 % | G-FR1-NTN-A3-7 | pos1 | [-3.5] |

Table 8.2.5.5-4: Test requirements for PUSCH with 70% of maximum throughput, Type B, 10 MHz channel bandwidth, FR1-NTN, 30 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (annex G) | Fraction of maximum throughput | FRC (annex G) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | Normal | NTN-TDLA100-200 Low | 70 % | G-FR1-NTN-A3-8 | pos1 | [-2.8] |
|  | 2 | Normal | NTN-TDLA100-200 Low | 70 % | G-FR1-NTN-A3-8 | pos1 | [-6.1] |

<End of Change 1>

# <Start of Change 2>

### 11.2.5	Performance requirements for PUSCH with DMRS bundling

#### 11.2.5.1	Definition and applicability

The performance requirement of PUSCH with DMRS bundling is determined by a minimum required throughput for a given SNR. The required throughput is expressed as a fraction of maximum throughput for the FRCs listed in annex A. The performance requirements assume HARQ re-transmissions.

Which specific test(s) are applicable to SAN is based on the test applicability rules defined in clause [11.1.3].

#### 11.2.5.2	Minimum Requirement

For SAN type 1-O, the minimum requirement is in TS 38.108 [2], clause [11.2.2.1].

#### 11.2.5.3	Test purpose

The test shall verify the receiver's ability to achieve throughput under multipath fading propagation conditions for a given SNR with DMRS bundling.

#### 11.2.5.4	Method of test

##### 11.2.5.4.1	Initial conditions

Test environment: Normal, see annex [B.2].

RF channels to be tested for single carrier: M, see clause [4.9.1.]

Direction to be tested: OTA REFSENS receiver target reference direction (see [D.54] in table 4.6-1).

##### 11.2.5.4.2	Procedure

1)	Place the SAN with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex [D.7].

2)	Align the manufacturer declared coordinate system orientation of the SAN with the test system.

3)	Set the SAN in the declared direction to be tested.

4)	Connect the SAN tester generating the wanted signal, multipath fading simulators and AWGN generators to a test antenna via a combining network in OTA test setup, as shown in annex E.3. Each of the demodulation branch signals should be transmitted on one polarization of the test antenna(s).

5)	The characteristics of the wanted signal shall be configured according to the corresponding UL reference measurement channel defined in annex A, and according to additional test parameters listed in table 11.2.5.4.2-1.

Table 11.2.5.4.2-1: Test parameters for testing PUSCH

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence (Note 1) | 0,0,0,0 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port(s) | {0} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain resource assignment | PUSCH mapping type | A, B |
|  | Start symbol | 0 |
|  | Allocation length | 14 |
|  | PUSCH aggregation factor | n4 for 15kHz SCS n8 for 30kHz SCS |
| pusch-TimeDomainWindowLength |  | 4 for 15kHz SCS 8 for 30kHz SCS |
| Frequency domain resource assignment | RB assignment | 6 RBs in the middle of the channel bandwidth |
|  | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
| SAN type |  | 1-O |
| NOTE 1: The effective RV sequence is {0, 2, 3, 1} with slot aggregation. |  |  |

6)	The multipath fading emulators shall be configured according to the corresponding channel model defined in annex J.

7)	Adjust the test signal mean power so the calibrated radiated SNR value at the SAN receiver is as specified in clause 11.2.5.5.1 for SAN type 1-O, and that the SNR at the SAN receiver is not impacted by the noise floor.

The power level for the transmission may be set such that the AWGN level at the RIB is equal to the AWGN level in table 11.2.5.4.2-2.

Table 11.2.5.4.2-2: AWGN power level at the SAN input

| SAN Type | Sub-carrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- | --- |
| Type 1-O | 15 | 5 | -86.5 - ΔOTAREFSENS dBm / 4.5 MHz |
|  | 30 | 10 | -83.6 - ΔOTAREFSENS dBm / 8.64 MHz |
| NOTE 1: ΔOTAREFSENS as declared in D.53 in table 4.6-1 and clause 7.1. NOTE 2: The AWGN power level contains an AWGN offset of 15dB by default. If needed for test purposes, the AWGN level can be reduced from the default by any value in the range 0dB to 15dB. Changing the AWGN level does not impact the validity of the test, as it reduces the effective base band SNR level.. |  |  |  |

8)	For reference channels applicable to the SAN, measure the throughput.

#### 11.2.5.5	Test Requirement

##### 11.2.5.5.1	Test requirement for SAN type 1-O

The throughput measured according to clause 11.2.5.4.2 shall not be below the limits for the SNR levels specified in table 11.2.5.5.1-1 ~ 11.2.5.5.1.4

Table 11.2.5.5.1-1: Test requirements for PUSCH with 70% of maximum throughput, Type A, 5 MHz channel bandwidth, FR1-NTN, 15 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (annex G) | Fraction of maximum throughput | FRC (annex G) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | Normal | NTN-TDLA100-200 Low | 70 % | G-FR1-NTN-A3-7 | pos1 | [-0.1] |
|  | 2 | Normal | NTN-TDLA100-200 Low | 70 % | G-FR1-NTN-A3-7 | pos1 | [-3.6] |

Table 11.2.5.5.1-2: Test requirements for PUSCH with 70% of maximum throughput, Type A, 10 MHz channel bandwidth, FR1-NTN, 30 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (annex G) | Fraction of maximum throughput | FRC (annex G) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | Normal | NTN-TDLA100-200 Low | 70 % | G-FR1-NTN-A3-8 | pos1 | [-2.9] |
|  | 2 | Normal | NTN-TDLA100-200 Low | 70 % | G-FR1-NTN-A3-8 | pos1 | [-6.1] |

Table 11.2.5.5.1-3: Test requirements for PUSCH with 70% of maximum throughput, Type B, 5 MHz channel bandwidth, FR1-NTN, 15 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (annex G) | Fraction of maximum throughput | FRC (annex G) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | Normal | NTN-TDLA100-200 Low | 70 % | G-FR1-NTN-A3-7 | pos1 | [-0.0] |
|  | 2 | Normal | NTN-TDLA100-200 Low | 70 % | G-FR1-NTN-A3-7 | pos1 | [-3.5] |

Table 11.2.5.5.1-4: Test requirements for PUSCH with 70% of maximum throughput, Type B, 10 MHz channel bandwidth, FR1-NTN, 30 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (annex G) | Fraction of maximum throughput | FRC (annex G) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | Normal | NTN-TDLA100-200 Low | 70 % | G-FR1-NTN-A3-8 | pos1 | [-2.8] |
|  | 2 | Normal | NTN-TDLA100-200 Low | 70 % | G-FR1-NTN-A3-8 | pos1 | [-6.1] |

<End of Change 2>