3GPP TSG-RAN4 Meeting #112	R4-2413497

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | 0573 | rev | 1 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 38.101-4 on FDD 4Rx requirements for advanced receiver for MU-MIMO |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_demod_enh3-Perf |  |  |  |  |  | Date: |  |  | 2024-08-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Requirements for MU-MIMO with Enhanced Receiver Type 2 were tentatively agreed and introduced in spec with “[ ]”. Update the results from updated simultion results. “[ ]” to be removed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated requirements based on simultion results removed square brackets Minor editorial corrections |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirements will not be based on latest simultion results. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.3.1.16 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-4 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from R4-2411380 |  |  |  |  |  |  |  |  |

##### 5.2.3.1.16	Minimum requirements for PDSCH with intra-cell inter-user interference

The performance requirements are specified in Table 5.2.3.1.16-3 and Table 5.2.3.1.16-4, with the addition of test parameters in Table 5.2.3.1.16-2 and the downlink physical channel setup according to Annex C.3.1.

The performance requirements for UE supporting Enhanced receiver Type 2 for intra-cell inter-user interference are specified in Table 5.2.3.1.16-6 and Table 5.2.3.1.16-7, with the addition of test parameters in Tables 5.2.3.1.16-2, 5.2.3.1.16-5 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2.3.1.16-1.

Table 5.2.3.1.16-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify PDSCH performance under 4 receive antenna conditions, when the PDSCH transmission of target UE is interfered by co-scheduled UE. | 1-1, 2-1 |
| Verify PDSCH performance under 4 receive antenna conditions, when the PDSCH transmission of target UE is interfered by co-scheduled UE with Enhanced receiver Type 2 when modulation order for co-scheduled UE is explicitly signaled by DCI. | 3-1, 4-1 |
| Verify PDSCH performance under 4 receive antenna conditions, when the PDSCH transmission of target UE is interfered by co-scheduled UE with Enhanced receiver Type 2 when modulation order for co-scheduled UE is detected. | 3-2, 4.2 |

Table 5.2.3.1.16-2: Test parameters

| Parameter |  | Unit | Target UE | Co-scheduled UE |
| --- | --- | --- | --- | --- |
| Duplex mode |  |  | FDD |  |
| Active DL BWP index |  |  | 1 |  |
| PDSCH configuration | Mapping type |  | Type A |  |
|  | k0 |  | 0 |  |
|  | Starting symbol (S) |  | 2 |  |
|  | Length (L) |  | 12 |  |
|  | PDSCH aggregation factor |  | 1 |  |
|  | PRB bundling type |  | Static |  |
|  | PRB bundling size |  | 2 |  |
|  | Resource allocation type |  | Type 0 |  |
|  | RBG size |  | Config2 |  |
|  | VRB-to-PRB mapping type |  | Non-interleaved |  |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |  |
| PDSCH DMRS configuration (Note 1) | DMRS Type |  | Type 1 |  |
|  | Number of additional DMRS |  | 1 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |  |
|  | Antenna ports indexes |  | {1000} for test 1-1, 3-1, 3-2 {1000, 1001} for test 2-1, 4-1, 4-2 | {1001} for test 1-1, 3-1, 3-2 {1002, 1003} for test 2-1, 4-1, 4-2 |
|  | Number of PDSCH DMRS CDM group(s) without data |  | 1 for test 1-1, 3-1, 3-2 2 for test 2-1, 4-1, 4-2 | 1 for test 1-1, 3-1, 3-2 2 for test 2-1, 4-1, 4-2 |
| PDSCH & PDSCH DMRS Precoding configuration |  |  | Single Panel Type I, Randomized precoder selection for every PRB bundle and updated per slot, with equal probability of each applicable i1/i2 combination or codebook Index, chosen from section 5.2.2.2.1 of TS 38.214 [12]. | Single Panel Type I, Randomized precoder selection for every PRB bundle and updated per slot, with equal probability of each applicable i1/i2 combination or codebook Index, chosen from section 5.2.2.2.1 of TS 38.214 [12]. Any column of precoder matrix is not equal to any column of precoder matrix of Target UE for test 1-1 Select the precoder to ensure any column of precoder is orthogonal to any column of precoder for the target PDSCH for test 2-1, 3-1, 3-2, 4-1, 4-2 |
| MU-MIMO Beamforming Model |  |  | As specified in B.4.2 |  |
| Number of HARQ Processes |  |  | 4 | N/A |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | 2 | N/A |
| Note 1: DMRS scrambling ID is the same for both target and co-scheduled UEs. |  |  |  |  |

Table5.2.3.1.16-3: Minimum performance for target UE with Rank 1

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate |  | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Target UE | Co-scheduled UE |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.5-1.1 FDD | 10 / 15 | 16QAM, 0.48 | Random 16QAM symbols | TDLC300-100 | 2x4, ULA Low | 70 | 11.5 |

Table5.2.3.1.16-4: Minimum performance for target UE with Rank 2

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate |  | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Target UE | Co-scheduled UE |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 2-1 | R.PDSCH.5-1.2 FDD | 10 / 15 | 16QAM, 0.48 | Random 16QAM symbols | TDLA30-10 | 4x4, ULA Low | 70 | 15.3 |

The parameters in Table  5.2.3.1.16-5 are configured for requirements with Enhanced receiver Type 2 for intra-cell inter-user interference.

Table 5.2.3.1.16-5: Assitance Information parameters for requirements with Enhanced receiver Type 2

| Parameter |  | Value |
| --- | --- | --- |
| AdvancedReceiver-MU-MIMO-r18 | precodingAndResourceAllocation | True |
|  | pdsch-TimeDomainAllocation | True |
|  | mcs-Table | qam256 |
|  | advReceiver-MU-MIMO-DCI-1-1 | Enabled |
| Co-scheduled UE information in DCI (Table 7.3.1.2.2-12 of TS38.212) |  | 1 for Test 3-1 2 for Test 4-1 6 for Test 3-2, 4-2 |

Table 5.2.3.1.16-6: Minimum performance for target UE with Rank 1 with Enhanced receiver Type 2

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate |  | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Target UE | Co-scheduled UE |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 3-1 | R.PDSCH.5-1.1 FDD | 10 / 15 | 16QAM, 0.48 | Random QPSK symbols | TDLC300-100 | 2x4, ULA Medium | 70 | 15.5 |
| 3-2 | R.PDSCH.5-1.3 FDD | 10 / 15 | 64QAM, 0.43 | Random 16QAM symbols | TDLC300-100 | 2x4, ULA Medium | 70 | 24.2 |

Table5.2.3.1.16-7: Minimum performance for target UE with Rank 2 with Enhanced receiver Type 2

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate |  | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Target UE | Co-scheduled UE |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 4-1 | R.PDSCH.5-1.4 FDD | 10 / 15 | 64QAM, 0.43 | Random 16QAM symbols | TDLA30-10 | 4x4, ULA Low | 70 | 19.4 |
| 4-2 | R.PDSCH.5-1.2 FDD | 10 / 15 | 16QAM, 0.48 | Random QPSK symbols | TDLA30-10 | 4x4, ULA Low | 70 | 14.3 |